Reg.No. ____________

End Semester Examination – Nov/Dec – 2018

| Code : | 18ME2004 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | MACHINE DESIGN | Max. marks : | 100 |

| Q. No. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- |
|  | PART-A(20X1=20 MARKS) |  |  |
| 1. | Define factor of safety. | CO3 | 1 |
| 2. | What is eccentric load and eccentricity? | CO1 | 1 |
| 3. | List out the factors involved in arriving at factor of safety | CO3 | 1 |
| 4. | Why normal stress theory is not suitable for ductile materials? | CO1 | 1 |
| 5. | What are the factors affecting selection of material for machine element? | CO3 | 1 |
| 6. | How hollow shaft are superior to solid shaft? | CO4 | 1 |
| 7. | Is what situation flexible coupling are used? | CO4 | 1 |
| 8. | Why a hallow shaft has great strength and stiffness then solid shaft of equal weight? | CO2 | 1 |
| 9. | What is the effect of keyway cut into the shaft? | CO2 | 1 |
| 10. | What is a stud? | CO2 | 1 |
| 11. | How is welding classified? | CO2 | 1 |
| 12. | Name the possible modes of failure of riveting joint | CO3 | 1 |
| 13. | Define Resilience of a spring | CO1 | 1 |
| 14. | What is the function of a flywheel? | CO1 | 1 |
| 15. | State any two important applications of leaf spring. | CO2 | 1 |
| 16. | Classify the helical springs. | CO6 | 1 |
| 17. | State the different failure theories and the type of materials for which these are applicable. | CO2 | 1 |
| 18. | Define the term ‘fluctuation of speed’ and ‘fluctuation of energy | CO5 | 1 |
| 19. | What is known as self – acting bearing? | CO6 | 1 |
| 20. | Name the type of lubricant used in journal bearing. | CO6 | 1 |

|  | PART B(10 X 5= 50 MARKS) (Answer any 10 from the following) |  |  |
| --- | --- | --- | --- |
| 21. | Explain the general Procedure in Machine Design. | CO2 | 5 |
| 22. | Write note on types of Fits. | CO3 | 5 |
| 23. | A hollow shaft of 40 mm outer diameter and 25 mm inner diameter is subjected to a twisting moment of 120 N-m, simultaneously, it is subjected to an axial thrust of 10 kN and a bending moment of 80 N-m. Calculate the maximum compressive and shear stresses. | CO4 | 5 |
| 24. | The maximum load on a petrol engine push rod 300 mm long is 1400 N. It is hollow having the outer diameter 1.25 times the inner diameter. Spherical seated bearings are used for the push rod. The modulus of elasticity for the material of the push rod is 210 kN/mm2. Find a suitable size for the push rod, taking a factor of safety of 2.5. | CO2 | 5 |
| 25. | A helical torsion spring of mean diameter 60 mm is made of a round wire of 6 mm diameter. If a torque of 6 N-m is applied on the spring, find the bending stress induced and the angular deflection of the spring in degrees. The spring index is 10 and modulus of elasticity for the spring material is 200 kN/mm2. The number of effective turns may be taken as 5.5. | CO5 | 5 |
| 26. | A spiral spring is made of a flat strip 6 mm wide and 0.25 mm thick. The length of the strip is 2.5 metres. Assuming the maximum stress of 800 MPa to occur at the point of greatest bending moment, calculate the bending moment, the number of turns to wind up the spring and the strain energy stored in the spring. Take E = 200 kN/mm2. | CO5 | 5 |
| 27. | The load on the journal bearing is 150 kN due to turbine shaft of 300 mm diameter running at 1800 r.p.m. Determine the following. - Length of the bearing if the allowable bearing pressure is 1.6 N/mm2, and - Amount of heat to be removed by the lubricant per minute if the bearing temperature is 60°C and viscosity of the oil at 60°C is 0.02 kg/m-s and the bearing clearance is 0.25 mm. | CO1 | 5 |
| 28. | A 150 mm diameter shaft supporting a load of 10 kN has a speed of 1500 r.p.m. The shaft runs in a bearing whose length is 1.5 times the shaft diameter. If the diametral clearance of the bearing is 0.15 mm and the absolute viscosity of the oil at the operating temperature is 0.011 kg/m-s, find the power wasted in friction | CO2 | 5 |
| 29. | A leather belt 9 mm × 250 mm is used to drive a cast iron pulley 900 mm in diameter at 336 r.p.m. If the active arc on the smaller pulley is 120° and the stress in tight side is 2 MPa, find the power capacity of the belt. The density of leather may be taken as 980 kg/m3, and the coefficient of friction of leather on cast iron is 0.35. | CO2 | 5 |
| 30. | A belt 100 mm wide and 10 mm thick is transmitting power at 1000 metres/min. The net driving tension is 1.8 times the tension on the slack side. If the safe permissible stress on the belt section in 1.6 MPa, calculate the maximum power, that can be transmitted at this speed. Assume density of the leather as 1000 kg/m3. Calculate the absolute maximum power that can be transmitted by this belt and the speed at which this can be transmitted. | CO3 | 5 |
| 31. | Determine the thickness of a 120 mm wide uniform plate for safe continuous operation if the plate is to be subjected to a tensile load that has a maximum value of 250 kN and a minimum value of 100 kN. The properties of the plate material are as follows: Endurance limit stress = 225 MPa, and Yield point stress = 300 MPa. The factor of safety based on yield point may be taken as 1.5. | CO5 | 5 |
| 32. | A steel bar 2.4 m long and 30 mm square is elongated by a load of 500 kN. If poisson's ratio is 0.25, find the increase in volume. Take E = 0.2 × 106 N/mm2. | CO6 | 5 |

|  | PART C(2 X 15= 30 MARKS) (Answer any 2 from the following) |  |  |  |
| --- | --- | --- | --- | --- |
| 33. | a. | A steel shaft 35 mm in diameter and 1.2 m long held rigidly at one end has a hand wheel 500 mm in diameter keyed to the other end. The modulus of rigidity of steel is 80 GPa. - What load applied to tangent to the rim of the wheel produce a torsional shear of 60 MPa? - How many degrees will the wheel turn when this load is applied? | CO1 | 8 |
|  | b. | A hollow shaft is required to transmit 600 kW at 110 r.p.m., the maximum torque being 20% greater than the mean. The shear stress is not to exceed 63 MPa and twist in a length of 3 metres not to exceed 1.4 degrees. Find the external diameter of the shaft, if the internal diameter to the external diameter is 3/8. Take modulus of rigidity as 84 GPa | CO2 | 7 |
| 34. | a. | Design a sleeve and cotter joint to resist a tensile load of 60 kN. All parts of the joint are made of the same material with the following allowable stresses : σt = 60 MPa ; τ = 70 MPa ; and σc = 125 MPa. | CO5 | 8 |
|  | b. | A steel spindle transmits 4 kW at 800 r.p.m. The angular deflection should not exceed 0.25° per metre of the spindle. If the modulus of rigidity for the material of the spindle is 84 GPa, find the diameter of the spindle and the shear stress induced in the spindle. | CO1 | 7 |
| 35. | a. | A vertical screw with single start square threads of 50 mm mean diameter and 12.5 mm pitch is raised against a load of 10 kN by means of a hand wheel, the boss of which is threaded to act as a nut. The axial load is taken up by a thrust collar which supports the wheel boss and has a mean diameter of 60 mm. The coefficient of friction is 0.15 for the screw and 0.18 for the collar. If the tangential force applied by each hand to the wheel is 100 N, find suitable diameter of the hand wheel. | CO5 | 8 |
|  | b. | A flat belt is required to transmit 30 kW from a pulley of 1.5 m effective diameter running at 300 r.p.m. The angle of contact is spread over 11/24 of the circumference. The coefficient of friction between the belt and pulley surface is 0.3. Determine, taking centrifugal tension into account, width of the belt required. It is given that the belt thickness is 9.5 mm, density of its material is 1100 kg /m3 and the related permissible working stress is 2.5 MPa. | CO6 | 7 |