Reg.No. ____________

End Semester Examination – Nov/Dec – 2018

| Code : | 14AE2014 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | AIRCRAFT PERFORMANCE | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Draw the force diagram of airplane during a steady level flight. | CO1 | 4 |
|  | b. | Define gliding. Explain in detail about gliding flight with neat diagram. | CO3 | 16 |
| (OR) |  |  |  |  |
| 2. | a. | Consider the flow over a small plate that is 5cm long in the flow direction and 1cm wide. The free stream conditions correspond to standard sea level. The flow velocity is 120 m/s and the viscosity µ = 1.789 × 10-5 kg/m.s . Assuming laminar flow, calculate - The boundary layer thickness at the downstream edge (the trailing edge) - The drag force on the plate | CO1 | 10 |
|  | b. | Discuss about the types of drag that act upon an aircraft. | CO1 | 10 |
| 3. | a. | Describe the concept of power available and power required for propeller driven aircraft. | CO3 | 10 |
|  | b. | Explain in detail about Takeoff performance of flight with neat sketch. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Derive an expression for endurance of a Propeller-driven airplane. | CO3 | 10 |
|  | b. | A single engine propeller driven airplane has following characteristics: span =10.91 m, wing area = 16.16 m2 , W = 1340 kg, e = 0.8, CDo = 0.022. Calculate the thrust required at 12000 m for flight velocity of 120 m/s. | CO3 | 10 |
| 5. | a. | Derive the Breguet range equation with suitable assumptions. | CO3 | 15 |
|  | b. | Define service and absolute ceilings. | CO1 | 5 |
| (OR) |  |  |  |  |
| 6. |  | Define V-n diagram and explain it with neat sketch. | CO3 | 20 |
| 7. | a. | Describe the various phases of take-off flight and derive the expression to estimate the take-off ground roll distance of an aeroplane. | CO3 | 10 |
|  | b. | Derive the expression to estimate the landing ground roll distance. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. |  | Write short notes on: a. Fixed pitch propeller. b. Variable pitch propeller. c. Thrust augmentation. | CO2 CO2 CO1 | 5 5 10 |
|  |  | Compulsory: |  |  |
| 9. |  | Derive the equations of motion for an airplane in steady, level flight. | CO1 | 20 |