Reg.No. ____________

End Semester Examination – Nov/Dec – 2018

| Code : | 14AE2015 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | AIRCRAFT STABILITY AND CONTROL | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Derive an expression for the wing contribution to the pitching moment of an aircraft about center of gravity. | CO1 | 10 |
|  | b. | A wing body model is tested in a subsonic wind tunnel. The lift is found to be zero at a geometric α = -1.5º .At α = 5º, the CL is measured as 0.52. Also at α = 1º and 7.88º, the CMcg are measured as -0.01 and 0.05 respectively. The c.g is located at 0.35 C. calculate the location of Aerodynamic Center and the value of CMac,wb | CO1 | 6 |
|  | c. | Why the airplane is considered as a dynamic system in six degrees of freedom? | CO1 | 4 |
| (OR) |  |  |  |  |
| 2. | a. | Describe the criteria for longitudinal static stability. Explain the contributions of fuselage and nacelles to Cmcg and Cmα. | CO1 | 12 |
|  | b. | Distinguish between static and dynamic stability | CO1 | 4 |
|  | c. | The dynamic pressure over tail is different from the free stream dynamic pressure. True/False. Justify your answer | CO1 | 4 |
| 3. | a. | Briefly explain the methods used to improve the stick force gradient? | CO2 | 8 |
|  | b. | Explain the various types of aerodynamic balancing of a control surface with neat sketches | CO2 | 10 |
|  | c. | Differentiate Stick fixed and Stick free longitudinal stability | CO2 | 2 |
| (OR) |  |  |  |  |
| 4. | a. | For an airplane the pitching moment co-efficient is given by the following expression when c.g. lies at 0.25 c. Cmcg = 0.05- 0.10CL-0.01δe where δe is in degrees. Determine the following: - Is the airplane statically stable? Justify. - What is the static margin? - What is the location of neutral point stick fixed? - What is the value of CL for which equilibrium is achieved with zero elevator deflection? - What is the value of elevator effectiveness (Cmδ)? - If the elevator deflection is limited to ± 25º, locate the most forward c.g. location for which trim is obtained at CL=1.5 in free light. | CO2 | 15 |
|  | b. | Write a brief note on Elevator hinge moment. | CO2 | 5 |
| 5. | a. | Explain the effect of dihedral on static lateral stability with neat sketch. | CO2 | 10 |
|  | b. | Briefly explain the contribution of fuselage and vertical tail on static lateral stability. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Based on strip theory derive an expression for aileron control power. | CO2 | 12 |
|  | b. | Discuss briefly on: Aileron reversal and Adverse yaw. | CO2 | 8 |
| 7. | a. | Discuss in detail the contribution of various components of the airplane on static directional stability. | CO2 | 12 |
|  | b. | Describe in detail basic requirements of the rudder. | CO2 | 8 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the phenomenon of rudder lock and methods to prevent rudder lock. | CO3 | 10 |
|  | b. | Explain: Weather cocking effect and One engine inoperative condition. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss in detail the phenomena of autorotation and spin and explain how the pilot can recover from that situation. | CO3 | 10 |
|  | b. | Explain the stability derivatives in longitudinal dynamics. | CO3 | 5 |
|  | c. | Explain: Dutch roll and Spiral instability. | CO3 | 5 |