Reg.No. ____________

End Semester Examination – Nov/Dec – 2018

| Code : | 14AE2030 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | BASICS OF AEROSPACE ENGINEERING | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain the contribution of Otto Lilienthal to the early developments of aviation. | CO1 | 15 |
|  | b. | What are the uses of whirling arm apparatus and wind tunnels? | CO2 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Differentiate Biplane and Monoplane. Illustrate the advantages of Monoplane over Biplanes. | CO1 | 15 |
|  | b. | Write a note on the silver disc inscription of sir George Cayley. | CO1 | 5 |
| 3. | a. | Explain the three principal movements of an aircraft and how they are performed with neat sketches. | CO2 | 15 |
|  | b. | Draw symmetrical aerofoil and explain its nomenclature. | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Briefly explain the working of a pitot tube and how it is used to actuate an airspeed indicator for velocity measurements. | CO2 | 15 |
|  | b. | Name 3 gyroscopic instruments used in an aircraft and state its uses. | CO2 | 5 |
| 5. | a. | Explain the use of non-metallic materials used for the construction of aircraft structures over the years. | CO2 | 15 |
|  | b. | State the use of thrust reversers and spoilers in aircrafts. | CO2 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the different control surfaces on a wing and explain its operation with sketches. | CO2 | 15 |
|  | b. | Illustrate the use of Slats and Flaps in high lift generation. | CO2 | 5 |
| 7. | a. | Explain the Ideal disk theory of propeller. | CO2 | 10 |
|  | b. | Draw and explain the components of a liquid propellant rocket engine. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the working principle of aircraft engine used for propelling. | CO2 | 15 |
|  | b. | Draw the stress-strain curve of a typical composite material. | CO2 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain the working of a turbofan engine with a neat sketch. | CO1 | 15 |
|  | b. | Justify why the thrust generated by a turboprop engine is more than a turbojet engine at low attitudes. | CO1 | 5 |