Reg.No. ____________

End Semester Examination – Nov/Dec – 2018

| Code : | 14BT2049 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | PROCESS EQUIPMENT DESIGN | Max. Marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

(Additional  graph sheet permitted)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Describe on the Engineering materials properties and give its merit and demerits. | CO1 | 10 |
|  | b. | Write down the various unit operations symbol with neat sketch. | CO2 | 10 |
| (OR) |  |  |  |  |
| 2 | a. | Distinguish between metal and non-metal physical properties. | CO1 | 6 |
|  | b. | Illustrate the citric acid plant layout with neat sketch and What are the main criteria required for location of plant layout. | CO1 | 14 |
| 3 | a. | Describe the design procedure of venturi-meter with neat sketch. | CO3 | 10 |
|  | b. | Discuss the working principles of gate valve and globe valve with sketch. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4 | a. | A fluid jet discharging from a 50mm diameter has a diameter 40mm at its vena contract. Find the coefficient of contraction. | CO3 | 5 |
|  | b. | A venturimeter with a throat diameter of 30cm and inlet opening dia of 60 cm is inserted in a pipe line of 60cmcarrying dry chloride gas. The pressure differential across the venturi meter as indicated by a U tube mercury manometer is 1.3 cm of Hg. Calculate the flow rate of chlorine in kg/ minutes. Where density of chlorine = 0.009 gm/cc and density of mercury = 13.6 g. m/cc. | CO3 | 15 |
| 5 | a. | In a double pipe heat exchanger , one fluid enters at 120 ºF and leaves at 500 ºF .The other fluid enters at 900 ºF and leaves at 520 ºF. Find LMTD for counter flow and parallel flow? | CO2 | 5 |
|  | b. | Cold fluid is flowing through the heat exchanger at a rate of 15 m3/hr. It enters the heat exchanger at 303K and leaves at 328K.The hot thermic fluid enters the heat exchanger at the rate of 21m3/hr at a temperature of 388K .Find out the area of heat transfer required assuming the flow is counter current and overall heat transfer coefficient be 2490 W/m2 k. Data : Density of cold fluid= 1000 kg /m3, density of thermic fluid = 950 kg/m3,Specific heat of cold fluid= 4.187 kJ/kg.K, Specific heat of thermic fluid=2.93 kJkg.K. Draw a neat block diagram. | CO2 | 15 |
| (OR) |  |  |  |  |
| 6. |  | Calculate the amount of steam required for concentrating the solution of caustic soda from 28% by weight of solids to 40% by weight of solids in a single effect evaporator. The feed rate in 25000 kg / hr and its temperature is 60 ºC. .The absolute pressure in the evaporator is 0.2 kg/ cm2.Saturated steam at 1.4 kg/ cm2 is to be used as heating medium. The elevation in boiling points is 25 ºC. If the overall heat transfer coefficient is 670 kcal/ hrm2 ºC. Calculate the heating surface required and steam economy. The enthalpy data for various streams are as follows. Vapour at 0.2 kg/ cm2 = 623 kcal/ kg. 28% NaOH at 60ºC = 50 kcal / kg, 40% NaOH at 85 ºC =90 kcal / kg. Latent heat of steam at 1.4 kg/ cm2=534 kcal / kg. | CO2 | 20 |
| 7. | a. | A mixture of 35mole % A and 65Mole % B is to be separated in a fractionating column. The concentration of A in the distillate is 93Mole % and 65% of all product A is in the distillate. The feed is half vapor and the reflux ratio is to be 4.0.The relative volatility of A to B is 2.0. Calculate the number of theoretical plates in the column and locate the feed plate. | CO2 | 10 |
|  | b. | A bubble cap fractionating column of 12 plates working at an average efficiency of 75% is being used to distill. 1000kg/hr of aqueous methanol at its bubble point enters the tower. The feed, over head product and bottom product are 50 mole %, 90 mole% and 10 mole% methanol. A total condenser is provided. The reflux is sent at its saturation temperature. If the reflux ratio is 1.7 times the minimum check whether the column available is satisfactory. The VLE data are: X; 8; 18; 20; 30; 40; 50; 70; 80; 95; 100. y; 36.5; 41.8; 57.9; 66.5; 72.9; 77.8; 87.0; 95.8; 97.9; 100. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8 | a | Describe the fractional distillation column design procedure step by step with neat sketch. | CO2 | 15 |
|  | b. | The feed to a distillation column is 10,000 kg/ hr of Methyl alcohol and water, containing 65 % alcohol by weight. The distillate product containing 80% alcohol by weight and bottom product contains 5% alcohol by weight. Calculate the amount of distillate and bottom product. Data given: molecular wt of 32 kg/K mole and 18 kg/K mole. | CO2 | 5 |
|  |  | Compulsory: |  |  |
| 9 | a | Differentiate between batch reactor and continuous reactor. | CO2 | 5 |
|  | b | Write down the design procedure of batch reactor. | CO2 | 15 |