Reg.No. ____________

End Semester Examination – Nov/Dec – 2018

| Code : | 14CE3021 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | PRESTRESSED CONCRETE STRUCTURES | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Define prestressing. What are its advantages? | CO1 | 8 |
|  | b. | A post tensioned beam of span 20m was designed for the live load of 20 kN/m and the mid span section is I section having top flange width 750 mm, thickness 150 mm and bottom flange width 300mm and thickness 250 mm. The total depth of the section is 1500mm and web thickness is 200 mm. The beam is prestressed by a parabolic cable with zero eccentricity at supports and 500mm eccentricity below the centroid at mid span. The effective prestressing force on the section is 2000kN. Sketch the stress distribution at mid span and at quarter span. | CO1 | 12 |
| (OR) |  |  |  |  |
| 2. | a. | Explain briefly about pretensioning and post-tensioning systems with neat sketch. | CO1 | 10 |
|  | b. | A rectangular beam 100mm x 250mm spanning over 8m is prestressed by a straight cable carrying an effective prestressing force of 200kN located at an eccentricity of 40mm. The beam supports a live load of 1.2kN/m. Calculate the resultant stress distribution for the central cross section of the beam. | CO2 | 10 |
| 3. | a. | What are the different ways of improving the shear resistance of a prestressed member? | CO1 | 8 |
|  | b. | A pretensioned, T section has a flange 1200mm wide and 150mm thick. The width and depth of the rib are 300 and 150mm respectively. The high-tensile steel has an area of 4700mm2 and is located at an effective depth of 1600mm. if the characteristic cube strength of the concrete and the tensile strength of steel are 40 and 1600N/mm2 respectively; calculate the flexural strength of the T section. | CO2 | 12 |
| (OR) |  |  |  |  |
| 4. | a. | What is the basis for the procedure recommended by I.S.Code for the design of end block. | CO3 | 8 |
|  | b. | Design a suitable end block for a beam of 550mm wide and 1350mm deep subjected to three cable anchorage each exerting 403 kN. The top and bottom cable is located with their center at 225mm from the top and bottom faces respectively, third one being placed at the center of the depth. The anchorage plate has a diameter of 122mm. The allowable stress in steel is 140 N/mm2. Show the details of reinforcement by a neat sketch. | CO2 | 12 |
| 5. | a. | What are the assumptions made in working stress design of prestressed concrete beams? | CO3 | 5 |
|  | b. | An unsymmetrical I-section has a top flange width of 1000mm and thickness of 150mm, a web thickness of 200mm, and the effective depth of cross section of 900mm. it is prestressed with 48m numbers of high tensile wires of 8mm diameter, which has ultimate tensile strength of 1500N/mm2. The effective centre of steel is at 100mm from the soffit of the beam. The steel is prestressed to a level of 900N/mm2 after losses. Estimate the ultimate moment capacity of the section using Indian Standard Code regulations. | CO2 | 15 |
| (OR) |  |  |  |  |
| 6. | a. | List out the various factors influencing deflections of prestressed concrete members. | CO3 | 8 |
|  | b. | A post tensioned concrete beam of span 10m has a section 300 x 800mm. The beam is prestressed by a parabolic cable concentric at the supports and with a eccentricity of 250mm at the centre of the span. The cross sectional area of high tensile wires in the cable is 500mm2. The wires are stressed using a jack at the left end such that the initial prestressing force in the cable at the right end is 250 kN, using the following data, calculate the total loss of stress in the wires and the jacking force required at the left end. Coefficient of friction for curvature effect = 0.55. Friction coefficient for wave effect = 0.003m Anchorage slip at jacking end = 3mm Relaxation of steel = 4% Shrinkage of concrete = 0.0002 Es = 210 kN/mm2 and Ec = 35 kN/mm2 | CO3 | 12 |
| 7. | a. | List out the losses of prestress that occur in a post tensioned member. | CO1 | 8 |
|  | b. | A prestressed concrete beam 250 mm x 500 mm is simply supported over a span of 8 m and is prestressed with 3 cables each of area 200 mm2. The cables are located at a distance of 100 mm from the soffit throughout the span. Effective stress in the cable is 900 MPa, determine; - Maximum deflection due to prestress and self weight. - Uniformly distributed load required to cause a resultant downward deflection of L/240, where L is the span. Use M40 Concrete of unit weight 24kN/m3 | CO3 | 12 |
| (OR) |  |  |  |  |
| 8. | a. | What are shear connectors? Explain how they are designed? | CO3 | 8 |
|  | b. | State the advantage of composite construction and explain the step by step procedure of analysis of a composite section. | CO2 | 12 |
|  |  | Compulsory: |  |  |
| 9. |  | Write short notes on - Partial prestressing. - Prestreeing sleepers. - Continuous prestressing. - Prestressing pipes. | CO1 | 20 |