Reg.No. ____________

End Semester Examination – Nov/Dec – 2018

| Code : | 14EC2006 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ELECTRONIC CIRCUITS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. |  | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. | a. |  | With a neat diagram, explain the working principle of half wave rectifier and derive the expression of efficiency and ripple factor. | CO1 | 15 |
|  | b. |  | In half wave rectifier, a diode whose internal resistance is 20Ω is to supply power to a 1kΩ load from a 110V (rms) source of supply. Calculate the peak load current and dc load current. | CO1 | 5 |
| (OR) |  |  |  |  |  |
| 2. | a. | Explain the operation of transistorized series voltage regulator with neat circuit diagram and derive the output voltage expression. |  | CO1 | 10 |
|  | b | Derive the ripple factor of full wave rectifier with capacitor filter. |  | CO1 | 10 |
| 3. | a. | Derive the expression for collector current IC and VCE for the fixed biased BJT with neat circuit diagram and find the stability factor. |  | CO1 | 15 |
|  | b. | Discuss the various operating points within the limits of operation of a BJT. |  | CO1 | 5 |
| (OR) |  |  |  |  |  |
| 4. | a. |  | Explain briefly about the fixed-bias arrangements for the JFET with its circuit configuration. Also perform dc analysis for the same to identify the Q-point. | CO1 | 10 |
|  | b. |  | Perform the dc analysis for voltage-divider bias arrangement applied to a FET transistor. Also discuss on how to obtain a Q-point graphically for the same. | CO1 | 10 |
| 5. | a. |  | Discuss the operation of RC Coupled Amplifier with neat circuit diagram and its frequency response. | CO2 | 15 |
|  | b. |  | Find the transformer turns ratio required to match a 8Ω speaker load so that the effective load resistance seen at the primary is 10 kΩ | CO2 | 5 |
| (OR) |  |  |  |  |  |
| 6. | a. |  | Explain the operation of Class B transformer coupled push-pull amplifier circuit. Also derive the expression of efficiency. | CO2 | 12 |
|  | b. |  | Draw the functional block diagram of Class D power amplifier and explain its operation. | CO2 | 8 |
| 7. | a. |  | Draw the block diagram representation of Voltage Series feedback amplifier and derive the expression of input impedance, output impedance and gain of the amplifier. | CO2 | 15 |
|  | b. |  | List out the significance of negative feedback amplifier. | CO2 | 5 |
| (OR) |  |  |  |  |  |
| 8. | a. |  | Draw the block diagram representation of current shunt feedback amplifier and derive the expression of input impedance, output impedance and gain of the amplifier. | CO2 | 15 |
|  | b. |  | Discuss the effect of cross-over distortion in Class B power amplifier. Also how it can be eliminated. | CO2 | 5 |
|  |  |  | Compulsory: |  |  |
| 9. | a. |  | With a neat circuit diagram, explain the operation of RC phase shift oscillator. | CO3 | 10 |
|  | b. |  | Explain the working principle of single tuned amplifier and its frequency response. | CO3 | 10 |