Reg.No. ____________

End Semester Examination – Nov/Dec – 2018

| Code : | 14EE2010 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | POWER ELECTRONICS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Write a note on the Switching characteristics of a power BJT. | CO1 | 15 |
|  | b. | Explain the formation of parasitic BJT and parasitic Diode in a PMOSFET. Can the parasitic diode be used in any power electronic application? | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Describe the different modes of operation of a thyristor with the help of its static I-V characteristics. | CO1 | 15 |
|  | b. | Design a circuit to avoid the false triggering of the SCR by high value of dv/dt and explain its operation. | CO1 | 5 |
| 3. | a. | Illustrate the working of a single phase full converter with RL load in rectifier mode with its corresponding waveforms for source voltage, output voltage and output across one thyristor. | CO2 | 15 |
|  | b. | A single-phase semi-converter circuit is supplying power to a motor load. The average value of load voltage is 176.72 V and the rms value is 219.3 V. Find the voltage ripple factor. | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. |  | Explain the operation of a three phase fully controlled full bridge converter in line commutated inverter mode with the corresponding circuit diagram and output waveform. | CO2 | 20 |
| 5. | a. | Design a circuit to produce an output as given in figure also explain its operation and state its application. | CO3 | 14 |
|  | b. | A single phase full wave AC voltage controller feeds a load of R= 20Ω with an input voltage of 230V, 50 Hz. Firing angle for both the thyristors is 45 . Calculate - RMS Value of output voltage. - RMS Value of load current. - Load Power. | CO3 | 6 |
| (OR) |  |  |  |  |
| 6. | a. | Recommend a circuit which helps in Forward Motoring, Reverse Motoring , Forward regeneration and revers regeneration. | CO3 | 14 |
|  | b. | Brief on the various control strategies used for varying the duty cycle . | CO3 | 6 |
| 7. |  | Describe the operation of three phase inverter in 180 mode conduction with necessary circuit diagram, waveforms of Line voltages and phase voltages. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Cosine Firing Scheme provides a linear transfer characteristic between the average output voltage and the control voltage – Validate the statement. | CO2 | 14 |
|  | b. | Present a circuit scheme for generating timing pulses for a three –phase chopper. | CO2 | 6 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss in detail the following: - Rotating type UPS system. - Short break static UPS configuration. - No break UPS configuration. | CO3 | 15 |
|  | b. | Draw the layout for a solar based home inverter. | CO3 | 5 |