Reg.No. ____________

End Semester Examination – Nov/Dec – 2018

| Code : | 14EI2023 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | OPTO-ELECTRONICS AND LASER BASED INSTRUMENTATION | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome |  | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. | a. | With a neat diagram explain the complete fiber optic communication system. Also discuss its advantages compared to other communication systems. | CO1 |  | 10 |
|  | b. | Describe the types of fiber optic cable based on the mode of transmission. | CO1 |  | 6 |
|  | c. | Optical fibers are generally composed of silica, with an index of refraction around 2.4. Calculate - The velocity of light. - The Energy if the wavelength of the light is 1.5μm. | CO1 |  | 2+2 |
| (OR) |  |  |  |  |  |
| 2. | a. | List the features of a good photo detector and describe in detail the construction and working of p-i-n detector. |  | CO1 | 10 |
|  | b. | If the quantum efficiency of light is 72% and the number of photons incident is 12 x 104 . - Calculate the number of electron hole pairs generated. - Calculate the responsivity if the wavelength of emitted light is 850nm |  | CO1 | 2+2 |
|  | c. | Describe the different type of losses that occur in fiber optic cable resulting in the attenuation of the signal. |  | CO1 | 6 |
| 3. | a. | Comprehend on the use of fiber optic sensors in Industry and also explain how fiber optic cables are used in measuring Temperature, Pressure and Liquid level. |  | CO1 | 15 |
|  | b. | Comment on the dual nature of light. |  | CO2 | 5 |
| (OR) |  |  |  |  |  |
| 4. | a. | Differentiate between intrinsic and extrinsic fiber optic sensors, Intensity modulated and phase modulated fiber optic sensors. | CO1 |  | 12 |
|  | b. | Discuss how fiber optic sensors are used for measuring electric field. | CO1 |  | 4 |
|  | c. | A typical LASER operates with a current of 10mA at a DC voltage of 2500V and gives an optical output of 5mW. Calculate the overall power efficiency. | CO1 |  | 4 |
| 5. | a. | Explain the construction and working of a Semiconductor LASER. | CO2 |  | 12 |
|  | b. | Differentiate: - Pulsed LASERS and Continuous Wave LASERS. - Solid LASER and Liquid LASER. | CO2 |  | 4+4 |
| (OR) |  |  |  |  |  |
| 6. | a. | LASER applications for material processing and medical applications requires high intensity short duration pulses. Justify the statement. Elaborate on the methods of producing such short duration pulses. | CO2 |  | 12 |
|  | b. | Analyze how LASERS can be used in measuring velocity? | CO2 |  | 8 |
| 7. | a. | Explain the different types of LASER interaction with tissues. | CO3 |  | 5 |
|  | b. | Discuss in detail the different reactions that occurs on tissues when it is exposed to light. | CO3 |  | 15 |
| (OR) |  |  |  |  |  |
| 8. |  | Elaborate on the application of LASER in Material Processing. | CO3 |  | 20 |
|  |  | Compulsory: |  |  |  |
| 9. | a. | Compare Holography with Photography and discuss the advantages of Holography over photography. | CO3 |  | 5 |
|  | b. | Explain in detail about the construction and reconstruction of hologram and the conditions involved in producing and reproducing good holograms. | CO3 |  | 15 |