Reg.No. ____________

End Semester Examination – Nov/Dec – 2018

| Code : | 17ME3003 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ADVANCED MECHANICAL VIBRATIONS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | An aircraft radio transmitter weighs 26kg and is mounted on five springs which deflect 8mm when the transmitter is placed upon them. Neglecting damping, find the percentage of engine vibration received by the transmitter for engine speeds of 1500 rpm and 2500 rpm. | CO5 | 10 |
|  | b. | A gun barrel having mass 560kg is to be designed with the following data: Initial recoil velocity =36m/sec; recoil distance on firing 1.5m. Calculate (i) Spring constant. (ii) Damping coefficient. | CO5 | 10 |
| (OR) |  |  |  |  |
| 2. |  | Explain the types of damping used in mechanical systems. | CO1 | 20 |
| 3. | a. | With illustration, explain semi-definite system. | CO1 | 10 |
|  | b. | A reciprocating machine weighing 25 N running at 6000 rpm after installation has natural frequency very close to the forcing frequency of vibrating system. Design a dynamic absorber of the nearest frequency of the system which is to be at least 20% from the excitation frequency. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. |  | With necessary equations, explain the working principle of dynamic vibration absorber. | CO3 | 20 |
| 5. |  | Determine the natural frequency of the spring mass system shown in Figure using Stodola method. Take K1 = K2 = K3=K and m1 = m2 = m3 =m | CO2 | 20 |
| (OR) |  |  |  |  |

| 6. |  | Use matrix iteration method to determine the natural frequencies of the system shown in Figure. | CO2 | 20 |
| --- | --- | --- | --- | --- |
| 7. |  | With neat sketch, explain any two vibration measuring instruments. | CO4 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | A seismic instrument is mounted on a machine running at 1000 rpm. The natural frequency of the seismic instrument is 20 rad/sec. the instrument records relative amplitude of 0.5 mm. Compute the displacement, velocity and acceleration of the machine. Neglect the damping in seismic instrument. | CO4 | 10 |
|  | b. | Write short notes on ‘Data Acquisition system’ | CO4 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | With illustration, explain the mechanism of noise generation in compressors | CO6 | 10 |
|  | b | Explain the Framework Standards for Noise Reduction at Machines | CO6 | 10 |