Reg.No. ____________

End Semester Examination – Nov/Dec – 2018

| Code : | 18EE3001 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ENERGY ENGINEERING | Max. marks : | 100 |

ANSWER ANY FIVE  QUESTIONS  (5 x 16 = 80 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Justify the need for renewable energy by showing the reserves/production ratio of fossil fuels. | CO1 | 8 |
|  | b. | A commercial building has 200, 15W CFL lamps. It is proposed to replace them with LED lamps providing an equivalent light output. The following data are given, Cost of a 7 W LED lamp is Rs.6, Average Electricity Price – 16 p/kWh, Lamps operate an average 5 hours per day, The building is occupied 250 days/year. Calculate the simple pay back period of replacement. | CO1 | 8 |
| 2. | a. | Develop the estimated energy resources available in india as on 2016, with the the help of pie charts. | CO2 | 8 |
|  | b. | Discuss about Energy consumption in various sectors and projected energy consumption for the future. | CO2 | 8 |
| 3. | a. | Draw a block diagram to represent the major components of a horizontal axis wind turbine and include the main options in wind machine design and construction. | CO3 | 8 |
|  | b. | Deduce the expressions used to calculate the wind power parameters in i] Direct use of data averaged over a short time interval ii] The method of bins. | CO3 | 8 |
| 4. |  | Discuss biomass as a renewable energy resource as well as the ways and technologies involved in extracting power from biomass energy is converted into electricity. | CO4 | 16 |
| 5. |  | Summarize the factors to be considered in design and operation towards management of radioactive wastes at nuclear power plants. | CO5 | 16 |
| 6. |  | Design and develop a Standalone Hybrid Wind-Solar Power Generation System for Indian remote area. | CO3 | 16 |
| 7. |  | Outline the major application possibilities of photovoltaics, ranging from large scale electricity production installations to on-device portable chargers. | CO3 | 16 |
| COMPULSORY QUESTION (1 x 20 = 20 Marks) |  |  |  |  |
| 8. | a. | Develop a framework for integrated energy network plan with 10 key challenges for future electric system resource planning. | CO6 | 10 |
|  | b. | Discuss the current state of various advanced Smart Grid components, as well as the core technologies upon which they depend. | CO6 | 10 |