Reg.No. ____________

End Semester Examination – Nov/Dec – 2018

| Code : | 18ME3001 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | THERMODYNAMICS AND COMBUSTION | Max. marks : | 100 |

ANSWER ANY FIVE  QUESTIONS  (5 x 16 = 80 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Steam enters a turbine steadily at 3 MPa and 450 °C at a rate of 8 kg/s and exits at 0.2 MPa and 150 °C. The steam is loosing heat to the surrounding air at 100 kPa and 25 °C at a rate of 300 kW, and the kinetic and potential energy changes are negligible. Determine i) the actual power output ii) the maximum possible power output iii) the second law efficiency iv) the exergy destroyed, and v) the exergy of the steam at the inlet conditions. | CO1 | 16 |
| 2. |  | Prove that the compressibility factor at critical point for Vander Waals gas is 0.375. | CO2 | 16 |
| 3. | a. | Derive Clapeyron-Clausius equation and mention is significance? | CO3 | 10 |
|  | b. | Using Clapeyron-Clausius equation estimate the saturation pressure of the refrigerant 134a at -45 °C, using the data available in the refrigerant tables. | CO3 | 6 |
| 4. | a. | One kmol of octane (C8H18) is burned with air that contains 20 kmol of O2. Assuming the products contain only CO2, H2O, O2 and N2, determine the mole number of each gas in the products and the air-fuel ratio for this combustion process. | CO4 | 8 |
|  | b. | Write short notes on i) heat of combustion, and ii) adiabatic flame temperature. | CO4 | 8 |
| 5. |  | Methane (CH4) gas enters a steady flow adiabatic combustion chamber at 25 °C and 1 atm. It is burned with 50 % excess air that also enters at 25 °C and 1 atm. Assuming complete combustion, determine i) the temperature of the products ii) the entropy generation, and iii) the reversible work and exergy destruction. Assume that T0= 298 K and the products leave the combustion chamber at 1 atm. | CO5 | 16 |
| 6. |  | The ultimate analysis of coal (by mass) as C-84.36%, H2- 1.89%, O2- 4.4% N2- 0.63% S- 0.89%and ash-7.83% is burned with theoretical amount of air. Disregarding the ash content, determine the mole fractions of the products and the apparent molar mass of the product gases. Also determine the air fuel ratio required for the combustion process. | CO1 | 16 |
| 7. | a. | As an alternative to electrolysis, hydrogen gas can be produced thermally in accordance with the dissociation process H2O→H2+1/2 O2 by heating water vapour to very high temperatures. Determine the percentage of water vapour that can be separated into hydrogen and oxygen when this reaction occurs at 4000 K and 10 kPa. | CO2 | 8 |
|  | b. | A mixture of 2 kmol CO and 3 kmol of O2 is heated to 2600 K at a pressure of 304 kPa. Determine the equilibrium composition, assuming the mixture consists of CO2, CO and O2. | CO2 | 8 |
| COMPULSORY QUESTION (1 x 20 = 20 Marks) |  |  |  |  |
| 8. | a. | State and prove Nernst heat theorem. | CO6 | 14 |
|  | b. | Write any three important probability theorems in statistical thermodynamics? | CO6 | 6 |