Reg.No. _____________

End Semester Examination – Nov/Dec – 2018

| Code : | 09BT229 / 12BT221 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | CHEMICAL THERMODYNAMICS AND BIO THERMODYNAMICS | Max. marks : | 100 |

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART-A(10X1=10 MARKS) |  |  |
| 1. | Define Ideal gas and its equation. | 1 |
| 2. | The compressibility factor accounts for the deviation of a system from __________. | 1 |
| 3. | Define rate constant. | 1 |
| 4. | Gibbs defined chemical potential as __________. | 1 |
| 5. | What is meant by heterogenous reaction? | 1 |
| 6. | Define multiple reaction | 1 |
| 7. | Name any one advantage of Carnot cycle. | 1 |
| 8. | Define air standard efficiency of a cycle. | 1 |
| 9. | Cell growth obeys __________. | 1 |
| 10. | Define anaerobic microorganisms. | 1 |

| PART B(5 X 3= 15 MARKS) |  |  |
| --- | --- | --- |
| 11. | Write a short note on Gibbs free energy. | 3 |
| 12. | Define fugacity. | 3 |
| 13. | What are the criteria for chemical equilibrium? | 3 |
| 14. | Define air standard efficiency for a Diesel cycle. | 3 |
| 15. | Explain Degree of reduction. | 3 |

| PART C(5 X 15= 75 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 16. |  | Derive Maxwell’s relations from the first principles. | 15 |
| (OR) |  |  |  |
| 17. |  | How is the compressibility factor determined for a mixture of real gases? | 15 |
| 18. | a. | What does Gibbs-Duhem equation relate? Show the various forms of Gibbs-Duhem equation. | 10 |
|  | b. | Explain activity and activity co-efficient. | 5 |
| (OR) |  |  |  |
| 19. |  | Compute the fugacity of gaseous Propane at 13.80 bar and 55.5oC assuming: a. Ideal gas behavior for Propane b. law of corresponding states applicable to Propane. Data: Tc = 370oK, Pc = 42 atm. | 15 |
| 20. |  | What is the effect of pressure on equilibrium constant and equilibrium composition? Discuss how this effect is accounted for. | 15 |
| (OR) |  |  |  |
| 21. |  | Estimate the equilibrium constant for the reaction C2H4 (g) + H2O(g)  C2H5OH(g) at 600oK . Data: ∆F o298 = 2030 Cal/ g.mole ∆H of 298 for C2H5OH(g) = 56,240 Cal/ g.mole ∆H of 298 for C2H4 (g) = 12,496 Cal/ g.mole ∆H of 298 for H2O(g) = 57,798 Cal/ g.mole | 15 |
| 22. |  | Discuss the characteristics of various processes in Otto cycle, diesel and dual cycles. | 15 |
| (OR) |  |  |  |
| 23. |  | An engine working on Dual Cycle has a cylindrical bore of 20 cm and stroke of 40 cm. Determine the air standard efficiency if: a. v = 1.4 b. r = compression ratio = 15 c. rp = ratio of pressure at the end and beginning of heat addition at constant volume = 2 d. Constant pressure addition cut off takes place at 5% of the stroke. | 15 |
| 24. |  | Corn steep liquor contains 2.5% invert sugars and 50% water. The rest of the feed is considered as residual solids. Beet molasses containing 50% sucrose, 1% invert sugar, 18% water and remainder solids are mixed with corn steep liquor in a mixing tank. Water is added to produce a diluted mixture with 2% invert sugar, 125 kg corn steep liquor and 45 kg molasses, which is fed into an enzymatic hydrolysis tank. a. How much water is required? b. What is the concentration of sucrose in the final mixture? | 15 |
| (OR) |  |  |  |
| 25. |  | Discuss in detail about the growth of stochiometric and energy balances. | 15 |