Reg.No. _____________

End Semester Examination – Nov/Dec – 2018

| Code : | 10EI214/12EI215/EI248 | Duration : | 3 hrs |
| --- | --- | --- | --- |
| Sub. Name : | COMMUNICATION ENGINEERING | Max. marks : | 100 |

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART-A(10X1=10 MARKS) |  |  |
| 1. | Moudlation index of an Amplitude Modulated Signal is ____________. | 1 |
| 2. | Audio signals are preferably Frequency Modulated. True/False | 1 |
| 3. | What is the difference between low level and high level transmission? | 1 |
| 4. | Give two examples of Frequency Demodulators. | 1 |
| 5. | Amplitude shift keying is also known as ____________. | 1 |
| 6. | __________ is the most prevalent encoding technique used for TDM digital signals. | 1 |
| 7. | Parity is a method for error correction. True/False | 1 |
| 8. | Fiber optics uses light signals of wavelength ranging from ____________. | 1 |
| 9. | Write the expression for maximum radar range. | 1 |
| 10. | Name the primary colors used in color TV signal transmission. | 1 |

| PART B(5 X 3= 15 MARKS) |  |  |
| --- | --- | --- |
| 11. | What is the need for modulation? | 3 |
| 12. | Compare AM and FM. | 3 |
| 13. | Define PAM, PPM, & PDM. | 3 |
| 14. | What are the advantages of fiber optic communication? | 3 |
| 15. | List the advantages of cellular communication. | 3 |

| PART C(5 X 15= 75 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 16. |  | Derive the expression for single tone AM signal with necessary diagram. Draw the frequency spectrum of AM signal and calculate its bandwidth of total power in AM wave. | 15 |  |
| (OR) |  |  |  |  |
| 17. | a. | Describes the generation of frequency modulation using Armstrong method. |  | 10 |
|  | b. | The antenna current of an AM transmitter is 8 amperes (8A) when only the carrier is sent, but it increase to 8.93A when the carrier is modulated by a single sine wave. Find the percentage modulation. Determine the antenna current when the percent of modulation changes to 0.8. |  | 5 |
| 18. |  | Describe in detail about the modulation and demodulation process of FM wave. |  | 15 |
| (OR) |  |  |  |  |
| 19. |  | Explain in detail about the various noises in communication systems. | 15 |  |
| 20. |  | Discuss the important features of transmitter and receiver of pulse code modulation system. | 15 |  |
| (OR) |  |  |  |  |
| 21. |  | Define FSK. Explain with neat diagrams FSK generation and detection. Draw and discuss about the power spectrum of FSK. | 15 |  |
| 22. |  | Write short notes on: a) Successive approximation converter. b) Time division multiplexing. | 15 |  |
| (OR) |  |  |  |  |
| 23. |  | Explain the various types of photo diodes used for detective light in optical fiber. | 15 |  |
| 24. |  | Explain about satellite system link models with suitable diagrams. | 15 |  |
| (OR) |  |  |  |  |
| 25. |  | Draw the block diagram for monochrome TV receiver and explain the components. | 15 |  |