Reg.No. _____________

End Semester Examination – Nov/Dec – 2018

| Code : | 12BT208 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | HEAT TRANSFER OPERATIONS | Max. marks : | 100 |

#### Answer ALL questions

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART-A(10X1=10 MARKS) |  |  |
| 1. | Fourier’s law of heat conduction applies to ___________. | 1 |
| 2. | The unit of thermal conductivity is __________. | 1 |
| 3. | The absorptivity of a black body is ___________. | 1 |
| 4. | The energy radiated from a surface Q at absolute temperature T is related as ___________. | 1 |
| 5. | The purpose of floating head in a heat exchanger is to avoid ___________. | 1 |
| 6. | In a shell and tube heat exchanger, floating head is used for __________ temperature difference. | 1 |
| 7. | The kg of liquid evaporated per hour in an evaporator is defined as ___________. | 1 |
| 8. | Heat sensitive materials can be concentrated in a __________ evaporator. | 1 |
| 9. | Drop wise condensation occurs on __________ surfaces. | 1 |
| 10. | Nucleated boiling is promoted on ___________ surfaces. | 1 |

| PART B(5 X 3= 15 MARKS) |  |  |
| --- | --- | --- |
| 11. | State Fourier's law of heat conduction and its application. | 3 |
| 12. | Differentiate natural convection from forced convection. | 3 |
| 13. | Why counter current flow in shell and tube heat exchanger is better than co-current flow? | 3 |
| 14. | Why multiple effect evaporator is preferred over single effect evaporator? | 3 |
| 15. | What are the factors that influence the heat transfer coefficient during nucleate boiling? | 3 |

| PART B(5 X 3= 15 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 16. | a. | A wall of a furnace 0.244m thick is constructed of material having thermal conductivity 1.30w/m K. The wall will be insulated on the outer side with the material having average thermal conductivity 0.346w/m K, so that the heat loss from the furnace will be equal to or less than 1830w/m2. The inner surface of the wall is at 1588K, while the outer surface is at 299K. Calculate the thickness of insulation required. | 9 |
|  | b. | Derive an expression for determining the rate of heat transfer for conduction through a hollow sphere. | 6 |
|  |  | (OR) |  |
| 17. | a. | Derive an expression for overall heat transfer coefficient from individual heat transfer coefficient based on outside area. | 10 |
|  | b. | Discuss the different modes on heat transfer. | 5 |
| 18. | a. | What is radiation? State the different laws of radiation. | 5 |
|  | b. | By dimensional analysis show that Nusselt number is a function of Grashoff number and Prandtle number for natural convection. | 10 |
|  |  | (OR) |  |
| 19. | a. | Derive the equation for radiation exchange between large grey concentric cylinders. | 10 |
|  | b. | Write note on heat transfer in packed bed. | 5 |
| 20. | a. | Derive an expression for LMTD of the counter flow heat exchanger with graphical representation. | 10 |
|  | b. | What is fouling factor? How it can be prevented in heat exchanger? | 5 |
|  |  | (OR) |  |
| 21. | a. | A reaction mixture having Cp = 2.85KJ/kg K is flowing at a rate of 7260 kg/hr is to be cooled from 377.6 to 344.3 K. Cooling water at 288.8 K is available and the flow rate is 4536 kg/hr. The overall heat transfer coefficient is 653 W/m2K. (a) For counter flow, calculate the outlet water temperature and area of heat exchanger (b) For co-current flows calculate the area of heat exchanger. | 10 |
|  | b. | Discuss the principle of double pipe heat exchanger with neat diagram. | 5 |
| 22. | a. | A feed of 4535 kg/hr of a 2% by weight salt solution at 311 K enters continuously a single effect evaporator and being concentrated to 3% by weight. The evaporation is at atmospheric pressure and the area of the evaporator is 69 m2. Saturated steam at 383.2 K is supplied for heating. Since the solution is dilute, it can be assumed to have same boiling point of water. Cp of feed is 4.10 KJ/kg K. Calculate the overall heat transfer coefficient. | 10 |
|  | b. | What are the liquid characteristics which affects the design and operation of evaporators? Explain. | 5 |
|  |  | (OR) |  |
| 23. |  | Explain the principle, operation and application of different types of evaporators with neat diagram. | 15 |
| 24. | a. | Explain the mechanisms involved in drop wise condensation and film type condensation. | 8 |
|  | b. | For a laminar film condensation on a vertical plate, develop an expression for film thickness, heat transfer coefficient, and steam condensation rate in term of fluid properties, temperature difference and the plate dimensions. Are the fluid properties involved for the vapour phase? If not, how are they evaluated? | 7 |
|  |  | (OR) |  |
| 25. |  | Explain the various steps involved in the boiling mechanisms for water at atmospheric pressure with graphical representation. | 15 |