Reg.No. _____________

End Semester Examination – Nov/Dec – 2018

| Code : | 12EI204 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | THERMODYNAMICS AND FLUID MACHINERY | Max. marks : | 100 |

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART-A(10X1=10 MARKS) |  |  |
| 1. | What is a thermodynamic cycle? | 1 |
| 2. | What is a cyclic heat engine? | 1 |
| 3. | Define Fourier law of heat conduction | 1 |
| 4. | Define ‘Emissivity’? | 1 |
| 5. | Distinguish between ideal fluid and real fluid? | 1 |
| 6. | Write the classification of reciprocating pumps? | 1 |
| 7. | Define steady incompressible fluid flow? | 1 |
| 8. | Define ‘hydraulic efficiency of the turbine’? | 1 |
| 9. | What is ‘evaporative capacity’ of the boiler | 1 |
| 10. | Why is a counter flow heat exchanger more effective than the parallel flow heat exchanger? | 1 |

| PART B(5 X 3= 15 MARKS) |  |  |
| --- | --- | --- |
| 11. | Write Kelvin Planck and Clausius statements of second law of thermodynamics? | 3 |
| 12. | Emissivities of two large parallel plates maintained at 800 °C and 300 °C are 0.3 and 0.5 respectively. Find the net radiant heat exchange per square meter for these plates? | 3 |
| 13. | Find the surface tension in a soap bubble of 40mm diameter when the inside pressure is 2.5 N/m2 above atmospheric pressure | 3 |
| 14. | Write the classification of turbines? | 3 |
| 15. | Write the differences between Fire-tube and water-tube boilers? | 3 |

| PART C(5 X 15= 75 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 16. | a. | State and prove ‘Carnot theorem’? | 10 |
|  | b. | A domestic food freezer maintains a temperature of -150C. The ambient air temperature is 300C. If heat leaks in to the freezer at the continuous rate of 1.75 kJ/s what is the least power necessary to pump this heat out continuously. | 5 |
| (OR) |  |  |  |
| 17. |  | In a steady flow apparatus, 135 kJ of work is done by each kg of fluid. The specific volume of the fluid, pressure and velocity at the inlet are 0.37 m3/kg, 600kPa and 16 m/s. The inlet is 32m above the floor and the discharge pipe is at the floor level. The discharge conditions are 0.62 m3/kg, 100kPa and 270m/s. The total heat loss between the inlet and the discharge is 9 kJ/kg of fluid. In flowing through this apparatus, does the specific internal energy increase or decrease, and by how much? | 15 |
| 18. |  | Derive general differential equation of heat conduction in Cartesian coordinates? | 15 |
| (OR) |  |  |  |
| 19. |  | Air stream at 27 °C is moving at 0.3 m/s across a 100 W electric bulb at 127 °C. If the bulb is approximated by a 60 mm diameter sphere, estimate the heat transfer rate and the percentage of power lost due to convection | 15 |
| 20. |  | Explain the construction and working of Babcock and Wilcox boiler with neat sketch. | 15 |
| (OR) |  |  |  |
| 21. |  | Hot oil with a capacity rate of 2500 W/K flows through a double pipe heat exchanger. It enters at 360 °C and leaves at 300 °C. Cold fluid enters at 30 °C and leaves at 200 °C. If the overall heat transfer coefficient is 800 W/m2 K determine the heat exchanger area required for a) parallel flow b) counter flow. | 15 |
| 22. | a. | Calculate the specific weight, density and specific gravity of one litre of a liquid which weighs 7N | 5 |
|  | b. | The space between two square flat parallel plates is filled with oil. Each side of the plate is 60cm. the thickness of the oil film is 12.5mm. The upper plate which moves at 2.5 m/s requires a force of 98.1 N to maintain the speed. Determine i) the dynamic viscosity of the oil in poise ii) the kinematic viscosity of the oil in stokes if the specific gravity of the oil is 0.95. | 10 |
| (OR) |  |  |  |
| 23. | a. | A U-tube manometer is used to measure the pressure of water in a pipe line, which is in excess of atmospheric pressure. The right limb of manometer contains mercury and is open to atmosphere. The contact between the water and mercury is in the left limb. Determine the pressure of water in the main line, if the difference in level of mercury in the limbs of U-tube is 10cm and free surface of the mercury is in level with the centre of the pipe. If pressure of water in the pipe line is reduced to 9810N/m2, calculate the new difference in the level of the mercury. Sketch the arrangement in both the cases. | 15 |
| 24. |  | A Kaplan turbine working under a head of 20 m develops 11772 kW shaft power. The outer diameter of the runner is 3.5 m and hub diameter is 1.75 m. The guide blade angle at the extreme edge of the runner is 35°. The hydraulic and overall efficiencies of the turbine are 88% and 84% respectively. If the velocity of the whirl is zero at the outlet, determine: - Runner vane angles at inlet and outlet at the extreme edge of the runner - speed of the turbine | 15 |
| (OR) |  |  |  |
| 25. |  | The penstock supplies water from a reservoir to the pelton wheel with a gross head of 500m. One third of gross head is lost in friction in the penstock. The rate of flow of water through the nozzle fitted at the end of the penstock is 2 m3/s.The angle of deflection of the jet is 165 0 . Determine the power given by the water to the runner and also hydraulic efficiency of the pelton wheel. Take speed ratio=0.45 and Cv=1. | 15 |