Reg.No. _____________

End Semester Examination – Nov/Dec – 2018

| Code : | 12EI208 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ELECTRICAL MEASUREMENTS | Max. marks : | 100 |

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART-A(10X1=10 MARKS) |  |  |
| 1. | The burden of a current transformer is expressed in terms of VA rating of transformer.(Y/N) | 1 |
| 2. | State any one function of an instrument. | 1 |
| 3. | A PMMC galvanometer with a short pen arc writes in a ___________ Manner. | 1 |
| 4. | The quantity intended to be measured is called ___________. | 1 |
| 5. | ___________ bridge is suited for low Q coils measurement. | 1 |
| 6. | __________ is an electrical circuit for the precise measurement of resistances. | 1 |
| 7. | An ____________ is an electronic device which measures resistance in an electronic component or circuit. | 1 |
| 8. | A multimeter is also known as ___________. | 1 |
| 9. | Thermistors have ___________ temperature coefficient of resistance. | 1 |
| 10. | A Magnetic tape recording permits ___________ channel recording. | 1 |

| PART B(5 X 3= 15 MARKS) |  |  |
| --- | --- | --- |
| 11. | Present the structure of the fundamental elements of an instrumentation system. | 3 |
| 12. | Draw the D’Arsonval Galvanometer with all parts marked? | 3 |
| 13. | Present the Maxwells bridge with its Phasor diagram. | 3 |
| 14. | Draw the potentiometric type Recorder. | 3 |
| 15. | What are the precautions to be taken for connecting a potential Transformer? | 3 |

| PART C(5 X 15= 75 MARKS) |  |  |
| --- | --- | --- |
| 16. | Justify the statement “In a deflection Type Instrument, the value of measured quantity depends upon the calibration of an instrument”also support it with a neat diagram. | 15 |
| (OR) |  |  |
| 17. | List out the static and dynamic characteristics of Instruments. | 15 |
| 18. | Explain in detail the PMMC Instrument with corresponding expression and diagram. | 15 |
| (OR) |  |  |
| 19. | Prove that Rsh=Rm/(m-1) in an ammeter shunt. Also state why manganin is preferred for shunts of DC instruments. | 15 |
| 20. | Explain the principle and construction of a moving iron instrument in detail. |  |
| (OR) |  |  |
| 21. | Explain the operation of watt-hour meter with corresponding diagram. | 15 |
| 22. | Derive the condition for balance in a Wheatstone’s bridge. | 15 |
| (OR) |  |  |
| 23. | Derive the balance equations for Anderson’s bridge supporting it with corresponding diagram. | 15 |
| 24. | Elaborate on a graphic recorder, which records one or more variables with respect to time. | 15 |
| (OR) |  |  |
| 25. | Detail out a graphic recorder, which records one or more dependent variables with respect to an independent variable. | 15 |