Reg. No. ________

End Semester Examination – Nov/Dec – 2018

Code              :	12EI210	                                                                                      Time : 3 hours

Sub. Name    :	CONTROL SYSTEM				 		  Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

| Q.No | Questions | Marks |
| --- | --- | --- |
|  | PART – A (10 x 1 = 10 MARKS) |  |
| 1 | Differentiate between open loop and closed loop control system. | 1 |
| 2 | List the characteristic of a good control system. | 1 |
| 3 | The transfer function of a phase-lead controller is ___________. | 1 |
| 4 | For a type one system, the steady – state error due to step input is equal to___________. | 1 |
| 5 | List the types of error. | 1 |
| 6 | Differentiate between lead and lag compensator. | 1 |
| 7 | List the condition for stability of a system. | 1 |
| 8 | The equation 2S4+S3+3S2+5S+10=0 has ___________ roots in the left half of s–plane. | 1 |
| 9 | Define state of a system. | 1 |
| 10 | If the Nyquist plot of the loop transfer function G(s) H(s) of a closed-loop system encloses the (−1, j0) point in the G(s) H(s) plane, the gain margin of the system is ___________. | 1 |

| PART – B (5 x 3 = 15 MARKS) |  |  |
| --- | --- | --- |
| 11. | Draw the equivalent of the block diagram in the following figure G1 G2 H | 3 |
| 12. | The open loop transfer function of the given system is 2 / (s(s+4)(s+6)). Determine the positional, velocity and acceleration error constants for the above system. | 3 |
| 13. | Draw the polar plot for type 0, order 1 system. | 3 |
| 14. | Draw the typical bode plots for lag compensator. | 3 |
| 15. | What is state model? Also explain the need for state model. | 3 |

| PART – C (5 x 15 = 75 MARKS) |  |  |
| --- | --- | --- |
| 16. a. | Explain the operation of synchro transmitter and synchro receiver with neat sketch | 7 |
| b. | Describe a two phase AC servomotor and derive its transfer function. | 8 |
|  | (OR) |  |
| 17 | Determine the transfer function C(s)/R(s) for the block diagram shown in Fig by first drawing its signal flow graph and then formula using the Mason’s gain | 15 |
| 18 | For the system shown in the block diagram of figure determine the values of gain K1 and velocity feedback constant K2 so that the maximum overshoot with a unit step input is 0.25 and the time to reach the first peak is 0.8 sec. Thus obtain the rise time and settling time for 5% tolerance band. | 15 |
|  | (OR) |  |
| 19 | The open loop transfer functions of two systems are given as (a) 4 / ((s +1)(s +2)) (b) 5/(s2(s +3)(s +10)) Determine respectively the positional, velocity and acceleration error constants for these systems. Also for the system given in (ii) determine the steady state errors with step input r(t) = u(t) , ramp input r(t) = t and acceleration input r(t) = 1/ 2 t2. | 15 |
| 20 | Sketch the root loci for the system shown in the given Figure. K s(s+1) (s2+4s+13) R(s) C(s) | 15 |
|  | (OR) |  |
| 21 | Consider the characteristic equation s4 + 2s3 + (4 + K)s2 + 9s + 25 = 0. Using the Routh’s stability criterion, determine the range of K for stability. Also Find the number of roots of characteristic equation which lie in the right half of s-plane for K =100. | 15 |
| 22 | Explain the procedural design steps of lag and lag- lead compensator. | 15 |
|  | (OR) |  |
| 23 | The open-loop transfer function of a control system is G(s)H(s) =10 / (s (1+ 0.5s) (1 +0.1s)) Draw the Bode plot and determine the gain crossover frequency, phase and gain margin. | 15 |
| 24. | Consider a DC motor electrical system to estimate the state model analysis. | 15 |
|  | (OR) |  |
| 25. | Determine the state equation of the given transfer function G(s)= Y(s)/ U(s) = (S2+7S+2 )/ (S3+9S2+26S+24) | 15 |