Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	ELECTRONIC CIRCUITS							      	     Time : 3 hours

Subject Code:	09EE201								   		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is ripple factor (Γ)?

2.	The switching regulator is also known as __________.

3.	What is wave shaping?

4.	How is an attenuator compensated?

5.	What is a BiFET amplifier?

6.	Draw the ideal differential amplifier.

7.	What is the operating cycle for a class B amplifier?

8.	What is the gain of the amplifier with negative feedback?

9.	What are the essential conditions for maintaining oscillations?

10.	Give any two applications of Astable multivibrator.

PART – B  (5 x 3 = 15 MARKS)

11.	A half wave rectifier is used to supply 24V DC to a resistive load of 500Ω and the diode has a forward resistance of 500Ω. Calculate the maximum value of the AC voltage required at the input.

12.	Give the applications of the pulse transformer.

13.	Compare BJT and JFET amplifier.

14.	What are the four different combinations of negative feedback connections?

15.	What is piezo-electric effect?

PART – C  (5 x 15 = 75 MARKS)

16.	In a centre tapped full wave rectifier, the rms half secondary voltage is 9V. Assuming ideal diodes and load resistance RL=1K Ω find 		a.	peak current		b.	DC load voltage

c.	rms current		d.	ripple factor		e.	efficiency.

(OR)

17.	a. 	Explain the different types of voltage regulators.									(10)

b. 	Distinguish shunt and series regulator.											(5)

18. 	Explain how high pass RC circuit acts as differentiator.

(OR)

19. 	With a neat diagram, explain the clamping operation.

20. 	Explain about JFET amplifier and its equivalent circuit with a neat diagram.

(OR)

21.	With a neat diagram, explain about differential amplifier and derive the expression                                    	for differential gain, common mode gain and CMRR.

22. 	Explain the operation of class A and class B amplifier with a suitable diagram.

(OR)

[P.T.O.]

23. 	a. 	Explain the concept of positive and negative feedback with a neat diagram.		 	(7)

b. 	Discuss the four effects of negative feedback.									(8)

24.	a. 	Explain the operation of RC phase shift oscillator with a diagram and derive the    	expression. 																	(9)

b. 	Explain the operation of a crystal oscillator. 										(6)

(OR)

25. 	Explain the operation of a Schmitt trigger with a suitable diagram and give its application.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	DIGITAL ELECTRONICS							      	     Time : 3 hours

Subject Code:	09EE202										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Divide 1100 by 100.

2.	Apply De-Morgan’s theorems to the following expression.

[(A+B+C)D]'.

3.	When is the output of an OR gate LOW?

4.	If the number on the A inputs is 0110 and the number on the B inputs is 0011, then which input of the 4 bit comparator will be made HIGH?

5.	Mention the primary difference between JK flip flop and SR flip flop.

6.	Classify the shift registers based on the input – output relationship.

7.	In Mealy model, the outputs are the functions of ____________.

8.	What is Excitation table?

9.	Name three major types of PLDs.

10.	What are the types of DRAM?

PART – B  (5 x 3 = 15 MARKS)

11.	Perform the subtraction of 1000 1000 – 1110 0010.

12.	Draw the logic circuit with four input variables that will only produce a 1 output when exactly three inputs are 1’s.

13.	Define Propagation delay, set up time and hold time of a flip flop.

14.	Give out the characteristic table for a JK flip flop.

15.	What are the features of Logic Element in FPGA?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Simplify the Boolean expression [AB'(C+BD)+A'B']C.                              			(5)

b.	Use a Karnaugh map to minimize the following standard SOP 5 – variable expression:

X=A'B'C'D'E'+A'B'CD'E'+A'BC {D'E'+A'BC'D'E'+A'B'C'D'E+A'BC {D'E+A'BCDE       +AB'C'D'E'+AB'C'D'E+ABC {D'E+ABCDE+AB'CDE.						(10)

(OR)

17.	a.	Convert the Gray code 10101111 to equivalent Binary.                                			(3)

b.	The code 101101010 is received at the receiver. Correct any errors. There are four parity 	bits and odd parity is used.                                                                        				(12)

18.	a.	Show the carry generation and propagation that happens in the four bit parallel adder. 	Support your answer with a logic design and truth table.                                   		(12)

b.	Give out the logic diagram of a 4 bit binary to gray code converter.            			(3)

(OR)

19.	a.	What is Parity? How does a 9 bit parity generator / checker operate?          			(10)

b.	With a complete circuit and truth table brief the working of a full adder.      			(5)

20.	What is Race around condition? Suggest any two suitable solutions to overcome the same.

(OR)

[P.T.O]

21.	Construct a 4 bit synchronous Binary counter using T flip flops and elaborate its operation with timing diagram.

22.	A sequential circuit with two D flip flops A and B, two inputs X and Y and one output Z is specified by the following state equations.

A(t+1)   =  X'Y+XA.

B(t+1)   =  X'B+XA

Z  = B

a.	Draw the logic diagram.

b.	Derive the state table.

c.	Derive the state diagram.

(OR)

23.	What is state reduction? For the given state table, draw the state diagram and apply state reduction technique and obtain reduced state table.

| Present State | Next state |  | Output (Y) |  |
| --- | --- | --- | --- | --- |
|  | X = 0 | X= 1 | X= 0 | X= 1 |
| A | B | F | 0 | 1 |
| B | C | D | 0 | 1 |
| C | D | E | 1 | 1 |
| D | B | F | 0 | 1 |
| E | F | A | 1 | 1 |
| F | D | E | 1 | 1 |

24.	Draw the basic structure of PAL. Show how is a PAL programmed for the following three variable logic function.

X=AB'C+A'BC'+A'B'+AC

(OR)

25.	a.	Compare the performance of CMOS and TTL.                                             			(5)

b.	Explain the basic CPLD macrocell architecture with a neat diagram.         			(10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	COMMUNICATION ENGINEERING				      	     Time : 3 hours

Subject Code:	09EE203										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The maximum efficiency transmission in AM with large carrier is ___________.

2. 	The purpose of a balanced modulator is to suppress ___________.

3. 	Draw the waveforms of Pulse Position Modulation.

4. 	Define Staircase Approximation.

5. 	Give some Digital Modulation techniques.

6. 	Give the expression for average error probability function of the matched filter.

7. 	Give one advantage of optical fiber over coaxial cable.

8. 	Define Total Internal Reflection.

9. 	Define Horizontal scan rate/horizontal frequency.

10. 	In a Television transmission the filter which is used to suppress a portion of one side band is called ___________.

PART – B  (5 x 3 = 15 MARKS)

11. 	Compare Narrowband and Wideband FM.

12. 	What are the merits and demerits of delta modulation?

13. 	Derive the expression for probability of error in ASK.

14. 	Explain the concept of multiplexing and its types.

15. 	Explain the synchronization operation used in Television System.

PART – C  (5 x 15 = 75 MARKS)

16. 	With a neat block diagram, explain the working of an FM transmitter.

(OR)

17. 	With a neat block diagram, explain the working of an AM receiver.

18. 	Explain Pulse Amplitude Modulation in detail with relevant expressions.

(OR)

19. 	Discuss about Pulse Position Modulation in detail.

20. 	Derive the expression for probability of error and explain briefly about FSK transmission system.

(OR)

21. 	Explain in detail about the matched filter technique.

22. 	Explain the concept of FDM in detail.

(OR)

23. 	Discuss in detail the block diagram of Optical Transmission Link.

24. 	Describe the construction and working principle of a Cathode Ray Tube.

(OR)

25. 	Describe the phase error cancellation process in a PAL TV system in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	ENERGY SYSTEMS									      Time : 3 hours

Subject Code:	09EE205			       						      	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is difference between solar thermal and solar electric systems?

2.	What is vertical axis wind Turbine?

3.	Is MHD generated power in D. C. Type or AC type?

4.	What is difference between 1-basin and 2-basin ocean energy?

5.	Define candle power.

6.	What is eddy current in induction heating?

7.	What is Rheostatic braking?

8.	Define specific energy consumption.

9.	What is the principle of van de Graff  generation?

10.	What is meaning of 1.2/50 sec impulse.

PART – B  (5 x 3 = 15 MARKS)

11.	What is difference between synchronous and asynchronous generators used in wind 	generation?

12. 	What is closed-loop MHD generation?

13.	Which type of Lamps are used in stadium floodlight?

14.	What is coasting and Braking used in quadrilateral speed-time curve?

15. 	What is Cockroff-Walton in D. C. generation?

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain different solar energy conversion schemes in detail.

(OR)

17. 	Derive that the power of wind is directly proportional to the cube of velocity.

18.	Explain 2-basin tidal energy generation and mention its advantages compared to 1-basin systems.

(OR)

19. 	Explain closed-loop MHD generation with a neat diagram.

20. 	Discuss about Dielectric heating and its application and show that it depends on loss angle 𝛿.

(OR)

21. 	Explain Direct and indirect resistance heating with their merits and demerits.

22. 	The distance between two stations is 1.92 KM. The schedule speed and duration of stops are 	respectively 40 kmph and 20 seconds. Assume quadrilateral approximation, the coasting and 	braking retardation as 0.16 kmph and 3.2 kmph. Determine acceleration if the speed at the end 	of the 	acceleration period is 60.8 kmph. Also find duration of coasting period.

(OR)

23. 	Explain about the series – parallel D.C motor control for electric traction.

24. 	 Discuss about the 3-stage cascade transformer for generating high A. C. voltage.

(OR)

25.	Explain about the generation of high DC voltage using voltage multiplier and discuss about 	output  	waveform and its ripple content.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	ELECTRIC CIRCUITS AND NETWORKS				      Time : 3 hours

Subject Code:	10EE201							      		       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	If a 100 Ω resistor is connected across the terminals of a 2.5 V battery what is the power dissipated in the resistor?

2.	The number of independent loops for a network with ‘n’ nodes and ‘b’ branches is ___________.

3.	Superposition theorem is valid only for____________.

4.	What is meant by duals and duality?

5.	What is the phase angle of a series RLC circuit at resonance?

6.	If the lower cut-off frequency is 2400 Hz and the upper cut-off frequency is 2800 Hz, what is the bandwidth?

7.	Wattmeter deflection in AC circuit is proportional to the __________.

8.	Mutually induced voltage is present independent of,  and in addition to, the voltage due to self induction. Say true or false.

9.	Which parameters are widely used in transmission line theory?

10.	A band pass filter may be obtained by using a high pass filter followed by a low pass filter. Say true or false?

PART – B  (5 x 3 = 15 MARKS)

11.	An electric heater draws 1000 W from a 230 V supply. What power will it take if the voltage is 210 V? What is the resistance of heater?

12.	State the reciprocity theorem.

13.	List the restrictions on pole zero locations for driving point functions.

14.	Define coefficient of coupling.

15.	Draw and write the overall ABCD parameters for a two port networks in cascade.

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the current delivered by the source in the circuit shown in the figure.

[P.T.O.]

(OR)

17.	Use nodal analysis to find the power dissipated in the 6Ω resistor for the circuit shown in the figure.

18.	Verify the reciprocity theorem for the given circuit shown in the figure.

(OR)

19.	Use Thevenin’s theorem to find the current through the 5Ω resistor in the figure.

20.	Derive the sinusoidal response of a RLC series circuit.

(OR)

21.	A 50µF capacitor and 20000 ohm resistor are connected in series across a 100 V battery at          t = 0. At t = 0.5 s, the battery voltage is suddenly increased to 150 V. Find the charge on capacitor at t = 0.75 s.

22.	Derive the coefficient of coupling of a coupled circuit.

(OR)

23.	A three-phase delta connected RYB system with an effective voltage of 400 V, has a balanced load with impedances 3+j4Ω. Calculate the 		i.	phase currents     ii.	line currents

iii.	power in each phase.                                       								(5+5+5)

24.	The Z parameters of a two-port network are Z11=10Ω, Z22=15Ω; Z12=Z21=5Ω. Find the equivalent T network and ABCD parameters.

(OR)

25.	Design and derive a constant K low pass filter.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	DIGITAL SIGNAL PROCESSING					      	     Time : 3 hours

Subject Code:	10EE202										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define discrete sinusoidal signal.

2.	The phenomenon of high frequency components acquiring the identity of low frequency component is called __________.

3.	Appending zeros to a sequence in order to increase its length is called__________.

4.	How many multiplications are involved in radix-2 FFT?

5.	Butterworth filters have

a.	Wideband transition region				b.	Sharp transition region

c.	Oscillation in the transition region			d.	None

6.	Type -2 Chebyshev filter contains oscillation in __________ region.

7.	FIR filters are always unstable (True/ False).

8.	Write the equation for Hamming window function.

9.	The _________ unit in DSPs can perform multiplication and addition in a single cycle.

10.	What is CALU?

PART – B  (5 x 3 = 15 MARKS)

11.	Define standard discrete time signals.

12.	List the application of wavelet transform.

13.	Obtain the Direct Form-II realization for the system described by the difference equation

y(n)−18y(n−1)+17y(n−2)+16y(n−3)=x(n)−12x(n−1)+13x(n−2)−14x(n−3).

14.	Illustrate about optimum equiripple approximation. Why is it followed?

15.	What is pipelining in DSPs?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Test the linearity of the system y(n) = x(n) – x(n – 1)								(3)

b.	Test whether the given system is invariant y(n) = x(n) x(n + 1)						(3)

c.	Test if the given signal x(n) = u(n)  is an Energy or Power	 signal.					(3)

d.	Find the odd and even component of x(n)={ 3 , 2 ,1 , 4 , 5 }						(3)

e.	Test  the causality of the given system   y(n)=x(k).							(3)

(OR)

17.	a.	Explain about the basic operation of the signal based on dependent and independent           	variable.																		(8)

b.	Find x(2n+3) for the given signal x(n)={ 1 , 1 , 1 , 1 , 1 , 1 , 1 }.					(3)

c.	Express the given signal x(n) ={ 1 , 2 , 8 , 4 , 5 , 3 , 7 } sequence as a time shifted          impulse.																		(4)

[P.T.O.]

18.	An 8 point sequence is given by x(n) ={ 2 , 1 , 2 , 1 , 1 , 2 , 1 ,2 }. Compute 8 point DFT of x(n) by radix-2 DIT FFT

(OR)

19.	a.	Find the Inverse FFT of the given sequence Y(K) = { –12 , 4 , 0 , 4 } by DIF FFT.	(6)

b.	Determine the 4 point DFT  of the given signal x(n)=1,0≤n≤30,elsewhere				(5)

c.	Distinguish between Linear and Circular Convolution.	   	             				(4)

20.	Design a Butterworth band pass filter with following specifications:

ωL = 0.25π  rad/sample	; ωU = 0.35 π rad/sample ; αp= –3 db ; αs= –15 db;  ω1=0.1π ; ω2=0.7π      using Bilinear Transformation.

(OR)

21.	State the procedure to design Chebyshev filter.

22.	Design an ideal low pass filter using rectangle window of N = 9 whose desired frequency response is  Hd(ejω)=1,π3≥ω≥−π30,otherwise   and determine:

i.	h(n) 		ii.	H(z) 		iii.	 H()

(OR)

23.	Design an ideal BPF whose desired frequency response Hd(ejω)=1,2π3≥ω≥π30,otherwise  using Hanning window. Determine:		a.	The impulse response for N = 9 	b.	H(Z)

24.	Describe about the architectures of Von Neumann and Harvard. Also differentiate the above two architectures.

(OR)

25.	Explain any two types of pipeline architecture based Digital Signal Processors.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	RENEWABLE ENERGY SOURCES						      Time : 3 hours

Subject Code:	10EE203			       				               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by direct radiation?

2. 	What is Photovoltaic effect?

3. 	Define power co efficient in wind mill.

4.  	What is meant by yaw control?

5.  	Mention the classification of biogas plants.

6.  	What is fermentation?

7. 	Give few utilizations of hydrogen gas.

8.  	List the main components of a fuel cell.

9. 	What are spring tides?

10. 	List few applications of geothermal energy.

PART – B  (5 x 3 = 15 MARKS)

11.	Mention the main types of concentrating collectors.

12. 	What is meant by cut in speed in wind mill?

13.  	Mention the disadvantages of floating drum plant.

14.  	What are the methods considered for hydrogen storage system?

15.  	What are the factors to be considered while selecting a site for hydro power plant?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the various types of solar heating systems.

(OR)

17. 	With a neat sketch explain the working principle of photovoltaic cell. What are the applications of photovoltaic system?

18. 	Explain the basic components of wind energy conversion system.

(OR)

19. 	Explain the construction and operation of various types of wind machines.

20. 	Discuss the factors affecting biodigestion in a biogas plant.

(OR)

21. 	Explain any two types of biogas plants with a neat sketch.

22. 	Explain the construction and working principle of a fuel cell.

(OR)

23. 	Discuss any one method to produce hydrogen gas with a neat sketch.

24. 	With a neat sketch, explain the working of open cycle ocean thermal energy conversion system.

(OR)

25. 	Write in detail about geothermal energy plants types and sites selection process.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	ELECTRONIC  DEVICES							      	     Time : 3 hours

Subject Code:	11EE203								   		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The reverse breakdown voltage of a semiconductor diode will increase or decrease with temperature depending on the ____________.

2. 	Draw the equivalent circuit of Zener diode.

3. 	Early effect is also known as _____________.

4. 	If both the emitter-base and the collector-base junctions of a bipolar transistor are forward biased, the transistor is in the ___________region.

5. 	In JFET voltage divider biasing, increasing the values of Rs result in lower quiescent values of __________and more negative values of__________.

6. 	In JFET, self biasing configuration eliminates the need of___________.

7. 	For values of Vgs less than the threshold level, the drain current of an enhancement-type MOSFET is __________mA.

8. 	What is CMOS Logic?

9. 	What are the two methods used to fabricate monolithic IC Capacitors?

10. 	What are the two common isolation techniques of monolithic IC fabrication?

PART – B  (5 x 3 = 15 MARKS)

11. 	Determine the DC resistance levels for the diode of the figure shown. For 		a. Id = 2mA,           b.	 Id = 20 mA 		c.	Vd = –10 V.

12. 	Write a short note on early effect of BJT.

13. 	Configure a MOSFET as a Resistor.

14. 	Find the ohmic resistance level of n-channel JFET, if r 0 = 10 kohm, Vgs = –3 V and Vp = –6V.

15. 	Design a 4 K ohm diffused resister, if the sheet resistance of p-type diffusion is 200 ohm / sq.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Calculate the junction capacitance of a pn junction. Consider a silicon pn junction at           	T = 300 K, with doping concentrations of Na = 1016 cm−3 and Nd = 1015 cm−3. Assume 	that ni = 1.5 × 1010 cm−3 and let Cjo = 0.5 pF. Calculate the junction capacitance at           	VR = 1V and VR = 5 V.															    (5)

[P.T.O.]

b.	Explain Zener diode characteristics with the equivalent model for each region and the effect of temperature coefficient. 												  (10)

(OR)

17. 	a.	Find the output voltage for the given circuit. 										    (8)

b.	Determine V0 for the given circuit.												   (7)

18. 	In detail, explain about Ebers-Moll model and equations of BJT.

(OR)

19. 	a.	Distinguish Common Emitter (CE) and Common Collector configuration.			   (6)

b.	Explain in detail about the common emitter mode of operation of BJT. 				   (9)

20. 	a.	Distinguish Self and Fixed Biasing of FET. 										  (10)

b.	Compare pMOS and nMOS devices. 											   (5)

(OR)

21. 	Draw and explain the drain and transfer characteristics of n-channel enhancement type MOSFET and derive the drain current equation.

22. 	In detail, explain about the construction and VI characteristics of n-channel and p-channel JFET.

(OR)

23. 	a.	Explain in detail about voltage divider biasing of JFET. 							  (10)

b.	Describe the effect of Rs on resulting Q point of Voltage divider biasing JFET. 		   (5)

24. 	a.	Write short note on diffusion process of IC technology. 							   (5)

b.	Define sheet resistance and feature size. 										   (4)

c.	Explain in brief about any two methods of realizing monolithic IC resistors. 			   (6)

(OR)

25. 	With a neat sketch, explain in detail about n-channel MOSFET fabrication process.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	ELECTROMAGNETIC FIELDS						      	     Time : 3 hours

Subject Code:	11EE206						      			       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give the types of Co-ordinate systems.

2.	What is the divergence of curl of a vector?

3.	Define electric field intensity.

4.	Define Polarization.

5.	Mutual inductance between two coils is related to their self inductances by M =_________.

6.	Define magnetic vector potential.

7.	What is motional emf?

8.	State Faraday’s law of induction.

9.	Define propagation constant.

10.	Write down the instantaneous Poynting vector equation.

PART – B  (5 x 3 = 15 MARKS)

11.	Show that the vector H = 3 y4z2 ax + 4 x3z2ay +3 x2y2 az is solenoidal.

12.	Write down the Poisson and Laplace equations.

13.	What are the types of magnetic materials?

14.	Write short notes on transformer emf.

15.	Define skin depth.

PART – C  (5 x 15 = 75 MARKS)

16.	Check the validity of the divergence theorem considering the field D = 2xy ax+x2 ay c/m2 and the rectangular parallelopiped formed by the planes x = 0, x = 1, y = 0, y = 2 and z = 0, z = 3.

(OR)

17. 	a.	State and prove Divergence Theorem.                                                                               (8)

b.	State and prove Stokes Theorem.                                                                                       (7)

18. 	Derive the boundary conditions of the normal and tangential components of electric field at the interface of two media with different dielectrics.

(OR)

19. 	Derive an expression for potential due to an infinite uniformly charged line and also derive the 	potential due to electric dipole.

20.	Derive the boundary conditions at the interface between two different magnetic materials.

(OR)

21. 	a.	Find the magnetic flux density and magnetic field intensity at any point along the axis of

a circular coil.                                                                                                                    (10)

b.	Find the force between two parallel conductors.                                                               (5)

[P.T.O.]

22. 	Write short notes on the following:

a.	Displacement current.                                                                                                   	(8)

b.	Eddy currents.                                                                                                                	(7)

(OR)

23. 	With necessary explanation, derive the Maxwell’s equation in differential and integral forms.

24.  	Derive a wave equation for non dissipative medium making use of Maxwell equations and field vectors E and H.

(OR)

25.	Briefly explain about the wave incident:

a.	Normally on a perfect conductor.     											(7)

b.	Obliquely to the surface of perfect conductor. 									(8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	ELECTROMAGNETIC FIELDS						      	     Time : 3 hours

Subject Code:	11EE206						      			       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give the types of Co-ordinate systems.

2.	What is the divergence of curl of a vector?

3.	Define electric field intensity.

4.	Define Polarization.

5.	Mutual inductance between two coils is related to their self inductances by M =_________.

6.	Define magnetic vector potential.

7.	What is motional emf?

8.	State Faraday’s law of induction.

9.	Define propagation constant.

10.	Write down the instantaneous Poynting vector equation.

PART – B  (5 x 3 = 15 MARKS)

11.	Show that the vector H = 3 y4z2 ax + 4 x3z2ay +3 x2y2 az is solenoidal.

12.	Write down the Poisson and Laplace equations.

13.	What are the types of magnetic materials?

14.	Write short notes on transformer emf.

15.	Define skin depth.

PART – C  (5 x 15 = 75 MARKS)

16.	Check the validity of the divergence theorem considering the field D = 2xy ax+x2 ay c/m2 and the rectangular parallelopiped formed by the planes x = 0, x = 1, y = 0, y = 2 and z = 0, z = 3.

(OR)

17. 	a.	State and prove Divergence Theorem.                                                                               (8)

b.	State and prove Stokes Theorem.                                                                                       (7)

18. 	Derive the boundary conditions of the normal and tangential components of electric field at the interface of two media with different dielectrics.

(OR)

19. 	Derive an expression for potential due to an infinite uniformly charged line and also derive the 	potential due to electric dipole.

20.	Derive the boundary conditions at the interface between two different magnetic materials.

(OR)

21. 	a.	Find the magnetic flux density and magnetic field intensity at any point along the axis of

a circular coil.                                                                                                                    (10)

b.	Find the force between two parallel conductors.                                                               (5)

[P.T.O.]

22. 	Write short notes on the following:

a.	Displacement current.                                                                                                   	(8)

b.	Eddy currents.                                                                                                                	(7)

(OR)

23. 	With necessary explanation, derive the Maxwell’s equation in differential and integral forms.

24.  	Derive a wave equation for non dissipative medium making use of Maxwell equations and field vectors E and H.

(OR)

25.	Briefly explain about the wave incident:

a.	Normally on a perfect conductor.     											(7)

b.	Obliquely to the surface of perfect conductor. 									(8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	INDUCTION AND SYNCHRONOUS MACHINES	     	     Time : 3 hours

Subject Code:	11EE209										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In what respect does the Squirrel Cage Induction Motor differ from Slip Ring Induction Motor?

2.	Define slip of a three phase induction motor.

3.	Draw the torque-slip characteristic of a single phase induction motor.

4.	What will be the direction of rotation of a shaded pole single phase induction motor?

5.	How does electrical angle differ from mechanical angle?

6.	Why is fractional pitch winding preferred over full pitch winding?

7.	Name any two methods of starting a Synchronous Motor.

8.	In what operating condition is a synchronous motor referred as a Synchronous condenser?

9.	List the advantages of Salient Pole Synchronous Machines.

10.	Write down the equation for power developed in a Salient Pole Synchronous Machine.

PART – B  (5 x 3 = 15 MARKS)

11.	What is Crawling?

12.	State the reasons for the inferior performance of a Single Phase Induction motor compared to a Three Phase Induction motor.

13.	State the conditions of Synchronism.

14.	Explain the term Hunting.

15.	Based on two reaction theory, draw and explain the phasor diagrams of a salient pole alternator supplying loads at lagging PF and leading PF.

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	Derive the torque equation of a three phase induction motor and draw the torque slip    	characteristics for various rotor resistances.  										(8)

b. 	A three phase induction motor on full load, delivers a torque of 73.3Nm with speed of 1433rpm when the supply frequency is 49.5Hz.  The friction and windage losses are 250W and the stator copper and iron losses amount to 900W.  Calculate the 	i.	 mechanical power developed 		ii.	rotor copper loss 		iii.	efficiency at this load condition. 																				(7)

(OR)

17.	Briefly describe the various methods of speed control adopted for three phase induction motors.

18.	Using double revolving field theory, explain why a single phase induction motor is not self starting.

(OR)

19.	Discuss about the different methods of starting a single phase induction motor, indicating the limitation and application of each method.

[P.T.O.]

20.	A 1.1MVA, 2.2 kV, three phase alternator gave the following test results:

| Field Current (A) | 10 | 20 | 30 | 40 | 50 |
| --- | --- | --- | --- | --- | --- |
| Open circuit voltage(kV) | 0.88 | 1.65 | 2.2 | 2.585 | 2.86 |
| Short circuit test (A) | 200 | 400 | - | - | - |

The effective resistance of the winding is 0.22ohm/phase. Estimate the full load voltage regulation at 0.8pf lagging by:

a.	Synchronous Impedance Method 		b.	Ampere turn method.

(OR)

21. 	a. 	Describe any two methods of synchronizing the alternators. 						(6)

b. 	Derive an expression for the synchronizing torque of a three phase synchronous generator 	on no load. 																	(9)

22.	Obtain expressions for the power input and power developed of a synchronous motor in terms of the load angle.

(OR)

23.	Explain V and Inverted V-curves of a Synchronous motor.

24.	A three phase star connected 11kV, 12 pole, 50Hz, salient pole synchronous motor has reactance of Xd = 5Ω and Xq = 3Ω. At certain load, the motor draws 20MW at unity pf.  Compute:

a.	the excitation voltage 		b.	power developed 		c.	synchronizing power per electrical degree and corresponding torque 		d.	synchronizing power per mechanical degree and the corresponding torque.

Neglect the resistance of the winding.

(OR)

25.	Describe the slip test for finding Xd and Xq experimentally.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	GENERATION, TRANSMISSION AND DISTRIBUTION	      Time : 3 hours

Subject Code:	11EE215										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Why do we endeavor to use water power for the generation of electrical energy?

2. 	The knowledge of diversity factor helps in determining ___________.

3. 	D.C. transmission is ___________ to A.C. transmission

4. 	Primary transmission is done by 3-phase ___________wire A.C. system

5. 	What is Corona?

6. 	Define Sag.

7. 	The most commonly used insulation in high-voltage cables is ___________.

8. 	In capacitance grading of cables, we use a ___________.

9. 	The main consideration in the design of a feeder is the ___________.

10. 	The service mains connect the ___________ and the ___________.

PART – B  (5 x 3 = 15 MARKS)

11. 	How do load curves help in the selection of size and number of generating units?

12. 	What are the advantages and disadvantages of high transmission voltages?

13. 	List out the desirable properties of an Insulator.

14. 	A single-core cable has a conductor diameter of 1cm and insulation thickness of 0.4 cm. If the specific resistance of insulation is 5 × 1014 Ω-cm, calculate the insulation resistance for a 2 km length of the cable.

15. 	Draw a single line diagram showing a typical distribution system.

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	Explain the working of a gas turbine power plant with a schematic diagram.		 	(8)

b. 	A generating station has a maximum demand of 25MW, a load factor of 60%, a plant  capacity factor of 50% and a plant use factor of 72%. Find:	i.	the reserve capacity of the plant 	ii.	the daily energy produced 		iii.	maximum energy that could be produced daily if the plant while running as per schedule, were fully loaded.			(7)

(OR)

17. 	Give the comparison of steam power plant, hydro-electric power plant, diesel power plant and

nuclear power plant on the basis of operating cost, initial cost, efficiency, maintenance cost and availability of source of power.

18. 	a.	Deduce an expression for line to neutral capacitance for a 3-phase overhead transmission

line when the conductors are:

i.	Symmetrically placed 		ii.	Unsymmetrically placed but transposed			(10)

b. 	A single phase overhead transmission line delivers 1100 kW at 33 kV at 0·8 p.f. lagging.

The total resistance and inductive reactance of the line are 10 Ω and 15 Ω respectively.

Determine : 		i.	sending end voltage 	ii.	sending end power factor

iii.	transmission efficiency													(5)

[P.T.O.]

(OR)

19. 	A (medium) single phase transmission line 100 km long has the following constants: Resistance/km = 0.25 Ω; Reactance/km = 0.8 Ω; Susceptance/km = 14 ×10-6 siemen; Receiving end line voltage = 66,000 V. Assuming that the total capacitance of the line is localized at the receiving end alone, determine:

a.	the sending end current 	b.	the sending end voltage 	c.	regulation 	d.	supply power factor.

The line is delivering 15,000 kW at 0.8 power factor lagging. Draw the phasor diagram to illustrate your calculations.

20. 	a.	Discuss the advantages and disadvantages of:	i.	pin-type insulators ii.	suspension 			type insulators.											 					(5)

b.	Define String Efficiency. Show that in a string of suspension insulators, the disc nearest to

the conductor has the highest voltage across it.  									(10)

(OR)

21. 	a.	What are the factors which affect corona? Discuss the advantages and disadvantages of      		Corona.																		(5)

b.	Deduce an approximate expression for sag in overhead lines when:			 		(10)

i.	supports are at equal levels       ii.	supports are at unequal levels.

22. 	a.	Write down the Classification of Cables based on their voltage range and draw the sketch

of a single-core low tension cable and label the various parts. 						(5)

b.	Explain the following methods of cable grading:

i.	Capacitance grading  		ii.	Inter sheath Grading. 							(10)

(OR)

23. 	a.	Compare the merits and demerits of undergrouand system versus overhead system.	(5)

b.	A single-core 66 kV cable working on 3-phase system has a conductor diameter of 2 cm and a sheath of inside diameter 5.3 cm. If two inter sheaths are introduced in such a way that the stress varies between the same maximum and minimum in the three layers, find:

i.	positions of inter sheaths 		ii.	voltage on the inter sheaths 	iii.	maximum and minimum stress. 																(10)

24. 	Explain the following systems of distribution:

a.	Radial system		b.	Ring main system		c.	Interconnected system	.

(OR)

25. 	a.	How do A.C. distribution calculations differ from D.C. distribution? 				(5)

b.	Explain briefly two methods of solving A.C. distribution problems.					(10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	CONTROL SYSTEMS									      Time : 3 hours

Subject Code:	11EE219				       			      		       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Transfer Function.

2.	What is a synchro?

3.	How will you identify the type of a system?

4.	What are the advantages of generalized error series?

5.	What are the disadvantages of frequency response analysis?

6.	Define Resonant frequency.

7.	State Nyquist stability criterion.

8.	Define System stability.

9.	What are the advantages of phase variable method?

10.	What are stable variables?

PART – B  (5 x 3 = 15 MARKS)

11.	Explain Mason’s gain formula.

12.	A system has a closed loop transfer function, T(s)=4s2+1.6s+4.	Find the settling time for a unit step input.

13.	The dampling ratio and natural frequency of oscillation of second order system is 0.5 and 8 rad/sec respectively.  Calculate the resonant peak and resonant frequency.

14.	Explain how the breakaway points of a root locus are determined.

15.	Mention the procedure for controllable canonical form realization.

PART – C  (5 x 15 = 75 MARKS)

16.	Obtain the transfer function X1(s)f(s), for the mechanical system shown in the figure.

(OR)

17.	Draw the signal for graph for the block diagram shown in the figure and obtain the overall transfer function using Mason’s gain formula.

[P.T.O]

18.	A unity feedback system has the forward transfer function.

G(s)=k(2s+1)s(5s+1)(s+1)2. The input r (t) = 1+16t is applied to the system. Determine the value of k for a steady state error to be less than 0.1.

(OR)

19.	Consider a system as shown in the figure. The system is initially at equilibrium with r (t) =1 and d (t) = 0.  A step function d (t) = u(t) is applied at t = 0.  Determine the output response c (t) for t > 0.

20.	A Unity feedback system has a forward loop transfer function, G(s)=4s(s+1).	Determine the closed loop bandwidth, closed loop peak magnitude, gain margin and phase margin.

(OR)

21.	The open loop transfer function of a unity feedback system is given by. G(s)=ks(1+0.02s)(1+0.04s). 	Draw the Bode plot.  Find the gain margin and phase margin of the system.  Find the value of open loop gain so that the system has a phase margin of 45º.

22.	For a unity feedback system’s open loop transfer function is given by, G(s)=ks(s+1)(s+2)(s+5). Using Routh-Hurwitz criterion, find:		a.	The range of k for stability		b.	the value of k for marginal stable condition		c.	the location of closed loop poles when the system is marginally stable.

(OR)

23.	For a unity feedback system, whose openloop transfer function is given by, G(s)=5s(s+1)(s+2). Draw the Nyquist plot and hence findout whether the system is stable or not.

24.	Obtain the cascade form realization of the transfer function,

H(s)=5(s2+3s+2)(s2+8s+15)(s+0.5).

(OR)

25.	Obtain the state space representation of a system whose transfer function is given by

T(s)=10(s+4)s(s+1)(s+3), using controllable canonical form.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	LINEAR INTEGRATED CIRCUITS					      	     Time : 3 hours

Subject Code:	11EE220										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List the important characteristics of ideal operational amplifier.

2.	What is PSRR? What should be its ideal value?

3.	Give the comparison between ideal inverting and non inverting amplifier.

4.	Determine the output voltage (Vo) of an inverting summing amplifier with Rf = 10KΩ, R1 = 4KΩ, R2 = 2KΩ, V1 = 3V and V2 = 2V.

5.	Give the differences between Schmitt trigger and comparator.

6.	Draw the circuit diagram of a positive half wave precision rectifier.

7.	What is a voltage regulator?

8.	State any two applications of a monostable multivibrator.

9.	Determine the resolution of an 8-bit analog to digital converter for a 10V input range.

10.	Define lock range of PLL.

PART – B  (5 x 3 = 15 MARKS)

11.	A 741C operational amplifier whose slew rate is 0.5V/µs is used as an inverting amplifier with a gain of 50. The voltage gain against frequency curve of 741C is flat up to 20kHz. What maximum peak to peak input signal can be applied without distorting the output?

12. 	Draw the circuit of a voltage follower using operational amplifier and prove that its gain is exactly equal to unity.

13. 	Explain non-inverting zero crossing detector.

14. 	Write the applications of a VCO.

15. 	State the advantages of R-2R DAC.

PART – C  (5 x 15 = 75 MARKS)

16.	Briefly explain external compensation techniques used in operational amplifier circuits.

(OR)

17.	List and explain the non-ideal DC characteristics of an operational amplifier.

18.	What are the requirements of good instrumentation amplifier? Discuss the working of three

operational amplifier based instrumentation amplifier.

(OR)

19.	Draw the circuit diagram of a second order Butterworth active low pass filter and derive an expression for its transfer function.

20.	Briefly explain the following:

a.	Antilog amplifier 															(10) 	b.	Positive clipper circuit using operational amplifier and a diode. 					(5)

(OR)

[P.T.O.]

21.	a.	Design an inverting Schmitt trigger to have supply of trigger voltage of ±4V. Use  	operational amplifier with a supply of ±15V, IB(max) = 500nA.  					(7)

b.	With a neat circuit diagram explain inverting AC amplifier. 						(8)

22.	Explain how IC 555 can be used as a monostable multivibrator. Derive an expression for its

pulse width.

(OR)

23.	a.	With a neat functional diagram explain 723 general purpose regulator. 				(10)

b.	A 555 timer is configured in astable mode with RA=2kΩ, RB=4kΩ and C=0.1µF. 				Determine the frequency of output and duty cycle. 								(5)

24.	a.	Explain the working of a successive approximation A/D converter. 					(8)

b.	Discuss the working of a weighted resistor digital to analog converter. 				(7)

(OR)

25.	Describe the working of a PLL using appropriate block diagram. Derive the expression for lock-in range.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	C++ AND DATA STRUCTURES						      	     Time : 3 hours

Subject Code:	11EE223								      	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a scope resolution operator?

2.	____________header file should be included to use exit function.

3.	List out any two operators that cannot be overloaded in C++.

4.	____________ allows you to create a derived class that inherits properties from more than one base class.

5.	A class that declares or inherits a virtual function is called ___________.

6.	Among public, private and protected scope keyword, the ___________ and ___________ keywords do not apply to friend functions, as the class has no control over the scope of friends.

7.	A ___________ is a linear list in which additions and deletions take place at the same end.

8.	After creating a linked list’s head pointer, one should make sure it points to ___________ before using it in any operations.

9.	Average case performance of Heap sort is ___________.

10.	In a complete binary tree of ‘n’ levels, there are ___________ and ___________ non-leaf nodes.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the difference between class and structure?

12.	What are the advantages of inheritance?

13.	What is a template?

14.	When is contiguous storage preferred for implementing a list?

15.	Differentiate linear search and binary search.  Compare its complexity also.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the constructor facility available in C++.  Also, explain how to overload a constructor.

(OR)

17.	Write a C++ program to add two complex numbers by passing objects from and to a function.

18.	Write a C++ program to overload ( ) operator.

(OR)

19.	Define Inheritance. With a suitable example, explain different types of inheritance supported by C++.

20.	With a necessary example, elucidate data file handling in C++.

(OR)

21.	With an example, discuss Function template and Class template.

22.	a.	List out the disadvantages of arrays over pointers. 								(5)

b.	Write a code to push an element as the head node of the list. 						(5)

c.	Write a code to find the length of the list.										(5)

[P.T.O.]

(OR)

23.	Explain the algorithm with code for a Circular queue.

24.	Discuss Insertion sort and implement it in C++.

(OR)

25.	a.	Define Binary Search Tree.													(2)

b.	Convert the following tree into binary search tree.								(6)

c.	Delete 20 from the binary search tree and reconstruct the tree.						(7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	MEASUREMENT AND INSTRUMENTATION		      	     Time : 3 hours

Subject Code:	11EE225										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give the method of obtaining experimental data.

2.	Write the expression of controlling torque developed in a spring controlled instrument.

3.	What is the working principle of an energy meter?

4.	What is ratio error?

5.	Write the bridge balance condition.

6.	What are the two types of potentiometer?

7.	What is an inductive transducer?

8.	Mention any two applications of LVDT.

9.	What is purpose of an error detector in a recorder?

10.	What is the use of a spectrum analyzer?

PART – B  (5 x 3 = 15 MARKS)

11.	What is a standard? What are the different types of standards?

12.	Write a brief note on creeping in an energy meter. How is it prevented?

13.	What is Maxwell’s bridge? Draw its circuit diagram.

14.	Discuss Piezoelectric effect.

15.	Draw the block diagram of a function generator.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the construction and working of PMMC instruments with a neat sketch. Also derive the torque equation.

(OR)

17.	Discuss the operating principle of dynamometer type instruments with a neat sketch and derive its torque equation.

18.	With a neat sketch explain induction type single phase energy meter.

(OR)

19.	Compare the working principle of CT and PT with necessary diagrams.

20.	Derive the Bridge balance condition of a Wheatstone bridge and explain it with Thevenin’s equivalent circuit.

(OR)

21.	a.	How will you measure capacitance using Schering’s Bridge?

b.	Explain about Hay Bridge. 													(8+7)

22.	Write a detailed note on various types of strain gauges.

(OR)

23.	a.	What are resistance potentiometers?

b.	Discuss about Thermistor.  													(7+8)

24.	Draw the Block diagram of a digital CRO and explain.

(OR)

25.	Write a note on strip chart and X-Y recorders with schematic diagrams.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	SEMICONDUCTOR MEMORY DESIGN AND TESTING																					Time : 3 hours

Subject Code: 	12EC375										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Explain in detail about the SRAM architecture and cell structure.

(OR)

2. 	Discuss in detail about SOI SRAMs technology.

3. 	What is DRAM? Explain the principle of DRAM.

(OR)

4. 	Present the rapid technology development in DRAM.

5. 	What is flash memory? Explain in detail about EPROM and EEPROM.

(OR)

6. 	Discuss in detail about flash memory architecture.

7. 	Compulsory:

Explain in detail about RAM fault models.

8. 	Elaborate in detail about reliability design.

(OR)

9. 	Discuss in detail about ferroelectric RAM with a required diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	ELECTRIC CIRCUITS AND NETWORKS			      	     Time : 3 hours

Subject Code:	12EE201										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Give one example for a distributed parameter network.

2. 	In dual circuits, capacitance is analogous to ____________.

3.	What is the resistance to be connected across a and b for maximum power transfer? Also find the maximum power.

4. 	Write the relationship between Thevenin’s voltage and Norton’s current.

5. 	Define unit step function. How it is useful in circuit analysis?

6. 	State initial value theorem. Mention its application in circuit analysis.

7.  	Define double tuned circuit.

8.  	Draw the voltage waveforms for the phase sequence RBY.

9.  	ABCD parameters are widely used in transmission line theory. True/False.

10. 	Draw the characteristics of an ideal low pass filter.

PART – B  (5 x 3 = 15 MARKS)

11. 	A 1.5-kW electric heater is connected to a 230-V source.

a.	How much current does the heater draw?

b.	If the heater is on for 45 minutes, how much energy is consumed in kilowatt-hours (kWh)?

c.	Calculate the cost of operating the heater for 45 minutes if energy costs ` 3/kWh.

12.  	State superposition theorem.

13.  	For the network function H(s)=s+9s2+5s+16 draw the pole-zero diagram. What is your comment about the stability?

14.  	Two identical coupled coils in series have equivalent inductance values of 0.08 H and 0.0354H. 	Find the values of L1, L2, M and k.

15. 	Two amplifier stages have to be connected in cascade. What is the main consideration about 	their connection?

[P.T.O.]

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Find the resistance between the terminals a and b. 								(5)

b.	Find the power dissipated in the 6Ω resistor using mesh analysis. 					(10)

(OR)

17.	a.	Find the resistance between the terminals a and b. 								 (7.5)

b.	Find the current through 4Ω resistor using nodal analysis. 							(7.5)

18. 	Find the current through 2Ω resistor using Norton’s theorem.

(OR)

19. 	A loud speaker is to be connected between the output terminals. What should be the impedance of the loud speaker to receive maximum power? Find the maximum power.

20.  	A series RL circuit with R = 10 Ω and L = 2H has a DC voltage V=80 volts applied at t=0. Determine the transient current, time constant of the circuit, current at t=0.1s, initial rate of rise of the current, the voltage across the resistor and the voltage across the inductor. Sketch the transient current.

(OR)

21.  	Derive the expression for transient current of a RC series circuit when excited by a sinusoidal 	source.

22. 	In the circuit shown, calculate the load current and load power. Take  Z1 = 60 − j100 Ω, Z2 = 30 + j40 Ω , and ZL = 80 + j60 Ω.

(OR)

23.  	Prove that by using the readings of two watt meters the total power and power factor of three phase circuits can be measured.

24. 	For the network in the figure below, determine the Y-parameters in the s domain.

(OR)

25.  Design a constant k-high pass filter (both T-section and π-section) to cut off at 5kHz. Assume a 	characteristic impedance of 600 Ω.  Find the frequency at which the section will have an 	attenuation of 15 dB.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	ELECTRON DEVICES								      	     Time : 3 hours

Subject Code:	12EE203										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In what rate the forward biased characteristics of the silicon diode shift to the left when the temperature of the diode increases?

2.	State an application of Zener Diode.

3.	Which configuration is suitable for the transistor to act as an amplifier?

4.	In which regions of the output characteristics does the BJT act as a switch?

5.	Draw the symbols of P channel and N channel JFETs.

6.	State the Shockley’s equation for JFET.

7.	What is the threshold voltage of MOSFET?

8.	What is the range of input impedance for the MOSFET?

9.	What is polysilicon?

10.	State different package of ICs.

PART – B  (5 x 3 = 15 MARKS)

11.	Compute the current through the resistance of the silicon diode circuit shown below.

4.3Ω

10V

12.	Draw the Ebers-Moll Model of BJT.

13.	Draw the circuit diagram for fixed biased JFET amplifier and write the expression for Vgs and ID.

14.	Draw the structure of n-channel enhancement type MOSFET.

15.	State the differences between ion implantation and diffusion.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Discuss the behavior of PN junction. 											(5)

b.	With necessary diagrams explain the Volt-Ampere characteristics of the Avalanche diode. 																					(10)

(OR)

17.	a.	Write short notes on Schottky barrier diode.										(7)

b.	Discuss the principle and operation of Light Emitting diodes.						(8)

18.	a.	What is the need of biasing for a transistor acting as an amplifier?					(3)

b.	With necessary diagrams and expressions discuss in detail about the voltage divider 	biasing of BJT amplifier. 														(12)

(OR)

19.	a.	Derive the relation between current gain α and β	of BJT.							(3)

b.	With a neat circuit diagram draw and explain the input and output characteristics of a CE 	configuration bipolar junction transistor.										(12)

[P.T.O]

20.	a.	Discuss the self biasing of the JFET. 											(7)

b.	Briefly discuss about the mathematical and the graphical analysis on fixing the Q-point for 	the self biased JFET Amplifier.													(8)

(OR)

21.	a.	With necessary diagrams discuss the operation of the JFET. 						(7)

b.	Draw and explain the drain and transfer characteristics of the JFET.				(8)

22.	a.	With relevant diagrams explain the operation of mosfet in enhancement and depletion 	mode. 																		(7)

b.	Draw the drain and transfer characteristics of the mosfet in the enhancement and 	depletion mode of operation.													(8)

(OR)

23.	a.	Compare PMOS and NMOS devices.											(3)

b.	Derive the expression for the drain current of the MOSFET.						(12)

24.	Write short notes on:

a.	Photolithography	.															(5)

b.	Ion Implantation.																(5)

c.	Oxidation.																	(5)

(OR)

25.	a.	With neat diagrams explain the fabrication steps of  Enhancement NMos Transistor.

(10)

b.	Write short notes on junction and dielectric isolation.								(5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	DC MACHINES AND TRANSFORMERS			      	     Time : 3 hours

Subject Code:	12EE207										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is flux density?

2.	List the types of electromechanical energy conversion systems.

3.	What is the function of a commutator in a D.C.generator?

4.	What is armature reaction?

5.	What is the purpose of a starter?

6.	What is the condition for maximum power of a D.C. motor?

7.	How can the leakage reactance of transformer winding be reduced?

8.	For a better transformer, should regulation be low or high?

9.	What are the losses in a transformer?

10.	What is an auto transformer?

PART – B  (5 x 3 = 15 MARKS)

11.	Define field energy and co energy.

12.	Draw the schematic diagram indicating flow of energy in the conservation of mechanical energy to electrical form.

13.	Define Regulation of transformers.

14.	Why are transformers rated in KVA?

15.	What are the necessary conditions to be satisfied for operating transformers in parallel?

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the expression for field energy and co energy in a singly excited system assuming constant voltage system.

(OR)

17.  a.	Write a note on flow of energy in electromechanical devices.       					    (5)

b.	Derive the expression for field energy and co energy in a doubly excited system assuming 	constant voltage system.                                     				             				  (10)

18.	a.	Define armature reaction. What are the effects of armature reaction? Name the various 	methods of decreasing the effects of armature reaction.	                                           	  (8)

b.	A 4 pole, lap connected d.c shunt generator has a useful flux per pole of 0.07Wb. The armature winding consists of 220 turns each of 0.004 resistance. Calculate the terminal voltage when running at 900 rpm, if the armature current is 50 A.	      		      		    (7)

(OR)

19.	a.	A four pole lap connected armature of DC shunt generator is required to supply the     	loads connected in parallel (1) 5 kW geyser at 250 V and (2)2.5kW lighting load also at 	250 V. The generator has an armature resistance of 0.2 ohm and a field resistance of       	250 ohm. The armature has 120 conductors in the slots and runs at 1000 rpm, allowing 1 V 	per brush for contact drops. Find 	i.	Flux per pole		ii.	Armature current per 	parallel path.																	   (9)

[P.T.O.]

b.	With a neat schematic diagram explain the principle working of a d. c. generator.	    (6)

20.	a.	With neat sketch, explain the working of 3 point starter with all protective devices. 	    (9)

b.	Derive the torque equation of DC motor.   		   			      	 				    (6)

(OR)

21.	a.	A 250 volt, DC shunt motor has an armature resistance of 0.5 ohms and a field resistance 	of 250 ohms. When driving at 600 rpm, a load, the torque of which is constant, the 	armature current is 20 Amps. If it is desired to raise the speed from 600 rpm to 800 rpm, 	find the resistance that must be inserted in the shunt field circuit, assuming magnetization 	curve to be unsaturated. 		                         				   		 			  (10)

b.	What is regenerative braking?  							  						    (5)

22.	a.	Derive the EMF equation of Transformer. 					    	 				    (8)

b.	A 30 kVA, 2.4 KV/120 V, 50 Hz transformer has a high voltage winding resistance of     0.1 ohms and a leakage reactance of 0.22 ohms. The low voltage winding resistance is 0.035 ohms and the leakage reactance is 0.012 ohms. Find the equivalent winding resistance, reactance and impedance referred to the: 	i.	high voltage side 		ii.	the low-voltage side.																    (7)

(OR)

23.	Explain the principle of working of a transformer with a neat diagram and deduce the equivalent circuit.

24.	a.	Give a detailed comparison of auto transformer with two winding transformer. 		   (6)

b.	Explain polarity test applicable for transformers. 		   			    			   	   (9)

(OR)

25.	With necessary circuit diagram explain how OPEN CIRCUIT and SHORT CIRCUIT tests are conducted on 1-phase transformer. Explain how its equivalent circuit can be obtained.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	C++ AND DATA STRUCTURES						      	     Time : 3 hours

Subject Code:	12EE227								   		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the difference between singly linked list and doubly linked list?

2.	Write the applications of a queue.

3.	What do you mean by linear search?

4.	How does a bubble sort differ from selection sort?

5.	What is the difference between a class and an object?

6.	What is an array? How can an array be declared?

7.	List out the advantages of using pointers.

8.	Describe about the new operator.

9.	What is the use of a friend function?

10.	Differentiate between a virtual function and a pure virtual function.

PART – B  (5 x 3 = 15 MARKS)

11.	State the difference between arrays and linked lists.

12.	Brief about Merge sort.

13.	Write a C++ program to find the factorial of a given number.

14.	Differentiate between a private and a public variable with an example.

15.	What is an exception? Why do we need to handle exceptions?

PART – C  (5 x 15 = 75 MARKS)

16.	What is a stack? Explain the operations of a stack. Write a suitable C++ program to perform operations of a stack using array implementation.

(OR)

17.	Explain about array implementation of Queue.

18.	Write a C++ coding to implement the following searching algorithms:

a. 	Binary search 		b. 	Linear search. 										(8+7)

(OR)

19.	Explain the Bubble sort procedure with an example.

20.	a.	What are constructors and destructors? Explain with an example how a constructor can be 	overloaded?																	    (8)

b.	Explain about copy constructor with an example.									    (7)

(OR)

21.	Write a C++ program using a class to print prime numbers between two limits.

22.	What is inheritance? Explain the types of inheritance with examples.

(OR)

23.	a.	Explain operator overloading with an example.									    (8)

b.	How are strings handled in C++?												    (7)

[P.T.O]

24.	What is a function template? Write its syntax and usage. Design a function template in C++ to find the maximum among two numbers of integer and long format

(OR)

25.	What is a virtual function?  Use a base class and two derived classes to demonstrate the use of a virtual function.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	RENEWABLE ENERGY SOURCES					      	     Time : 3 hours

Subject Code:	12EE240										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Solar radiation that has not been absorbed or scattered and reaches the ground directly from the sun is called as ______________.

2. 	What is Photovoltaic effect?

3. 	India is ranked _____________in the world for wind energy generation.

4. 	What is meant by yaw control?

5. 	Mention the classifications of biogas plants.

6.  	What is fermentation?

7. 	Give few utilizations of hydrogen gas.

8.  	List the main components of a fuel cell.

9. 	What is a spring tide?

10. 	Mention the name of the lowest level of tidal water.

PART – B  (5 x 3 = 15 MARKS)

11.  	Mention the different standards used to measure the Sun’s radiation.

12. 	What is meant by cut in speed in a wind mill?

13.  What are the factors affecting biomass generation?

14. 	List the various methods of production of hydrogen for use as an energy carrier.

15.  	What are the factors to be considered while selecting a site for a hydro power plant?

PART – C  (5 x 15 = 75 MARKS)

16.  	List the types of solar radiation. Explain any one type of solar radiation measuring instrument.

(OR)

17. 	Explain with a neat sketch the operation of a solar PV system and discuss its applications.

18. 	Discuss the wind energy conversion system with a suitable sketch.

(OR)

19. 	Briefly explain the factors influencing wind and site selection in wind energy system.

20.  a.	Discuss the factors affecting biodigestion in a biogas plant. 						  (10)

b.	Explain the process of photosynthesis. 											    (5)

(OR)

21. 	With a neat sketch describe the biogas power generation system.

22. 	Explain any one method to produce hydrogen gas with a neat sketch.

(OR)

23. 	Discuss with a neat diagram the working of a fuel cell.

24. 	With a neat sketch, explain the working of an open cycle ocean thermal energy conversion 	system.

(OR)

25. 	Draw the basic elements of a geo thermal energy plant and explain its working.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	RENEWABLE ENERGY SOURCES					      	     Time : 3 hours

Subject Code:	12EE240										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Solar radiation that has not been absorbed or scattered and reaches the ground directly from the sun is called as ______________.

2. 	What is Photovoltaic effect?

3. 	India is ranked _____________in the world for wind energy generation.

4. 	What is meant by yaw control?

5. 	Mention the classifications of biogas plants.

6.  	What is fermentation?

7. 	Give few utilizations of hydrogen gas.

8.  	List the main components of a fuel cell.

9. 	What is a spring tide?

10. 	Mention the name of the lowest level of tidal water.

PART – B  (5 x 3 = 15 MARKS)

11.  	Mention the different standards used to measure the Sun’s radiation.

12. 	What is meant by cut in speed in a wind mill?

13.  What are the factors affecting biomass generation?

14. 	List the various methods of production of hydrogen for use as an energy carrier.

15.  	What are the factors to be considered while selecting a site for a hydro power plant?

PART – C  (5 x 15 = 75 MARKS)

16.  	List the types of solar radiation. Explain any one type of solar radiation measuring instrument.

(OR)

17. 	Explain with a neat sketch the operation of a solar PV system and discuss its applications.

18. 	Discuss the wind energy conversion system with a suitable sketch.

(OR)

19. 	Briefly explain the factors influencing wind and site selection in wind energy system.

20.  a.	Discuss the factors affecting biodigestion in a biogas plant. 						  (10)

b.	Explain the process of photosynthesis. 											    (5)

(OR)

21. 	With a neat sketch describe the biogas power generation system.

22. 	Explain any one method to produce hydrogen gas with a neat sketch.

(OR)

23. 	Discuss with a neat diagram the working of a fuel cell.

24. 	With a neat sketch, explain the working of an open cycle ocean thermal energy conversion 	system.

(OR)

25. 	Draw the basic elements of a geo thermal energy plant and explain its working.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	BASICS OF MEDICAL ELECTRONICS					      Time : 3 hours

Subject Code:	12EE251							      		       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The voltage developed at an electrode is called as__________.

2.	Which one is called the power house of the cell?

3.	What is the purpose of electrode paste?

4.	Give the different types of Surface electrodes.

5.	Define scale range.

6.	Why is the calibration of an instrument important?

7.	The principle of operation of the Plethysmograph depends on_________.

8.	Graphical recording of the heart sounds is called__________.

9.	Define macro shock.

10.	Give the types of leakage current.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the parts of the central nervous system?

12.	List the different lead systems in an ECG measurement.

13.	Define CMRR and mention some of the active and passive transducers.

14.	Define cardiac output.

15.	What is gross current shock?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain different ways of transport of ions through the cell membrane.	 			(7)

b.	Explain the constituents of the Human Cell with a neat sketch. 					(8)

(OR)

17.	Draw the physiology of the heart and explain the circulatory system of human body.

18.	What is a Micro Electrode? Classify it. Explain it in detail with a neat diagram.

(OR)

19.	Draw the diagram for the recording setup of an EEG measurement and explain in detail the 10-20 electrode system.

20.	Discuss the different characteristics for selecting a transducer.

(OR)

21.	Explain in detail the different types of temperature transducers with a suitable diagram.

22.	What are the methods used for the measurement of cardiac output? Explain any one method in detail.

(OR)

23.	a.	Describe with a suitable diagram the working of a Heart-Lung Machine.			(9)

b.	Explain the block diagram of a single channel telemetry system.	                         	(6)

[P.T.O.]

24.	Write short notes on:

a.	Micro current shock.															(7)

b.	Leakage current.																(8)

(OR)

25.	Discuss in detail about the safety standards.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	BASICS OF MEDICAL ELECTRONICS					      Time : 3 hours

Subject Code:	12EE251							      		       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The voltage developed at an electrode is called as__________.

2.	Which one is called the power house of the cell?

3.	What is the purpose of electrode paste?

4.	Give the different types of Surface electrodes.

5.	Define scale range.

6.	Why is the calibration of an instrument important?

7.	The principle of operation of the Plethysmograph depends on_________.

8.	Graphical recording of the heart sounds is called__________.

9.	Define macro shock.

10.	Give the types of leakage current.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the parts of the central nervous system?

12.	List the different lead systems in an ECG measurement.

13.	Define CMRR and mention some of the active and passive transducers.

14.	Define cardiac output.

15.	What is gross current shock?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain different ways of transport of ions through the cell membrane.	 			(7)

b.	Explain the constituents of the Human Cell with a neat sketch. 					(8)

(OR)

17.	Draw the physiology of the heart and explain the circulatory system of human body.

18.	What is a Micro Electrode? Classify it. Explain it in detail with a neat diagram.

(OR)

19.	Draw the diagram for the recording setup of an EEG measurement and explain in detail the 10-20 electrode system.

20.	Discuss the different characteristics for selecting a transducer.

(OR)

21.	Explain in detail the different types of temperature transducers with a suitable diagram.

22.	What are the methods used for the measurement of cardiac output? Explain any one method in detail.

(OR)

23.	a.	Describe with a suitable diagram the working of a Heart-Lung Machine.			(9)

b.	Explain the block diagram of a single channel telemetry system.	                         	(6)

[P.T.O.]

24.	Write short notes on:

a.	Micro current shock.															(7)

b.	Leakage current.																(8)

(OR)

25.	Discuss in detail about the safety standards.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	BUILDING AUTOMATION							      	     Time : 3 hours

Subject Code:	12EE253						      			       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Differentiate transducers and actuators.

2.	What is the purpose of Earthing?

3.	What is reactive power? Give an example.

4.	List the ways to improve Power factor.

5.	What are the parameters considered for cable selection?

6.	List any two installation guidelines of cables.

7.	Define Biometrics.

8.	List the blocks in Digital video management system.

9.	Define an integrated system.

10.	List the issues in integrated systems.

PART – B  (5 x 3 = 15 MARKS)

11.	Discuss about level control in a building automation system.

12.	How do you practically estimate power quality? Indicate the instrument used.

13.	Write briefly about fire extinguishing principles.

14.	Describe the utility of Video door phone.

15.	What is the need for an Energy management system? List the benefits of an integrated system.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Write a brief note on Building Management system.								(7)

b.	Explain the meaning of analog and digital signals and describe the concept of controller 	IOs.																			(8)

(OR)

17.	Write a brief note on following sensors and transducers:

a. 	Temperature.																	(7)

b.	Pressure.																	(8)

18. 	a.	Explain in detail about Energy meter types and meter networking.					(7)

b.	Write a note on Energy conservation and the importance of energy saving.			(8)

(OR)

19. 	a.	Explain the effect of power quality on energy consumption. Enumerate its merits and 	demerits.																	(7)

b.	Discuss in detail about reactive power requirement and power quality issues.		(8)

20.	a.	Write a brief note on fire sensors and detectors.									(7)

b.	Briefly explain the fire development stages and installation guidelines of fire sensors and 	detectors.																	(8)

[P.T.O.]

(OR)

21.	a.	Describe fire alarm and pre-alarm systems.										(7)

b.	Describe the best installation practices for fire safety.								(8)

22.	Explain the concept of security systems briefly, with necessary sketches and terminologies.

(OR)

23.	Write a note on:

a. 	TCP/IP Networking.															(7)

b. 	Intrusion detection systems.													(8)

24.	Discuss in detail about:

a. 	Security system.																(7)

b. 	Future prospects of integrated systems.											(8)

(OR)

25.	Write short notes on:

a. 	Energy management system.													(7)

b. 	Video management.															(8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	BUILDING AUTOMATION							      	     Time : 3 hours

Subject Code:	12EE253										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Sensor.

2.	Why earthing plays an important role in building automation?

3.	What is instantaneous power?

4.	Define Power Factor.

5.	How manual fire alarm can be activated?

6.	Name the cables used in fire alarm system.

7.	What is anti-pass back concept?

8.	What are the components of typical analogue CCTV system?

9.	What is the necessity of integrating different systems in a building?

10.	List out the benefits of an integrated system.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the role of input and output unit in a controller?

12.	What is the relationship between voltage and current? Define Ohm’s law.

13.	Define the principle behind fire extinguisher. Give its classification.

14.	Briefly explain digital video management system.

15.	What is energy management system?

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Explain in detail about the different functions of a building management system.            (9)

b. 	Discuss about the benefits of a building automation system.                                             (6)

(OR)

17. 	With a neat diagram explain how pressure, flow and temperature are controlled in an automated building.

18. 	What are the effects of power quality on energy consumption? What is the difference between energy saving and energy conservation?

(OR)

19. 	Explain in detail any two types of energy meters. How energy parameters are monitored in an automated system.

20. 	Discuss in detail about the components and configurations of a fire alarm system.

(OR)

21. 	What are the types of cables used in fire alarm system? Also explain the installation guidelines and best installation practices of a fire alarm system.

22. 	Discuss about the different types of access control system employed for security purpose.

(OR)

23. 	With a neat sketch explain how analog CCTV system used for surveillance purpose.

[P.T.O]

24. 	What are the challenges faced by the integrated systems in building automation? What are the future prospects of integrated systems?

(OR)

25. 	Describe the components of integrated systems. What are the roles of safety and security systems in an integrated system?

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	HVDC AND FACTS									      	     Time : 3 hours

Subject Code:	12EE263								   		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The total installed HVDC capacity now exceeds ___________ MW.

2. 	HVDC Transmission is ___________ to HVAC Transmission.

3. 	Homopolar link is always operated with ___________ or ___________ return.

4. 	Mention the two applications of HVDC Transmission consideration.

5. 	Define Flexible AC Transmission System (FACTS).

6. 	Define Static Var Compensator (SVC).

7. 	Define Thyristor Controlled Switched Capacitor (TCSC).

8. 	Define Static Compensator (STATCOM).

9. 	Define Unified Power Flow Controller (UPFC).

10. 	HVDC and FACTS are ___________ Technologies.

PART – B  (5 x 3 = 15 MARKS)

11. 	Tabulate the detailed list of HVDC Back To Back and LINE projects in India.

12. 	Define and draw the three types of HVDC Link configurations.

13. 	List out the types of FACTS controllers.

14. 	Write down the control attributes of SVC, TCSC and UPFC.

15. 	Draw the conceptual representation of the UPFC in a two-machine power system.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the principle of operation of a typical HVDC converter station with a neat schematic diagram and twelve pulse converter unit diagram.

(OR)

17. 	Discuss in detail, the problems affecting reliable operation and major difficulties in the adoption of HVDC Technology by system planners.

18. 	Explain in detail, the relative merits of the two modes of transmission (HVAC and HVDC)  which need to be considered based on 	a.	Economics of Transmission	b.	Technical Performance 		c.	Reliability.

(OR)

19. 	Discuss the Modern Trends in HVDC Technology to reduce the cost of converter stations	 while improving the reliability and performance.

20.	Explain the basic concepts with relevant diagram and its associated waveforms of:

a.	Thyristor Controlled Reactor (TCR)	b.	Thyristor Switched Capacitor (TSC).

(OR)

21. 	Explain the basic concepts with relevant diagram and its associated waveforms of:

a.	Thyristor Switched Series Capacitor (TSSC).

b.	Thyristor Controlled Series Capacitor (TCSC).

[P.T.O.]

22. 	Explain the Voltage Control Characteristics of SVC connected at midpoint of a Transmission Line and determine the operating point of SVC.

(OR)

23. 	Explain the different modes of operation of TCSC and compute the TCSC Reactance.

24. 	Explain the principle of operation of STATCOM with a circuit diagram and its control characteristics.

(OR)

25. 	Explain the principle of operation of Unified Power Flow Controller (UPFC) with a schematic diagram and analyze its equivalent circuit parameters.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	POWER SEMICONDUCTOR DEVICES					Time : 3 hours

Subject Code: 	12EE301								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a. 	Explain the series and parallel operation of a power diode.								(10)

b. 	Discuss the need of a Heat sink for a Power Semiconductor device.						(10)

(OR)

2.	With a neat diagram explain the turn-on and turn-off characteristics of a power diode.

3.	Explain with a diagram the operation of a Thyrisor along with the two transistor analogy.

(OR)

4.	Discuss about the various gate driver circuits for the thyristor.

5.	a. 	Explain about the switching and steady state characteristics of TRIAC.					(8)

b. 	Describe the TRIAC Switching and gate trigger circuit. 								(12)   (OR)

6.	Explain the working of the gate driver circuit of GTO.

7.	Discuss about Base Drive design and Power Darlington.

(OR)

8.	Discuss about the MOSFET Gate Drive Circuitry.

9.	Compulsory:

Explain the structure and principle of working of IGBT.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	POWER CONVERTER ANALYSIS – I					Time : 3 hours

Subject Code: 	12EE302									       	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a. 	Describe the working of a three phase fully controlled bridge converter with RL load in

the rectifying mode and inversion mode. Also derive the expressions for average output

voltage and rms output voltage.											      (12)

b. 	Briefly explain the performance parameters of three phase fully controlled bridge 	converter.																	(8)

2.	a. 	With a relevant circuit diagram and waveforms, explain in detail the operation of a single 	phase half controlled thyristor bridge converter feeding RLE load for discontinuous 	current conduction mode.			                                                            		     (10)

b. 	With necessary waveforms, explain the operation of single phase dual converter.	     (10)

(OR)

3.	A single phase full converter is connected to a 120V, 60Hz supply. The load current Ia is continuous and its ripple content is negligible. The turns ratio of the transformer is unity.

a.	Express the input current in a Fourier series; determine the Harmonic Factor of the input 	current, Displacement Factor, and input Power Factor.						      (10)

b.	If the delay angle is   , calculate average output voltage, normalized average 	output voltage, rms output voltage Harmonic Factor, Displacement Factor, and input 	Power Factor. 															      (10)

4.	a. 	Explain the operation of the step down chopper with associated output voltage and 			current waveforms for continuous current mode with RL load.					      (10)

b. 	The buck regulator has an input voltage of . The required average output     		voltage is  at R = 500 Ω and the peak-to-peak output ripple voltage is 20 mV.     		The switching frequency is 25 kHz. If the peak-to-peak ripple current of inductor is     		limited to 0.8A, determine:	a.	the duty cycle k		b.	the filter inductance L

c.	the filter capacitor C		d.	the critical values of L and C.			              (10)

(OR)

5.	a. 	Draw and explain the circuit diagram of buck-boost regulator and also derive the 			expression for the average output voltage, peak to peak ripple current, critical value of

the inductor and the capacitor.												      (10)

b. 	Briefly explain the four-quadrant operation of DC converter.					      (10)

6.	a. 	With relevant circuit diagram and waveforms, explain the operation of a single phase full 	wave voltage controller with Inductive loads.								      (10)

b. 	The single phase full-wave voltage controller supplies an RL load. The input rms voltage 	is , 60 Hz. The load is such that L = 6.5 mH and R = 2.5Ω. The delay angles 	of 	thyristors are equal: . Determine:

a.	the conduction angle of thyristor, 	b.	the rms output voltage		c.	the rms 	thyristor current		d.	the 	rms output current	e.	the 	average current of a 	thyristor		f.	the input Power factor.					      				     (10)

(OR)

[P.T.O.]

7.	With relevant circuit diagram and waveforms, explain the operation of a three phase                 full wave controller with resistive load. Derive the expression for the rms output phase voltage in the range of  0o to 60o delay angle.

8.	a. 	Describe the basic principle of a single phase to single phase cyclo-converter for both 	continuous and discontinuous conductions for a bridge type cyclo-converter.	      (10)

b. 	The input voltage to the cyclo converter is 120V rms. 60 Hz. The load resistance is 5 Ω

and the load inductance is L = 40mH. The frequency of the output is 20Hz. If the converters are operated as semi-converter such that 0 ≤ α ≤ π,  and the delay  angle is

αp = 2π/3, determine:														      (10)

i.	the rms value of output  voltage Vo

ii.	the rms current of each thyristor IR

iii.	the input power factor.

(OR)

9.	a. 	With relevant circuit diagram and waveforms, explain the operation of 3-phase to           	1-phase half wave cyclo converter.											      (12)

b. 	Explain about load commutated cyclo converter.									(8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	POWER CONVERTER ANALYSIS – II					Time : 3 hours

Subject Code: 	12EE303								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

With a neat circuit diagram, waveforms and equations, explain the operation of transistorized three - phase bridge inverter with resistive load in 180 degree conduction mode.

2.	a.	Explain the principle of single phase half - bridge inverter with neat diagrams. 			(12)

b.	A single phase half-bridge inverter has R = 2.4 and Vs = 48V. Determine:

i. rms output voltage						ii.	output power

iii.	average and peak currents of transistor 	iv.	THD.							(8)

(OR)

3.	Briefly explain any three methods for the voltage control of single phase inverters.

4.	a.	Derive the average input voltage equation of a single phase capacitor - commutated current 	source inverter for R load, with neat circuit diagram and waveforms. 					(15)

b.	Compare voltage source inverter and current source inverter.							(5)

(OR)

5.	a.	A single phase auto-sequential commutated CSI is fed from 220V DC source. The load is          	R = 10Ω. Thyristors have turn-off time of 20 µs and inverter output frequency is 50 Hz. Take a 	factor of safety of 2. Determine a suitable value of source inductance assuming a maximum 	current change of 0.5A in one cycle. Neglect all losses. Find also the values of commutating 	capacitors. 	 																(5)

b.	Discuss the various modes of operation of a single phase auto – sequential commutated inverter 		with required diagrams and waveforms. 											(15)

6.	a.	What is a multilevel inverter? List out its types.	 									(5)

b.	Explain the operation, features and advantages of diode-clamped inverter with neat diagrams. 																					(15)

(OR)

7.	a.	Describe the operation of a single phase multilevel cascaded H-bridge inverter with the required 		diagram and waveforms.   														(10)

b.	Describe the advantages and disadvantages of flying – capacitor inverter. 				(5)

c.    	Compare the component requirements per leg of three types of multilevel inverter.	     		(5)

8.	Explain the operation of series resonant inverters with unidirectional switches with the required equations, diagram and waveforms.

(OR)

9.	a.	Discuss the detailed working of resonant dc – link inverter.							(10)

b.	Describe various modes of operation of Class E resonant inverter. 						(10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	SOLID STATE DC DRIVES								Time : 3 hours

Subject Code: 	12EE304					                 			 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Explain the different duty classes of a motor.									(10)

b.	Discuss the Ward-Leonard method of speed control of a DC motor.				(10)

2.	a.	Discuss with a neat diagram, the operation of a single phase half controlled rectifier fed 	DC 	series motor. 														(10)

b.	A 220W, 960 rpm, 12.8A separately excited DC motor has armature circuit resistance   	and inductance of 2Ω and 150mH respectively. It is fed from a single phase half 	controlled rectifier with an AC voltage source of 230V, 50Hz. Calculate:

i.	motor torque for α = 60°, N = 600 rpm.

ii.	motor speed for  α = 60°, T = 20 N-m.									(10)

(OR)

3.	a.	Explain with a neat diagram, the operation of a single phase fully controlled converter 	fed 	DC separately excited motor in discontinuous conduction mode.			(10)

b.	A 200V, 875 rpm, 150A separately excited motor has an armature resistance of 0.06 Ω. 		It is fed from a single phase fully controlled rectifier with an AC voltage source of 220V, 		50Hz. Assuming continuous, calculate:

i.	Firing angle for rated torque, N=750 rpm.

ii.	Motor speed for α = 160° at rated torque.					 			 (10)

4.	a.	Explain the regenerative and dynamic braking of separately excited DC motor by 	chopper control with neat diagrams.										(10)

b.	A 230V, 960 rpm, 200A separately excited motor has armature resistance of 0.02 Ω. The		motor is fed from a chopper for both motoring and braking modes. The source voltage is 		230V. Assuming continuous conduction,

i.	Calculate duty ratio for motoring mode at rated torque, N= 350 rpm.

ii.	Calculate duty ratio for braking mode at rated torque, N= 350 rpm.			(10)

(OR)

5.	a.	Describe the chopper controlled DC series motor in various modes with a neat diagram.

(10)

b.	Explain the motoring control of separately excited motor using chopper with neat 	diagrams and also, brief about time ratio control of a chopper.					(10)

6.	a.	Explain the closed loop control of current, speed and torque of drives.			(12)

b.	Discuss about phase locked loop (PLL) control of speed in drives.				(8)

(OR)

7.	Derive the transfer functions of armature controlled and field controlled DC motor.

8.	Draw and explain the block diagram of a separately excited DC motor drive using a 	microprocessor.

(OR)

9.	Explain the simulation of single quadrant converter fed DC drive with a flowchart.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	GENERALIZED THEORY OF ELECTRICAL MACHINES																					Time : 3 hours

Subject Code: 	12EE306									        Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Explain in detail about the steady state operation of Synchronous Machine.

2. 	a. 	Write the Voltage equations for Kron’s primitive machine in matrix form. What 	observations are made from the impedance matrix of this machine?			      	      (10)

b. 	Obtain an expression for the electrical torque of the Kron’s Primitive machine. Show that 	no torque is produced by interaction between the flux and current on the same axis.   (10)

(OR)

3. 	a. 	A coil moves in a time varying magnetic field whose spatial distribution along the air-	gap Periphery is sinusoidal. Find an expression for voltage induced in the coil identifying 	the terms therein.															      (12)

b. 	Explain the conventions adopted in the development of the generalized machine theory.																				(8)

4. 	a. 	A three phase Induction motor has the following per phase parameters referred to stator:

Stator resistance					- 0.30 

Rotor resistance 					- 0.45 

Stator and Rotor leakage reactance 	- 2.1  each

Magnetizing reactance			- 30.00 

Find out the parameters of an equivalent 2-phase Induction motor if its per phase turns 	are:

i.	Same as that of the 3-phase Induction motor.

ii.	3/2 times that of the 3-phase Induction motor.

iii.	√3/2 times of the 3-phase Induction motor.								      (12)

b. 	Explain the term ‘invariance of power’ as applied to electrical machines.			(8)

(OR)

5. 	Derive the transformations for currents between a rotating balanced 2-phase (α, β) winding and a pseudo-stationary two-phase (d, q) winding. Assume equal turns on all coils. Show that the transpose of current transformation matrix is equal to its inverse.

6. 	a. 	A Separately-excited d. c. generator is connected to RL. The generator is driven at 	constant speed, but the voltage source connected to field is such that the expression for 	field current  can be written as,

if=Ifo+Ifsinωt.

Obtain expressions for 	i.	load current 		ii.	torque 	iii.	power connected from mechanical to electrical. Neglect armature inductance.					      (12)

b. 	Explain the concept of Regenerative braking of d. c. motor in detail.				(8)

(OR)

7. 	Write the voltage and torque equation of the separately excited d. c. motor and explain in detail about its steady state and transient condition.

8. 	Derive a three phase induction machine stator and rotor voltage, flux linkages, torque equations in arbitrary qdo reference frame.

(OR)

9.	Explain the transient model of three phase induction machine in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	ADVANCED DIGITAL SIGNAL PROCESSING			Time : 3 hours

Subject Code: 	12EE308										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	The input to the system y(n) = 0.999 y(n – 1) + x(n) is quantized to b = 8bits. What is the 	power produced by the quantization noise at the output of the filter?                   		(4)

b.	The first order filter given by the difference equation y(n) = α y(n1) + x(n) is 	implemented in four bit (including sign) fixed-point two’s complement fractional 	arithmetic. Products 	are rounded to four bit representation. Using the input x(n) = 	0.1δ(n), determine:

i.	The first five outputs if α = 0.5. Does the filter go into a limit cycle?      			(8)

ii. 	The first five outputs if α = 0.75. Does the filter go into a limit cycle?  			(8)

2.	a.	Determine the convolution y(n) of the signals				      				      (10)

x(n)=13≤n≤60elsewhere

h(n)=1−4≤n≤−30elsewhere

b.	State and prove the properties of linear time invariant system with example.	      	      (10)

(OR)

3.	a.	Consider the signal x(n) given by sum of the causal and anticausal sequence, that is

x(n)=anu(n)+bnu(−n−1)

Determine the z-transform. Does the z-transform exist if :

i.	|b| < |a| 		ii.	|a| < |b| 		      										      (15)

b.	Obtain direct form I and direct form II for the system described by

y(n)=−0.1y(n−1)+0.72y(n−2)+0.7x(n)−0.252x(n−2)	 					(5)

4.	a.	Define and prove Parseval’s Theorem.				                   				      (10)

b.	A finite duration sequence of x(n) of length 8 has 8-point DFT X(k) as shown in the 			figure.

A new sequence y(n) of length 16 is defined by

y(n)=xn2;n even0;n odd

Find the 16 point DFT of y(n).					                  					      (10)

[P.T.O.]

(OR)

5.	a.	Compute the DFT for N = 8 if x(n)= {1, 2, 2, 3, 5, 5, 3, 1} using Decimation in time 			algorithm. 					      											      (15)

b.	State and prove any five properties of DFT.	   			        				       (5)

6.	a.	Convert the analog filter with system function

Ha(s)=s+2s+1 s+3

into the digital IIR filter by means of the impulse invariant method.                	      (10)

b.	Write short notes on:

i.	Bilinear transform.				      					      				      (10)

ii.	Warping effect.

iii.	Matched Z-Transform.

(OR)

7.	a.	The desired frequency response of a low pass filter is given by

Hdω=e−j2ωω≤π20π4≤ω≤π

Determine the filter coefficients hd(n)  if

h(n)=hd(n)ω(n)	 where

ω(n)=10≤n≤40otherwise

Then determine the response H(ω).					      						      (15)

b.	Mention the different types of FIR filter windows			        				(5)

8.	With a neat diagram explain the architecture of TMS320C Processor with its key features.

(OR)

9.	a.	Explain the different types of addressing modes for TMS320 series with suitable 	examples.  																      (10)

b.	Explain the following terms.												      (10)

i.	Barrel Shifter.

ii.	CSSU.

iii.	MAC.

iv.	Harvard Architecture.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	PHOTOVOLTAIC SYSTEMS							Time : 3 hours

Subject Code: 	12EE313								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1. Compulsory:

a. 	Briefly write about the geometric and atmospheric effects of sunlight with suitable 			sketches.                                                                                                          		      (15)

b. 	Define Irradiance and mention the irradiation level for Standard testing condition with 		proper unit. 																	(5)

2.  	a. 	Draw the equivalent circuit of an ideal solar cell and derive its Fill Factor.                (15)

b. 	Derive the PV Module current equation by considering the series resistance of a solar

cell. 																		(5)

(OR)

3.	a. 	Explain the shadow effect and the usage of by-pass diodes in the solar cell with a neat 	diagram and graph.														     (10)

b. 	Write short notes about the hot pot problem and the safe operating area.                    (10)

4. 	a. 	With a neat sketch explain the operation of pumped energy storage.                           (10)

b. 	Draw and explain the different characteristics of lead acid batteries.                           (10)

(OR)

5. 	a. 	Discuss about the compressed energy storage system with neat sketches.                     (8)

b. 	Define memory effect in batteries.                                                                                  (4)

c. 	Explain the modelling of batteries with proper derivation.                                             (8)

6. 	a. 	Analyse the inverter operation at fundamental frequency.                                             (10)

b. 	Draw and explain the working of residential PV system.                                              (10)

(OR)

7. 	a. 	Mention the different inverter control topologies for standalone PV system and briefly 	write about each methodology. 											      (15)

b. 	Draw and explain the PV water pumping system.                                                          (5)

8. 	a. 	What is the need of MPPT?                                                                                             (5)

b. 	Explain any three MPPT algorithms with suitable flowchart and graph.                      (15)

(OR)

9. 	a. 	Write about the modelling and simulation of standalone PV system with diagrams.   (15)

b. 	Explain the active power filtering process in PV systems.                                              (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	POWER ELECTRONIC CIRCUITS						Time : 3 hours

Subject Code: 	12EE314										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Illustrate the principle of phase control in single phase half wave AC voltage controller with the corresponding power circuit diagram and its output voltage and output current waveforms. Also derive an equation for its average output voltage and RMS output voltage

2.	a.	Explain the modes of operation of a thyristor in detail. Also sketch its VI Characteristic.

(10)

b.	Draw the switching curves of thyristor and explain various times involved with it      (10)

(OR)

3.	a.	Explain in detail the operation and switching characteristics of an IGBT.                    (12)

b.	What is the difference between β and βf  for BJTs?                                                    	(3)

c.	For a power diode the reverse Recovery time is 3.9µs and the rate of diode current decay is 50A/µs. For a softness factor of 0.3 calculate the peak inverse current and the storage charge.								                   								(5)

4.	a.	Derive the Performance parameters of a single phase full converter in detail.              (12)

i.	DF		ii.	CDF		iii.	VRF		iv.	PF

v.	THD	vi.	Pi			vii.	Qi

b.	Discuss the single phase symmetrical semiconverter with RL load including suitable waveforms and circuit diagrams. Also derive its Average output voltage and RMS output voltage.	      									      								(8)

(OR)

5.	a.	For a 3-phase full converter, explain how an output voltage wave for a firing angle of 30˚ 		is obtained by using 	i.	Phase Voltages	ii. 	Line Voltages                                  (12)

b.	Describe the working of single-phase fully controlled bridge converter in the rectifying 	mode. Also sketch the following waveforms for α=30°.

i. Load voltage 						ii.	Load current 	waveforms

iii.	 Thyristor voltage waveforms 		iv.	Supply voltage and current waveforms.	(8)

6.	a.	With the help of voltage and current waveforms, explain the working of first quadrant 	chopper. Give the complete Time domain analysis of class A chopper.  		     	      (15)

b.	Derive an expression for average current in the freewheeling diode for a continuous load current and  prove that average value of load current Ioav =		       			(5)

(OR)

7.	a.	A single Quadrant type A chopper is operated with the following specifications:

Ideal battery of 220V; On-time ton= 1ms; Off-time toff= 1.5ms.

Determine the following:

i.	RMS output voltage				ii.	Average output Voltage

iii.	Ripple Factor					iv.	Form Factor							(8)

b.	Explain the working of a four quadrant chopper in detail.		                                   (12)

8.	Discuss in detail the operation of a 3-phase six step 180˚ mode voltage source inverter with a balanced star connected load, with its equivalent circuits.

(OR)

[P.T.O.]

9.	a.	List out and discuss the various methods for the control of output voltage of inverters. 											     									      (10)

b.	Discuss in detail the single pulse modulation technique, with suitable diagram and 	corresponding equations.								     					      (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	SOLAR THERMAL ENERGY CONVERSION			Time : 3 hours

Subject Code: 	12EE315									       	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Explain the preparation of anti-reflective coating and its characterization.        	(10)

b.	Describe the reflection from real surface and obtain expression for reflectance. 	(10)

2.	a.	Obtain expression for energy balance equation for flat plate collector.         		(10)

b.	With a neat sketch explain working of a flat plate collector.                             	(10)

(OR)

3.	a.	Explain the thermal analysis of Liquid flat- plate collector and useful heat gained

by the fluid.                                                                                                    		(10)

b.	With a diagram describe working of Air flat plate collector.	              			(10)

4.  	a.	Explain Storage method of storing solar thermal energy. 		                 		(10)

b.	Describe the heat transport system effect in design of flat- plate collector solar energy

storage system.		                                                                           			(10)

(OR)

5.	a.	With a block diagram describe thermo chemical solar storage system.			(10)

b.	Explain the effect of selective surface and spacing in the design of solar storage

system.                                                                                           				(10)

6.  	a.	Explain the working of parabolic concentrating solar collector.                      	(5)

b.	Describe the performance analysis of cylindrical parabolic concentrating collector

and obtain the expression for collector efficiency.                                          		(15)

(OR)

7.	a.	Explain construction and working of a heliostat array and central receiver system.	 																				(10)

b.	Describe with a layout the working of a solar pond power plant.             			(10)

8.   	a.	With a schematic diagram explain the working of a neutral circulation solar water heater. 	                                                                                                   					(10)

b.	Describe the working of solar hot air heating system.	                     				(10)

(OR)

9.	With a diagram explain the working of a solar- operated absorption air conditioner.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	ADVANCED CONTROL TECHNIQUES FOR INDUCTION 						GENERATORS											Time : 3 hours

Subject Code: 	12EE316										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1. 	a.	A four pole induction generator is rated at 300 kVA and 480 V.  It has the following 	parameters: Xls = Xlr = 0.15 , Rls = 0.014 , Rlr = 0.0136 , Xm = 5 .  How much 	power does it produce at a slip of 0.025? How fast is it turning at that time? Also, find 	the torque, power factor and efficiency (Ignore mechanical losses. Assume f = 60 Hz).																			      (10)

b.	Obtain the expression for generated power and induced torque in an induction generator.																				      (10)

(OR)

2. 	Write down the state equations of SEIG with RLC load and partition the SEIG state matrix to separate the machine parameters from load and self-excitation parameters.

3. 	What are the parameters on which the performance of a self-excited induction generator depends on?  Derive the mathematical description of the self-excitation process and find the expression for efficiency of the SEIG.

(OR)

4. 	A 75 kW, three-phase, 400 V, 50 Hz, four pole wound rotor induction motor, with star-connected stator windings, runs at a slip of 3.5% under rated shaft torque. It has the following per phase equivalent circuit parameters referred to the stator: Rs = 0.075 , Rr = 0.06 ,

Xls = 0.15, Xlr = 0.12 , Xm = 6.1 .  A constant rotational loss of 1.7 kW may be assumed for operation at speeds close to the synchronous speed.  The induction machine is now driven at a speed of 1530 rpm (2% slip) with the stator terminals remaining connected to the 400 V supply.  Find the power fed into the supply for the following conditions:

a.	Rotor terminals are short-circuited.

b.	1.15 V per phase (referred to the stator) injected into the rotor, and in phase with the supply.

5.  	Compulsory:

Highlight the merits and demerits of scalar control of induction generators. Discuss with a neat block diagram the following scalar control schemes:

a.	Voltage-fed link system for grid connection with current control for generator side. (10)	b.	Static VAR compensator scalar-based control with voltage regulation.			      (10)

6. 	With neat block diagrams and expressions, discuss the direct vector control based induction generator control for grid connected and stand alone systems.

(OR)

7. 	In the per-phase equivalent circuit model of a star-connected 415-V, 50 Hz, six-pole, squirrel cage induction motor, the impedances are as follows: Rs = 0.085, Rr = 0.07, Xm = 7 , Xls = 0.18 , Xlr = 0.18 .  The machine runs at a slip of 0.025, supplying power to a utility network at 415 V, 50 Hz.  Find the values of the stator and the rotor currents, the electromagnetic torque, and the power input using a synchronously rotating stator voltage vector oriented d-q axis model.  Also, draw the space-phasor diagram showing the relative positions of the voltage, current and flux linkage vectors of the stator and the rotor.

[P.T.O.]

8.  	Discuss with neat block diagrams and flowchart the application of hill climbing control for 	induction generators.

(OR)

9.  	The optimum fuzzy logic operating control of an induction generator requires three different fuzzy logic controllers. Discuss in brief the block diagram, rule table and membership functions of the three controllers.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	ENERGY ENGINEERING								Time : 3 hours

Subject Code: 	12EE317								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	What is Energy? Explain any two forms of Energy in detail. 					      (10)

b.	What are fossil fuels? Explain with examples.  								      (10)

2.	a.    	Discuss about Coal and Oil Energy resources.								      (16)

b.	Explain Energy Consumption in Urban and Rural areas.							(4)

(OR)

3.	a. 	Discuss about Hydroelectricity Potential.									      (12)

b.	Describe renewable Energy resources.											(8)

4.	With a neat schematic diagram explain about Oceanic Energy system.

(OR)

5.	a.	Discuss about Biomass Resources.											      (12)

b.	Explain about scope of Fuel Cell and its energy conversion.						(8)

6.	a.	Discuss about Kyoto Protocol. 												      (10)

b.	Give some example of primary and secondary air pollutants and explain.		      (10)

(OR)

7.	a.	Explain the consequence of ozone depletion. 								      (10)

b.	What is global warming? Explain its effect.									      (10)

8.	Discuss about various components of a Smart grid in detail.

(OR)

9.	a.	Explain about the wireless sensor network. Give its application.					(8)

b.	Discuss about Deregulation.												      (12)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	HYDROGEN AND FUEL CELLS						Time : 3 hours

Subject Code: 	12EE319								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Name the different technologies for hydrogen production.			      				(2)	b.	Mention the various applications of each of these technologies.		      			(2)

c.	Explain at least four hydrogen production techniques in detail.			    		     (16)

2.	What are the various challenges that will be involved in hydrogen storage process? Explain each one of them with respect to each of the storage options that is available.

(OR)

3.	Compare compressed hydrogen gas with liquid hydrogen in terms of production, storage, application and safety concerns.

4.	a.	Derive an equation for theoretical fuel cell potential.			    	       				(8)

b.	Show, with appropriate derivations, how the voltage losses in the cell occurs through 	activation polarization and ohmic losses.						      			      (12)

(OR)

5.	a.	Compare the technological difference between a battery powered vehicle and a fuel cell 	vehicle. 										    							      (10)

b.	Derive Butler-Volmer equation for both anode and cathode material of a fuel cell.    (10)

6.	Compare the working principle, applications and limitations of solid oxide fuel cells with PEM fuel cells.

(OR)

7.	a.	State the type of electrolyte used in each of the different types of fuel cells. 	       		(4)

b.	Write down the chemical reaction that happens at the electrode/electrolyte interface for 	different types of fuel cells	.					       								(6)

c.	Demonstrate the working principle of DMFC.					     			      (10)

8.	Explain the various technical problems that you may face while you stack up your PEM fuel cell for large scale application.

(OR)

9.	Explain the various concerns, accessories required, applications and limitations of fuel cell application in automobile and space applications.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	ENERGY MODELLING, ECONOMICS AND PROJECT MANAGEMENT									      Time : 3 hours

Subject Code: 	12EE322										Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.   	Compulsory:

Describe the Framework for Energy Demand Forecast Models with the aid of a schematic diagram.

2.	Explain the architecture towards the implementation of various investment projects by public and private stakeholders or as a part of public- private partnership.

(OR)

3.	Describe the economics of energy pertaining to the following:

a.	Properties of Energy Resources and Energy Commodities.

b.	Energy Conversion Processes.

c.	Demand for Energy as a Derived Demand.

d.	Demand Substitution among Energy Commodities.

4.	Discuss energy aggregation as well as the approaches for governmental aggregation.

(OR)

5.	Explain the following energy demand models:

a.	Econometric Models.

b.	Engineering Process Models.

c.	Engineering Economic Models.

6.	Discuss the various energy conservation measures for a typical energy-conserving building highlighting the investment aspects and the potential energy savings.

(OR)

7.	Explain in detail on how to assess the potential for waste heat recovery with a typical example.

8.	Propose a scaling up renewable energy program and discuss the linkages between priority areas of renewable energy sector and the strategic axes of the proposed program.

(OR)

9.	a.	Describe the features, objectives and limitations of budgetary control. 			      (10)

b.	Explain budgets based on time-period, conditions, capacity and coverage and explain.																			      (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	SIMULATION OF POWER ELECTRONICS SYSTEMS																						Time : 3 hours

Subject Code: 	12EE325										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	What is the need for simulation? What are the challenges in simulation?              	(10)

b.	Derive the state equation for the linear system shown below in matrix form.         (10)

2. 	a.	Explain Trigonometric functions, Logical relations, Exponential, operators in MATLAB 	with  appropriate examples. 	                                                                            		(8)

b.	Discuss For loop – While – If else options in MATLAB.						      (12) (OR)

3. 	a.	Write a MATLAB program to simulate a Half wave controlled rectifier with R and RL 		load.   																	     (10)

b.	Explain the graphic functions used in MATLAB.								     (10)

4. 	a.	i.	Mention the procedure for building systems using Simulink. 					(5)

ii.	Build in a dynamic system to convert Celsius to Fahrenheit with Simulink blocks.

(5)

b.	Explain in detail about the sim power electronics library for a diode, thyristors, universal

bridge and MOSFET diode switch.											      (10)

(OR)

5. 	a.	Explain the simulation phases in SIMULINK. 									(8)    b.	Discuss about the Sources and Sinks model analysis using SIMULINK.	            	     (12)

6. 	a.	i.	Draw the circuit structure of PSIM.                                                                		(2)

ii.	Illustrate with an example, how to control  3 phase thyristor and voltage source 				inverter modules.															(8)

b.	What is the ‘Power modelling block and Thermal module’ in Psim? Mention the 			parameters related to this block and explain the usage of this block. 			      (10)

(OR)

7. 	a.	What is Leakage flux path? Mention its uses.		                                            	(8)

b.	Give details about the Switch mode and linear type in Psim. 		                            (12)

[P.T.O.]

8. 	a.	Mention the small-signal and the static models that are generated by Pspice for a 			reverse biased diode and explain in detail.  									      (10)	b.	Write brief notes on dot commands of Pspice.                                                        	      (10)

(OR)

9. 	a.	Draw the Pspice models of BJT and MOSFET Choppers and explain in detail.	      (10)    	b.	Explain about the various types of analysis that are allowed in Pspice. 			      (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December – 2013

Subject Title: 	POWER ELECTRONICS IN WIND AND SOLAR POWER CONVERSION											Time : 3 hours

Subject Code: 	12EE328										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Discuss the classifications of energy resources.								(10)

b.	Briefly explain the industrial base created in India in the various renewable energy 	techniques.																(10)

2.	Explain the various issues and constraints in PV power conversion.

(OR)

3.	a.	Briefly discuss the Maximum Power Point Tracking algorithms. 				(10)

b.	Explain the measurements available for solar radiation. 						(10)

4.	Discuss the basic components and classifications of wind energy conversion system.

(OR)

5.	a.	Explain the potential of wind energy system in India. 							(10)

b.	Evaluate the selection of a wind generator. 									(10)

6.	Discuss the issues related to grid interconnection of wind farms.

(OR)

7.	Explain the various power conditioning schemes.

8.	Describe the wind and solar interconnected grid system and discuss its problems.

(OR)

9.	Briefly explain 	a.	The essentiality of optimization in a power system. 			(10)

b.	Storage of power. 										(10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	ELECTRIC AND HYBRID VEHICLES					Time : 3 hours

Subject Code: 	12EE331								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Explain the concept of Hybrid Electric drive train and discuss about the series and parallel hybrid electric drive train design.

2.	Explain	a.	The traction motor characteristics and tractive effort. 					      (10)

b.	Electronic control system for electric vehicles.						      (10)

(OR)

3.	Write shorts notes on:

a.	Energy consumption in Electric Vehicles.									      (10)

b.	Transmission requirements and Vehicle performance.							      (10)

4.	a.	Explain in detail about photovoltaic cells and tracking.						      (10)

b.	Write a note on solar cars.													      (10)

(OR)

5.	a.	Explain in detail about fuel cells, their construction and working with required equations.

(10)	b.	Describe the design of a fuel cell and make a cost comparison with solar cell.	      (10)

6.	Draw and explain the characteristics, regenerative braking, control system principles and speed-torque characteristics of AC motors used in electric propulsion system of hybrid electric vehicles.

(OR)

7.	Draw and explain the characteristics, regenerative braking, control system principles and speed-torque characteristics of DC motors used in electric propulsion system of hybrid electric vehicles.

8.	a.	Write short notes onLithium based batteries with the electro chemical reactors.	      (10)

b.	Define Thermodynamic voltage, specific energy, specific power and energy efficiency.

(10)

(OR)

9.	Write notes on:

a.	Ultra capacitors.															      (10)

b.	Voltage and frequency regulators of AC generators.							      (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	FLEXIBLE AC TRANSMISSION SYSTEMS				Time : 3 hours

Subject Code: 	12EE335									        Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a.	Compare SVC and STATCOM.												(5)

b.	What is Bang-Bang control? 													(2)

c.	Explain with a neat sketch, the principle of operation of thyristors controlled reactor and 	static condenser.															      (13)

(OR)

2.	With a neat sketch explain the principle of operation of TCSC, different modes of operation and list the advantages.

3.	Compulsory:

Explain the following controls of TCSC in its closed loop operation with their block diagram.

a.	Constant current control.

b.	Constant angle control.

c.	Enhanced current control.

d.	Constant power control.

4.	Explain the principle of operation of UPFC and also explain how the power transfer capability can be improved with the help of UPFC.

(OR)

5.	With a neat sketch, explain the concept of implementation of UPFC.

6.	What is meant by variable structure Series Capacitor Control? Discuss its applications.

(OR)

7.	Briefly explain the multimodal decomposition concepts in FACTS application.

8.	a.	Distinguish clearly between UPFC and IPFC.									(5)

b.	Give the block diagram for generalized IPFC and explain. 				   	      (15)

(OR)

9.	a.	Describe the procedure to locate the FACTS device in an electrical network.		      (12)

b.	Explain the basic concept of C-UPFC. 											(8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	GREEN BUILDING										Time : 3 hours

Subject Code: 	12EE339								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Explain in detail about the benefits and economics of green buildings.

2.	a.	Write in detail about the principles and integrated approach of green buildings in 	building design 											  				      (15)

b.	Mention the salient features of a green building.					     				(5)

(OR)

3.	What are the five basic principles of ecological design? Explain the principles in detail.

4.	a.	Discuss in detail the recommendations and practices for a sustainable landscape design. 																				      (15)

b.	What is meant by Grey water system for irrigation?					   			(5)

(OR)

5.	Write in detail about the basic steps for reducing water consumption and water conservation in a building.

6.	a.	Write about the recommended check list for site selection.				     		      (10)

b.	Explain how site inventory characteristics affect the site and building design 	elements. 																      (10)

(OR)

7.	a.	What are the limitations and features of natural ventilation?				     	      (10)

b.	Discuss in detail about the low energy strategies for a mechanical ventilation system.          																			      (10)

8.	a.	Mention the services offered by IGBC.						      				      (10)

b.	Explain the process to achieve LEED India rating for a building.			             (10)

(OR)

9.	a.	What is LEED?									        						(5)

b.	Write in detail about the LEED criteria that are being assessed for certification. Mention 	the point based rating system followed by LEED.					      	      		      (15)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	DATA MINING FOR RENEWABLE ENERGY TECHNOLOGY																				Time : 3 hours

Subject Code: 	12EE340										  Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	With a block diagram describe the frame work of a data warehouse with the example of

a multidimensional data cube.	                                                                                      (10)

b.	Explain data cleaning with missing values, noisy data and data cleaning as a process.(10)

2.	a.	Describe three-tier data warehouse architecture.							     	      (10)

b.	Explain how to index OLAP data by join indexing.							      (10)

(OR)

3.	a.	Describe an example of multiway array aggregation for full cube computation.	      (10)

b.	Explain mining multilevel association rules with the example of hierarchy for all 	electronic computer items.								                  			      (10)

4.	a.	Discuss issues regarding preprocessing the data for classification and prediction.	      (10)

b.	With an example describe accuracy and error measures.						      (10)

(OR)

5.	a.	Describe Bayesian belief network with an example.							      (10)

b.	Explain with an example about Lazy learners.								      (10)

6	a.	Explain various types of data that often occur in cluster analysis.				      (10)

b.	With an example describe mining time-series data trend analysis.				      (10)

(OR)

7.	a.	Explain partitioning centroid based technique by K-means method.			             (10)

b.	With an example explain the concepts and primitives of sequential pattern mining.    (10)

8.	a.	Discuss the application of data mining in a wind power system.				      (10)

b.	Describe how data mining is used in the analysis of solar power plant performance.  (10)

(OR)

9.	a.	How can one apply data mining for wind power prediction?					      (10)

b.	Explain how to perform the forecasting of solar radiation data.					      (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	SOFT COMPUTING TECHNIQUES						Time : 3 hours

Subject Code: 	12EE341								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Define fuzzification and defuzzification. Explain the various methods available for 	fuzzification and defuzzification. 											(10)

b.	With a neat block diagram, explain the fuzzy logic control system. Discuss the steps 	involved in designing the fuzzy logic controller. 								(10)

2.	a.	What are the different architectures used for intelligent control. Explain them briefly. 																				(10)

b.	What is a rule based system? Explain how knowledge representation is done in various 		intelligent techniques. 													(10)

(OR)

3.	a.	Describe the various fuzzy set operations – Union, Intersection, Negation taking two 	example fuzzy sets.														(10)

b.	Explain where intelligent and soft computing techniques can be effectively used.	(10)

4.	a.	Discuss from basic principles the error back propagation algorithm.			(10)

b.	Draw the structure of the Hopfield network and explain the storage and retrieval 				algorithm.																(10)

(OR)

5.	a.	Draw and explain the basic structure of a biological neuron and artificial neuron. Explain 		the basics of ANN and its learning. 											(10)

b.	What is a self organising network? Name and explain the operation of any one self 	organizing network and its training. 										(10)

6.	a.	Define optimization. Explain the ant colony search optimization technique. 		(10)

b.	Explain the concept and use of Tabu search for optimization problems.  		(10)

(OR)

7.	a.	Discuss in detail the genetic algorithm steps with a flowchart. 					(10)

b.	What are the parameters available for tuning the genetic algorithm? Explain each of them 		and their significance.  													(10)

8.	Explain the use of Fuzzy logic toolbox in Matlab.

(OR)

9.	Identify a typical problem in Power system requiring optimization and explain how the genetic algorithm can be used for the same.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	DESIGN OF TURBINES FOR RENEWABLE ENERGY SYSTEMS																			Time : 3 hours

Subject Code: 	12EE346										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Enumerate the different types of concentrating type collectors. Describe a collector used in a power plant for generation of electrical energy.							      (10)

b.	At a tidal power plant site, the difference between the high and low water tide is 8 m. The basin area is 12 km2 and average head is 6.5 m. The plant can generate a power for   3 h in each cycle and the overall efficiency of the generation is 65%. Calculate the instantaneous power generation and the yearly power output. Average specific weight of sea water is 1025 kg/m3 and there are 705 full tide cycles in a year.				      (10)

2.	a.	Calculate the useful energy gain and collection efficiency of a shell and tube flat 	collector between 10 and 11 a.m. on April 15, for Bhopal (latitude 23° N). Collector is 	tilted towards equator at 38°. The measured radiation is 480 kcal / hr m2, over a 	horizontal surface. Ambient temperature ta = 25°C. For the collector, assume overall loss 	coefficient UL = 6.88 kcal / hr m2 °C.

Thermal conductivity of the plate                = 180 kcal / hr m2 °K.

Centre to centre distance of tubes                = 15 cm.

Fin thickness                                                = 0.05 cm.

The tube outer diameter                               = 1.0 cm.

Fluid to tube heat transfer coefficient  = 1200 kcal / hr m2 °C with negligible bond resistance and wall resistance.

Cover transmittance for solar radiation is 0.88, solar absorptance (a) is 0.95, collector size is 1 m x 2 m, water flow rate is 72 kg / hr.

Water inlet temperature is constant and is equal to 60°C. 						      (14)

b.	What is the principle collection of solar energy used in a non-convective solar pond? Describe a non-convective solar pond for solar energy collection and storage. 		(6)

(OR)

3.	a.	Draw and explain the roles of a bypass diode and a blocking diode in a PV module.   (10)

b.	Calculate the efficiency and peak power of a Si solar cell operating at 27°C, with short circuit current of 2.2 A, and operating under standard illumination of 1000 W/m2. The area of the solar cell is about 100 cm2. If the operating temperature of the solar cell increases to about 35°C, calculate the efficiency. 							      	      (10)

4.	a.	With the help of a neat sketch explain the operation of a Savonius rotor and Darrieus 	rotor. List out its advantages and disadvantages.									(6)

b.	Define 	i.	Yaw control		ii.	Pitch control 		iii.	Stall control.				(6)

c.	Wind at 1 standard atmospheric pressure and 15°C has velocity of 15 m/s. Calculate:

a.	the total power density in the wind stream,

b.	the maximum obtainable power density

c.	a reasonably obtainable power density,

d.	the total power and

e.	the torque and axial thrust. Given: turbine diameter = 120 m, and turbine operating speed = 40 r.p.m. at maximum efficiency. Propeller type wind turbine is considered. 	(8)

(OR)

[P.T.O.]

5.	a.	Derive an expression for energy that can be extracted from wind. 				      (13)

b.	A WEG generates 1500 W at rated speed of 24 kmph at atmospheric pressure and temperature of 20°C. Calculate the change in output if the wind generator is operated at an altitude of 1800 m, temperature 10°C, wind speed 30 kmph, and air pressure 0.88 atmosphere. 																	(7)

6.	a.	A Pelton wheel driven by two similar jets transmits 4000 kW to the shaft when running 	at 400 r.p.m. The head from the reservoir level to the nozzle is 200 m and the efficiency 	of power transmission through the pipelines and nozzles is 90%. The jets are tangential 	to a 1.5 m diameter circle. The relative velocity decreases by 10% as the water traverses 	the buckets, which are so shaped that they would, if stationary, deflect the jet by 165°. 	Neglecting windage losses, estimate:	a.	the efficiency of the runner 	b.	the 	diameter of each jet. 														      (10)

b.	Discuss with neat sketches the characteristic curves related to the performance of 			hydraulic turbine. 														      (10)

(OR)

7.	a.	A Francis turbine of specific speed 210 is to develop 30 MW at 180 r.p.m. An 	experimental model is to be made to operate at a flow of 0.6 m3/s and head of 4.5 m. 	Assuming an efficiency of 88%, estimate the speed, power and scale ratio for the model. 	Using the same efficiency estimate the flow through the turbine. 				      (10)

b.	What are Kaplan turbines? How is a Kaplan turbine different from a propeller turbine? 		Explain the characteristic features of a Kaplan turbine.						      (10)

8.	An OTE power plant uses propane in the cycle. The warm water enters the evaporator at 28◦C and leaves at 25◦C. The cold water enters the condenser at 5◦C and leaves at 8◦C. Evaporating and condensing temperatures of the propane are 22◦C and 11◦C, respectively. The turbine machine efficiency is 0.87 and that of the generator 0.93. The power plant output is 130 MW. Operation of the station auxiliary equipment requires 15% of the output power. Calculate:

a.	the propane flow rate.

b.	the thermal efficiency of the power plant.

c.	the area of the heat transfer surface of the evaporator.

d.	the area of the heat transfer surface of the condenser.

(OR)

9.	a.	A total flow hot water geothermal power plant receives water at 225◦C. The pressure at 		turbine inlet is 1.03 MPa. The plant uses a direct contact condenser operating at 34.34 		kPa. The turbine has a polytropic efficiency of 0.70. For a cycle net output of 12 MW, 		calculate:

i.	the hot water flow (kg/h).

ii.	the condenser cooling water flow (kg/h) if cooling water is available at 25◦C.

iii.	The thermal efficiency.

iv.	Plant heat rate (kJ/kWh).												      (12)

b.	Draw and explain the working principle of vapour-dominated geothermal power plant.
																			(8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	BASIC ELECTRICAL ENGINEERING				      	     Time : 3 hours

Subject Code:	EE101/DEE101									      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The two laws that form the basis of circuit analysis were enunciated by

a.	Faraday					b.	Ohm

c.	Bohr					d.	Kirchhoff

2.	The unit of inductance is Henry. It is represented by

a.	V/A						b.	V sec/A

c.	V sec					d.	V/sec

3.	The effect of temperature on metals and insulating materials is that the

a.	Resistance of both increases

b.	Resistance of both decreases

c.	Resistance of metal decreases and that of insulating material increases

d.	Resistance of metal increases and that of insulating material decreases

4.	Heat losses in any electrical device, such as a generator, motor, etc., are independent of

a.	Voltage					b.	Current

c.	Power factor				d.	All the above

5.	The root mean square value of an alternating quantity is another name for its

a.	Instantaneous value		b.	Peak value

c.	Effective value			d.	Average value

6.	Which of the following describes a transformer?

a.	Mechanical device		b.	Static device

c.	Electrostatic device		d.	Rotating device

7.	The first presentation of a poly phase induction motor was made by

a.	Tesla					b.	Kirchhoff

c.	Faraday					d.	Lenz

8.	Which of the following is not a subsystem of an electrical energy system?

a.	Distribution system

b.	Protection and control system

c.	Transmission system

d.	None of the above

9.	Which of the following is responsible for producing a deflecting torque?

a.	Magnetic effect			b.	Damping effect

c.	Controlling system		d.	All the above

10.	Which of the following applies to a wattmeter

a.	It has a voltage coil

b.	It has a current coil

c.	The voltage coil has a high resistance

d.	All the above

PART – B  (5 x 3 = 15 MARKS)

11.	A coil consists of 2000 turns of copper wire having a cross sectional area of 0.8 sq.mm. The mean length per turn is 80cm and the resistivity of copper is 0.02 micro ohms-metres. Find the resistance of the coil and power absorbed by the coil when connected across 110 V DC supply.

[P.T.O.]

12.	An alternating current is expressed as i=14.14 sin 314t. Determine:

a.	rms current			b.	frequency

13.	List out the types of distribution systems.

14.	A three-phase induction motor is wound for four poles and supplied from a 50Hz supply.

Calculate:	a.	the synchronous speed 	b.	and the speed of the rotor when the slip is 3%

15.	A 250 Volt moving iron voltmeter takes a current of 0.05 A when connected to a 250 volt DC supply. The coil has an inductance of 1 Henry. Determine the reading on the meter when connected to a 250 V, 100Hz AC supply.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the construction and working principle of DC generator with a neat diagram.

(OR)

17.	a.	A platinum coil has a resistance of 3.146 ohms at 40οC and 3.767 ohms at 100οC. Find the 	resistance at 0οC and the temperature co-efficient of resistance at 40οC.   			(7)

b.	List the various types of DC generators and give the applications of any one of the 	generators. 																	(8)

18.	a.	Compute the average and RMS value of the square voltage wave shown in figure.  	(5)

b.

Find the form factor of the waveform shown in the figure.							(10)

(OR)

19.	Explain the construction and working principle of a transformer with necessary diagram.

20.	Draw the schematic layout of a nuclear power station and explain the basic parts.

(OR)

21.	Explain the basic layout of a power system network with a neat diagram.

22.	Why single phase induction motor is not self starting? Explain any one method by which the motor is made self starting.

(OR)

23.	Explain the principle of operation of synchronous motor. List out the application of synchronous motor.

24.	Explain the various torques which act on the moving system of the instruments for their satisfactory operation.

(OR)

25.	Write short notes with neat diagram on:

a.	Earthing							b.	Fluorescent tube wiring					(8+7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	ELECTRIC CIRCUIT ANALYSIS						       Time: 3 hours

Subject Code:	EE201				       						              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	State Kirchoffs current law.

2.	Draw the VI relationship of practical current source.

3.	Define series resonance.

4.	For the circuit shown in the figure, determine the value of capacitive reactance and impedance at resonance.

5.	Give the expression for star to delta transformation.

6.	State the dot rule.

7.	For perfect coupling, the coefficient of coupling is ___________.

8.	What is the condition for superposition theorem?

9.	Define time constant for RC circuit.

10.	Write the condition for critical damping.

PART – B (5 x 3 = 15 MARKS)

11.	Find resistance across AB for the figure below.

12.	An alternating current is expressed as i=14.14sin314t. Determine rms current and frequency when t = 0.02ms.

13.	Using nodal analysis, determine the current in the 20 Ω resistor shown in the figure.

14.	Write briefly on Millman’s theorem.

[P.T.O]

15.	A series RC circuit consists of resistors of 10 Ω and capacitor of 0.1F as shown in the figure. A constant voltage of 20V is applied to the circuit at t = 0. Obtain the current equation. Determine the voltage across the resistor and the capacitor.

PART – C (5 x 15 = 75 MARKS)

16.	a. 	Determine the total current taken from the source for the circuit shown in the figure.	(7)

b. 	Determine the voltages V1, V2, V3 and V4 in circuit shown in the figure.				(8)

(OR)

17.	a. 	Find the equivalent resistance between A and B for the circuit shown in figure.		(8)

b. 	Compare series and parallel circuits. 											(7)

18.	a. 	Derive the following expression for a series resonant circuit: Resonant frequency, Half-      		power frequencies, quality factor and bandwidth. 								(10)

b.	A series RLC circuit shown in the figure, has R = 20Ω, L = 0.005H and C =  0.2 x10 -6F. It is fed from a 100V variable frequency source. Find: 	i. 	frequency at which current is maximum 	ii.	 impedance at this frequency 	iii.	 voltage across inductance at this frequency. 																	(5)

[P.T.O]

(OR)

19.	Explain the two wattmeter method of measurement of the three phase power with a neat phasor diagram.

20.	a. 	Determine the current supplied by each battery in the circuit shown in the figure using 			mesh analysis.																(7)

b. 	Calculate the voltage across the 15Ω resistor in the network shown in the figure below 	using nodal analysis.															(8)

(OR)

21.	a. 	Derive the expression for coefficient of coupling between two coils and mention its 	significance. 																(10+5)

b. 	Two identical coupled coils in series have equivalent inductance values of 0.080 H and 	0.0354 H. Find the values of the inductance, mutual inductance and coefficient of 	coupling.

22.	a. 	State and explain superposition theorem. 										(8)

b. 	Find the maximum power across the load impedance for the circuit shown in figure.	(7)

(OR)

23.	a. 	State and explain Thevenin’s theorem 											(8)

b. 	Using Norton’s theorem, find current through 5Ω resistance shown in the figure.		(7)

[P.T.O]

24.	a. 	Define time constant and derive the expression for the transient current and voltage across 	the resistance and capacitance of a series RC circuit when connected across supply voltage 	E. 																			(8)

b. 	A series RC circuit consists of resistor of 10 Ω and capacitor of 0.1F as shown in figure. 	A constant voltage of 20V is applied to the circuit at t=0. Obtain the current equation. 	Determine the voltage across the resistor and the capacitor.						(7)

(OR)

25.	a. 	Derive the current i (t) for series RLC circuit. Also explain the nature of current for 				parameter variations.															(8)

b. 	For the circuit shown in the figure determine the current i1 and i2 when the switch is at 			t = 0. Assume that the initial current through the inductor is zero. 					(7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	ELECTRICAL TECHNOLOGY						      	     Time : 3 hours

Subject Code:	EE205									   		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	For a 4-pole, 2-layer, D.C lap-winding with 20 slots and one conductor per layer determine the number of commutator bars.

2.	If the __________ winding of a DC shunt motor is opened, the speed of the motor will be increased to an unstable value.

3.	When will a Bucholz relay operate in a transformer?

4.	Full load copper loss in a transformer is 1600 watts. What will be the loss at half load?

5.	Which type of synchronous generators are used in Hydroelectric plants?

6.	Write down the formula for distribution factor.

7.	How can the direction of rotation of the motor be reversed?

8.	Give the condition for maximum torque for 3-phase induction motor when the motor is running?

9.	A __________ motor is an incremental motion machine.

10.	What type of motor is used for low power applications?

PART – B  (5 x 3 = 15 MARKS)

11.	What is back emf in D.C motors?

12.	Why transformers are rated in kVA?

13.	Define the term voltage regulation of an alternator.

14.	Can we add extra resistance in series with squirrel cage rotor? State the reason?

15.	Differentiate between capacitor-start induction run motor and resistance-start induction run motor.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	A 4-pole, lap-wound, D.C. shunt generator has a useful flux per pole of 0.07 Wb. The 	armature winding consists of 220 turns each of 0.004 Ω resistance. Calculate the terminal 	voltage when running at 900 r.p.m. if the armature current is 50 A. 					(8)

b.	Explain the term ‘critical field resistance’ and ‘critical speed’, of a DC shunt generator 	with reference to relevant characteristics. 										(7)

(OR)

17.	a.	Draw and explain in detail the speed-torque characteristic of D.C. series motor and state	why the series motors should not be started without load. 							(5)

b.	What is the importance of starter in a dc motor? Draw a neat sketch of a three point starter inclusive of protective relays and explain clearly its operation.						(10)

18.	a.	Derive an equation for e.m.f. induced in a transformer.							(3x5)

b.	A 4500 V/225 V, 50 Hz single-phase transformer is to have an approximate e.m.f. per turn of 15 V and operate with a maximum flux of 1.4 T. Calculate:		i.	the number of primary and secondary turns 	ii.	the cross-sectional area of the core.

c.	With a neat diagram explain the Scott connection of transformers for 3-phase to 2-phase conversion.

[P.T.O.]

(OR)

19.	a.	A 400 kVA transformer has a primary winding resistance of 0.5 Ω and a secondary 	winding resistance of 0.001 Ω. The iron loss is 2.5 kW and the primary and secondary 	voltages are 5 kV and 320 V respectively. If the power factor of the load is 0.85, determine 	the efficiency of the transformer 	i.	on full load	ii.	on half load.				(10)

b.	Explain the principle of operation of an auto-transformer. 							(5)

20.	a.	Describe the main constructional features of cylindrical rotor and salient pole alternators.																						(10)

b.	Find the no-load phase and line voltage of a star-connected 3-phase, 6-pole alternator which runs at 1200 rpm, having flux per pole of 0.1 Wb sinusoidally distributed. Its stator has 54 slots having double layer winding. Each coil has 8 turns and the coil is chorded by 1 slot. 																		(5)

(OR)

21.	a.	With the help of a neat sketch, explain the synchronizing of three phase alternators.	(8)

b.	Draw and explain the working principle of a synchronous motor.					(7)

22.	a.	Why an induction motor will never run at its synchronous speed?					(5)

b.	A 415 V, three-phase, 50 Hz, 4 pole, star-connected induction motor runs at 24 rev/s on full load. The rotor resistance and reactance per phase are 0.35 Ω and 3.5 Ω respectively, and the effective rotor-stator turns ratio is 0.85:1. Calculate:

i.	the synchronous speed	ii.	the slip		iii.	the full load torque	iv.	the power output if mechanical losses amount to 770 W	v.	the maximum torque.			 (10)

(OR)

23.	a.	Explain the various types of speed control of slip ring induction motor.				  (8)

b.	The power supplied to a three-phase induction motor is 32 kW and the stator losses are 1200 W. If the slip is 5%, determine:		i.	the rotor copper loss		ii. the total mechanical power developed by the rotor	iii.	the output power of the motor if friction and windage losses are 750 W		iv.	the efficiency of the motor, neglecting rotor iron loss.																		   (7)

24.	a.	Write a brief note on linear induction motor.										    (5)	b.	With the help of a neat sketch, explain in detail the different modes of operation of stepper 			motor.																		   (10)

(OR)

25.	Find the mechanical power output at a slip of 0.05 of the 185 W, 4 pole, 110 V, 60 Hz single-phase induction motor, whose constants are given below:

Resistance of the stator main winding                      R1 = 1.86 Ω

Reactance of the stator main winding                      X1 = 2.56 Ω

Magnetizing reactance of the stator main winding  Xm = 53.5 Ω

Rotor resistance at standstill                                     R2 = 3.56 Ω

Rotor reactance at standstill                                      X2 = 2.56 Ω.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	MICROPROCESSORS AND MICRO CONTROLLERS	      Time : 3 hours

Subject Code:	EE238								      			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List the 16 – bit registers of 8085 microprocessor.

2.	List the allowed register pairs of 8085.

3.	Compare Procedure and Macro.

4.	What is the purpose of segment registers in 8086?

5.	What are the various programmed data transfer methods?

6.	Give the different types of command words used in 8259A.

7.	What are the modes used in keyboard display interface?

8.	What is synchronous data transfer?

9.	Specify the single instruction, which clears the most significant bit of B register of 8051, without affecting the remaining bits.

10.	Give the DJNZ instruction of Intel 8051 microcontroller.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the steps involved fetching a byte in 8085?

12.	Discuss the function of an instruction queue in 8086.

13.	What are the basic modes of operation of 8255?

14.	What are the different types of write operations used in 8253?

15.	Compare Microprocessor and Microcontroller.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe in detail with a neat diagram the Pin Configuration of the 8085 Processor with its functions.

(OR)

17.	Explain in detail the addressing modes of 8085 with suitable examples.

18.	Elaborate on the architecture of 8086 Microprocessor with a block diagram and also explain its functions in detail.

(OR)

19.	Write an 8086 ALP to sort an array of ten bytes in ascending order. Add comments to your program.

20.	a.	Using a model, write a program to communicate between two microprocessors using 8255. 	(10)

b.	Show the control word format of 8255 and explain how each bit is programmed. 			(5)

(OR)

21.	With a neat block diagram, explain the description and functions of 8259.

22.	Discuss in detail the interfacing of a Temperature Controller using 8085 Processor.

(OR)

23.	Explain in detail the procedure and the block diagram involved in the Traffic Light Controller using 8085.

24.	Describe the architecture of 8051 with a neat diagram.

(OR)

25.	Discuss in detail the various modes available for a timer in 8051.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	ELECTRIC CIRCUITS								      	     Time : 3 hours

Subject Code:	EE245											      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Unit of conductance is _____________.

2. 	True or false: Inverse of impedance is known as susceptance.

3. 	Form factor = _____________.

4. 	Peak factor =  _____________.

5. 	Define the term mesh current.

6. 	Define the term node voltage.

7. 	Draw the Thevenin’s equivalent circuit.

8. 	Maximum power is transferred when load resistance is equal to _____________.

9. 	Inverse of selectivity is known as _____________.

10. 	True or false: Bandwidth is the range of frequencies.

PART – B  (5 x 3 = 15 MARKS)

11.	State Kirchhoff’s voltage law.

12.	Define the term power factor.

13.	Define the term dependent source voltage.

14.	State superposition theorem.

15.	Define Anti-resonance.

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the unknown currents through the branches of the network given.

(OR)

17.	For the given network determine the equivalent resistance between PQ.

[P.T.O]

18.	For the pure sinusoidal waveform determine the rms value, average value, form factor and peak factor.

(OR)

19.  Derive the equations for active power, reactive power, apparent power and power factor for the 	RLC series circuit supplied by V (t)=Vm sinωt.

20. 	Determine the voltage across 10Ω using nodal analysis.

(OR)

21.	Determine the branch currents using mesh current method

22.	For the given network, determine the value of R for maximum power transfer through it and the maximum power.

(OR)

23.	Determine the current through 5Ω resistance using superposition theorem

24.	Determine the value of capacitance in order that the circuit in the figure is resonant at 6366 Hz.

(OR)

25.	A current source is applied to a parallel combination of R, L and C. Where R=10 Ω, L=1 H

and C= 1µF, compute 		a.	resonant frequency	b.	Q factor		c.	bandwidth

d.	lower and upper half frequency points of the bandwidth.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	NETWORK THEORY								      	     Time : 3 hours

Subject Code:	EE246									   		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by phase sequence?

2.	Write the phase relation between the line voltages and their respective phase voltage in a three phase balanced star connected system.

3.	What is magnetic coupling?

4.	Write any one application of double tuned circuits.

5.	Inductor does not allow sudden changes in ____________.

6.	State the final value theorem of  Laplace transform.

7.	The Y-parameters Y11 and Y21 are obtained by _____________ output terminals.

8.	What is band pass filter?

9.	The poles and zeros of the stable system must lie on the ___________ half of S-plane.

10.	In second Cauer LC network, the last element is an inductor when the driving point function consists of ____________.

PART – B  (5 x 3 = 15 MARKS)

11.	What do you mean by balance load and unbalance load?

12.	State the dot rule for coupled coils.

13.	What is meant by poles and zeros of network function?

14.	What is an attenuator? Mention its application.

15.	List the properties of Hurwitz polynomial.

PART – C  (5 x 15 = 75 MARKS)

16.	An unbalanced four-wire, star connected load has a balanced voltage of 400V, the loads are     Z1 = (4+j8) Ω, Z2 = (3+j4)Ω,  Z3 = (15+j20)Ω. Calculate the:	a.	line currents

b.	current in the neutral wire 		c.	the total power.

(OR)

17.	In the two wattmeter method of power measurement, the power registered by one wattmeter is 3500W, while the other reads down scale. After reversing the latter, it reads 300W. Determine the total power in the circuit and the power factor.

18. 	Two identical coupled coils in series have equivalent inductance values of 0.084H and 0.0354H. Find the values of L1 , L2 , M and coefficient of coupling K.

(OR)

[P.T.O.]

19.	Consider the single tuned circuit shown in the figure. Determine:	a.	resonant frequency

b.	 the output voltage at resonance 	c.	the maximum output voltage. Assume  Rs >>ωrL1, and K = 0.9.

20.	For the circuit shown in the figure, find the expression for the current. Assume initial charge on the capacitor is zero.

(OR)

21.	Determine the driving point impedance of the network shown in the figure.

22. 	Find the ABCD parameters for the network shown in the figure.

(OR)

23.	Design a π-type attenuator to give 20 dB attenuation and to have a characteristic impedance of 100 Ω.

24.	Check the positive realness of the function  s2+6s+5s2+9s+14.

(OR)

25.	Find the first and second Cauer forms of the RC network whose driving point impedance

is (s+2)(s+4)s(s+3).

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	ELECTRON DEVICES								      	     Time : 3 hours

Subject Code:	EE247									   		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the barrier potential of silicon (Si)?

2.	Write down the diode current equation.

3.	Give one advantage of semiconductor device over vacuum tubes.

4.	In which region the BE and CB junctions are forward biased?

5.	JFET is a __________ controlled device.

6.	Draw n-channel and p-channel symbol of FET.

7.	What are the two types of MOSFET?

8.	Name the three terminals of JFET.

9.	What is thin film IC?

10.	In ion implantation __________ passive layer forms effective barrier.

PART – B  (5 x 3 = 15 MARKS)

11.	Write short notes on Schottky barrier diode and draw the characteristics of the diode.

12.	Calculate αdc and βdc for the given transistor for which Ic = 5mA, Ib = 50µA, Ico = 1µA.

13.	Distinguish between BJT and FET.

14.	Write short notes on CMOS devices.

15.	What is metallization?

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the expression for temperature dependence of diode characteristics.

(OR)

17.	Explain briefly about Zener diode and its characteristics with a neat diagram.

18.	With a neat diagram, explain input and output characteristics of CB configurations.

(OR)

19.	Explain in detail about transistor switching times.

20.	Explain the operation of JFET with a suitable diagram.

(OR)

21.	a. 	How JFET act as a switch?													(7)

b. 	How JFET act as a variable resistor?											(8)

22. 	With a suitable diagram, elaborate on the Enhancement Of MOSFET.

(OR)

23. 	a. 	How does MOSFET act as a resistor?

b. 	Explain briefly about CMOS devices.

24. 	Discuss in detail about BJT fabrication with a suitable diagram.

(OR)

25. 	Explain in detail about IC capacitors.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	ELECTRO MAGNETIC FIELDS					      	     Time : 3 hours

Subject Code:	EE248								   			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A vector A is said to be irrotational if its curl is _________.

2.	Write down the condition for two vectors A and B to be perpendicular.

3.	What is electric flux density?

4.	Write down the Poisson’s and Laplace’s equation.

5.	What is the magnetic field due to a toroid?

6.	Define magnetic moment.

7.	What are eddy currents?

8.	Define Lenz’s law.

9.	The characteristic impedance of free space is _________ ohms.

10.	Define circular polarization.

PART – B  (5 x 3 = 15 MARKS)

11.	Find the distance from A(r =5, θ = 20°, Ф=120°) and B(r =2, θ = 80°, Ф=30°) in spherical

coordinates.

12.	State Gauss law.

13.	State Biot-Savart law.

14.	What is displacement current?

15.	Mention the properties of uniform plane wave.

PART – C  (5 x 15 = 75 MARKS)

16. 	State and prove Stokes Theorem.

(OR)

17. 	State and prove Divergence Theorem.

18. 	Derive the boundary conditions at the interface between two perfect dielectrics.

(OR)

19. 	Derive the point form and integral form of continuity equation for current.

20. 	Find the magnetic flux density and magnetic field intensity due to infinitely long straight conductor.

(OR)

21. 	Find the expressions for the inductance of:

a.	Solenoid.                                                                                                                          (7)

b.	Co-axial cable.                                                                                                                	(8)

22.	a.	Write short notes on Faraday’s law of electromagnetic induction.                                	(8)

b.	For a 1 A conductor current in a copper wire find the corresponding displacement current at 100 	MHz. Assume for copper σ = 5.8 × 107 mho/m.                                                     	(7)

(OR)

[P.T.O.]

23. 	Derive the Maxwell’s curl equation from Ampere’s law and Faraday’s law. Express the equations in phasor form for time harmonic fields.

24. 	Define wave. Derive the free space electromagnetic wave equation.

(OR)

25. 	State Poynting’s theorem and derive the Poynting’s vector in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	DC MACHINES AND TRANSFORMERS			      	     Time : 3 hours

Subject Code:	EE250								      	       	       	      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name the rule that gives the direction of motion of DC motors.

2.	What are the main effects of armature reaction?

3.	A single phase transformer designed for 50 Hz operation is connected to a supply of 60 HZ. What will happen?

4.	What are the methods of speed control of DC motors?

5.	How does the no-volt release protect the motor?

6.	What is the condition for maximum efficiency of a transformer?

7.	Define all-day efficiency.

8.	What is the use of Swinburne’s test?

9.	What are the different types of testing of transformers?

10.	What is the condition for maximum efficiency of DC machines?

PART – B  (5 x 3 = 15 MARKS)

11.	What are the requirements to be satisfied for parallel operation?

12.	A DC motor connected to a 460V supply takes an armature current of 120A on full load. The armature circuit has a resistance of 0.25Ω. Calculate the value of the back EMF at this load.

13.	Draw the circuit diagram of a transformer for resistance and reactance referred to primary.

14.	What are the advantages and disadvantages of Hopkinson’s test?

15.	Why does the external characteristic of a DC shunt generator droop downwards?

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Describe with sketches the construction of a DC machine. 								(9)

b.	Derive the EMF equation of a DC generator. 										(6)

(OR)

17. 	a.	A 40kW, 250V DC shunt generator has armature and field resistances of 0.05Ω and 150Ω 	respectively. Calculate the total power developed by the armature when it delivers full load output.																				(7)

b.	Explain the operation of external characteristics of different types of DC generators.		(8)

18. 	a.	Discuss the operation of a three point starter with a neat sketch. 						(9)

b.	Explain the methods of speed control of a DC series motor.							(6)

(OR)

19. 	Derive the torque, speed equations of a DC motor.

20. 	In a 50kVA, 1100/20 V transformer, the iron and copper losses at full load are 350 W and 425W respectively. Calculate the efficiency at 	a.	full load with unity power factor 		b.	half load unity power factor 		c.	full load with 0.8 power factor lagging. Also, determine the maximum efficiency and the load at which maximum efficiency occurs assuming the load to be resistive.

(OR)

21. 	a.	Draw and explain the no-load phasor diagram for a single-phase transformer.         			(10)

b.	How can the equivalent impedance of a transformer be determined? 						(5)

[P.T.O.]

22. 	Draw the circuit diagrams for conducting OC and SC tests on a single phase transformer. Also explain how the efficiency and voltage regulation can be estimated by these tests.

(OR)

23. 	a.	Explain in detail about parallel operation of single phase transformers. 					(8)

b.	What is auto- transformer? State the application of auto- transformer.					(7)

24.  	List the four possible ways of connecting a bank of three transformers for three-phase service. State the applications of each of the connections listed above.

(OR)

25. 	What are the instrument transformers? Give the connection diagrams for current and potential transformers.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	INDUCTION AND SYNCHRONOUS MACHINES	     	     Time : 3 hours

Subject Code:	EE251										   	      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the principle of operation of induction motor?

2.	What is the crawling of an induction motor?

3.	State the application of linear induction motor.

4.	Name the two different theories with which principle of single phase induction motors are 	explained.

5.	Why is the stator core of alternator laminated?

6.	What is meant by load angle of an alternator?

7.	Why a synchronous motor is a constant speed motor?

8.	At what load angle does the power developed in a synchronous motor become its maximum 	value?

9.	What is the necessity for predetermination of voltage regulation?

10.	Why synchronous generators are to be constructed with more synchronous reactance and negligible resistance?

PART – B  (5 x 3 = 15 MARKS)

11.	A 3 phase, 4 pole, 50 Hz induction motor is running at 1440 rpm.  Determine the slip speed and slip.

12.	Name and write about inherent characteristics of any four types of single phase induction motors.

13.	What are the advantages of salient pole type construction used for synchronous machines?

14.	How the synchronous motor can be used as a synchronous condenser?

15.	Name the various methods for predetermining the voltage regulation of 3 phase alternator.

PART – C  (5 x 15 = 75 MARKS)

16.	Develop an equivalent circuit for a three phase induction motor by conducting suitable tests.

(OR)

17.	Discuss in detail about the theory of star – delta starter.

18.	Explain the double field revolving theory for operation of 1φ induction motor.

(OR)

19.	Explain with a neat diagram the following types of 1φ induction motor.

a.	Split phase induction run motor.

b.	Capacitor start capacitor run motor.

20.	a.	Describe the principle and construction of slow speed operation generator with a neat 			diagram 																		(8)

b.	Derive the emf equation of an alternator.										(7)

(OR)

21.	Explain the condition for parallel operation of 3 phase alternator with a neat diagram.

[P.T.O.]

22.	Derive the mechanical power developed per phase of a synchronous motor.

(OR)

23.	a.	Explain the methods of starting synchronous motor against high – torque loads.		(8)

b.	Explain the significance of V and inverted V curves.								(7)

24.	Explain how you will determine the d and q axis reactance of a synchronous machine in your laboratory.

(OR)

25.	For the salient synchronous machine, derive the expression for power developed as a function of load angle.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title:	ELECTRICAL MACHINE DESIGN					       Time: 3 hours

Subject Code:	EE252			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

# 1.	Write the formula for the output coefficient of dc machine.

# 2.	List the different dimensions of slots.

# 3.	The range of specific electric loading is _____________.

# 4.	What are the materials used for brushes in dc machines?

# 5.	Classify the transformers according to their construction.

# 6.	What are the factors to be considered for selecting the cooling method of a transformer?

# 7.	What are the main dimensions of induction motor?

# 8.	Write the formula for length of mean turn of stator of induction motor.

# 9.	What are the two types of poles used in salient pole machines?

# 10.	In turbo-alternators, the diameter is limited by_______________.

PART – B (5 x 3 = 15 MARKS)

# 11.	Define specific electric and magnetic loading.

# 12.	List the advantages of choosing larger number of poles in DC machines.

# 13.	Write the formulae for the no-load current, temperature rise in plain walled tanks and magnetizing current per phase in a transformer.

# 14.	What are the ranges of efficiency and power factor in different types of induction motor?

# 15.	List the advantages of large air gap in synchronous machines.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Explain about the various factors that decide choice of specific electric loading. 		(8)

b.	Calculate the MMF required for air gap of a dc machine having the following data: axial 			length = 20 cm (no ducts), air gap = 6 mm, useful flux per pole = 25 mWb, pole arc = 18 			cm, slot pitch is 27 mm and slot opening 12 mm. Take Carter’s coefficient for slot as 0.3. 																						(7)

(OR)

17.	a.	Explain about the different methods for calculating MMF required for teeth. 		(8)

b.	A 350 KW, 500 V, 450 rpm, 6 pole dc generator is built with an armature diameter of 0.87m and core length of 0.32m. The lap wound armature has 660 conductors. Calculate the specific electric and magnetic loadings. 										(7)

18.	a.	Discuss about the various factors related to fixing of length of air gap of a DC machine. (6)

b.	Design a suitable commutator for a 350 KW, 600 rpm, 440 V, 6 pole DC generator having 		an armature diameter of 0.75 m. The number of coils is 288. Assume suitable values 				wherever necessary. 															(9)

(OR)

19.	a.	Derive the output equation of a DC machine. 									(7)

[P.T.O]

b.	Calculate the main dimensions of a 200 KW, 250 V, 6 pole, 1000 rpm generator. The maximum value of the flux density in the air gap is 0.87Wb/m2 and the ampere conductors per metre of armature periphery are 31000. The ratio of pole arc to pole pitch is 0.67 and the efficiency is 0.91. Assume the ratio of length of core to pole pitch = 0.75. 		(8)

20.	Estimate the dimensions of the core , the number of turns, winding conductor area in primary and secondary side of a 1-phase, core type transformer rated at 100 KVA, 2200/480 V, 50 Hz. Assume the following data: Maximum flux density =1.2 Wb/m2, Ratio of effective cross-sectional area of core to square of diameter of a circumscribing circle is 0.6, Ratio of height to width of window is 2, δ = 2.5A/mm2, Approximate Volt per turn = 7.5 V, Window space factor = 0.28.

(OR)

21.	The tank of 1250 KVA, natural oil cooled transformer has the dimensions of length, width and height as 1.55 X 0.65 X 1.85 m respectively. The full load loss = 13.1KW, loss dissipation due to radiations = 6 W/m2-0C, loss dissipation due to convection = 6.5 W/m2-0C, Improvement in convection due to provision to tubes = 40%, Temperature rise = 400C, Length of each tube =1m, diameter of tube = 50 mm. Find the number of tubes for this transformer. Neglect the top and bottom surface of the tank as regards the cooling.

22.	Estimate the stator core dimensions, number of stator slots, number of stator conductors per slots for a 3 phase, 100 KW, 3300 V, 12 pole, 50 Hz, star connected slip ring Induction motor.  Bav = 0.4 Wb/ m2  and Specific electric loading = 25000 amp. cond./m., p.f. =0.9, η = 0.9. Choose the main dimensions to give best power factor. The slot loading should not exceed 500 amp. cond.

(OR)

23.	Estimate the main dimensions, number of stator slots, stator turns per phase and for a 3 phase, 70 HP, 415 V, 6 pole, 50 Hz Induction motor. The motor is star connected. Bav = 0.51 Wb/ m2    and Specific electric loading = 30000 amp. cond./m. L/г = 1, p.f. =0.91, η = 0.9. Estimate the number of stator conductors required for a winding in which the conductors are connected in two parallel paths. Choose a suitable number of conductors per slot, so that the slot loading does not exceed 750 amp. cond.

24.	Determine the output coefficient for a 250 KVA, 1100 V, 3 phase, 500 rpm, 12 pole, 50 Hz Alternator. Assuming Specific electric loading = 30000 amp. cond./m. and L/г = 1.5, find D and L. Assume average air gap density as 0.6 Wb/m2. Find also the number of stator conductors, number of stator slots and cross-section of stator conductors.

(OR)

25.	Write short notes on the following:

a.	Short Circuit Ratio.															(7)

b.	Design of turbo alternator.														(8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	GENERATION, TRANSMISSION AND DISTRIBUTION      Time : 3 hours

Subject Code:	EE253								      			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the practical voltage levels of transmission and distribution in Indian electricity grid?

2.	List the types of water turbines.

3.	Mention the types of line supports.

4.	What is surge impedance loading?

5.	What is Corona?

6.	What is a stringing chart?

7.	Define insulation resistance.

8.	Mention few advantages of underground cables over overhead lines.

9.	What is a feeder?

10.	Why 3-phase, 4-wire system is preferred for AC distribution system?

PART – B  (5 x 3 = 15 MARKS)

11.	Discuss the merits and demerits of a hydro power plant.

12.	What are the advantages of high transmission voltage?

13.	Enumerate the factors affecting corona.

14.	Draw the sketch of a single core low tension cable.

15.	What is an interconnected system?

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the schematic diagram of a modern thermal power station and explain its operation 	in detail.

(OR)

17.	a.	Discuss the important points to be considered in selecting the number and size of 				generating units. 							                         						(5)

b.	A generating station with the installed plant capacity of 40MW has the following 	daily 	load cycle:                                                                                         					(10)

| Time (Hrs) | 0 – 6 | 6 – 10 | 10 – 12 | 12 – 16 | 16 – 20 | 20 – 24 |
| --- | --- | --- | --- | --- | --- | --- |
| Load (MW) | 20 | 25 | 30 | 25 | 35 | 20 |

Draw the load curve and compute the load factor and plant capacity factor.

18.	a.	Derive the expression for inductance of a 3-phase single circuit line with unsymmetrical 	spacing. 						                         									(9)

b.	Write short notes on skin effect and proximity effect.                                          		(6)

(OR)

19.	a.	Using rigorous method, derive the expressions for sending end voltage and current for a 	long transmission line.                                                                          				(12)

b.	The series impedance and shunt admittance per phase of a transmission line is          (20+j52) ohms and 315 x 106 mho respectively. Using nominal T-method, calculate the ABCD constants of the line.                                                                                  		(3)

[P.T.O.]

20.	A string of 5 insulators is connected across a 100KV line. If the capacitance of each disc to earth is 0.1 of the capacitance of the insulator, calculate the distribution of voltage on the insulator discs and string efficiency.

(OR)

21.	What is sag in overhead lines? Deduce an expression for sag in overhead lines when the supports are at equal levels. Also discuss the effect of wind and ice loading in sag calculations.

22.	Derive an expression for insulation resistance and capacitance of a single core cable with a neat sketch.

(OR)

23.	What is grading of cables? Explain in detail the capacitance and inter-sheath methods of grading.

24.	a.	Write brief notes on radial and ring main system.                                              		(6)

b.	Derive an expression for voltage drop in a uniformly loaded dc distributor fed at one end.                                                                                                                        																			(9)

(OR)

25.	A single phase AC distributor 2 km long supplies a load 120A at 0.8pf  lagging at its far end and a load of 80A at 0.9pf lagging at its midpoint. Both power factors are referred to the voltage at the far end. The resistance and reactance per km are 0.05 ohm and 0.1 ohm respectively. If the voltage at the far end is maintained at 230 V, calculate the voltage at the sending end.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title: 	POWER ELECTRONICS								      Time: 3 hours

Subject Code: 	EE254			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	IGBT is voltage controller device (Yes / No).

2.	Compare turn-off current and turn-on current of GTO.

3.	Express the average output voltage for fully controlled Thyuristor (1- ø) gridge with R – L

load.

4.	Write the purpose of freewheeling diode.

5.	What is difference between step-down and step-up cyclo converter?

6.	Define duty ratio of DC chopper.

7.	What is pulse – width modulation?

8.	How is turn – off performed in inverter operation?

9.	Which device is used to control an induction motor?

10.	What is Bi-polar HVDC?

PART – B  (5 x 3 = 15 MARKS)

11.	What is Triac? Write its applications.

12. 	What is four quadrant in dual converter.

13.	Draw the step-up DC chopper circuit.

14.	Write the difference between voltage source and current source inverter.

15. 	How are the D.C motor field and armature controlled using a 1 – ø converter.

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain static and dynamic characteristics of 										(8+7)

a.	SCR							b.	MOSFET

(OR)

17. 	a.	What is string efficiency? Find the string efficiency of SCR series stack with 10 numbers 			of each 100 volts and total voltage is 900V.										(5)

b.	Explain static and dynamic equalization of series SCR connection.					(10)

18.	a.	Explain the 1 – ø fully controller converter with 									(8)

i.	RL load							ii.	RL with freewheeling diode

b.	Derive necessary output voltage Equation in both cases.							(7)

(OR)

19. 	A single – Phase series full converter is operated from 120V, 50Hz supply with load resistance 	R = 10 . If the average output voltage is 75% of maximum possible average output voltage,

calculate:

a.	Delay angle 						b.	RMS and average output currents

c.	input power factor.

[P.T.O]

20. 	Explain																			(8+7)

a.	Two – quardrant 				b.	Four quadrant DC chopper with RL and RLC load

(OR)

21. 	A 1 – ø AC voltage controller has resistive load R = 10 and input voltage is 120 V, 50Hz.

The delay angle α =π2. Determine:

a.	output RMS voltage 			b.	input power factor 		c.	average input current

22.	a.	What is the difference between 1 – ø half and full bridge inverter. Explain in detail.	(7)

b.	Derive the output voltage expression with harmonics for 1 – ø full bridge inverter.	(8)

(OR)

23. 	Explain 3 – ø bridge inverter operation in 1800 mode of operation.

24. 	Explain different control schemes for: 												(8+7)

a.	DC motor 						b.	induction motor using thyristor device

(OR)

25. 	Mention different pulse width modulation techniques used in inverter AC output generation. 	Explain sinusoidal pulse width modulation in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	POWER SYSTEM ANALYSIS						      	     Time : 3 hours

Subject Code:	EE255									   		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Given: Base voltage = 11,000V and Base power = 5 MVA what is the value of base impedance?

2.	1+a+a2+a3= __________.

3.	What is bolted or solid fault?

4.	In type 1 modification of Z-bus building algorithm, off diagonal elements are always __________.

5.	What is the need of load flow study?

6.	List the different types of bus.

7.	Define Incremental efficiency.

8.	Write down the Power balance equations.

9.	How can we classify stability studies?

10.	Define swing curve.

PART – B  (5 x 3 = 15 MARKS)

11.	Prove that neutral current can flow only if zero – sequence current is present.

12.	State the properties of Bus admittance matrix.

13.	Compare Newton Raphson and Gauss-Seidal method of load flow solution.

14.	What are the different types of constraints used in economic dispatch problem?

15.	List the various causes for voltage instability.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail the need for system analysis in planning and operation of a power system.

(OR)

17.	The one line diagram of an unloaded power system is shown in the figure. Reactances of the two sections of transmission line are shown in the diagram. The generator and transformer rated as follows,

G1-20MVA, 13.8KV, X=0.2P.u, G2-30MVA, 18KV, X=0.2P.u, G3-30MVA, 20KV, X=0.2P.u, T1-25MVA, 20/220KV, X=0.2P.u, T2-3-single phase transformer, each unit 10MVA, 127/18KV, X=10%, T3-35MVA, 220/22KV, X=10%. Draw the impedance diagram with all the reactance marked in p.u choose the base as 50MVA, 13.8KV in the circuit of generator1 (G1).

[P.T.O.]

18.	Obtain the ZBus for the 3-bus sample system shown in the figure below.

(OR)

19.	a.	Derive the fault current expression for a single line to ground fault that occurs in a power 			system network.																(5)

b.	For a three phase solid fault on bus number 1 shown in the figure below, determine the 	fault current supplied by each generator.											(10)

20.	For the network shown in the figure below, obtain the complex bus bar voltages at the end of first iteration using G-S method. Bus -1 is slack bus with 10.  Take P2 +jQ2= 5.96+1.46j, P3= 6.02 and V3=1.020 and V2=10. Line impedances are in p.u.

(OR)

21.	With a flow chart and necessary equations, explain the NR Iterative method for solving load flow problem.

22.	a.	The Fuel cost equation for three thermal plants in Rs/hr are given by

F1 = 0.004P12+5.3P1+500

F2 = 0.006P22+5.5P2+400

F3 = 0.009P32+5.8P3+200 where P1, P2 and P3 are in MW. The total load is 800 MW. Neglecting line losses and generating limits, finds the optimal dispatch and the fuel cost in Rs/hr.      						        										(10)

b.	Derive the coordination equation for transmission loss in economic scheduling problem.                                                                                               																					(5)

(OR)

23.	a.	Explain the various constraints in unit commitment problem with suitable examples.	(7)

b.	Give the elementary idea of optimal load scheduling of Hydro - Thermal plants. 		(8)

24.	Derive swing equation for a single machine connected to an infinite bus system. State the assumptions if any and state the usefulness of this equation. Damping is not to be neglected.  State the reasons for non-linearity of this equation.

(OR)

25.	Draw a flow chart which shows the various steps involved in the modified Euler’s method to solve the swing equation.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	POWER SYSTEM PROTECTION AND SWITCHGEARS     Time : 3 hours

Subject Code:	EE256			       						                     Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	List the types of faults in power systems.

2.	What is a current limiting reactor?

3.	What is pick-up current?

4.	What is a differential relay?

5.	Define arc voltage.

6.	Mention the high resistance methods of arc extinction.

7.	What is a voltage surge?

8.	What is arcing ground?

9.	What is meant by earthing?

10.	What is insulation coordination?

PART – B  (5 x 3 = 15 MARKS)

11.	Discuss the need for a protective scheme.

12. 	What is a protective relay? Mention the fundamental requirements of protective relays.

13.	Write a brief note on circuit breaker ratings.

14.	Explain the types of lightning strokes.

15. 	Write a brief note on reactance grouding.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Explain the analysis of a symmetrical fault.										(9)

b.	State the applications of CTs and PTs.											(6)

(OR)

17. 	A small generating station has two alternators of 2500 KVA and 5000 KVA and reactances of

8% and 6% respectively. The circuit breakers are rated at 150 MVA. Due to increase in system load, it is intended to extend the system by a supply from the grid via a transformer of 10,000 KVA rating and 7.5% reactance. If the system voltage is 3.3 KV, compute the reactance to protect the switchgear.

18.	a.	Explain with a neat diagram the construction and working principle of non-directional 	induction type over-current relay.												(7)

b.	Explain the working principle of an impedance relay.								(4)

c.	Determine the time of operation of a 5A, 3 sec overcurrent relay having a current setting of

120% and a time multimer of 0.06 connected to the supply circuit through a 400/5 current transformer when the circuit carries a fault current 400 A. Take time of operation as 3.5 sec for PSM of 8.																(4)

(OR)

19. 	a.	Explain with a neat diagram the application of Merz-Price circulating current principle for 		the protection of an alternator.													(8)

b.	Explain the Translay protection scheme for feeders.								(7)

[P.T.O]

20. 	a.	An air blast circuit breaker designed to interrupt a transformer magnetizing current of 11

amps (RMS), chops the current at a instantaneous value of 7 amp. If the values of L and C in the circuit are 35.2H and 0.0023 µF find the value of the voltage that appears across the contacts of the breaker. Assume that all the inductive energy is transferred to the capacitance.																	(5)

b.	Discuss the principle of operation of an air blast circuit breaker. What are the advantages

and disadvantages of using air as the arc quenching medium.						(10)

(OR)

21. 	Draw and explain with a neat sketch the principle of operation of a sulphur hexafluoride circuit

breaker. Mention its advantages and disadvantages.

22.	Explain the operation of any three types of lighting arresters with neat sketches.

(OR)

23. 	What is a surge absorber? Discuss in detail about Ferranti surge absorber.

24.	Write short notes on:																(7+8)

a.	Arc Suspension Coil grounding		b.	Earthing transformers.

(OR)

25. 	What is insulation level? Discuss in detail the insulation levels of various substation equipments.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	LINEAR INTEGRATED CIRCUITS					      	     Time : 3 hours

Subject Code:	EE258									   		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define CMRR.

2.	The slew rate of Op-Amp is measured in volt per micro second. True/False.

3.	The gain of voltage follower is infinity. True/False.

4.	Active filters cannot provide voltage gain. True/False.

5.	Schmitt trigger is a circuit which can convert any wave into _________wave.

6.	The cutoff voltage of Precision rectifier is exactly 0.2V. True/False.

7.	The value of Line regulation in a regulator should be maximum. True/False.

8.	Astable Multivibrator has two stable states. True/False.

9.	There is no relationship between number of bits and resolution in DAC. True/False.

10.	Which ADC is the fastest?

PART – B  (5 x 3 = 15 MARKS)

11.	List important characteristics of an Ideal Op-Amp.

12.	Draw the circuit for an Integrator using Op-Amp.

13.	Mention the different techniques employed in multipliers.

14.	Define Load regulation.

15.	Define lock in range and capture range in PLL.

PART – C  (5 x 15 = 75 MARKS)

16.	Mention and explain different DC Characteristics of an Op-Amp.

(OR)

17.	Describe in detail different AC characteristics of an Op-Amp.

18.	With a neat sketch, explain the operation of instrumentation amplifier.

(OR)

19.	Derive the Transfer function of second order low pass Butterworth active filter and hence design a low pass filter with a cutoff frequency = 1 kHz.

20.	Draw the circuit of a square wave generator and explain its operation.

(OR)

21.	Draw the circuits of precision - half wave and full wave rectifiers and explain.

22.	Draw the circuit of an IC 723 regulator and explain its operation.

(OR)

23.	Explain in detail any three applications of PLL.

24.	Draw the diagram of an R-2R ladder DAC and explain its working principle.

(OR)

25.	With a neat sketch explain the operation of a dual slope ADC.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title:	CONTROL SYSTEMS									       Time: 3 hours

Subject Code:	EE259				       					                     Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is meant by the mathematical model of a system?

2.	Write the Mason’s gain formula for signal flow graphs.

3.	Determine the percentage overshoot of the system, whose closed loop transfer unction is

1000 / (S2 + 34.5 S + 1000), for unit step input.

4.	Define delay time and peak time.

5.	What are the frequency domain specifications? Explain.

6.	Obtain the corner frequencies of the system. G(s)H(s) = (s+0.5)/(s+0.25)(s+4)

7.	What are the characteristics of an unstable system?

8.	State the rule for finding out the root loci on the real axis.

9.	What is state and state variable?

10.	 List the properties of state transition matrix

PART – B (5 x 3 = 15 MARKS)

11.	Mention the equivalent electrical elements for the mass – damper – spring elements in mechanical    system.

12.	Explain the function of a PID controller.

13.	List any two advantages of frequency response analysis.

14.	Distinguish between relative stability and absolute stability.

15.	Draw the block diagram of state space equation.

PART – C (5 x 15 = 75 MARKS)

16.	Write the dynamic equation in respect of the mechanical system given in the figure. Then using the force-voltage analogy obtain the equivalent electrical network. K1, K2- spring constant

B1- viscous friction damping co-efficient, M1, M2- inertial constant of masses, x1, x2- 	displacements, F(t)- force

[P.T.O]

(OR)

17.	Determine the transfer function C(s) / R(s) for the block diagram shown in figure by drawing signal flow graph and then using Mason’s gain formula.

18.	The open loop transfer function of the three system are given as

Determine the positional, velocity and acceleration error constants for these systems.  Also for the system given in (ii) determine the steady state errors with step input r(t)= u(t), ramp input r(t) = t and acceleration input r(t) = (½) t2

(OR)

19. Obtain the unit – step response of a unity feedback control system whose open –	loop transfer 	function is G(s) = 1/ s(s+1). Obtain also the rise time, peak time, maximum 	overshoot and 	settling time.

20.	Consider a unity feedback control system with the following open-loop transfer function G(s)=Ks(s2+s+4).	Determine the value of the gain K such that the phase margin is 50º.       What is the gain margin for this case?

(OR)

21.	For the standard second order system shown in the figure with r (t) = u (t) explain how the time domain specifications corresponding to resonant peak and bandwidth can be inferred.

22.	Consider the characteristic equationS4+2S3+(4+K)S2+9S+25=0.  Using the Routh’s stability criterion, determine the range of K for stability.

(OR)

23.	Sketch the Root loci for the system shown in the figure

[P.T.O]

24.	Obtain all three canonical state variable model and realize for the given transfer function G(z)=Y(Z)R(z)=−2z3+2z2−z+2z3+z2−z−34

(OR)

25.	a.	Describe the conversion of transfer function to canonical state variable models in second 	companion form.

b.	Consider the system x1(k+1)x2(k+1)=01−2−3 x1(k)x2(k)+01(−1)k

Find y(k) for k >1.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	DIGITAL SIGNAL PROCESSING						      Time : 3 hours

Subject Code:	EE260							   	       			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A discrete time sinusoid will not be periodic if 2πω0is not a rational number. (True / False).

2.	Plot the signal u [n–2], where u [n] is a unit step sequence.

3.	What is the value of W42?

4.	Write the expression to compute DFT of the sequence x [n].

5.	What are the methods available for IIR filter design?

6.	Name the commonly used windows for FIR filter design.

7.	Express the fraction (–7/8) in sign magnitude form.

8.	What are the methods available for quantization?

9.	MAC operation in a digital signal processor can be completed in one clock cycle. (True / False)

10.	Name a common approach adopted for increasing the efficiency of digital signal processors.

PART – B  (5 x 3 = 15 MARKS)

11.	Determine the response y[n] of the system given the impulse response h[n]=1, 2 and input

x[n]=1, 2, 3.

12.	How many complex multiplications and additions are required to compute N-point DFT using DIF FFT algorithm?

13.	Given the analog filter transfer function H(s)=1s+1.Obtain the digital filter transfer function using impulse invariant technique.

14.	What is the effect of quantization on pole locations?

15.	The performance gains that can be achieved with VLIN architecture depend on what factors?

PART – C  (5 x 15 = 75 MARKS)

16.	The input –output relationship of a discrete time system is given byy[n]=x[−n−1]+x[n].  Determine whether the system is casual, memoryless, linear, time invariant and stable.

(OR)

17.	A casual DT LTI system is described y[n]−56y[n−1]+16y[n−2]=x[n] where x[n] and y[n] are the input and output of the system.  Determine the impulse response and step response.

18.	Give x[n]=1, 2, 1, 2, 1, 2, 1, 2compute the DFT using DIT algorithm.

(OR)

[P.T.O]

19.	Given x[n]=1, 2, 3, 1, 2, 3, 1, 2, 4, 1, 2and h[n]=1, 2, 3. Use overlap add method and compute convolution and verify the result using any one method used for obtaining linearly convolved output.

20.	Using Bilinear transformation design a Butterworth filter for satisfying the constraints.

0.8≤H(ejω)≤1, 0≤ω≤0.2π

H(ejω)≤0.2, 0.6π≤ω≤π

(OR)

21.	Given H(z)2z+4z(z−0.1)(z+0.5)(z+0.4). Draw the cascade realization.

22.	Discuss in detail about the different types of errors that occur due to finite register length in a DSP system.

(OR)

23.	Consider the IIR filter,

y[n]=x[n]+ay[n−1]where x[n]=12δ[n]and a=−12.

Illustrate the limit cycle behavior assuming b = 3 bits excluding the sign bit.

24.	Draw the block diagram of the Von Neumann and Harvard architecture and explain the difference between the two for a computer. Which architecture is preferred for DSP applications and why?

(OR)

25.	a.	Draw the block diagram of VLIW architecture and explain the same.				(8)

b.	Write short notes on pipelining.												(7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	C++ AND DATA STRUCTURES						      	     Time : 3 hours

Subject Code:	EE261											      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is called a Stack?

2.	Draw a binary Tree for the expression :   A * B  (C + D) * (P / Q).

3.	What is called Heap?

4.	Compare Binary search and Linear search.

5.	What is called a Constructor?

6.	What is an Array?

7.	What is Inheritance?

8.	List out the uses of Pointers.

9.	What is called Dynamic binding?

10.	What are Manipulators?

PART – B  (5 x 3 = 15 MARKS)

11.	How do you delete an element from a Queue?

12.	What are the differences between Selection Sort and Insertion Sort?

13.	Write notes on Overloaded Constructors.

14.	How do you overload the unary ++ operator? Give an example.

15.	Write notes on ‘this’ pointer.

PART – C  (5 x 15 = 75 MARKS)

16.	Demonstrate how a node can be inserted, deleted using Singly Linked List. Explain briefly.

(OR)

17.	Explain the Binary Tree Traversal techniques with a suitable example.

18.	Discuss briefly about the Quick Sort. Sort the following elements.

26, 5, 37, 1, 61, 11, 59, 15, 48, 19

(OR)

19.	Illustrate briefly about the Binary Tree Search technique with suitable example.

20.	a. 	How do you pass objects as function arguments? Give a suitable example.        		(10)

b. 	Explain briefly about the Static Class data.				           					(5)

(OR)

21. 	a.	Write a C++ program to create a user defined String Data type.	 	           		(10)

b. 	Explain briefly about the Copy Constructors.										(5)

22.	Explain briefly about the multiple inheritance with an example.

(OR)

23.	a. 	Explain the use of new and delete operator.				            					(5)

b. 	Write a C++ program to sort person objects using array of pointers.    	          		(10)

24.	Explain briefly about the friend function with an example.

(OR)

25. 	a.	What is called Template? Write a function template used for absolute value function?(10)

b. 	Write notes C++ Exception Handling Keywords.									(5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title:	MEASUREMENTS AND INSTRUMENTATION			       Time: 3 hours

Subject Code:	EE262				       						              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	__________ is defined as a device for determining the value or magnitude of a quantity or variable.

2. 	State any two disadvantages of PMMC instruments.

3. 	__________ is the process of comparing the instrument with a standard to correct the accuracy.

4.	Write down the deflecting torque equation in dynamometer type wattmeter.

5.	Name the device used to measure medium resistance.

6.	What is the use of Hay’s bridge?

7. 	List any two applications of LVDT.

8.	Name any two devices used to measure high pressure.

9.	List two advantages of digital multimeter over analog multimeter.

10.	What is a recorder?

PART – B (5 x 3 = 15 MARKS)

11. 	What are the different types of standard of measurement?

12. 	List the classification of Errors.

13. 	How the earth resistance is measured? Give the working principle.

14. 	Define Transducer. List the classification of Transducers.

15. 	Compare Strip chart and X-Y recorders.

PART – C (5 x 15 = 75 MARKS)

16. 	With a neat diagram explain the construction, working, torque equation and advantages, disadvantages of PMMC instrument.

(OR)

17. 	a.	Explain D’Arsonval Galvanometer with a neat sketch.			       				(8)

b.	Briefly explain Dynamometer type moving coil Instruments.		  	       			(7)

18. 	Explain how the power is measured using Instrument Transformers.

(OR)

19.	a.	Explain the calibration of energy meters with a neat sketch.		       				(8)

b.	Briefly explain power factor meter.	    											(7)

20. 	a.	Explain Kelvin’s double bridge method for the measurement of low resistance.        	(8)

b.	Describe how inductance is measured by using Maxwell’s bridge.     		       		(7)

(OR)

21.	a.	Explain the working principle of Anderson’s bridge and also derive its balance equations. 																				(8)

b.	Describe the working principle of Schering Bridge and also derive its balance equations.                            					      															(7)

[P.T.O]

22.	a.	How are transducers classified on the basis of principle of operation?	   	   		(5)

b.	Describe the different modes of operation of piezo-electric transducers.                   	(10)

(OR)

23. 	a.	How is low pressure measured? Explain any one type of low pressure measurement with 	a 	neat sketch.								       									(8)

b.	What is Thermocouple? Explain its construction and working principle with a neat sketch. 									       												(7)

24. 	Describe a Spectrum analyzer with a neat sketch.

(OR)

25. 	With a neat sketch, explain the construction and working principles of Strip chart and X-Y recorders.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	MICROPROCESSORS AND MICROCONTROLLERS   	     Time : 3 hours

Subject Code:	EE263										   	      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Flag register and accumulator together are called as ___________.

2.	The instruction size of LXI H, 16 bit is __________.

3.	Bidirectional buffers are used for__________ bus.

4.	8255 is a __________ interface.

5.	_____________ is a non maskable interrupt.

6.	On receiving _________ signal from a microprocessor, DMA goes into master mode.

7.	D/A and A/D are called as______________.

8.	Define conversion time.

9.	8051 has __________ number of register banks.

10.	Name a 16 bit register in an 8051 microcontroller.

PART – B  (5 x 3 = 15 MARKS)

11.	Write an Assembly level language program to add two 8 bit numbers.

12.	Name the various machine cycles of 8085.

13.	List the various types of hardware interrupts available in 8085.

14.	Draw the R/2R Ladder network D/A convertor.

15.	Explain the instruction DJNZ R0 16 bit Address.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain about the Architecture of 8085 with a neat functional block diagram.

(OR)

17.	Write an assembly level language program to find the smallest of five - 8 bit numbers.

18.	Explain the operation of the instruction IN C0H with timing diagram.

(OR)

19.	Explain about the need of 8255 Programmable peripheral interface and its various modes of operation.

20.	Discuss about the various interrupts available in 8085 and what is the need for the interrupt?

(OR)

21.	What is the role of DMA? With a neat diagram explain the procedure for interfacing the DMA controller with 8085.

22.	With a neat sketch explain about the multiplexed seven segment display system.

(OR)

23.	Describe with a neat sketch the working of ADC0801 A/D Converter.

24.	Explain with examples, the various addressing modes of 8051.

(OR)

25.	a. 	Discuss about the various instructions in the instruction set of 8051.				(8)

b. 	Explain about the Counters and Timers of 8051.									(7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	MOBILE COMMUNICATION						      	     Time : 3 hours

Subject Code:	EE264									   		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give any one need for modulation.

2.	Give one advantage of spread spectrum.

3.	What is the multiple access scheme used in 2G GSM?

4.	Give any one application of TETRA.

5.	What is an ad-hoc network?

6.	Give one application of Bluetooth.

7.	List the working groups in WATM.

8.	What is the need for Mobile IP?

9.	What is advantage of selective retransmission?

10.	Give one application of WAP.

PART – B  (5 x 3 = 15 MARKS)

11.	Comment on the nature of signal propagation in wireless transmission.

12.	Compare MEO and LEO.

13.	Distinguish Infrared vs. Radio Transmission.

14.	What is DHCP? Explain.

15.	Write a brief note on World Wide Web.

PART – C  (5 x 15 = 75 MARKS)

16. 	With a neat block diagram explain Direct Sequence Spread Spectrum System.

(OR)

17. 	Distinguish FDMA and TDMA.

18. 	With a neat block diagram explain about the DECT architecture.

(OR)

19. 	Explain the GSM channels and services offered by GSM.

20. 	Discuss about the MAC frames in IEEE 802.11.

(OR)

21.	Explain in detail the architecture and radio aspects of HiperLAN.

22. 	Describe the WATM Functions and Radio Access layer.

(OR)

23.	Explain handover, addressing and location management in WATM.

24.	Explain in detail about Indirect and Snooping TCP.

(OR)

25. 	Write notes on:

a.	Fast Retransmission/Fast Recovery.

b.	File system.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	BIOMEDICAL INSTRUMENTATION				      	     Time : 3 hours

Subject Code:	EE265											      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define Cell.

2. 	Define Neuron.

3. 	What is Arrhythmia?

4. 	Mention any 2 widely used Blood Flow meters.

5. 	Define Diathermy.

6. 	What is the most commonly used Anesthetic?

7. 	Define Micro shock.

8. 	Define Macro shock.

9. 	Define Ultrasonography.

10. 	What is the efficiency of X-Ray machine?

PART – B  (5 x 3 = 15 MARKS)

11. 	Mention the different systems of the Human body.

12. 	Define Oximeter. Mention its types.

13. 	What is an External Defibrillator? List out its types.

14. 	Define Chromatography. Mention its types.

15. 	Mention a few applications of Computer Tomography.

PART – C  (5 x 15 = 75 MARKS)

16. 	With a neat block diagram, explain the working of an ECG Machine.

(OR)

17.	Describe the Recording set up used in EMG.

18. 	Discuss about Cardiac Output measurement with its types. List out its advantages.

(OR)

19.	What are Oximeters? Describe any one of the Oximeters types with a suitable diagram.

20. 	Describe the block diagram of Anaesthesia machine and explain the different components in it.

(OR)

21.	Differentiate between External Pacemaker and Implanted Pacemaker. List out the different 	modes of operation.

22. 	Describe a Flame Photometer with a suitable diagram.

(OR)

23.	Briefly mention the Physiological effects of currents at commercial frequencies.

24. 	Explain about the MRI system with a neat block diagram.

(OR)

25.	Briefly mention the different elements involved in Bio-Telemetry. Mention its uses.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	OPERATING SYSTEMS							      	     Time : 3 hours

Subject Code:	EE268							      			       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	How does a multitasking system work?

2.   	State the advantages of multiprocessor systems.

3.   	What is paging?

4.   	Why are page sizes always a power of 2?

5.   	What is meant by context switch?

6.   	Define the term ‘safe state’.

7.   	What is FCB?

8.   	What is seek time?

9.   	List down the components of a Linux system.

10.  	What is meant by a portable operating system?

PART – B  (5 x 3 = 15 MARKS)

11.	Mention the features of real time systems.

12.	Compare fixed with variable partition methods.

13.	Write briefly about monitors.

14.	What do you know about free-behind and read-ahead optimization techniques?

15.	State the features of ‘signals’ in Unix.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe the features of mainframe systems in detail.

(OR)

17.	Write notes on the following:

a.	Distributed systems.     														(8)		b.	Clustered systems.    															(7)

18.	a.	Explain first-fit, best-fit and worst-fit allocation algorithms.    						(6)

b.	Given memory partitions of 100K, 500K, 200K, 300K and 600K (in order), how would each of the algorithms (first-fit, best-fit, worst-fit)  in parts a, b and c place processes of 212K, 417K, 112K and 426K(in order)? Which algorithm makes the best use of memory? 																			(9)

(OR)

19.	a.	Illustrate the concept of demand paging.      										(6)

b.	Consider the following page-reference string: 									(9)

1, 2, 3, 4, 2, 1, 5, 6, 1, 2, 2, 3, 7, 6, 3, 2, 1, 2, 3, 6

How many page faults would occur for the following replacement algorithms assuming 	one, two, three, or four frames?

a.	LRU     	b.	FIFO      c.	optimal

[P.T.O.]

20.	Discuss any 3 CPU scheduling algorithms with a suitable example.

(OR)

21.	Describe about any 3 concurrency control algorithms.

22.	Explain linked and indexed disk space allocation methods in detail.

(OR)

23.	Explain the various disk scheduling methods.

24.	How is process management implemented in Linux? Explain.

(OR)

25.	Describe the architecture of Windows 2000 system with a neat sketch.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	COMPUTER COMMUNICATION					      	     Time : 3 hours

Subject Code:	EE269									   		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Mention the Network support layers.

2.	What is the access method in Ethernet?

3.	Name any one error detection and correction code.

4.	What is the function of a repeater?

5.	What are the protocols in the transport layer?

6.	What is ATM?

7.	What are the services provided by ISDN?

8.	Mention the configurations in HDLC.

9.	Define topology.

10.	What is queuing?

PART – B  (5 x 3 = 15 MARKS)

11.	What is a protocol? What are its functions?

12.	Assume six devices are arranged in a mesh topology. How many cables are needed? How many ports are needed for each device?

13.	Draw the PLP packet format in X.25.

14.	Differentiate router and gateway.

15.	Mention the ATM layers.

PART – C  (5 x 15 = 75 MARKS)

16.	With a neat diagram explain the responsibilities of OSI model.

(OR)

17.	A message is broken up into three pieces. Discuss the transmission of packets through different approaches in packet switching.

18.	Explain the types of contention schemes in LAN access techniques.

(OR)

19.	Discuss in detail about IEEE 802.5 with neat diagrams.

20.	a.	Given a remainder of 111, a data unit of 10110011, and a divisor of 1001, is there any error 	in data unit when you use CRC method?											(10)

b.	Find LRC for the following block of data 100110001, 011011111.  				(5)

(OR)

21.	a.	Find checksum for 1001001110010011, 1001100001001101.						(5)

b.	Explain the frame formats of HDLC. 											(10)

22. 	a.	Discuss the TCP segment in detail. 												(10)

b.	Find the sub network address for the following IP address 140.11.36.22; Mask 					255.255.255.0. 																(5)

(OR)

[P.T.O]

23. 	Discuss about Distance Vector and Link State routing with an example.

24. 	Write in detail about the subscriber access to ISDN.

(OR)

25.	Explain the architecture of the ATM network.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	VIRTUAL INSTRUMENTATION						      Time : 3 hours

Subject Code:	EE270							      	       			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the colour coding for a cluster data type?

2.	What are the contents of a block diagram in a VI?

3.	List the types of sequence structures.

4.	What is the significance of connector pane?

5.	Why is a PGA required in DAQ systems?

6.	List two advantages of transferring data by DMA technique.

7.	What does VISA stand for? List the hardware interfaces it can handle.

8.	Which of the two is preferred to connect 10 external devices to a PC? RS232C or GPIB.

9.	Define cut off frequency in filters.

10.	List the VIs available for signal processing

PART – B  (5 x 3 = 15 MARKS)

11.	Bring out the differences between data flow programming and text based programming.

12.	How is a global variable created? Is using a global variable recommended while programming? Why?

13.	What is the need for timers in a DAQ system? Give the scheme for measurement of frequency using timers.

14.	Why is the use of USB more convenient for data acquisition?

15.	What is spectral leakage? How can it be minimized?

PART – C  (5 x 15 = 75 MARKS)

16.	Give the block diagram of a virtual instrument and discuss about the wide spectrum of applications that can be done through virtual instrumentation.

(OR)

17.	Draw and explain the general functional description of a conventional digital instrument. Compare it with a virtual instrument and list the advantages.

18.	Discuss in detail about the FOR loop and WHILE loop structures. How can an array of data be created using a FOR loop. Explain the concept with an example VI.

(OR)

19.	When are sequence structures used? Give its general format and explain its operation with an example VI.

20.	Explain the working of a Successive approximation ADC. Discuss about the specifications of ADCs and issues related to sampling.

(OR)

21.	Explain how data transfer takes place through a DMA controller. Also explain the DMA bus cycles with a timing diagram.

[P.T.O.]

22.	Give the structure and configurations of the GPIB interface and discuss the same with the associated signals and pin details.

(OR)

23.	With a neat sketch detail on the working of the universal serial bus for data transfer.

24.	Detail on the various signal processing VIs.

(OR)

25.	Construct a VI for a temperature monitoring and control system and explain the same.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	COMPUTER ARCHITECTURE						      	     Time : 3 hours

Subject Code:	EE272											      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Instructions.

2.	What is meant by Interrupt?

3.	Draw the general register organization unit.

4.	Write the instruction format of Berkeley RISC 1.

5.	Define ALU.

6.	Give the polynomial form of decimal numbers.

7.	List out any two functions to be performed by an I/O processor.

8.	What is meant by Cache?

9.	Expand SISD and MISD.

10.	_________ transfer is very useful in many microcomputer system applications.

PART – B  (5 x 3 = 15 MARKS)

11.	List out any five Arithmetic logic and any five shift micro operations.

12.	Differentiate between RISC and CISC Characteristics.

13.	What are the steps involved in the design of an ALU.

14.	Enumerate the Asynchronous data transfer Scheme.

15.	Write the special features of uniprocessor systems.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the timing and Control unit with a neat diagram.

(OR)

17.	Describe the memory sequence instructions with examples.

18.	Give the organization of micro programmed control unit and explain its operation.

(OR)

19.	Explain the various addressing modes with example instructions. Also discuss the importance of each of the addressing mode.

20.	A computer has floating-point arithmetic instructions. But the CPU hardware does not have the floating point ALU. How should the system handle floating- point arithmetic. Also discuss micro operations.

(OR)

21.	Explain the following. 						  								(5+5+5)

a.	Booth Multiplication  		b.	Restoring Division 	c.	Non-Restoring Division

22.	What is the need for a DMA transfer? Explain how DMA operation takes place.

(OR)

23.	Discuss the concept on Auxiliary memory and Associative memory.

24.	With a neat sketch, explain the modes of memory access.

(OR)

25.	With a schematic diagram, discuss the shared memory and distributed memory.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	POWER SYSTEM CONTROL							      Time : 3 hours

Subject Code:	EE275			       						                     Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Electric power system.

2.	What are different methods of voltage control?

3.	Define Load Frequency Control.

4.	What is the necessity of frequency bias tie line control?

5.	What is the objective of power factor correction?

6.	What are the types of feeder regulators?

7.	Expand SCADA.

8.	Why do we need SCADA in Power Systems?

9.	Define incremental heat rate.

10.	Define incremental fuel cost curve.

PART – B  (5 x 3 = 15 MARKS)

11.	Write short notes on P-F and Q-V control loops.

12.	What are the limitations of Prime Mover systems?

13.	Write short notes on synchronous Condensers.

14.	What is the role of security monitoring and control in power systems?

15.	Write short notes on Incremental cost curve.

PART – C  (5 x 15 = 75 MARKS)

16.	Briefly explain the need for voltage and frequency regulation in a power system.

(OR)

17.	Explain any three methods of voltage control.

18.	Explain the characteristics of Automatic Generation Control.

(OR)

19.	A small system consists of 4 identical 500 MVA generating units feeding a total load of 1020 MW. The inertia constant H of each unit is 5 on 500 MVA base. The load varies by 1.5% for a 1% change in frequency. When there is a sudden drop in load by 20 MW:

a.	Determine the system block diagram with constants H and D expressed on 2000 MVA 	base.

b.	Find the frequency deviation, assuming that there is no speed governing action.

20.	Explain the operation of SVC in detail.

(OR)

21.	Explain the principle of operation and Dynamic response of TSC.

22.	What are the main functions of Energy control centers? Explain.

(OR)

23.	With a neat block diagram explain the SCADA hardware configuration.

24.	Discuss briefly about Base point and Participation factors.

(OR)

25.	Explain dispatch control including Transmission loss and derive the coordination equation including 	losses.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	MATERIAL SCIENCE								      	     Time : 3 hours

Subject Code:	EE276						      				       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List the two categories of solids based on the arrangement of atoms or molecules.

2.	The total mean free time of electrons for impure metals is made up of two components. What are they?

3.	List the two types of semiconductors classified based on the concentration of electrons and holes in the material.

4.	The display device which modulates the incident radiations to provide the necessary display information is known as ______________ device.

5.	Liquid crystal displays must be used in ______________ areas.

6.	The intrinsic breakdown of dielectrics is also known as ______________ breakdown.

7.	Any two opposite poles separated by a distance constitute a ______________.

8.	List any two materials used in storage media.

9.	Nanophase materials are materials with grain sizes at ______________.

10.	The resistivity of mercury suddenly dropped to zero at ______________ degree Kelvin.

PART – B  (5 x 3 = 15 MARKS)

11.	State the important features of Miller Indices.

12.	What is Hall Effect? Mention any two applications of it.

13.	Mention the different types of polarization.

14.	List the properties of diamagnetic materials.

15.	State Josephson Effect. Also give the expression for the frequency of the ac components.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail, Debye-Scherrer method of determination of crystal structure.

(OR)

17.	Explain in detail, Wiedemann-Frenz law. Also mention the advantages and drawbacks of classical free electron theory.

18.	Deduce the mathematical expression for the concentration of electrons and holes respectively in the conduction band and valence band for an intrinsic semiconductor and hence show that the Fermi level lies at the middle of an intrinsic semiconductor.

(OR)

19.	Explain the experimental technique used to determine Hall Effect voltage and Hall coefficient.

20.	Discuss in detail, the effect of temperature and frequency on dielectric properties.

(OR)

21.	Explain in detail, the classifications of electrical insulating materials on the basis of temperature. Give few examples for each class.

22.	Explain in detail, Heisenberg and Domain Theory of Ferromagnetism.

(OR)

[P.T.O.]

23.	Explain in detail, the following applications of magnetic materials:

Floppy disks and Magnetic optical recording.

24.	Describe in detail, the characteristics and application of Shape Memory Alloys.

(OR)

25.	Define metallic glasses. Explain how metallic glasses are prepared and how are they different from their crystalline counterparts.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	ILLUMINATION ENGINEERING					      	     Time : 3 hours

Subject Code:	EE293									   		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Radiation of only one wavelength is called _____________.

2.	The energy in the form of light waves radiated per second from a luminous body is called _____________.

3.	_____________ is the most common material used as filament.

4.	Luminance is classified as _____________ and _____________.

5.	Polar curve for vertical plane is equivalent to _____________ of any object.

6.	Inverse square law holds good only if the source is a _____________ source.

7.	Space/Height ratio is given by _____________.

8.	In industries _____________ type of lighting scheme is adopted.

9.	High ratings of _____________ and _____________ lamps are employed for flood lighting.

10.	_____________ control is one of the important criteria required to provide good illumination.

PART – B  (5 x 3 = 15 MARKS)

11.	A room 20m x 10m is to be illuminated by eight lamps and the average illumination is to be 50 lumens/m2. If the utilisation factor is 0.48 and depreciation factor 1.2, calculate the mean spherical power per lamp.

12.	When does the life of sodium discharge lamp end?

13.	A lamp giving 300 c.p in all directions below the horizontal is suspended 2 m above the centre of a square table of 1 m side. Calculate the maximum and minimum illumination on the surface of the table.

14.	Define utilization factor. Write the factors which decide this factor.

15.	What is flood lighting?

PART – C  (5 x 15 = 75 MARKS)

16.	What do you mean by photometry? Mention the four factors that affect illumination and discuss how you measure these factors.

(OR)

17.	a.	Define the following:

i.	Luminous flux 	ii.	Luminous intensity 	iii.	Degree of illumination

iv.	Lumen     																(8)

b.	What do you mean by solid angle?  Obtain the relationship between solid angle and plane 	angle.                                                                                                                     		(7)

18.	a.	Compare the filament lamp and fluorescent tube.                                                      	(7)

b.	Give the construction and working of arc lamps.                                                     	(8)

(OR)

19.	Describe about the two types of LCDs. Mention the advantages of LCDs.

[P.T.O.]

20. 	a.	Explain the laws of illumination.                                                                                 	(8)

b.	What is the effect of voltage variation on light output and life of the lamp?			(7)

(OR)

21. 	a.	Two lamps of 200 candle power and 400 candle power are arranged as shown in the figure.
   		Calculate the illumination in the middle of the lamps. 							(7)

b.	What do you understand by polar curves as applied to light sources?                         	(8)

22. 	a.	Enumerate the various factors which have to be considered while designing factory
     		lighting?                                                                                                                           (8)

b.	It is required to provide an illumination of 100 m. candle  in a factory hall 40 m x10 m.
     		Assume that the depreciation factor is 0.8, co-efficient of utilization is 0.4 and efficiency
     		of lamp is 14 lumens per watt.  Calculate the number of lamps and their disposition.(7)

(OR)

23. 	a.	Discuss about the factors which are considered while designing any lighting scheme.	(7)
   	b.	A drawing hall 30 m by 13 m with a ceiling height of 5 m is to be provided with a general 		illumination of 120 lux.  Taking a coefficient of utilization of 0.5 and depreciation factor of 		1.4, determine the number of fluorescent tubes required, their spacing mounting height and 		total wattage.  Taking luminous efficiency of fluorescent tube as 40 lumens/watt for 80 			watt tube.                                                                      								(8)

24.  	a.	The front of a building is to be flood lighted by means of projectors placed at a distance
     		of 8m from the wall.  The dimensions of the front portion are 80 m x 10 m.  The average 
     		illumination required is 50 m candle. Determine the number and size of projectors 
     		required.  Assume that waste light factor is 1.2, depreciation factor 1.3 and coefficient of 
     		utilization 0.4.  Given that :

Wattage		: 	300  	500      1000

Lumens   	: 	5000    	9000    18000										(8)

b.	Write the lighting design procedure for street lighting.                                             	(7)

(OR)

25. 	Write short notes on the following:

a.	Aviation lighting    	b.	Transport lighting

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	AUTOMOTIVE ELECTRONICS						      Time : 3 hours

Subject Code:	EE294							      	       			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.  	What is an oxygen sensor?

2.	Name the phenomenon in which the resistance of the sensing element changes in proportion to the applied manifold pressure.

3.  	Why is a series motor used as a starter motor?

4.  	What is the purpose of a starter?

5.  	List the functions of a carburetor.

6.  	What is the advantage of electronic ignition system?

7.  	What is the significance of earthing?

8.   	Changing a car’s ___________ can throw off a speedometer’s accuracy.

9.  	What is a throttle angle?

10.	What is the purpose of using acceleration enrichment?

PART – B  (5 x 3 = 15 MARKS)

11.  	What is the function of throttle position sensor?

12.  	Explain the importance of starter switches.

13.  	List the variables that will be affected by ignition timing.

14.  	Write the principles of wiper system.

15.  	Write short notes on Antilock Braking system.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.  	Draw the construction, principle of operation and signal conditioning circuit of Air flow 	sensor.

b. 	Describe how the crank shaft position is sensed by an optical sensor.

(OR)

17. 	a. 	Describe the importance of actuators and sensors for an automotive control system with a 	suitable example.

b. 	Explain in detail, how the Exhaust Gas Oxygen sensor is working for different types of 			air/fuel ratio mixture concentrations.

18. 	a. 	What are the requirements of an engine starting system?

b. 	Describe the working of self starter with solenoid switch.

(OR)

19.  Explain the operation and different parts of starter motor with neat sketches.

20.  Explain the principle of operation of fuel injection with a neat diagram.

(OR)

21.   What are the requirements of a spark plug? How does the spark plug work in the contact less 		electronic ignition system?

[P.T.O.]

22. 	a.  	Write the principle of operation of electrical fuel pump with a neat diagram.

b. 	Describe the functions of oil and temperature gauges.

(OR)

23.	Discuss the following:

a.   Different types of lights used in car.

b.   Head light reflector.

c.   Positive and negative earthing systems.

24.  With neat sketches, explain principle of operation of electronically controlled electric power 	steering and four wheel steering in an automotive electrical system.

(OR)

25.  Explain in detail about:

a.	Integrated engine control system.

b.	Electronically controlled electric brakes.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/ December - 2013

Subject Title : 	NETWORK ANALYSIS AND SYNTHESIS			      	     Time : 3 hours

Subject Code:	EE295											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the difference between one port network and two port network?

2.	What is the significance of pole and zeros of network functions?

3.	What is meant by frequency response plot?

4.	Frequency response plot can be obtained by __________ method and __________ method.

5.	What is planar graph?

6.	The branches removed from the graph in forming tree are called __________.

7.	How will you find the Y-parameters from the Z-parameters of a two-port network?

8.	How are filters classified?

9.	The poles and zeros of the RL driving point impedance are located on the ___________ of the S-plane.

10.	In the first Foster form of LC network, the presence of first element capacitor C0 indicates __________ at ω = 0.

PART – B  (5 x 3 = 15 MARKS)

11.	Define the terms ‘driving point impedance’ and ‘transfer impedance’ of a network.

12.	State the importance of magnitude and phase of the network function in the network design.

13.	Distinguish between trees and co-trees of a graph.

14.	What is m-derived filter? What is its advantage over a constant K-filter?

15.	What are the conditions to be fulfilled for a function to be positive real?

PART – C  (5 x 15 = 75 MARKS)

16. 	Determine the driving point admittance of the network shown in figure.

(OR)

17.	The transform current I(s) of a network is given by I(s)=2s(s+1)(s+2). 	Plot pole-zero 	pattern in the S-plane and hence obtain the time domain response i(t).

18.	Draw the magnitude and phase angle plot of V2(s)/ V1(s) as ω is varied from 0 to ∞ for the RC network shown in the figure.

[P.T.O]

(OR)

19.	Draw the complex locus plot of voltage transfer function G21(s) of the transform RC network shown in the figure.

20. 	Determine the Tie-set matrix for the graph shown in the figure.

(OR)

21. 	What is a dual network? Explain with suitable example the principle of duality in electrical network.

22. 	Determine the h-parameters for the network shown in the figure.

(OR)

23.	Design a m-derived low pass filter (both T and π sections) having cut-off frequency of 1KHz, design impedance of 400Ω and the resonant frequency 1100Hz.

24.	Test whether the following polynomials are Hurwitz:

a.	s4+3s2+2

b.	s3+4s2+5s+2

(OR)

25.	The driving point impedance of a one-port reactive network is given by

Z(s)=5(s2+4)(s2+25)s(s2+16). Find the first and second Foster forms of the LC network .