Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	BASIC CIVIL ENGINEERING						      	     Time : 3 hours

Subject Code:	12CE101						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	“Civil Engineering is associated with Society and Civil life” Give some examples.

2.	Define: Environment Impact Assessment (EIA).

3.	Write down the different bench marks used in surveying.

4.	What do you understand about ‘Hooke’s law’?

5.	Mention the dimensions of a standard or metric brick.

6.	Recommend a suitable grade of concrete for RCC.

7.	Give three examples for public building.

8.	What is ATC?

9.	Define: Penstock.

10.	What should be the pH value for drinking water?

PART – B  (5 x 3 = 15 MARKS)

11.	What are the advantages of Infrastructural development?

12.	Mention about the basic principles of surveying.

13.	Write down briefly on pozzolana cement.

14.	Differentiate between port and harbour.

15.	What are Clariflocculators?

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss in detail about Impact of infrastructure development on socio-economic status of the nation and civilization.

(OR)

17.	Make a detailed note on scope of civil engineering.

18.	Classify the surveying based on all the aspects.

(OR)

19.	a.	Derive the formula for computing the areas of irregular shapes by using Trapezoidal rule 	and Simpson’s rule								           						    (9)

b.	The perpendicular offsets taken at 10 m intervals from a survey line to an irregular boundary are 2.18 m, 3.2 m, 4.26 m, 6.2 m, 4.8 m, 7.20 m, 8.8 m, 8.2 m and 5.2 m. Determine the area enclosed between the boundary, survey line, the first and the last offsets by (a) Trapezoidal rule (b) Simpson’s rule.                        		           		    (6)

20.	a.	Discuss the characteristics of good building stones.				           			    (8)

b.	Explain the following test procedure on cement:				  					(4+3)

i .	Soundness test.

ii.	Crushing strength test.

(OR)

[P.T.O.]

21.	a.	Explain different types of bricks.        											    (8)

b.	Discuss on the different types of paints.                  								    (7)

22.	a.	Draw a neat sketches of cross section of a two lane divided highway in an urban area.

b.	Bring out the components permanent way and its ideal requirements.    				(7+8)

(OR)

23.	a.	Briefly explain about the airport Master plan.                                                    		    (9)

b.	Write short notes on the following:

(i)	Jetty   (ii)   Dolphins                                       								(3+3)

24.	Discuss briefly about various irrigation methods adopted in India.

(OR)

25.	Explain the various components of waste water treatment process.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FLUID MECHANICS AND MACHINERY			      	     Time : 3 hours

Subject Code:	30								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.  	Distinguish between ideal fluid and real fluid.

2.  	Define Capillarity.

3.  	Define path line.

4.  	Write the continuity equation for one dimensional flow.

5.  	List any two types of the minor losses.

6.  	What is a venturimeter?

7.  	Define slip in a reciprocating pump.

8.   	Define impulse momentum principle.

9.   	What is the basic classification of Turbine?

10. 	List the various types of miscellaneous pumps

PART – B  (5 x 3 = 15 MARKS)

11. 	One litre of oil at 40˚c temperature has mass 900 gms. Calculate the mass density,  specific weight , and specific gravity of oil .

12. 	What is velocity potential function list the properties of velocity potential function?

13. 	Derive an expression for energy loss due to expansion.

14. 	Explain briefly the performance characteristics curves for centrifugal pumps.

15. 	Explain briefly the classification of turbines	.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	What is capillarity? Derive expression for height of capillary rise.

b.	A plate having area of 0.6 m2 sliding down the inclined plane at 30° to the horizontal with a velocity of 0.36 m/s. There is a cushion of fluid 1.8mm thick between the plane and the plate. Find the viscosity of the fluid if the weight of the plate is 280N.

(OR)

17.	a.	What is a manometer? Explain briefly the various types of manometer.

b.	An annular plate 2m external diameter and 1m internal diameter with its greatest and least   depth below the surface being 1.5m and 0.75m respectively. Calculate the magnitude, direction and location of the force acting upon one side of the plate due to water pressure.

18.	a.	Explain briefly the various types of flow.

b.	In a 2 dimensional incompressible flow. The fluid velocity component are given by

u = x–4y and v = –y – 4x. Show that velocity potential exists and find its stream function .

(OR)

19.	a.	Derive Euler’s Equation and hence deduces Bernoulli’s Equation.

b.	A pipe line gravity 0.87, changes in diameter from 200mm diameter at a position A to     500 mm   diameter at a position B, which is 4m at higher level of the pressure at A and B is 9.81x104 n/m2 and 5.86 x 104 n/m2 and the discharge is 200lit/s. Determine  the loss of head and direction of flow.

[P.T.O.]

20.	a.	Derive the Darcy’s equation.

b.	A Pipeline of 600 mm diameter is 1.5 km long. To increase the discharge another line of the same diameter is introduced parallel to the first in the second-half of the length. If           f = 0.01 and  head at inlet is 300mm calculate the increase in discharge.

(OR)

21.	a.	Derive an expression for discharge over a rectangular notch?

b.	Determine the height of rectangular weir of length 6m to be built across a rectangular channel. The maximum depth of water on the upstream side of the weir is 1.8m and discharge is 2000 lit/sec. Take Cd=0.6 and neglect end contraction.

22.	a.	List the factors for selection of pump.

b.	A double acting reciprocating pump, running at 40 rpm is discharge 1.0m3 of water/minute, the pump has a stroke of 400mm.the diameter of piston in 200mm. The delivery and suction head are 20m and 15m respectively , and the slip of the pump and lower required to derive the pump.

(OR)

23.	a.	Explanation of ideal indicator diagram with a neat sketch.

b.	A centrifugal pump is to discharge 0.118m3/s at a speed of 1450rpm against a head of 25m.     the impeller diameter is 250mm, its width outlet is 50mm and Manometric efficiency is 75%.  Determine the vane angle at the outer periphery of the impeller.

24.	The following data is given for a Francis turbine .Net head H=60m , speed N=700rpm , Shaft power=294.3KN. ηa=84%, ηh=93% flow ratio =0.2, breadth ratio η=0.1 outer diameter of runner =2 inner diameter of runner . The thickness of vane occupies 5% of circumferential area of runner. Velocity of is constant at inlet and outlet and discharge is radial at outlet. Determine a) Guide blade angle  b) runner vane angle at inlet and outlet , c) diameter of runner inlet and outlet iv) width of wheel at inlet .

(OR)

25.	A Pelton wheel is receiving water from a penstock with a cross head of 510m. one-third of cross head is lost in friction in the penstock. The rate of flow through the nozzle fitted at the end of the penstock is 2.2m3/s. The angle of deflection of the jet is 165°. Determine

a.	the power given by water to the runner

b.	hydraulic efficiency of the pelton wheel.

Take cv (coefficient of velocity) = 1.0 and speed ratio = 0.45.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	SURVEY											      	     Time : 3 hours

Subject Code:	12CE201						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define a contour.

2.	Write the types of leveling.

3.	What are accidental errors?

4.	Write stadia constants.

5.	What is subtense bar?

6.	Write the types of tachometric measurements

7.	Define lead lines.

8.	What is the use of a curve?

9.	List the types of signals.

10.	Write the types of triangulation system.

PART – B  (5 x 3 = 15 MARKS)

11.	List the temporary adjustments of a dumpy level.

12.	What are omitted measurements?

13.	Explain tachometric constants.

14.	List the types of curves.

15.	Write the uses of total station.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain with sketches the characteristics of contours and its uses.

(OR)

17.	a.	Explain in detail reciprocal leveling.											  (10)

b.	Discuss closing error, with a diagram.											    (5)

18.	a.	Explain the temporary and permanent adjustments of a theodolite.	       				  (10)

b.	Illustrate the Transit rule.														   (5)

(OR)

19.	a.	Explain the measurement of horizontal angles by repetition method.	  			    (7)

b.	Write the salient components of transit theodolite.								    (8)

20.	Two sets of tachometric readings were taken from an instrument station A, the reduced level of which was 100.06m to a station B.

Instrument P  -  multiplying constant is 100, additive constant  is   0.06  staff held vertical

Instrument Q  - multiplying constant is 100, additive constant  is   0.06 staff held normal

| Instrument | At | To | Height of Inst | Vertical Angle | Staff Readings |
| --- | --- | --- | --- | --- | --- |
| P | A | B | 1.5m | 26o | 0.755, 1.005, 1.225 |
| Q | B | B | 1.45m | 26o | ? ? ? |

What should be the stadia readings with instrument Q

[P.T.O.]

(OR)

21.	a.	What are the advantages of tachometric survey?									    (5)

b.	Explain how you would determine the constants of a tachometer.					  (10)

22.	Explain in detail about the   various components of a simple curve.

(OR)

23.	List the various methods of locating the soundings. Explain any two of them in detail.

24.	A nominal distance of 30m was set out with a 30m steel tape from a mark on the top of another, 	the tape being in  catenary under a pull of 100 N and at a mean temperature of 700F . The top of 	one peg was 0.25m below the top of another. The top of the higher peg was 40m above the 	mean sea level, if the tape was standardized at a temperature 600F  in catenary  under a pull of

a) 80N b) 120N  c) 100N.    Take radius of earth as 6370 Km, density of the is 7.86 gm/Cm3,   sectional area is 0.08sq Cm. Coefficient of expansion is 6x10-6 per m, Young’s modulus is  2x107 N/Cm2.  Determine the following

i) Correction for slope ii) tension correction   iii)  Sag correction and Total corrections for all 	the cases.

(OR)

25.	a.	Explain the various types of signals in detail.										    (8)

b.	Explain the different methods used for baseline measurement.						    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MECHANICS OF SOLIDS							      	     Time : 3 hours

Subject Code:	12CE202						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give the relationship between stress due to suddenly applied load and the stress due to gradually applied load.

2.	Define Poisson’s ratio.

3.	In a thin cylindrical vessel, the hoop stress developed in the walls is 50 N/mm2. Determine the longitudinal stress.

4.	The principal planes carry only___________ stresses.

5.	What is point of contraflexure?

6.	Give an example of indeterminate beam

7.	Draw the variation of shear stress in an I section.

8.	What is meant by flexural rigidity of beam?

9.	Give the equation of power transmitted by a solid circular shaft.

10.	Define the term stiffness.

PART – B  (5 x 3 = 15 MARKS)

11.	Find the modulus of rigidity of the material; the Young’s modulus and Poisson’s ratio are     1.18 x105 N/mm2 and 0.32 respectively.

12.	The principal stresses at a point in a material are 45 N/mm2 tensile and 60 N/mm2 compressive. 13.	Find out the maximum shear stress.

14.	Derive the relationship between rate of loading, shear force and bending moment

15.	Write the pure bending equation. What are the assumptions made in the theory of simple bending?

PART – C  (5 x 15 = 75 MARKS)

16.	A steel rod of 20 mm diameter passes centrally through a copper tube of 50 mm external diameter and 40 mm internal diameter. The tube is closed at each end by rigid plates of negligible thickness. If the temperature of the assembly is raised by 50 ͦ C, calculate the stresses developed in copper and steel. The coefficient of thermal expansions of steel and copper are 12x10-6 per  ͦ C and 18x10-6 per  ͦ C respectively. Take Es =200 GN/m2, ECu =100 GN/m2.

(OR)

17.	a.	A bar of 20mm diameter is tested in tension. It is observed that when a load of 37.7 kN is 	applied, the extension is measured over a gauge length of 200 mm is 0.12 mm and 	contraction in diameter is 0.0036mm. Determine the value of Poisson’s ratio and elastic 	constants E, G and K.															  (12)

b.	Draw the stress-strain diagram of mild steel and explain the salient points.			    (3)

18.	At a point within a body subjected to two mutually perpendicular stresses 80 N/mm2 tensile and 40 N/mm2 tensile. Each of these stresses is accompanied by a shear stress of 60 N/mm2. Determine the normal stress, shear stress and resultant stress on an oblique plane inclined at an angle 45 ͦ with the axis of minor tensile stress.

[P.T.O.]

(OR)

19.	A cylindrical vessel whose ends are closed by means of rigid flange plates is made up of steel plate 3mm thick. The internal length and diameter of the vessel are 50 cm and 25 cm respectively.  Determine the longitudinal and hoop stresses in the vessel due to an internal fluid pressure of 3 MN/m2.

Also calculate i) the change in diameter, ii) change in length and iii) change in volume of the vessel, if E= 200 GN/m2 and Poisson’s ratio is 0.3

20.	Draw the bending moment and shear force diagram for the given beam.

(OR)

21.	Draw the bending moment and shear force diagram for the given beam.

22.	A beam of length 8m is simply supported at the ends and carries a uniformly distributed load of 15 kN/m throughout the span of the beam. Draw the shear stress distribution at supports and bending stress distribution at mid span. The cross section of the beam is a rectangle with 150 mm wide and 300 mm deep.

(OR)

23.	A leaf spring is made of seven steel plates 65mm wide and 6.3mm thick. Determine the length of the spring so that it may carry a central load of 2750 N the stress being limited to 160 N/mm2. The Modulus of Elasticity E = 2 x 105 N/mm2.

24.	A closely coiled helical spring made of 1 cm diameter steel wire has 15 coils of 100 mm mean diameter. The spring is subjected to an axial load of 100 N. Calculate:

a.	The maximum shear stress induced.

b.	The deflection.

c.	Stiffness of the spring.

Take modulus of rigidity as 4.5 x 10 4 N/mm2.

(OR)

25.	A hollow shaft is to transmit 300 kW at 80 r.p.m. If the shear stress is not to exceed 60 MN/m2 and internal diameter is 0.6 of the external diameter, find the external and internal diameters assuming that the maximum torque is 1.4 times the mean.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ENGINEERING MECHANICS						      	     Time : 3 hours

Subject Code:	12CE203						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the expression for Newton’s second law?

2.	Write the value of acceleration due to gravity near the earth surface.

3.	Write any one equation of equilibrium.

4.	How do you call a beam having projection beyond the support?

5.	How a vector which is having a specified point of application is called?

6.	In method of joints what is the number of members at a joint with unknown member forces?

7.	What is the rate of change of displacement with respect to time?

8.	How the motion of a particle along a straight line called?

9.	What is the unit of power?

10.	Write the expression for kinetic energy.

PART – B  (5 x 3 = 15 MARKS)

11.	Define free body diagram.

12.	What is a resultant force?

13.	State Varignon’s theorem.

14.	What is a projectile?

15.	State the principle of conservation of momentum.

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the resultant force for the system of forces shown in figure.

(OR)

C

17.	State and prove Lami’s theorem.

18.	Find the reactions at supports A and B of the beam shown in figure.

(OR)

[P.T.O.]

19.	Determine the magnitude of the force P required just to start the motion of a block of weight 500 kN, up the plane inclined at 25 ̊ to the horizontal. What is the force P needed to keep it just moving up at a constant speed. The coefficient of static and dynamic friction are 0.3 and 0.25.

20.	Find the forces in all the members of the pin jointed truss shown in figure by the method of joints, indicating whether the forces are compressive or tensile.

(OR)

21. 	Find the position of centroid of the channel section shown in figure.

300 mm

12 mm

250 mm

12 mm

22.	A stone was thrown vertically upwards from the ground with a velocity of 55 m/sec. After 3 seconds, another stone was thrown vertically upwards from the same place. If both the stones strike the ground at the same time, find the velocity with which the second stone was thrown.

(OR)

23.	The motion of a particle is described by the equations x = 5t3 + 6t2 and y = 6.5t2 – 10t, where x and y are expressed in metres and t in seconds. Determine the velocity and acceleration

(a) when t = 4 sec and (b) when t = 6 sec.

24.	Ball A of mass 1 kg moving with a velocity of 2 m/sec strikes directly on a ball B of mass 2 kg at rest. The ball A after striking comes to rest. Find the velocity of ball B after striking and coefficient of restitution.

(OR)

25.	Briefly explain the following: (a) Coefficient of restitution, (b) Oblique impact, (c) Impulse momentum and (d) Newton’s second law of motion.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ENGINEERING GEOLOGY						      	     Time : 3 hours

Subject Code:	12CE204						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the composition and nature of the inner and outer core of the earth?

2.	List any two coastal protection structures.

3.	What are the symmetry elements of normal class of isometric system?

4.	Name any two mica group minerals.

5.	List any two textures of igneous rocks.

6.	What is dolerite?

7.	What is a normal fault?

8.	Write any two uses of geological maps.

9.	Write the application of seismic method of geophysical investigation.

10.	List the types of landslides.

PART – B  (5 x 3 = 15 MARKS)

11.	What do you mean by the term plate tectonics and its significance?

12.	List the physical properties of Garnet minerals.

13.	What are the agents of metamorphism?

14.	What is fold?

15.	What are the geological conditions that influence the construction of dams?

PART – C  (5 x 15 = 75 MARKS)

16.	Briefly explain about the branches of Geology.

(OR)

17.	Describe the various geological works of rivers.

18.	What are clay minerals? Discuss the types of clay and its engineering significance.

(OR)

19.	Describe the composition, properties, occurrence and uses of Quartz family.

20.	Write in detail about the types of metamorphism.

(OR)

21.	Briefly explain about the structure of sedimentary rocks.

22.	Discuss in detail the significance of folds in construction of dams and tunnels.

(OR)

23.	Explain the engineering significance of fault.

24. 	Give an elaborate account of the various geological features to be considered for the construction of dams.

(OR)

25. 	Explain the electrical methods of survey in civil engineering.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MECHANICS OF FLUIDS							      	     Time : 3 hours

Subject Code:	12CE205						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the Newton’s law of viscosity.

2.	What is a Newtonian fluid?

3.	Define Stream tube.

4.	Express continuity equation in polar co-ordinates.

5.	Name two flow measuring device that use the Bernoullis principle.

6.	Define Reynold’s number.

7.	What is meant by an equivalent pipe?

8.	Define hydraulic gradient line.

9.	What is meant by Dimensional Homogenity?

10.	What are repeating variables?

PART – B  (5 x 3 = 15 MARKS)

11.	An isosceles triangular plate of base 3m and attitude 3m is immersed vertically in an oil of specific gravity 0.8. The base of the plate coincides with the free surface of the oil. Determine the total pressure on the plate.

12.	List various types of flow.

13.	When 2500 litres of water flows per minute through a 0.3m diameter pipe which later reduces to a 0.15m diameter pipe, calculate the velocity of flow in the two pipes.

14.	Discuss major and minor energy losses in pipes.

15.	Define Similitude. Discuss its types.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Calculate density, specific weight and weight of one litre of petrol of specific gravity 0.7.

(8)

b.	Find the surface tension of a soap bubble of diameter 40 mm, when the inside pressure of the bubble 2.5N/m2 above atmospheric pressure.									    (7)

(OR)

17.	a.	A circular plate 1.5m diameter is submerged in water with its greatest and least depths 	below the surface being 2m and 0.75m respectively. Find the total pressure and centre of 	pressure.																		    (8)

b.	The space between two square flat plates is filled with oil. Each side of the plate is 720mm. The thickness of the oil film is 15mm, the upper plate which moves at 3m/sec requires a force of 120N to maintain the speed. Determine dynamic viscosity of oil and 	kinematic viscosity of oil, if the specific gravity of oil is 0.95.								    (7)

18.	a.	The diameters of a pipe at two sections 1-1 and 2-2 are 200mm and 300mm respectively. If 	the velocity through the pipe at section 1-1 is 4 m/s. Find the discharge through the pipe 	and the velocity of water at section 2-2.											    (5)

b.	Derive the continuity equation in cartesian coordinates.							  (10)

[P.T.O.]

(OR)

19.	a.	For a two dimensional flow the velocity function is given by φ=x2−y2. (i) Determine 	the velocity components in x and y directions.(ii) Show that the velocity component 	satisfies the condition of flow continuity.(iii) Determine stream functions and flow rate 	between the streamlines (2,0) and (2,2).(iv) Show that the stream line and potential line 	intersect  orthogonally at the point (2,2)	.										  (10)

b.	State Bernoullis theorem. List the assumptions made.								    (5)

20.	a.	A horizontal venturimeter with inlet and throat diameter 300 mm and 100mm respectively 	is used to measure the rate of flow. The pressure intensity at the inlet is 130kN/m2 and the 	vacuum pressure at the throat is 350mm of mercury. Assuming 3% head lost between inlet 	and throat, find the coefficient of discharge and rate of flow.						  (10)

b.	A rectangular notch 2.0 m wide has a constant head of 500 mm. Find the discharge over 	the notch if Cd is 0.62.															    (5)

(OR)

21.	a.	The following data relate to an orificemeter.

Diameter of the pipe = 240mm

Diameter of the orifice = 120mm

Specific gravity of oil	= 0.88

Reading of differential manometer = 400 mm of mercury

Co efficient of discharge of the meter = 0.65

Determine the rate of flow of oil.												  (10)

b.	Differentiate Laminar and Turbulent flow.										    (5)

22.	a.	At a sudden expansion of water main from 240mm to 480mm diameter hydraulic gradient 	rises by 100mm. Calculate the rate of flow.										    (7)

b.	Determine the rate of flow of water through the pipe of diameter 20 cm and length 50m 	with one end of the pipe connected to a tank and the other end is open to atmosphere. 	The pipe is horizontal and the height of water in the tank is 4m above the centre of the 	pipe. Consider all the minor losses and take f = 0.009.								    (8)

(OR)

23.	a.	Three pipes of length 800 m, 500 m, and 400 m and of diameter 500 mm, 400 mm, and 	300 mm respectively connected in series. These pipes are to be replaced by a single pipe of 	length 1700 m. Find the diameter of the single pipe.								  (10)

b.	Write short notes on Energy gradient line and Hydraulic gradient line.				    (5)

24.	The efficiency  of a fan depends on density, dynamic viscosity µ of the fluid angular velocity ω, diameter D of the rotor and the discharge Q. Express in terms of dimensionless parameters using Buckingham’s method.

(OR)

25.	a.	Write the dimensions for the following (i) kinematic viscosity (ii) Discharge

(iii) 	Resistance. 																    (9)

b.	Write short notes on model laws.												    (6)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	APPLIED HYDRAULICS AND HYDRAULIC MACHINERY														      								    Time : 3 hours

Subject Code:	12CE209										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define specific energy.

2.	Define surges.

3.	Define boundary layer.

4.	What are the different types of drag?

5.	What is an impulse momentum equation?

6.	Give the expression for force exerted by a jet on normal flat fixed surface.

7.     How would you classify turbines based on the direction of flow in the runner?

8.     What do you mean by jet ratio?

9.     What are the functions of multistage pumps?

10.   When do you go for jet pumps?

PART – B  (5 x 3 = 15 MARKS)

11. 	Distinguish between Draw down and back water curve.

12. 	What is boundary layer separation?

13. 	What are the applications of linear momentum principle?

14. 	What are the differences between impulse and reaction turbine.

15. 	What is negative slip in a reciprocating pump? When it happens?

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	Write about the types of flow in open channels.                                                       	(8)

b. 	Find the rate of flow and conveyance for a rectangular channel 7.5 m wide for uniform flow at a depth of 2.25m. The channel is having bed slope as 1 in 1000. Take Chezy’s constant C=55.                                                                                                                   (7)

(OR)

17. 	a. 	For a trapezoidal channel with bottom width 40m and side slopes 2H: 1V , Manning’s N is 	0.015 and bottom slope is 0.0002. If it carries 60 m3/s discharge, determine the normal 	depth.                                                                                                                               (10)

b. 	What is hydraulic jump and what are the conditions for the formation of hydraulic jump.

(5)

18.	Find the displacement thickness, the momentum thickness and energy thickness for the velocity distribution in the boundary layer given by uU=yδ,where u is the velocity at a distance y from the plate and u=U at y=δ where  δ=boundary layer thickness.  Also calculate value of δ*/θ.

(OR)

[P.T.O.]

19. 	A laminar boundary layer is formed on a 1m by 1 m smooth flat plate exposed to a fluid stream at zero incidence. If the drag on one side of the plate is 1.6 N and the maximum boundary layer thickness is 1.25 cm, determine the shear stress at the trailing edge. If the free stream velocity is doubled, what would be the maximum boundary layer thickness and the value of the mean drag coefficient?

20.  	250 liters/sec. of water is flowing in a pipe having a diameter of 300 mm. If the pipe is bent by 135 degree find the magnitude and direction of the resultant force on the bend. The pressure of the water flowing is 400 kN/m2. Take specific weight of water as 9.81 kN/m3.

(OR)

21. 	A jet of water of diameter 5 cm having a velocity of 25 m/s strikes a curved vane which is moving with a velocity of 15 m/s in the direction of the jet.  The jet leaves the vane at an angle of 60 degree to the direction of motion of vanes at outlet.  Find out the force exerted by the jet on the vane in the direction of motion and work done per second by the jet.

22. 	A Pelton wheel is to be designed for a head of 60m when running at 200 r.p.m. The Pelton wheel develops 130 shaft horse powers. The velocity of buckets is equal to 0.45 times the velocity of the jet. Overall efficiency = 0.85 and the coefficient of velocity = 0.98.

(OR)

23. 	a. 	Explain the working principle of Kaplan turbine with neat sketch.                        		    (7)

b. 	What do you mean by characteristics curves of turbines, explain with an example write the 	significance of characteristics curves                                                                         	    (8)

24. 	Find the power required to drive a centrifugal pump which delivers 0.04 m3/s of water to a height of 20 m through at 15 cm diameter pipe and 100 m long. The overall efficiency of the pump is 70 % and coefficient of friction f=0.015.

(OR)

25.	Explain the working of a single acting reciprocating pump with neat sketch and draw an indicator diagram, considering the effect of acceleration and friction, in suction and delivery pipes.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ADVANCED STRENGTH OF MATERIALS			      	     Time : 3 hours

Subject Code:	12CE210								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	If a member is subjected to a uniform bending moment (M) the radius of curvature of the deflected form of the member is given by _________.

2.	The expression EId2y/dx2 at a section of a member represents _________.

3.	Define slenderness ratio.

4.	Write the Euler’s critical load for a column of length ‘L’ with both ends hinged.

5.	The hoop stress in case of thick cylinders across the thickness is _________.

6.	What is the other name for Rankin’s theory?

7.	Define shear centre.

8.	The stress at any point in case of curved bar subjected to a bending moment is given by _________.

9.	When unsymmetrical bending will occur?

10.	For sections having symmetry about the two axes the shear centre coincides at the _________.

PART – B  (5 x 3 = 15 MARKS)

11.	What is conjugate beam method?

12.	What are the limitations of Euler’s Column theory?

13.	Write short note on strain energy theory.

14.	Write down the expression for Winkler Bach formula.

15.	How will you find the resultant stresses in unsymmetrical bending?

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the slope at the left support deflection under the load and maximum deflection of a simply supported beam of length 5m which is carrying a point load of 5kN at a distance of 3m from the left end. Take E =2x105N/mm2 and I=1x108mm4.

(OR)

17.	A beam of length 6m is simply supported at its ends and carries two point loads of 48kN and 40kN at a distance of 1m and 3m respectively from the left support. Find (a) deflection under each load (b) maximum deflection (c) the point at which maximum deflection occurs.

18.	A 1.5m long column has a circular cross section of 5cm diameter. One of the ends of the column is fixed in direction and position and other end is free. Taking factor of safety as 3, calculate the safe load using, (a) Rankine’s formula, take yield stress σc=560 N/mm2 and a =1/1600 for pinned ends. (b) Euler’s formula, Young’s modulus of C.I. =1.2x105N/mm2.

(OR)

19.	A hollow alloy tube of 5m long with external and internal diameters 40mm and 25mm respectively was found to extend 6.4mm under a tensile load of 60kN .Find the buckling load for the tube when used as a column with both ends pinned. Also find the safe load for the tube, taking a factor of safety =4.

[P.T.O.]

20.	The principal stresses at a point in an elastic material are 22N/mm2, (Tensile),110N/mm2 (Tensile) and 55N/mm2 (Compressive).If the elastic limit in simple tension is 220N/mm2 and     µ =0.3,then determine whether the failure of materials will occur or not according to all 5 failure theories.

(OR)

21.	A steel cylinder of 300mm external diameter is to be shrunk to another steel cylinder of 150mm internal diameter. After shrinking the diameter at the junction is 250mm and radial pressure at the common junction is 28N/mm2.Find then original difference in radii at the junction. Take E=2x105N/mm2.

22.	Determine the position of shear centre of a channel having dimensions: flanges 120mmx20mm and web 160mmx10mm.

(OR)

23.	Determine (a) location of neutral axis (b) maximum and minimum stress (c) ratio of maximum stress when curved beam of rectangular cross section of width 20mm and depth 40mm is subjected to pure bending moment of +600Nm. The beam is curved in a plane parallel to depth. The mean radius of curvature is 50mm.Also plot the variation of the stress across the section.

24.	Figure shows an unequal angle of dimensions 100mm by 60mm and 10mm thick. Determine (a) position of principal axes (b) magnitude of principal moments of inertia of the given angle.

(OR)

25.	A simply supported beam if span 3m carries a load of 500N at its centre. The section of the beam is an equal angle of size 100mm and 12.5 mm thick as shown in figure. The load line passes through centroid G of the section and is along line YG. Determine (a) stresses at point A, B and C of mid section of the beam (b) deflection of beam at the mid section and                (c) position of neutral axis. Take E = 200000N/mm2.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	SOIL MECHANICS									      	     Time : 3 hours

Subject Code:	12CE211						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define void ratio.

2.	List the main types of soils.

3.	Write the two methods to determine the particle size distribution of soil.

4.	What is the uniformity co-efficient for a well graded soil?

5.	Define permeability.

6.	What is seepage velocity?

7.	What are the stages of consolidation?

8.	What are the reasons for compression of the soil?

9.	Give the Coulomb shear strength equation.

10.	Define shear strength of a soil mass.

PART – B  (5 x 3 = 15 MARKS)

11.	Write the significance of soil mechanics.

12.	Define hydrometer analysis and write the correction to hydrometer readings.

13.	Write the steps in the construction of a flownet.

14.	Distinct between compaction and consolidation.

15.	Write the laboratory tests for determination of shear strength parameters.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain with a neat procedure for determining the specific gravity of the given soil in the laboratory by using the pycnometer apparatus.

(OR)

17.	An undisturbed sample of clayey soil is found to have a wet weight of 285N, a dry weight of 250N and a total volume of 14 x 103cm3,if  the specific gravity of soil solids is 2.7,determine the water content, void ratio and the degree of saturation.

18.	Explain the factors affecting the Atterberg Limits.

(OR)

19.	Explain the soil Classification system.

20.	In a laboratory Constant head permeability test, a cylindrical sample 100mm in diameter and 150mm height is subjected to a upward flow of 540ml(cm3) /min .The head loss over the length of the sample is measured to be 360mm.Calculate the co-efficient of permeability in m/s.

(OR)

21.	Explain the protective filters condition for proper selection of filter material.

[P.T.O.]

22.	A laboratory compaction test on soil having specific gravity of 2.7 gave a maximum dry density of 18kN/m3 and a water content of 15%. Determine the degree of saturation, air content and percentage air voids at the maximum dry density. What would be the theoretical maximum dry density corresponding to zero air voids at the optimum water content?

(OR)

23.	Explain with a neat sketch the Terzhaghis one dimensional consolidation theory.

24.	The results of a direct shear test on a specimen of dry sand are as follows

Normal stress= 96.6kPa

Shear stress at failure =67.7kPa

By means of Mohr’s circle of stresses, find the magnitude and direction of the principal stresses acting on a soil element within the zone of failure.

(OR)

25.	a.	What is the shear strength in terms of effective stress on a plane within a saturated soil 	mass at a point 295kN/m2 and the pore water pressure 120kN/m2? The effective stress 	parameters for the soil are c ̍̍ =12 kN/m2 and φ̍=30˚

b.	Explain Mohr –Coulomb failure criterion.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	HIGHWAY ENGINEERING						      	     Time : 3 hours

Subject Code:	12CE215										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the four stages of engineering surveys?

2.	State the classification of roads according to Nagpur road plan.

3.	Mention any four factors which control the geometric design of highway.

4.	What are the factors governing the super elevation of a road surface?

5.	Write any two factors to be considered in the design of pavements.

6.	Define ‘Modulus of sub-grade reaction’.

7.	What are the different types of joints provided in the cement concrete pavement?

8.	Mention the grades of bitumen used for bituminous macadam.

9.	Give examples for surface defects in pavements.

10.	What are the two main factors on which the cement concrete pavement fails?

PART – B  (5 x 3 = 15 MARKS)

11.	State the functions of Indian Road Congress.

12.	Define stopping sight distance.

13.	List the empirical methods of design of flexible pavement.

14.	Define ‘softening point of bitumen’.

15.	What are the causes of mud-pumping?

PART – C  (5 x 15 = 75 MARKS)

16.	Describe the salient features of 20 year road development plans of India.

(OR)

17.	Discuss the features of rural and urban roads.

18.	A state highway passing through a rolling terrain has a horizontal curve of radius equal to the ruling minimum radius. Design all the geometric features of this curve. Assume suitable data.

(OR)

19.	Explain the factors affecting sight distances.

20.	Explain the CBR method of flexible pavement design with its limitations.

(OR)

21.	Design a flexible pavement for the following data. No. of commercial vehicles in each direction = 500; Annual traffic growth rate = 7.5%; CBR = 6%; Road type = SH; Pavement width = 7cm; Design period = 20 years. Consider IRC method.

22.	What is WBM? Describe the construction procedure of a WBM road.

(OR)

23.	What are the desirable properties of road aggregates? Explain them briefly.

24. 	Explain the ‘Benkelman beam deflection’ test procedure with neat sketches.

(OR)

25. 	Describe about the causes of pavement failure.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	COMPUTATIONAL METHODS					      	     Time : 3 hours

Subject Code:	12CE216						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the interval of the root of the equation x3−9x+1=0?

2.	Find the value of α for the matrix A=1334 by Jacobi Method.

3.	State Simpson’s one-third rule.

4.	Write the standard values of ξ and w for the polynomial of degree 3 in Gaussian quadrature.

5.	Write down all the types of  straight line equation.

6.	Write down the Lagrange’s interpolation formula.

7.	Write the Euler’s formula.

8.	Write down the Taylor’s series for one variable.

9.	Give an example for the parabolic equation equation

10.	Write down the Crank-Nickolson scheme to solve the parabolic equation.

PART – B  (5 x 3 = 15 MARKS)

11.	Solve the following system of equation by Gauss seidel method

10x−5y−2z=3;4x−10y+3z=−3;x+6y+10z=−3

12.	Find a positive root of 3x−1+sinx=0 by iteration method.

13.	Write a short note on Newton’s interpolation formula.

14.	Compute y at x=0.25 by Modified Euler’s method given y'=2xy,y(0)=1

15.	Identify the equationfxx+2fxy+fyy=0.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Solve by Gauss elimination method

3x+4y+5z=18;2x−y+8z=13;5x−2y+7z=20.								  (10)

b.	Write down the working rule for finding the Eigen values by Jacobi method.			    (5)

(OR)

17.	a.	Find the Eigen values and Eigen vectors for the given matrix using power 	method161120003.

18.	a.	Solve x3+x2−1=0 by method of iteration.									   (8)

b.	Find an iterative formula to find the reciprocal of given number N and hence find the value  of 119.																		    (7)

(OR)

[P.T.O.]

19.	a.	Evaluate  06dx1+x2 using Trapezoidal rule and Simpson’s rule. Verify your answer with 	actual integration.															  (12)

b.	Using Gaussian three-point formula evaluate −11(3x2+5x4)dx.						    (3)

20.	From the following data find θ at x=43 and x=84

X 	:	40	  50	         60	      70	  80	     90

θ	:	184	  204	 226	     250	 276	    304

(OR)

21.	Fit a second degree parabola to the following data taking y as dependent variable

X: 	1	2	3	4	5	6	7	8	9

Y: 	2	6	7	8	10	11	11	10	9

22.	Compute y(0.2) given dydx=−y,y(0)=1 by taking h=0.1 using fourth order Range Kutta method.

(OR)

23.	Obtain y(0.6) given dydx=x+y,y(0)=1 with h=0.2 by Adam’s method.

24.	Using Crank-Nicholson’s scheme solve uxx=16ut,0<x<1,t>0 given u(x,0)=0,u(0,t)=0,u(1,t)=100t.Compute u in t direction taking h=1/4.

(OR)

25.	Solve ∇2u=−10(x2+y2+10) over the square mesh with sides x=0,y=0,x=3,y=3 with u=0 on the boundary and mesh length l unit.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	REINFORCED CONCRETE STRUCTURES			      	     Time : 3 hours

Subject Code:	12CE217								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is design strength?

2.	What is the drawback of working stress method?

3.	What is a two way slab?

4.	What is the minimum rise and tread in residential buildings?

5.	What is a doubly reinforced beam?

6.	What is a lintel?

7.	What is a long column?

8.	Define punching shear.

9.	Mention the three factors that must be considered while designing a RCC tank.

10.	List the types of retaining walls

PART – B  (5 x 3 = 15 MARKS)

11.	Compare balanced, under reinforced and over reinforced sections.

12.	List various types of staircases and mention the places where they can be used.

13.	Determine the area of tensile reinforcement required for a singly reinforced beam section of size 300 x 550 mm effective to carry a factored moment of 175 kNm. Take M20 grade concrete and Fe415 steel.

14.	Mention the reinforcement details that should be provided in a water tanks.

15.	What is a counterfort retaining wall? What are the forces acting on a retaining wall?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain (a) working stress method, (b) Ultimate load method and (c) Limit state design.

(OR)

17.	A singly reinforced concrete beam with an effective span of 4 m has a rectangular section with a width of 250 mm and an overall depth of 550 mm. The beam is reinforced with 3 bars of 10 mm diameter Fe415 HYSD bars at an effective depth of 500 mm. The self-weight of beam together with the dead load is 4 kN/m. Calculate the maximum permissible live load on the beam. Assume M20 grade concrete.

18.	Design a simply supported RC slab 6.3 x 4.5 m supported on all the four sides. It has to carry a characteristic live load of 10 kN/m2,  in addition to its dead weight. Assume M25 concrete Fe 415 steel and the exposure condition to environment as mild.

(OR)

19.	Design one of the flights of stairs of a school building spanning between landing beam to suit the following data:

Type of staircase = Waist slab type

Number of steps in flight = 12

Tread, T = 300 mm, Riser, R = 160 mm.

Width of landing beams = 400 mm

Materials = M20 concrete and Fe415 HYSD bars

[P.T.O.]

20.	Design a singly reinforced concrete beam to suit the following data:

Clear span = 3 m

Width of supports = 200 mm

Working live load = 6 kN/m

Concrete = M20 grade, Steel = Fe415 HYSD bars

(OR)

21. 	A reinforced concrete beam of rectangular section 350 mm wide is reinforced with 4 bars of    20 mm diameter at an effective depth of 550 mm out of which 2 bars are bent up near the support section where a factored shear force of 400 kN is acting, Using M20 grade concrete and Fe415 grade HYSD bars, design suitable shear reinforcements at the support section.

22.	Design an axially loaded tied column 400 x 400 mm pinned at both ends with an unsupported length of 3m for carrying a factored load of 2300 kN. Adopt M20 concrete and Fe 415 steel.

(OR)

23.	A solid footing has to transfer a dead load of 1000 kN and an imposed load of 400 kN from a square column of size 400 mm x 400 mm with 16 mm bars. Assuming that fck = 20 N/mm2,    fy = 415 N/mm2, and a safe bearing capacity of 200 kN/m2. Design the footing.

24. 	Design a cylindrical water tank of capacity 500,000 litres resting on the ground and having a flexible base. The overall height of the tank is restricted to 5 m with a free board of 300 mm. The bearing capacity of soil at the site is 150 kN/m2. Use M25 grade concrete and HYSD bars of grade Fe415.

(OR)

25.	Design a cantilever retaining wall to retain earth embankment 4 m high above ground level. The density of earth is 18 kN/m3 and its angle of repose is 30 degrees. The embankment is horizontal at top. The safe bearing capacity of the soil may be taken as 200 kN/m3 and the coefficient of friction between soil and concrete is 0.5. Adopt M20 grade concrete and Fe415 HYSD bars.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	WATER SUPPLY ENGINEERING					      	     Time : 3 hours

Subject Code:	12CE218										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write any two types of water demand.

2.	Define rate of demand.

3.	Write the pH value of portable water.

4.	Define total solids

5.	What is detention period?

6.	Define Aeration.

7.	What is fluoridation?

8.	Write the uses of a pump.

9.	Write the types of distribution.

10.	Define equivalent pipe.

PART – B  (5 x 3 = 15 MARKS)

11.	Discuss the functions of WHO.

12.	Explain the significance of BOD and COD in water.

13.	Write short notes on demineralization of water.

14.	Write the various piping materials.

15.	Write briefly about house connection.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Write the various factors governing the design period.   							    (7)

b.	Explain geometric increase method used for forecasting the population.			           (8)

(OR)

17.	a.	Explain the arithmetic increase method for forecasting the future population. 		    (7)

b.	The population of 5 decades from 1960 to 2000 is given below in table.  Find out the population after one, two and three decade beyond the last known decade by using arithmetic increase method.													    (8)

| Sl.No | Year | Population |
| --- | --- | --- |
| 1. | 1960 | 8000 |
| 2 | 1970 | 12000 |
| 3 | 1980 | 17000 |
| 4 | 1990 | 22500 |
| 5 | 2000 | 30000 |

18.	a.	Explain with a neat sketch the removal of iron and manganese from water. 			  (10)

b.	Write the various methods adopted for removal of colour,odour and taste from water.  (5)

(OR)

19.	Explain the significance of E-coli in water analysis.

20.	Write the various methods of purification of water, Explain any one method in detail.

[P.T.O.]

(OR)

21.	The maximum daily demand at a water purification plant has been estimated at 12 million liters per day. Design a suitable sedimentation tank (fitted with mechanical sludge removal arrangements)   for the raw supplies, assuming a detention period of 6 hours and a velocity of flow as 20 cm per minute.

22.	Explain various types of pumps and selection of pumps.

(OR)

23.	a.	Explain the following with neat sketches

i)  Stop cocks 		ii)  Water tap   				 							  (10)

b.	Explain Gravity conduits and pressure conduits.		                         				    (5) 24.	a.	Explain the various requirements for proper functioning of distributary system.       	    (8)

b.	Compare the merits and demerits of intermittent and continuous water distribution 	systems.																		    (7)

(OR)

25.	a.	Explain the various methods adopted for detection of leakage in the distribution pipes. (5)

b.	Explain the Hardy cross method procedure for water distribution.  					  (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FOUNDATION ENGINEERING						      	     Time : 3 hours

Subject Code:	12CE219						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State geostatic stresses.

2.	Suggest the type of failure in Net Safe Bearing Capacity.

3.	Justify : A disturbed sample is not a truly representative sample.

4.	Recommend a method followed in soil preliminary investigation.

5.	State negative skin friction.

6.	Suggest the choice of foundation for various soil formations.

7.	State the use of Rehbann’s construction.

8.	Sketch Gravity Retaining wall.

9.	Justify: Bishop’s simplified method of slices satisfies.

10.	State soil stabilization.

PART – B  (5 x 3 = 15 MARKS)

11.	Discuss the limitations of Elastic Theories.

12.	List the methods used for drilling the holes.

13.	Sketch any three types of foundation with their classification.

14.	Discuss the requirements for safe design of Retaining wall.

15.	Describe in brief cement stabilization.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Determine the vertical stress at a point P which is 3 m below and at a radial distance of 3m 	from the vertical load of 100 kN. Use Westergaard’s solution (v=0.0).				    (7)

b.	Determine the ultimate bearing capacity of  strip footing 1.20m wide, and having a depth of foundation of 1.0m. Use Terzahi’s theory and assume general shear failure. Take        Ф’= 350  ,ϒ = 18 kN/m3 , and c’ = 15 kN/m2 .										    (8)

(OR)

17.	a.	Discuss briefly Boussinesq’s vertical stresses due to a concentrated load Assumptions and 	Limitations.																	    (7)

b.	Discuss in detail Terzahi’s bearing capacity theory showing Terzahi’s bearing capacity 	equation.																	    (8)

18.	a.	Enumerate the stages in sub-surface explorations.								    (7)

b.	Discuss in detail any four types of borings for exploration with neat sketches.		    (8)

(OR)

19.	a.	Suggest the difference in geophysical methods by briefing the Seismic method and 	Electrical resistivity method.													    (7)

b.	Describe cone penetration tests. How these tests differ from standard penetration test. Draw neat sketches.															    (8)

20.	a.	Brief short notes on the following :

i.	Necessity of pile foundations

ii.	Classification of piles														    (7)

[P.T.O.]

b.	A concrete pile, 40 cm diameter, is driven 25 m into a soft clay (cu = 25.0 kN/m2  ,

ϒ = 19 kN/m3 ) . Determine the allowable load using (F.S. =2.5). The water table is at the ground surface. 																    (8)

(OR)

21.	a.	Discuss briefly how would you estimate the load carrying capacity of a pile in                     	(i) 	cohesionless soils, (ii) cohesive soils.										    (7)

b.	A 40 cm pile, 11 m long, has a bell of 2 m diameter and 1height . If the soil has cu = 20.0 	kN/m2 ,ϒ =19 kN/m3 , Ф = 250 . Estimate the allowable pull out resistance (FS = 3).	    (8)

22.	a.	Brief short notes on the following  :

Types of Retaining walls

Types of Sheet pile walls.														    (7)

b.	Determine the Coulomb active force on the retaining wall  for the following details :

i = 150  , ϒ =17.5  kN/m3 , Ф’= 300   , β = 750  , δ = 200 ,H = 5m .					    (8)

(OR)

23.	a.	Discuss the Rankine’s earth pressure theory in with neat sketches.					    (7)

b.	Detail the difference between Gravity retaining walls and Cantilever retaining walls with stability checks against  sliding and over turning.									    (8)

24.	a.	A long natural slope in an over consolidated clay (c’=10kN/m2 , ϒsat =20  kN/m2 , Ф’= 250) 	is inclined at 100 to the horizontal. The water table is at the surface and the seepage is 	parallel to the slope. If a plane slip had developed at a depth of 5m below the surface, 	determine the factor of safety. Take ϒw =10 kN/m3.								    (7)

b.	Discuss the friction circle method for the stability analysis of slopes. Can this  method be used for purely cohesive soil?													    (8)

(OR)

25.	a.	Describe briefly the following :

i.	Types of chemicals used in stabilization of  soils.

ii. 	Stability of Geo textile in soil.												    (7)

b.	Discuss the use of lime in stabilization of soils. What are the chemical and physical changes which take place in lime stabilization?									    (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DESIGN OF STEEL STRUCTURES					      	     Time : 3 hours

Subject Code:	12CE223										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Bolted connection may fail due to _____________.

2.	Name different types of weld used for steel connections.

3.	Design strength of tension member governed by _____________.

4.	Design compressive strength of compression member depends on _____________.

5.	Write the reason for failure of laterally unrestrained beam sections _____________.

6.	Define slenderness ratio.

7.	Write the purpose of using stiffeners in welded plate girder.

8.	What are all the different components of gantry girder?

9.	Name the different parts of truss member.

10.	What is the Indian standard used to calculate the wind pressure on roof truss?

PART – B  (5 x 3 = 15 MARKS)

11.	Classify different types of steel sections according to local buckling of plates.

12.	Define block shear in tension members.

13.	Describe lateral torsional buckling failure of beams with neat sketches.

14.	Draw the section of welded plate girder and write the components of plate girder.

15.	How do you select the configuration of roof truss? Sketch some of the simple truss configuration.

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the strength and efficiency of the lap joint with bolts of 20mm diameter and of grade 4.6. The two plates to be jointed are 10mm and 12mm thick. (Steel is of grade Fe 410).

(OR)

17.	A tie member in a truss girder is 250mmx14mm in size. It is welded to 10mm thick gusset plate by a fillet weld. The overlap of the member is 300mm and the weld size is 6mm. Determine the design strength of the joint, if the welding is done as shown in Figure. What is the increase in strength of the joint if the welding is done all around? Assume shop welding.

[P.T.O.]

18.	Compute the tensile strength of an angle section ISA 150x115x8mm of Fe 410 grade of steel connected with gusset plate as shown in figure.

(OR)

19.	Design a column to support a factored load of 1050kN. The column has an effective length of 7m with respect to Z axis and 5m with respect to Y axis.

20.	Determine the design bending strength and shear capacity of ISLB350 @ 486N/m considering the beam as laterally supported with the span of 3m.

(OR)

21.	Find the axial load capacity of buildup column as shown in figure. Length of the column 6m, thickness of flange 30mm, thickness of web 20mm.

22.	Design a slab base for a column ISHB350 @ 710.2N/m subjected to a factored axial compressive load of 1500kN. Assume M20 grade concrete.

(OR)

23.	Write short notes on the following.

a.	Welded plate girder.

b.	Gantry girder.

24.	Design an I-section purlin for an industrial building. Spacing of truss is 6m, Spacing of purlin is 5m, Wind pressure is 2kN/m2 and Weight of GI sheet is 130N/m2.

(OR)

25.	Draw different types of roof truss and write the loads to be considered for the analysis of truss.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ADVANCED STRUCTURAL ANALYSIS			      	     Time : 3 hours

Subject Code:	12CE224								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by statically indeterminate beam?

2.	Write the general slope deflection equation.

3.	A beam is simply supported at end A and fixed at B. If a moment M is applied at the simple end, the moment developed at the fixed end will be ____________.

4.	What is the sum of distribution factors at any joint?

5.	Define:	Degree of freedom.

6.	What is “lack of fit”?

7.	When do we prefer stiffness method?

8.	Which unknowns are treated as primary unknowns in stiffness method?

9.	Define:  Frequency and Period of oscillation.

10.	Give the expression for damping coefficient.

PART – B  (5 x 3 = 15 MARKS)

11.	A fixed beam AB of length 2l, propped at C midpoint of AB, carries a UDL of intensity w/m over the span AC. Formulate the slope deflection equation for the span AC.

12.	Explain the concept of distribution factor in moment-distribution method with an example.

13.	Give an example of a structure where stiffness matrix exists but flexibility matrix is zero.

14.	Define:	Degree of static and kinematic indeterminacy.

15.	Explain damped and un-damped vibration.

PART – C  (5 x 15 = 75 MARKS)

16.	Analyze the continuous beam shown in figure below and draw the BMD. Use slope deflection method.

(OR)

[P.T.O.]

17.	Analyze the rigid frame shown in figure below using slope deflection method and determine the reactions and moments.

18.	Analyze the continuous beam shown in figure below and draw the BMD using moment distribution method.

(OR)

19.	A rigid portal frame is loaded as shown in figure below. Determine the member end moments using moment – distribution method.

20.	Analyze the continuous beam shown below using flexibility method.

(OR)

21.	Analyze the rigid frame shown below using force method.

22.	Analyze the continuous beam shown below using stiffness method.

(OR)

23.	Analyze the rigid frame shown below using displacement method.

24.	Give step-by-step procedure of analyzing a multi – storey building using portal method.

(OR)

25.	Determine the plastic moment capacity of the frame for the loading as given in the figure below by:

a.	Beam Mechanism;

b.	Sway Mechanism; and

c.	Combined Mechanism

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	SANITARY ENGINEERING						      	     Time : 3 hours

Subject Code:	12CE226						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name the sewer which transports the sewage to the point of treatment.

2.	Mention the appropriate percentage of water in sewage.

3.	List any four sewer appurtenances.

4.	List the test available for sewer pipes.

5.	Label the sewage treatment units, which work on anaerobic decomposition of organic matter.

6.	What do you understand by the term, activated sludge?

7.	List the methods available to dispose sewage effluents.

8.	What are the methods available to dispose materials removed from skimming?

9.	List the types of sanitary fittings provided in buildings.

10.	Mention the testing methods available to test house sewers.

PART – B  (5 x 3 = 15 MARKS)

11.	List the routine tests carried out to know the physical and chemical characteristics of sewage.

12.	Draw the general layout of a municipal sewage treatment plant.

13.	Recall the principle behind sedimentation process.

14.	List the factors affecting sludge digestion.

15.	List the principle systems adopted in plumbing of drainage work in a building.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the factors affecting the storm water discharge.

(OR)

17.	Summaries the different characteristics of sewage.

18.	Elaborate the advantages and disadvantages of different types of sewer materials.

(OR)

19.	Discuss in detail about different types of sewer appurtenances.

20.	Design the primary sedimentation tank to treat wastewater with average flow rate of 10 MLD and peak flow of 22.5 MLD.

(OR)

21.	Draw schematic of the skimming tank and explain the purpose of providing it and how removal of pollutant occurs in this tank.

22.	Compile a detailed report about sludge thickening.

(OR)

23.	Discuss in detail about self purification of natural stream.

24.	With neat sketch explain about two pipe system and one pipe system in building drainage work.

(OR)

25.	Describe the working principle of septic tank with neat sketch.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	WATER RESOURCES ENGINEERING				      	     Time : 3 hours

Subject Code:	12CE227										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is surface runoff? How it is measured?

2.	Differentiate between Recording and Non-Recording types of Rain Gauge.

3.	Define Permeability.

4.	What do you mean by perched water table?

5.	Define consumptive use and soil moisture deficiency.

6.	What is meant by Paleo Irrigation?

7.	List any 3 methods to forecast population for water demand.

8.	What is water prizing?

9.	How are dams classified? Name any three types of dams.

10.	Define water power potential.

PART – B  (5 x 3 = 15 MARKS)

11. 	Define UH. What are assumptions and uses of UH?

12. 	Define the terms transmissibility and storage coefficient.

13.	Write short notes on participatory irrigation management.

14. 	Explain in detail about chlorination.

15.	Write a brief note on types of turbines in hydro electric scheme.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Describe hydrological cycle with a neat sketch.									(7)

b.	Explain any one type of automatic rain gauge with a neat sketch.					(5)

c.	What are the various forms of precipitation?										(3)

(OR)

17.	a.	What are the factors affecting infiltration capacity?

b.	Given the ordinates of a 4hr UH as below, Derive the ordinates of 12hr UH for the same

catchment.

| Time | 0 | 4 | 8 | 12 | 16 | 20 | 24 | 28 | 32 | 36 | 40 | 44 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 hr UH | 0 | 20 | 80 | 130 | 150 | 130 | 90 | 52 | 27 | 15 | 5 | 0 |

18.	a.	Derive an expression for steady state discharge for a fully penetrating in unconfined 	aquifer.

b.	What is an aquifer? Explain briefly the various types of aquifer with neat sketches.

(OR)

19.	a.	i.	Write short notes on Artificial Recharge of Groundwater.

ii.	Interference of open well.

b.	Explain how yield of well is obtained by recuperation test.

[P.T.O.]

20.	a.	Explain briefly about sprinkler irrigation and drip irrigation with neat sketches.

b.	10 cumecs of water is delivered to a 32 hectare field, for 4 hours. Soli probing after the

irrigation indicates that 0.3m of water has been stored in the root zone. Compute water

application efficiency.

(OR)

21.	a.	What is to be grown at a certain plate? The useful climatologically conditions are tabulated   	below. Determine evapotranspiration and consumptive irrigation requirement of wheat 	crop.  Also determine field irrigation requirement if water application efficiency is 80%. 	Make use of Blaney Criddle Equation and crop factor = 0.8

| Month | Monthly trip in degree Celsius (t) | Monthly percent of day time hours (p) | Useful rainfall Re |
| --- | --- | --- | --- |
| Nov | 18.00 | 7.2 | 1.7 |
| Dec | 15.00 | 7.15 | 1.42 |
| Jan | 13.50 | 7.30 | 3.01 |
| Feb | 14.50 | 7.10 | 2.25 |

b.	What do you mean by tank sluice? Describe different types of tank sluice indicating 	regulator arrangements

22.	a.	What are the two major sources of water supply? Discuss the comparative merits and 	demerits of both these types from these sources.				   					    (7)

b.	The population of 5 decades from 1930 to 1970 is given below in table. Find out the population after one, two and three decades beyond the last known decade, by using a incremental increase method.													    (8)

| Year | 1930 | 1940 | 1950 | 1960 | 1970 |
| --- | --- | --- | --- | --- | --- |
| Population | 25000 | 28000 | 34000 | 42000 | 47000 |

(OR)

23.	a.	Draw a neat sketch of rapid gravity filter and describe how it works.

b.	Explain briefly the general methods of distribution of water employed in municipal water supply scheme.

24.	a.	What are the factors governing the selection of particular type of dam?

b.	Briefly explain how you would fix the capacity of a reservoir and height of dam required 	for this storage.

(OR)

25.	a.	Explain the neat sketch the basic components of hydroelectric scheme.

b.	Write short notes on:

a.	Reservoir sedimentation.

b.	Flood control measure.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	RAILWAYS, AIRPORTS AND HARBOURS			      	     Time : 3 hours

Subject Code:	12CE228								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by adzing?

2.	Define grade compensation.

3.	What do you understand about MSP?

4.	How ventilations are provided in tunnels?

5.	What is meant by wind coverage?

6.	Define gate capacity.

7.	What are the functions of quay walls?

8.	When the neap tide and spring tide will occur?

9.	Why linguistic variables are used in fuzzy logic?

10.	Define perceptron.

PART – B  (5 x 3 = 15 MARKS)

11.	List the functions of any three permanent way components.

12.	What is the necessity of inter-locking between points and signals?

13.	State the importance of instrumental landing system.

14.	Why is dredging so essential in port operations?

15.	What are the uses of weight matrix of ANN model?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	What are the ideal characteristics of elastic fastenings?                                      	    (5)

b.	Draw a typical cross section of B.G. double track in cutting & embankment and mark its components and dimensions.                                                                         			(5+5)

(OR)

17.	a.	Determine the length of transition curve for a BG track having design speed of train on 	curve is 90 kmph.                                                                                           			    (7)

b.	If the wheel base of a vehicle moving on a BG track is 6 m, the diameter of wheel is 1.5 m and the depth of flanges below the top of rail is 3.17 m, Determine the extra width required to be provided on gauge, if the radius of curve is 260 m. Also compute the shift of this curve, if the length of transition curve is 90 m.                                   				    (8)

18.	Calculate all the necessary elements required to set out a 1 in 8½ turnout taking off from a straight B.G track with its curve starting from the toe of the switch (i.e.) tangential to the gauge face of the outer main rail and pass through theoretical nose of crossing. The heel divergence (d) is 11.4 cm.

(OR)

19.	a.	Explain the various railway track construction methods.                                  	 	  (8)

b.	Compare the conventional and modern methods of railway alignment survey.             (7)

[P.T.O.]

20.	a.	What are the factors to be considered for the selection of site for a commercial airport? 	Explain the importance of them.                                                                    			    (8)

b.	What survey has to be conducted to orient a runway? Explain the method of orienting the runway using wind rose diagrams.                                                                        		    (7)

(OR)

21.	a.	Discuss the capacities of different runway configurations.                              			    (8)

b.	Describe the factors to be considered for the design of taxiway and its special features.

(7)

22.	a.	Explain the operations and working principles of wet and dry docks.                              (8)

b.	Bring out the various kinds of buoys used as navigational aids.                                       (7)

(OR)

23.	Write short notes on the following:

a.	Harbour Entrance.                                                                                                   		 (5)

b.	Piers and Bulk Heads.                                                                                               	 (5)

c.	Breasting Dolphins.                                                                                                          (5)

24.	a.	Describe the evolution of Artificial Intelligence.                                                  	        (8)

b.	Discuss the applications of AI in engineering researches.                                              (7)

(OR)

25.	Explain the use of Fuzzy Logic and Genetic Algorithms in solving transportation problems with examples.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FUNDAMENTALS OF REMOTE SENSING AND GIS	    Time : 3 hours

Subject Code:	12CE235								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Classify Remote Sensing.

2.	Define: EMR.

3.	What do you understand by the term orbital period?

4.	List the types of Orbit.

5.	Recall the sources of non systematic error.

6.	Name any three software used for image processing.

7.	Give real world examples of point, line and polygon features.

8.	Classify map scale.

9.	Name the types of data model used in GIS.

10.	Define the term LIS.

PART – B  (5 x 3 = 15 MARKS)

11.	Demonstrate different stages of remote sensing.

12.	Explain the term Swath.

13.	Recall the features of GCP.

14.	Define the term map.

15.	List five applications of RS and GIS.

PART – C  (5 x 15 = 75 MARKS)

16.	Demonstrate the principle components of Remote Sensing.

(OR)

17.	Explain energy interaction with earth surface features.

18.	Discuss in detail about different types of orbits.

(OR)

19.	Elaborate the on features of Remote Sensing satellites.

20.	Discuss in detail about Geometric corrections in RS.

(OR)

21.	Summarize features of different image processing software.

22.	Label and discuss components of GIS

(OR)

23.	Demonstrate the role of DBMS in GIS.

24.	Explain different data model in GIS.

(OR)

25.	Compose LIS as urban development modeling tool.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	CONSTRUCTION PRACTICES						      	     Time : 3 hours

Subject Code:	12CE238						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define seasoning of timber.

2.	Which type of plastic cannot be reused?

3.	What do you mean by shoring?

4.	Suggest the foundation method for a single column located on a deep well.

5.	List out the Bouge’s compounds in cement.

6.	Suggest the methods of anti-termite treatment adopted in the construction.

7.	For a residential building construction, suggest the methods of construction that can be adopted.

8.	What do you mean by underpinning?

9.	Suggest the sources of equipment usage in the construction field.

10.	Suggest the dredgers type used in marine construction.

PART – B  (5 x 3 = 15 MARKS)

11.	Outline the building materials that are used in the construction.

12.	Reason out the causes of failure of foundation and the preventive measures.

13.	Reason out the necessity for curing of concrete and brief with different methods.

14.	Recommend the type of concrete construction method for a speedy construction and state its advantages.

15.	Suggest the factors that affect the cost of owning and operating the equipment.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss the lifting appliances used in stone construction with neat sketches.

(OR)

17.	Discuss elaborately on the properties of various metals.

18.	Explain in detail the available methods of site exploration stating its importance in the construction process.

(OR)

19.	Discuss the essential requirements of substructure and elaborate the method of construction on hard strata at shallow depth.

20.	Explain the stages involved in the manufacturing process of cement and its final composition.

(OR)

21.	Discuss the stages in concrete hardening and the tests to verify the workability of concrete in the field.

22.	a.	Discuss the suitability of various bonds in brickwork with neat sketches.			  (10)

b.	Discuss the connections in steel work and their advantages.						    (5)

(OR)

[P.T.O.]

23.	a.	Suggest the building material used in the load bearing construction and compare their 	usage in the construction.														    (7)

b.	Reason out the causes for the dampness in the construction and discuss their preventive 	measures.																	    (8)

24.	Discuss the equipments used in the concrete construction with neat sketches.

(OR)

25.	Discuss the safe practices with formwork in constructions.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ENVIRONMENTAL IMPACT ASSESSMENT		      	     Time : 3 hours

Subject Code:	12CE251								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by Environmental Impact Assessment?

2.	What are the objectives of EIA?

3.	Mention the components of EIA.

4.	State the various software packages being used for EIA.

5.	What are the prediction tools in EIA?

6.	What do you understand by Socio-economic aspects?

7.	Define Environment Management plan.

8.	What is environmental audit?

9.	Define Mining.

10.	Name few nuclear power plants in India.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the role of public in environmental decision making?

12.	Write short notes on cost benefits analysis.

13.	Discuss about models in prediction of EIA.

14.	Write short notes on Mitigation plan in Environmental Management Plan?

15.	Give a note on Ecological impacts of hydroelectric power plant.

PART – C  (5 x 15 = 75 MARKS)

16.	What are the various types of EIA. Discuss the benefits and limitations of EIA.

(OR)

17.	a.	Explain the salient features of Environmental Impact Statement.					    (8)

b.	Discuss the role of Ministry of Environment and Forest in sanctioning infrastructure 	development projects.															    (7)

18.	Describe the check list method and network method of assessment of environmental impacts

(OR)

19.	Discuss the screening and scoping processes in EIA.

20.	How are modeling techniques applied in the prediction of impacts of a project? Explain with examples

(OR)

21.	Explain in detail the various methods for the prediction of impacts of the affected environment.

22.	a.	Explain in detail rehabilitation plan in EMP.										  (8)

b.	Discuss on ethical aspects of EIA. 									         		  (7)

(OR)

23.	Discuss in detail about the environment management plan.

24.	Explain about the environmental impact assessment for mining.

(OR)

25.	Present in detail a case study of a big dam

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	INTELLIGENT BUILDINGS						      	     Time : 3 hours

Subject Code:	12CE257						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name the three dimensions of building intelligence.

2.	List at least two components of security system.

3.	State any one difference between close control A/C systems and comfort A/C.

4.	Write the function of the main switch.

5.	Write the application of foam systems in case of fire.

6.	What does means of escape refer to?

7.	Define intrusion detection with reference to security systems.

8.	What is a central station?

9.	What do you understand by the term PLC?

10.	How much does the base frequency of optic fiber vary from that of copper cable?

PART – B  (5 x 3 = 15 MARKS)

11.	Write three points to state how a building can become intelligent.

12.	What are structure, system services and management in intelligent building?

13.	State the different factors affecting comfort air conditioning.

14.	Examine the need for security systems.

15.	Give the basic elements of satellite communications.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Give an account of holistic approach to energy efficiency in buildings – An Integrated 	approach.																	    (8)

b.	Explain the green building concept in the 21st century buildings.					    (7)

(OR)

17.	a.	Explain the basic concept of intelligent building bringing out the technology systems and 	evolutions of intelligent buildings.												    (8)

b.	What are the major aspects of intelligent building ? Highlight the integration and enhanced 	comfort of the building?														    (7)

18.	a.	Enumerate on the centralized  air conditioning system  and explain the working of VAV 	and CAV with the help of a neat sketch.											    (8)

b.	Write notes on any five protective devices in electrical installations.				    (7)

(OR)

19.	Explain the basic requirements   transmission and distribution  system should satisfy. Also draw an elementary diagram of the transmission and distribution system explaining its principle components.

20.	a.	Write notes on side lightings and top lightings  with the  help  of sketches.			    (8)

b.	Detail on automatic fire alarm and detection systems.								    (7)

(OR)

[P.T.O.]

21.	Give an account of management of fire safety is an essential element in averting fire.

22.	a.	With the help of neat sketch explain the access control system used in intelligent buildings.																					    (8)

b.	Explain about the different types of cards used in access control system.			    (7)

(OR)

23.	Explain the  analog CCTV,  digital CCTV and its combination with diagrams. Also bring out the difference between them.

24.	a.	Discuss about the microprocessor system hardware and its operation.				  (10)

b.	Write the uses of satellite communication.										    (5)

(OR)

25.	a.	Define a PLC and illustrate its system components and discuss its working principles.

(8)

b.	Detail about the working of fiber optic sensors.									    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	INDUSTRIAL WASTE TREATMENT AND DISPOSAL 	     Time : 3 hours

Subject Code:	12CE259								 		      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name any Five industries which are polluting water ?

2.	List out any two “Air pollution “ sources.

3.	Name any two coagulants using for the removal of industrial wastes.

4.	Define  the ”Residue management”

5.	What is the pH value suitable for drinking water ?

6.	Where you can use the “Sedimentation tank” ?

7.	What are the raw materials used in cement manufacturing industry?

8.	List out any two waste water characteristics from the industries ?

9.	What are agro-industries?

10.	Mention the by-products of Sugar Industry.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the effects of industrial wastes on “streams” ?

12.	Explain any one method for “ removal of dissolved  organic & inorganic matters”.

13.	Explain about the “Oxidation  ditch”  with  sketch.

14.	What are the Fibres used in textile industry ?

15.	Briefly explain about the poultry effluents.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain  in detail about the effect of industrial waste on waste water treatment plant.

(OR)

17.	Explain the methods for  “Process modification and material changes to reduce the strength of wastes”.

18.	Explain  a)   Equalization       b) Neutralization

(OR)

19.	Explain in detail about the “ combined  treatment of industrial and municipal wastes and its advantages .

20.	Explain about  “Aerobic and anaerobic digestion of sludge”.

(OR)

21.	Explain about a) Nitrogen removal and b) Phosphate removal.

22.	Explain the manufacturing process and waste water characteristics of Textile Industries.

(OR)

23.	Explain the manufacturing process and waste water characteristics of “Petro chemical Industry”.

24.	Explain the manufacturing processes in Paper and Pulp Industry and the treatment processes of the wastes.

(OR)

25.	Explain the Leather Tanning process with neat flow chart. Also explain the characteristics of  wastes and their treatment process.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FLUID MECHANICS AND MACHINERY			      	     Time : 3 hours

Subject Code:	12CE263								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the unit of dynamic viscosity of fluid.

2.	How do you measure pressure in case of pipe flow?

3.	Write the continuity equation in three dimension.

4.	Write the practical applications of Bernoulli’s theorem.

5.	How do you classify the loss of energy in pipes?

6.	Give an example for positive displacement pump?

7.	How will you measure discharge in case of open channel.

8.	Distinguish between Laminar and Turbulent flow.

9.	Classify turbines.

10.	Define specific speed.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Total Pressure and Centre of pressure.

12.	State Bernoulli’s theorem.

13.	What do you mean by power transmission through pipes?

14.	Draw the inlet velocity triangle and outlet velocity triangle of an impeller in a centrifugal pump.

15.	What do you mean by Negative slip?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	State and explain the Newtons law of viscosity.									    (5)

b.	The velocity distribution of flow over a plate is parabolic with vertex 30 cm from the plate where the velocity is 180 cm/s.  If the viscosity of the fluid is 0.9 N.s/m2 , find the velocity gradients and shear stresses at distances of 0 cm, 15 cm and 30 cm from the plate.

(10)

(OR)

17.	a.	Simple U-tube manometer is installed across an orificemeter.  The manometer is filled 	with mercury (sp gravity = 13.6) and the liquid above mercury is carbon tetra chloride (sp 	gravity = 1.6).  The manometer reads 200 mm.  What is the pressure difference over the 	manometer.																	    (5)

b.	A circular plate 1.5 m diameter is submerged in water, with its greatest and least depths below the surface being 2 m and 0.75 m respectively.  Determine the total pressure on one face of the plate and the position of the centre of pressure.							  (10)

18.	a.	The velocity components in x and y directions are given as u=2xy33−x2yand 	v=xy2−2yx33.  Indicate whether the given velocity distribution is

i.	a possible field of flow.

ii.	not a possible field of flow.												    (5)

b.	Derive the continuity equation.													  (10)

[P.T.O.]

(OR)

19.	a.	In a two dimensional incompressible flow, the fluid velocity components are given by

u=x–4y and v= –y–4x.  Show that velocity potential exists and determine its form as well 	as stream function.															  (10)

b.	What are the uses and limitations of flow net?									   (5)

20.	a.	A pipe line carrying oil (sp.gr 0.8) changes in the diameter from 300 mm at position  1 to 	600 mm diameter at position 2 which is 5 meters at a higher level. If the pressures at 	positions 1 and 2 are 100 kN/m2 and 60 kN/m2 respectively and discharge is 300 liters/sec.  	Determine loss of head and direction of flow.									    (8)

b.	Derive the expression for rate of flow of an venturimeter.							    (7)

(OR)

21.	a.	Differentiate between a venturimeter and orificementer.							    (5)

b.	A vertical venturimeter carries a liquid of relative density 0.8 and has inlet and throat diameters of 150 mm and 75 mm respectively.  The pressure connection at the throat is 150 mm above that at the inlet.  If the actual rate of flow is 40 lit/sec and the cd=0.96.  Calculate the pressure difference between inlet and throat in N/m2.					  (10)

22.	a.	Explain the working principle of reciprocating pump with neat sketch.				  (10)

b.	Explain indicator diagram.														    (5)

(OR)

23.	The following data relate to a centrifugal pump :  Outlet diameter of impeller = 800 mm

Width of impeller vanes at outlet	= 100 mm

Angel of impeller vanes at outlet = 30°.

The impeller runs at 600 rpm and delivers 0.9m3/s under an effective head of 30 m.  A 450 kW motor is used to derive the pump.  Determine the manometric, mechanical and overall efficiencies of the pump.  Assume water enter the impeller vanes radially at inlet.

24.	The powerhouse is equipped with impulse turbines of Pelton type.  Each turbine delivers a maximum power of 1400 kW.  When working under a head of 840 m and running at 600 rpm.  Find the least diameter of each jet and the mean diameter of the wheel. What would be the approximate diameter of the orifice at the nozzle tip?  Determine the value of the jet ratio and state whether it is within limit or not.  Specify the number of buckets for the wheel.  Take the overall efficiency of the turbine as 89%.  Assume Cv=0.988 and Ku=0.45.

(OR)

25.	Explain the working principle of submersible pump with neat sketch.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	APPLIED ELASTICITY AND PLASTICITY			       Time : 3 hours

Subject Code: 	12CE302									      	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

- A hollow circular shaft of inner radius 7cm and outer radius 15cm is subjected to a twisting couple of  4500 Nm .If the shear stress -strain diagram for the shaft material is given by τ = 320 γ^0.3. Determine the maximum shear stress induced in the shaft and angle of twist per unit length.    												      (15)

- Give the assumptions in plastic analysis.  										(5)

2.	a.	Displacement field in a homogeneous isotropic elastic body is given by 				u=k(2x2z+60x)i+(6z2+10xy)j+(6z2+3xyz)k		where k=1x10⁻⁶mm.

E=2x10⁵N/mm², r=0.25,Evaluate the stress components at a point P(5,10,‐15)mm.  (15)

b.	Give the biharmonic equation for the case of plane strain.  						(5)

(OR)

3.	The state of stress at a point wih respect to the xyz system is

21−220−1−2−11kN/m2

Determine the stress tensor relative to the x’y’z’ coordinate system obtained by a rotation         	through 30° about the z‐axis.

4.	a.	State Cauchy integral theorem. 												(5)

- Show that the Airy’s stress function ф=A(xy³‐3/4  xyh²) represents stress distribution in a cantilever  beam loaded at the free end with load P. Find the value of A if

τxy=0 at y=±h2.where b and h are width  and depth respectively of the cantilever.  																		      (15)

(OR)

5. 	a.	A square shaft rotating at transmits torque to a crane which is designed to lift maximum 	load of 175 kN at a speed of 10m/min.If the efficiency of crane gearing is 60% ,	estimate the size of the shaft  for the maximum permissible shear stress of 35MPa.Also

calculate the angle of thrust of the shaft for a  length of 3m.Take G=100 GPa.  	      (15)

b.	Derive the torque equation of a thin rectangular section.							(5)

6.	Calculate the strain energy for a simply supported rectangular beam AB of span 10m with

1kN point load at the centre of the span .The beam size is 40x60mm.

(OR)

7.	Derive the expression for deflection of a rectangular plate by the principle of virtual work.

[P.T.O.]

8.	Solid circular shaft of radius 12cm is subjected to transmit 600 kW at 540rpm.

The maximum torque is 30 percent greater than the mean torque.If the shear stress strain

curve for the shaft materials is  given by τ=280γ^0.25,determine the maximum stress induced 	in the shaft and the corresponding angle  of twist.What would be these values if the shear

stress‐strain curve is a linear one? G=0.84x10⁵ N/mm².

(OR)

9.	For the stress tensor given below,determine the principal stresses and the direction cosines

associated with the normals to the surface of each principal stress.

00010001000100002000100020000 Nm2

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	ADVANCED CONCRETE TECHNOLOGY				Time : 3 hours

Subject Code: 	12CE306									      	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	List the various methods of proportioning.										(4)

b.	Design a concrete mix by ACI method for construction of an elevated water tank. The specified design strength of concrete (characteristic strength) is 30 MPa at 28 days measured on standard cylinders. Standard deviation can be taken as 4 MPa. The specific gravity of FA and C.A. are 2.65 and 2.7 respectively. The dry rodded bulk density of C.A. is 1600 kg/m3, and fineness modulus of FA is 2.80. Ordinary Portland cement (Type I) will be used. A slump of 50 mm is necessary. C.A. is found to be absorptive to the extent of 1% and free surface moisture in sand is found to be 2 per cent. Assume any other essential data.														      (16)

2.	a.	Briefly explain different types of cement.   									      (8)

b.	Explain the methods of finding the fineness of cement.						     (12)

(OR)

3.	a.	Write a detailed note on Alkali-Aggregate reaction.							     (10)

b.	Discuss the factors affecting workability of concrete with Super plasticizers.		     (10)

4.	a.	Distinguish between segregation and bleeding.								     (10)

b.	Brief about different methods of curing.										     (10)

(OR)

5.	a.	Bring out the impact of w/c ratio on durability of concrete.					     (10)

b.	Describe the methods of controlling the sulphate attack on concrete.			     (10)

6.	a.	Write a detailed note on aerated concrete.									     (10)

b.	Bring out the features of no-fines concrete.									     (10)

(OR)

7.	a.	Brief the properties of Sulphur infiltrated concrete.							     (10)

b.	How the cold weather affects the properties of concrete.						     (10)

8.	a.	Bring out the symptoms, causes, preventive measures and remedial measures for the 	defects occurring during construction.

(OR)

9.	a.	Explain the various cracking on hardened concrete.							      (10)

b.	Briefly explain about the common types of repairs carried out on concrete structures.(10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	STABILITY OF STRUCTURES						       Time : 3 hours

Subject Code: 	12CE307								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

Derive Euler’s critical load for the following boundary conditions:

a.	Bothe ends fixed.															      (10)

b.	One end fixed and one end free.											      (10)

2.	a.	Explain Double Modulus theory.											      (10)

b.	Explain Shanley’s theory of inelastic column beaviour.						      (10)

(OR)

3.	a.	Derive the maximum moment for a beam column with concentrated lateral load.       (10)

b.	Derive the critical load for a frame fixed at its base and free to translate at the top using 	neutral equilibrium.														      (10)

4.	Determine the critical loading for a cantilever I beam subjected to concentrated load acting at the free end.

(OR)

5. 	a	Explain  (i) Uniform torsion and (ii) Non-uniform torsion with neat sketches.	      (10)

b.	Derive 	the expressions for strain energy of torsion for the St.Venant torsion and warping torsion.									 						      (10)

6.	Derive the differential equations for a plate with constant biaxial compression forces and constant in-plane stresses.

(OR)

7.	Determine the critical load of a plate in shear by Galerkin’s method.

8.	What are the assumptions while deriving the shell equations? Derive Donnel’s equations for a cylindrical shell.

(OR)

9.	a. Differentiate (i) Perfect and imperfect systems and (ii) Sensitive and insensitive 	systems.																             (10)

b. Explain symmetric and asymmetric bifurcation and its limit points.				      (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	EXPERIMENTAL TECHNIQUES AND INSTRUMENTATION													       							Time : 3 hours

Subject Code: 	12CE317										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Explain the concept of photo elasticity with a suitable example.				    (10)

b.	Explain the working principle of pressure gauges and proving rings.			    (10)

2.	a.	Explain the working principle of LVDT.									    (10)

b.	Explain the procedure to find velocity and acceleration using transducers.		    (10)

(OR)

3.	a.	Explain the seismographs and vibration analyzer.								    (10)

b.	Describe the procedure of digital data acquisition system.						    (10)

4.	a.	Explain the concept of sound level meters.									    (10)

b.	Describe wind tunnel and state its uses.										    (10)

(OR)

5.	Explain direct and indirect model analysis with suitable examples

6.	a.	Explain half cell potential test.												    (10)

b.	Describe the controlled blasting for demolition.								    (10)

(OR)

7.	a.	Describe the procedure to find slope and deflection of beams.					    (10)

b.	Explain the creep test on concrete.											    (10)

8.	a.	Explain pull out test.														    (10)

b.	Explain ultrasonic testing.													    (10)

(OR)

9.	a.	Explain impulse echo and GPR tests.										    (10)

b.	Explain resonant frequency methods of testing.								    (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	MATLAB											       Time : 3 hours

Subject Code: 	12CE343								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a. 	Mark the right answer for the following questions.				              				(10)

i.	What does Matlab stand for?

a) Math Laboratory	b) Matrix Laboratory 		c) Mathworks

d) Nothing   			e) none of the above

ii. 	What symbol precedes all comments in Matlab?

a) “ 					b) %					c) //

d) <					e) none of the above

iii. 	Which of the following is not a pre-defined variable in Matlab.

a) pi 				b) inf 					c) i			d) gravity 		e) j

iv. 	This Matlab command clears all data and variables stored in memory:

a) clc			b) clear			c) delete			d) deallocate		e) none of the above

v. 	Characters in Matlab are represented in their value in memory.

a) decimal		b) ASCII 		c) hex			d) string 		e) none of the above

vi. 	To print a newline in a fprintf statement, you must use the following escape character:

a) \t 			b) \nl			c) \nxt			d) \n			e) none of the above

vii. 	In Matlab, this keyword immediately moves to the next iteration of the loop:

a) update		b) goto			c) continue		d) break			e) none of the above

viii. Which of the following will correctly define x, y, and z as symbols?

a) sym (x, y, z) 	b) syms x y z		c) syms x, y, z	d) sym x, y, z	e) none of the above

ix. 	Which of these is the way to access the first element in a vector named v (assuming there is at least one element in the vector)?

a) v(0) 			b) v(1) 			c) v 				d) v(: , 0) 		e) none of the above

x.	Which of the following is used to see if two elements are equal in MATLAB?

a) !=			b) == 			c) isequal		d) =				e) none of the above

b. 	What are variables and arrays and list down the rules governing them?			       			(5)

c. 	What are the different data types? Give examples.				                   				(5)

(OR)

2.  a. 	Please write the Matlab commands to create all of the Matrices above.                             		(5)

b. 	Solve each of the following problems by hand using the data given in section (a) (do not use Matlab code for this section).   If a problem is unsolvable, please state that and state why it is unsolvable.  																			(5)

i)  A +B   		ii)  4 x A    		iii)  A x E     		iv)  A x C      	v)  A x A-1

c. 	Write about the operator precedence in MATLAB and list down different types of operators used in MATLAB									                                					     (10)

[P.T.O.]

3.	a. 	Write the Matlab command you would use to create a vector of the even whole numbers 	between 31 and 175 named evenNums.  						                                  	(3)

b. 	Write the Matlab command you would use to create a vector of 50 equally spaced values on a linear scale between 31 and 175 named evenNums. 					                      	(3)

c. 	What are the conditional statements in MATLAB and explain the syntax with examples and the rules governing them.					                                                                     (14)

(OR)

4. 	a.  	Evaluate the following Boolean expressions in Matlab. 				        			(4)

i) x = 22 &&  (130 - 41 == 44) || (12 + 8 > 20);

ii) y = (24 <= 72) &&  3 || (56 – (22 * 3) == 0) &&  (25 == 24);

b. 	Explain in detail about loop control statements with examples				       		     (16)

5.  	a. 	List down formatted input and ouput statements in MATLAB with examples.		     (10)

b. 	Define user define functions. List down the rules governing the function file		       	     (10)

(OR)

6.   	a. 	How will you read from and write to a text file in MATLAB. Explain with an example (10)

b. 	Given the following matrix, show the results generated by the MATLAB command.       (3)

R = [1.22  3.78  2.41]

fprintf('R = %3. 1f  \n', R)

c. 	Write a MATLAB program to read data from the file ab1.txt. The data stored is as follows:																						(7)

1. 	Anil		234	 2k1	  50

2. 	Bhanu	215	 2k2	  54

3. 	Charu	231	 2K1  62

7.  	a. 	Plot the function z=2−1.5x2+y2over the domain −4≤x≤4 and −4≤ y≤4 using            	     (15)

i.	Mesh plot

ii.	Surface plot

iii.	Mesh curtain plot

iv.	Mesh and contour plot

v.	Surface and contour plot

b. 	How will you plot the four different graphs obtained from curve fitting using ‘subplot’    (5)		command.

(OR)

8.  How will you create different 2-D plots using MATLAB commands? Explain with an example.

9. 	Compulsory :

a. 	Explain in detail about the steps used in creating different objects in a GUI  layout with their     significance.																	     (14)

b. 	Write short notes on								            						      (6)

i. 	Handles

ii. 	Call back functions

iii. 	GUI layout

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	FINITE ELEMENT METHODS IN ENGINEERING

Time : 3 hours

Subject Code: 	13CE303										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

Consider a 1 mm diameter, 50 mm long aluminium pin- fin as shown in figure is used to enhance the heat transfer from a surface wall maintained at 300°C. The governing differential equation and the boundary conditions are given by

Wall

Tw

h   T∞

L

K(d2T/dx2) = Ph/A(T-T∞)

T(0) = Tw = 300°C.

dT/dx(L) = 0 (insulated pipe)

K( thermal conductivity) = 200 W/m°C For aluminium, h= 20 W/m2,

T∞(ambient temperature) = 30°C

Estimate the temperature distribution in the fin using Galerkin weighted residual Method.

2. 	a. 	Explain briefly about Iso, Super and Sub parametric elements.						(5)

b. 	A triangular membrane element of thickness t = 0.1 cm, with the (x,y) coordinate of nodes indicated besides the node numbers is shown in fig. If the material of the   element is steel with Young’s modulus E = 207 GPa and Poisson ratio υ = 0.3. Determine the following.

i.	Shape functions of the element, Ni(x,y), Nj(x,y), Nk(x,y).

ii.	Matrix [B] that relates the strains to the nodal displacements.

iii.	Elasticity matrix [D].

iv.	Element stiffness matrix, [k(e)].                                              				      (15)

[P.T.O.]

(OR)

3.	a. 	Derive the element stiffness for linear serendipity rectangle.					      (10)

b. 	On a four nodal quadrilateral plane stress element the nodes are (0, 0), (6, 2), (6, 6), (1,5). A concentrated load whose x and y components are 10 kN, 16 kN respectively is applied at a point (4,5). Find the equivalent nodal forces. 						      (10)

4. 	a. 	Derive the shape function for ZIB 20 element.								      (15)

b. 	What are parallel piped elements? 												(5)

(OR)

5. 	a. 	Explain in detail Numerical Integration using Gaussian Quadrature.                            (10)

b. 	Derive the shape functions and element stiffness matrix for an axis symmetric Annular 	ring element.																      (10)

R2

R1

r, u

2                     1

2

1

6. 	Discuss in detail about the Mindlin Plate elements.

(OR)

7. 	Explain with a neat sketch the various three dimensional elements used in analysis of Shells.

8.	Write short notes on

a. 	Pre processor and Post processor. 											      (10)

b. 	Explain H method of mesh refinement. 										      (10)

(OR)

9. 	Explain the various sources of non linearities and explain their solution procedure.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	EXPERIMENTAL TECHNIQUES AND INSTRUMENTATION METHODS												Time : 3 hours

Subject Code: 	13CE307										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

Briefly explain the following tests bringing out its advantages and disadvantages.

a.	Accelerated Curing test.													      (10)

b.	Ultrasonic pulse test.														      (10)

2.	a.	Justify the statement using Strain gauges the errors can be reduced than using a dial 	gauge.																	      (10)

b.	Discuss on strain rosettes and its applications.								      (10)

(OR)

3.	Briefly discuss on the concept of Moire freezing techniques.

4.	If n is the number of variables and m is the dimensions, (n-m) dimensionless variables are to be derived for a static problem. Enumerate the variables and how they are used for similarity between prototype and model.

(OR)

5.	Explain how cyclic load test are conducted on frames? Discuss on the hysteresis curve of concrete and steel.

6.	The Young’s modulus of a tor steel rod is to be determined using strain measurement readings taken from a UTM. Explain the procedure and principle behind the operation of UTM with suitable sketches.

(OR)

7.	Explain the working principle of Linear Variable Differential Transformer(LVDT).

8.	How the earthquake motion is simulated using a Shake table? Briefly discuss on its working Principle.

(OR)

9.	a.	Explain the working principle of a Cathode Ray Oscilloscope.					      (12)

b.	Briefly discuss the transducers used for measuring structural vibrations.				(8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December -2015

Subject Title :	PRESTRESSED CONCRETE STRUCTURES		      	     Time : 3 hours

Subject Code:	14CE202								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the advantage of using prestressed concrete in construction?

2.	Long-term deflection in prestressed concrete is due to _________.

3.	What will happen when the local principle tensile stress exceeds the tensile strength of concrete?

4.	Draw the profile of parabolic cable.

5.	What are all the factors that affect the deflection of prestressed concrete?

6.	Spalling/Bursting reinforcement provided in which part of pre-stressed concrete members?

7.	Define the term primary moment in continuous beams?

8.	Write the advantages of continuous members in PSC beams?

9.	Give examples for pre-stressed tension and compression members?

10.	Write the Indian standard to be used for the design of pre-stressed concrete water tank?

PART – B  (5 x 3 = 15 MARKS)

11.	List the different method of pre-stressing system?

12.	Draw the different types of tendon profile used in prestressed concrete beams.

13.	Sketch the pattern of anchorage zone reinforcement in end blocks.

14.	Define the term concordant cable.

15.	Write the main advantages of pre-stressed concrete pole?

PART – C  (5 x 15 = 75 MARKS)

16.	Briefly explain the different losses of pre-stressed concrete.

(OR)

17.	A rectangular concrete beam of cross section 30cm deep and 20cm wide is pre-stressed by means of 15 wires of 5mm diameter located 6.5cm from the bottom of the beam and 3 wires of 5mm diameter located 2.5cm from the top. Assuming the pre-stress in the steel as 810N/m2, Calculate the stress at the extreme fibers of the mid span section when the beam is supporting its own weight over a span of 6m. Live load on the beam is 6kN/m2.

18.	A post tensioned bonded pre-stressed concrete beam is of rectangular section 200mm wide and 400mm deep. The beam is pre-stressed by 300mm2 high tensile steel located at an eccentricity of 100mm. The effective pre-stress after loss is 800N/mm2. The characteristic tensile strength of pre-stressing steel is 1600 N/mm2 and cube compressive strength is 40 N/mm2. The ratio of yield to ultimate tensile stress is 0.90. Estimate the ultimate moment capacity of the section or flexural strength of the section as per Indian standard.

(OR)

19.	A pre tensioned pre-stressed concrete beam of rectangular section is required to support a design ultimate moment of 100kN/m. Design the section for fck 50N/mm2 and ultimate tensile strength of steel is 1600 N/mm2. Assume the depth of beam 300mm and width of beam 150mm.

[P.T.O.]

20.	A pre-stressed concrete beam of rectangular section 120mm wide and 300mm deep, span of 6m. The beam is pre-stressed by a straight cable carrying an effective force of 180kN at an eccentricity of 500mm. Live load of 4kN/m and young’s modulus of 38kN/mm2. Compute the deflection for the following cases,

a.	Upward deflection under pre-stress and self weight.

b.	Final downward deflection (pre-stress+ live load and self weight).

(OR)

21.	The end block of a post tensioned pre-stressed member is 550mm wide and 550mm deep. Four cables each made up of 7 wires of 12mm diameter strands and carrying a force of 1000kN are anchored by plate anchorages, 150mmx150mm located with their centers at 12mm from the edges of the end block. The cable duct is 50mm diameter, fck - 45 N/mm2 and fci - 25 N/mm2. Yield stress of steel anchorage reinforcement is 260 N/mm2. Design suitable anchorages for end block.

22.	A precast Pre tensioned beam of rectangular section of size 100x200mm, Effective span of beam is 5m. The initial force in the tendon is 150kN. Loss in pre-stress is 15%, The beam is incorporated in a composite T-beam by casting a top flange of breadth 400mm and thickness 40mm, Live load on beam 8kN/m2. Calculate the resultant stress developed in the precast and insitu concrete block.

(OR)

23.	Two simply supported beams, AB=BC=10m of rectangular cross section, each post tensioned by means of two parabolic cap cables(p=300kN) with eccentricities of zero at supports and 150mm @ mid span. The ends of cap cable are located at 3m from the central support. The cable centre is 50mm from the top of the beam over the central support B. The beam is 200mm wide and 600mm deep.

a.	Calculate the secondary moment at B.

b.	Locate the resultant line of thrust through the beam AB.

c.	Evaluate the resultant pre-stress.

24.	a.	Discuss the design consideration of pre-stressed concrete sleepers.

b.	Explain the criteria of designing pre-stressed concrete pipes.

(OR)

25.	A cylindrical pre-stressed concrete water tank of internal diameter 30m is required to store water over a depth of 7.5m. The permissible compressive stress in concrete at transfer is 13 N/mm2 and the minimum compressive stress under working pressure is 1 N/mm2. The loss ratio is 0.75; wires of 5mm diameter with an initial stress of 1000 N/mm2 are available for circumferential winding and freyssinet cables made up of 12 wires of 8mm diameter stressed to 1200 N/mm2. fck - 40 N/mm2, Design the tank walls assuming the base as fixed.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	DESIGN OF STRUCTURES FOR DYNAMIC LOAD

Time : 3 hours

Subject Code: 	14CE3013										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

Explain the behavior of concrete under impact loading & cyclic loading. Draw the stress strain relationships.

2.	Differentiate soft impact and hard impact. How do you determine the penetration depth in the case of hard impact in a semi-infinite concrete slab?

(OR)

3.	a.	Brief the soil response to monotonic loading.								      (10)

b.	Specify the guidelines for design against earthquake in the case of hydraulic structures.  																				      (10)

4.	Explain the current state of seismic analysis and design for towers and tunnels

(OR)

5.	a.	What is the difference between pushover analysis and response spectrum method? Which 	is a suitable method for earthquake analysis?									      (10)

b.	Mention a few suggestions for maximizing the potential of a building to offer protection against explosive loads.  													      (10)

6.	Explain the basic concepts of seismic analysis & design of tall buildings.

(OR)

7.	What is vibration control? Mention the types of vibration control devices. Explain the elements of active vibration control system with sketches.

8.	Discuss the inverse problems in the vibration of discrete systems that are closely related 	to matrix inverse eigen value problems.

(OR)

9.	Explain procedures to establish discrete equations of motion for a structure from its measured responses and inputs.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	DESIGN OF TALL BUILDINGS						       Time : 3 hours

Subject Code: 	14CE3015								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Explain design principle &factors affecting growth and height of tall buildings.	    (10)

b.	Explain the structural forms of tall buildings.								    (10)

2.	a.	Discuss the modeling for approximate analysis.								    (10)

b.	Explain reduction techniques.												    (10)

(OR)

3.	a.	Explain analytical and approximate drift analysis.							    (10)

b.	Explain approximate analysis for vertical and horizontal forces.				    (10)

4.	a.	Explain the behaviour of shear wall structures.								    (10)

b.	Explain the analysis of shear wall.											    (10)

(OR)

5.	a.	Explain the behaviour of coupled shear wall.									    (10)

b.	Explain the continuous medium method.									    (10)

6.	Explain the behaviour and design of wall frame structures.

(OR)

7.	Explain the behaviour and analytical models of tubular structures.

8.	a.	Explain the behaviour of core structures.									    (10)

b.	Explain the method of analysis of outriggered structures.						    (10)

(OR)

9.	a.	Explain the influence of buckling on stability of high rise building.				    (10)

b.	Explain the dynamic analysis of high rise building.							    (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	INTRODUCTION TO IWRM						       Time : 3 hours

Subject Code: 	14CE3026								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Write the Principles of IWRM.  Present a case study of a city with 1 million population (like Coimbatore) and discuss the application of IWRM concepts to restore all the water bodies (Tanks, Rivers and Channels).										      (15)

b.	Write the concept of Sustainable Development.  									(5)

2.	a.	Discuss the Global Climate change and its impacts on water resources.  			      (15)

b.	Write a short note on “National water Policy on IWRM”			 				(5)

(OR)

3.	a.	Discuss the role of forest and cultivated watersheds in sustainable water resources 	management.																      (15)

b.	How to manage the Water logging? 											(5)

4.	a.	Present the impact of anthropogenic activities on water bodies using a case study and 	propose suitable measures for the restoration of it.							      (15)

b.	Write the impacts of Sand mining on water resources.                 			 		(5)

(OR)

5.	a.	Discuss the Role of Water in development, peace and happiness of community.         (10)

b.	Write the various diseases caused due to the Water contamination and describe its impact on human health.									  			                	      (10)

6.	a.	Why Water Conservation is the need of the hour? Describe the various methods of it.

(10)

b.	Describe the need and concept of Inter basin Transfer with case study.			      (10)

(OR)

7.	a.	Discuss 3R concept and discuss various methods for Reuse and Recycling of 	wastewater/grey water.													      (10)

b.	Write the Role of IWRM in eradication of poor sanitation and health conditions in under developed nations in Africa.											      (10)

8.	a.	What do you mean by Water economics and relate it with water Pricing. 	       	      (10)

b.	Write the role of PPP in IWRM context.						       				      (10)

(OR)

9.	a.	Describe the role of government in implementing IWRM concept.  			            (10)

b.	Write the problems that our country is facing due to private water markets   and how it is 	going to affect the citizens in future. 										     (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	OPEN CHANNEL FLOW & SEDIMENT TRANSPORT																						Time : 3 hours

Subject Code: 	14CE3042										      aximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.   	Describe the various classifications of open channel flow.         				      (10)

b.   	A 3 m wide rectangular channel conveys 12 m3/sec of water at a depth of 2 m. Calculate 	the specific energy, critical depth, critical velocity and minimum specific energy.      (10)

2.	a.   Discuss the computation of Gradually Varied Flow profile using direct step method.   (10)

b.   Draw the expansions and contractions design diagrams in channel transitions. 	      (10)

(OR)

3.	a.    Derive the differential equation for the Gradually Varied Flow. 				      (10)

b.    Explain the computation of Spatially Varied Flow profile using numerical methods. (10)

4	a.    A hydraulic jump is formed in a 5 m wide rectangular channel carrying a discharge of

20 m3/sec. The pre-jump depth is 0.5 m. Find the post-jump depth, post-jump Froude number and energy loss in the jump.   										      (10)

b.   Discuss the profile of an ogee spillway with diagram.         						      (10)

(OR)

5.	a.   Explain the different types of hydraulic jump. 								      (10)

b.   How the hydraulic jump is useful in stilling basin? Explain.         				      (10)

6.	a.   Distinguish between kinematics and dynamics. 								      (10)

b.   Discuss the Continuum concept in fluid element.     							      (10)

(OR)

7.	a.   Explain about macroscopic and microscopic continuity relation. 				      (10)

b.   Brief about diffusion equation used in open channel flow. 						      (10)

8.	a.    Discuss about the meandering and stream networks in river system.				      (10)

b.    How will you control the reservoir sedimentation?        						      (10)

(OR)

9.	a.     Elaborate the Universal Soil Loss Equation. 									      (10)

b.     Differentiate between bed load and suspended load.   						     (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	REMOTE SENSING AND GIS						       Time : 3 hours

Subject Code: 	14CE3044								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Give the importance of digital image processing. 								      (10)

b.	What do you mean by image enhancement, how it is achieved?                           	      (10)

2.	a.	Draw the sketch of electromagnetic spectrum and write the lands suitable for various 	remote sensing applications. 												      (10)

b.	Explain atmospheric effects on remote sensing.                                               	      (10)

(OR)

3.	a.	Describe various types of platforms employed in remote sensing.            		      (10)

b.	Describe his working of multispectral scanners with sketch.                       		      (10)

4.	a.	Explain various components of GIS.                                                               		      (10)

b.	Explain raster data structure with sketch with their advances over vector data structure                                                                                                                                               																		      (10)

(OR)

5.	a.	What are the sources errors in output?

b.	What different types of errors occur in output and how they are rectified?

6.	Write short notes on

a.	Spatial measurement methods.                                                                   		      (10)

b.	Digital elevation modelling and applications.                                              		      (10)

(OR)

7.	What is object oriented GIS? How these are applied to the problems? What are their limitations?

8.	Estimate water quality parameters with this help of GIS and remote sensing data of a particular watershed.

(OR)

9.	What are the parameters to be selected for site section for artificial recharge? How these parameters useful? What are the limitations?

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	ECOSYSTEM MANAGEMENT						       Time : 3 hours

Subject Code: 	14CE3047								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	What are the structural and functional components of ecosystem?

b.	What are the scopes of ecology?

2.	a.	Explain energy flow and material cycling in ecosystems.                                      	      (10)

b.	Explain the mechanism of steady state maintenance in open and closed systems.       (10)

(OR)

3.	a.	Describe the different types of food chains.                                               		      (10)

b.	What do you understand by mature and immature ecosystems? Explain how a tropical rain forest is an example of a mature ecosystem.								      (10)

4.	a.	Explain tolerance and adaptability of living organisms.						      (10)

b.	Explain how ecological succession can be used in the treatment of waste water.         (10)

(OR)

5.	a.	Explain the salient features of oligotrophic, mesotropic and entropic lakes.

b.	Explain desert ecosystems.

6.	a.	Explain pE-pH diagram? What are their uses in Engineering?                       		      (10)

b.	What are the acid –base reactions? Write their applications in environment engineering.                                                                                                                                  																		      (10)

(OR)

7.	Explain carbon sequestration? How they are applied in Industrial wastewater treatment?

8.	What are ecosystems models? Describe in detail about modelling and modelling procedure involved in ecological modelling.

(OR)

9.	a.	What are self-organising design and process?

b.	How these principals applied in suitable treatment technologies?