Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	APPLIED CHEMISTRY							      	     Time : 3 hours

Subject Code:	11CH101						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give an example for bifunctional monomer.

2.	An example for a thermosetting polymer is ___________.

3.	The CaSO4 forms ___________ type of hardness.

4.	Give an example for coagulant.

5.	The efficiency of fuel is measured in terms of ___________.

6.	What is LPG?

7.	The oxidation potential of Zinc electrode is ___________.

8.	Corrosion resistance of stainless steel is due to the presence of ___________ in it.

9.	Give an example for smart material.

10.	The size of a nano particle is in the order of ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	Write a note on the preparation of a polymer by addition polymerization reaction.

12.	Calculate the total, permanent and temporary hardness of a water sample containing Ca(HCO3)2 -162 mg/l, Mg(HCO3)2 -73 mg/l, CaSO4 -136 mg/l, MgCl2 - 95 mg/l,

NaCl - 23 mg/l.

13.	Give the significance of proximate analysis of coal.

14.	How corrosion can be controlled on iron related alloys.

15.	What are biomaterials? Give two examples.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain in detail about various ingredients added in compounding of plastics.     	     (10)

b.	Explain the preparation of PVC. Give its properties and uses.						    (5)

(OR)

17.	a.	Explain the preparation of nylons. Give its uses. 									  (10)

b.	What is tacticity? Explain the classification of polymers based on tacticity. 			    (5)

18.	a.	What is break point chlorination? Explain its significance.     						    (5)

b.	How hard water can be softened by zeolite process? 								  (10)

(OR)

19.	a.	How is hardness of water estimated by EDTA method?    							  (10)

b.	What is desalination? Describe reverse osmosis process of desalination.			         (5)

20.	a.	How flue gas analysis is carried out in Orsat’s apparatus. Give its significance.   		  (10)

b.	How anti-knocking characteristics of a fuel can be improved? 						    (5)

(OR)

21.	a.	With a neat diagram explain the manufacturing process of metallurgical coke by Otto 	Hofmann’s method.         														  (10)

b.	Write on the characteristics of a good fuel.       									    (5)

[P.T.O.]

22.	a.	Discuss the construction and working of Lead acid storage cell.					  (10)

b.	Derive an expression to calculate EMF using Nernst equation. 					         (5)

(OR)

23.	a.	What is electrochemical corrosion?      											    (2)

b.	A zinc rod immersed partially in sodium chloride solution undergoes corrosion.   Explain the mechanism.												        			    (5)

c.	Explain how corrosion can be controlled by cathodic protection and by using inhibitors.

(8)

24.	a.	What are nanomaterials? 														    (3)

b.	Explain any two methods for the preparation of nanomaterials. 					  (12)

(OR)

25.	a.	What are ceramics? Give examples. 											    (3)

b.	Explain the preparation of silica and alumina. Write any three properties and applications.

(12)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	INSTRUMENTAL METHODS OF ANALYSIS		       Time : 3 hours

Subject Code: 	CH343										        Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a. 	Explain the principle involved in atomic absorption spectroscopy. 					(5)

b. 	What are the advantages of atomic absorption spectroscopy?  						(5)

c. 	Define detection limit and sensitivity of an atomic adsorption spectroscopy. 		       (5)

d. 	What are the applications of atomic absorption spectroscopy? 						(5)

2. 	a. 	Discuss various types of electronic transitions involved in the electronic spectroscopy.(5)

b. 	Define the following. (5) (i) Bathochromic shift	(ii) Hypsochromic shift.

c. 	Describe Woodward – Fiesher rules for calculating the  absorption maximum in dienes.

(5)

d. 	Explain the various applications of ultra-violet spectroscopy. 					(5)

(OR)

3. 	a. 	Define Infra – red spectroscopy. Describe the various molecular vibrations in the 	technique. 																	(5)

b. 	Write a note on Fermi resonance. 												(5)

c. 	What is Finger print region in Infra – red spectroscopy?Explain. 					(5)

d. 	Write any two applications of Raman  spectroscopy. 								(5)

4. 	a. 	Define the term Chemical shift and describe the factors which influence it. 		      (10)

b. 	Explain clearly the term ‘spin – spin’ coupling. 									(5)

c. 	A compound having molecular formula C9H11Br showed the following signals in PMR 	data.

(i) multiplet  (δ 2.25), 2H  (ii) triplet (δ 2.75),2H  (iii) triplet (δ 3.38) 2H, (iv) singlet

(δ 7.22), 5H. Assign the structure of the compound giving reasons.					 (5)

(OR)

5. 	a. 	Explain the mechanism of Hyper fine interaction in the ESR spectra of organic radicals.

(5)

b. 	Explain the applications of ESR spectroscopy. 								      (10)

c. 	Explain the relaxation process in flurecent spectra. 								(5)

6. 	a. 	Write a note on McLafferty rearrangement. Explain with example. 					(5)

b. 	Explain (i) Nitrogen rule (ii) Even electron rule, with examples. 					(5)

c. 	Explain (i) Molecular ion peak 	(ii) Base peak (iii) Meta stable peak	 			      (10)

(OR)

7. 	a. 	What are the factors affecting thermogravimetric curves? 							(6)

b. 	What is chromatography? Write any two applications. 							(4)

c. 	Explain the principle and Instrumentation of gas chromatography. 				      (10)

8. 	a. 	Deduce Bragg’s equation and discuss its use in X – ray spectroscopy. 			      (10)

b. 	Discuss the application of X – ray diffraction methods. 						      (10)

(OR)

9. 	a. 	Write short notes on Auger electron spectroscopy. 								(5)

b. 	Sketch the Instrumentation of electron spectroscopy. 								(7)

c. 	What do you understand by optical activity? Define specific rotation. 				(4)

d. 	Write short notes on optical rotatory dispersion. 									(4)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	APPLIED CHEMISTRY							      	     Time : 3 hours

Subject Code:	12CH201						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Why bicarbonate of calcium is a temporary hardness forming salt?

2.	Give an example for coagulant.

3.	Name any one resin as a hetero polymer.

4.	The number of reactive sites in a monomer is called ___________.

5.	What is CNG?

6.	What is the indication if considerable amount of CO is present in flue gas?

7.	When zinc and copper are electrically connected and exposed to electrolyte ___________ undergoes corrosion?

8.	___________ is the metal used as sacrificial anode.

9.	Define abrasive.

10.	Give an example for a lubricant.

PART – B  (5 x 3 = 15 MARKS)

11.	Give briefly on the problems of using hard water.

12.	Write a short note on conducting polymers.

13.	Define octane number. Give two examples for anti-knocking agents.

14.	What is EMF Series? Write any two importances.

15.	What is a refractory? How are they classified?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Describe ion exchange process of softening the hard water with a neat diagram.	          (10)

b.	A sample water is found to contain 18.6mg/l Mg(HCO3)2, 15mg/l MgCl2,  10.5mg/l CaCl2. 	Calculate the permanent and temporary hardness of water.  						    (5)

(OR)

17.	a.	Explain the methods of removal of micro organisms from water using bleaching powder 	and chlorine.					         											    (9)

b.	How is sea water desalinated by reverse osmosis?								    (6)

18.	a.	Explain the different types of polymerization reactions.  							    (6)

b.	Differentiate between thermoplastics and thermosetting plastics.					    (4)

c.	Give the preparation, properties and uses of polyethylene.						           (5)

(OR)

19.	a.	How is bakelite prepared? Mention its uses.										  (10)

b.	Write on any four additives added in compounding of plastics.   					    (5)

20.	a.	How is flue gas analyzed by Orsat’s apparatus? 									  (10)

b.	Write a short note on the characteristics of a good fuel. 							    (5)

(OR)

[P.T.O.]

21.	a.	Describe the manufacture of coke by Otto-Hoffman method. 						  (10)

b.	Mention the importance of proximate analysis.									    (5)

22.	a.	Explain how corrosion can be controlled in the case of marine equipments.		         (5)

b.	Explain the factors influencing corrosion with respect to the nature of the metal. 	      (10)

(OR)

23.	a.	Describe the construction and working of hydrogen – oxygen fuel cell.				    (7)

b.	Explain the mechanism of oxidation corrosion. How the nature of metallic oxide is 	influencing oxidation corrosion?   												    (8)

24.	a.	Explain the manufacturing process of Boron carbide and its uses.					    (8)

b.	Explain the preparation of alumina. Write any three properties and applications. 		    (7)

(OR)

25.	a.	Explain the selection criteria of a lubricant.									         (10)

b.	Explain the manufacturing process of silicon carbide and its uses.					    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ENVIRONMENTAL STUDIES						      	     Time : 3 hours

Subject Code:	12CH203						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write any one reason for deforestation.

2.	The major constituent of biogas is ____________.

3.	Water moves in a cycle known as ____________ cycle

4.	Name two hot spots in India.

5.	The intensity of noise is measured in ____________ unit

6.	Minamata disease is caused by pollution due to heavy metal ____________.

7.	Name a suitable substitute used in place of CFC’s.

8.	Mention any one benefit of water shed management.

9.	Write any one social injustice against women in India.

10.	Richter scale is used to measure the intensity of ____________.

PART – B  (5 x 3 = 15 MARKS)

11.	Write any three effects of over utilization of ground water.

12.	List any six consequences of enhanced green house effect.

13.	What are the causes of thermal pollution?

14.	What is sustainable development?

15.	Expand the following: GIS, GPS, AIDS.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Discuss the direct and indirect uses of forest resources.							  (10)

b.	How solar energy is used as renewable source of energy?                                  		    (5)

(OR)

17.	a.	Suggest measures which can be adopted in daily life to conserve natural resources.	    (7)

b.	Explain the measures adopted to control soil erosion.								    (8)

18.	Explain the terms. a) types of food chain  b) bio-magnification   c) abiotic component  d) levels of biodiversity   e) endangered and extinct species

(OR)

19.	a.	Discuss the stages involved in the process of ecological succession. 				    (8)

b.	Write a short note on insitu conservation of biodiversity.							    (7)

20.	a.	Explain the effects of air pollution on human health, plants and climate in detail.		    (9)

b.	Discuss the sources and effects of noise pollution.                                        			    (6)

(OR)

21.	a.	Mention any 6 solid wastes generated in our locality.  Explain the 5R’s adopted to 	minimize generation of solid waste.												    (8)

b.	Discuss the principles of green chemistry.                     							    (7)

[P.T.O.]

22.	a.	What do you mean by ozone hole? How it is formed? What are it’s effects?			    (9)

b.	List the objectives of creating awareness about environment among public. Discuss the methods adopted to create awareness.											    (6)

(OR)

23.	a.	What is acid rain? How it is formed? What are it’s effects? Suggest measure for it’s 	control.                                                   											  (12)

b.	Write a short note on forest act.                                  								    (3)

24.	a.	List the ways by which AIDS can be transmitted.                                      			    (5)

b.	Explain the factors responsible for high birth rate in India.                              		  (10)

(OR)

25.	a.	What are land slides? Discuss the causes and effects. Suggest measures for control.	  (10)

b.	Discuss the role of genetic and behavioral influences on human health.                         (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	PRINCIPLES OF ORGANIC CHEMISTRY			      	     Time : 3 hours

Subject Code:	12CH205								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the structure of a) 1,2-dimethyl cyclobutane       b) 5-ethyl-2,2-dimethyl heptane

2. 	Short note on arynes with structure.

3. 	Write a shot note on hyperconjugation effect with example.

4. 	What is the meaning of addition reactions and types?

5. 	Define recemization with different method.

6. 	Draw Sawhorse, Newman projection formula for ethane.

7. 	Which of the following chiral/achiral

a)                                        b)

2HN

COOH

H00C

NH2

8. 	Define stereo selectivity.

9. 	State Zeisel’s method

10. 	Give an account of isoprene rule

PART – B  (5 x 3 = 15 MARKS)

11. 	Define heterocyclic compounds? Write the structural formulas of pyrrole, pyridine and  thiophene.

12. 	Explain Hammond postulate.

13. 	Write a short note on Walden Inversion

14. 	What are stereo specific reactions? Explain.

15. 	Elaborate discuss on how identify the presence of nitrogen group, presence of unsaturation in natural product.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Describe electrophile and nucleophiles? Elaborate neutral, positive electrophile and 	negative, neutral nucleophiles with an example. 									  (10)

b.	Define alicyclic compound.

Write the formula of   2-bromo-4-chlorohexane

3-methyl-1-cyclohexanol									    (5)

(OR)

17.  a. 	Give an account of carbanion. Explain its stability with adjacent groups.			    (8)

b.	Describe free radicals with different types of reactions.							    (7)

18. 	a. 	Define Inductive effect and electromeric effect. Why tertiary alkyl group more inductive 	effect than secondary and primary alkyl group?     								    (7)

b.	Discuss briefly rearrangement reaction with different types. 						    (8)

[P.T.O.]

(OR)

19. 	a. 	Explain with suitable example of kinetic and thermodynamic control of reaction. 	    (8)

b. 	How resonance changed by steric inhibition and steric enhancement. 				    (7)

20. 	a. 	Explain Cahn-Ingold-Prolog rules for R-S nomenclature with example. 				    (8)

b. 	Elaborate about optical isomerism of chiral & achiral molecule.					    (7)

(OR)

21. 	a. 	Discuss cis-trans isomerism with maleic acid and fumaric acid.					    (8)

b. 	Brief discussion about conformation of cyclohexane and disubstituted cyclohexane.	    (7)

22. 	a. 	Explain more stable and less stability of steric compound by Curtin-Hammet principle.(8)

b. 	State asymmetric synthesis.  Discuss with two examples.							    (7)

(OR)

23. 	a. 	In the case of disubstituted  cucylohexane when a conformation is called cis or trans.	    (8)

b. 	Explain the effect of conformation on reactivity in cyclohexanes. 					    (7)

24. 	a. 	Write a note on ‘Exhaustive methylation degradation method’ as applied to find out the 	heterocyclic moiety present in the molecule of an unknown alkaloid.			    	  (10)

b. 	What are flavonoids? How are they classified?									    (5)

(OR)

25. 	a. 	Discuss the structural features of flavonoids.  				                        		    (8)

b. 	Write a short note on the structural features of Terpenoids.						    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	INORGANIC AND CO-ORDINATION CHEMISTRY	      Time : 3 hours

Subject Code:	12CH206										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the molecular orbital electronic configuration for N2- molecule?

2.	Is AlCl3 a Lewis acid or base? Give reasons for your answer.

3.	Which one is fissionable? U235or  U238

4.	Fill up the missing entities             ? U?    +  0 n1    ? Kr?   +  56Ba139  + ?

5.	Name one ore of vanadium with formula.

6.	What are transuranium elements?

7.	Find out the coordination number of central metal atom in the given complex Na2[Pt(CN)4].

8.	Name the complex [Al(CN)6]Br3

9.	Vitamin B12 present in _________ part of our body.

10.	Give one example for metallocene.

PART – B  (5 x 3 = 15 MARKS)

11.	Account for the increase in bond angle in the following compounds in the given order?              H2O < NH3 < CH4

12.	Give three differences between nuclear and chemical reactions.

13.	Name three actinides with the name of Scientists.

14.	Find out the type of isomerism involved in the following complexes and how will you identify the anions.[Co(NH3)5Br]SO4  and  [Co(NH3)5 SO4] Br

15.	What is Wilkinson’s Catalysts? Give its molecular formula.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Draw the MO diagram for O2 molecule, give its bond energy and bond order.      	  (12)

b. 	Arrange the following molecules in the increase order of bond energy?

F2, F2-, F2+                                                                                                                                                           			      (3)

(OR)

17.	Use the VSEPR model describes the bonding in the following molecules according to VB theory: (a) chlorine trifluoride and (b) triiodide ion. Use orbital diagrams for the central atoms to show how hybridization occurs.

18. 	Explain in detail about nuclear reactor and its components.

(OR)

19.	a. 	Which equation is an example of artificial transmutation? Give reason.                		    (2)

92U238  2He4 + 90Th234

13Al27 + 2He4  15P30 + 0n1

6C14  7N14 + -e0

88Ra226  2He4 + 86Ra222

b. 	Calculate the packing fraction of Argon isotope, 40 Ar18. Mass is 39.96amu. What does the value of packing fraction imply?                                                                          		    (5)

c. 	The observed mass of 26Fe 56 is 55.9375amu. The masses of one proton, neutron and

electron are 1.00732, 1.00866 and 0.0005486 amu respectively. Calculate the binding

energy per nucleon in Mev and in Joules.                                				  	            (8)

[P.T.O.]

20.	a.	Give in detail the extraction, isolation and purification of thorium metal. 	                 (10)

b. 	Give the application of chloroplatinic acid.                                                                  	    (3)

c. 	Even though titanium occurs as 9th most abundant element it is very costly, why?    	    (2)

(OR)

21.	a. 	Give in detail the extraction, isolation and purification of uranium metal.              	  (10)

b.	Give the uses of titania.                                                                                    			    (5)

22.	a. 	Name the following complexes                                                                                 	    (4)

i.	[Al(H2O)6]Br3

ii.	[Cr(NH3)6]Cl3

iii.	K3[Fe(F)6]

iv.	[Zn(OH)4]-2

b.	Write the oxidation state of the central metal ion.                                           		    	    (2)

i.	K[Cu(Br)2]

ii.	K2[Ni(CN)2]

c.	Explain the stability of complexes on basis of the central metal atom?                  		    (6)

d.	What is chelate effect? Give example.				                  					    (3)

(OR)

23.	a. 	Give the formula for the following complexes.                                                 		    (2)

i.	tetraamminecopper (II) sulphate.

ii.	pentacarbonyliron (0)

b. 	EDTA – what type of ligand it is?                                                                  			    (1)

c.	Explain the splitting of d-orbitals in octahedral and tetrahedral field using crystal field 	theory. 							             										  (12)

24. 	a. 	Explain the mechanism involved in using Zieglar Natta catalyst in the polymerization of      		alkenes?                                                                 					     				  (10)

b. 	Briefly discuss platinum anticancer drugs.				    						    (3)

c.	What is 18 electron rule?   					                						    (2)

(OR)

25. 	Give the formula of Wacker’s catalyst and state its few applications in synthesis. Give the entire mechanism involved in using this catalyst, along with the advantage of the reaction.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ORGANIC REACTIONS AND MECHANISMS		      	     Time : 3 hours

Subject Code:	12CH207								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	How is banzyne intermediate is expressed?

2.	What are ambident nucleophiles?

3.	Write Hammett equation.

4.	Write an example for nitration reaction in aromatic compounds.

5.	State Bredt’s rule.

6.	What is the formula for diborane? Where it is used?

7.	Finely divided copper is used as a reagent in ____________.

(a) Gattermann reaction	(b) Sandmeyer reaction 	(c) Reimer-Tiemann reaction

8.	In Wolff-Krishner reduction, carbonyl group is converted into ____________.

9.	The reagent used for Fries rearrangement is ____________.

10.	The reaction of benzylic quartenary ammonium salts with alkali-metal amides is called ____________ rearrangement.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain Chichibabin reaction with an example.

12.	What is Stork-enamine reaction? What is its application?

13.	Explain Chugaev reaction with an example.

14.	Write a note on Clemmenson reduction.

15.	With an example, explain Steven’s rearrangement.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain about 								                  							(3x5)

a.	SNAr Mechanism 	b.	Bucherer reaction

c.	effect of reaction medium on aliphatic nucleophilic substitution reactions

(OR)

17.	a.	Write a detailed account on SN2 Mechanism.					     					   (8)

b.	What is meant by neighbouring group participation? Write two examples of non-classical carbocations.								     									    (7)

18.	a.	Explain the aromatic electrophilic substitution reaction mechanism. Write any two 	reactions.										   							  (10)

b.	What are activating and deactivating groups? Give examples.			     			    (5)

(OR)

19.	Explain the mechanism of the following reactions in aliphatic compounds.    			(8+7)

a.	Diazonium coupling reaction	b.	Acylation reaction

20.	a.	Describe the Electrophilic addition reactions to Carbon-Carbon double bonds. Discuss the 	stereochemistry in these reactions.					    							  (10)

b.	Write a note on Michael Addition.						      						    (5)

(OR)

[P.T.O.]

21.	a.	State and explain the Hoffmann and Saytzeff rules in elimination reactions.   		    (8)

b.	Write a detailed account on Hoffmann degradation reaction.			      			    (7)

22.	Discuss the mechanism of the following condensation reactions.           					(5x3)

a.	Aldol Condensation	b.	Perkin Condensation		c.	Dieckmann Condensation

(OR)

23.	a.	What are ylides and how are they prepared? Give examples.             		      		    (8)

b.	Write a detailed account on Wittig reaction. What are their applications?	      		    (7)

24.	Explain the following rearengement reactions with examples.				       	       (8+7)

(a) Baeyer- Villiger rearrangement	        (b) Benzil-Benzilic acid rearrangement.

(OR)

25.	a.	Discuss the mechanism of Curtius and Lossen rearangements. In what way they are similar 	to Hoffmann rearrangement?								 			    		  (10)

b.	Explain the Favorski rearrangement. What is the main intermediate formed during the reaction?										      							    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MATERIALS CHEMISTRY							      	     Time : 3 hours

Subject Code:	12CH208						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define the term orbital.

2.  	State the Hund’s rule of maximum multiplicity.

3.  	Define the term Unit cell.

4.  	What is meant by a reciprocal lattice?

5.  	Define metallic bonding.

6.  	What is meant by ductility?

7.  	How are polymers classified?

8.  	Define the term glass transition temperature.

9.  	What are ceramics?

10. 	How are ceramic materials classified?

PART – B  (5 x 3 = 15 MARKS)

11.  What is hydrogen bonding? Explain the types of hydrogen bonding.

12.  What is screw dislocation? Explain.

13.  What is metallography? Explain.

14.  Explain briefly about the crystalline nature of polymers.

15.  What is FRP? Mention its uses.

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain the following with suitable examples: (a) Ionic radius (b) ionization potential and

(c) Electron affinity.

(OR)

17. 	Briefly explain about: (a) leptons (b) quarks and (c) fermions.

18. 	Discuss about the different types of crystal systems with examples.

(OR)

19. 	Discuss about the following crystal defects: (a) Point defects (b) Line defects and (c) planar defects.

20. 	What are alloys? Discuss briefly about the following: a) base metals b) Ferrous metals

c) Noble metals and d) precious metals.

(OR)

21. 	Elaborate in detail about the following properties of the metals (a) thermal spray

(b) Case hardening and (c) plating

22. 	a.	Explain the free radical type of mechanism of polymerization. 						    (9)

b.	Discuss the method of preparation and applications of silicones. 					    (6)

(OR)

23. 	Write down the methods of preparation, properties and uses of (a) PVC (b) Bakelite and                      (c) Polyethylene.

24.  	What are polymer composites? How are they classified? Give uses of polymer composites.

(OR)

25. 	Explain the mechanical and electrical properties of ceramics.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	BIOCHEMISTRY									      	     Time : 3 hours

Subject Code:	12CH209						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Give one example for Simple amino acid.

2. 	Define Zwitterion.

3. 	Give one example for polysaccharide.

4. 	Define simple lipids.

5. 	What are proteolytic enzymes?

6. 	Define enzyme.

7. 	Define metabolism.

8. 	What is meant by amphibolism?

9. 	Define Receptors.

10. 	What is meant by Glycolysis?

PART – B  (5 x 3 = 15 MARKS)

11. 	Explain Isoelectric point.

12. 	Write a short note on Spingolipids.

13. 	Give the nomenclature of enzymes.

14. 	Write a short note on Bioenergetics.

15. 	Explain Glycolysis.

PART – C  (5 x 15 = 75 MARKS)

16. 	Discuss in detail about the structure of proteins.

(OR)

17. 	Discuss briefly about Separation and Purification of Amino acids and Proteins by Gel		filtration Chromatography and Gel Electrophoresis.

18. 	Write a brief notes on Classification of lipids and Carbohydrates.

(OR)

19. 	Write detailed notes on (i) Polysaccharides and (ii) Conformation of furanose and pyranose 	rings.

20. 	Explain briefly about specificity of enzyme action using Lock and key and induced fit theories.

(OR)

21. 	Derive Michaelis-Menten Enzymatic rate equation.

22.  	Discuss in detail about the role of hormones and signal transduction in the metabolic pathways.

(OR)

23. 	Write a short note on (i) Oxidation of Carbon fuels and (ii) Metabolic pathways with                                                                                                                                                                                                  recurring motifs.

24.  	Discuss in detail about an overview of amino acid metabolism.

(OR)

25. Write a short note on (i) Glycogenolysis (ii) Glyconeogenesis and (iii) Intracellular receptors.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	CHARACTERIZATION AND INSTRUMENTAL TECHNIQUES													          	     					             Time : 3 hours

Subject Code:	12CH210						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Distinguish between SEM and TEM analysis.

2.	State the role of EXAFS for a research sample.

3.	Define reflection high energy electron diffraction.

4.	Mention the advantages of dynamic light scattering technique.

5.	Give the selection rule for all the transitions in an anharmonic oscillator.

6.	Name any two types of detectors used in IR spectroscopy.

7.	Write the various types of transitions in organic molecules and its order of energy level observed in UV spectroscopy.

8.	Among 1,2-diphenyl ethylene and 1,3-diphenyl butadiene, which one will have greater λmax and why?

9.	What is meant by X-ray photoelectron spectroscopy and give its use.

10.	Define nanotribology. Write the major tool used in its study.

PART – B  (5 x 3 = 15 MARKS)

11.	Write short notes about scanning tunneling microscopy.

12.	Explain reflected electron energy loss spectroscopy.

13.	Calculate the number of fundamental vibrational frequencies for the following molecules:

a.	Carbondioxide.

b.	Benzene

c.	Water.

14.	Explain chromophore and auxochromes with suitable examples.

15.	Write an informative note about quartz crystal microbalance.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Discuss in detail about the various types of X-ray detectors used in X-ray diffractometers.

(5)

b.	Write elaborately about the principle and instrumentation behind EXAFS technique.

(10)

(OR)

17.	a.	Write an informative note about the principle and instrumentation of TEM analysis.

(3+7)

b.	Indicate the advantages of EXAFS technique.									    (5)

18.	a.	Explain the low energy electron diffraction technique.							  (10)

b.	Briefly explain the principle and salient features of dynamic light scattering technique.(5)

(OR)

19.	a.	Write short notes about the electron energy loss spectrometer.						    (8)

b.	Explain the principle and advantages of neutron diffraction technique.				(4+3)

[P.T.O.]

20.	a.	Discuss about the concept of diatomic molecule as anharmonic oscillator.			    (8)

b.	Write elaborately about the instrumentation and working of IR spectroscopy.	 	    (7)

(OR)

21.	a.	Explain why we should consider the diatomic molecule as an anharmonic oscillator in 	vibrational spectroscopy.														    (6)

b.	Write briefly about the applications of IR spectroscopy.							    (9)

22.	a.	Explain the application of woodward fieser rule for any two conjugated dienes.		    (6)

b.	Describe the principle and instrumentation of UV visible spectroscopy.				    (9)

(OR)

23.	a.	Mention the factors affecting absorbance.										    (3)

b.	How absorption spectra arise? Explain.											    (6)

c.	Write the applications of UV visible spectroscopy.								    (6)

24.	a.	Write short notes about nanotribometer.				        						    (7)

b.	Elaborately explain the ESCA technique.										    (8)

(OR)

25.	a.	Describe Friction force microscope.												    (5)

b.	Discuss in detail about the principle and instrumentation of ultraviolet photoelectron 	spectroscopy.																  (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MEDICINAL CHEMISTRY							      	     Time : 3 hours

Subject Code:	12CH211						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A prodrug is an inactive compound that requires __________.

2.	Drug interactions with targets are known as __________.

3.	Mention an example of a neurotransmitter?

4.	__________ binds to DNA by inserting themselves between the stacked base pairs.

5.	__________ exists mainly in the liver and catalyze the reaction.

6.	Polar drugs are unable to cross __________ unless they make use of a carrier protein.

7.	Isotopes commonly used for labeling studies in drug metabolism are __________ and __________.

8.	Minerals required by the body are __________ and __________.

9.	Name a drug, which was accidentally discovered?

10.	What is ab initio synthesis of drugs?

PART – B  (5 x 3 = 15 MARKS)

11.	Mention any two classifications of drugs.

12.	Differentiate between competitive and non – competitive inhibitors.

13.	Explain briefly High – throughput screening.

14.	What is a metal cluster? Give an example.

15.	List any two factors affecting drug development.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Briefly trace the history of medicinal chemistry.				             				    (8)

b.	How the use of computers has revolutionized the drug design?			 			    (7)

(OR)

17.	a.	What is a soft drug? How does it differ from a pro drug?							    (7)

b.	Explain the various concepts taken into consideration for drug design. 				    (8)

18.	a.	What is the role played by the enzymes?											    (3)

b.	Write a note on reversible and irreversible inhibitors.								(6+6)

(OR)

19.	a.	State Paton’s rate theory?														    (3)

b.	Discuss the effects of PH, PKa and polarity on the solubility of a drug?				  (12)

20.	Write short notes on 																(7+8)

a.	Testing drugs in vivo  	b.	Testing drugs in vitro

(OR)

21.	Discuss the Metabolism and Excretion process of a drug? 								(8+7)

[P.T.O.]

22.	a.	What is a convergent synthesis? What are its advantages? 							(4+3)

b.	Write a note on drug action with the help of metal chelation? 						    (8)

(OR)

23.	Describe the various phases involved in pre clinical testing & clinical trials?       		    (5+10)

24.	Explain the following procedures in drug design

a.	Isolation of bioactive compounds.												    (5)

b.	Examination of metabolites.													    (5)

c.	Interference with fundamental life processes. 									    (5)

(OR)

25.	Discuss in detail any three methodologies of molecular modification of lead compounds with suitable examples? 															       (3x5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ANALYTICAL CHEMISTRY AND SPECTROSCOPY    	     Time : 3 hours

Subject Code:	12CH212						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The mobile and stationary phase in HPLC is _________ and  __________.

2.	Oxygen is unsuitable as a carrier gas for Gas chromatography, why?

3.	13C NMR is active, whereas 12C NMR is not. Give reason?

4.	Chemical shift of CDCl3 in 13C NMR will be observed at __________ ppm.

5.	Give any two detectors used in confocal microscope.

6.	If a compound has even number of nitrogen then its molecular mass will be __________.

7.	What is meant by base peak?

8.	How many fragments are formed from 1-Butanol?

9.	Write Bragg’s equation.

10.	Give any two problems associated with crystallisation.

PART – B  (5 x 3 = 15 MARKS)

11.	Write a short note on axial haloketone rule.

12.	How many kinds of protons are there in the following compounds?

a.	CH3-CH2-CH2-CH2-CH3          b.	            c.	 CH3-CH2CH3OH

13.	Briefly explain fluorescence correlation spectroscopy.

14.	What is metastable ion peak? Give an example.

15.	What are the applications of  X- ray crystallography?

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Discuss the principle, instrumentation and application of gas chromatography.		    (7)

b. 	Write short notes on: i) cotton effect,        ii) octant rule                         				    (8)

(OR)

17.	a. 	Give the qualitative and quantitative application of coloumn chromatography.		    (8)

b. 	Out line the basic principle of Thin layer chromatography?        					    (5)

c. 	Name the 2 most widely used detectors in HPLC?	                    					    (2)

18.	a. 	What is chemical shift? Explain the factors affecting chemical shift.       			    (8)

b. 	What is proton decoupling in 13C NMR spectroscopy? Explain it with an example.	    (7)

(OR)

19.	a. 	What is spin-spin coupling? Explain with an example.                                     		    (7)

b. 	Explain why TMS is used as a standard in NMR Spectroscopy	.	              	  		    (3)

c. 	Write short notes on FT – NMR?							     					    (5)

20.	a. 	Explain using the Jablonski diagram the various photophysical processes that occur      	during absorption and emission.							    					  (10)

b.	Explain the FRET techniques and its application. 	     			     				    (5)

[P.T.O.]

(OR)

21.	a.	Explain the instrumentation involved in a confocal microscope with a diagram.	   	  (10)

b.	Describe the FISH and its application?					                  				    (5)

22.	a. 	Write about the Retro Diels Alder rearrangement in Mass spectra.		     	 		  (10)

b. 	Give the fragmentation pattern of an Aromatic compound.       	  	                          	    (5)

(OR)

23.	a.	What is Mc Lafferty Rearrangement? Explain with an example.		               	  (10)

b.	Write about the fragmentation pattern of aldehyde.				     				    (5)

24.	a. 	What are the problems associated with growing  biomolecular crystals? 	     		    (7)

b. 	Discuss the instrumentation of XRD and its application.				     			    (8)

(OR)

25.	a.	What is meant by isomorphic replacement in solving crystal structure? 		    		    (5)

b.	Explain the Protein crystallography?                                    		               		  (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	CHEMISTRY IN EVERYDAY LIFE					      	     Time : 3 hours

Subject Code:	12CH220										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define clinical chemistry.

2.	Give the name of virus responsible for polio.

3.	In perfume industry a ‘nose’ is called ____________.

4.	By using which reagent, the common group is incorporated in explosives like TNT, Gun Cotton and RDX.

5.	What is ‘Umami’ taste?

6.	Give the chemical structure and name of aginomotto.

7.	The burning sensation of chilly is due to __________.

8.	The maximum % of air in branded and quality ice cream should be __________.

9.	Reason out- Why organic foods are costly?

10.	What is the range of Fluoride in tooth paste?

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between: antiseptic, sterilization and disinfection?

12.	Write a note on preventing diabetes through proper diet and stress free life?

13.	The foaming in tooth paste is due to what? How much % of that is allowed?

14.	Give reason. Kitchen gas burner burns yellow when a pot of boiling water overflows?

15.	What is a balanced diet?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Write short notes on diagnosing the diseases through fluid samples of our body?

(Blood, urine)          														      (4+4)

b.	Give the name of the first antibiotic and its inventor. Give the general function of an antibiotic? 																	(2+2)

c.	 Give the structure and chemical names of aspirin and its function.                     		   (3)

(OR)

17.	a.	What is AIDS? Explain its causes, sign and symptoms.                                                (9)

b.	The common cold & cough drugs like Vicks Action 500, and D’Cold and Analgin (Baralgon) are banned in India. Mention the ill effects of using these drugs.                (6)

18.	a.	Give two names of natural dyes and give its significance and disadvantage compared to the 	synthetic dyes.                                                                                        				   (5)

b.	Explain the chemistry of perfumes highlighting their classifications.   				  (10)

(OR)

19.	a.	Discuss on the chemistry involved in ice cream making.           						    (9)

b.	What is liquid nitrogen ice cream?                                                                                     (2)

c.	Write a short note on hair dye.                                               						    (4)

20.	a.	What are the various ingredients of tooth paste and its role?                  				    (7)

b.	Write a short note on: Microban technology		                                       			    (3)

c.	 AlCl3 acts as antiperspirant.                                                                   				    (5)

[P.T.O.]

(OR)

21.	Write the Advantages and Disadvantages of using the following as food items

a. 	aginomotto

b. 	lycopene

c. 	polyphenols in Tea

d.	Aspartame

e. 	chloral hydrate																(5x3)

22.	a.	Give reason for the following?

i. 	Peeling of oinons irritate eyes.

ii. 	Puff pastries expand when baked.

iii.	Skin feel cooling effect when cosmetic cream is applied.						(4x3)

b.	 Is it good firing crackers, justify your answer?									    (3)

(OR)

23.	a.	Explain the chemistry involved in disappearing inks with chemical equations? 		    (9)

b.	Give reason

i. 	Flesh of the fish smells different than other meat.

ii. 	Why cotton fabrics absorb more water?										    (6)

24.	a.	How are the following food adulterated? Bring about the harmful effects of using them? 	What are the possible tests to identify these adulterations?

- Apple

- Banana

- Ghee																		    (6)

b.	Write short notes on certification mark of pulses, oil and fruits and why is it needed.  (5)

c.	Write down the ill effect of consuming alcohol? 									    (4)

(OR)

25.	a.	i.	What do you mean by organic food?

- Write a note on PAH?

- What is iodized salt? Reason out the need for it?								(3x3)

b.	Write a note on molecule of happiness?											    (3)

c.	How do dopamine play a role in addiction to various fun killers like drugs, alcohol,

porn etc?																	    (3)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	ORGANIC CHEMISTRY – II						       Time : 3 hours

Subject Code: 	12CH304								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a. 	Discuss about the synthetic importance of organocuprates. 						(8)

b. 	How can one transfer the following by using wittig reaction? 						(8)

i) CH2CH(CH2)4CHCH2	ii)

c. 	How will you synthesis indole. 												(4)

(OR)

2.	Write the mechanism of the following reactions. 									  (5 x 4)

a. 	Michael addition, 	b. 	Stork-enamine reaction, 	c. 	Mannich reaction,

d. 	Barton reaction, 		e. 	oppeauer oxidation

3.	a. 	Write the structure of the reaction product from succinimide with Br2 and aqueous KOH. 																					 (5)

b. 	What product one expect from the Baeyer-Villiger oxidation. 						 (5)

c. 	Predict the product of the reaction and give the mechanism. 						 (5)

d. 	Why the trans-decalin undergoes ring contraction on treatment with acid? 			 (5)

(OR)

4.	Predict the product and the mechanism.  										    (4x5)

a.				c.

b.								d.

5.	a. 	Describe the Norrish type I and II reactions. 									      (10)

b. 	Discuss about the photosensitization and quenching. 							      (10)

(OR)

6.	 Discuss the following

a. 	Paterno-Buchi reaction 			b. 	Cis-trans isomerisation

c. 	Zimmerman rearrangement 		d. 	Photochemical elimination

[P.T.O.]

7.	a.	Explain the fate of [A] in case of the following pericyclic reaction with correlation 	diagram.

[A]

i. 	Photochemical electrocyclic closure. 									      (10)

ii. 	Thermal cycloaddition reaction with ethylene. 								(5)

b. 	Write the product of thermal sigmatropic reaction of   							(5)

(OR)

8.	a. 	Using frontier orbital overlap explains why diels-Alder reaction between butadiene and 	ethylene is thermally allowed but is not catalysed by UV light. 				      (10)

b. 	Predict the product of the following reactions. 								      (10)

i)		ii)

9.	Compulsory :

a. 	Explain one group and two group disconnections with examples. 				      (10)

b. 	Give the synthetic equivalents for the following synthons. 					      (10)

i)	ii)   		iii) -CH2CH2CH2-		iv) CH2CH-	v)   -CH2CH2-

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	ANALYTICAL CHEMISTRY - II					       Time : 3 hours

Subject Code: 	12CH314								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a. 	Explain why the aromatic proton is more shielded than the ethylenic proton although 	both the types of protons are attached to sp2hybridised carbons. 					(5)

b. 	PMR spectrum of dibromopropane shows a quintet and downfield triplet of about twice the intensity. Is the isomer 1,1-  1,2-  1,3- or 2,2- dibromopropane? Support your answer by suitable reasoning.  														(5)

c. 	Explain briefly the basic theory of NMR spectroscopy. 						      (10)

(OR)

2.	a. 	How many NMR signals would you expect from cyclohexane at -600C? Explain. 	(5)

b. 	What are enantiotropic and diastereotopic protons? Explain.  Why the former have the same but the latter have different chemical shifts? 								(5)

c. 	Discuss the principles of 2D –NMR. 										      (10)

3.	a. 	Cyclooctatetraene has a conjugated eight carbon ring system. How its PMR spectrum can 	help in deciding if the compound is aromatic or not? 								(5)

b. 	How can the two isomeric hydrocarbons with molecular formula C3H4 are distinguished 	by 13C NMR. 																(5)

c. 	Discuss about Kramer’s degeneracy and zerofield splitting. 					      (10)

(OR)

4.	a. 	Explain briefly the factors affecting 13C chemical shifts. 							(8)

b. 	Discuss the ‘g’ value and factors affecting ESR lines. 							(7)

c. 	How many signals are expected for ClF3 and ClF5?								(5)

5.	a. 	How would you distinguish among ethylamine, diethylamine and triethylamine on the 	basis of their mass spectral studies? 											 (8)

b. 	Explain with suitable illustrations the significance of retro Diels-Alder fragmentation in mass spectral analysis. 														 (7)

c. 	Define base peak in mass spectrum. Is it true that molecular ion peak should always be the base peak?																 (5)

(OR)

6.	a. 	Explain McLafferty rearrangement taking the example of CH3CD2CH2CH2COCH3. 	(7)

b. 	Discuss briefly the factors affecting fragmentation. 								(7)

c. 	Predict the fragmentation pattern of diphenylether. 								(6)

7.	Compulsory :

a. 	Explain the MB spectra of spimn free and spin paired Fe complexes. 				(8)

b. 	Explain in detail the photo ionization process. 									(6)

c. 	Write short notes on Quadrupole effect. 											(6)

8.	a. 	Explain the principle, procedure and applications of DTA. 					      (10)

b. 	What are the factors affecting TG curves. 										(6)

c. 	Discuss SEM. 																(4)

(OR)

9.	a. 	Explain the principle, procedure and applications of DSC. 					      (10)

b. 	What are the factors affecting DTA curves. 										(6)

c. 	Discuss TEM. 																(4)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	POLYMER CHEMISTRY							       Time : 3 hours

Subject Code: 	12CH315								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	What are conducting polymers and composites? Explain the applications in sensors and 	batteries.

b.	Describe the advantages and limitations of nanofillers.

c.	Give a brief account on filler matrix interaction.              						(7+7+6)

2.	a.	Discuss the cross linking and cyclisation reactions of polymers.

b.	Explain the Flory-Huggins theory of polymer solutions.  						(10+10)

(OR)

3.	a.	Write about the cellucose and its derivatives.

b.	Discuss the structure-property relationship in polymers.

c.	Write notes on ‘Biopolymers’                                           						(7+7+6)

4.	a.	Describe coordination polymerization with an example.

b.	Explain the characteristics of step growth polymerization.

c.	What is the effect of inhibitors or retarders on polymerization?

(OR)

5.	a.	Discuss the mechanism of polyaddition polymerization.

b.	Explain the reactivity of functional groups in polymers.

c.	Write briefly on fractional copolymerization.             							(8+6+6)

6.	a.	How is molecular weight of a polymer determined by any one method?

b.	Explain the geometrical isomerism in polymers.

c.	Describe the electric effect of polymers.                    							(7+7+6)

(OR)

7.	a.	What is glass transition temperature? What is its importance? Explain the factors 	influencing Tg.

b.	Describe the thermal behaviour of polymers.

c.	Outline the concept of degree of crystallinity. What are the factors affecting 	crystallinity?            														(8+6+6)

8.	a.	Explain the bulk and emulsion polymerization processes.

b.	How are the following prepared?

i) Polystyrene    ii) PTFE

c.	What are the additives added in the polymerization processes? What is their 	significance?                                       									     (10+5+5)

(OR)

9.	a.	Discuss the injection and extrusion moulding

b.	How are nylon 66 and PET prepared?

c.	Write notes on compounding of plastics.            							     (10+5+5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	ADVANCED PHARMACEUTICAL CHEMISTRY	       Time : 3 hours

Subject Code: 	12CH316										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

Outline the synthesis and mechanism of action of

a.	Idoxuridine.																      (10)

b.	Captodiame.																      (10)

2.	a.	Explain the classification of drugs based on their pharmacological effect?			(4)

b.	Write short notes on:

i.	QSAR																	(8)

ii.	Chirality in drug design.													(8)

(OR)

3.	a.	Differentiate between a prodrug and a soft drug.									(8)

b.	Enumerate the various aspects that are taken into consideration while designing a drug.

(12)

4.	a.	Differentiate between

i.	Competitive & non – competitive inhibitors.

ii.	Reversible & Irreversible inhibitors.                                           				(8)

b.	State Clark’s theory.															(2)

c.	Write a note on DNA – drug interaction. 									       (10)

(OR)

5.	a.	Write suitable examples highlight the effect of Pka and polarity on drug solubility.	(8)

b.	What are chemical messengers? Give an example.								(4)

c.	Discuss the role played by (i) DNA intercalators and (ii) Drug smugglers.			(8)

6.	a.	What are the ideal requirements of a drug?										(4)

b.	Describe the mechanism of drug action by

i.	Absorption and 	ii.	Elimination										      (10)

c.	Write a note on High – throughput screening.									(6)

(OR)

7.	a.	Explain ‘Therapeutic index’.													(4)

b.	Discuss the mechanism of drug action by

i.	Metabolism and 		ii.	Distribution									      (10)

c.	Write briefly on testing of drugs in vitro.										(6)

8.	a.	Discuss the factors affecting drug development.									(5)

b.	What are Analgesics? Explain their classification and mechanism of action with few examples.																(3+6+6)

(OR)

9.	a.	What are Anesthetics? How are they classified?									(3)

b.	Describe the various methodologies adopted for molecular modification of lead 	compounds.   															  (4+13)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	MOLECULAR AND MATERIALS SELF-ASSEMBLY	Time : 3 hours

Subject Code: 	12CH319										   Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	What are the driving forces behind molecular and materials self-assembly? 	     (5+5+10)

b.	Discuss the five principles that need to be taken into consideration in forming self-assembled structures.

c.	Illustrate with examples (i) self-assembled monolayers and the methods of analyzing them and (ii) SAM crystal engineering.

2.	a.	Discuss organic polyelectrolyte multilayers.                           				     (5+5+10)

b.	What are electrostatic superlattices?

c.	Give a brief account of polyelectrolyte–colloid multilayers, citing out examples.

(OR)

3.	Give a detailed account of (a) patterned multilayers (b) non-electrostatic LbL assembly.

(10+10)

4.	a.	Discuss modulated diameter gold nanorods

b.	Describe self-assembling nanorods with an illustration.					       (10+10)

(OR)

5.	a.	Illustrate and explain magnetic nanorods bunching up.						      (10)

b.	Discuss elaborately a case of 	hierarchical ordering of nanorods.				      (10)

6.	Discuss the following:

a.	Synthesis of capped semiconductor nanoclusters.								      (10)

b.	Electrons and holes in nanocluster boxes.									      (10)

(OR)

7.	Illustrate and explain (a) capped gold nanoclusters (b) water soluble nanocluster (c) core-shell magnetic nanoclusters	.				    									     (10+5+5)

8.	Illustrate and explain making micelles and harnessing rig rods of block copolymers.

(OR)

9.	Discuss elaborately block co-polypeptide self assembly.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	CHEMICAL APPROACH TO NANOMATERIALS		Time : 3 hours

Subject Code: 	12CH320										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Explain forces involved in the formation of Self-assembling Supra-micron Shapes with   capillary bond method. 														(8)

b.	Define autonomous self-assembly method, write difference between autonomous self-assembly and dynamic self-assembly. 											(7)

c.	Describe the steps involved in the formation of polyhedra with patterned faces. 		(5)

2. 	a. 	Write detailed account on defect patterning through topologically directed etching. 	(8)

b. 	Explain the enzymatic nanolithography using dip-pen nanolithography.                   	(8)

c. 	Define SPM nano electrochemistry. How it is differ beyond DPN Nanostructures?    	(4)

(OR)

3.    a. 	Describe feature of microcontact printing by living bacteria.      				      (12)

b. 	Discuss the principle of nanoscale patterning of PDMS stamps with DPN way. 		(8)

4.  	a. 	Write a detailed account on basics of microsphere self-assembly with different crystals   	formation.																      (10)

b.     Explain Combined Bragg–Snell laws for optical properties of colloidal crystals.  	      (10)

(OR)

5.   	a.  Write short notes on:

i.	Thermochromic colloidal photonic crystal switch.

ii.	Liquid crystal photonic crystal.    									    	   (5 +5)

b. What are internal light sources? How it is differ from photonic inks? Explain.       	     (10)

6.	a. 	Explain mesomorphology with patterned films, soft lithography, micromolding.        (10)

b.  	Discuss the synthesis of mesoporous silica through in situ functionalisation  and post-	synthetic functionalisation. 												      (10)

(OR)

7.  	a.  	Write a detailed account on the mesoscale building blocks. Discuss the factors that 	decide the geometry of the microspheres.      			            					     (10)

b.   Write short notes on mesoepitaxy.											 	(5)

c.  	Explain gases absorbtion by porous coordination polymers.						(5)

8.    a. 	Write short note on Bacteria directed materials self-assembly. Explain intramolecular

self- assembly and intermolecular self-assembly. 								      (12)

b.  	Discuss basic principle and application of Faux fossils. 							(8)

(OR)

9.    a.  	Explain Biological functions of the S-layers										(7)

b.  	How natural history affected by Siliceous sculptures? 							(7)

c. 	Describe Bone replacement and repairing through better bone synthesis by chemical 	method. 																		(6)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	MOLECULAR MACHINES AND MATERIALS		       Time : 3 hours

Subject Code: 	12CH321									        Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	How are fullerenes synthesized?

b.	Explain fullerenes as super conducting intercalation compounds.

c.	Give a brief account on epitaxial growth on silicon carbide.         				(6+8+6)

2.	a.	Discuss the various molecular electronic devices.                           				      (10)

b.	Write briefly about 	(i) replicating nanorobots

(ii) molecular shuttle                                        			    (5+5)

(OR)

3.	a.	Describe the smart materials and nanosensors.                                 				      (10)

b.	Explain the following:

(i) molecular logic gates

(ii) molecular propeller                                                                    				      (10)

4.	a.	Discuss the various dentritic  photochemical   devices.

b.	How are dendrimers classified?

c.	Write the method of synthesis of dendrimers.                                   				(8+5+7)

(OR)

5.	a.	Explain the properties and application of dendrimers.

b.	Give a detailed account of types of dendrimers.

c.	Write about the divergent dendrimer growth and convergent dendrimer growth.  (8+6+6)

6.	a.	How are nanotubes synthesized?

b.	Explain the usage of nanotubes in electrical circuits.

c.	Discuss the kinetic, thermal and electrical properties of carbon nanotubes. 		(6+6+8)

(OR)

7.	a.	How are cabon nanotubes classified?

b.	Describe the structural, electro-acoustic and electromagnetic applications of nanotubes.

c.	Explain the defects in carbon nanotubes.                                      			     (5+10+5)

8.   	a.	“The zero dimensional nanomaterials are referred as Quantum dots”. Justify.

b.	Explain the application of quantum dots in computing.

c.	How do the nanowires and wells differ in their shapes?                         			(7+7+6)

(OR)

9.   	a.	Highlight the electrochemical assembly of quantum dots.

b.	With any one example, explain the fabrication technique of quantum dots.

c.	Discuss the application of quantum dots in photovoltaic devices.         			(7+7+6)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	ANALYTICAL CHEMISTRY						       Time : 3 hours

Subject Code: 	14CH3004								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Describe the relaxation processes in NMR.									      (10)

b.	What is FT-NMR? Explain it briefly.										        (5)

c.	A compound with molecular formula C8H8O gives the PMR spectrum

Multiplet (δ 7.28), 5H		Doublet (δ 2.8), 2H		Triplet (δ 9.78), 1H

Give the structural formula.													(5)

2.	a. 	Discuss the migration parameters in paper chromatography.					      (10)

b.	Mention any five applications of Thin layer chromatography.						(5)

c.	What is column packing in HPLC? Explain it.									(5)

(OR)

3.	a.	Explain the quantitative analysis of gas chromatography.						      (10)

b.	What are various methods of preparation of thin layers in platee (TLC)? 		      (10)

4.	a.	List out the factors that influence vibrational frequencies of molecules with a suitable 		example.																      (10)

b.	What is the fingerprint region? How it is helpful for IR analysis?		      			(5)

c.	Write a note on selection rule.													(5)

(OR)

5.	a.	Write a short note on i) Factors affecting column efficiency and ii) applications of 	column chromatography													      (10)

b.	Explain the fundamental vibrations and Overtones in IR spectroscopy			      (10)

6.	a.	List out any five applications of UV spectroscopy.								(5)

b.	Describe the Woodward Fisher rule for the calculation of absorption maxima in , -   unsaturated carbonyl compounds with an example.			 		      		      (15)

(OR)

7.	a.	Discuss the absorption and intensity shifts in UV spectroscopy	.				      (10)

b.	Explain the different types of electronic transitions with an example.	      		      (10)

8.	a.	A compound having molecular formula C9H11Br showed the following signals in PMR 	data

Multiplet (δ 2.25), 2H		Triplet (δ 2.75), 2H

Triplet (δ 3.38), 2H		Singlet (δ 7.22), 5H

Assign the structure of the compound giving reasons.								(6)

b.	Discuss the factors influencing the chemical shift.							      (10)

c.	Why TMS is used as a standard in NMR Spectroscopy?							(4)

(OR)

9.	a.	A compound with molecular formula C6H12O2 shows four signals:

Singlet (δ 1.1), 6H		Singlet (δ 2.1), 3H

Singlet (δ 2.6), 2H 		Singlet (δ 3.9), 1H

Propose a structure consistent with the given data.							      (10)

b.	Describe the spin-spin coupling in AX type spin system with suitable example.	      (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	SUPRAMOLECULAR CHEMISTRY				       Time : 3 hours

Subject Code: 	14CH3007								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	What do you mean by selectivity? Give the examples.

b.	State the principle of Lock and Key and explain it.

2.	a.	Write a detail about the induced fit model.

b.	Difference between Lock and Key principle and induced fit model.

(OR)

3.	What is chelate effect? How cooperativity in supramolecular chemistry can be measured?

4.	Explain in detail about the solution host –guest chemistry.

(OR)

5.	Discuss the anion binding with suitable examples.

6.	Define self assembly. Describe the biological self assembly in supramolecular chemistry

(OR)

7.	Explain solid state supramolecules with suitable example.

8.	Discuss the supramolecular host-guest system in zeolites with suitable example.

(OR)

9.	Describe the Supramolecular coordination polymer. Give its applications.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	POLYMERS FOR NANOTECHNOLOGY			       Time : 3 hours

Subject Code: 	14CH3010								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	State the advantages of polymers over the metals.  								(4)

b.	Describe the principle, advantages and limitations of injection and blow moulding 	fabrication techniques of polymers. 										      (16)

2.	a.	Write an elaborate note on classification of composites according to reinforcements. (10) b.	Explain the role of interface in composites.									      (10)

(OR)

3.	With suitable examples discuss the compounding of plastics and its importance.

4.	Discuss the following polymer composite fabrication techniques                            	    (4x5)

a.	Hand lay up method.

b.	Bag moulding.

c.	Pultrusion.

d.	Filament winding.

(OR)

5.	a.	Discuss the structure and properties of various polymer matrixes.				      (10)

b.	With a neat sketch explain the working principle of compression moulding of plastics and its essential features. 													      (10)

6.	a.	What are nano fillers?  Give examples and write their advantages over conventional 	fillers.																	      (10)

b. 	Discuss the various surface treatments adopted for nano fillers. Also state their necessity  																				      (10)

(OR)

7.	a.	Discuss the various methods of preparation of polymer nano composites with suitable 		example. 																      (10)

b.	Classify CNT and discuss their structure and properties.						      (10)

8.	Compare the tribology of clay and CNT based polymer nonocomposites.

(OR)

9.  	a.	Explain various types of wear process taking place in polymer nano composites.       (10)

b.	Write an elaborate note on measurement of adhesive wear.                                   	      (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	APPLIED CHEMISTRY							      	     Time : 3 hours

Subject Code:	CH106							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is meant by Functionality of a polymer?

2. 	List the monomers used for the preparation of Nylon – 66.

3. 	What are the units used to express the hardness of water?

4. 	What is meant by alkalinity of water?

5. 	Define calorific value of the fuel.

6. 	The  antiknocking compound used in gasoline is __________.

7. 	Define: Fuel Cell

8.	What are the two types of corrosion?

9. 	 1 nm = ____m; ____Ao

10. 	What is meant by Fermentation?

PART – B  (5 x 3 = 15 MARKS)

11. 	Write a note on conducting polymers.

12. 	Write the sources of water. What are the impurities present in water?

13. 	State the importance of proximate and ultimate analysis

14. 	Derive Nernst Equation.

15. 	Discuss the properties of carbon nanotubes.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the free radical polymerization process.					       				    (8)

b.	Write a note on Ziegler-Natta Polymerization.					       				    (7)

(OR)

17.	With a diagram describe the following processes.									       (8+7)

a.	Compression Moulding 		b.	Injection moulding

18.	Describe the following water softening methods. 					 					(8+7)

a.	Lime soda process 			b.	Ion exchange process

(OR)

19.	Write a note on (a) Boiler Corrosion (b) Caustic embrittlement (c) Water for drinking purpose. 									          										   (5+5+5)

20.	a.	What are the characteristics of metallurgical coke.				       				    (5)

b.	Explain the manufacture of coke by Otto-Hoffmann method.			     			  (10)

(OR)

21.	a.	Define octane number. What is its significance?					       				    (3)

b.	Write  about water gas manufacturing process.					       				    (8)

c.	Write a note on biogas.								       						    (4)

[P.T.O.]

22.	Write a detailed account on (a) lechlanche cell  (b)Lead-acid battery               		     (5+10)

(OR)

23.	a.	Discuss the various factors influencing the corrosion.				     				  (10)

b.	Describe in detail any one corrosion control methods.				       				    (5)

24.	a.	What are the different types of nanomaterials? Give examples for each types.	           (5)

b.	What are the applications of nanomaterials?					    		 			  (10)

(OR)

25.	a.	Explain any one method for the manufacturing of ethyl alcohol and acetic acid by 	fermentation.									     								  (10)

b.	Write a short note on photolithography.                                                             		    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	ORGANIC CHEMISTRY - I							       Time : 3 hours

Subject Code: 	CH303									              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Discuss the principle of Asymmetric synthesis.									(5)

b.	Draw the most stable conformation of Cis 1-tert-butyl-4-methyl cyclohexane.		(3)

c.	Write a note on :

i.	Conformation analysis of Cyclohexane.										(6)

ii.	Conformation of Decalins.													(6)

2.	a.	What is Hammett equation? Why is it called LFER?								(4)

b.	What does the sign and magnitude of the ρ value imply? Illustrate with examples.	(8)

c.	Highlight the operation of the steric effect using specific examples. 				(8)

(OR)

3.	a.	Explain the rules of resonance by giving suitable examples.						(8)

b.	What is Taft equation? Explain the significance of each term in the Taft equation.	(6)

c.	State the Huckel’s rule? Give examples where Huckel rule is obeyed and violated.	(6)

4.	a.	What is a NGP reaction?														(3)

b.	State the evidences for the operation of a NGP reaction.							(5)

c.	Give examples of reactions in which ‘O’ and ‘S’ participate as a neighbouring group.(6)

d.	Outline the mechanism of aromatic substitution reaction proceeding via the benzyne 	intermediate.																       (6)

(OR)

5.	a.	Taking suitable examples, discuss the mechanism of nucleophilic substitution at vinylic 	and allylic carbon atoms.														(8)

b.	Compare and contrast the mechanism of SN1 and SN2 reaction. 					(6)

c.	What is the effect of substrate and reaction medium on the rate of SN1 and SN2 reactions?																	(6)

6.	a.	Write the mechanism for the following reactions  							      (12)

i.	Hydroxylation   ii.  Hydro halogenation    and 	 iii.	Hydrogenation

b.	What is E1cB mechanism? Illustrate with an example.							(4)

c.	Write a note on operation of Hoffman’s rule in elimination reactions.				(4)

(OR)

7.	a.	Discuss the orientation and reactivity that is in operation in mono substituted benzene 		rings.																		(8)

b.	Highlight the salient features of a E2 and E1 mechanistic pathway.  				(6)

c.	What are the factors that favor the operation of E1 and E2 mechanism?				(6)

[P.T.O.]

8.	a.	Differentiate between enantiomers and diastereomer.								(4)

b.	Comment on the statement “The presence of a stereomer is not a necessary and sufficient condition for molecular dissymmetry”.										      (10)

c.	Assign R & S descriptors to each of the stereo center present in,					(6)

(OR)

9.	a.	Assign absolute configuration to the stereogenic atoms in the compounds

- ii

(3+3)

b.	Write a short notes on

i.	Planar Chirality and 	ii.	Helicity										    (5+5)

c.	Write the Saw Horse and Newman Projection of the given compound

(2+2)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	ORGANIC CHEMISTRY - II						       Time : 3 hours

Subject Code: 	CH328									              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Choose alcohols as example and explain the synthetic importance with one group 	disconnection approach. 													      (12)

b.	Write down the disconnection scheme for a simple ketone and a simple carboxylic acid 	taking one example for each class. 												(8)

2.	a.	Write down the reaction mechanisms of

(i) Mannich reaction and (ii) Favorskii reaction                 						      (10)

b.	Elaborate in detail the synthetic applications of (i) LDA and (ii) DDQ 			      (10)

(OR)

3.  	a. 	State and explain the following name reactions:

(i) Enamine reaction and (ii) Sapiro reaction									      (10)

b. 	Bring out the synthetic applications of the following organic reagents with specific 	examples.

(i) Gilman’s reagent and (ii) OsO4 											      (10)

4 . 	With suitable examples, discuss about the following pericyclic reactions: (a) Electrocyclic

reactions (b) Cyclo addition reactions (c) cheletropic reactions and (d) the ene reactions.

(OR)

5. 	a.	 Explain about the stereochemistry involved in electrocyclic reactions. 				(6)                                                                                  b.	Give the mechanistic explanation for the electrocyclic reaction with FMO approach.           																				(8)

c.	With a suitable example, discuss about a simple (2+ 2) cheletropic reaction. 			(6)

6. 	a.	Discuss briefly about sensitization and quenching as applicable to photochemical 	reactions. 																      (10)

b.	Explain the different steps involved in the following reactions: (i) Paterno-Buchi reaction

and (ii) cis-trans isomerization reaction. 									      (10)

(OR)

7. 	Outline the following photochemical reaction mechanisms: (i) Norrish type-I reaction and

(ii) Zimmermann rearrangement.

8. 	What are pyrazoles and thiazoles? Discuss the preparation, and properties of at least TWO

pyrazoles and thiazoles.

(OR)

9. 	a.	What are zeolites?  Explain the role of zeolite as a solid adsorbent to carry out organic 	reactions with suitable examples.        										      (12)

b.	Discuss the synthetic applications of cyclodextrins. 								(8)