Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MICROWAVE AND OPTICAL COMMUNICATION ENGINEERING  													      	     					     Time : 3 hours

Subject Code:	09EC223								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Mention any two application of Directional Coupler.

2.	Define VSWR.

3.	What is the condition for oscillation in Reflex Klystron?

4.	TWT’s used in Radar and Microwave receivers are having _________ power and _________ noise.

5.	Operating Frequency of TRAPATT devices range from _________ to _________.

6.	Define Insertion Loss.

7.	What is Snell’s Law?

8.	Define acceptance angle.

9.	What are the types of Photodiodes?

10.	Define external quantum efficiency.

PART – B  (5 x 3 = 15 MARKS)

11. 	Why is S-matrix used in Microwave analysis?

12. 	What is the purpose of slow wave structures used in TWT amplifiers?

13. 	What are the factors reducing efficiency of IMPATT diode?

14. 	Define Numerical Aperture of a step index fiber.

15. 	Define Photo current.

PART – C  (5 x 15 = 75 MARKS)

16. Write technical notes on Isolators and Circulators.

(OR)

17. 	Give the technical details about E-plane , H-plane Tee and hybrid  Tee.

18. 	Describe in detail about 2- cavity Klystron Amplifier.

(OR)

19.  	Explain the working of Reflex Klystron with neat diagram.

20.  	What is negative resistance in GUNN diode? Describe the operation of GUNN Diode.

(OR)

21. 	Describe in detail about the power measurement.

22.	 Discuss about the step index fiber and graded index fiber.

(OR)

23. 	Write technical note on a) Dispersion method 	b) Polarization. 						(7+8)

24.	Explain in detail about the LED with neat diagram.

(OR)

25. 	Describe in detail about the principle and operation of LASER diode.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	NETWORK ANALYSIS AND SYNTHESIS			      	     Time : 3 hours

Subject Code:	EE295								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define zero of a network function.

2. 	Define transfer function.

3. 	Define immittance.

4. 	Give the importance of complex locus diagram.

5. 	Say true or false: Every tree has a co-tree.

6. 	Branches of tree are known as ___________.

7. 	Define Attenuator.

8. 	Unit of the attenuation constant is ___________.

9. 	In the second Cauer network, first element is a ___________ when the driving point

function consists of pole at zero.

10. 	Say true or false: All the quotients in the Hurwitz polynomials are positive.

PART – B  (5 x 3 = 15 MARKS)

11.	Give the properties of transfer function.

12.	Define phase plot.

13.	Define cut set.

14.	List the applications of attenuators.

15.	List the properties of positive real function.

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the parameters Z21(s) and G21(s) for the network given

(OR)

17.	For the network function, Vs=4s+2ss+1s+3.

Draw the pole zero diagram and hence obtain the time domain response v(t).

[P.T.O.]

18.	For a series RL circuit, (a)Write the expression of admittance function Y(s) in standard form.

(b)Show the phasor representation of

(c) Using the s-plane phasors obtained, sketch the complex locus of Y (jω)

(OR)

19.  Apply Routh criterion and determine the number of roots of the given polynomial (i) with

Positive real roots (ii) with zero real parts (iii) with negative real roots

Qs=s4+4s3+8s2+12s+15	.

20. 	Determine the fundamental loop matrix and cut-set matrix of the network given( each 1).

(OR)

21.	Determine the loop equations and branch currents using tie set schedule.

22.	Design a low pass filter (both T and π sections) having a zero frequency characteristic

impedance of 300 Ω and a cutoff frequency of 1 KHz.

(OR)

23. 	a.	Design and draw a π-type attenuator to give 40 dB attenuation and to have a characteristic 	impedance of 400 Ω.

b.	Design and draw a symmetrical lattice attenuator to have characteristic impedance of

600 Ω and attenuation of 30 dB.

24.   Find the first and second Cauer forms of the function, zs=2s2+8s+6s2+2s.

(OR)

25.   Find the Foster realizations of the function, 	zs=3s2+1s2+16ss2+9.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DIGITAL SIGNAL PROCESSING					      	     Time : 3 hours

Subject Code:	09EC224						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give an example for random signal.

2.	List two applications of FFT algorithms.

3.	List the well known design techniques for FIR filter design.

4.	What are the properties of FIR filter?

5.	What is prewarping?

6.	Write any one property of Chebyshev type –1 filters.

7.	What are the different formats of fixed point representation?

8.	What is rounding?

9.	List any two digital signal processing application with TMS 320 family.

10.	What are the disadvantages of Harvard architecture?

PART – B  (5 x 3 = 15 MARKS)

11.	Convolve the two sequences x(n)={1,2,3} and h(n)={5,4,6,2} and fill up the blank

y(n)={__, 14 ,__ ,__ ,__ , 6}.

12.	What are the advantages and disadvantages of Digital filters?

13.	Compare the Butterworth and Chebyshev Type-1 filters.

14.	What is meant by product quantization error?

15.	What are four phases available in pipeline technique?

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Determine the 4-point DFT of the given data sequence  xn=13,13,13.    			    (5)

b. 	Find the output y(n) of a filter whose impulse response h(n) = {1,1,1} and input signal

x(n) = {3,-1,0,1,3,2,0,1,2,1} using i) overlap-save method  ii) overlap-add method.  	  (10)

(OR)

17.	Compute eight point DFT of a sequence x (n) = {0, 1, 2, 3, 4, 5, 6, 7} using DIF algorithm.

18.	a.	What is meant by Optimum equiripple design criterion? Why it is followed?       		    (4)

b.	Write the procedure for FIR filter design by frequency sampling method             		    (4)

c.	Obtain the cascade realization of system function Hz=1+2z−1−z−21+z−1−z−2.          																				    (7)

(OR)

19.	Design an Low Pass Filter using rectangular window if N=9 whose desired frequency response is

[P.T.O.]

20.	Design a digital Butterworth filter with the following specification

0.8≤Hejω≤1,0≤ω≤0.2πHejω≤0.4,0.32π≤ω≤π

with T=1 sec. Apply impulse invariant transformation.

(OR)

21. 	a.	Explain the design procedure involved in designing a low pass digital Chebyshev filter. (5)

b.	For the analog transfer function, Has=2s+1s+2,determine H (z) if (i) T=1 sec and (ii) T=0.1 sec using bilinear transformation                                                                     (10)

22.	In the IIR system given below the products are rounded to 4-bits (including sign bit).

HZ=11−0.35z−11−0.62z−1. Find the output roundoff noise power in direct form realization.

(OR)

23.	a. 	Explain finite word length effects in realization of digital filters.                                     (10)

b. 	Write short notes:   (i) Quantization (ii) Scaling                                                                 (5)

24.	Discuss about adaptive noise cancellation using LMS algorithm in detail.

(OR)

25.	Draw and describe the features of Van Neumann and Harvard architecture.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December – 2015

Subject Title :	DIGITAL COMMUNICATION						      	     Time : 3 hours

Subject Code:	09EC225						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	__________ phenomenon will occur if sampling theorem is not satisfied.

2. 	ISI stands for __________.

3. 	List any two digital modulation schemes.

4.	Sketch the  Phasor diagram for output of BPSK method.

5. 	When the carrier signal is used in receiver for signal demodulation then the detection scheme is called as _________.

6. 	The main objective of matched filter is __________.

7. 	Define Entropy.

8. 	What are cyclic codes?

9. 	What is the need for ‘Pseudo random sequence’?

10. 	Define ‘processing gain’ of spread spectrum.

PART – B  (5 x 3 = 15 MARKS)

11. 	What is aliasing? How it is eliminated?

12. 	Sketch the binary output waveform for BPSK and FSK for the input data ‘1110101’.

13. 	Write a short note on Differential PSK with example.

14.	How convolution codes are different from other codes?

15. 	Compare slow and fast frequency hop spread spectrum system.

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain the fundamental block digram of Pulse Code modulation System.(PCM).

(OR)

17.	With neat diagram explain the functional principle of  time division multiplexing.

18. 	With neat functional block diagram explain the operation of BPSK transmitter and receiver also deduce an expression for the probability of symbol error.

(OR)

19. 	With neat functional block diagram explain the data transmission and detection in QPSK scheme.

20.	a.	Derive the expression for the output of the match filter.						  	  (10)

b.	List down the properties of Match Filter											   (5)

(OR)

21. 	Explain the operation of FSK transmitter and receiver. Also deduce an expression for the  Probability of symbol error.

22.	With suitable example discuss Viterbi Algorithm in detail.

(OR)

23.	Explain any four important classes of cyclic code in detail.

24. 	With neat functional block diagram explain the operation of Direct Sequence Spread (DSSS) System. Also sketch relevant waveforms and spectrums.

(OR)

25.	With neat diagram explain what is Antijam and Multipath performance?

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MICROPROCESSORS AND MICROCONTROLLERS

Time : 3 hours

Subject Code:	09EC233										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write down the control and status signals of 8085.

2.	Which interrupt has highest priority in 8085?

3.	Write down the additional flags included in the 8086.

4.	Mention the size of data bus of 8086.

5.	Which mode generates square wave in 8253?

6.	Specify the bit of a control word for the 8255, which differentiates between the I/O mode and the BSR mode.

7.	An alternate function of port pin P3.4 in the 8051 is  ____________.

8.	How many bytes of internal RAM and ROM supported by 8051 micro controller?

9.	What is the basic difference between counter and timer?

10.	The rate at which serial data is transmitted is called  ____________.

PART – B  (5 x 3 = 15 MARKS)

11.	Describe the function of the following pins in 8085.

a) ALE    		b) IO/M   		c) SID

12.	Why memory segmentation is done for 8086 Microprocessor?

13.	List the major components of 8251A programmable communication interface.

14.	Show the status of the CY, AC and P flag after the addition of 38H and 2FH in the following  instructions.

MOV A, #38H

ADD A, #2FH

15.	Draw the format of SCON register. Explain different bits in it.

PART – C  (5 x 15 = 75 MARKS)

16.	With a neat functional block diagram, explain the architecture of 8085 Microprocessor.

(OR)

17.	a.	Draw the timing diagram for the instruction MVI B, Data. Assume all the relevant details.

(8)

b.	Write an Assembly Language Program to find the value of :  X2   			      		    (7)

18.	a.	Discuss the different addressing modes that are supported by 8086 processor with  	example.																	  (10)

b.	Distinguish   Minimum and Maximum mode operation of 8086 processor.	                  (5)

(OR)

19.	With a neat diagram, explain in detail about the architecture of 8086.

20.	What is the function of 8253 Programmable Interval Timer? Discuss any one of its applications

in detail.

(OR)

[P.T.O.]

21.	With a neat block diagram, explain 8259A Programmable interrupt controller.

22.	With a neat diagram, explain in detail about the architecture of 8051.

(OR)

23.	a.	How many addressing modes are available in 8051? Explain them with an example.	  (10)

b.	List the interrupts of Microcontroller according to its priority.						    (5)

24.	Draw Format of timer mode control register TMOD and describe four modes of operations of Timer of 8051 microcontroller.

(OR)

25. 	Write short notes on:

a. 	Interfacing ADC with 8051.													    (8)

b.	Interfacing Keyboard with 8051.												    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	COMPUTER COMMUNICATION					      	     Time : 3 hours

Subject Code:	09EC234										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	A computer communication technology that provides a way to interconnect multiple computers across short distance is ____________.

2. 	What is point-point link?

3. 	Define terms single bit error.

4. 	What is redundancy?

5. 	Tool that is used to transfer data/files among computers on the Internet is ____________.

6. 	A device that forwards data packet from one network to another is called a ____________.

7. 	What is meant by brouter?

8. 	In ATM, the ____________ class is designed for customers who need real-time audio or video services. The service is similar to that provided by a dedicated line such as a T line.

9. 	What is encoding and decoding?

10. 	Secret-key encryption is also known as ____________.

PART – B  (5 x 3 = 15 MARKS)

11. 	What are the three criteria necessary for an effective and efficient network?

12. 	List out the available detection methods.

13. 	Give classification of Packet switching in detail.

14. 	State duties of Transport layer.

15. 	What is a digital signature?

PART – C  (5 x 15 = 75 MARKS)

16. 	Describe the ISO – OSI layer model of data communications system in detail with neat sketch.

(OR)

17. 	What is meant by Guided medium? Explain in detail based upon its classification.

18. 	Discuss the various techniques involves in flow control and error control in computer networks.

(OR)

19.	Write a detailed note on IEEE standards related to Ethernet and wireless LAN.

20. 	With relevant diagram in detail about IPv4 and IPv6 along with addressing mode.

(OR)

21. 	Explain in detail about routing Distance vector routing and Link state vector routing.

22. 	Why UDP is said to be a Connectionless protocol and explain the operation of TCP.

(OR)

23. 	Discuss the process involved in congestion control with various algorithms. And state the factors determine the QoS.

24. 	Explain the Concept behind Domain Name System with necessary diagram.

(OR)

25.	Write a detailed note on SMTP along with its applications.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ELECTRONICS AND MICROPROCESSORS		      	     Time : 3 hours

Subject Code:	09EC235								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the ripple factor value for Half wave and full wave rectifiers?

2.	Give the relations among α, β and γ.

3.	What is thermistor?

4.	What are Universal gates? Why they are called so?

5.	Draw the characteristic table for JK FF.

6.	Define Combinational logic circuits. Give Example.

7.	Mention the flags of 8085.

8.	‘SUI 8-bit’, Give the operation for this mnemonics when it is executed.

9.	Expand PPI.

10.	Classify data transferring schemes in 8085.

PART – B  (5 x 3 = 15 MARKS)

11.	State Barkhausen criterion.

12.	Write a short note on transducers.

13.	Convert (72.625)10 into Binary Number.

14.	What are the registers available in 8085?

15.	What are the operating modes in PPI?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Draw and explain the CE configuration of the transistor with its input / output 	characteristics curve.															  (10)

b.	With necessary circuit explain the RC phase shift oscillator.						    (5)

(OR)

17.	a.	Explain the circuit which regulates voltage.										    (6)

b.	Draw the circuit diagram for single stage class A power amplifier.					    (4)

c.	List out ideal characteristics of an Op amp.									    (5)

18.	a.	How DVMs are classified?													    (3)

b.	Comprehend on various types of potentiometer with necessary diagram.				    (6)

c.	Briefly explain bonded resistance strain gauges.									    (6)

(OR)

19.	a.	Explain about Dual slope DVM with neat block diagram.							    (7)

b.	Draw the block diagram of simple multimeter and explain its operation.				    (8)

20.	a.	Draw the AND,OR,NOT using NAND gate with truth table.						    (6)

b.	Explain about decoder with necessary diagram.									    (4)

c.	Design full adder with necessary diagram.										    (5)

(OR)

21.	a.	Explain the circuit of RS flip flop. How the problem of FF is avoided?		                  (8)

b.	Design 4:1 Multiplexer with its truth table.										    (7)

[P.T.O.]

22.	a.	Draw and explain the architecture of 8085.										  (10)

b.	Write the various types of addressing modes in 8085. Explain with suitable example.	   (5)

(OR)

23.	a.	Classify and explain the Instruction set in 8085.									  (10)

b.	Write an assembly language program to add two 8 bit numbers and store the result in 	memory address.																    (5)

24.	a.	Write an algorithm and flowchart for asynchronous data transfer with handshaking.	    (8)

b.	Draw the operator panel for temperature monitoring system and explain its input and 	output subsystem.															    (7)

(OR)

25.	a.	Classify and explain in detail about the memory unit.								  (10)

b.	Paraphrase on Interrupts in 8085.												   (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ELECTRON DEVICES								      	     Time : 3 hours

Subject Code:	10EC201						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The unit of electric field intensity is ________.

2. 	The ________ is low enough so that the mutual repulsive force may be neglected.

3. 	The equation for the drift current density, Jp due to holes is given by ________.

4. 	Which capacitance dominates in the reverse bias region?

(a) Depletion (b) Conversion (c) 40Ω diffusion (d) None of the above

5. 	How much is the base-to-emitter voltage of a transistor in the “ON” state?

(a) 0V (b) 0.7V (c) 0.7mV (d) undefined

6. 	A diode conducts current when reverse-biased and blocks current when forward-biased.

(a) True (b) False

7. 	Schotky diodes are known as ________.

8. 	The normal operating region of a Zener diode is

(a) forward bias region (b) reverse bias region

(c) zero crossing region (d) reverse breakdown region.

9. 	The forward voltage across an LED is typically 0.7V.State True or False.

10. 	As the optocoupler provides electrical isolation between circuits, it is also called ________.

PART – B  (5 x 3 = 15 MARKS)

11. 	An electron is accelerated through a potential of 40V before it enters a magnetic field density       of  0.91 Wb/m2 at an angle of 30 degrees with the field. Find the position of the electron after which it has completed one revolution in the field.

12. 	What are the limiting values of PN-junction diode?

13. 	State Miller’s theorem.

14.	Draw the structure of Gunn diode.

15. 	List out the merits and demerits of LCD.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Discuss the motion of an electron between two parallel plates under the influence of   	applied potential.                                                                                                                 (7)

b.	An infinitely large parallel plane plates are spaced 0.8cm apart. The voltage at one of the plates is raised from 0 to 5 V in 1 ns at a uniform rate, with respect to the other. After duration, the potential difference between the plates is suddenly dropped to 0 V and remains the same thereafter. Find (i) the position of the electron, which started with Zero initial velocity from the negative plate, when the potential difference drops to zero volt,   (ii) the total time of transit of the electron from the cathode to the anode. 			    (8)

(OR)

17. 	a.	Describe in detail about Electrostatic Deflection in Cathode Ray Tube.                          (10)

b.	Calculate the force acting on a conductor kept in a magnetic field.                                   (5)

18.  	Define Hall effect. Derive the Continuity equation of PN junction diode.

[P.T.O.]

(OR)

19. 	a.	With neat sketch explain the energy band structure of Open Circuited PN junction.         (8)

b.	Explain about Space Charge region Capacitance (CT) of PN diode.                                   (7)

20. 	a.	Explain in detail about Forward and Reverse Characteristics of PN diode.                      (10)

b.	Draw the Ebers-Moll model for a PNP transistor and give the equations for emitter   	current and collector current.                                                                                         	    (5)

(OR)

21. 	Determine the h-parameters from the characteristics of CB configuration.

22. 	a.	Describe the working principle of an SCR with V-I characteristics.                                  (8)

b.	What is TRIAC? Sketch its characteristics and describe its operation.                               (7)

(OR)

23. 	a.	What is “tunneling”? From the energy band diagram explain the V-I characteristic of                                     	a tunnel diode.                                                                                                                   (10)

b.	Show that a reverse biased PN junction can be used as a Variable Capacitor.                   (5)

24. 	a.	Draw and explain a seven-segment LED display.                                                               (8)

b.	Describe the principle of operation of an LCD.                                                                  (7)

(OR)

25. 	a.	Describe the operation of optocoupler.                                                                                (7)

b.	Explain the structure of a colour plasma display panel and draw the V-I characteristics of 	the PDP cell and identify the operating region.                                                                   (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ELECTRIC CIRCUIT ANALYSIS					      	     Time : 3 hours

Subject Code:	10EC202										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Find the equivalent current source for a voltage source of 100V with series resistance of 2Ω.

2.	A series circuit consists of a 4.7k, 5.6 k, 9k and 10k resistor. Which resistor has the most voltage across it?

3.	A 1 kHz sinusoical voltage is applied to an RL circuit, what is the frequency of the resulting current?

a.	1 kHz 		b.	0.1 kHz		c.	100 kHz			d.	2 kHz

4.	A power factor of ‘1’ indicates

(a)purely resistive circuit (b) purely reactive circuit (c) combination of (a) and (b)

(d) none of these

5.	Coefficient of coupling of an ideal transformer is ________.

6.	Dot convention in coupled circuits is used to determine ________.

7.	Superposition theorem is valid only for ______________ systems.

8.	Mention the condition for maximum power to be  transferred to the load according to maximum power transfer theorem

9.	Time constant of a series RC circuit is ________.

10.	What is meant by transient time?

PART – B  (5 x 3 = 15 MARKS)

11.	Three resistors 13 Ω, 12 Ω and 14 Ω are connected in delta network. What are the equivalent star resistors?

12.	Determine the quality factor of a coil for the series circuit consist of R=10 Ω , L=0.1 H, C=10μF

13.	Two inductively coupled coils have self inductances L1=50 mH, L2=200 mH. If the coefficient of coupling is 0.5 Find the value of mutual inductance between coils.

14.	State Compensation theorem.

15.	Derive the DC response of series RC circuit.

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the voltage drop across the 10Ω resistance in the circuit as shown in the figure. The units of all the resistors are in ohm

[P.T.O.]

(OR)

17.	a.	State Kirchoff’s laws.

b.	Derive the equivalent resistances Ra,Rb&Rc of  Delta-Star Electric Network.

18.	Derive and prove that product of half power frequencies is equal to square of resonant frequency.

(OR)

19.	A three phase balanced delta connected load of (4+j8) Ω is connected across a 400V, 3Ø balanced supply. Determine the phase currents and line currents. Assume the phase sequence to be RYB. Also calculate the power drawn by the load.

20.	Determine the value of V2 such that current through (3+j4) Ω impedance is zero.

(OR)

21.	Derive the input impedance of the ideal transformer using turns ratio.

22.	Verify Thevenin’s theorem for the circuit shown in the figure. The units of all resistors are in ohm.

(OR)

23.	Verify superposition theorem for the given circuit. The units of all resistors are in ohm.

24.	A series RL circuit with R = 30  and L =15 H has a DC voltage of 60 V applied through a switch at t = 0. Determine the current and voltages across the different elements.

(OR)

25.	Derive the current expression for RLC series circuit when sinusoidal excitations are applied.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DIGITAL COMMUNICATION						      	     Time : 3 hours

Subject Code:	10EC204						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define companding.

2.	Give the other name of delta modulation.

3.	What are antipodal signals?

4.	What is Gaussian MSK?

5.	Differentiate coherent and non coherent receivers.

6.	Why Gaussian noise is termed as white noise?

7.	Define code rate.

8.	Give the dimension of a code.

9.	Enlist the advantages of spread spectrum.

10.	The PN sequence length for shift register length m = 4 is _______.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the power spectra of PAM signals.

12.	In a DEPSK system the error always occurs in pairs. Why?

13.	What is non coherent detection?

14.	State Shannon’s Capacity theorem.

15.	Define jamming margin and processing gain.

PART – C  (5 x 15 = 75 MARKS)

16.	With suitable block diagram and wave forms explain Delta modulation and Pulse Code Modulation.

(OR)

17.	What is ISI? Obtain the Nyquist criterion for distortionless baseband binary transmission. Explain ideal Nyquist channel with the help of ideal and practical solution.

18.	With a neat sketch of a transmitter, receiver, signal constellation and wave forms demonstrate the BPSK system?

(OR)

19.	Why is MSK termed as shaped QPSK? With a neat block diagram and suitable wave forms explain MSK.

20.	Calculate the probability of error and the transfer function in an Optimal filter?

(OR)

21.	a. 	Explain the detection of non-coherent FSK.

b. 	Give the probability of error of matched filter.

[P.T.O.]

22.	A convolution encoder has a single shift register with two stages (i.e constraint length k=3),three modulo 2 adders and an output multiplexer. The generator sequences are as follows:

g(1) = (1,0,1), g(2) =(1,1,0), g(3) =(1,1,1).

Draw the block diagram of the encoder.

Determine the output digit sequence for the data digits 11010100.

(OR)

23.	Consider the generator of a (7,4) cyclic code by generator polynomial g(D) =1+D+D3.

a.	Calculate the systematic code word for a message sequence 1010

b.	Draw the diagram of encoder .

24.	Describe direct sequence spread spectrum technique with suitable diagram.

(OR)

25.	Describe frequency hopping spread spectrum technique with suitable diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	VLSI DESIGN										      	     Time : 3 hours

Subject Code:	10EC205						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the transistors CMOS technology provides?

2.	Design costs for standard cell ASICs are ________ those for MPGAs.

a. lower than  b. about the same as     c. higher than       d. none of the above

3.	_________ is about twice as fast as P-MOS.

a.  CMOS  	b.  DMOS    c.  MOD  	d.  N-MOS

4.	Give the different types of CMOS process.

5.	In HDL, LITERALS is/are:

a.	digital systems.      b. scalars.      c. numbering system

6.	The carry propagation can be expressed as ________.

a.	Cp = AB    	b.	Cp = A + B    	c. 	cp=A~b

7.	Give two examples for variable declaration.

8.	Give the different types of oxidation.

9.	Conditional statements like if else  can be used only in _________.

a.	Data Flow 	b. 	Behavorial 	c. 	Structural 	d.	none

10.	IF multiplexer is implemented in behavorial modeling the following is used

a.	Expression    b. 	Function 	c. 	if else 	d.	for

PART – B  (5 x 3 = 15 MARKS)

11.	Discuss about Circuit Design in VLSI design Process.

12.	Define Threshold voltage.

13.	What are the three regions of operation of a MOS transistor?

14.	What are the different arithmetic operators?

15.	Compare sequential and concurrent statements with example.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the Silicon Gate NMOS fabrication process.

(OR)

17.	Explain the different fabrication steps of SOI process with neat diagram.

18.	How N well CMOS transistor in designed using Mead   and Conway design rules?

(OR)

19.	With neat characteristics curve derive the expression for complementary MOS inverter circuit.

20.	Explain in detail about sheet resistance and how it is applied to MOS transistors and inverter.

(OR)

21.	Explain in detail about capacitance MOS Logic with necessary diagrams.

22.	Write a VHDL Program for four bit full adder, using the structural description. Draw its basic structure showing the inputs and outputs signals.

(OR)

[P.T.O.]

23.	a.	Design a 3 : 8 Decoder in Behavioral modeling using VHDL.

b.	Explain in detail about different types of scalar   types.

24.	Explain in detail about concurrent signal assignment statement and also compare between concurrent and sequential signal assignment.

(OR)

25.	Write a VHDL code for 2-bit Comparator using any one modeling.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	SOLID STATE CIRCUITS							      	     Time : 3 hours

Subject Code:	11EC201						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Mention the merits of Full wave rectifier.

2.	The voltage regulation for an ideal power supply is infinity. State True/False.

3.	Write the saturation collector current equation for fixed bias BJT network with emitter resistor.

4.	What is the disadvantage of fixed bias configuration over self bias configuration in JFET?

5.	What are the characteristic requirements of an ideal wide band amplifier?

6.	What is the efficiency of series fed class A power amplifier?

7.	Negative feedback circuit reduces the effect of noise and non-linear distortion.State True/False.

8.	Define Common Mode Rejection Ratio in a differential Amplifier.

9.	Name any two high frequency oscillator.

10.	Give the mathematical formula for the resonant frequency of a parallel tuned circuit.

PART – B  (5 x 3 = 15 MARKS)

11.	Suggest some method to protect a regulated power supply from overload condition.

12.	Draw the self bias configuration of JFET and derive VGS for the same.

13.	Define collector efficiency, distortion and power dissipation capability of a power transistor.

14.	The voltage gain of negative voltage shunt feedback circuit is reduced by a factor of 1+Aβ. Justify.

15.	The Q of a tuned amplifier is 60. If the resonant frequency for the amplifier is 1200 KHz, find the bandwidth of the amplifier.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the working principle of Half wave rectifier and derive its rectifier efficiency and transformer utilization factor .Also discuss the merits and demerits of the circuit.

(OR)

17.	a.	A full wave rectifier with a load resistance of 15 kohm uses an inductor filter of 15 H. The 	peak value of the applied voltage is 250 V and the frequency is 50 cycles/second. Calculate 	the dc load and ripple factor.													    (5)

b.	With neat diagram explain the operation of controlled transistor series regulator and derive the output voltage expression.													  (10)

18.	a.	Explain the fixed bias configuration of common emitter Bipolar Junction Transistor and 	derive the parameters related to it.												  (10)

b.	Discuss the variation of Q point with respect to Vcc , IB and Rc of the circuit.			  (5)

(OR)

19.	a.	Calculate IB for voltage divider biasing of BJT with the following specifications.

Vcc = 22 V, R1 = 39 kΩ, R2 = 3.9 kΩ, Rc = 10 kΩ, RE = 1.5 kΩ, β = 140			           (7)

b.	Discuss the self bias configuration of JFET and also the procedure to find the operating point.																		    (8)

[P.T.O.]

20.	a.	Calculate the effective resistance seen looking into the primary of a 15:1 transformer 	connected to an 8 Ωload.														    (4)

b.	With neat circuit diagram explain the operation of RC coupled transistor amplifier.	    (7)

c.	Discuss the advantages and disadvantages of transformer coupled transistor amplifier. (4)

(OR)

21.	Mention the special features of a power transistor and derive the collector efficiency of series fed and transformer coupled class-A power amplifier.

22.	What are the advantages of negative feedback in a system? Derive the gain, input impedance and output impedance for voltage shunt feedback connection.

(OR)

23.	Draw the ac equivalent circuit of differential amplifier and derive the output voltage for all the three modes of operation.

24.	State Barkhausen criteria. With neat sketch explain the operation of RC phase shift oscillator. Also derive the frequency of oscillation.

(OR)

25.	a.	Explain the single tuned transistor amplifier with its frequency response.			  (10)

b.	Discuss the advantages of tuned amplifier.										    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	NETWORK ANALYSIS AND SYNTHESIS			      	     Time : 3 hours

Subject Code:	11EC202								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	For a one port network, the driving point impedance is given by Z(s) = _________.

2.  	A network has transfer function Z(s) = . The pole is at _________ and zero is at _________.

3. 	A capacitor with an initial charge acts as a _________.

4. 	In Laplace  transform, s is equal to _________.

5. 	If there exists at least one path between any two nodes, that path is known as _________.

6. 	The basic principle of dual networks is the exchange of _________ and _________.

7.  	In constant k filters, the series and shunt impedances (Z1 and Z2) satisfy the relation _________.

8. 	The y parameters are also called as _________.

9. 	A LC network can be synthesized using _________.

10. 	If H(s) =P(s)/Q(s) is a positive real function, then Q(s) is a _________.

PART – B  (5 x 3 = 15 MARKS)

11. 	Mention the necessary conditions for driving point functions.

12. 	With reference to pole - zero plot, how is stability of a network defined?

13. 	What is incidence matrix? List  its properties.

14. 	Define the h parameters for a two port network.

15. 	Check whether the polynomial  is Hurwitz polynomial or not.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Determine the current transfer ratio of the network shown with R = 2 ohm and C = ½ F.

(8)

b. 	Explain how time domain behavior of a network can be studied from its pole zero plot. (7)

(OR)

[P.T.O.]

17.	a. 	Determine the condition under which the input impedance of the network shown in fig. 		below will be equal to R.

(8)

Zin

b. 	A transfer function is given by Z(s) = . Find the pole zero plot.      	    (7)

18. 	Explain the Routh Hurwitz criterion for stability of a network function. Verify the stability of the given denominator polynomial Q(s) =  using Routh criteria.

(OR)

19. 	Consider a RLC network. Obtain its magnitude and phase response for a step input and explain its complex loci plot.

20. 	In graph theory, define tree, branch, twigs and links. For the incidence matrix, draw the oriented graph of the network.

Nodes				Branches

1	 2	 3	 4	 5	 6	 7	 8

1 	 0	 0	 1	 0	 0	 0	 1

-1	 1	 0	 0	 1	 0	 0	 0

[A] =	  0	 -1	 -1	 0	 0	 -1	 0	 -1

0	 0	 1	 0	 0	 0	 -1	 0

0 	 0	 0	 -1	 -1	 1	 1	 0

(OR)

21. 	Write the basic elements and their duals. Prove that the two networks are duals of each other.

22.	a.	Explain the z parameters and ABCD parameters of a given two port network. Derive the 			relation between z parameter in terms of ABCD parameters. 						    (8)

b. 	Find the Input impedance of two port network in terms of ABCD parameters.		    (7)	(OR)

23. 	Discuss the configuration of m-derived T section prototype filter. Design an m-derived low    pass T section with resistance of 600 ohms, cut off frequency 1600Hz and infinite attenuation frequency of 2200Hz.

24. 	Synthesize the impedance function Z(s) =   using Foster Form I and             	Foster 	Form II .

(OR)

25. 	Explain the properties of positive real functions. Check whether the functions

Z(s) =     and      Z(s) =         are  positive real functions.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	SIGNALS AND SYSTEMS							      	     Time : 3 hours

Subject Code:	11EC203						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define a discrete time system.

2.	What is an energy signal?

3.	Differentiate Natural response and Forced response.

4.	Write any two properties of ROC of Z-transform.

5.	State Dirichlet condition for Fourier series.

6.	Define frequency convolution property of Fourier transform.

7.	Under-sampling is due to ___________.

8.	State initial value theorem.

9.	Define Discrete Time Fourier Transform (DTFT) of a function.

10.	What is the relation between Z-transform and DTFT?

PART – B  (5 x 3 = 15 MARKS)

11.	State the linearity property of a system with one example.

12.	Find the Z-transform of x(n) = (1/2)n u(–n).

13.	Prove the Differentiation in frequency property of CTFT.

14.	For the given signal x(t) = sinc2(200πt) determine the Nyquist rate and the sampling interval.

15.	Determine the DTFT of x(n)=anu(n).

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Find the even and odd components of the following signals.

i.	x(t) = cos t + sin t + cos t sin t

ii.	x(n) = {1, 0, –1, 2, 3}														    (8)

b.	Check whether the following signals are energy signal or not.

i.	xt=e−3tu(t)

ii.	x(n) = 13n u(n)															    (7)

(OR)

17.	Determine whether the following continuous time systems are

a.	Static or Dynamic

b.	Linear or Non-linear

c.	Shift invariant or variant

d.	Causal or Non- causal

i.	y(t) d2y(t)dt2+3t dy(t)dt+y(t)=x(t)										    (8)

ii.	y(n)=x(n)×x(n−1)                   											    (7)

[P.T.O.]

18.	a.	Compute the convolution of the signals

x (n)	=	u(n) – u(n–7).

h (n)	=	u(n–1)–u(n–4).                                                                             	    (7)

b.	Determine the impulse response for the systems given by the difference equation of y(n)=0.6y(n−1)−0.08y(n−2)+x(n)	.				           					    (8)

(OR)

19.	a.	Find the z-transform and ROC of  x(n)=an ; a<1 .							    (5)

b.	Obtain the cascade form realization of the system described by the difference equation:

y(n)−14y(n−1)−18y(n−2)=x(n)+3x(n−1)+2x(n−2).						  (10)

20.	Determine the Fourier series representation of periodic square wave defined over one period

x(t) 	  = { 1 for 1 ≤  t  ≤ 1

–1 for  1 ≤  t  ≤  3 }

(OR)

21.	a.	Find the continuous time Fourier transform for the signal xt=e−t.				    (5)

b.	The input and Output of a causal LTI system are related by the differential equation d2y(t)dt2+6dy(t)dt+8y(t)=2x(t)

i. 	Find the impulse response of the system.

ii. 	What is the Response of this system if xt=te−2tu(t). 						  (10)

22.	Discuss the discrete time processing of continuous time signals with necessary block diagrams and equations.

(OR)

23.	a.	Determine the z transform of x(n) =an u(n)–bn u(–n–1) and find the region of convergence.

(5)

b.	Find the inverse Z transform of

14Z−11−12 Z−11−14Z−1 , |Z| > 12  by using partial fraction expansion technique.		  (10)

24.	State and prove any five important properties of discrete time Fourier series.

(OR)

25.	Determine and sketch the magnitude and phase response of the system

y(n) = 1/3[x(n) + x(n–1) + x(n–2)]

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DIGITAL ELECTRONICS							      	     Time : 3 hours

Subject Code:	11EC204						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Convert the following decimal number (133)10  to binary.

2. 	State De-morgan’s theorem.

3. 	Write the equivalent gray code for the given binary number (1000)2

4. 	Draw the truthtable for Ex-NOR gate.

5. 	Define Characteristics equation.

6. 	What do you mean by sequence generator?

7. 	List the application of shift register.

8. 	The bit sequence 0010 is serially entered (right-most bit first) into a 4-bit parallel out shift

register that is initially clear. What are the Q outputs after two clock pulses?

9. 	The size of the PLA specified by __________.

10. 	List the 3 types of output configuration of TTL gates.

PART – B  (5 x 3 = 15 MARKS)

11. 	Obtain the canonical SOP form of the function Y = AB + ACD.

12. 	Design the logic circuit of a Half adder and Half Subtractor.

13. 	Differentiate synchronous counter and asynchronous counter.

14. 	Explain the operation of ring counter.

15. 	Compare the TTL, ECL and CMOS digital logic families.

PART – C  (5 x 15 = 75 MARKS)

16. 	Simplify the Boolean function using Quine Mcclusky method.

F(a,b,c,d)= Σm(0,2,4,5,6,7,8,10,14,17,18,21,29,31) + Σd(11,20,22)

(OR)

17. 	a. 	Using the K-map method, simplify the Boolean function                                   		    (8)

Y = Σm(0,2,3,6,7) + Σd (8,10,11,15)   and obtain

(i) Minimal SOP expression and

(ii) Minimal POS expression

b.	Simplify the Boolean function Y = Σm(0,4,8,12,16,18,20,22) + Σd (24,26,28,30,31) using

K-map and implement using NOR gate.                                                            		    (7)

18. 	a.	Implement the following Boolean function using 8:1 multiplexer.

f(A,B,C,D) = Σ m(2,4,5,7,10,14)                                                                         		  (10)

b.	Design a 2-bit Magnitude comparator.                                                                   		    (5)

(OR)

19. 	Design a Circuit to convert 8421 BCD code to EXCESS-3 code.

20.	a.	Explain with neat logic circuit and working of SR flipflop and JK master slave flipflop.

(10)

b.	Write short notes on triggering of flipflops.                                                         		    (5)

(OR)

[P.T.O.]

21. 	Design a sequential circuit using D-flipflop for the given state diagram.

22. 	Design a synchronous MOD – 6 counter using JK flip flop.

(OR)

23. 	a.	What is a shift register? Explain Serial in Serial out and Serial in parallel out registers with 	suitable diagram.                                                                                                		    (8)

b. 	Draw a 4-bit Johnson counter and explain with truth table and waveform.             	    (7)

24. 	a.	Implement the following Boolean functions using PLA.                                       		    (7)

F1(A,B,C) =∑(0,1,2,4) ; F2(A,B,C)=∑(0,5,6,7)

b.	Implement the following Boolean functions using PAL.                                      		    (8)

F1(A,B,C) =∑(0,1,4,7) ; F2(A,B,C)=∑(0,2,4,5); F3(A,B,C)=∑(0,1,3,6,7)

(OR)

25.	a.	Sketch open collector and totem pole TTL logic with a neat diagram and explain briefly.                                                                                                             																				  (10)

b.	Describe about the ECL with neat diagrams.                                                              	    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DIGITAL COMMUNICATION						      	     Time : 3 hours

Subject Code:	11EC205						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Entropy.

2.	What are the line coding techniques?

3.	What is the function of synchronizer?

4.	What do you mean by coherent reception?

5.	When aliasing occurs?

6.	Mention the disadvantage of PAM signal?

7.	How will you calculate Hamming Distance?

8.	Define code efficiency.

9.	What is the application of frequency hopping technique?

10.	State any one property of PN sequence.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the advantages and disadvantages of digital communication?

12.	Draw a correlation receiver and explain.

13.	What is sampling? Mention its types.

14.	Differentiate systematic and non systematic code word.

15.	What is spread spectrum? What are its applications?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the functional description of digital communication system in detail.

(OR)

17.	a.	State and explain Shannon’s channel capacity theorem.							    (8)

b.	Explain Huffman coding with an example.									          (7)

18.	Explain in detail about Gram- Schmidt  orthogonalization procedure.

(OR)

19.	a.	What is Matched Filter? Mention the properties of Matched Filter. 					    (6)

b.	Derive an expression for probability of error of Matched filter.					         (9)

20.	Explain in detail about M- ary PSK system?

(OR)

21.	a.	Explain  in detail about Delta Modulation and also state its demerits.				  (10)

b.	What are the types of quantization?												    (5)

22.	a.	Decode the input [11 10 11 11 01 01 01 11] using code tree and code trellis.			    (7)

b.	Discuss about convolutional coding.											    (8)

(OR)

23.	Obtain the generator matrix corresponding to G (p) =p3+p+1 for a systematic (7, 4) cyclic code. Also find parity check matrix

24.	With neat block diagram explain frequency hopping technique.

(OR)

25.	Explain Direct sequence spread spectrum with an example.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	COMMUNICATION THEORY AND SYSTEMS		     	     Time : 3 hours

Subject Code:	11EC206								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define depth of modulation.

2.	A transmitter supplies 8 kW to the antenna when modulated. Determine the total power radiated when modulated to 30%.

3.	Define noise figure.

4.	What is called image frequency?

5.	What is the figure of merit of an AM system with 100 percent modulation?

6.	Define Tracking.

7.	What is Capture effect in FM system?

8.	What is Pre-emphasis?

9.	What is FSK?

10.	List the application of CDMA.

PART – B  (5 x 3 = 15 MARKS)

11.	Compare AM with DSB-SC and SSB-SC system.

12.	What are the applications of PLL?

13.	What is super heterodyne principle?

14.	What are the types of External noise and internal noise?

15.	Compare the FDMA and TDMA techniques.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Derive the expression for AM & its Power and Efficiency calculation.     			    (8)

b.	Describe the generation of AM wave by Non-Linear modulators.	       				    (7)

(OR)

17.	a.	Write short notes on De Modulation of AM wave.		                    				    (8)

b.	Discuss the generation of DSB-SC-AM system.			        					    (7)

18.	a.	Explain the Narrow Band Frequency Modulation.		       	        				    (8)

b.	Explain the method of Generation of FM signal. 			        					    (7)

(OR)

19.	a.	Write short notes on balanced slope detector.			                    				    (8)

b.	Explain about the Angle Modulation.				        						    (7)

20.	Explain the Super heterodyne receiver with block diagram.

(OR)

21.	Discuss about the SSB transmitter with eat diagram.

22.	a.	Explain in detail about Noise temperature.                                                  			   (8)

b.	Describe the effect of noise in reactive circuits. 	                               				   (7)

(OR)

23. 	Explain the FM transmitter with block diagram.

24.	Explain about the PDM technique and PAM technique with diagram.

(OR)

25.	Discuss the SDMA and CDMA technique with diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December-2015

Subject Title :	C++ AND DATA STRUCTURES						      	     Time : 3 hours

Subject Code:	11EC207						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is class and object in C++?

2. 	List out an advantages of C++.

3. 	What is the difference between base class and derived class in c ++?

4. 	What is the operator overloading in C++?

5. 	What is friend function in C+?

6.  	List out the get file pointer in C++.

7.  	State about linked list types.

8. 	What are the basic operations on circular linked list?

9.  	What is sorting algorithm?

10. 	What is merge sort?

PART – B  (5 x 3 = 15 MARKS)

11. 	Give short notes on applications of OOPs.

12. 	Define pointer and array.

13. 	Explain the class templates with multiple parameters using the programs.

14. 	Give short notes on operations on linked list.

15. 	Explain the operations of binary search trees.

PART – C  (5 x 15 = 75 MARKS)

16.  Write out C++ Program for display the Name, Age, employee salary using Structures.

(OR)

17. 	Write out C++ Program for display the names such as “Hello”, “world” and count the string length using C++ strings.

18. 	Explain about the UML class using levels of inheritance and multiple inheritances with example.

(OR)

19. 	Explain about the array using pointer and pointer to function.

20. 	Describe the type of exception with example.

(OR)

21.	Explain about the function templates with example.

22.	Explain about the implementation of doubly linked list.

(OR)

23.	Detail about the queue with suitable example.

24.	Write about the selection sort with example.

(OR)

25.	Explain brief about the performances of binary search with example.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	LINEAR INTEGRATED CIRCUITS AND APPLICATIONS													      	     								    Time : 3 hours

Subject Code:	11EC208								 	  	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Gate of a MOSFET is formed using ____________ material.

2.	____________ technology is used to make passive components like resistors and capacitors.

3.	Clamper is also known as ____________.

4.	____________ is a circuit which compares a signal voltage applied at one input and the known reference voltage at the other input.

5.	____________ wave can be simply obtained by integrating a square wave.

6.	Frequency of oscillation of phase shift oscillator is given by ____________.

7.	A frequency selective electric circuit that passes electric signals of specified band of frequencies and attenuates the signals of frequencies outside the band is called an ____________.

8.	____________ is a highly stable device for generating accurate time delay or oscillations.

9.	A common type of VCO available in IC form is ____________.

10.	Which is the fastest ADC?

PART – B  (5 x 3 = 15 MARKS)

11.	List the advantages of using CMOS technology.

12.	Draw the circuit diagram of a sample and hold circuit.

13.	Design an RC phase shift oscillator to oscillate at 100Hz, Let C=0.1µf and R1=10kΩ.

14.	Sketch the frequency response of first order low pass filter.

15.	What are the basic blocks of PLL?

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss about the fabrication of monolithic NPN transistor and PNP transistor.

(OR)

17.	a.	Explain any one method of fabrication of resistor.					   			    (8)

b.	Describe the fabrication of JFET.							   					    (7)

18.  Explain the Op-amp differentiator and Integrator circuits.

(OR)

19.  Discuss the operation and characteristics of comparator with neat diagram.

20.  Draw and explain the operation of Wien bridge oscillator and obtain its frequency response.

(OR)

21. 	a.	Explain the function of IC 723 voltage regulator.					  				  (10)

b.	Discuss about current fold back and current limit protection circuits.			  	    (5)

22. 	a.	Draw the frequency response of band pass filter and band reject filter.				   (4)

b.	Derive the expression for the frequency of second order low pass filter.				  (11)

(OR)

23. 	Explain the Mono stable mode of operation of 555 timer.

24. 	a.	Draw and explain the working of Weighted resistor DAC.       	                         		    (8)

b.	Explain the operation of Successive approximation ADC.	  		             			    (7)

(OR)

25.	Describe the working of Voltage Controlled Oscillator with neat diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	PULSE AND WAVE SHAPING CIRCUITS			      	     Time : 3 hours

Subject Code:	11EC210								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The process whereby the form of a non-sinusoidal signal is altered by transmission through a linear network is called as ___________.

2. 	___________ is the time taken by the output to rise from 10% to 90% of its final steady state value for a step input.

3. 	A Schmitt trigger exhibits ___________ when loop gain is greater than one.

4. 	The ___________ capacitors are required to increase the speed of operation.

5. 	The ___________ circuit has two quasi stable states.

6. 	The ___________ triggering technique is adopted for monostable multivibrator.

7. 	The most important application of time base generators is ___________.

8. 	The rate of difference in slope at the beginning and end of sweep to the initial value of slope is called as ___________.

9. 	The ___________ blocking oscillator is used to obtain abrupt pulses from slowly varying input voltages.

10. 	A blocking oscillator uses ___________ feedback.

PART – B  (5 x 3 = 15 MARKS)

11. 	What is meant by biased clamping?

12. 	What is triggering? Mention its types.

13. 	Calculate the frequency of oscillation of the astable multivibrator with R1=R2=25 Kohms and C1=C2 = 0.2 microFarad.

14.	Mention the functioning of  exponential sweep circuit.

15. 	List the applications of blocking oscillators.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Derive the expression of the output of a low pass circuit excited by an exponential Input.																						    (8)

b.	Explain with diagram, the working of a clipper circuit clipping at two independent levels. 																					    (7)

(OR)

17. 	a.	With the circuit diagram and waveforms, explain the working of a positive clamping 	circuit.																		    (8)

b.	Mention the working of a high pass RC circuit as a differentiator.					    (7)

18. 	With circuit diagram and waveforms, explain the working of a Schmitt Trigger circuit. Derive expressions for UTP and LTP of the circuit.

(OR)

19. 	Describe the working of a fixed biased transistor bistable multivibrator. Mention its applications.

[P.T.O.]

20. 	Describe the working of a collector coupled monostable multivibrator. Derive the expression for gate width of the monostable multivibrator circuit.

(OR)

21. 	With circuit diagram and necessary waveforms, describe the working of an astable multivibrator. Explain how it can be used as a voltage controlled oscillator.

22.	Describe with circuit diagram, the working of a transistor Bootstrap time base generator. circuit.

(OR)

23. 	With circuit diagram, explain the principle of operation of a transistor current time base generator.

24. 	Describe with circuit diagram, the working of an astable blocking oscillator circuit.

(OR)

25. 	Explain the basic operating principle of sampling gates. Explain the working of a unidirectional diode gate.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ANTENNAS AND WAVE PROPAGATION			      	     Time : 3 hours

Subject Code:	11EC211								 	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Dipole of length ___________ is chosen as it has maximum radiation in the centre.

2.	Radiation resistance of a simple dipole is ___________ times of a folded dipole.

3. 	The directivity of an antenna is defined as the ratio of ___________.

4. 	In order to obtain a endfire array, the electric field of the linear array of n isotropic point sources should be  ___________.

5. 	The Yagi antenna is an example of ___________ array antenna.

6. 	An example of a non-resonant antenna is ___________.

7.  	The ___________ antenna is used as a feed for a parabolic reflector.

8. 	In a lens antenna, the lens is typically made of ___________.

9. 	The ionosphere is at a height of ___________ from the earth's surface.

10. 	Duct propagation occurs due to ___________.

PART – B  (5 x 3 = 15 MARKS)

11.  What is the use of loading coil for short antennas?

12. 	Define the reciprocity principle with respect to antenna.

13. 	Define the terms self and mutual impedance in coupled antenna.

14. 	Mention some important features of  biconical antennas.

15.	Define skip distance.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the equations for the magnetic and electric field components from a radiating small current element.

(OR)

17.  	Define radiation resistance of an antenna and derive the expression for radiation resistance of half wave dipole with linear current distribution.

18. 	Obtain the expression for the electric field of a linear array of n isotropic point sources with same amplitude and phase. Obtain its radiation pattern and maximum and minimum directions of radiation.

(OR)

19. 	a.	Explain the principle of radiation from a small loop.  								    (7)

b. 	Explain the principle of pattern multiplication.									    (8)

20. 	What are frequency independent antennas? Explain constructional details and applications of a log periodic dipole array.

(OR)

21. 	Describe the principle of radiation and design aspects of a rhombic antenna.

[P.T.O.]

22.	a. 	Describe how a rectangular aperture can be used as antenna.  						    (7)

b. 	Explain the equivalence of fields of Slot and complementary dipole. 				    (8)

(OR)

23.	a.	Explain the principle and practical considerations of lens antenna. 					    (8)

b. 	Explain how the geometry of parabolic reflectors is suitable for radiation.  			    (7)

24. 	Explain how different layers of the ionosphere reflect radio wave. Deduce expression 	for critical frequency of the ionosphere.

(OR)

25. 	Describe the ground wave propagation. Explain how the angle of tilt affects the 	field strength at a distance from the transmitter.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	TRANSMISSION LINES AND WAVE GUIDES		      	     Time : 3 hours

Subject Code:	11EC212								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define wavelength.

2.	Give the value of inductance in an ordinary unloaded telephone cable.

3.	Determine the standing wave ratio if the reflection co-efficient value is 0.5.                                           4.	___________ line is called as one to one transformer.

5.	What are Transverse Electric waves? What is its dominant mode in a parallel plate waveguide?

6.	Define wave impedance.

7.	What are transverse magnetic waves in a rectangular waveguide?

8.	Define Phase velocity and group velocity.

9.	Give the wave impedance in terms of phase constant for TE waves of circular waveguides.

10.	Give the dominant mode in the TE waves of a rectangular cavity resonator.

PART – B  (5 x 3 = 15 MARKS)

11.	A transmission line of length 2 miles has the following constants at 10kHz: R=30Ω/mile, C=80nF/mile, L=2.2mH/mile and G=20ns/mile. Find its Characteristic impedance.

12.	Give the principle of single stub matching in a transmission line.

13.	What are the characteristics of TEM waves?

14.	An air-filled rectangular waveguide with dimensions 6.5 × 3.5 cm operates in the dominant mode at 3GHz. Calculate the value of phase constant.

15.	What are the advantages of circular waveguides over rectangular waveguides?

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the general solution of a transmission line.

(OR)

17.	What is Loading? Explain this concept supported with Campbell’s equation.

18.	Derive the input impedance for the following transmission line sections:

a.	λ/8 line

b.	λ/4 line

c.	λ/2 line

Also give the applications of quarter-wave line.

(OR)

19.	A transmission line with characteristic impedance 50 Ω is connected to a load of (40+j10) Ω. The frequency is 750MHz and the transmission line is air filled. Find the shortest length of a short circuited stub and shortest distance from the load in order to provide a perfect match on the main line to the left of the stub.

20.	Derive the expression for TE waves between parallel planes of perfect conductors.

(OR)

21.	The spacing between plates of an air dielectric parallel plate waveguide is 5cm. The signal frequency is 10,000MHz. Find the propagating TEmo modes. Also calculate the guide wavelength and phase velocity for the modes.

[P.T.O.]

22.	Derive the expression for TM waves in a rectangular waveguides.

(OR)

23.	Derive the expression for the following characteristics of a rectangular waveguide:

a.	Cut-off frequency.

b.	Cut-off wavelength.

c.	Guide wavelength.

d.	Phase velocity.

e.	Group velocity.

24.	Using Bessel function, derive the solution of field equation in cylindrical co-ordinate.

(OR)

25.	Derive the Q-factor for TE101 mode of a rectangular cavity resonator.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	BASIC ELECTRONICS								      	     Time : 3 hours

Subject Code:	12EC101						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give any two examples for Passive components.

2.	Semiconductors exhibit  ___________ bond

3.	Portray the symbol of an Ex-NOR gate.

4.	What are the advantages of FM?

5.	Mention the types of optical fiber.

6.	Give one example for a unipolar device.

7.	FET is a _________controlled device.

8.	State Demorgan’s law.

9.	Define Multiplexer.

10.	Define Frequency Modulation.

PART – B  (5 x 3 = 15 MARKS)

11.	Describe the colour coding system in resistors.

12.	Draw the circuit and explain how zener diode is used as a voltage regulator?

13.	Draw the symbol and truth table of universal gates.

14.	Explain communication system with a block diagram.

15.	Portray about the orbital patterns in satellite communication.

PART – C  (5 x 15 = 75 MARKS)

16.	With neat sketches explain the different types of capacitors and inductors.

(OR)

17.	Give details about the different types of semiconductors with respect to dopping and explain their conduction with possible types of biasing.

18.	Explain the construction of BJT. Discuss the input and output characteristic of an NPN transistor in CE configuration.

(OR)

19.	Discuss in detail the construction of N-channel JFET and explain briefly its drain characteristics.

20.	Elucidate the operation of 4X1 Multiplexer with the help of truth table and circuit diagram.

(OR)

21.	Define De-multiplexer. With the help of truth table explain the principle of operation of 1X4 De-multiplexer.

22.	Portray in detail about Amplitude Modulation. Also derive the expression for total power in the Amplitude Modulated wave.

(OR)

23.	What is the need for modulation? Explain frequency modulation. Derive the equation for frequency modulation.

24.	Give a brief explanation on ISDN and RADAR systems.

(OR)

25.	Explicate in detail about a basic satellite system. Also discuss in brief the basic components of transmitter, transponder and receiver of satellite communication system.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ELECTRON DEVICES								      	     Time : 3 hours

Subject Code:	12EC201						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is energy level?

2.	What does doping mean?

3.	For a transistor, IC = 100mA, IB= 100μA, What is the value for IE.

4.	For a common Emitter configuration transistor, IC = 100mA, IB= 100μA, Compute the vale for β.

5.	Which configuration of BJT is ideal for amplifier application?

6.	State the application of Common collector configuration of a BJT.

7.	Can SCR be used for DC power control application? Write Yes/No.

8.	As the Bipolar Junction Transistors are voltage-controlled devices, for every application the Field Effect Transistors can replace them. State True or False

9.	Write at least two differences between PN Junction Diode and Zener Diode.

10.	State any two applications of Photo diodes.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the major difference between transition capacitance and diffusion capacitance?

12.	What is thermal runaway?

13.	What is the necessity of modeling a transistor?

14.	Draw the transfer Characteristics of a N channel JFET.

15.	Draw the structure of opto-coupler and state any one of its applications.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the differences between Conductors, semi conductors and insulators with 	necessary energy band diagram.												    (8)

b.	Draw the forward and reverse characteristics of a PN Junction diode and discuss on the same.																		    (7)

(OR)

17.	Derive the expression for the current through the PN Junction Diode.

18.	With necessary diagrams draw and explain the input and output characteristics of a common base configuration transistor. Also highlight an application of CB configuration using the characteristics.

(OR)

19.	a.	Write short notes on Current components of a transistor.							    (7)

b.	Discuss on the Eber Moll model of a Transistor.									    (8)

20.	Draw the hybrid equivalent model of a Common emitter configuration transistor and derive its various parameters.

(OR)

[P.T.O.]

21.	Draw the input and output characteristics of a common emitter configuration BJT amplifier and predict the h-parameters.

22.	With necessary diagrams explain the construction and characteristics of Depletion MOSFET.

(OR)

23.	Explain the construction of UJT and explain its static characteristics.

24.	a.	Discuss on the forward and reverse characteristics if a Zener diode.					(7)

b.	Briefly explain the functionality of Light Emitting Diode.							(8)

(OR)

25.	Write short notes on

a.	Varactor Diodes.

b.	Tunnel Diodes.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ELECTRIC CIRCUIT ANALYSIS					      	     Time : 3 hours

Subject Code:	12EC202						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Kirchoff’s current law.

2.	What is the voltage across 10Ω resistor?

3.	Define resonant frequency.

4.	What is the possible number of phase sequence in a three phase system and what are they?

5.	Define a mesh.

6.	Two inductively coupled coils have self inductances L1 =50mH and L2 = 200mH. If the coefficient of coupling is 0.5, find the value of mutual inductance between the coils?

7.	Norton’s theorem is dual of ______________ theorem.

8.	Which theorem is valid only for linear system?

9.	What is meant by forced response?

10.	The time constant of a series RL circuit is (a) LR   (b) L/R   (c) R/L.

PART – B  (5 x 3 = 15 MARKS)

11.	Determine the current in all resistors in the circuit.

12.	For the circuit given, determine the value of Q at resonance and bandwidth of the circuit.

13.	Determine the current through each resistor in the circuit using mesh analysis.

14.	Find the current passing through 3Ω resistor in the circuit by applying superposition principle.

15.	A series RL circuit with R=30Ω and L=15H has a constant voltage V=60V applied at t=0 as shown in the circuit. Determine the current I, the voltage across resistor and the voltage across the inductor.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Calculate power and voltage of the dependent source in the circuit.

b.	Determine Vx in the circuit.

(OR)

17.	a.	Use the technique of star-delta conversion to find the equivalent resistance of the given circuit. Each resistor R is 10Ω.														(12)

b.	Compute the current through 4.7KΩ resistor, in the circuit after transforming the 9mA source into an equivalent voltage source.									(3)

18.	a.	For the circuit shown, determine the impedance at resonant frequency, 10Hz above 				resonant frequency and 10Hz below resonant frequency.					      (12)

b.	Find the resonant frequency in the ideal parallel LC circuit shown.					(3)

(OR)

19.	a.	A three-phase, balanced delta-connected load of (4+j8)Ω is connected across a 400V, 3-	phase balanced supply. Determine the phase currents, line currents and power drawn by the 	load. Assume the phase sequence to be RYB.									(12)

b.	The two wattmeter method is used to measure power in a 3-phase load. The wattmeter readings are 400W and -35W. Calculate (a) Total active power (b) power factor

(c) reactive power.															(3)

20.	Use mesh analysis to determine the three mesh currents in the circuit.

(OR)

21.	Use nodal analysis, to determine the voltages at each node for the circuit shown.

22.	Determine the maximum power delivered to load in the circuit.

(OR)

23.	a.	Apply Norton’s theorem to determine the current through 30Ω resistor in the circuit shown 	in Figure.

b.	Find the current through 10Ω resistor using Millman’s theorem.

24.	a.	A series RC circuit consists of resistor of 10Ω and capacitor of 0.1F as shown in the 	circuit. A constant voltage of 20V is applied to the circuit at t=0. Obtain the current 	equation. Determine the voltages across the resistor and the capacitor.

b.	The circuit shown consists of resistance, inductance and capacitance in series with a 100V constant source when the switch is closed at t=0. Find the current transient.

(OR)

25.	Derive an expression for the current of the given RC circuit.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ELECTROMAGNETIC FIELDS						      	     Time : 3 hours

Subject Code:	12EC203						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Let P(x1,y1z1) & Q(x2,y2,z2) are two points in rectangular co- ordinate system. Write an equation for the distance vector between P &Q.

2.	Write the relation between electric field intensity and electric flux density.

3.	Define Curl of a vector F.

4.	Write an expression for potential difference in terms of electric field intensity.

5.	Calculate the energy stored in a magnetic field in which the magnetic field intensity is HAMand magnetic flux density is B.

6.	State  Biot Savart Law.

7.	Write the point form of Ohm’s law.

8.	Find the capacitance of a parallel plate capacitor with dielectric of εr =3.5 and area 0.9m2 with a separation of 5 mm.

9.	Write Maxwell’s first equation in integral form.

10.	Poynting vector has the dimension _________.

PART – B  (5 x 3 = 15 MARKS)

11.	Use spherical coordinates and integrate to find the area of the region 0≤φ≤ α on the spherical shell of radius a. What is the area if α=2π?

12.	Find the force of interaction between two charges spaced 10 M apart in vacuum.  The charges are 4 x 10-3 and 6 x 10-3 Coulombs respectively.

13.	State Ampere’s circuital law and list its applications.

14.	Write the boundary conditions of electric fields.

15.	Write Maxwell’s equations in point form.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Given the point A(x=2, y=3 & z= –1) in rectangular co-ordinate system. Obtain the 	Cylindrical co-ordinates and the Spherical co-ordinates of the point A.				    (8)

b.	State and explain Stoke’s theorem.												    (7)

(OR)

17.	a.	Obtain the conversion equations which convert rectangular co-ordinates into spherical co-	ordinates.																	    (9)

b.	Explain briefly about line, surface and volume integrals.							    (6)

18.	a.	Find the force on a 100  c charge at (0, 0, 3) m if 4 point charges of 20  c are located on 	the x and y axes at  4m .														    (8)

b.	Derive expressions for the electric field intensity due to an infinite uniformly charged line of linear charge density of ρ C/m at a point P.       								    (7)

(OR)

[P.T.O.]

19.	a.	Three point charges q1 = 10- 6 C, q2 = -10 -6 C and q3 = 0.5 x 10 -6 C are located in the 	corners of an equilateral triangle of 50 cm side. Determine the magnitude and direction of 	the force on q3.																    (9)

b.	An infinitely long, straight and non conducting wire carries a uniform line charge of          	3 μC/m. Determine the electric field 2m away from the wire.  						    (6)

20.	a.	Using Ampere’s force law find H due to infinitely long straight conductor.			    (8)

b.	A wire carrying a current of 100 A is bent into the form of a circle of diameter 100cm. Calculate the flux density at the centre of the coil. Assume the number of turns to be 1.  (7)

(OR)

21.	a.	Determine the total force exerted by both E and H on a charge if Q = -1.2C;

v =5a x + 2ay –3az; E = –18a x +5ay –10az  v/m ; H= –4a x + 4ay +3az Tesla.			    (9)

b.	Write short note on magnetic vector potential.									    (6)

22.	a.	Determine whether or not the following potentials fields satisfy the Laplace’s equation 	a)V=x2–y2+z2     b)V= rcosφ +z     c)V= rcosθ +φ.								    (8)

b.	Describe the boundary relations at the interface between two magnetic media.		    (7)

(OR)

23.	a.	Determine the inductance of a solenoid of 2500 turns wound uniformly over a length of 	0.5cm on a cylindrical paper tube of 4 cm diameter. The medium is air. 				    (8)

b.	Discuss the theory of continuity of current.       									    (7)

24.	a.	State Poynting theorem.  Obtain the integral and point form of Poynting theorem.	    (8)

b.	Derive Maxwell equations from Faraday’s law and Ampere’s law. Explain the applications of Maxwell’s equations. 														    (7)

(OR)

25.	a.	Derive wave equation for uniform plane waves in free space.						    (8)

b.	How does wave propagate in good conductors? Explain.							    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MEASUREMENTS AND INSTRUMENTATION		      	     Time : 3 hours

Subject Code:	12EC204								 		      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Eddy current damping is used in ____________ instruments.

2.	An attraction type moving iron instrument has ____________ inductance than its corresponding repulsion type moving iron instrument.

3.	A wattmeter has two coils known as ____________ and ____________.

4.	____________ is prevented by cutting two holes or slots in the disc on the opposite side of the spindle in an energy meter.

5.	____________ is an instrument for measure high resistances.

6.	____________ is used for measuring the inductance of high Q-coils.

7.	____________ has negative temperature coefficient of resistance.

8.	LVDT stands for ____________.

9.	The resolution of a digital voltmeter with 4-digit display is ____________.

10.	Strip chart recorder is also known as ____________ recorder.

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between moving coil and moving iron instruments.

12.	How do you compensate the error due to resistance of the pressure coil in a dynamometer type wattmeter?

13.	State See-back effect.

14.	Why is Kelvin’s double bridge superior to Wheat-stone bridge for measuring low resistances?

15.	Mention the applications of X-Y recorders.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the construction and working of a Permanent Magnet Moving Coil instrument with a neat sketch.

(OR)

17.	Explain the working principle of a moving iron instrument. Also derive the expression for torque in a moving iron instrument.

18.	Describe the construction and working principle of a dynamometer type wattmeter with a neat sketch.

(OR)

19.	Explain in detail about the construction and working of a single phase induction type energy meter with a neat sketch.

20.	Explain the working principle of a Wheat-stone bridge. Also derive the relation for sensitivity of the bridge.

(OR)

21.	Explain the working of a Maxwell’s bridge for the measurement of unknown inductance. List out its applications.

[P.T.O.]

22.	Explain in detail about the working principle of a Strain Gauge. Derive an expression for gauge factor.

(OR)

23.	State Piezo-electric effect. Explain the working principle of a Piezo-electric transducer.

24.	Explain the working principle of a Digital Multi-meter with a neat block diagram.

(OR)

25.	Describe in detail the working of a spectrum analyzer with a neat block diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DIGITAL ELECTRONICS							      	     Time : 3 hours

Subject Code:	12EC205						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Mention the base of an octal number system.

2.	In Boolean algebra, which property states that an algebraic expression can be obtained when the operator and the identity elements are interchanged?

3.	For a half adder circuit, what is the expression for SUM?

4.	Mention the logic circuit which selects binary information from one of many input lines and directs it to a single output line.

5.	What is the condition for JK flip flop to toggle?

6.	What is the use of excitation table?

7.	How many clock pulses are required to get the output of a 4 bit SIPO shift register?

8.	Mention the other name for Asynchronous counter.

9.	Expand PLD.

10.	What type of transistor is used in TTL logic family?

PART – B  (5 x 3 = 15 MARKS)

11.	Simplify F(X,Y,Z) = XZ + Z(X' + XY).

12.	Design a Half Subtractor circuit.

13.	Determine the characteristic equation of SR Flip Flop.

14.	With neat diagram show the difference between Johnson counter and ring counter.

15.	Implement F(A,B,C) = ∑ (3,4,6,7) using PLA.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Simplify the Boolean function by using Karnaugh map.

F(w, x, y, z) = Σ (0, 1, 2, 4, 5, 6, 8, 9, 12, 13, 14)									  (10)

b.	Simplify the following Boolean functions to a minimum number of literals:

i.   	xyz+x'y+xy {z'

ii.   	zx+zx'y																    (5)

(OR)

17.	a.	Represent the decimal number 8620.

(i) in BCD (ii) in excess-3 code 												    (4)

b.	Simplify the following Boolean function by using the Tabulation Method.

F (W, X, Y, Z) = € (1,4,6,7,8,9,10,11,15) 										  (11)

18.	a.	Design and implement a 4 bit Binary to gray code converter circuit.				  (10)

b.	Design a Full Adder Circuit.													    (5)

(OR)

19.	a.	Implement the following function with a multiplexer F(A,B,C,D) = ∑ (0,1,3,4,8,9,15)    (8)

b.	Design a Full subtractor Circuit.												    (7)

[P.T.O.]

20.	a.	Draw the logic diagram of RS Flip Flop and obtain the characteristic table, characteristic 	equation, excitation table, truth table and graphic symbol.							  (12)

b.	Mention the difference between level triggering and edge triggering clock pulse.		    (3)

(OR)

21.	a.	Determine the reduced state table and state diagram for the state table given

| Present State | Next State |  | Output |  |
| --- | --- | --- | --- | --- |
|  | X=0 | X=1 | X=0 | X=1 |
| A | A | B | 0 | 0 |
| B | C | D | 0 | 0 |
| C | A | D | 0 | 0 |
| D | E | F | 0 | 1 |
| E | A | F | 0 | 1 |
| F | G | F | 0 | 1 |
| G | A | F | 0 | 1 |

(8)

b.	Explain how to obtain D Flip Flop from SR Flip Flop using logic diagram. Mention the 	characteristic table, characteristic equation and excitation table of D Flip Flop.		    (7)

22.	a.	With neat diagram explain the difference between Johnson counter and Ring counter.	    (7)

b.	Design a Binary ripple counter with its timing waveform.							    (8)

(OR)

23.	a.	Design a Mod 6 counter using JK Flip Flop. 										    (8)

b.	Explain the operation of SISO shift register with neat timing diagram.				    (7)

24.	a.	Explain the NAND operation with TTL logic in open collector output gate configuration.

(8)

b.	With neat diagram explain the operation of TTL NAND gate with totem pole output.

(7)

(OR)

25.	Find minimum SoP for the following PoS and implement using

F(A,B,C,D)= π(3,4,6,7,11,12,13,14,15)

PROM										       									    (5)

PLA											        							    (5)

PAL											       								    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December – 2015

Subject Title :	C++ AND DATA STRUCTURES						      	     Time : 3 hours

Subject Code:	12EC206						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give the syntax of class.

2.	Differentiate declaration and definition.

3.	What is a delete operator?

4.	Name the operators that cannot be overloaded.

5.	How to type cast using explicit type conversion?

6.	What is an abstract class?

7.	Does a non-member function need to be friend to access a class’s members?

8.	What is a linked list?

9.	Enlist the different types of sorting.

10.	Enlist the various searching techniques.

PART – B  (5 x 3 = 15 MARKS)

11.	What do you mean by encapsulation and data hiding?

12.	Write short notes on “new” operator.

13.	Write notes on exception.

14.	What are the different types of linked list? Narrate on each of them.

15.	State the properties of a complete binary tree.

PART – C  (5 x 15 = 75 MARKS)

16.	What is a constructor? Give the special features of constructor? What are the different types of constructors. Explain each of them using a sample program.

(OR)

17.	a.	With a sample program explain how classes are used to represent physical objects.	    (8)

b.	With a sample program explain how classes are used to represent data types.		    (7)

18.	What is data type conversion? Explain in detail using a sample program.

(OR)

19.	What is inheritance? Enlist the different types of inheritance. With an example program explain hierarchical inheritance.

20.	With a sample program explain the concept of virtual functions and polymorphism.

(OR)

21.	With a sample program explain the concept of friend functions.

22.	With a sample program explain the basic operations in singly linked list.

(OR)

23.	With a sample program explain the basic operations in doubly linked list.

24.	Explain bubble sort. Sort the following sequence using it.

28	24	36	29	14	75	80	31	60	49

(OR)

25.	What is searching? Explain linear search in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	SOLID STATE CIRCUITS							      	     Time : 3 hours

Subject Code:	12EC207						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the peak inversion voltage of a half wave rectifier?

2.	Zener diode maintains a constant output voltage in its_________region.

3.	Mention any two factors affecting the stability of a Q point.

4.	State the operating region of the transistor when it is used as an amplifier.

5.	In the DC equivalent circuit of a transistor amplifier, the capacitors are considered as ____________.

a.	Short     b. 	Open   c. 	Partially short     d. 	None of the above

6.	Power amplifiers generally use transformer coupling because transformer permits__________.

a) Cooling of the circuit     b) Impedance matching

c) Distortion-less output    d) Good frequency response

7.	The voltage gain of an amplifier without feedback is 3000.  Calculate the voltage gain of the amplifier if negative voltage feedback is introduced in the circuit.  Given that the feedback fraction is 0.01.

8.	What is common mode rejection ratio (CMRR)?

9.	What type of feedback is used in oscillator?

10.	Name some piezo electric materials.

PART – B  (5 x 3 = 15 MARKS)

11.	Calculate the value of inductance to be used in the inductor filter connected to a full wave rectifier at 60 Hz to provide a dc output with 4% ripple for a 100 Ω load.

12.	Which biasing configuration of BJT provides least stability? Why?

13.	Write the functions of the following capacitors in Single Stage Amplifier.

(a) Base Capacitor  (b) Emitter Capacitor   (c) Collector Capacitor

14.	Compare positive feedback and negative feedback.

15.	State Barkhausen criterion.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	With the help of schematic diagram, explain the working of a full wave rectifier with 	center tap connection.  														    (6)

b.	Derive the expressions for ripple factor, regulation and efficiency for full wave rectifier.

(9)

(OR)

17.	With a neat circuit diagram explain the operation of transistorized series feedback voltage regulator.

18.	a.	Analyze fixed bias circuit of BJT with necessary diagrams.						    (8)

b.	Derive the stability factor for fixed biasing BJT.									    (7)

(OR)

[P.T.O.]

19.	Explain self biasing FET with essential circuit diagrams.

20.	a.	Explain two stage RC coupled amplifier with neat circuit diagram and frequency response 	curve.																		    (8)

b.	Derive the mid frequency range current gain expression of RC coupled amplifier.	    (7)

(OR)

21.	a.	Prove that the maximum efficiency of a series fed, directly coupled class A amplifier is 	25%.                																  (12)

b.	A power transistor working in class A operation has zero signal power dissipation of 10 Watts.  If the ac output power is 4 Watts, then find efficiency.						    (3)

22.	Derive voltage gain, input impedance and output impedance of voltage series feedback amplifier.

(OR)

23.	Draw the circuit diagram of single ended differential amplifier and derive the following parameters for single ended differential amplifier.

a.	AC base current.

b.	AC collector current.

c.	output voltage.

d.	AC voltage gain.

24.	a.	Explain the operation of Hartley oscillator with neat circuit diagram.				    (8)

b.	Derive the frequency of oscillation of Hartley oscillator.							    (7)

(OR)

25.	a.	What are the advantages of tuned amplifier?										    (3)

b.	Explain in detail about the single tuned amplifier with neat circuit diagram and frequency response curve.																  (12)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	LINEAR INTEGRATED CIRCUITS AND APPLICATIONS													      	     								      Time : 3 hours

Subject Code:	12EC208								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the need for frequency compensation in practical op-amps?

2.	Define slew rate.

3.	What is comparator?

4.	State Barkhausen criteria.

5.	What is switched capacitor filter?

6.	Mention the applications of 555 timer.

7.	What are the advantages of R-2R ladder DAC over weighted resistor DAC?

8.	What is VCO?

9.	What is the purpose of oxidation in IC fabrication process?

10.	What are the advantages of ICs?

PART – B  (5 x 3 = 15 MARKS)

11.	What are the ideal characteristics of op-amp?

12.	What are the applications of Comparator?

13.	Draw the frequency response of band-pass and band-reject filters.

14.	Calculate the value of LSB for an 8-bit DAC for the 0 to 10V range.

15.	Why aluminium is preferred for metallization?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Derive the voltage gain expression for inverting and non-inverting amplifier using op-amp 	with neat circuit diagrams. 													  (10)

b.	Write short notes on dominant pole compensation using op-amp.					    (5)

(OR)

17.	With neat circuit diagrams, explain about the operation of Log and antilog amplifier and determine its output expression.

18.	Explain the working principle of Astable multivibrator using op-amp and derive an expression for its frequency of oscillation.

(OR)

19.	Explain the working principle of a 723 general purpose regulator with necessary diagrams.

20.	Draw the circuit diagram of first order LPF using op-amp and derive its transfer function.

(OR)

21.	Explain in detail the function of 555 timer in monostable mode and derive the expression for the pulse width.

22.	Explain the operation of phase locked loop with neat block diagram.

(OR)

23.	With the help of a neat block diagram, explain the operation of Successive approximation ADC.

24.	Discuss the various steps involved in the fabrication of IC using silicon planar process with necessary diagrams.

(OR)

25.	With neat diagrams, explain the different methods of integrated resistors fabrication.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	NETWORK ANALYSIS AND SYNTHESIS			      	     Time : 3 hours

Subject Code:	12EC209								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In a transfer function the complex or imaginary poles and zeros must be __________.

2.	Write the necessary condition for a driving point function with regard to the degree of the     numerator polynomial, P(s) and the denominator polynomial Q(s).

3.	State the Routh criterion for stability.

4.	Define transfer function.

5.	The branches of a tree are called __________.

6.	What is meant by V shift?

7.	Give one application of ABCD parameters.

8.	__________ is used in filter sections and is also used as attenuators.

9.	In a Hurwitz polynomial, all coefficients are __________.

10.	The poles and zeros of Z(s) and Y(s) all lie on the __________ of the s plane.

PART – B  (5 x 3 = 15 MARKS)

11.	Define poles and zeros of a transfer function.

12.	What is phase margin and gain margin?

13.	Define:		a.	Tree		b.	Links		c.	Co-tree of the graph.

14.	Discuss the classification of filters.

15.	List out the properties of Hurwitz polynomials.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	State the significance of poles and zeros in network functions.						    (5)

b.	State the properties of driving point functions. 									  (10)

(OR)

17.	For the network shown in the figure, determine Z11(S), Z21(S) and G21(S).

20Ω

½s

10Ω

I1(s)

I2(s)

V1(s)

V2(s)

18.	Using Routh’s criterion, determine the stability and the number of roots in the right half of the s-plane for the system whose characteristic equation s5 + 2s4 + 4s3 + 8s2 + 5s + 10 = 0.

(OR)

19.	Sketch the complex locus plot of the admittance function for the series RLC network.

20.	What do you understand by the term ‘duality’? Discuss the procedure to find out the duality of a given network having both voltage and current sources. Explain with an example.

[P.T.O.]

(OR)

21. 	a.	Explain about the concept of formulation of network equilibrium equation on loopbasis.

(8)

b.	Explain the method of constructing dual of a network	.							    (7)

22.	Derive Z parameters in terms of other parameters (Y,ABCD, h).

(OR)

23.	Design an m-derived high pass filter having a cut-off frequency of 10 kHz; design impedance of 5Ω and m = 0.4. Draw T-section and π-section.

24.	Find the two Foster realizations of the given function Z(s)=s(s2+4)(s2+1)(s2+9).

(OR)

25.	a. 	Find the second Cauer form of the driving point impedance of an LC 	networkZ(s)= s4+4s2+3s3+2s.													  (10)

b.	Write any five properties of positive real function.								    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	TRANSMISSION LINES AND WAVE GUIDES		      	     Time : 3 hours

Subject Code:	12EC211								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Characteristic Impedance.

2.	Frequency distortion is reduced in the transmission of high quality radio broadcast programs over wire lines by use of __________ at the line terminals.

3.	Write down the expression for capacitance for an open-wire line with air dielectric.

4.	On a transmission line with standing waves, give the distance between a voltage maxima and adjacent current maxima.

5.	When a transmission line of length λ/2 is used, the input impedance is ____________.                               6.	What are the applications of Smith Chart?

7.	What is dominant mode? Give example.

8.	Give the cut-off frequency when the wave is propagated by between two parallel plates for 	TEM mode.

9.	Define Attenuation factor in a waveguide.

10.	Find the resonant frequency of a copper cubic rectangular cavity of dimension 1000 cubic cm for the TE101 mode. Conductivity of the copper is 5.8  107 S/m.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the equivalent circuit of a transmission line and also discuss the existence of the line constants in a transmission line.

12.	A transmission line with characteristic impedance of 500Ω is terminated by a purely resistive load. It is found by measurement that the minimum value of voltage upon it is 5µV and maximum voltage is 7.55µV. Calculate the value of load resistance.

13.	Design a quarter wave transformers to match a load of 200 Ω to a source resistance of 600 Ω. The operating frequency is 100 MHz?

14.	Explain why TE00 mode does not exist in a parallel plate.

15.	Why TEM mode does not exist in a rectangular waveguide?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Discuss the various types of transmission lines. 									    (8)

b.	Design the equivalent T section for a transmission line.							    (7)

(OR)

17.	a.	Briefly discuss about waveform distortion.										    (5)

b.	Observe the distortion present in an ordinary telephone cable and how it can be overcome by using continuous loading.                                                        					  (10)

18.	Discuss the about voltages and currents on a dissipationless transmission line and sketch the phasors for the following resistive loads:

a.	Short Circuited Load

b.	Open Circuited Load

c.	Matched Load

d.	Resistive Load RR = 3Ro

e.	Resistive Load RR = Ro/3

[P.T.O.]

(OR)

19.	a.	Derive the relationship between SWR and reflection coefficient.                                      (9)

b.	A UHF transmission line of characteristic impedance 75Ω is terminated in an unknown 	load. The VSWR measured in the line is 3 and the position of current minimum nearest to 	the load is one-fifth wavelength away from the load. Calculate the value of load 	impedance. 																	    (6)

20.	a.	Explain how impedance matching is done using quarter- wave section. Also derive the 	input impedance of the λ/4 section.                                                         				    (9)

b.	Discuss about stub matching. What are the advantages of using double stub over single 	stub?             																	    (6)

(OR)

21.	A lossless transmission line with a characteristic impedance of 50Ω is terminated in a load     (50 + j50) Ω. Using the Smith chart find the following:

a.	VSWR.

b.	The magnitude and the angle of reflection coefficient at the load.

c.	Load admittance.

d.	Reflection coefficient at a distance of λ/4 from the load.

e.	The position of the first voltage minimum from the load.

22.	Derive the expression for electric and magnetic fields for waves between two parallel plates.

(OR)

23.	The separation between parallel plates of a waveguide is 3cm. It is filled with a dielectric with relative permittivity of 4. The signal frequency is 6GHz. Find the propagating modes. For each propagating modes, Calculate the following:

a.	Cut-off frequency.

b.	Guide wavelength.

c.	Cut-off wavelength.

d.	Phase velocity.

24.	a.	With neat diagrams, explain the excitation of modes in Rectangular waveguides.		  (10)

b.	Derive the wave impedance for TE waves in a Rectangular waveguides.                          (5)

(OR)

25.	Discuss about circular and rectangular cavity resonators in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	COMMUNICATION THEORY						      	     Time : 3 hours

Subject Code:	12EC213						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The output spectrum of has ___________ number of significant side bands.

2. 	Product modulator is also called as ___________.

3. 	The output spectrum of FM has ___________ no of significant sideband.

4. 	Ratio Detector is used to detect a ___________ signal.

5.	TRF stands for ___________.

6.	If ‘fs’ and ‘f0’ are the two input frequencies to the Mixer then the output frequency of the mixer is ___________.

7. 	The ideal value of Signal to Noise ratio for any good communication system is ___________.

8.	The function of a noise limiter is ___________.

9.	___________ noise is also called as Johnson Noise.

10.	___________ noise is also called as Pop Corn noise.

PART – B  (5 x 3 = 15 MARKS)

11.	Is 100 % modulation practically possible in communication system? Support your answer.

12. 	Compare narrow band and wide band FM.

13. 	Write a short note on the importance of IF amplifiers.

14. 	Write a brief note on Squelch Circuit.

15. 	Define Signal to Noise Ratio. Comment about the SNR value of an ideal good Communication 	system.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the output equation of an AM system. Sketch the relevant signal in time domine and frequency domine.

(OR)

17. 	Explain the operation of a Ring Modulator. Sketch the relevant signals.

18. 	Derive the output equation of an FM. Sketch the related signals. Comment on the output equation.

(OR)

19. 	Explain Armstrong method of Indirect FM Generation. Also Sketch related Phasor.

20.	a.	Compare Low level and High Level Transmitters.                                                             (5)

b.	Explain the fundamental block diagram of an AM transmitter.                                        (10)

(OR)

21.	With help of neat block diagram explain AM super heterodyne receiver.

22.	With neat block diagram explain an FM transmitter.

(OR)

23. 	Explain the importance of Pre-Emphasis and De-emphasis circuits in FM. Draw the circuits and its frequency response characteristics.

24. 	Explain the Noise interference calculation in AM system.

(OR)

25. 	Explain the Noise interference calculation in FM system.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	SIGNALS AND SYSTEMS							      	     Time : 3 hours

Subject Code:	12EC214						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Sketch x(n) = (n+1) + (n–1).

2.	Define deterministic signal with an example.

3.	Give Parseval’s relation using continuous time Fourier transform.

4.	Write two properties of region of convergence (ROC).

5.	Mention the MATLAB command used for downsampling with proper format.

6.	Define Nyquist Rate.

7.	What is the condition for existence of Fourier transform of a signal x(n)?

8.	Write the formula for Inverse Z-transform.

9.	The output of the system due to unit step signal as input is called __________.

10.	Which command in MATLAB is used to plot the frequency response?

PART – B  (5 x 3 = 15 MARKS)

11.	Test whether the given signal is periodic or not. If periodic, find the fundamental period.

x(t) = sin(πt2) +  3cos (πt4+π3)

12.	The output y(t) of a causal LTI system is related to the input x(t) by the differential equation d2y(t)dt2+6dy(t)dt+8y(t)=2x(t). Find the frequency response of the system.

13.	Differentiate Natural and Flat-top sampling.

14.	Find the discrete time Fourier transform for the signal x(n)=n,−2≤n≤20,otherwise

15.	Write the advantages of using block diagram realization of system function.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Determine whether the given system is causal, time invariant and linear. Justify your 				answers.  y(n) = cos(3n)x(n).

b.	Consider the signal x(t) shown in figure. Sketch the following waveforms:

x(t)(t), x(–t+1), 4x(2t–1)

(OR)

17.	Using graphical procedure, find the output response of an system in which

x(n) = { 2, 3, -1, 4 } and  h(n)= {-1,2,3}.

[P.T.O.]

18.	State and prove the following properties of continuous time Fourier transform.

a.	Time Shifting   	b.	Time Reversal  		c.	Time Scaling

d.	Linearity     		e.	Convolution property.

(OR)

19.	a.	Determine the Laplace transform and its associated ROC for each of the following signals:

i.	xt=e−4tu(t)+e3tu(−t)

ii.	x(t) = e-tu(t+2)

b.	Find the inverse Laplace transform: X(s)=s+1(s3+9s2+26s+24).

20.	State Sampling theorem. Explain impulse train sampling and reconstruction of Continuous time signal from its samples with neat diagrams.

(OR)

21.	a.	Write the steps involved in discrete time processing of continuous time signals in detail.

b.	For the analog signal, x(t) = 3cos(500πt)+15sin(800πt)-20cos(100πt), what is the minimum sampling rate required to avoid aliasing? Also find Nyquist interval.

22.	a.	Determine x(n) for the given Fourier transform

i.	Xejω=e−jω(1+sinω).

ii.	Xejω=2+5e−jω−2ejω+e−2jω+e2jω−3e−jω.

b.	A causal and stable discrete-time LTI system is described by

y(n) – (1/6)y(n –1) – (1/6)y(n – 2)  =   x(n)

where x(n) and y(n) are the input and output of the system, respectively.

i.	Determine the frequency response of the system, H(ej).

ii.	Find the impulse response of the system.

(OR)

23.	a.	Find the Z-transform for x(n) = –2nu(–n–1) and depict the ROC.

b.	Using long division method, find the inverse Z-transform

X(z)=z+1(z2−3z+2)when (i) x(n) is causal and  (ii) x(n) is anticausal.

24.	Obtain the cascade and parallel form realization of the system described by the difference equation.

y(n) –1/4y(n–1) –1/8 y(n–2) = x(n) +3x(n–1)+2x(n–2)

(OR)

25.	a.	Compare FIR and IIR systems.

b.	Draw Direct form-II realization for the given transfer functions

i.	H(z)=zz+7			ii.	H(z)=4z+28z2+6z+1

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MICROPROCESSORS AND INTERFACING TECHNIQUES													      	     								      Time : 3 hours

Subject Code:	12EC215								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give the microinstructions for memory read and write operations.

2.	Write the microinstructions for the branch unconditional operation.

3.	Name the hardware interrupts of 8085 microprocessor.

4.	Give an 8085 instruction to convert an eight bit number in an accumulator to four-bit binary coded decimal digits.

5.	Give the flag register format of 8086 microprocessor.

6.	Name the chips those are used with 8086 microprocessor in maximum mode configuration.

7.	Mention the modem control signals of 8251 Programmable Communication Interface chip.

8.	Name the handshaking signals of 8255 Programmable Peripheral Interface chip in mode 1 output configuration.

9.	List the signals of cascade buffer in 8259 programmable interrupt controller.

10.	Mention the scanned keyboard modes of operation of 8279 Programmable keyboard/display controller.

PART – B  (5 x 3 = 15 MARKS)

11.	Compare hardwired with microprogrammed computer organization.

12.	Mention the number of machine cycles and T states required to execute the following 8085 instructions.

MVI A,05H

SBB M

LDA 2000H

13.	List the different 8086 CPU cycles along with their bus status signals.

14.	Write the control word of 8255 to set group B signals as input ports and group A signals as output ports in mode 0 I/O configuration.

15.	Explain the keyboard/display mode set command word of 8279.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Draw and explain the flow chart of an interrupt cycle of a basic computer.   			  (10)

b.	Explain with neat diagram the bus system for 4 registers using multiplexers.       		    (5)

(OR)

17.	a.	With neat diagram explain the control unit of a basic computer.        				  (10)

b.	Draw the flowchart of an instruction cycle of a basic computer.					    (5)

18.	a.	Explain the addressing modes of 8085 with examples.							    (5)

b.	Draw the timing diagram of the following 8085 instruction. The instruction is in the memory location 4000H.                                    									  (10)

4000 STA 2000H

(OR)

[P.T.O.]

19.	a.	Write an 8085 assembly language program to disassemble a byte stored at memory 	location 2050H.Store the result at memory location 2051H and 2052H.                          (5)

b.	Design a memory system for the 8085 microprocessor such that it should contain 8 Kbyte of EPROM and 8 Kbyte of RAM. Write the address map for each chip.  			  (10)

20.	a.	Draw the block diagram of 8086 microprocessor and explain each block in detail.	  (10)

b.	Explain the unique signals used in minimum mode configuration of 8086.			    (5)

(OR)

21.	a.	With neat diagram explain 8086 in maximum mode configuration.					  (10)

b.	Explain any five addressing modes of 8086 microprocessor.     					   (5)

22.	a.	Draw the block diagram of 8251 Programmable Communication Interface chip and explain 	each block in detail.															    (9)

b.	Explain the various word formats used in 8251 chip.								    (6)

(OR)

23.	a.	Draw the block diagram of 8255 Programmable Peripheral Interface chip and explain each 	block in detail.   																    (9)

b.	With neat diagrams explain 8255 in mode 1 input and output configuration.			    (6)

24.	a.	Explain the operation of 8259 programmable interrupt controller chip with neat block 	diagram.  																	  (12)

b.	Explain the priority modes of operation of 8259.   								   (3)

(OR)

25.	a.	Explain the different modes of operation of 8253 Programmable interval timer.  		  (12)

b.	Mention the features of 8279 Programmable keyboard/display controller.			    (3)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DIGITAL SIGNAL PROCESSING					      	     Time : 3 hours

Subject Code:	12EC216										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Linear Convolution of two sequences with length N1=5 and N2=6 results in a sequence of length __________.

2.	Arrange the 8 point sequence xn=1,2,1,2,−1,−2,−1,−2in bit reversed order.

3.	Give the bilinear transform equation between s-plane and z-plane.

4.	What is the advantage of direct form II realization over direct form I realization.

5.	An FIR filter is _________ (Recursive/Non Recursive).

6.	What is the width of side lobe of rectangular window?

7.	Which limit cycle oscillation is caused by addition?

8.	Define truncation.

9.	State the advantage of adaptive filters.

10.	Give the expression of Wiener-Hopf equation.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the butterfly diagram for a 4 point DIF FFT.

12.	What are the advantages of digital filters?

13.	What is Gibbs phenomenon?

14.	Express the fraction 7/8 and -7/8 in sign magnitude, 2’s complement and 1’s complement form.

15.	What are the practical limitations of the LMS algorithm?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Perform the circular convolution of the following sequences using DFT and IDFT method.

x(n) = {1,1,2,1};   h(n) = {1,2,3,4}

(OR)

17.	a.	Compute the eight-point DFT of the sequence

x(n) =1 0≤ n≤ 70 otherwise    by using DIT-FFT algorithm.								  (10)

b.	State and prove the periodicity and linearity property of DFT.						    (5)

18.	a.	Design a digital Butterworth filter satisfying the following specifications using

Bilinear transformation technique. Assume T= 1 sec.

0.707≤ H(ejω)≤ 1; 0≤ω≤π2 H(ejω) ≤ 0.2; 3π4 ≤ ω ≤ π.											  (10)

b.	Use Impulse invariance technique to obtain H[z] for the analog transfer function

H(s)=2(s+1)(s+2). Assume T= 1sec.											    (5)

(OR)

[P.T.O.]

19.	Obtain the direct form I, direct form II, Cascade and Parallel realization of the LTI system governed by the equation

y[n]=− 0.1y[n−1]+0.2y[n−2]+3x[n]+3.6x[n−1]+0.6x[n−2]

20.	Design an ideal low pass filterwith the following frequency response using a Hanning window. Find the values of h(n)for N=7. FindH(z).

Hd(ejω) = e−j3ω for −π4≤ ω≤ π40 for π4 ≤ω≤ π

(OR)

21.	Using frequency sampling, Determine the coefficients of a linear phase FIR filter of length N=15 which has a symmetric unit sample response and a frequency response that satisfies the conditions

H2πk15= 1 for k=0,1,2,30 for k=4,5,6,7

22.	a.	Explain input quantization error with an example.								    (8)

b.	Find the steady state variance of the noise in the output due to quantization of input for the first order filter y(n)=ay(n−1)+x(n).											    (7)

(OR)

23.	a.	Illustrate zero-input limit cycle oscillations with an example.						  (10)

b.	Consider a second order IIR filter with H(z)=1(1−0.5z−1)(1−0.45z−1).Find the effect on quantization on pole locations of the given system function in direct form and in cascade form. Assume b = 3 bits.														    (5)

24.	a.	Explain the basic Wiener filter theory in detail.									    (7)

b.	Draw the various blocks in the Harvard architecture and explain them in detail.		    (8)

(OR)

25.	a.	Explain the least mean square (LMS) adaptive algorithm in detail.					    (8)

b.	Explain the process of pipelining in detail.										    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ANTENNAS AND WAVE PROPAGATION			      	     Time : 3 hours

Subject Code:	12EC217								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Front to Back Ratio.

2.	The radiation resistance of half-wave dipole antenna is __________.

3.	What is an antenna array factor?

4.	Define BWFN.

5.	Give an example of resonant and non-resonant antenna.

6.	What is the relationship between C, α, S and L?

7.	State Huygens's Principle.

8.	Define Spill Over.

9.	What are the three modes of radio wave propagation?

10.	Define MUF.

PART – B  (5 x 3 = 15 MARKS)

11.	Relate the effective aperture and directivity of an antenna.

12.	Distinguish between collinear and parasitic arrays.

13.	Derive the emf equation of loop antenna.

14.	Give the relation between dipole and slot impedance.

15.	What are the effects of earth’s curvature on tropospheric propagation?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Calculate the power radiated and radiation resistance of half wave dipole antenna.	  (10)

b.	A dipole having a length 3cm is operated at 1GHz.The efficiency factor is k = 0.6. 	Calculate Rr  and RL.															    (5)

(OR)

17.	Verify Reciprocity principle with necessary diagram.

18.	Describe a broadside array. Deduce an expression for the radiation pattern of a broadside array with four isotropic point sources.

(OR)

19.	Describe an end-fire array. Deduce an expression for the radiation pattern of a end-fire array having four isotropic point sources

20.	Explain the construction, operation and design consideration for a Yagi-uda antenna.

(OR)

21.	Illustrate the radiated fields of helical antenna. Explain its modes of operation.

22.	Brief theoretically radiation from a rectangular aperture treated as an array of Huygens's source with neat diagram.

(OR)

23.	a.	Explain various types of Horn Antenna. 											    (8)

b.	Brief about parabolic reflector antenna.											    (7)

24.	Explain in detail the layers of atmosphere.

(OR)

25.	Derive an expression for the refractive index of the ionosphere in terms of the electron number density and frequency.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MICROCONTROLLERS AND APPLICATIONS		      	     Time : 3 hours

Subject Code:	12EC218								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	On reset, the stack pointer is initialized to ____________.

2.	The flag register in 8051 is also called as ____________.

3.	Find the value for TMOD to program timer 0 in mode 2.

4.	____________bit in SCON is used for multiprocessor communication.

5.	What will be the working mode in Motorola 68HC11 if both MOD A and MOD B lines are grounded?

6.	Which port performs the alternate function of timer in Motorola 68HC11?

7.	What are the two wake up techniques used in SCI?

8.	____________ flag indicates the completion of conversion process in ADC of 68HC11.

9.	Give the status register format in PIC microcontroller.

10.	Write the instruction to clear the working register.

PART – B  (5 x 3 = 15 MARKS)

11.	Debug the following instructions.

a.	MOV A, @DPTR

b.	MUL A,B

c.	DEC DPTR

12.	Mention the enabled interrupts after the execution of MOV   IE,#1F.

13.	Brief about PAI in Motorola 68HC11.

14.	Name the signals used in serial peripheral interface of 68HC11.

15.	Mention briefly about the timers in PIC.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the various addressing modes in 8051.									    (8)

b.	Write an assembly language program to find factorial of 5.						    (7)

(OR)

17.	a.	Find the value of A and the status of C flag after execution of every DA A instruction in 	the following program.

MOV A, #42

ADD A,#13

DA A

ADD A,#17

DA A

ADDC A,#34

DA A

ADDC A,#11

DA A

(8)

b.	Draw the block diagram of 8051 microcontroller and explain about the concept of memory segmentation.																    (7)

[P.T.O.]

18.	a.	Explain the various modes of timer in 8051.										  (10)

b.	Give the format for SMOD and specify the importance of each bit.					    (5)

(OR)

19.	a.	Assuming that XTAL = 11.0592 MHz, write a program to generate a square wave of 2 kHz 	frequency on pin P1.5.														    (8)

b.	Write a program for 8051 to transfer “YES” serially at 9600 baud, 8-bit data, 1 stop bit, do this continuously.																    (7)

20.	a.	Explain the various operating modes in Motorola 68HC11.						  (10)

b.	Give a short note on ports and its alternate functions in 68HC11.					    (5)

(OR)

21.	a.	With a neat diagram explain the concept of input capture technique in Motorola series 	microcontroller.																    (8)

b.	Explain about pulsed accumulator feature in 68HC11and its associated registers.		    (7)

22.	a.	Brief about the control registers used in serial communication interface.				    (8)

b.	Write short notes on wake up features in Motorola 68HC11.						    (7)

(OR)

23.	Explain the working of ADC in Motorola 68HC11.

24.	a.	Draw the architecture of PIC microcontroller and Explain about its CPU registers.	    (8)

b.	Explain in detail about the timer 0 in PIC and its interrupt.							    (7)

(OR)

25.	a.	Explain in detail about the data transfer and arithmetic instruction in PIC microcontroller.

(10)

b.	Draw the data transfer format in I2C bus.										   (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DIGITAL COMMUNICATION						      	     Time : 3 hours

Subject Code:	12EC219						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Sampling theorem.

2.	___________ and___________ noises occur in Delta modulation.

3.	What are the two modes of operation of an adaptive equalizer?

4.	Name the algorithm used in adaptive equalization.

5.	Give an example for Non coherent modulation techniques.

6.	Plot the BPSK signal for the given sequence 01001101.

7.	Define Information rate.

8.	For a (7, 4) Hamming Code, the number of parity bits is ___________.

9.	Give the expression of processing gain.

10.	State the Balance property.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the adaptive delta modulation transmitter diagram.

12.	Explain the term eye pattern.

13.	Sketch the DPSK transmitter block diagram.

14.	State the properties of syndrome decoding.

15.	Write short notes on Rake receiver.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	What is quantization? Explain the types of quantization with necessary diagrams.	    (8)

b.	Explain encoding. Using line coding techniques, illustrate any four electrical representation for the data stream "01101001".												    (7)

(OR)

17.	With a neat block diagram explain the basic building of PCM system.

18.	What is matched filter? Obtain the impulse response of the optimum filter.

(OR)

19.	Sketch the baseband binary data transmission system. Obtain the Nyquist’s criterion for distortion less baseband binary transmission.

20.	Draw the block diagram for the generation and detection of BFSK signal and explain the same with geometrical representation.

(OR)

21.	Explain the principle and operation of QPSK transmitter and receiver with the help of block diagram and signal space diagram.

22.	Design a Convolutional encoder of code rate r=1/2 and constraint length of K=3 for the given generator polynomial (i) input-top adder output path is 111  (ii) input-top adder output path is 101.

a.	Encode the message sequence 10011.

b.	Draw the code tree and state diagram for encoding.

(OR)

[P.T.O.]

23.	Consider the generator of a (7,4) cyclic code by generator polynomial g(D) = 1+D+D3

a.	Construct the systematic generator matrix G Also find out parity check matrix.

b.	Calculate the code word for the message sequence 1100.

24.	Briefly explain the direct sequence spread spectrum with neat sketches.

(OR)

25.	What is frequency hopping? Explain with suitable diagrams, how the PN sequence is used in fast frequency hopping spread spectrum.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MODERN CONTROL SYSTEMS					      	     Time : 3 hours

Subject Code:	12EC220						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Transfer function of a system.

2.	Write a real time example for negative feedback in a control system?

3.	Write the expression for the second order undamped system for unit step input?

4.	Define steady state error.

5.	List the advantages of Bode Plot.

6.	What is phase cross over frequency?

7.	In a root locus when does break away point and break in point occur?

8.	State the Routh Hurwitz criterion.

9.	What do you mean as the state variables of the system?

10.	What are the advantages of state space analysis?

PART – B  (5 x 3 = 15 MARKS)

11.	Write the analogues between mechanical translational systems and mechanical rotational systems.

12.	The damping ratio of a system is 0.75 and the natural frequency of oscillation is 12 rad/sec. 	Determine the peak time.

13.	Draw the magnitude and phase plot for a first order factor in the denominator of a system.

14.	Give the relation between the stability and characteristic equation of a system.

15.	Mention about Controllability and observability test.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Write the differential equations governing the Mechanical rotational system shown.

Obtain the transfer function of the system. 					                            		(10)

b.	Find the overall gain of the system for the signal flow graph below

(5)

(OR)

[P.T.O.]

17.	a.	Using block diagram reduction technique, find the closed loop transfer function C/R of the 	system whose block diagram is shown below

(8)

b.	Compare the analogous quantities in force current and force voltage analogy.		    (4)

c.	The dynamic behaviour of the system described by the equation dcdt+10c=40e  where e is the input and c is the output.  Determine the transfer function of the system.       	    (3)

18.	a.	A Unity feed back control system has an open loop transfer function  Gs=10ss+2  find 	the rise time, percentage overshoot, peak time and settling time for a step input of 12 Units.

b.	 A unity feedback system is characterized by the open-loop transfer function.		    (8)

CsRs=1s0.5s+10.2s+1.Determine the steady state errors for unit-step, unit-ramp and unit-acceleration inputs.														    (7)

(OR)

19.	Obtain the response of second order underdamped system when it is subjected to unit step input.

20.	Sketch the bode plot for the given transfer function and obtain the gain margin and phase margin . G(s)=10s(1+0.4s)(1+0.1s)

(OR)

21.	The open loop transfer function of a unity feedback system is given by 	G(s)=1s(s+1)(2s+1).

Sketch the polar plot and determine the gain margin and phase margin.

22.	Sketch the root locus for the system having an Open loop transfer function

G(s)=ks(s+2)(s+4)  .

Find the value of k so that the damping ratio of the closed loop system is 0.5

(OR)

23.	a.	A system is given by s6+2s5 +2s4+3s3+5s2+6s+1=0.  Check the stability of the system using 	Routh criterion.																  (10)

b.	Give a brief idea about Proportional, PI and PID control.							    (5)

24.	A system is described by

x1x2x3=010001−6−11−6 x1x2x3+001u

Check the controllability of the system.

(OR)

25.	A feedback system has the closed loop transfer function

Y(s)U(s)=10(s+4)s(s+1)(s+3)

Construct a state model and give block diagram representation.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ADHOC NETWORKS								      	     Time : 3 hours

Subject Code:	12EC230						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	TCP is connection oriented Protocol.  (True / False)

2.	RTS means

a. Ready To Send 	b. Request To Send   c. Read To Send   d. Reply To Send

3.	________ battery technology has higher energy density.

4.	An mobile host is said to exhibit a low state of mobility when it has ________ associativity ticks.

a.	1 		b.	0		c. 	High	d. 	Low

5.	Define route reconstruction time.

6.	TCP Snoop addresses ________ issues due to the presence of wireless links.

a. 	bandwidth	  b. 	Delay		c. 	Packet loss		d. 	Jitter

7.	In multicasting, connection through which encapsulated packets are transmitted is called _______ tunnel

a. 	broadcast	b. 	Multicast	c. 	Unicast

8.	What information is used by LBM protocol to distribute multicast traffic?

9.	What is NeSt (Network Status) module in Mobile MAN Architecture?

10.	________ is used as a reference routing protocol for the MobileMAN cross-layer architecture.

a. 	AODV		b. 	OLSR	  	c. 	DSR      d. 	DSDV

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate Synchronous and Asynchronous protocols.

12.	Write short notes on ORA (Optimum Routing Approach) and LORA (Least Optimum Routing Approach).

13.	What is an Additive Increase Multiplicative Decrease phenomenon in TCP Reno?

14.	What are the different operations ODMRP depends upon?

15.	What is SER integration in real world evaluation of MANETS?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the different types of Mobile host movements with diagrams.				    (8)

b.	State the need of ad hoc MAC protocols.  Explain in detail the Hidden Terminal problem in Ad hoc Channel access.														    (7)

(OR)

17.	a.	Elaborate the challenges faced by ad hoc mobile networks.						    (8)

b.	Write short notes on Wireless Sensor Networks.									    (7)

18.	a.	Illustrate the various power issues in Ad hoc Networks.							    (8)

b.	Write short notes on effects of beaconing on battery life.  							    (7)

(OR)

[P.T.O.]

19.	With neat diagrams and frame format describe Associativity based routing with its protocol description.

20.	a.	Explain in detail TCP Reno and TCP Tahoe versions.								  (12)

b.	What are the problems faced by TCP in wireless last hop?							    (3)

(OR)

21.	a.	What are the performance parameters of Ad hoc wireless network? Define them.		    (7)

b.	What are the factors that affect Route Discovery (RD)?							    (8)

22.	a.	Explain multicast route establishment in Distance Vector Multicast Routing Protocol with 	neat diagrams.																  (10)

b.	Compare multicast routing protocols based on protocol characteristics and multicast operations.																	    (5)

(OR)

23.	a.	What are the four components in ABAM? Explain them.   						  (10)

b.	Briefly explain the working CAMP protocol.										    (5)

24.	Discuss in detail about network topologies, node placement, and movement and traffic patterns in real-world evaluation of Mobile Ad hoc Networks.

(OR)

25.	a.	Draw the cross-layer and enhanced architecture of MobileMAN neatly and explain in 	detail.																		  (12)

b.	What is middleware? How it works?											    (3)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DIGITAL DESIGN USING VHDL					      	     Time : 3 hours

Subject Code:	12EC232						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Why PLDs are used?

2.	Name the different programming technologies.

3.	Define RTL.

4.	What do you mean by address mapping?

5.	What is Programmable Interconnect Points (PIPs)?

6.	Define Full Custom Design.

7.	Define Entity Declaration.

8.	Give the syntax of CASE Statement.

9.	With Example define Component Declaration statement.

10.	__________ delay is called default delay.

PART – B  (5 x 3 = 15 MARKS)

11.	Design the following functions using ROM

F1 (A1, A0)=Σm (1, 2)

F2 (A1, A0)= Σm (0,1, 3)

12.	With neat schematic explain the RTL notation Y←A ∨ B(A & B are 3 bits).

13.	With neat diagram briefly describe the architecture of FPGA.

14.	With examples describe in detail about WAIT Statements.

15.	By using Conditional signal assignment statements write VHDL code for a 4X1 multiplexer.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Realize the following functions using PAL:										(12)

W(A,B,C,D)= Σm(2,12,13)

X(A,B,C,D)= Σm(7,8,9,10,11,12,13,14,15)

Y(A,B,C,D)= Σm(0,2,3,4,5,6,7,8,10,11,15)

Z(A,B,C,D)= Σm(1,2,8,12,13)

b.	Write about PALASM syntax for Sum and Carry expression of Full Adder.			    (3)

(OR)

17.	a.	Implement the following Boolean logic equations in PLA.							    (5)

Z=Q2X+Q3X

D1=XQ2Q3

D2=Q2Q3+XQ1+XQ2Q3

D3=Q2X+Q3X

b.	Implement the binary to gray code converter (4 bits) using PROM where A, B, C, D are inputs and W, X, Y,Z are outputs.     											  (10)

[P.T.O.]

18.	a.	Write a RTL description for the following logic system and implement it. “The data on the 	X lines (2 in number) are to be loaded in a Register A. The data in A are then to be 	complemented in Register C. With the data in C, next shifted into Register B, the data in 	Register A and Register B is then to be ORed, with the result being stored in Register C 	again. Finally the data in register C are to be placed on Z output lines. All the registers are 	2-bit long”.																	  (10)

b.	Describe the organization of a  hypothetical tiny computer  with a neat diagram.		    (5)

(OR)

19.	a.	With the design flow, list the different operators and sub-programs used in ABEL. 	  (10)

b.	 Discuss about Horizontal and Vertical format in Micro-instruction.				    (5)

20.	Explain about Standard Cell design and different types in Gate–Array based design with neat diagrams.

(OR)

21.	With neat diagram explain in detail about Xilinx 3000 FPGA and also describe how a 32-bit LUT can realize different functions.

22.	a.	Write a VHDL Program for four bit full adder, using the structural description. Draw its 	basic structure showing the inputs and outputs signals.							  (10)

b.	List the types of data operators available in VHDL with examples.  				    (5)

(OR)

23.	a.	Explain in detail about different scalar data types.								  (10)

b.	Write a VHDL program for D Flip-flop using behavioral modeling. 				    (5)

24.	a.	Design a 2 bit magnitude comparator in Behavioral modeling using VHDL (use if-then-	else OR CASE statement).														  (10)

b.	With an example differentiate variable and signal assignment statement.			    (5)

(OR)

25.	a.	With neat block diagram discuss about test bench and write the test bench for half adder.

(10)

b.	Explain in detail about selected signal assignment statement with an example.		    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	COMPUTER COMMUNICATION					      	     Time : 3 hours

Subject Code:	12EC233										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Determine the number of links needed to connect 100 computers in mesh topology.

2.	Name the categories of guided transmission media.

3.	Determine the redundant bit for the data 1100001 using VRC method.

4.	What is piggybacking?

5.	Mention the channel access method used in IEEE 802.11.

6.	What are the different types of bridges?

7.	List the OSI layers used in routers.

8.	What are the types of masking used in subnet?

9.	Name the protocol used to copy the file from one host to another.

10.	Mention the categories of web documents.

PART – B  (5 x 3 = 15 MARKS)

11.	Discuss the components used in data communication networks.

12.	Explain the procedure for stop and wait protocol in flow control.

13.	Draw the frame format of token in 802.4  standard(token bus).

14.	Extract the sub network address from an IP address 86.25.93.7 if the mask is 255.255.0.0.

15.	List any six generic domain labels.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the structure of OSI model and explain the functions of each layer.

(OR)

17.	a.	Discuss the different types of topology used in computer networks.				    	  (10)

b.	Encode the data bits 1100110110 in

i.	NRZ – L      		ii.	NRZ-I												    (5)

18.	a.	With frame format explain the principle of bit stuffing in HDLC protocol.			  (10)

b.	Discuss the principle of Cyclic Redundancy Check(CRC).

Find the redundant code if the data is 1001101 and the divisor is 1101.				    (5)

(OR)

19.	a.	Describe the different methods of sliding window with relevant diagrams.			  (10)

b.	Detect and locate the error position of the data 1001101 using hamming code.		    (5)

20.	a.	Narrate the features of timer registers used in FDDI and write the station procedure.	    (8)

b.	Explain the channel accessing method used in IEEE 802.5 token ring. Draw the frame 	format.																		    (7)

(OR)

21.	a.	Describe the principle and features of SONET.									  (10)

b.	Explain the frame format of   IEEE 802.11.										    (5)

[P.T.O.]

22.	a.	Determine the class, netid, hostid and network address of the given IP addresses

i.	90.32.3.5		ii.	200.40.3.8

iii.	150.90.62.5		iv.	250.68.92.9											    (8)

b.	Illustrate the principle of distance vector routing algorithm with an example and write the routing table for any one router.												    (7)

(OR)

23.	a.	Determine the shortest path trees and routing table for the router C using dijikstra 	algorithm.

(10)

b.	Discuss the steps involved to communicate data in switched virtual circuit.			    (5)

24.	a.	Draw the TCP segment format.													    (5)

b.	With the diagram describe the concept of encryption and decryption and explain in  detail about any three conventional  encryption methods.								  (10)

(OR)

25.	a.	Explain in detail about UDP data gram.											    (5)

b.	Draw the entire e-mail system and narrate the features of SMTP protocol.			  (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MEMS AND NANOELECTRONICS					      	     Time : 3 hours

Subject Code:	12EC234							 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is oxidation?

2.	Define Sputtering.

3.	Define Reynolds number.

4.	What is capillary effect?

5.	What are the requirements for MEMS post packaging process?

6.	What are the materials used for MEMS manufacturing?

7.	Define scaling.

8.	What is channel width?

9.	What is RTD?

10.	Where superconducting devices used?

PART – B  (5 x 3 = 15 MARKS)

11.	Write short notes on Surface tension, Micro-pumping.

12.	Explain the heat conduction in multilayered thin films and heat conduction equation.

13.	Write short notes on Die level packaging.

14.	List the issues related to scaling.

15.	Explain tunneling effect.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain briefly about micro systems in automotive industry.

(OR)

17.	Explain about MEMS and micro system products.

18.	Explain in detail about the various processing techniques used in MEMS fabrication.

(OR)

19.	Describe any two types of micro actuation.

20.	Explain the materials used in bulk micro manufacturing technique in MEMS.

(OR)

21.	Describe about polymers and piezo electric crystals.

22.	Enumerate fundamentals of nano electronics.

(OR)

23.	Brief micro electronics to nano electronics.

24.	Describe Josephson tunneling diode.

(OR)

25.	Explain three terminal RTD technologies.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	CELLULAR MOBILE COMMUNICATION			      	     Time : 3 hours

Subject Code:	12EC236								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a forward Channel?

2.	Name any one third generation standard.

3.	Define Grade of Service.

4.	What is MIN?

5.	When the mobile is moving towards the direction of arrival of the wave, the Doppler shift is __________.

6.	On which radio propagation mechanism does Durkin’s model based on?                                                                        7.	Equalizer is a __________ filter of the channel.

8.	Name a time domain class of vocoder.

9.	Multiple access scheme used in AMPS is __________.

10.	What is SIM in GSM?

PART – B  (5 x 3 = 15 MARKS)

11.	WLL is preferred more than the traditional telephone networks. Why?

12.	For a cellular system using hexagonal geometry, find the Co-channel reuse ratio for a cluster size of 12.

13.	Why two ray ground reflection model is said to be reasonably accurate than Friis free space model?

14.	Compare LMS gradient DFE and Kalman RLS equalization algorithms.

15.	List any three significant features of IS-95.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	“Satellite systems offer global coverage”. Explain with a neat block diagram. 		  (10)

b.	Enumerate the trends in personal wireless systems. 								    (5)

(OR)

17.	a.	With a neat block diagram explain the cellular system. 							  (10)

b.	State the advantages and applications of 3G wireless networks. 					   (5)

18.	a.	What is Co-channel Interference? 												   (5)

b.	Show that the Co-channel interference determines frequency reuse and capacity of the system. 																		  (10)

(OR)

19.	A cellular system aims to achieve the following 	a) Increasing the system capacity b) Reducing co-channel interference c) Reducing Handoffs. Suggest suitable techniques to achieve them.

20.	Derive the equation for received power in a two-ray ground reflection model.

(OR)

21.	a.	What are the factors influencing small-scale fading?   							   (5)

b.	What are the different types of small-scale fading undergone by the signal propagating through mobile radio channel? 													  (10)

[P.T.O.]

22.	a.	With suitable diagram explain Space diversity.									  (10)

b.	What is a RAKE receiver? Briefly explain 										   (5)

(OR)

23.	a.	What are Vocoders? Explain with suitable diagram. 								   (5)

b.	With suitable block diagram, explain Linear predictive coder. 						  (10)

24.	a.	List AMPS radio interface specifications. 										   (5)

b.	With block diagram elucidate the GSM system architecture.  						  (10)

(OR)

25.	a.	Compare the protocol architecture of OSI model and SS7 model. 					  (10)

b.	Explain message transfer part of SS7. 											   (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	BIOMEDICAL INSTRUMENTATION				      	     Time : 3 hours

Subject Code:	12EC238										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	ECG Stands for __________.

2. 	The Bio Electrical generator of heart is situated at __________.

3. 	Define the term Thermistor.

4. 	List the methods for measuring blood flow.

5. 	What is diathermy?

6. 	Define Defibrillators.

7. 	What is the range of pH value of blood?

8. 	Mention the different types of Chromatography.

9. 	MRI Stands for __________.

10. 	List the applications of Bio- Telemetry.

PART – B  (5 x 3 = 15 MARKS)

11. 	Define action potentials.

12. 	What is the principle of Plethysmograph?

13. 	What is Pacemaker?

14. 	Write short note on Flame photometer.

15. 	Define X-ray generation efficiency.

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain Bio- Medical parameters of Cardio-Vascular System along with Characteristics and Corresponding transducer

(OR)

17. 	Explain Electromyography (EMG) - Lead system, recording setup with suitable diagram.

18.  	a.	Explain the working principle of any one method of Measuring Blood Flow.			    (8)

b.	Explain any one method of Measuring Blood Pressure.							    (7)

(OR)

19. 	a.	Explain in detail about the different methods employed in measuring the Cardiac output.

(8)

b.	Describe the method used to determine the Total Lung Capacity. 	             	         	    (7)

20.	Explain in detail about:

a.	Pacemaker Circuit. 					         	            							    (8)

b.	Defibrillator Circuit.															    (7)

(OR)

21.  Explain the working principle of Heart Lung Machine with neat diagram.

22. 	Explain in detail about the working principle of Spectrophotometer with neat diagram.

(OR)

23.	a.	With suitable diagram explain how pH of Blood is measured. 	             				    (8)

b.	Write short notes on Electrical Safety Hazards in hospitals.						    (7)

24.	a.	Explain the Working of X-Ray Machine.				            					    (8)

b.	Write a note on Magnetic Resonance Imaging System.		            				    (7)

(OR)

25.  Explain in detail about how 3D images can be formed using Computerized Tomography (CT).

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	NEURAL NETWORKS AND FUZZY SYSTEMS		      	     Time : 3 hours

Subject Code:	12EC239								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define artificial neural network.

2.	Write the Mc Culloch’s pit for the XOR function.

3.	What do you mean by biases and threshold in neural networks?

4.	State the equation for squared error for particular training pattern.

5.	Draw MAXNET.

6.	Sate various LVQ algorithms.

7.	Define fuzzy set.

8.	Name the operations that could be performed on fuzzy relations.

9.	State conjunction rule.

10.	List out the limitations of fuzzy output.

PART – B  (5 x 3 = 15 MARKS)

11.	How would you characterize neural network?

12.	State the transfer function of discrete Hopfield network.

13.	What do you mean by counter propagation in networks?

14.	State the non linear function for fuzzy ARK structure.

15.	State the difference between fuzzy and crisp classification.

PART – C  (5 x 15 = 75 MARKS)

16. 	List out the application areas of neural networks and bring out valiant features in it.

(OR)

17. 		Discuss about the various networks used in neural networks and list out the techniques used to train them.

18. 	Explain the ADALINE network and it’s algorithm with suitable examples.

(OR)

19. 	Discuss the architecture of perceptron and state few examples.

20. 	Kohonen self organizing maps- how does it aid in clustering networks.

(OR)

21. 	Discuss about the adaptive resonance models in detail with suitable examples.

22. 	Describe the methods available for defuzzifying fuzzy inputs in detail.

(OR)

23. 	How does the approximate reasoning method helps to evaluate the fuzzy rules?

24. 	How does pattern classification is performed using fuzzy C means technique.

(OR)

25.  	How does fuzzy logic technique helps in image detection?

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MICROWAVE AND OPTICAL COMMUNICATION ENGINEERING													      	     					      Time : 3 hours

Subject Code:	12EC243								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the function of waveguide flange?

2.	Name any one coaxial connector.

3.	Mention the time taken by the electron to traverse through drift space.

4.	What is the application of TWTA?

5.	What are the applications of GUNN diode?

6.	What is VSWR?

7.	Define Snell’s Law.

8.	What are the types of bending loss?

9.	What are the modes present in LASER diode?

10.	Mention the semiconductor materials that can be used for optical sources.

PART – B  (5 x 3 = 15 MARKS)

11.	State the properties of S matrix.

12.	What do you mean by coaxial connector and adapter?

13.	What is the main purpose of slotted section with line carriage?

14.	Explain Meridional and skew rays.

15.	Differentiate LASER and LED.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the scattering matrix for Magic Tee and mention its applications.

(OR)

17.	Explain the scattering matrix of circulator with example.

18.	Explain the operation of two cavity klystron and derive an expression for velocity modulation.

(OR)

19.	a.	Derive an expression for round trip transit time of Reflex klystron.		        		    (8)

b.	With neat diagram explain the working of TWTA.								    (7)

20.	Compare the working principles of IMPATT and TRAPATT diode.

(OR)

21.	a.	Explain the operation of GUNN diode.											    (8)

b.	How will you measure the impedance of unknown load using slotted line method?	    (7)

22.	a.	Differentiate types of fibers with neat diagrams.									    (8)

b.	A step index glass fiber has higher core index and lower cladding index of 1.50 and 	1.45.Find out critical angle, corresponding acceptance angle of fiber in air, numerical 	aperture.																		    (7)

(OR)

[P.T.O.]

23.	When an optical signal passes through the fiber, it is subjected to attenuation and dispersion. Explain the factors responsible for attenuation and dispersion.

24.	a.	Mention the criteria for choosing the photo detectors for optical communication. How does 	a reverse bias p-n diode act as a detector? 										    (8)

b.	What is the population inversion? Explain the mechanism of Population inversion with example.																	    (7)

(OR)

25.	a.	Discuss the impact ionization in avalanche photodiode. 							    (7)

b.	Differentiate the different structures of LED.									           (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MOBILE COMMUNICATION						      	     Time : 3 hours

Subject Code:	12EC244						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define spread spectrum.

2.	MAC is a __________.

3.	The modulation technique used in DECT is __________.

4.	Give main disadvantages of TETRA.

5.	The IEEE 802.11 standard for wireless LANs defines two services: __________ and ___________.

6.	ISM band of Bluetooth device is _________.

7.	List the types of routing protocol.

8.	What is VoIP service?

9.	Mention the role of transaction layer in WAP.

10.	What is mobile TCP?

PART – B  (5 x 3 = 15 MARKS)

11.	Define hand off in a mobile network.

12.	What are the four possible hand over scenarios in GSM?

13.	Mention the design goals of WLANs.

14.	What are the functions of Radio access layer?

15.	List the classes of transaction service of WAP.

PART – C  (5 x 15 = 75 MARKS)

16.	What is multiplexing? Explain the SDMA, FDMA, TDMA and CDMA.

(OR)

17.	a.	Discuss the concept of frequency reuse. 											    (8)

b.	Explain the handoff strategies in wireless system. 								    (7)

18. 	a.	Explain the GSM architecture with diagram.				 						    (8)

b.	Explain the mobility management in GSM. 										    (7)

(OR)

19. 	a.	Explain GPRS architecture in detail. 											    (8)

b.	Explain the GPRS procedures in detail. 											    (7)

20.	a.	Explain the various IEEE 802.11 standards in detail. 								    (8)

b.	Explain the service offered by IEEE 802.11 standard. 							    (7)

(OR)

21. 	a.	Explain about Bluetooth. 														    (8)

b.	Write short notes on Hyper LAN. 												    (7)

22.	a.	Describe in detail Dynamic host configuration protocol. 							    (8)

b.	Discuss the routing protocols in MANET.							 			    (7)

[P.T.O.]

(OR)

23. 	a.	Explain about WATM services with neat sketches.				 				    (8)

b.	Explain the quality of service access point control protocol.			 			    (7)

24. 	a.	Write notes on Indirect TCP and Mobile TCP. 									    (8)

b.	Discuss Transaction Oriented TCP in detail. 										    (7)

(OR)

25. 	a.	Explain the function of the components of the WAP architecture. 					    (8)

b.	Describe WAP user agent profile in detail. 										    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	TELEVISION AND VIDEO ENGINEERING			      	     Time : 3 hours

Subject Code:	12EC245								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What are the 3 parts of the composite video signal?

2. 	Define aspect ratio.

3. 	Why horizontal polarization is preferred for television receiving antenna?

4. 	Name the process which reduces the scanning beam current to zero.

5. 	What do you understand by Hue?

6. 	Name the commonly used color phosphors.

7. 	Name the essential components of RF section.

8. 	What is the use of comp filter?

9. 	List some merits of high definition television.

10. 	What are the advantages of EDTV over HDTV?

PART – B  (5 x 3 = 15 MARKS)

11.	What do you understand by flicker?

12. 	Write the significance of DC reinsertion?

13.  Why is (G-Y) not suitable for transmission?

14. 	Write short notes on color killer circuit.

15. 	Mention some of the applications of satellite TV.

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain the Interlaced scanning method and describe how the flicker is solved by interlaced scanning.

(OR)

17.	Give the constructional details of a Monochrome picture tube and explain the function of each

component.

18. Draw the block diagram of a monochrome TV receiver and briefly explain the operation of TV

receiver.

(OR)

19.  Explain in detail about the operation of Vision IF subsystem.

20.	Explain the gun structure and shadow mask structure of Trinitron colour picture tube and also explain the pin cushion correction techniques.

(OR)

21.	a. 	Explain colour television camera with neat sketch.                      					    (8)

b. 	Write briefly about generation of colour signals.                                     				    (7)

22.	Explain in detail about SECAM system.

(OR)

23.	Describe the functional block diagram of a PAL coder.

24. 	Explain the principles of digital video broadcasting and digitization of TV picture.

(OR)

25.  	Write in detail about flat panel television receivers.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MICROPROCESSOR AND MICROCONTROLLER	      	     Time : 3 hours

Subject Code:	12EC246								 		      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give an example for two byte instruction.

2.	What are the registers available in 8085 microprocessor?

3.	How many T-states are necessary to perform the following 8085 instruction:  MVI B, 40H

4.	Differentiate maskable and non maskable interrupts of 8085 microprocessor.

5.	Write any two handshake signals.

6.	How many bytes does the Restart Instruction have?

7.	Mention the purpose of SBUF register of 8051.

8.	Mention the size of DPTR and stack pointer in 8051 microcontroller.

9.	What is the need for a Watchdog Timer?

10.	Write the instruction to check the particular bit of a port.

PART – B  (5 x 3 = 15 MARKS)

11.	Write instructions to set the zero flag when a register pair (such as BC) is used as a down counter in 8085 microprocessor.

12.	What are the various types of machine cycles?

13.	Define DMA. What are the two pins available for DMA communication?

14.	Draw the internal organization of the RAM the 8051 microcontroller.

15.	Write any three addressing modes of 8051 microcontroller with an example.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the functional block diagram and explain the architecture of 8085A Microprocessor.

(OR)

17.	Classify the instruction set of 8085 in detail according to the functional group with examples.

18.	a.	Describe in detail, with a timing diagram, the memory read cycle and memory write cycle. 																				  (12)

b.	Write short notes on Instruction Cycle and T- State. 								    (3)

(OR)

19.	Define Machine Cycle. Also draw and explain the timing diagram for the given Instruction:

| Memory Location | Machine Code | Instruction |
| --- | --- | --- |
| 2000H 2001H | 0011 1110  3EH 0011 0010  32H | MVI A, 32H Load byte 32H in the Accumulator |

20.	With a block diagram explain each block of 8255A Programmable Peripheral Interface in detail.

(OR)

21.	 Explain the operation of the 8279 keyboard / display interface with a neat block diagram.

[P.T.O.]

22.	a.	Describe the various interrupts and their associated priorities in 8051 microcontroller.  (10)

b.	Explain the features of 8051 microcontroller.									    (5)

(OR)

23.	a.	Write an assembly language program to count the number of ones in a byte. If it is an odd 	number store the result as 00 or else FF using 8051 Microcontrollers. 				    (8)

b.	Write an ALP to find the factorial of a given number using 8051 Microcontroller.	    (7)

24.	Interface a stepper motor with 8051 and explain its operation with necessary diagrams.

(OR)

25.	With a neat sketch explain how a 4x4 keypad is interfaced with 8051 for keyboard scanning.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MICRO ELECTRO MECHANICAL SYSTEMS			     Time : 3 hours

Subject Code:	12EC248								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	__________ is the base material used in MEMS.

2. 	Give application of telecommunication in MEMS.

3. 	What is surface micromachining?

4. 	List the types of etching.

5. 	Metal oxide gas sensors works on a principle of __________ sensor.

6. 	What are the advantages of chemical sensor?

7. 	What is meant by an actuator?

8. 	Give the types micro motors used in micro machines.

9. 	Give one application of RF electronic devices.

10.	__________ is the fundamental particle for light.

PART – B  (5 x 3 = 15 MARKS)

11. 	Brief on isotropic etching.

12. 	Write the difference between bulk micromachining and surface micromachining.

13. 	List different types of chemical sensor.

14. 	Draw a schematic diagram of microvalves.

15. 	Write the importance of optical switch in MEMS.

PART – C  (5 x 15 = 75 MARKS)

16. 	Draw the intelligent Microsystems and discuss its operation in detail.

(OR)

17. 	Explain fabrication steps of LIGA process in MEMS and explain its importance in detail.

18. 	Explain the surface micromachining in detail.

(OR)

19. 	Write the mechanisms of gear train and explain its operation in detail with relevant diagrams.

20. 	With a neat sketch, describe the working principle of Mechanical sensor and its applications in detail.

(OR)

21. 	Write the operation of bio medical sensor and its application in MEMS in detail.

22.	What is the use of micromotor in MEMS and explain its operation with schematic diagram.

(OR)

23.  	Explain the constructional features of micropump with figure and describe its working.

24. 	What is the significance of RF MEMS systems and explain the various types of applications in detail.

(OR)

25. 	Draw the structure of optical switches and discuss its operation based on MEMS applications.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	WIRELESS SECURITY								      	     Time : 3 hours

Subject Code:	12EC249						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is MANET?

2.	Define black hole.

3.	What is an adhoc network?

4.	Define routing algorithm.

5.	How to prevent rushing attack?

6.	Define throughput.

7.	Write the name of the fields in the request initiation message.

8.	What is SEAD?

9.	What is MAC?

10.	What is the function of a correction scheme?

PART – B  (5 x 3 = 15 MARKS)

11.	What is impersonation?

12.	Give the classification of IDS.

13.	What are the effects of false detection?

14.	Discuss the disadvantages of byzantine failure.

15.	What are the classifications DoS attacks at MAC Layer?

PART – C  (5 x 15 = 75 MARKS)

16.	In Detail explain the types of attacks on routing.

(OR)

17.	a.	In detail explain the routing table overflow.	 									    (8)

b.	Explain in detail the security flaw in AODV. 									    (7)

18.	In detail explain network based and host based intrusion detection techniques.

(OR)

19.	In detail explain the intrusion detection in MANETs using distributed IDS and mobile agents.

20.	In detail explain the dynamic source routing algorithm.

(OR)

21.	Describe in detail about the packet leashes as a defense against worm hole attack.

22.	In detail explain self organized network layer security in MANETs.

(OR)

23.	In detail explain the on demand secure routing protocol resilient to Byzantine failures in wireless networks.

24.	Explain in detail various challenges in routing security.

(OR)

25.	In detail explain about detection and handling of MAC layer misbehavior.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FUNDAMENTALS OF NANOSCIENCE				      	     Time : 3 hours

Subject Code:	12EC250										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Nanotechnology.

2.	What is meant by top-down and bottom-up approach.

3.	Define Lithography.

4.	What is meant by diffraction?

5.	List some biohazards.

6.	Define sputtering.

7.	Define quantum dots?

8.	XRD stands for ___________.

9.	Differentiate SEM and TEM.

10.	What is meant by clean room?

PART – B  (5 x 3 = 15 MARKS)

11.	List the applications of nanotechnology on various fields.

12.	Define self assembly.

13.	Write short notes on Plasma etching.

14.	What is clean room? How can one define the clean room class?

15.	What is meant by nano indentation?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain different classification of nanostructured materials with example.

(OR)

17.	What are the nano scales involved properties? Explain nano scale effect on these properties.

18.	Discuss in detail about molecular beam epitaxy and atomic layer epitaxy.

(OR)

19.	a.	Explain about Vapour Phase Deposition with neat diagram.						(8)

b.	Discuss in detail about Mechanical Milling with neat diagram						(7)

20.	Define etch resists. Explain the processes and techniques involved in dip pen lithography?

(OR)

21.	Explain the construction and processes on X-ray lithography systems.

22.	Describe the chemical and biological contamination occurred in preparation environment.

(OR)

23.	Explain various hazards and safety issues in preparation environment.

24.	Describe X-ray diffraction technique and scanning electron microscopy.

(OR)

25.	Discuss in detail about various surface analysis techniques.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	BASICS OF MOBILE COMMUNICATION			      	     Time : 3 hours

Subject Code:	12EC251								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The actual radio coverage of a cell is called ____________.

2.	Write two examples for wireless communication system.

3.	Define propagation model.

4.	What occurs when the radio path between the transmitter and receiver is obstructed by sharp edges?

5.	Write the use of equalization in mobile communication.

6.	Mention the use of interleaving in digital communication.

7.	What are vocoders?

8.	____________ multiple access technique assigns individual channels to individual users.

9.	Why the second generation system was developed?

10.	What are the different 3G wireless networks available?

PART – B  (5 x 3 = 15 MARKS)

11.	Determine the number of cells per cluster and frequency reuse factor, if i=3 and j=2.

12.	List the three important small-scale fading effects.

13.	What are the different factors used to determine the performance of an algorithm?

14.	Write the features of TDMA.

15.	Differentiate TDD and FDD.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	What are the two major types of system-generated cellular interference? Explain each one 	in detail.

b.	Write short notes on trunking and Grade of Service

(OR)

17.	Define frequency reuse. Illustrate the cellular frequency reuse concept with neat diagrams.

18.	Classify Small-scale fading based on multipath time delay spread and Doppler spread.  Elaborate each in detail

(OR)

19.	a.	Using Friis free space propagation model, derive the expression for path loss in dB.   	(10)

b.	Illustrate Doppler effect with diagram.											    (5)

20.	With neat block diagrams, explain QPSK transmitter and receiver.

(OR)

21.	What is meant by Diversity Reception? Explain the different types diversity techniques.

[P.T.O.]

22.	a.	Explain in detail CDMA multiple access technique.

b.	Write short notes on Space Division Multiple Access Technique.

(OR)

23.	Describe the traditional speech generation model used as basis for vocoding systems. Explain Linear Predictive Coder in detail.

24.	With neat diagram, explain the various subsystems in GSM system architecture.

(OR)

25.	a.	Explain the functional concept of DECT.

b.	Differentiate GPRS, IS-95B and W-CDMA based on channel bandwidth, duplexing and infrastructure change required.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FUNDAMENTALS OF DIGITAL IMAGE PROCESSING

Time : 3 hours

Subject Code:	12EC252								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define N4(P).

2.	Digitizing the amplitude values is called __________ whereas digitizing the coordinate values is known as __________.

3.  	What is image enhancement?

4.  	List the three basic types of functions used for image enhancement.

5.  	Segmentation subdivides an image into its __________.

6.	List the two categories of segmentation algorithms for monochrome images.

7.	Give any two image compression standards.

8.	Give an example of variable length coding.

9.	What does ‘morphology’ denote in image processing?

10.	Define the term ‘Thinning’.

PART – B  (5 x 3 = 15 MARKS)

11.	Write the 2-D discrete Fourier transform pair.

12.	Describe any two implementation of intensity-level slicing.

13. State the three fundamental steps performed in edge detection.

14.	Draw the block diagram of a general image compression system.

15.	State the three properties to be satisfied by ‘closing’ operation.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Draw the block diagram of a general purpose image processing system and explain the role 		of various components.                                                                             				    (6)

b. 	Explain any three applications of digital image processing. 						    (9)

(OR)

17.	Explain RGB color model with a neat sketch. Describe the technique of converting colors from RGB to HSI.

18.	Explain the implementation of 2-D Laplacian second order derivatives and their use in image sharpening.

(OR)

19.	Describe the histogram equalization technique applied for image processing.

20.	a. 	Discuss the concept of point detection using second derivatives. 					    (7)

b. 	Discuss the concept of line detection using Laplacian mask. 						    (8)

(OR)

21.	Explain region growing, region splitting and merging with suitable algorithms.

[P.T.O.]

22.	Using Huffman coding technique, find the average length of the code and entropy for the code word “abcdef”. The probability for each symbol is given as:

a=0.1, b=0.4, c=0.06, d=0.1, e=0.04 and f=0.3.

(OR)

23.	a. 	Explain the three different coding systems defined in JPEG standard.                   		    (7)

b.	Describe the motion compensated video compression with a neat DPCM/DCT encoder   	system.                            								  						    (8)

24.	Explain the operations Erosion and Dilation with necessary sketches. Show that Erosion and Dilation are duals of each other.

(OR)

25.	Discuss how the following algorithms are used for image processing applications:

a) Boundary extraction       b) Region filling

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	EMBEDDED SYSTEMS								      	     Time : 3 hours

Subject Code:	12EC254						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Embedded systems.

2.	If an 8 bit ADC has a reference voltage as 5 volt and the lower limit as 0 v, find the resolution of the ADC.

3.	CPU load equals to __________ implies that CPU is underutilized for 80% of its time.

4.	Differentiate Critical and non critical tasks.

5.	When all the tasks are blocked, the problem is called _____________(Blocked , Deadlock, Priority Inversion)

6.	How is Dynamic Page allocation done?

7.	What is the process of erasing and programming a PROM.?                                                           8.	What is the use of introducing breakpoints in a program?

9.	Differentiate Source Codes from Object Codes.

10.	Mention any 1 advantage of high level language compared to assembly language.

PART – B  (5 x 3 = 15 MARKS)

11.	Sketch the H/W block diagram of a washing machine.

12.	Mention any 3 properties of a task.

13.	When and why does priority inversion occur?

14.	Differentiate emulators and simulators.

15.	Write a C program to add 2 numbers.

PART – C  (5 x 15 = 75 MARKS)

16.	a.  Discuss on the types of processors or microcontrollers used in embedded systems.  	  (5)

b. Explain the function of embedded systems with a neat block diagram.				  (10)

(OR)

17.	a. 	Design an embedded system to monitor and control temperature in an industry. Draw the 	flow chart of the S/W involved.                  						                     		    (8)

b. 	With a neat diagram, explain how a ROM image is obtained from an assembly language 			program.																  	   (7)

18.	a.	Explain the function of a real-time system with a neat block diagram. 				  (10)                              b. 	Round Robin scheduling is not effective in a computer controlling an aircraft. Justify.    (5)

(OR)

19.	a.	Mention the job list of a fully automated washing machine.  Mention the multitasking jobs                                             																					    (8)

b. 	Mention any 3 performance measures of a mobile phone and justify the performance measures based on their properties. 											           (7)

20	a.	Discuss the different task states with a state diagram. 								    (7)

b.	Discuss any two memory management strategies.         				        	  	    (8)

[P.T.O.]

(OR)

21.	a. 	With neat block diagram and timing diagram,explain the use of semaphore for a single

task.    									        								(10)

b.  	Discuss how mutex can be used between various processes.                                     (5)

22.	a.    For the given program :

LOOP: MOV A, # FF

DEC A

JNZ     LOOP

HLT

Justify the output of the debugger.                                                     					    (5)

b. 	Explain with a neat block diagram the function of an IDE.    						  (10)

(OR)

23. 	a. 	What is a compiler: What is the need for a cross compiler?       					    (5)

b. 	With diagrams explain the functioning of InCircuit Emulator. 						  (10)

24.   a. 	Elaborate on the Include directive used in C with examples.		        			    (7)

b. 	Explain the function of queue and circular queue with diagrams.                			    (8)

(OR)

25   a. 	Write an embedded C program with a flowchart to generate a square waveform from any 			port of a microcontroller.  													         (10)

b. 	Explain the use of pointers and null pointers with examples.			   			    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	OPTO ELECTRONIC DEVICES					      	     Time : 3 hours

Subject Code:	12EC256						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is polarization?

2.	What is Brewster's law?

3.	What is photoluminescence?

4.	What is LASER?

5.	What is photo detector?

6.	How to estimate detector performance?

7.	What is a magneto optic device?

8.	What is meant by optical switching?

9.	Name any one application of opto electronic integrated circuits.

10.	What are guided wave devices?

PART – B  (5 x 3 = 15 MARKS)

11.	Define diffraction? Explain the importance of diffraction in optical electronics.

12.	Explain photo luminescence with respect to photo emission process.

13.	What are the requirements of a thermal detector?

14.	Define digital modulation.

15.	What are components of integrated transmitters?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Write short notes on wave nature of light. 										    (7)

b.	Give the relationship between electric and magnetic field in optical wave equation.	    (8)

(OR)

17.	Derive the expressions for concentrations of electrons and holes in an intrinsic semiconductor, with relevant diagrams.

18.	Explain with necessary diagrams the principle, construction and working of liquid crystal display.

(OR)

19.	Discuss the theory of population inversion and threshold condition in two level laser system. Also explain various modes of laser.

20.	In detail explain the construction and working principle of thermal detector with neat diagram. (OR)

21.	In detail explain the construction and working principle of photo conductors with neat diagram. 22.	What is Electro-Optic Effect? Explain how this is suitable for electro-optic phase modulation 	and electro-optic amplitude modulation.

(OR)

23.	Write short notes on the following:

a.	Tunable threshold logic gates. 													    (8)

b.	Optical crossbar switching.  													    (7)

24.	Explain in detail any two applications of opto electronic integrated circuits.

(OR)

25.	In detail explain about integrated transmitters and receivers with diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	NETWORK ANALYSIS AND SYNTHESIS			      	     Time : 3 hours

Subject Code:	EE295								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define zero of a network function.

2. 	Define transfer function.

3. 	Define immittance.

4. 	Give the importance of complex locus diagram.

5. 	Say true or false: Every tree has a co-tree.

6. 	Branches of tree are known as ___________.

7. 	Define Attenuator.

8. 	Unit of the attenuation constant is ___________.

9. 	In the second Cauer network, first element is a ___________ when the driving point

function consists of pole at zero.

10. 	Say true or false: All the quotients in the Hurwitz polynomials are positive.

PART – B  (5 x 3 = 15 MARKS)

11.	Give the properties of transfer function.

12.	Define phase plot.

13.	Define cut set.

14.	List the applications of attenuators.

15.	List the properties of positive real function.

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the parameters Z21(s) and G21(s) for the network given

(OR)

17.	For the network function, Vs=4s+2ss+1s+3.

Draw the pole zero diagram and hence obtain the time domain response v(t).

[P.T.O.]

18.	For a series RL circuit, (a)Write the expression of admittance function Y(s) in standard form.

(b)Show the phasor representation of

(c) Using the s-plane phasors obtained, sketch the complex locus of Y (jω)

(OR)

19.  Apply Routh criterion and determine the number of roots of the given polynomial (i) with

Positive real roots (ii) with zero real parts (iii) with negative real roots

Qs=s4+4s3+8s2+12s+15	.

20. 	Determine the fundamental loop matrix and cut-set matrix of the network given( each 1).

(OR)

21.	Determine the loop equations and branch currents using tie set schedule.

22.	Design a low pass filter (both T and π sections) having a zero frequency characteristic

impedance of 300 Ω and a cutoff frequency of 1 KHz.

(OR)

23. 	a.	Design and draw a π-type attenuator to give 40 dB attenuation and to have a characteristic 	impedance of 400 Ω.

b.	Design and draw a symmetrical lattice attenuator to have characteristic impedance of

600 Ω and attenuation of 30 dB.

24.   Find the first and second Cauer forms of the function, zs=2s2+8s+6s2+2s.

(OR)

25.   Find the Foster realizations of the function, 	zs=3s2+1s2+16ss2+9.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ELECTRONICS AND MICROPROCESSORS			     Time : 3 hours

Subject Code:	12EC260								 		      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Barkihausen Criteria.

2.	The dc current gain is the ratio of __________.

3.	Give 2 advantages of Multimeter.

4.	What is the purpose of Digital Voltmeter?

5.	What is a Register? What are the different types of Registers?

6.	Convert (356 )16 to Decimal.

7.	The accumulator register of 8085 is _________ bit in length.

8.	Give an example for two byte instruction.

9.	What is an Interrupt?

10.	Give an application for 8085 Microprocessor.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Ripple Factor.

12.	Write short notes on Potentiometer.

13.	Design a data selector with many inputs and single output line.

14. 	What are the different addressing modes in 8085 microprocessor?

15. 	Draw the Control Word format for Input /Output modes(I/O) in 8255.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	What is Rectifier?Explain the operation of Half Wave Rectifier with its input and output 			waveform. 	    														                   (8)

b.	With a neat circuit diagram explain the operation of Voltage regulator with an example. (7)

(OR)

17 . 	a. 	What is an Oscillator? Explain Wein Bridge Oscillator with a neat circuit diagram.         (8)

b. 	Explain Class B Push pull amplifier with a neat circuit diagram.       				    (7)

18 . a. 	Explain the working principle of Cathode Ray Oscilloscope(CRO).			 	          (10)

b. 	With a neat illustration, explain the working and the characteristics of a photodiode.	    (5)

(OR)

19. 	a.  	Draw and explain the block diagram of Signal Generator. 						           (5)

b. 	Define Strain gauges.Discuss various types of Strain gauges in detail.			          (10)

20 . 	a. 	Design a 4 to 2 Encoder with its truth table and logic diagram.  		                         (8)

b.  	Draw the schematic of 1 to 4 demultiplexer and explain with function table. 			    (7)

(OR)

21. 	a. 	Construct  a 3 bit binary Ripple Counter with its timing waveforms. 				   (8)

b.  	With the logic diagram verify the truth table of SR Flip flop & T Flip flop.			   (7)

[P.T.O.]

22.  Draw the architecture of 8085 micro processor and explain the functions of each unit.

(OR)

23.	a.	What is a Flag? Discuss various types of flags with its format.                 	             	    (8)	 b. 	Write an assembly language program to subtract two 8 bit numbers from memory location 		4500 and 4501 and store the result in 4502.  	   								            (7)

24. 	a. 	Differentiate between memory mapped I/O and I/O mapped I/O.				 	    (8)

b. 	What is Direct Memory Access(DMA)?Discuss various types of data transfer in direct

memory access?                                                   									    (7)

(OR)

25. 	Explain with a flow diagram ,temperature monitoring system.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	STATISTICAL DIGITAL SIGNAL PROCESSING	       Time : 3 hours

Subject Code: 	12EC301										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

1.	a. 	Show that the power spectrum of WSS random process can be factorized as

P(z) = σ0 2 Q(z)Q*(1/z*)													      (10)

b. 	Find a filter which when driven by unit variance white noise will yield a random process                		having a power spectrum   Pxejω=5+4cos2ω10+6cosω.							      (10)

2.	a.	Compare parametric and non-parametric estimators.							      (10)

b. 	Show how the inconsistency in periodogram technique  of the spectral estimation can be 	eliminated by modified periodogram method	.								      (10)

(OR)

3.	a. 	Derive the Yule-Walker equation for ARMA process.						 	      (10)

b.  	Explain Welch method of power spectrum estimation.						      (10)

4.	Discuss in detail the FIR Wiener filter.

(OR)

5. 	From basic principles develop LMS error criterion for a simple linear estimation problem.

6.  	Explain Newton’s steepest descent method.

(OR)

7.	a. 	Describe LMS adaptive algorithm.											      (10)

b. 	Explain the application of adaptive filtering in channel equalization.			      (10)

8. 	Describe in detail Decimation and Interpolation by integer factors.

(OR)

9.  	Explain about wavelet transforms and filter bank implementation of wavelet expansion of signals.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	CMOS VLSI DESIGN								       Time : 3 hours

Subject Code: 	12EC313								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a.	Explain the construction and working of a MOS transistor.			    		     (10)

b.	Derive the expression for drain current of a MOSFET in various regions of operation.					    				               									     (10)

(OR)

2.	Explain the working of a CMOS inverter. Derive the expressions for noise margins.

3.	a.	Explain the working of a dynamic CMOS logic. Design a two input XOR gate using 	dynamic CMOS logic.						          	    					      (10)

b.	What is the problem associated with cascading dynamic CMOS gates? Give a solution to 	overcome this problem.                                                          					      (10)

(OR)

4.	a.	Implement the function F=m(1,3,4,6)using static CMOS logic.        	      		(6)

b.	Implement the function F=m(1,3,4,6,7,11,14,15) using 8 X 1 multiplexer and pass 	transistor logic.	                                                                                 				(8)

c.	Explain in detail about Ratioed logic .					      						(6)

5.	a.	Explain the working of a C2MOS D-Latch. Justify that it is insensitive to clock overlaps.  									    											      (12)

b.	Explain the working of a Master-Slave edge triggered register.                    			(8)

(OR)

6.	a.	Explain the working of True Single Phase Clocked register.	   	    			      (12)  	b.	Discuss the working of a multiplexer based D-Latch.			      					(8)

7.	a.	Design a 4 X 4 barrel right shifter. Explain its working.			   			      (10)

b.	Draw and explain the working of a 4 X 4 Wallace Tree multiplier.               		      (10)

(OR)

8.	a.	Design a 6-Transistor SRAM cell and explain its working.				    		      (10)

b.	Design a CMOS mirror adder and list out its features.                                  		      (10)

9.	Compulsory :

a.	Explain in detail the various sources of power dissipation in CMOS logic circuits.     (10)

b.	Explain in detail about CMOS n-well design rules.				    			      (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination –  November/December - 2015

Subject Title: 	SEMICONDUCTOR DEVICES AND MODELING	       Time : 3 hours

Subject Code: 	12EC314										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

Derive the expression for the extrinsic transconductance degraded by an emitter resistance.

2. 	a. 	Derive the expression for Hall factor and Hall mobility.						      (10)

b.	Explain how diffusion takes place in PN diode and derive the expression constant of P 	type semiconductor.														      (10)

(OR)

3. 	a.	Discuss the diffusion capacitance and derive the expression for diffusion capacitance in 	a PN junction.															     (10)

b. 	Explain any one type of  heterojunction. 									     (10)

4.	a.	Write a note on Knee current and Kirk effect.									(8)

b. 	Show that Gummel number dominates in emitter band gap narrowing.			     (12)

(OR)

5.	Explain the following:

a.	High voltage limit.															(5)

b.	High current effects.															(5)

c.	Thermal runaway	.															(5)

d.	Second breakdown.															(5)

6. 	With the help of relevant equations obtain the relationship between the drain current of a

MOSFET and its terminal voltages. Also plot the drain current versus source voltage in triode

region.

(OR)

7.	a.	Explain in detail about the second order effects in MOS structure.				     (15)

b. 	Discuss the MOS device capacitance.											(5)

8. 	Discuss in detail about the level 2 model of MOSFET with relevant expressions.

(OR)

9.	a. 	Discuss the level 1 model of MOSFET  with the parameters.					      (10)

b. 	Write short notes on BSIM model.											      (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	VLSI DIGITAL SIGNAL PROCESSING				       Time : 3 hours

Subject Code: 	12EC317								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a.	State and prove the properties of Retiming.					      					(8)

b.	Draw the bit serial adder for a Word-length W=4. Unfold the same to obtain the digital 	serial and bit parallel adder architecture for unfolding factors of J= 2 and J=4 	respectively.					    				    							      (12)

(OR)

2.	Formulate a three parallel FIR filter using Poly-phase decomposition.

3.	Derive a 2 X 3 linear convolution s(p)=h(p)x(p); where h(p)=h0+h1pand x(p)=x0+x1p+x2p2using Cook-Toom algorithm.

(OR)

4.	Construct a 4 X 4 cyclic convolution algorithm using CRT with m(p)=p4−1=(p−1)(p+1)(p2+1).

5.	a.	Design a 4 X 4 Baugh-Wooley carry-save multiplier.		          	    			      (10)

b.	Explain Modified Booths algorithm.                                                              			(5)

c.	Write an algorithm to compute CSD format of a W-bit binary number.         			(5)

(OR)

6.	a.	Explain the design of Lyon’s Bit-Serial multiplier using Horner’s rule.         		      (10)

b.	Write short notes on round-off noise.						    				      (10)

7.	a.	How sub-expression elimination can be applied to Multiple Constant Multiplication?			   					    														(8)

b.	Draw and explain the structure of a 8-digit parallel hybrid radix-2 adder.  		      (12)

(OR)

8.	a.	Draw and explain the architecture of TSPC n-latch.			   				      (10)  	b.	Explain the working of a Static D-latch and Dynamic D-latch.		   			      (10)

9.	Compulsory :

a.	Explain loop bound and iteration bound with a suitable example. 	    	      			(8)

b.	Draw a four tap FIR filter and its 2-parallel version. Determine the critical path in the 2-parallel version. Assume that multiplication takes 8 units of time and addition takes 1 unit of time. For power calculation purposes, assume that the capacitance of the multiplier is 8 times that of the adder. The two architectures are operated at a sample period of 9 units of time. Assume the device threshold to be 0.45 V. Also assume the sequential filter to be operated at an initial supply voltage of 3.3 V. Calculate the supply voltage of the 2-parallel filter. Also compute the power consumption of a 2-parallel filter as a percentage of the original filter.						    				      (12)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	TESTING OF VLSI CIRCUITS						       Time : 3 hours

Subject Code: 	12EC318								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Explain in detail about Boundary Scan architecture.

(OR)

2.	a.	Explain the Adhoc Design rules for Testability.				    				      (12)

b.	Discuss in detail about the need for testing.					      					(8)

3.	a.	Find the test vector to detect the fault on the line ‘5'  s_a_0 using D algorithm for the 	circuit shown in figure.						    							      (10)

b.	Given F=x1.(x2+x3). Use Boolean Difference method to find all single struck at faults 	present in the circuit.						    								      (10)

(OR)

4.	Explain in detail about parallel and concurrent fault simulation.

5.	Explain in detail about test pattern generation for built in self test.

(OR)

6.	a.	Draw and explain the working of a built in logic block observer.     	    		            (10)

b.	Obtain the signature for the input sequence “11110101” using a SISR whose 	characteristic polynomial is given by P(x)=1+x2+x4+x5.        	    			      (10)

7.	a.	Explain the basic concepts of fault dictionary. Mention its advantages and drawbacks.  																			      (10)

b.	Explain the steps involved in Inject and Evaluate Paradigm.                      		      (10)

(OR)

8.	a.	Draw the flowchart for the fault diagnosis process by UUT reduction.	      			(8)  	b.	Write short notes on Residue codes.		    			               				      (12)

9.	Compulsory

a.	Explain in detail about fault equivalence and fault dominance with suitable examples.										    										      (10)

b.	Show that the two single struck at faults ‘c’ s_a_0 and ‘f’ s_a_1 shown the in figure are 		equivalent.							    									      (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	LOW POWER VLSI DESIGN						       Time : 3 hours

Subject Code: 	12EC319								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Write down the need for low power VLSI chips. Explain the various sources of power 		consumption in CMOS.													      (10)

b.	Using any one example, describe how low power design using gate level logic simulation is achieved.													      (10)

2.	a.	Apply network reconstruction and reorganization to reduce the power consumption.  (10)

b.	Define signal gating. Explain the various logic implementation of signal gating.	      (10)

(OR)

3.	a.	Design asynchronous circuit and reduce the power using clock network.		      (14)

b.	Explain the state machine encoding technique.									(6)

4.	a.	Explain the BDD statistical techniques for computing the signal probability.		      (10)

b.	Describe the glitching power estimation and show how the glitching power can be reduced in a circuit. 														      (10)

(OR)

5.	a.	Explain about the switching activity reduction in detail.						      (10)

b.	What are the significances of voltage reduction? Explain the parallel architecture with voltage reduction.														      (10)

6.	a.	Discuss in detail the deep sub micro meter device design issues.				      (10)

b.	Explain circuit techniques for reducing power consumption in multipliers.		      (10)

(OR)

7.	a.	Explain the common forms of SRAM cells design using MoS transistors. Justify the 	organization technique of SRAM that targets total switched capacitance to achieve 	reduced power.															      (12)

b.	Describe the power reduction in write drives.									(8)

8.	Explain the energy recovery with partially reversible logic. Use any one suitable example.

(OR)

9.	a.	What are the need for low power latches and FF.?								(6)

b.	Explain about the evaluation of latches.											(6)

c.	What are the quality measures for latches and flip flops? Explain. 					(8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	EMBEDDED SYSTEM DESIGN						      	     Time : 3 hours

Subject Code:	14EC2029						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define an embedded system.

2.	What are the types of embedded system?

3.	What is the use of TXD and RXD signal in RS232?

4.	Give two examples for embedded microcontrollers.

5.	Name any two important RTOS.

6.	Name the HDLC’s frame structure.

7.	What is watchdog timer?

8.	Name the important terms of RTOS.

9.	State the functionalities of emulator.

10.	List down some applications of semaphores.

PART – B  (5 x 3 = 15 MARKS)

11.	Define RS232 standards.

12.	What are all the challenges faced in embedded system design?

13.	What are the characteristics of an embedded system?

14.	What is shared data problem ?

15.	List the functions of a kernel.

PART – C  (5 x 15 = 75 MARKS)

16.	With a detailed diagram elucidate the embedded design life cycle with an example.

(OR)

17.	a.	Explain about the interfacing techniques for IO ports, buses.  						  (10)

b.	How to interface LEDs in microprocessor and microcontroller? 					    (5)

18.	Elucidate the working of stepper motor interfaced with 8051 microcontroller

(OR)

19.	Explain the working of RS232 with 8051 Microcontroller.       							  (10)

List the handshaking signals of RS232. 												    (5)

20.	a.	Explain briefly the feature of IDE.												    (8)

b.	Explain the issues in hardware /software design and co-design.					   	    (7)

(OR)

21.	Explain the Host system and software development tools used in developing, testing and debugging the embedded software in development process.

22.	a.	Explain how UART used for communication. List its advantages.					    (8)

b.	Explain about the HDLC protocol. Give the format for network device port bits in HDLC 	protocol.																		    (7)

(OR)

[P.T.O.]

23.	a.	Write short notes on usage of In-Circuit Emulators for debugging the design of Embedded 	Systems.																		    (5)

b.	Write a short notes on synchronous serial data communication using SCI,SPI and SI ports. 																				  (10)

24.	Explain in detail about the memory management system of multitasking RTOS.

(OR)

25.	a.	Write about the semaphores with types in detail.									    (8)	b.	Write in detail about shared data operating systems.								    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FUNDAMENTALS OF WIRELESS COMMUNICATION

Time : 3 hours

Subject Code:	14EC2044								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give a difference between cellular telephone system and cordless telephone system.

2.	What are underlay systems?

3.	What is the concept behind cellular systems?

4.	What is adjacent channel interference?

5.	What is shadowing?

6.	Which realistic model takes into account both the LOS path and the reflected path?

7.	Define coherence time.

8.	__________ is the expected rate at which a signal crosses a specified level in the positive going direction.

9.	What is intersymbol interference?

10.  State Shannon’s coding theorem.

PART – B  (5 x 3 = 15 MARKS)

11.	Write short notes on geosynchronous satellites.

12.	Briefly explain the second generation mobile assisted handoff technique.

13.	Explain the simplified path loss model.

14.	Find the average fade duration for threshold level ρ = 0.1, when the Doppler frequency is 200 Hz.

15.	What is meant by capacity with outage?

PART – C  (5 x 15 = 75 MARKS)

16.	Describe in detail the following wireless systems.

a.	Cellular telephone systems.                                                                              			  (11)

b.	Bluetooth and Zigbee. 					                                      				   (4)

(OR)

17.	a.	What are the technical issues in wireless systems?                                 		   (9)

b.	Discuss in detail the methods for spectrum allocation.		  		       				   (6)

18.	Elaborate on the techniques to improve the capacity and coverage of a cellular system.

(OR)

19.	a.	Consider a cellular system with hexagonal cells of radius, R= 1km.Suppose the minimum 	distance between cell centers using the same frequency must be D= 6km to maintain the 	required SIR. Find the required reuse factor (Q) and the number of cells per cluster (N). If 	the total number of channels for the system is 1200, find the number of channels that can 	be assigned to each cell.														    (5)

b.	If a signal to interference ratio of 15 dB is required for satisfactory forward channel performance of a cellular system, what is the frequency reuse factor and cluster size that should be used for maximum capacity if the path loss exponent is  a) n= 4, b) n=3? Assume that there are 6 co-channels cells in the first tier and all of them are at the same distance from the mobile. Use suitable approximations.       								  (10)

[P.T.O.]

20.	a.	If a transmitter produces 50W of power, express the transmit power in units of (i) dBm 	and (ii) dBW. If 50W is applied to a unity gain antenna with a 900MHz carrier frequency, 	find the received power in dBm at a free space distance of 100m from the antenna. What is 	Pr(10km)? Assume unity gain for the receiver antenna. 							  (10)

b.	Derive the outage probability under path loss and shadowing.                        			   (5)

(OR)

21.	a.	Describe in detail the two ray propagation model.                                                		  (12)

b.	Find the far-field distance for an antenna with maximum dimension of 1m and operating 	frequency of 900MHz.                                                                                    			   (3)

22.	Derive the time varying channel impulse response of statistical multipath channel model.

(OR)

23.	Discuss in detail on the power delay profile, coherence bandwidth and the Doppler power spectrum of wideband fading models.

24.	Discuss about the capacity of time-invariant and time varying, frequency selective fading channels.

(OR)

25.	Derive the Shannon capacity of flat fading channels.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	BASICS OF SATELLITE COMMUNICATION		      	     Time : 3 hours

Subject Code:	14EC2050								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The orbital point far from earth is called __________.

2.	List out the components of satellite payload.

3.	__________ is the event when the sun is at the Zenith over the equator.

4.	What is the primary source of satellite power subsystem?

5.	Radio frequency range is __________.

6.	Spatial separation of two successive oscillations of a wave is called as __________.

7.	__________ is the figure of merit of satellite transmitter

8.	1EHz = __________ Hz.

9.	The major source of noise at radio frequency is __________.

10.	The implementation complexity of CDMA is __________ (very low/high/infinite).

PART – B  (5 x 3 = 15 MARKS)

11.	List out any five possible satellite services available.

12.	Define Prograde and Retrograde orbits of satellite.

13.	List out any three basic transmission parameters of RF link.

14.	State inverse square law of radiation. What is its significance?

15.	Draw the noise equivalent model of active devices.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the working of on-board processing type satellite transponder.				    (8)

b.	State and explain Kepler’s laws of planetary motion.								    (7)

(OR)

17.	a.	With neat sketch explain the working of frequency translation transponder.			    (7)

b.	Explain the satellite Telemetry Tracking Control & Monitoring system.				    (8)

18.	a.	Brief the working of satellite attitude control subsystem.							    (5)

b.	Explain the physical structure of satellite bus.									  (10)

(OR)

19.	a.	Derive the expressions for range and elevation angle to satellite.					    (5)

b.	Explain the classification of satellite orbits in detail.								  (10)

20.	a.	Derive the complete expression for free space path loss.							  (10)

b.	Derive the expression for satellite antenna gain.									    (5)

(OR)

21.	a.	Tabulate the RF spectrum with its range and applications.							    (5)

b.	Obtain the satellite link power budget equation.									    (4)

c.	Derive the gain (in dBi) of circular parabolic reflector antenna.						    (6)

[P.T.O.]

22.	a.	With necessary mathematical expressions, explain system noise effect in satellite link.(8)

b.	Brief the INSAT project of India.												    (7)

(OR)

23.	a.	Explain about INTELSAT.													    (6)

b.	Explain FDMA technique in detail.												    (9)

24.	a.	Explain the concept of CDMA using neat sketch.									  (10)

b.	List out various propagation mechanisms of radio waves.							    (5)

(OR)

25.	a.	Explain DTH services in detail.												    (6)

b.	Explain the working principle of GPS neatly.									    (9)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FUNDAMENTALS OF ELECTRONICS				      	     Time : 3 hours

Subject Code:	14EC2090										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Electrons in the outermost orbit of an atom are called ___________.

2.	Write an application of BJT.

3.	How many pins are there in an op amp.

4.	Draw the symbol of MOSFET.

5.	(1001.1111)2 = (              )16

6.	What is the output of EXOR gate when both the inputs are high?

7.	___________ is a non volatile memory.

8.	___________ is less affected by noise.

9.	An ___________ satellite receives amplifies and retransmits the signal back towards earth

10.	What is the intermediate frequency in super heterodyne receiver?

PART – B  (5 x 3 = 15 MARKS)

11.	Define intrinsic standoff ratio in UJT and write its expression.

12.	What are the ideal properties of op amp?

13.	Define Logic gates and list out some logic gates.

14.	What is a noise? What are the sources of noise?

15.	What is the need of demodulation?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	With neat sketch explain the characteristics, construction and working principle of PN 	diode.																		    (6)

b.	Draw and explain the energy band structure of a conductor?						    (4)

c.	Explain in detail the formation of intrinsic semiconductor and its characteristics.		    (5)

(OR)

17.	a.	Discuss the Common emitter configuration of BJT and plot the dc characteristics.	  (10)

b.	Explain the internal architecture of op amp with neat block diagram.				    (5)

18.	a.	Reduce the expression (A+B) (A’+C) (B+C).									    (7)

b.	(875.12)10 = (  )8   																    (2)

c.	(1110101)2  = (  )10  															      (2)

d.	Draw the symbol and the truth table of AND gate.								    (4)

(OR)

19.	a.	Explain the differentiator circuit with a neat diagram. How will you overcome the 	problems of basic differentiator circuit?											  (12)

b.	What are the problems in basic integrator circuit?									    (3)

20.	a.	Define Semi conductor memory. Explain the types of memory in detail.				  (12)

b.	Explain virtual instrumentation in brief.											    (3)

(OR)

[P.T.O.]

21.	Explain the architecture of 8085 micro processor in detail with neat diagram

22.	a.	Define amplitude modulation. Derive the modulated signal of amplitude modulation.	  (10)

b.	What is the need for modulation? Explain in detail.								    (5)

(OR)

23.	a.	Explain the working of LVDT in detail with neat diagrams.						  (10)

b.	Briefly discuss about the principle of aerial system.								    (5)

24.	a.	Explain the principle of super heterodyne receiver with neat block diagram.			  (10)

b.	List out the advantages of satellite communication.								    (5)

(OR)

25.	a.	Explain the principle of optical fiber in detail.									  (10)

b.	List out the applications of MEMS and Nano electronics.							    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	WIRELESS COMMUNICATION NETWORKS		       Time : 3 hours

Subject Code: 	14EC3013										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Describe the different types of small scale fading and any two methods of small scale 	multipath measurements.                                                                                    	      (15)

b.	The mean excess delay for a multipath channel is 4.38 µs and the rms delay spread is 1.37 µs. Determine whether an equalizer is required if this channel is used for GSM service.    																	(5)

2.	a.	Explain the Okumura and Hata propagation models.                      				      (12)

b.	Explain the indoor propagation models and bring out the difference between the indoor and outdoor models.                                                                                                		(8)

(OR)

3.	a.	Explain the multipath propagation model for a wireless channel. Bring out the 	differences between free space and multipath propagation.                     			      (15)

b.	A transmitter power of 50 W is applied to a unity gain antenna. Find the received power at a free space distance of 100 m from the transmitting antenna.  The operating frequency is 900 MHz.  Assume unity gain for the receiver antenna.                                              (5)

4.	a.	Explain the principle, advantages and applications of MIMO communication in detail.

(15)

b.	Compare diversity and MIMO techniques with respect to improving wireless system performance.                                                                                                       		(5)

(OR)

5.	a.	Explain the channel capacity for MIMO channels considering both static and fading 	conditions.                                                                                                             	      (10)

b.	Explain the concept of spatial multiplexing and blast architectures.              		      (10)

6.	Discuss the random access techniques used in wireless systems and discuss the needs for scheduling and power control in these systems.

(OR)

7.	a.	Explain the principle of UMTS and discuss the UMTS air interface.  			      (12)

b.	Compare SDMA with other multiple access techniques.                             			(8)

8.	a.	Draw and explain the 3GPP network architecture.  							      (15)

b.	Compare the different generations of wireless communication. 					(5)

(OR)

9.	a.	Draw and explain the IMS architecture.                							      (12)

b.	Briefly compare the different standards of WLANs.    							(8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	ADVANCED RADIATION SYSTEMS				       Time : 3 hours

Subject Code: 	14EC3014								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

- The radial component of the radiated power density of an infinitesimal linear dipole of length l <<λ is given by

Wav=arWr=arAsin2θr2wm2

Where A0 is the peak value of the power density, θ is the usual spherical coordinate, and ˆar is   the radial unit vector. Determine the maximum directivity of the antenna and express the directivity as a function of the directional angles θ and φ.			      (10)

b.	Verify that the radiation resistance of a 3 λ /2dipole with sinusoidal current distribution is equal to 105.3 ohms.						                  					      (10)

2.	a.	Explain the geometries of BSA and EFA with neat diagrams. Derive expressions for 3dB   	beamwidth of Broad side and end fire patterns.					       			      (10)

b. 	An YAGI- UDA array of linear elements is excited at 200 MHz with a regular resonant

HWD as the feed element in the array. The Zin is approximately 70 ohms. A twin- lead 300 ohm feed line is used for signal connection to TV.

i. 	Find the smallest Zin of the array for VSWR = 1.2

ii. 	State the best method to modify the present feed to meet the desired VSWR

specification.									       						      (10)

(OR)

3.	a.	Draw the structure of LPDA for VHF and UHF ranges. Explain its radiation mechanism.

Derive its design equations.    												      (10)

b. 	Design an LPDA which operates from 400 to 800 MHz with 10dBi gain. This antenna

should be matched to a 50 ohm cable of length 5m, which is left open. The length to

diameter ratio is 25. (Assume suitable data).					         			      (10)

4.	a.	Show the application of Babinet’s principle to short dipole antennas. Explain it in 	detail.																	      (10)

b.	Derive the far electric field component of an aperture antenna using Fourier transform

technique.Hence compute the far electric field for a circular aperture of known 	dimensions.																      (10)

(OR)

5.	a. 	A transmitting antenna and a receiving antenna are separated by a distance   of 103m. If 	the transmitting antenna radiates a power of 100W, calculate the available power at the 	receiving antenna if the D of transmitting antenna is 1.64 and effective area of receiving 	antenna is 0.25 m2. 											   			      (10)

b.	Design a horn antenna to give a HPBW of 35 deg. in both the E and H planes at a 		frequency of 8GHz. The horn is to be mounted on an X band waveguide. Assume the 	flare angle to be 20deg.  Calculate the gain of the horn.						      (10)

[P.T.O.]

6.	a.	Explain the cessegrain feed geometry of reflector antennas. Bring out its design details.

(10)

b. 	Discuss the various feed mechanisms for parabolic reflector with diagrams.		      (10)

(OR)

7.	a.	Differentiate  1.E plane sectoral horn from H plane sectoral horn.

2.  Horn antenna 	from microstripantenna.						      (10)

b.	Give an account on  “ CAD tools for Antenna analysis”.			                	      (10)

8.	a.	Explain the radiation mechanism of a rectangular patch antenna with a neat diagram.(10)

b.	Enumerate the choice of substrate materials for microstrip antennas.		     	      (10)

(OR)

9.	a.	Explain in detail the characteristics and potential applications of dielectric antennas.

(10)

b.	 Design a rectangular microstrip patch with dimensions W and L over a single substrate,

whose center  frequency is 10 GHz. The dielectric constant of the substrate is 10.2 and the height of the substrate is 0.127 cm. Determine the physical dimensions W and L of the patch taking into account fringing fields. 									      (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	VLSI DIGITAL SIGNAL PROCESSING				       Time : 3 hours

Subject Code: 	14EC3027								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Briefly explain the pipelining of FIR Digital filter.							      (10)

b. 	In the signal flow graph shown,the computation time for each node is assumed to be 1 u.t.

Calculate the critical path computation time.									      (10)

The critical path has been reduced to 2 u.t by inserting 3 extra delay lines shown in figure(b).Is this a valid pipelining? If not,obtain an appropriate pipelined circuit in figure (c)with critical path if 2 u.t

Fig(a)      				     Fig(b)			          Fig(c)

2. 	What is a cutset retiming and pipelining? How this can be obtained? Explain with an example.

(OR)

3. 	a.	Discuss discrete cosine transform and its inverse.									 (5)

b.	Construct 8 point fast DCT architecture using 2 point IDCT butterfly architecture.     (15)

4. 	a.	Write Cook Toom algorithm.													 (5)

b.	Construct a 2 X 2 convolution algorithm using Cook-Toom algorithm with β=0, +1.  (15)

(OR)

5.	Express cyclic convolution and construct a 4 X 4 cyclic convolution algorithm using CRT

with m(p)=p4–1=(p–1)(p+1)(p2+1)

6. 	Discuss scaling operation and roundoff noise in detail.

(OR)

7. 	a.	Explain the architecture for computing inner product of two length N vectors using

distributed arithmetic with ROM decomposition.								      (15)

b.	Write short notes on redundant number representation.							(5)

8. 	a.	What is hybrid radix 2 addition? Explain how it is carried out.					      (10) b.	Explain how redundant number is converted to non-redundant number with example.(10)

(OR)

9. 	a.	Describe the process involved in additive and multiplicative number splitting.	      (10)

b.	Briefly explain the steps involved in row based additive number splitting. Apply this to 	the following linear circuit.

Y0n+1Y1n+1Y2n+1=. 4 . 7 . 4 . 8. 3 . 6 . 2 . 9. 5 .3 . . 7y0ny1ny2nXn

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	BIO SIGNAL PROCESSING						       Time : 3 hours

Subject Code: 	14EC3030								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a.	Define the following terms:

i.	Deterministic signal        	ii.	correlation

iii.	ergodicity                      	iv.	stationarity                                       	     		      (12)

b.	What is system?  What are the types of signals? 									(8)

(OR)

2.  	a.	Write a note on spectral estimation in biomedical signals. 							(5)

b.	Explain two types filtering techniques for biomedical signals. 					      (15)

3.  	a.	Explain how time frequency analysis is helpful in biomedical signal processing. 	      (10)

b.	What is heart rate variability? What noises are encountered in it? Discuss.		      (10)

(OR)

4. 	Design an optimal filter to remove noise from a signal, given that the signal and noise Processes are independent, stationary, random processes.

5. 	a.	List out a few physiological interferences that may be encountered while acquiring a

signal of   interest.									 					      (10)

b.	Enumerate the origin of bio potentials with example.           			                    (10)

(OR)

6.	Propose an algorithm to detect QRS complexes in an ongoing ECG signal.

7. 	Discuss about:

a.	Wavelets.

b.	Multivariate Analysis.

(OR)

8.  	a.	Mention the types of artifacts interfering with the EEG acquisition and the method by 	which they are processed.    								 				      (12)

b.	Write a note on Joint Time-frequency analysis of biomedical signals.                        	(8)

9.	Compulsory :

Discuss in detail how the Back propagation neural net is used for Multivariate analysis?

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	SOLID STATE CIRCUITS-I							      	     Time : 3 hours

Subject Code:	EC205											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The ripple factor of a half-wave rectifier is ________.

(a) 2 		(b) 1.21 			(c) 2.5			 (d) 0.48

2. 	The ________ filter circuit results in the best voltage regulation

(a) choke input (b) Capacitor input (c) resistance input (d) none of the above

3. 	What is the function of Q-point?

4. 	Draw the collector to base bias circuit.

5. 	The coupling method which does impedance matching is________.

6. 	Class D amplifiers are mainly used in ________.

7. 	Negative feedback is employed in________.

8. 	Give the output voltage VO of Common Mode Rejection Ratio.

9. 	A tuned amplifier uses________ load.

10. 	State Barkhausen criterion for oscillation.

PART – B  (5 x 3 = 15 MARKS)

11. 	Define Half-Wave rectifier.

12. 	State the relation between region and biasing of transistor junctions.

13. 	Distinguish between Class A and Class C amplifiers.

14. 	How does negative feedback reduce distortion in an amplifier?

15. 	In a Wien-bridge oscillator, if the value of R is 100kΩ and frequency of oscillation is 10kHz, find the value of capacitor C.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Derive an expression for a ripple factor in a full-wave rectifier.                                         (7)

b.	With neat sketch, explain the working principle of full-wave rectifier.                              (8)

(OR)

17. 	Explain in detail about Voltage and Current regulators.

18. 	Describe in detail about different types of biasing circuits in FET.

(OR)

19. 	a.	Design a Voltage divider bias circuit for the specified conditions. VCC =12V, VCE = 6V,

IC =1mA, S=20, β=100 and VE=1V                                                                                    (8)

b.	Draw the circuit of CE amplifier with Voltage divider bias. Derive the expression for VB

and VCE.                                                                                                                               (7)

20. 	Give a brief account on Transformer coupled Class A amplifier

(OR)

21. 	Draw the circuit diagram of Push pull and complementary symmetry amplifiers and explain

its principle of operation.

[P.T.O.]

22. 	a.	Describe with necessary derivations, the effect of negative feedback on the bandwidth and 	distortion in an amplifier.                                                                                                    (7)

b.	Write short note on basic concept of feedback.                                                                   (8)

(OR)

23. Draw the circuit diagrams of various configurations of differential amplifier.

24. Draw the circuit diagram of RC phase shift oscillators and briefly explain its operation.

(OR)

25. 	Draw the equivalent circuit of single tuned amplifier and derive the equation for 3dB 	bandwidth 	of capacitance coupled single tuned amplifier.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	SOLID STATE CIRCUITS – II						      	     Time : 3 hours

Subject Code:	EC206											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Why high-pass RC circuit is called Differentiator?

2. 	__________circuit is designed to shift the input waveform either above or below the DC

reference level without altering the waveform shape.

3. 	What are the different names of bistable Multivibrator?

4. 	What is Schmitt trigger?

5.	Why is monostable Multivibrator called gating circuit?

6. 	Mention the applications of one short multivibrator.

7. 	What do you mean by voltage time base generators?

8. 	Define restoration time or flyback time.

9. 	The two important elements of Blocking Oscillator are __________.

10. 	The other name of astable blocking Oscillator is __________.

PART – B  (5 x 3 = 15 MARKS)

11. 	What is Low pass RL circuit? Why it is called low-pass filter?

12. 	How a Schmitt trigger is different from a multivibrator?

13.	What is astable multivibrator? List the applications of Astable multivibrator.

14. 	Define transmission error and displacement error.

15.	What is bidirectional sampling gate?

PART – C  (5 x 15 = 75 MARKS)

16.	Obtain the response of a low pass RC circuit to step and pulse inputs.

(OR)

17. 	What is clipper? Explain any four types of clippers. Also mention its applications.

18. 	Describe the working of a fixed – bias transistor bistable multivibrator with a neat circuit diagram.

(OR)

19. 	With the aid of necessary circuit diagram, explain the working of a schmitt trigger. Sketch its transfer characteristics. If a periodic triangular input is applied to this circuit, sketch the output wave form.

20. 	Using neat circuit and waveforms explain the working and derive the expression for the gate

width of a collector coupled monostable multivibrator.

(OR)

21. 	Describe with suitable waveforms and working of an emitter coupled astable multivibrator with merits and demerits of emitter coupled astable multivibrator.

22. 	Explain the operation of transistorized Bootstrap sweep generator and transistorized Miller sweep generator.

(OR)

[P.T.O.]

23.	Explain current-time base generator working principle and derive sweep speed error expression.

24. 	With circuit diagram and waveforms, explain the operation of a RC controlled astable transistor blocking oscillator.

(OR)

25.	 Explain the working of base timing transistor blocking oscillator and draw its waveform.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	LINEAR INTEGRATED CIRCUITS AND APPLICATIONS													      	     								     Time : 3 hours

Subject Code:	EC211								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List the type of IC packaging available.

2.	Classify IC’s based on their chip size.

3.	Define slew rate.

4.	Draw the divider circuit.

5.	Draw the waveform of non inverting comparator with positive reference voltage.

6.	Mention the principle of SMPS.

7.	List out the commonly used filters.

8.	Name the modes of operation of a timer.

9.	Define lock in range.

10.	Classify DAC’s based on their output.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the Schottky transistor and list out the valiant features of the transistor.

12.	List out the charactersistics of ideal OPAMP.

13.	Why do switching regulators have better efficiency than series regulators?

14.	Why do we use higher order filters?

15.	Define conversion time of ADC.

PART – C  (5 x 15 = 75 MARKS)

16.	List out the planar process involved in the fabrication of IC.

(OR)

17.	With necessary diagrams explain the fabrications of MOSFET.

18.	Design suitable circuit to produce large bandwidth and low closed loop gain. Suggest some suitable compensation techniques for the circuit designed.

(OR)

19.	a.	Derive the output voltage equation for log amplifier with saturation current and 	temperature compensation. 													    (8)

b.	Define CMRR and point out how it important in determining the sensitivity of an opamp.																						    (4)

20.	Explain the working principle of RC phase shift sine wave generator using op-amp and derive the expression for ‘f’.

(OR)

21.	Draw and explain the fundamental block diagram of a 723 voltage regulator and how this IC can be used as High voltage regulator.

22.	a.	Design a fourth order Butterworth low pass filter having upper cut off frequency 2kHZ. 																						(10)

b.	Derive the output frequency operation of VCO.									(5)

(OR)									[P.T.O.]

23.	a.	Design a 1 kHz square waveform using 555 for duty cycle D = 0.6 and D = 0.35.		(10)

b.	How would you realize a resistor using single capacitor in a switched capacitor network.(5)

24.	List out various phase detectors employed in PLL and derive the necessary equation.

(OR)

25.	With necessary diagrams explain direct type ADC’s in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ELECTROMAGNETIC FIELDS						      	     Time : 3 hours

Subject Code:	EC233							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Curl of a vector.

2.	Formulae for Electric flux density is __________________

3.	Write the expression for Ampere’s force law.

4.	Formulae for permeability is __________________

5.	Write the expression for Polarization.

6.	Write the equation for point form of ohm’s law.

7.	What do you mean by magnetic dipole?

8.	Draw the hysteresis curve.

9.	Give the relation between self and mutual inductance.

10.	Define Poynting vector.

PART – B  (5 x 3 = 15 MARKS)

11.	Check whether the vector F=xyz(ax+ay+az) solenoidal or not.

12.	Define the terms:

a.	Magnetomotive force.

b.	Magnetic field intensity.

13.	Calculate the energy density of the electrostatic field of 8 volts/metre in a medium of relative permittivity 4.

14.	Define the terms:

a.	Reluctance.

b.	Permeance.

15.	Write the modified Ampere’s law, Maxwell’s equation in integral and differential form derived from it.

PART – C  (5 x 15 = 75 MARKS)

16.	State and prove Stoke’s and Divergence theorem.

(OR)

17. 	Explain the different applications of Gauss’s law.

18.	Explain the different applications of Ampere’s law.

(OR)

19. 	Explain about vector magnetic potential.

20. 	Derive the expression for the capacitance of a co-axial cable with two dielectric material.

(OR)

21. 	With E1=2 ax−3 ay+5 az V/m at the charge free dielectric interface, z=0 (x – y plane).  Find E2, D1, D2 and the angle of incident and refraction, with εr1=2  and εr2=5.

[P.T.O.]

22. 	a.	State and derive the boundary relations for Magnetic fields.						    (9)

b.	Derive the expression for inductance of Solenoids.								    (6)

(OR)

23. 	a.	A solenoid of length d=30cm, N=3000 & I=40 mA & a radius of 1.2 cm is available. 	Assume =o. Find B & H within the solenoid and total energy.  					    (6)

b.	Derive the expression for the inductance between a pair of transmission lines.		    (9)

24. 	a.	Explain about the Maxwell’s equation derived from Faraday’s law.      				    (7)

b.	With the necessary equations, explain the three ways of generating induced emf. 	    (8)

(OR)

25. 	a.	Write a detailed note on comparison between field theory and circuit theory.		          (10)

b.	Write a short note on circuit application of poynting vector.					    	    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MICROPROCESSORS & APPLICATIONS			      	     Time : 3 hours

Subject Code:	EC238								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	8085 is a __________bit processor.

2.	MVI A,30 is an example for __________ addressing mode instruction.

3.	8086 has __________ bit address lines to access memory.

4.	List  the functional units of 8086.

5.	Function of 8255 IC is __________.

6.	Direct data transfer between an IO device and memory is called __________.

7.	Mention the number of 16 bit counters available in the timer 8253.

8.	8275 IC is a __________ controller IC.

9.	DAC will accept a __________ and convert it in to analog voltage or current.

10.	8279 requires the internal clock frequency of __________ Hz.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the use of an Accumulator?

12.	Compare minimum and maximum mode of operation of 8086.

13.	Discuss the features of 8237.

14.	What is the significance of programmable interval timer?

15.	What are the applications of temperature controller?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the architecture of 8085 microprocessor with neat diagram.

(OR)

17.	What are the different types of instructions used in 8085? Explain with examples.

18.  	Explain the architecture of 8086 with neat sketch.

(OR)

19.  	a.	Explain various addressing modes of  8086.	                                                                   (10)

b.	Write an assembly language program for 16 bit addition  using 8086.	                  	    (5)

20. 	Explain the functional block diagram of 8255 PPI.

(OR)

21. 	How the serial communication takes place  between a microprocessor and  peripherals    	using 8251?

22. 	Describe the functional block diagram of programmable interrupt controller 8259.

(OR)

23. 	Explain the different operating modes of timer 8253 with suitable diagram.

24. 	With the help of interface diagram explain the operation of keyboard display controller IC	 8279.

(OR)

25. Explain the operation of traffic light controller using microprocessor 8085 with neat diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	COMMUNICATION THEORY & SYSTEMS		      	     Time : 3 hours

Subject Code:	EC242								 			      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define modulation.

2.     Write the other name for baseband signal.

3.	What is the bandwidth for narrow band FM?

4.	Write the maximum permitted frequency deviation for FM.

5.	TRF is __________.

6.	What is meant by Double superhetrodyne receiver?

7.	List the performance parameter of receivers.

8.	Define fidelity of the radio receiver.

9.	Define noise.

10.	Define figure of merit.

PART – B  (5 x 3 = 15 MARKS)

11.	 A certain AM wave is observed on an oscilloscope, & it’s maximum & minimum amplitudes are measured as 6.5V & 1.5V respectively. Calculate its Depth of Modulation.

12.	Compare PM & FM.

13.	Draw the block diagram for low level modulator.

14.	A transmitter radiates 9kW without modulation and 10.125 kW after modulation. Determine depth of modulation.

15.	What is thermal noise?

PART – C  (5 x 15 = 75 MARKS)

16. 	Write the need for modulation and Draw the basic block diagram of communication systems and explain the components.

(OR)

17. 	Discuss the various types of modulation techniques.

18. 	Derive the mathematical analysis for FM signal and obtain the spectrum of FM wave.

(OR)

19. 	Derive the power relations in AM.

20. 	Explain the Balance modulator method of generating suppressed carrier AM signal.

(OR)

21. 	With neat diagram explain Foster seeley discriminator.

22. 	With neat diagram explain the function of super heterodyne receiver.

(OR)

23. 	Draw the block diagram of Armstrong FM transmitter system and explain its functions.

24. 	Derive expression for signal to noise ratio for DSB-SC AM system.

(OR)

25. 	Explain about the types of noise.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MICROWAVE & OPTICAL COMMUNICATION ENGG.													      	     								     Time : 3 hours

Subject Code:	EC244								 			      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	TM modes in a circular guide is characterized by____________.

2.	The other name for magic TEE is ____________.

3.	The efficiency of two cavity Klystron is ____________.

4.	Crossed field tubes derive their name from the fact that

a.	DC electric field & DC magnetic field are parallel to each other.

b.	DC electric field & DC magnetic field are perpendicular to each other.

c.	AC electric field & AC magnetic field are parallel to each other.

d.	AC electric field & AC magnetic field are perpendicular to each other.

5.	The abbreviation LSA in LSA diodes stands for

a.	Laser Stimulated Accumulation.

b.	Limited Space Charge Accumulation.

c.	Liquid Space Charge Argon.

d.	Limited Space Charge Assimilator.

6.	The operating frequency of TRAPATT devices are

a.	4MHz - 1GHz.

b.	40 KHz – 12MHz .

c.	400 MHz - 12GHz.

d.	40 MHz - 12GHz.

7.	Which one of the following is not a reason for losses in optical fibers __________.

a.	Impurities.

b.	Microbending.

c.	Attenuation.

d.	Stepped index operation.

8.	If the core/cladding dimensions of an optical fiber are given by 50/125µm, the fiber type would be _____________.

a.	Single mode step index.

b.	Single mode graded index.

c.	Multimode step index.

d.	Multimode graded index.

9.	Which of the following photo detector has the highest multiplication factor __________ .

a.	P-N diode.

b.	Avalanche photodiode.

c.	PIN diode.

d.	Photo transistor.

10.	The term responsivity as it applies to a light detector is best described as

a.	The time required for the signal to go from 10% to 90% of maximum amplitude.

b.	The ratio of diode output current to optical input power.

c.	The ratio of input power to output power.

d.	The ratio of output current to input current.

PART – B  (5 x 3 = 15 MARKS)

11.	Give three differences between isolator and circulator.

12.	What is the effect of transit time?

13.	Define Gunn Effect.

14.	Write the expression for the refractive index in graded index fibers.

15.	What is current mode of operation of photodiode?

PART – C  (5 x 15 = 75 MARKS)

16.	For a rectangular waveguide, show that the propagation modes depend on its physical dimensions.

(OR)

17.	a.	Derive and explain the significance of S Matrix of a Directional Coupler.                (8)

b.	Draw the structure of Hybrid Ring.  Obtain its S-matrix. Why it is also called Rat-Race 	circuit?                                                                                                                   		    (7)

18.	a.	Derive an expression for beam-coupling coefficient of the input cavity gap in Klystron and 	also draw the profile of beam coupling coefficient versus gap transit angle.        	        (10)

b.	Explain the Amplification process in helix type travelling-wave tube.                 	         (5)

(OR)

19.	a.	What is a magnetron Oscillator? What are its various types?  			       			    (5)

b.	An X band pulsed magnetron has the following operating parameters.

Anode voltage (V) = 26 kV

Beam current I0 = 27 A

Magnetic flux density B0= 0.336 Wb/m2

Radius of cathode cylinder a = 5 cm

Radius of vane edge to center b = 10 cm

Compute:  	(i) Cyclotron Angular frequency

(ii) Cut off voltage for a fixed B0

(iii) Cut off magnetic flux density for a fixed V0  			        	        (10)

20.	a.	Explain the principle of operation of a tunnel diode. Draw its I-V characteristics. 	  (10)

b.	Explain two valley model theory of Gunn diodes. 				       				    (5)

(OR)

21.	Explain with block diagram the measurement of low and high VSWR.

22.	a.	What is dispersion? How does it play an important role in determining information 	capacity of a waveguide? Classify the different types of dispersions. 		       		    (8)

b.	Discuss the design-optimization characteristics of single-mode fibers with suitable 				diagrams. 										       							    (7)

(OR)

23.	Derive the expression for Numerical aperture in Step index fiber and graded index fiber.

24.	Explain with suitable diagrams the construction and working principle of LASER diodes

(OR)

25.	Explain with suitable diagrams the construction and working principle of pin and Avalanche photo detectors.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DIGITAL SIGNAL PROCESSING					      	     Time : 3 hours

Subject Code:	EC245								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	When the system is said to be linear?

2.	Write down the condition to check for stability of digital filters.

3.	What form of structure, the FIR Filter is implemented?

4.	Define windowing process.

5.	Elliptic filters is sometimes called __________ filter.

6.	Write down the mapping formulae in bilinear transformation.

7.	Which process in finite word length effect degrades the system performance?

8.	What is called sampling?

9.	Explain the role of BNZ instruction in TMS processor.

10.	What the letter C in TMS320C5416 represents?

PART – B  (5 x 3 = 15 MARKS)

11.	Determine the DFT of the sequencexn=1/5,for n varies from −1 to +10,otherwise.

12.	List the advantages of FIR filters over IIR Filters.

13.	Using the impulse invariant method. Convert H(Z) to H(S) where H(S)=2/(s+1)(s+3) and T=0.1S.

14.	Illustrate the issues connected with finite word length effects.

15.	Define pipelining in TMS Processor.

PART – C  (5 x 15 = 75 MARKS)

16.	For the given sequence x(n)={0,1,2,3,4,5,6,7} , Find DFT using DT-FFT algorithms. Show the flow graph for all the stages clearly.

(OR)

17.	An FIR Digital filter has the unit impulse response sequence h(n)=[2,2,1]. Find the output sequence in response to the input sequence x(n)=[3,0,-2,0,2,1,0,-2,-1,0] using overlap-add and overlap save convolution method.

18.	The desired response of a low pass filter is Hdejw=e−j3w−3π4<=w<=3π40,3π4<w<=π.

Determine the value H(ejw) for N=7 using Hamming window.

(OR)

[P.T.O.]

19.	A Low pass filter has the desired response as given below Hdejw=e−j3w0<=w<π20, π2<=w<=π

Determine the filter co-efficient h(n) for M=7 , using type-I frequency sampling technique.

20.	Design a digital Butterworth filter that satisfies the following constraint using bilinear    transformation .Assume T=1s.

0.9 <= |H(ejw)}<=1         	;    	0<=w<=∏/2

|H(ejw)|<=0.2      	;  	3∏/4 <=w<=∏

(OR)

21.	Draw the structure of cascade and parallel realizations of H(z)=(1–z-1)3/ (1–0.5z-1)(1–1/8z-1) .

22.	A Cascaded realization of the two first order digital filters is shown below. The system functions of the individual sections are H1(z)= z/(1–0.9z-1)  and H2(z)= 1/(1–0.8z-1).Draw the product quantization noise model of the system and determine the overall output noise power.

(OR)

23.	a. 	Determine the deadband of the system for the system with difference equation

y(n)=0.9y(n–1)+x(n). with x(n)=0 and initial condition y(–1)= 12. 					  (10)

b. 	Why rounding is preferred to truncation in realizing digital filter?					    (5)

24.	a. 	Explain in detail about wiener filter.											    (8)

b. 	Write short notes on LMS algorithm.											    (7)

(OR)

25.	a. 	Explain the architecture of DSP Processor TMS320C5X.	                                               (7)

b. 	Write any two simple programs of TMS320C5X.				           				    (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DIGITAL COMMUNICATION						      	     Time : 3 hours

Subject Code:	EC249											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Mathematically Sampling Theorem is written as __________.

2. 	What is Pulse Amplitude Modulation?

3. 	List any two digital modulation schemes?

4. 	Draw the constellation diagram for Phasor diagram for output of BPSK method.

5. 	When the carrier signal is used in receiver for signal demodulation then the detection scheme is called as __________..

6. 	Mention one application of Frequency shift Keying (FSK) Method.

7. 	Define Entropy.

8. 	What are cyclic codes?

9. 	What is the need for ‘Pseudo random sequence’?

10. 	Define ‘processing gain’ of spread spectrum.

PART – B  (5 x 3 = 15 MARKS)

11. 	What is aliasing? How it is eliminated?

12. 	Sketch the binary output waveform for BPSK and FSK for the input data ‘1100101’.

13. 	Write a short note on Differential PSK.

14.	Write a short note on convolution codes.

15. 	Compare slow and fast frequency hop spread spectrum system.

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain the fundamental block diagram of Pulse Code modulation System (PCM).

(OR)

17.	a.	With neat diagram explain the functional principle of time division multiplexing.      (10)

b.	What is Inter symbol Interference (ISI)? How it is eliminated?						    (5)

18. 	With neat functional block diagram explain the operation of BPSK transmitter and receiver also deduce an expression for the probability of symbol error.

(OR)

19. 	With neat functional block diagram explain the data transmission and detection in QPSK scheme.

20.	a.	Show that a match filter maximizes output Signal to Noise Ratio (SNR).			  (10)

b.	List down the properties of Match Filter											    (5)

(OR)

21. 	Explain the operation of FSK transmitter and receiver. Also deduce an expression for the Probability of symbol error.

22.	With suitable example discuss Viterbi Algorithm in detail.

(OR)

23.	Explain any four important classes of cyclic code in detail.

[P.T.O.]

24. 	With neat functional block diagram explain the operation of Direct Sequence Spread               (DSSS) System. Also sketch relevant waveforms and spectrums.

(OR)

25. 	With neat functional block diagram explain the operation of Frequency Hop Spread Spectrum (FHSS) system. Also sketch relevant waveforms and spectrums.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DIGITAL SYSTEM DESIGN USING VHDL			      	     Time : 3 hours

Subject Code:	EC250								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is PLE?

2. 	Which type of PLD should be used to program basic logic functions?

3. 	Resistor–transistor logic (RTL) is a class of digital circuits built using resistors as the input network and _________  as switching devices.

4. 	Give the significance of advanced Boolean expression language.

5. 	What is LCA?

6. 	PIP is ___________.

7. 	Define entity.

8. 	What are the two main parts of a design entity?

9. 	What is defined within a port list?

10.	 How are statements evaluated within a process?

PART – B  (5 x 3 = 15 MARKS)

11. 	List the design steps using ASM chart.

12. 	What is ROM simulation?

13. 	Give the significance of  configurable logic block.

14. 	What are the data operators?

15. 	Write the VHDL coding for a sequential statement (d flip flop).

PART – C  (5 x 15 = 75 MARKS)

16. 	Discuss on the concept of working and applications of following memories.

a. 	ROM

b. 	EPROM

c. 	PLA

(OR)

17. 	A combinational circuit is defined by the functions.

a. 	F1 (a, b, c) = ∑ m (3, 5, 6, 7).

b. 	F2 (a, b, c) = ∑ m (0, 2, 4, 7) implement the circuit with a PLA.

18. 	Explain the designing of micro programmed controller.

(OR)

19. 	Write short notes on

a. 	ROM simulation. 												    			    (7)

b. 	Bit sliced computers in detail.						    							    (8)

20. 	Explain the logic cell array and Input and output block of Xilinx 3000 series.

(OR)

21. 	With neat diagram, describe the structure of PLD and CPLD.

[P.T.O.]

22. 	a. 	Explain the software tools used for VHDL.                                                         		    (8)     	b. 	Explain in detail about data types with an example.                                            		    (7)

(OR)

23. 	a. 	Write a HDL code for 8:1 MUX using behavioral model.		              				    (8)

b. 	Write the HDL description of the circuit specified by the Following Boolean equations

i. 	S = xy’ + x’ y

ii. 	C = xy          								  								    (7)

24. 	a. 	Write an HDL data flow description of a 4 bit adder subtraction of Unsigned numbers use 	the conditional operator.                                                             						    (8)

b. 	Write the HDL gate level description of the priority encoder.                        			    (7)

(OR)

25.	a. 	Write VHDL code for a full sub tractor using logic Equation.                       	 		    (8)

b. 	Write a VHDL description of an S-R latch using a process.                           			    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	TRANSMISSION LINES AND WAVE GUIDES		      	     Time : 3 hours

Subject Code:	EC280								 			      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The practical condition for a distortion-less line is ___________.

2. 	Consider a lossless line with characteristic impedance Ro and VSWR = S. Then the impedance at    the point of voltage maxima equals ___________.

3. 	What is a Flat line?

4. 	The centre point of smith chart represents ___________.

5. 	TEM wave travelling between a pair of parallel perfectly conducting plane is a special type of ___________ wave in which an electric field E along the direction of propagation is  zero.

6. 	The average power transmitted along a parallel plane guide is given byWav=voI0e−2α z2. Find the rate of decrease of transmitted power along the guide.

7. 	The non-zero field components of the TM11mode in a rectangular waveguide are    ___________.

8. 	A waveguide can be considered to be equivalent to a ___________ filter.

9. 	Define the quality factor of the cavity resonator.

10. 	Write any two parameters describing the performance of a microwave resonator.

PART – B  (5 x 3 = 15 MARKS)

11. 	Let us consider a uniformly loaded cable circuit for which it may be assumed that conductance     is zero and for which inductance has been increased so that ωL is large with respect to R. Show that

a.	α=R2CL and

b.	β=ωLC

12.  Derive an expression for input impedance of a Quarter Wave Transformer.

13.	Between parallel conducting plates, the E field and H field are perpendicular to each other and both of them are perpendicular to the direction of propagation also. Which type of electromagnetic wave is this? Find the following for this electromagnetic wave if air is the dielectric medium between the parallel conducting plates.

a.	Propagation constant, γ

b.	Phase constant, β

c.	Velocity of propagation, v

d.	Cutoff frequency, fc.

14. 	Why TM01, TM10,and TEM mode does not exist in a rectangular waveguide?  Give your answer supported with mathematical equations governing wave components for electric and magnetic fields.

15. Mention the differences between waveguide and cavity resonator.

[P.T.O.]

PART – C  (5 x 15 = 75 MARKS)

16. 	A generator of 1.0 volt, 1000 cycles supplies power to a 100 – mile open wire line terminated in 200 ohms resistance. The line parameters are R = 10.4 ohms / mile, L = 0.00367 Henry / mile, G = 0.8 micro mho / mile and C = 0.00835 micro farad / mile. Determine Z, Y, Zo, γ, α and β and velocity of propagation.

(OR)

17. 	A coaxial line for carrier telephony is 900 miles long, from New York City to Chicago. The distributed inductance of the line is 0.425 millihenries / mile and its distributed capacitance is 0.0715 microfarads / mile. The distributed resistance R and the distributed conductance G are small enough to have no effect on the values of the phase factor β and the characteristic impedance Zo at the frequency of operation (R and G do cause attenuation, however, which is offset by amplifiers at intervals along the line). There are no reflected waves on the line. [a] How long is the telephone system delayed in transmission from New York City to Chicago? [b] When the RMS signal voltage at a particular point on the line is 5.0 volts, what is the RMS signal current at that point and what is the signal power passing that point?

18. 	Determine the following [a] standing wave ratio(VSWR)  [b] load admittance [c] impedance of       the transmission line at the maximum and minimum of the stationary wave along the line [d] distance between the load and first voltage maximum for a transmission line with characteristic impedance of 50 ohm with a receiving end impedance of 100 + 121j using smith chart. The wavelength of the electrical signal along the line is 2.5m.

(OR)

19.  Discuss about the single stub impedance matching on a line. Derive the expression for the 	distance d from voltage minimum to locate the stub.

20.	A pair of perfectly conducting planes is separated by 8 cm in air. For frequency of 5000 MHz with the TM11mode excited, find the following.

a.	Cutoff frequency.

b.	Wave impedance.

c.	Attenuation constant if operating frequency equals 0.95 times the cutoff frequency.

d.	Phase constant.

(OR)

21. 	Let us consider a parallel plane conducting guiding medium where there is a magnetic field component in the direction of propagation but no electric field component in the propagation direction. Start with Maxwell’s equations and develop a wave equation. Make the choice of coordinates in such a way that boundary conditions can be easily applied later. Introduce the restrictions of harmonic variations with respect to time and a wave travelling in Z direction. Obtain the general solution of the electric and magnetic field components if dominant mode is propagating. Sketch the electric and magnetic field lines when the dominant mode is propagating between parallel planes.

22.	Obtain the solution of electric and magnetic fields of TM waves guided along rectangular

waveguides.

(OR)

23. 	For an air filled copper X – band rectangular waveguide with dimensions a = 2.286 cm,

b = 1.016 cm,

a.	Determine the cutoff frequency of the first four propagating TE modes.

b.	Find the attenuation factor for first four modes, if the length of the guide is 2 m and 	operating frequency is 10 GHz.

24. 	Derive the Q factor of a cavity resonator for TE101mode.

(OR)

25.	Derive the characteristic equations of TM waves in circular waveguides.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ANTENNAS AND WAVE PROPAGATION			      	     Time : 3 hours

Subject Code:	EC282								 		       	      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the radiation resistance of a half wave dipole?

2.	Give the expression for retarded current.

3.	The advantage of binomial array is ___________.

4.	How to increase the radiation resistance of a loop antenna?

5.	What is meant by parasitic elements?

6.	State the limitations of rhombic antenna.

7.	A rectangular aperture with a constant field distribution, with a=3λ and b=2λ is mounted on an infinite ground plane. Compute FNBWE in the E-plane.

8.	What features of horn antenna are responsible for its widespread applicability?

9.	Which layer is suitable for propagation of high frequency signals? Why?

10.	Define duct propagation.

PART – B  (5 x 3 = 15 MARKS)

11.	A dipole having a length of 3 cm, operates at 1 GHz. Calculate the radiation resistance if efficiency factor, k =0.6.

12.	List the applications of helical antenna.

13.	Define self impedance and mutual impedance.

14.	A widely used microwave antenna is the circular aperture. Justify the statement.

15.	What are the factors that affect the propagation of radio waves?

PART – C  (5 x 15 = 75 MARKS)

16.	Derive expression for the power radiated in the far field and radiation resistance of the half wave dipole.

(OR)

17.	a.	Explain the fields associated with Hertzian dipole.								  (10)

b.	Mention the effect of ground in a quarter wave or monopole antenna.				   (5)

18.	a.	Derive the array factor of N- isotropic element linear array with uniform amplitude and 	spacing along z-axis.  															    (8)

b.	With neat sketch, explain the operation of helical antenna.							    (7)

(OR)

19.	a.	Derive the expression for far field components of a small loop antenna.  			  (10)

b.	With an example, illustrate the pattern multiplication.								    (5)

20.	What is Yagi-uda antenna? Explain the construction and operation of Yagi-uda antenna. Also explain its general characteristics.

(OR)

[P.T.O.]

21.	a.	Explain the construction and design of a rhombic antenna.							    (5)

b.	Explain the geometry of a log periodic antenna? Give the design equations and uses of log periodic antenna?																  (10)

22.	a.	Describe the construction and operation of the paraboloid reflector antenna.			  (10)

b.	Prove that slots and dipoles are dual antennas.									    (5)

(OR)

23.	Derive the various field components radiated from the circular aperture antenna and also find beam-width and effective area.

24.	a.	Explain the ground wave propagation.											    (5)

b.	In detail explain the diversity reception methods.									  (10)

(OR)

25.	a.	Discuss the effects of earth’s magnetic field on ionosphere radio wave propagation.	  (10)

b.	Discuss briefly about the Faraday rotation in sky wave propagation.				    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ELECTRON DEVICES								      	     Time : 3 hours

Subject Code:	EC283							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Energy band gap in pure germanium is ______

(A) 1.1eV (B) 0.5eV (C) 0.67eV (D) 1eV

2.	The force acting on a conductor kept in magnetic field is given by ________.

3.	Avalanche breakdown of PN junction diode occurs_______.

(A) in forward bias (B) in reverse bias

(C) due to manufacturing defect (D) due to decrease in temperature

4.	In p-type semiconductors, number of holes __________ number of electrons.

(a) Equal (b) Greater than (c) Less than (d) Twice

5.	In CE configuration, the input V-I characteristics are drawn by taking

a.	VCE versus IC for constant value of IE.

b.	VBE versus IE for constant value of VCE.

c.	VBE versus IB for constant value of IC.

d.	VBE versus IB for constant value of VCE.

6.	The magnitude of electric current ICBO

a.	depends largely upon the emitter doping.

b.	depends largely upon emitter-base junction base potential.

c.	increases with the increase in temperature.

d.	is generally greater in silicon than in germanium transistor.

7.	In a MOSFET, the polarity of the inversion layer is the same as that of the

a.	charge on the gate electrode.

b.	minority carriers in the drain .

c.	majority carries in the substrate.

d.	majority carries in the source .

8.	In the constant-current region, the IDS change in an n-channel JFET______

a.	As VGS decreases ID decreases.

b.	As VGS increases ID increases.

c.	As VGS decreases ID remains constant.

d.	As VGS increases ID remains constant.

9.	In the second approximation, the total voltage across the zener diode is the sum of the breakdown voltage and the voltage across the

a.	source.

b.	series resistor.

c.	zener resistance.

d.	zener diode.

10.	The turn-on time of a SCR with inductive load is 20 μs. The pulse train frequency is 2.5 kHz with a mark/space ratio of 1/10, then SCR will

a.	Turn on.

b.	Not turn on.

c.	Turn on if inductance is removed.

d.	Turn on if pulse frequency us increased to two times.

[P.T.O.]

PART – B  (5 x 3 = 15 MARKS)

11.	Define potential and how it is expressed in terms of electric field intensity.

12.	What is transition capacitance?

13.	Define collector efficiency in a BJT.

14.	Why is FET called as square-law device?

15.	Write three applications of optocouplers.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain with necessary diagrams the electrostatic deflection in CRT and magnetic deflection in CRT.

(OR)

17.	a.	Show that the two dimensional motion of electron between parallel plates is parabolic in 	nature.   										       								    (8)

b.	Explain the motion of a charged particle in magnetic field with necessary equations.	    (7)

18.	Derive the continuity equation in semiconductor.

(OR)

19.	Explain the working of a PN junction diode and derive the expression for diode current.

20.	Derive the Eber-Moll equations for BJT.

(OR)

21.	Draw and explain the characteristics of BJT in CC configuration.

22.	Explain the construction and working on enhancement type MOSFET.

(OR)

23.	Explain the construction, working and characteristics of SCR.

24.	Explain the construction and working of Tunnel diode with energy band diagram.

(OR)

25.	Explain the working of Plasma display and nano crystal display

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DIGITAL ELECTRONICS							      	     Time : 3 hours

Subject Code:	EC284							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Convert (153)10 to Octal number.

2.	Convert gray code 101011 into Binary equivalent.

3.	The universal gates are _________ and __________.

4.	Give the truth table for half adder.

5.	Define flip flop.

6.	The time required to change the voltage level from 10% to 90% is called as _________.

7.	Define Finite State machine.

8.	Define state table.

9.	What is Tri state logic?

10.	CMOS is a combination of _________ and _________.

PART – B  (5 x 3 = 15 MARKS)

11.	State De Morgan’s Theorem.

12. 	What is a priority encoder and binary decoder?

13. 	Give the comparison between synchronous and asynchronous counters?

14. 	Explain the procedure for state minimization.

15. 	Define emitter coupled logic(ECL).

PART – C  (5 x 15 = 75 MARKS)

16. 	Convert the following Hexadecimal number to their equivalent Decimal numbers.

a.	(E7A9)16		b.	(632)16			c.	(2647)16								(3x5)

(OR)

17. 	a.	Simplify the Boolean Function using k maps and express the results in SOP and POS form. 
	FA, B, C, D=m(0, 1, 2, 4, 5, 6, 8, 9, 12, 13, 14)	.									    (8)

b.	Minimize the Boolean Function 	FA,B,C= ABC +ABC+ABC+ABC+ABC

using K – map and draw the logic diagram.					 					    (7)

18. 	a.	Design a 4 bit Magnitude comparator with neat diagram.							  (10)

b.	Design a full adder with the truth table and logic diagram.							   (5)

(OR)

19. 	a.	Design and explain the operation of 4 : 1 Multiplexer.								    (7)

b.	Explain the parity checker and generator.										    (8)

20. 	a.	Draw the logic diagram of clocked JK Flip Flop and explain its working. 			    (7)

b.	Explain the Excitation Table for SR,JK,D, T Flip Flop.							    (8)

(OR)

21. 	a.	Explain Ripple Counter with neat diagram.										   (7)

b.	Give the comparison between synchronous and Asynchronous counters.	 		   (8)

[P.T.O.]

22. 	a.	Give the design of Synchronous sequential circuits.                                      			   (7)

b.	Give the comparison of Melay and Moore Machine		           					   (8)

(OR)

23.	Design a sequential circuit with 4 flip flops, A,B,C,D. The next states of A are equal to the Exclusive – OR of the present state of C and D. The next states of B,C and D are equal to the present states of A,B,C respectively.

24.	Explain the following terms  a)DTL  b) RTL                                           					(7+8)

(OR)

25. 	Write technical note on a.	CMOS Inverter.                                                          			  (10)

b.	Salient features of ECL.                                               			  (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	SIGNALS AND SYSTEMS							      	     Time : 3 hours

Subject Code:	EC285							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Sketch the DT unit ramp signal.

2.	The unit parabolic signal can be obtained by integrating ___________ signal.

3.	The FT of a rectangular pulse existing between t = − T/2 to t = T/2 is a ___________ function.

4.	If the signal is expanded in time domain then it is ___________ in frequency domain.

5.	What is meant by aliasing?

6.	Laplace transform of unit step signal is ___________.

7.	Fourier transform, Laplace transform, and Z transform of the impulse signal is unity. State True or False.

8.	Find DTFT of δ[n–k]= ___________.

9.	An LTI system is stable if and only if the ROC of its system function H(Z) ___________.

10.	Two sided Z transform also called as ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	Determine the response of the system y(n) for the given input  x(n) = u(n) – u(n–7) and the impulse response

h(n) =u(n–1) –u(n–4).

12.	Define the Fourier transform pair for continuous time signal.

13.	Find initial and final values of the signal corresponding to the Laplace Transform 				X(S)= (7S+10)/S(S+2).

14.	State and prove the time reversal property of DTFT.

15.	Determine the Z transform of the signal x(n)=n u(n).

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Determine whether the following CT system  y(t)=10 x(t)+5 is Static or Dynamic.

b.	Linear or Non Linear.

c.	Causal or Non Causal.

d.	Time Invariant or Time Variant. 												  (10)

e. 	Find the output response of the system y(n) using   convolution sum. Input x(n)=2n u(–n) Impulse response h(n)=u(n).													   (5)

(OR)

17.	The square pulse signal x(t) is given below. Sketch each of the following signals derived from x(t).

x (t)

t

1

-1

1

a.	x (3t)

b.	x (3t+2)

c.	x(–2t–1)

d.	x(2(t+2))

e.	x(3t)+x(3t+2)

[P.T.O.]

18.	a.	State and prove Convolution property of CTFT.   								    (5)

b.	Find the Convolution of the two signals xt=e−atu(t)and h(t)=e−btu(t)using FT.	  (10)

(OR)

19.	Obtain the Cosine Fourier series expansion of a half wave rectified sine wave.

20.	How will you reconstruct a signal from its samples? Explain with neat diagrams.

(OR)

21.	a. 	Find x(t) if X(S)= (4s2+6)/ (s3+s2-2).											  (10)

b. 	Find X(S) if x(t)=e−btand plot the signal in S domain.							   (5)

22.	State and prove any five properties of DTFT.

(OR)

23.	Find the FT of the following signals

a.	x(n)= δ(6–2n)+ δ(6–2n).														    (8)

b.	x(n)= –anu(–n–1)																    (7)

24.	Perform long division method to find x(n)

X(Z)=(z+1)/(z2–3z+2)

a.	x(n) is causal. 																    (8)

b.	x(n) is anti causal.															    (7)

(OR)

25.	a.	What is meant by ROC of Z transform? Explain its significance.					    (5)

b.  	Determine Z transform for the signal

x(n) = 7(1/3)n u(n) – 6(1/2)n u(n) and plot its  RoC.								  (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	SOLID STATE CIRCUITS							      	     Time : 3 hours

Subject Code:	EC288							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Ripple factor of a half wave rectifier is ____________.

2.	In series regulation the control element is placed in ____________ between input voltage and output voltage.

3.	In cut off region of transistor operation emitter base junction is ____________ biased.

4.	JFET has ____________ temperature coefficient.

5.	Coupling capacitor in RC coupled amplifier is to ____________ dc signal.

6.	Cross over distortion in class B amplifier is due to ____________ potential.

7.	Amplifier uses ____________ feedback.

8.	Common Mode Rejection Ratio is ____________.

9.	Crystal oscillator inductor is replaced by ____________.

10.	Lower the resistance of an inductor ____________ the Q factor.

PART – B  (5 x 3 = 15 MARKS)

11.	Give the limitations of Zener regulator.

12.	Define pinch off voltage.

13.	Brief out the need for multistage amplifier.

14.	An amplifier has an open loop gain of 1000 and a feedback ratio of 0.004.If the open loop gain changes by 10% due to temperature. Find the % change in gain of the amplifier with feedback.

15.	Define Barkhausen criteria.

PART – C  (5 x 15 = 75 MARKS)

16.	A half wave rectifier having resistive load of 1000 Ohm, rectifies an alternating voltage of 325 V peak value and the diode has a forward resistance of 100 Ohm. Calculate                                a) Peak,average,rms values of current b) dc output power c) ac input power and d) efficiency of the rectifier.

(OR)

17.	Explain the working principle of full wave rectifier circuit with  filter. Derive its ripple factor.

18.	Explain collector to base bias and fixed bias arrangement of transistor in detail.

(OR)

19.	Derive the source self bias and voltage divider bias circuit for FET.

20.	a.	Explain transformer coupling in cascaded stages.									  (10)

b.	Explain direct coupling in cascaded stages in detail.				     				    (5)

21.	Explain with necessary equations and diagrams any two types of power amplifiers in detail.

[P.T.O.]

22.	Explain

a.	Voltage series feedback. 														(3)

b.	Current series feedback. 														(4)

c.	Current shunt feedback. 														(3)

d.	Voltage shunt feedback.														(4)

(OR)

23.	Explain the stability of feedback amplifier with necessary equations.

24.	Explain Hartley and Colpitts Oscillator in detail.

(OR)

25.	Explain capacitance coupled single tuned amplifier in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	PULSE AND WAVE SHAPING CIRCUITS			      	     Time : 3 hours

Subject Code:	EC289								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the reason for terming the capacitor C in high pass RC circuit as a “blocking capacitor”.

2.	Mention the two classes of non-linear waveshaping circuits.

3.	A bistable multivibrator consists of (a) Two stable state (b) two quasi stable state (c) one stable and one quasi stable state.

4.	List the applications of Schmitt trigger.

5.	Monostable multivibrator has two stable states. Say True or False.

6.	Write an expression for gate width T of a collector coupled astable multivibrator.

7.	Give the mathematical relationship between displacement error and transmission error for an exponential waveform.

8.	What is meant by Restoration time?

9.	List the two types of sampling gates.

10.	Define Blocking oscillator.

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish between clippers and clampers.

12.	Mention the methods for triggering bistable multivibrator and distinguish between them.

13.	List the advantages of emitter-coupled astable Multivibrator over collector coupled astable Multivibrator.

14.	Obtain expressions for slope error and sweep speed in the miller sweep voltage waveform generator.

15.	Write some of the applications of blocking oscillator.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	A symmetrical square wave of peak–to-peak amplitude V and frequency f is applied to a 	high pass RC circuit. Show that the percentage tilt is given by

P=T2RC×100%																  (10)

b.	Obtain the criterion for good integration for low pass RC circuit.					    (5)

(OR)

17.	a.	Explain the working principle of positive and negative series biased diode clippers with 	circuit diagram and sketch the output waveform.									    (8)

b.	State Clamping circuit theorem and mathematically prove it.						    (7)

18.	Calculate the stable state currents and voltages for the fixed bias bistable multivibrator consisting of cross coupled inverter circuits whose parameters are Rc1=Rc2=2.2KΩ;              R1= 15kΩ; R2= 100kΩ; Vcc= +12V; Assume hfe= 20, VC2= VB2 = 0V,-VBB= -12V ,transistor Q1 is OFF and Q2 is in saturation (ON).

(OR)

[P.T.O.]

19.	With a help of circuit diagram, make a quantitative analysis of the transistor Schmitt trigger to obtain an expression for the critical voltages VUTP and VLTP.

20.	Explain the collector coupled monostable multivibrator with neat circuit diagram and obtain an expression for Gate width T when the transistor Q1 is in saturation.

(OR)

21.	Consider a practical emitter coupled astable multivibrator employing two NPN transistors with the following circuit parameters: VCC = 30V, VBB=20V, VBE2 =0.6V,  RC2 =0.2 kΩ, hfe=30, R2=2R1>>R’’, R’=R’’=1kΩ, RE1 = RE2 =3.3 kΩ, VBE(sat)=0.7V, VCE(sat)=0.3V, V=0.5V and C=0.1µF. Calculate the voltage levels of the waveforms at t=t1- and t=t1+ and the frequency of oscillation.

22.	Derive the relationship between displacement error (ed), transmission error (et) and slope error (es).

ed= es / 8 = et /4

(OR)

23.	Explain in detail how the sweep circuit is employed in television with necessary circuit diagrams.

24.	Make use of neat sketches of circuits and analyse the triggered transistor monostable blocking oscillator with base timing to obtain an expression for the pulse width tp.

(OR)

25.	Sketch the circuit of a simple unidirectional diode sampling gate and explain the effect of the higher voltage level –V2 of the control input on the output of the circuit.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	INFORMATION THEORY AND CODING			      	     Time : 3 hours

Subject Code:	EC290								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is entropy?

2.	What is channel redundancy?

3.	What is the advantage of delta modulation over PCM?

4.	Give one advantage of adaptive delta modulation.

5.	Define Code efficiency.

6.	Why cyclic codes are extremely well suited for error detection?

7.	Give two drawbacks of data compression.

8.	What are the types of compression?

9.	What is MPEG?

10.	Give the main application of linear predictive coding.

PART – B  (5 x 3 = 15 MARKS)

11.	State the Shannon’s capacity theorem in communication systems.

12.	What are the advantages of differential pulse code modulation?

13.	What is meant by cyclic code?

14.	Differentiate lossy and lossless compression.

15.	What is perceptual coding?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	State and prove continuous channel capacity theorem.								  (10)

b.	Write a short note on discrete channel capacity.									    (5)

(OR)

17.	A discrete source emits symbols with the following probabilities p(x)={0.07, 0.08, 0.04, 0.26, 0.14, 0.09, 0.07, 0.25}.  Using Huffmann code determine the coding efficiency and redundancy.

18.	Explain DPCM transmitter and receiver with block diagram with block diagram. How it is different from PCM?

(OR)

19.	Explain delta modulation in detail with suitable diagram.  Explain adaptive delta modulation and compare its performance with delta modulation.

20.	a.	Define linear block codes.														    (3)

b.	How to find the parity check matrix.											    (6)

c.	Give the synchrome decoding algorithm.										    (6)

(OR)

21.	Verify whether g(x) =1+x+x2+x3+x4 is a valid generator polynomial for generating a cyclic code for message [1 1 1].

22.	Write short notes on

a.  Graphics Interchange file format.												    (7)

b.  Tagged image file format.														    (8)

[P.T.O.]

(OR)

23.	a.	Discuss the various stages in JPEG standards.									    (7)

b.	Explain arithmetic coding with suitable example.									    (8)

24.	Explain the principles of perceptual coding.

(OR)

25.	Discuss in brief, the following audio coders.

a.  MPEG audio coders.

b.  DOLPY audio coders.