Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	BASIC ELECTRONICS AND INSTRUMENTATION      	     Time : 3 hours

Subject Code:	09EI101								 	Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the function of the rectifier.

2.	List the terminals in the transistor.

3.	List any two registers.

4.	Write the merits of counter.

5.	Define error in the measuring instrument.

6.	Name the type of instrument to measure AC quantity.

7.	What is the output of transducer?

8.	How do you measure moisture?

9.	Write the function of galvanometer.

10.	Name the device to measure time interval.

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish between PN junction diode and Zener diode.

12.	Write the applications of binary number system.

13.	Distinguish between accuracy and precision.

14.	Write the classification of transducers.

15.	List the merits of multimeter.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe the characteristics of CB and CE bipolar junction transistors with neat diagrams.

(OR)

17.	Explain the operating principles of SCR and UJT with neat schematic diagrams.

18.	Enumerate the principles of RS and JK flip flops in detail.

(OR)

19.	Write brief notes on half and full adders with illustrations.

20.	Explain the static and dynamic characteristics of measuring instrument.

(OR)

21.	Describe any three methods for the measurement of errors.

22.	Explain the principles of temperature and velocity transducers.

(OR)

23.	Describe the working of viscosity and pH meters.

24.	Enumerate the concept of A/D and D/A converters in detail.

(OR)

25.	Write short notes on a) Calibration of meters  b) CRT display

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	INSTRUMENTATION AND PROCESS CONTROL FOR FOOD ENGINEERS											      Time : 3 hours

Subject Code:	11EI215								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Sensor.

2.	What is the Laplace transform of unit step function?

3.	Define transfer function.

4.	What is the essential element of closed loop control system?

5.	Mention the use of branch point in signal flow graph.

6.	Find the order of the system whose characteristic equation is S3+S2+S+1=0 and give reason?

7.	What is Gauge factor?

8.	The resistance of a conductor _________ with length and _________ with area.

9.	What is mass spectrometer?

10.	Define accuracy.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the block diagram of heated tank system.

12.	What is servo mechanism?

13.	State the Routh stability criterion for stability analysis.

14.	What are the applications of thermistor?

15.	Write short notes on capacitive sensing method for level measurement.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain in detail about different elements involved in process control with block diagram.																						  (12)

b.	What is steady state error?														  (3)

(OR)

17.	a.	Describe the different test functions involved in process control.					    (8)

b.	Write short notes on damped and cyclic response of process control.				    (7)

18.	a.	With neat sketch, Illustrate the functions of open-loop and closed loop temperature control 	systems.																		    (8)

b.	Drive the transfer function of pneumatic system.									    (7)

(OR)

19.	a.	Describe the performance responses of the different control valves.					  (12)

b.	Write short notes on Proportional controller.										    (3)

20.	Draw a signal flow graph and evaluate the closed loop transfer function of a system whose block diagram is shown as

(OR)									[P.T.O.]

21.	Using Routh criterion, determine the stability of the system represented by the characteristic equation s6+2s5+8s4+12s3+20s2+16s+16=0. Also determine the location of the roots of Characteristic equation.

22.	a.	What are the different methods for pressure measurement?						    (3)

b.	Explain the working principle of Bourdon tube pressure gauge for pressure measurement.

(12)

(OR)

23.	a.	Explain in detail about the construction and working principle of Resistance Thermometer 	with suitable diagram.															  (10)

b.	State the various application laws of thermocouple.								  (5)

24.	a.	Discuss the role of simple float system for level measurement.						    (8)

b.	Write short notes on the technique of buoyancy meter.							    (7)

(OR)

25.	a.	Explain in detail about dual beam non dispersive infra red analyzer.				    (7)

b.	Explain the working of gas chromatograph with schematic diagram.				    (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	CONTROL SYSTEMS								      	     Time : 3 hours

Subject Code:	09EI219							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by a ‘system’?

2.	Define transfer function.

3.	State the order and type of the following system.

GsHs=200s2+20s+200

4.	Enumerate the advantages of generalized error coefficients.

5.	Draw the log-magnitude plot of 1jω2.

6.	Define Bandwidth.

7.	Can derivative mode of control be used alone? Why?

8.	When a system is said to be stable?

9.	What is a Synchro?

10.	Draw the block diagram of fuzzy logic controller.

PART – B  (5 x 3 = 15 MARKS)

11.	Write the mason’s gain formula.

12.	How the system is classified depending on the value of damping?

13.	Write the Nyquist Stability Criterion.

14.	Sketch the polar plot of

Gs=1s21+sT11+sT21+sT3
15.	Compare the features of AC and DC servomotors.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Write the differential equations governing the mechanical rotational system shown in 	figure and determine the transfer function θ(s)/T(s)

(10)

b.	Differentiate between open loop and closed loop systems.							   (5)

(OR)

[P.T.O]

17.	Determine the overall transfer function of the system C(s)/R(s).

18.	Derive the mathematical expressions for the time response of first order system with unit step input. Sketch the response.

(OR)

19.	a.	A unity feedback control system has an open loop transfer function G(s)=10/s(s+2).  Find 	the rise time, percentage over shoot, peak time and settling time.					    (8)

b.	For a servomechanisms with open loop transfer Function C(s) / R(s) = 10/(s+2) (s+3). What type of input signal gives constant steady state error and calculate its value.	    (7)

20.	A Unity feed back control system has an open loop transfer function G(s) = K / s(s+1)(s+5). Design a suitable Lead compensator is to meet the following  specifications.

(a) Kv(Velocity error constant) ≥ 50; (b) Phase margin ≥200(degree)

(OR)

21.	For the transfer function given below, sketch the Bode plot and hence find Gain cross over frequency,     Phase cross over frequency, Gain margin and Phase margin.

G(s) = 5(1+2s) / (1+4s) (1+0.25s)

22.	a.	Using Routh- Hurwitz criterion determine the stability of the system represented by   the 	characteristic equations s4 s4+8s3+18s2+16s+5 = 0. Comment on the location of the roots of 	the characteristic equation.													 	    (7)

b.	What are the characteristics of P, PI, PD and PID (Proportional+Integral+Derivative)  	Controller?																	    (8)

(OR)

23.	The open loop transfer function of unity feed back control system is

G(s) = K(s+9) / s(s2+4s+11). Sketch the root locus of the given system.

24.	a.	With a neat sketch explain the AC Tachogenerator.								    (7)

b.	With an example, explain how fuzzy logic controller can be used to control the temperature of a process.														    (8)

(OR)

25.	Explain the function of Synchro as error detector.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ELECTRICAL MACHINES							      	     Time : 3 hours

Subject Code:	09EI220							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name the rule which give the direction rotation of a DC motor.

2.	List the major two factors on which the speed of dc shunt motor depend.

3.	Define transformation ratio of a transformer.

4.	Mention the two components of no load current of a transformer.

5.	Define slip of an induction motor.

6.	Write the two types of rotor construction of synchronous generators..

7.	Write the use of tachogenerator.

8.	Name the motor which can be operated both at AC and DC suuply.

9.	List the methods of electrical welding.

10.	What is meant by electric heating?

PART – B  (5 x 3 = 15 MARKS)

11.	List the applications of three types of DC motor.

12.	Distinguish between shell type and core type transformer.

13.	Write the principle of operation of three phase induction motor.

14.	Write the principle and applications of linear induction motor.

15.	Outline the recent trends in electric traction.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Explain the constructional features of DC machine with relevant diagram.       		  (10)

b.	From the first principles, derive the emf equation of a DC generator.                 		    (5)

(OR)

17.	a.	Explain the electrical and mechanical characteristics of DC series motor.       		    (7)

b.	Draw the diagram of 3 point starter and explain various protective devices  available in 3 	point starter.                                                                      							    (8)

18.	a.	Explain the operation of single phase transformer under loaded condition.  			    (7)

b.	From the first principles, obtain the approximate equivalent circuit of transformer. 	    (8)

(OR)

19.	A 25 kVA, 2200/220V, 50-Hz transformer is tested for efficiency and  regulation as follows:

O.C. Test: 220V, 4A, 150W

S.C. Test: 90V, 10A, 350W

a.	Regulation of transformer at 0.8 power factor lagging current

b.	Efficiency at full load and 0.8 power factor lagging current.

20.	a.	Draw the torque slip characteristics of three phase induction motor with different rotor 	resistance values and explain.                 											    (8)

b.	Explain the working principle of auto transformer starters with a neat diagram. Bring out 			its advantages.                                                                                        				    (7)

(OR)

[P.T.O.]

21.	a.	From the first principles, derive the emf equation of a three phase alternator with short 	pitched and distributed winding.                                                         				    (7)

b.	Draw the V curves of synchronous machine and justify its shape.                      		    (8)

22.	a.	Explain the working principle and applications of AC servo motor.                  		    (8)

b.	Explain the working principle and application of variable reluctance stepper motor. 	    (7)

(OR)

23.	a.	Explain the double field revolving theory for a single phase induction motor.    		    (8)

b.	Explain the operating principle of capacitor start capacitor run single phase  induction 	motor.                                                                                                   				    (7)

24.	Explain the working principles of different methods of electric heating.  Bring out their applications.

(OR)

25.	Write the significant features of electric traction motors and explain the various methods of control.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	NETWORKS & PROTOCOLS FOR INSTRUMENTATION & CONTROL													      	     				      Time : 3 hours

Subject Code:	09EI226								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define the term protocol.

2. 	UART stands for _________.

3. 	List the functions of transport layer.

4. 	List the advantage of USB.

5. 	The term IEEE stands for _________.

6. 	Which OSI layer defines the standards for data formats and encryption?

7. 	The term HART stands for _________.

8. 	Mention the merits and demerits of field bus.

9. 	List the different kinds of switching technique.

10. 	Mention few examples of TCP/IP transport layer protocols.

PART – B  (5 x 3 = 15 MARKS)

11. 	Differentiate the bit oriented protocol and character oriented protocol.

12. 	What is the need for 4-20mA current loop in process industries?

13. 	List any four salient features of HART protocol.

14. 	How the reliability of token ring topology can be improved?

15. 	In what situation, contention based MAC protocols are suitable?

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain in detail about the building blocks of communication network.	Also discuss in detail about the communication principles and its modes.

(OR)

17. 	Discuss about the synchronous and asynchronous systems and explain in detail about the             transmission characteristics data coding.

18.  Explain in detail about any one serial communication interface standard.

(OR)

19.  Explain in detail about the IEEE 488 serial communication standard.

20.   Explain in detail about HDLC & SDLC.

(OR)

21.  Explain in detail about the functions of the different layers of OSI Model.

22. 	Explain any one popular communication protocol used in field (or) flow control.

(OR)

23.  	a.	Explain in detail about HART communication protocol. 	             					    (8)

b.	Compare HART & OSI model.													    (7) 24. 	a.	Discuss in detail about circuit and packet switching.				           			    (8)

b.	Write a note on Ethernet.		            											    (7)

(OR)

25.  	Discuss in detail on various internet control protocols.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December -2015

Subject Title :	CIRCUIT ANALYSIS & NETWORKS				      	     Time : 3 hours

Subject Code:	10EI201								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Any closed path of a circuit is called as ________.

2. 	State kirchoff’s voltage law.

3. 	Norton’s equivalent circuit consists of _________.

4. 	State millman’s theorem.

5. 	What is resonant frequency?

6.  	Draw the frequency response of RLC parallel circuit.

7. 	Give the relation between self and mutual inductances.

8.  	What is double-tuned circuit?

9. 	Write the performance equations of h-parameters.

10. 	Define output impedance of a two-port network.

PART – B  (5 x 3 = 15 MARKS)

11. 	Convert the following current source into voltage source.

12. 	Write the limitations of ohms law.

13. 	What is free and forced response?

14. 	Write the expression for equivalent inductance of two coupled coils connected in parallel.

15.  	What are the assumptions made in the analysis of two-port networks?

PART – C  (5 x 15 = 75 MARKS)

16.  a.	What are the classifications of dependent and independent sources?

b.	Determine currents I1, I2 and I3 in the given network.

[P.T.O.]

(OR)

17. 	Determine the voltages at each node for the below circuit.

18. 	State super position theorem. Find the voltage across the 2  resistor in below circuit.

(OR)

19.  	a.	State and explain maximum power transfer theorem. 								    (6)

b.	State Thevenin’s theorem. Find the current in 3  resistor in the below circuit.		    (9)

20.	Derive the expression for the sinusoidal response of R-L-C circuit.

(OR)

21. 	Derive the expression for the DC response of an R-L circuit.

22.	a.	Prove that in a coupled circuit the mutual inductance M=k L1L2. 					    (7)

b.	Find the mutual reactance Xm in the coupled coils shown in the below figure , if the average power in 6  resistance is 80 W. 										    (8)

(OR)

23.	a.	Discuss about single tuned circuits. 												  (10)

b.	List out the characteristics of an ideal transformer.								    (5)

24. 	Find the transformer admittance I2 (s) / V2 (s) of the below network.

(OR)

25. 	Test results for a two port network are (a) port 2 open circuit I1 = 0.010 , V1 = 1.4 45, V2=2.3–26.5 &  (b) port 1 open circuit I2 = 0.010; V1=-90, V2=1.5-68. The source frequency in both the test was 103 Hz. Find the Z parameters & draw equivalent circuit at this frequency.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ELECTRONIC CIRCUITS							      	     Time : 3 hours

Subject Code:	10EI202							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the semiconductor devices used as switches?

2.	Define ripple factor.

3.	What is the gain of emitter follower?

4.	Which one has the greatest value among the four h-parameter of a transistor?

5.	What is meant by current mirror?

6.	Write the average power and efficiency of class A amplifier.

7.	Write the basic difference of enhancement and depletion mode of FET

8.	Define the pinch-off voltage.

9.	Which feedback amplifier is used as oscillator? Why?

10.	Write the application of astable multivibrator.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the operation of clipping circuit.

12.	Explain the miller theorem with its basic circuit.

13.	What is the need of Darlington connection in BJT? Explain.

14.	Compare the BJT and FET amplifiers.

15.	A voltage series negative feedback amplifier has a voltage gain without feedback of A = 500, input resistance = 3KΩ, output resistance = 20 KΩ and the feedback ratio = 0.01. Calculate the voltage gain and input resistance of the amplifier with feedback.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the operation of full wave rectifier with its block diagram and write the advantageous over half wave rectifier.

(OR)

17.	Explain the operation of regulator with its types

18.	Discuss the analysis of transistor amplifier using h- parameters.

(OR)

19.	a.	Draw and explain the hybrid equivalent of Common base transistor

b.	A transistor connected in CE configuration has the following h-parameters. hie = 1.1KΩ, hre = 2.5 x 10-4, hfe = 50, hoe = 25µsiemens and rs=rL= 1kΩ. Calculate current gain, input impedance and voltage gain.

c.	Explain the concept of cascading transistor amplifier with its advantageous.

20.	Explain the operation of RC coupled amplifier.

(OR)

[P.T.O.]

21.	Discuss the push pull power amplifier with its neat circuit.

22.	Explain the working principle of FET amplifier with its V-I characteristics, dynamic drain resistance and amplification factor.

(OR)

23.	Short notes on a) Low FET amplifier.    b) BIFET amplifier

24.	Derive the effect of negative feedback parameters Ri, Ro, gain, stability, distortion and bandwidth of current series feedback amplifier.

(OR)

25.	Explain the concepts of multivibrator. Briefly discuss the monostable vibrator with its applications.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	SENSORS AND TRANSDUCERS					      	     Time : 3 hours

Subject Code:	10EI203											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	A null type of instrument as compared to a deflection type instrument has a higher accuracy. Say True or False.

2. 	What are the desirable characteristics in measurement system?

3. 	Give an example for an active transducer.

4. 	In a transducer, the experimentally obtained transfer function is different from the theoretical transfer function. What is the name of error result from this difference?

5. 	What is an order of a resistance potentiometer instrument?

6. 	The resistance of a thermometer is 5Ω at 30°C and 6.5Ω at 60°C. If linear approximation is used, calculate the value of resistance temperature co-efficient at 45°C.

7. 	What is the order of minimum displacement that can be measured with capacitive transducers?

8. 	List the applications of Hall Effect transducer.

9. 	How many outputs does a tachometer encoder have?

10. 	What are properties of alpha- particles of radioactive materials?

PART – B  (5 x 3 = 15 MARKS)

11. 	A thermometer is calibrated for 150°C to 200°C. The accuracy is specified within ± 0.25 percent of instrument span. What is the maximum static error?

12. 	Derive the equation for time response of a first order system when subjected to unit ramp input. And also draw its response curve.

13. 	List the advantages of LVDT.

14. 	A capacitive transducer uses two quartz diaphragms of area 750 mm2 separated by a distance of 3.5 mm. A pressure of 900 kN/m2 when applied to the top diaphragm produces a deflection of 0.6 mm. The capacitance is 370 pF when no pressure is applied to the diaphragms. Find the value of capacitance after the application of a pressure of 900kN/m2.

15. 	Write short notes on optical encoders.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Draw the basic blocks of a generalized instrumentation system.					    (5)

b.	Define input filtering and output filtering. Explain with suitable examples.			  (10)

(OR)

17. 	Classify the errors in measurements. Explain these errors by giving suitable examples.

18.	 Describe with neat sketches the various types of primary detecting elements:

a.	Bourdon tubes			b.	Bellows  		c.	Diaphragms

(OR)

19. 	Describe how a liquid filled thermometer can be mathematically modelled as a first order system. Find expressions for static sensitivity and time constant.

20.	Describe the construction, theory and working of thermocouples. Describe the different types of compensation used.

(OR)

[P.T.O]

21. 	Explain the construction of resistance potentiometer with neat sketch and derive the expression for its output   when connected across a meter of finite impedance.

22. 	Sketch the schematic diagram of parallel plate capacitive transducer and explain its principle. How change in area of plates used in transducers?

(OR)

23. 	Draw the complete setup for measurement of displacement using piezo-electric transducer. Derive the voltage output of the transducer under load and no load conditions.

24. 	Describe how fibre optic transducers are used for:

a.	Measurement of temperature.

b.	Measurement of sound level.

c.	Measurement of liquid level.

(OR)

25. 	a.	Explain construction of encoders for displacement measurement.					  (10)

b.	Write short notes on pH measurement.											    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	SIGNALS AND SYSTEMS							      	     Time : 3 hours

Subject Code:	10EI204											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Represent x(n)= {1,2,3} in  terms of  impulses.

2. 	Draw the signal .

3. 	x(n)= {1} and h(n)={1}. convolve x(n) with h(n).

4. 	Write the convolution integral formula.

5. 	The coefficient a0 in Fourier series is always _________ for odd signals.

6. 	Write the formula to find CTFT of a signal x(t).

7. 	Determine the DTFT of .

8. 	A system is described by difference equation. Draw the   magnitude response of the system.

9. 	The two sided Z-transform of  x(n) is X(z).The Z -transform of  2x(n–1) is  _________.

10. 	 represents -transform of . Find .

PART – B  (5 x 3 = 15 MARKS)

11.  A discrete time signal is given by x(n)={4,5,6,7}. Draw x(2n) and x(n/2).

12. 	A continuous  time  system is described by differential equation  where .

13. 	Find the Fourier transform of a signal .

14. 	Determine the Nyquist frequency for the signal .

15. 	State the properties of ROC of Z-transform.

PART – C  (5 x 15 = 75 MARKS)

16. 	A signal x(t) is shown below.

Sketch the following wave forms

- b.	  		c.	 	         	d.

(OR)

[P.T.O.]

17. 	Check linearity, time invariance, static, causal and stability of the following systems

a.

b.

c.

18. 	a.	  and  Find the convolution between x(t) and h(t).    	    (9)

b.	Find the convolution between x(n) and h(n).	         	    (6)

(OR)

19. 	Find the output response of a system described by a differential equation

. The input signal The initial conditions are 	  and

20. 	a.	Prove Parseval’s relation in continuous time Fourier transform.					    (6)

b.	Find the trigonometric Fourier series representation of the signal x(t).				    (9)

(OR)

21. 	Derive convolution, time shifting, frequency shifting and time scaling properties of continuous time Fourier transform.

22.	Determine the discrete time Fourier transform of the following signals.

a.								         						    (4)

b.								         							    (6)

c.								         						    (5)

(OR)

23. 	Derive any five properties of discrete time Fourier series.

24. 	Find the output response of the discrete time system described by difference equation

The initial conditions are  and . The input is   x(n)=

(OR)

25. 	Find inverse Z-transform for the following signals.

a.	Xz=7z−23z−3z−4 ROC: z > 4												    (7)

b.	Xz=10zz+2z+42 ROC: z>4											    (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	CONTROL SYSTEMS								      	     Time : 3 hours

Subject Code:	10EI205							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name the energy storage elements in electrical systems.

2.	Define Transfer function.

3.	What do you mean by feedback?

4.	Define damping ratio.

5.	What is meant by zero-state response?

6.	Where does the root locus start and terminate?

7.	Define band width.

8.	Write the purpose of frequency response.

9.	Define the term stability in control system.

10.	What is an asymptote?

PART – B  (5 x 3 = 15 MARKS)

11.	State Mason’s gain formula.

12.	What is a Synchro? Write its principle.

13.	What do you mean by Steady – State response? On what does it depend?

14.	Define the phase crossover frequency and phase margin.

15.	Write the applications of root locus.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the basic structure of control system and describe the importance of open loop and closed loop systems.

(OR)

17.	Write brief note on block diagram reduction technique using the Mason’s gain formula with a sample system.

18.	Enumerate the modeling of automatic control system and describe the operation of AC and DC servo motors.

(OR)

19.	Describe the operating principle of stepper motor with neat diagram and explain its control parameters.

20.	A second order system has a damping ratio of 0.7 and its natural frequency of oscillation is 12 radians per second. Determine the damped frequency of oscillation, the rise time, peak time, peak over shoot and settling time. Also determine the closed – loop transfer function.

(OR)

21.	A unity feedback system has the forward path transfer function.

Gs=10ss+2

Find the steady state error and the generalized error coefficients for r(t) = t.

[P.T.O.]

22.	Obtain the polar plot of the following transfer function

Gs=1s+14s+1

(OR)

23.	Sketch the Bode plot for the following transfer function.

Gs=KS2S+10.1S+11.0.01S

24.	The open loop transfer function of a unity feedback control system is given by

GsHs=KSS+13S+12S+1

Determine the range of values of K for which the system is stable. Also determine the value of K for sustained oscillations and the frequency of oscillations.

(OR)

25.	Draw the complete root locus for the system with

Gs=KS+3SS2+S+2

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MICROPROCESSORS AND MICROCONTROLLERS    	     Time : 3 hours

Subject Code:	10EI207								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Each manufacturer of a microprocessor has devised a symbolic code for each instruction, called a __________.

2.	What operation can be performed by using the instruction ADD A?

3.	The contents of the accumulator are 93H and the contents of register C are B7H.Add both contents and write the answer.

4.	The function of POP instruction is __________.

5.	__________ has the highest priority interrupts.

6. 	The BSR mode is concerned only with the __________ bits of PORTC.

7. 	The most widely used registers in 8051 are __________,

8. 	The advantage of 8751 is __________.

9. 	The ASCII system uses __________ bits to represent each code.

10. 	The microcontroller is used to measure ON and OFF time of a perfect __________ pulse.

PART – B  (5 x 3 = 15 MARKS)

11. 	Distinguish between troubleshooting and debugging a program.

12. 	Specify the memory location and its contents after the following instructions are executed.

MVI B,F7H

MOV A,B

STA XX75H

HLT

13. 	Explain the functions of Hand shake signals.

14. 	Show the status of the CY, AC and P flag after the addition of 9CH and 64H in the following instructions.

MOV A, #9CH

ADD A, #64H; after the addition A=00H, CY=1

15. 	Compare 8051 and 8031.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Assume register B holds 93H and the accumulator holds 15H. Illustrate the results of the 	instructions ORA B, XRA B and CMA.											    (7)

b.	Write a program using the ADI instruction to add the two hexadecimal numbers 3AH and 48H and to display the answer at an output port.									    (8)

(OR)

17.	a.	Discuss branch operations with their classifications.								    (5)

b.	Draw the Functional block diagram of 8085A microprocessor. 						  (10)

18.	a.	Explain Five Vectored interrupts of 8085.										    (5)

b.	Draw the block diagram of interfacing 8237A-DMA Controller with the 8085.		  (10)

(OR)

[P.T.O.]

19.	a.	Discuss about DMA Controller and DMA signals.								    (8)

b.	Explain multiple interrupts and priorities.										    (7)

20.	a.	Calculate the values of the MSB, LSB and full scale output for an 8-bit DAC for the 0 to 	10V range.																	    (5)

b.	Illustrate and discuss the interfacing an 8-bit D/A converter with the 8085.			  (10)

(OR)

21. 	Discuss the interfacing a keyboard and a seven segment LED using 8255A.

22. 	a.	Write output of the 8051 microcontroller program given below.

MOV A, #25H;

MOV R2, #34H;

ADD A, R2;										    							   (5)

b.	Discuss steps to create assembling and running an 8051 program.					  (10)

(OR)

23.	a.	Find the sum of the values 79H, F5H, E2H. Put the sum in registers R0 (low byte) and R5 	(high byte) using 8051 micro controller program. 								    (7)

b.	Discuss the various addressing modes in 8051.									    (8)

24.	Explain the role of 8051 in A/D conversion.

(OR)

25. 	Discuss about the interfacing of 8051 with keyboards and display devices.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	PROCESS DYNAMICS AND CONTROL			      	     Time : 3 hours

Subject Code:	10EI208								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Draw Piping and Instrumentation diagram for pneumatic and Hydraulic line?

2.	What is meant by Degrees of Freedom?

3.	What is Neutral Zone?

4.	What is integral windup?

5.	What is the need for controller tuning?

6.	What is ISE?

7.	What are the parts of a control valve?

8.	What is need for positioner in control valve?

9.	What is multivariable process control?

10.	What is the advantage of feed-forward control?

PART – B  (5 x 3 = 15 MARKS)

11.	Explain self regulating process with example.

12.	What is floating control mode?

13.	Explain damped oscillation method of tuning.

14.	Brief about Control valve sizing.

15.	Explain ratio control with example.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the Transfer function of a Two tank process connected Non - Interacting.

(OR)

17.	a. 	Explain about the modeling of temperature and gas systems.						  (10)

b. 	Draw P& ID symbol for instrument terms and symbols.							    (5)

18.	Explain Composite controller modes in detail.

(OR)

19.	a. 	Explain the response of controller for different test inputs.							    (6)

b. 	Explain about Proportional, Integral and Derivative controller modes				    (9)

20.	Explain process reaction curve method of tuning with formula.

(OR)

21.	Explain Ziegler – Nichol’s method of tuning in detail with formula.

22.	Write short notes on:

a.	Cavitation.																	    (5)

b.	Flashing	.																	    (5)

c.	Selection of control valve based on the inherent characteristics.					    (5)

(OR)

23.	a. 	Explain the working of control valve along with its constructional features.			  (10)

b. 	Explain about control valve with positioner.										    (5)

[P.T.O.]

24.	Explain the following with example.

a.	Cascade Control.																    (8)

b.	Split range control.															    (7)

(OR)

25.	a. 	Explain boiler drum level control.												    (7)

b. 	Explain chemical reactor control.												    (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	INDUSTRIAL INSTRUMENTATION				      	     Time : 3 hours

Subject Code:	10EI209								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	_________ pressure is the difference between total pressure and static pressure.

2.	Bellow type gauges are used to measure _________.

3.	The principle of electromagnetic flow meter is _________.

4.	Flow rate calibration depends on standards of _________ and _________.

5.	_________ is used to convert a temperature change into mechanical displacement and thus acts as a temperature sensor.

6.	The anemometer is used to measure the flow rate of fluids by measuring velocity of ________.

7.	Air purge system is also known as _________.

8.	Which type of rays is commonly used in radiation level detector?

9.	A solution which reached the limit of solubility is called as _________.

10.	When there is water vapour contained in the atmosphere, it is called as _________.

PART – B  (5 x 3 = 15 MARKS)

11.	Write down the formula for calculating the pressure in a Mcleod gauge.

12.	What is the principle of turbine flow meter.

13.	Define triple point.

14.	List the direct and indirect level measuring methods.

15.	State Henry’s law.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the testing and calibration of pressure gauge using dead weight tester.

(OR)

17.	Mention the different types of manometers and explain any two with neat sketch.

18.	a.	Pressure before orifice plate rises and pressure after it reduces but velocity increases-	Justify the statement. Describe its construction and Working in detail.				    (7)

b.	Draw the different types of orifice plates and explain how it is used for particular fluids.(8)

(OR)

19.	a.	Explain the type of mass flow meter which works under the principle of Newton’s Second 	law of angular motion. 														    (7)

b.	With a neat diagram explain about the construction and working operation of Electromagnetic flow meter and also discuss its advantages and limitations. 			    (8)

20.	Illustrate the construction and working of total radiation pyrometer.

(OR)

21.	Classify the different types of filled system thermometer in detail.

22.	a.	How liquid level is measured using float and displacer sensor? 					    (7)

b.	Explain about the operation of solid flow measurement with a neat sketch.			    (8)

(OR)

[P.T.O.]

23.	a.	Discuss electrical methods of level measurement. 								    (7)

b.	Explain how to measure level using ultrasonic method. 							    (8)

24.	Describe the working principle of Saybolt viscometer and also give their merits, demerits and applications.

(OR)

25.	Explain in detail about the densitometer with neat diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	LOGIC AND DISTRIBUTED CONTROL SYSTEMS	      	     Time : 3 hours

Subject Code:	10EI210								 			      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Draw the block diagram of DAS.

2. 	Expand the acronym SCADA.

3. 	Name the two I/O modules used in PLC hardware.

4. 	Name the different types of timers used in PLC.

5. 	What is meant by program scan in PLC?

6. 	Write the advantage of sequencer program over conventional program.

7. 	What is the function of LCU?

8. 	List the applications of DCS.

9. 	Write the two operational modes of HART.

10. 	CAN stands for ____________.

PART – B  (5 x 3 = 15 MARKS)

11. 	List the applications of DDC.

12. 	List the advantages of PLC over Relay logic.

13.	Draw ladder logic for flasher light.

14. 	Write any two functions of controller in DCS.

15. 	List the requirements of field bus.

PART – C  (5 x 15 = 75 MARKS)

16. 	Discuss in detail about:

a. 	SCADA						                                                						    (8)

b. 	Data Loggers					                                                            			    (7)

(OR)

17. 	Explain the working of direct digital control with relevant block diagram.

18.	a. 	Explain the architecture of PLC with a neat diagram.	                                                     (10)

b. 	Write short notes on analog input interface module.		                                    		    (5)

(OR)

19. 	With suitable example discuss the auxiliary, timing and counter functions.

20.	a. 	Discuss various DATA compare instructions with their ladder logic in PLC.	           	  (10)

b. 	Explain the function block programming for sequence control application. 			    (5)

(OR)

21. 	Discuss the various aspects relevant to installation practices, editing and troubleshooting of PLC.

22. 	Write short notes on Process interfacing issues and the concept of redundancy.

(OR)

23. 	a. 	List the general and performance requirements of a DCS communication system.	   (8)

b. 	What is sub network? List its advantages and disadvantages.                                  	   (7)

24. 	Explain the HART communication protocol and different communication modes.

(OR)

25. 	Describe the field bus concept with a block diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	BIOMEDICAL INSTRUMENTATION				      	     Time : 3 hours

Subject Code:	10EI211								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Electrical activity of the heart is analyzed through __________.

2.	What is resting potential?

3.	Define Heart rate.

4.	What is vital capacity?

5.	What are the two types of dialysers?

6.	What is the use of heart lung machine?

7.	Define pH.

8.	Name the various blood cell counters.

9.	Mention the advantages of CT over X ray.

10.	Draw simple model of a telemetry system.

PART – B  (5 x 3 = 15 MARKS)

11.	Name three bioelectric potentials generated in human body. Also mention how they are related to human physiology.

12.	What are the needs of physiological transducer?

13.	How ventilators useful in hospitals?

14.	What is the use of spectrometer?

15.	State the NMR principle.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail electrical activity of the heart with neat diagram.

(OR)

17.	Explain 10-20 electrode system EEG measurements. Also name various types of EEG signals and their significance.

18.	How do you measure lung volumes and capacities?

(OR)

19.	Explain about microwave diathermy.

20.	Explain the function of any one DC defibrillators in detail.

(OR)

21.	Explain in detail about patient safety from various micro and macro shocks in hospitals.

22.	With neat diagram explain the operation of Flame photometer and Spectrophotometer.

(OR)

23.	Discuss various typed and methods for blood cell counting.

24.	Explain the basic principle and various modes of Ultrasound scanning.

(OR)

25.	Explain the functions of Echocardiograph and Vector cardiograph.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DIGITAL CONTROL SYSTEMS					      	     Time : 3 hours

Subject Code:	10EI213											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define analog and digital signals.

2.	List any two types of analog to digital converters.

3.	State Z transform.

4.	List the steady state errors.

5.	Define Controllability.

6.	Compare the cascade and parallel realization methods.

7.	Define stability of digital control system.

8.	State digital quantization

9.	List any two applications of dead-beat algorithm.

10.	What is the need of microcontroller in control algorithms?

PART – B  (5 x 3 = 15 MARKS)

11.	State shannon's sampling theorem.

12.	Why is z-transform useful in analysis of digital control systems?

13.	Sketch the sampled data model for continuous system.

14.	Outline the state sequences for sampled data systems.

15.	Compare the dahlin's and kalman's algorithms.

PART – C  (5 x 15 = 75 MARKS)

16.	Enumerate basic elements of a digital control system and show the block diagram representation of such a system. Also discuss briefly about functioning these elements.

(OR)

17.	Illustrate the working of dual slope and successive approximation analog to digital conversion types.

18.	a.	Outline the relationship between the Laplace transform and z-transform.			    (5)

b.	Evaluate z-transform of the following functions:

f1t=sinωt

f2t=e−αtcosβt

(10)

(OR)

19.	a.	Define Pulse T.F.Also mention limitations of z-transform.							    (5)

b.	Enumerate steps involved in obtaining z-transform of a function f(t). Obtain z-transform of 	the following

f1t=e−αtsinβt

f2t=cosωt																  (10)

[P.T.O.]

20.	Define controllability and observability. Consider a digital control system described by input-output relation in form of following difference equation

c(k + 2) + 2c(k + 1) + c(k) =u(k + 1) + u(k) check if the above system is completely controllable.

(OR)

21.	Outline the design concepts of state feedback and output feedback control.

22.	Define stability of digital control systems. Enumerate various methods used for stability investigation of such systems. Why is R-H criterion is not directly applicable in stability analysis of such systems – explain.

(OR)

23.	Explain the Bilinear transformation and Jury's stability tests on digital control systems.

24.	Summarize the concepts of Dead-beat algorithm, Dahlin's and Kalman's algorithms.

(OR)

25.	a.	Outline the Implementation of control algorithms using microcontroller.			    (8)

b.	Illustrate the block diagram study of digital implementation.						    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December – 2015

Subject Title :	COMMUNICATION ENGINEERING				      	     Time : 3 hours

Subject Code:	10EI214								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define the modulation index and write its limit.

2.	Write the equivalence between PM and FM.

3.	List the commonly used filters for Amplitude modulation system.

4.	An amplifier operating over the frequency range from 18 to 20MHz has a 10KΩ input resistor. 	What is the rms noise voltage at the input to this amplifier if the ambient temperature is 27⁰C?

5.	Write an example for the application of PAM.

6.	List the types of shift keying techniques.

7.	A multimode step index fiber has a dispersion of 4ns/Km and is to be operated at a bit rate of 10Mbps. Find the bandwidth – distance product of the fiber.

8.	How do you correct the single bit error?

9.	Define the index of cooperation.

10.	Find the video bandwidth required for the US NTSC system.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the need of modulation for communication.

12.	Compare the AM, FM and PM modulation techniques.

13.	Draw the block diagram of delta modulator and explain the slope overload error.

14.	Draw and explain the parts of coaxial cable.

15.	Write the functions of modem.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Discuss the principle and operation of AM modulation and demodulation.			  (10)

b.	A standard AM transmission, sinusoidally modulated to a depth of 30%, produces side frequency of 4.928 and 4.914MHz. The amplitude of each side frequency is 75V. Determine the amplitude and frequency of the carrier.								    (5)

(OR)

17.	Compare the AM with SSBSC. Explain the methods of single sideband generation.

18.	Discuss the operation and the important blocks of FM transmitter and receiver system.

(OR)

19.	Discuss the types, causes and effects of the various forms of noise which may be created within a receiver or an amplifier.

20.	Explain the blocks and operation of PCM system.

(OR)

21.	Discuss the modulation and demodulation of PSK system.

22.	Explain the fiber optic system in detail.

(OR)

23.	Briefly explain the multiplexing techniques.

24.	Explain the TV receiver operation with its block diagram.

(OR)

25.	Explain: what is meant by geostationary orbit? Why there is only one such orbit? Calculate the minimum delay time for a signal transmitted from the geostationary satellite to reach the earth.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ELECTRICAL AND ELECTRONIC INSTRUMENTATION													      	     								      Time : 3 hours

Subject Code:	11EI201								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The use of ___________ instruments is merely confined within laboratories as standardizing

Instruments.

2.	___________ instruments are those which measure the total quantity of electricity delivered in a particular time.

3.	Resistances can be measured with the help of __________.

4.	An induction meter can handle current up to __________.

5.	Most common form of A.C. meters met with in every day domestic and industrial installations are __________.

6.	The function of shunt in an ammeter is to __________.

7.	Lissajous figure is used in CRO for __________.

8.	Input impedance of CRO is __________.

9.	Digital instrument works on the principle of __________.

10.	Magnetic deflection is inversely proportional to __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Compare Moving coil with Moving iron instruments.

12.	What is the working principle of megger?

13.	Sketch the resistive measurement circuit of electronic multimeter.

14.	Draw the internal structure of CRT and list its functions.

15.	Comparison between line printer and dot matrix printer.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the operating principle of PMMC type ammeter with a neat diagram. What special features are incorporated in its construction in order to reduce the errors?

(OR)

17.	Explain in detail about the working principle of D’Arsonval galvanometer and derive its torque equation.

18.	Explain the working principle of Anderson’s bridge and derive its balance equation.

(OR)

19.	Explain Kelvin’s double bridge method for the measurement of low resistance.

20.	Sketch and explain the functioning of vector impedance meter.

(OR)

21.	a.	Discuss the measurement of DC and AC voltages and currents using an Electronic 	Multimeter. 							     										    (8)
b.	Draw the block diagram of True RMS reading voltmeter and explain its operation.	    (7)

[P.T.O.]

22.	Explain the block diagram of a general purpose oscilloscope and also describe about the observation of waveform on CRO.

(OR)

23.	a.	With an example, interpret the importance of vertical and horizontal deflection system in 	CRO.								                                         					    (7)

b.	Describe the working of sampling oscilloscope.				                 			    (8)

24.	With neat sketch explain the operation of X-Y Recorders.

(OR)

25.	Explain in detail about Seven Segment Display system.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DIGITAL SIGNAL PROCESSING					      	     Time : 3 hours

Subject Code:	11EI202											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Give the convolution sum formula.

2. 	Check the linearity of the system

3. 	Impulse response of a FIR filter is h(n)={1,2}. Find H(z).

4. 	Write the non-causal Hanning window formula.

5. 	Draw the frequency response of elliptic filter.

6. 	.Find cut off frequency of the filter.

7. 	RISC stands for _________.

8. 	Write any two applications of P-DSPs.

9. 	Give any two shift instructions in TMS320C5X.

10. 	Write some examples for fixed point DSPs.

PART – B  (5 x 3 = 15 MARKS)

11. 	The signal . Find DTFT of x(n).

12. 	Summarize the advantages of FIR filters over IIR filters.

13. 	What is the warping effect? What is its effect on magnitude and phase response?

14. 	State the advantages and disadvantages of VLIW architecture.

15.	 Write a simple assembly language program in TMS320C5X to add any two positive numbers.

PART – C  (5 x 15 = 75 MARKS)

16. 	Derive the radix-2 DIF algorithm for N=8. Find the eight-point DFT of the sequence        	  using radix-2 DIF algorithm.

(OR)

17.  a.	A system is described by the difference equation Draw the 	frequency response of the system.								   				    (5)

b.	Find the linear convolution between .			   		    (5)     	c.	Determine the circular convolution between (5)           18. 	Determine the filter coefficients h(n) obtained by frequency sampling method.

The order of the filter N=7.

(OR)

[P.T.O.]

19. 	Design a filter with

20. Apply impulse invariant method to find H(z)

a)  assume T=0.2 sec.

b)  assume T=1 sec.

(OR)

21. 	Obtain the direct form I, direct form II, cascade and parallel form realization for the system given by

22. 	Discuss the special addressing modes P-DSPs.

(OR)

23. 	Explain the following in a DSP processor.

a.	Multiplier and accumulator unit.							 					    (5)

b.	Pipelining.																	  (10)

24. 	Draw the internal architecture of TMS320C5X with neat diagram and explain the operations     	of each block.

(OR)

25. Write the function of the following instructions in TMS320C5X with a simple example.  	Assume suitable initial data.

a.	BLPD 00h																	    (5)

b.	NORM *+																	    (5)

c.	LST #1, 00h																	    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	INDUSTRIAL DATA COMMUNICATION  AND NETWORKS													      	     							      Time : 3 hours

Subject Code:	12EI226								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is parity bit?

2. 	What is meant by nipple?

3. 	What is the maximum data rate is possible in RS422 interface standard?

4. 	What is USB?

5. 	Define flow control.

6. 	What are the two most common modes of operation for HDLC?

7. 	Classify field bus.

8. 	What is  allen bradley protocol?

9. 	What is the function of repeaters?

10. 	Differentiate between token bus and token ring.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the principle of communication?

12. 	Write the importance of IEEE 488.

13. 	What are the features of BSC Protocols?

14. 	Draw the Modbus message structure.

15. 	How media is accessed in token passing technique?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Discuss the various factors affect the Transmission speed.				     			    (6)

b.	Explain the operation of universal asynchronous receiver/transmitter.              		    (9)

(OR)

17.	 List and explain the most common data codes used in communication networks.

18. 	Explain in detail about:

a.	Balanced and unbalanced transmission lines.    							    		    (7)

b.	Interface to printers.								    							    (8)

(OR)

19.	Describe the working RS232 serial data communications interface standards and its mechanical characteristics.

20. 	Explain in detail about HDLC protocol and compare with SDLC?

(OR)

21.	Draw the OSI model and explain the functions performed by each layer of the Model?    Explain how data is transmitted using this model.

22.	a.	Explain the origin and benefits of the field bus systems.     				      		    (6)

b.	Explain the architecture of field bus communication.			                              	    (9)

[P.T.O.]

(OR)

23. 	a.	Explain the various communication modes of HART.     				      			    (7)

b.	Describe the layers of the HART protocol.			                                          		    (8)

24. 	a.	What is a router? Explain the various routing techniques.	      			     			    (8)

b.	Discuss the issues involved in the design of bridges.		     		                		    (7)

(OR)

25. 	What are the various Ethernet technologies? Explain in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	C++ AND DATA STRUCTURES						      	     Time : 3 hours

Subject Code:	11EI217											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Libraries for the input/output operations in C++ are included using the command __________.

2.	Logical operators are used to make decisions in a programming language like C++. State True or False.

3.	A sequence of character pointers is called a __________.

4.	Structure is a __________ data type.

5.	Return type of a default constructor is an object. State True or False.

6.	Inheritance allows to remove methods defined in the base class. State True or False.

7.	A train represents a singular linked list. State True or False.

8.	Queue is a __________ data structure.

9.	__________ is a standard procedure that is used to build a heap.

10.	File pointers are defined as string pointers. State True or False

PART – B  (5 x 3 = 15 MARKS)

11.	What is the need for nested IF structure?

12.	Is string a character array pointer? If so, give a brief explanation.

13.	What is multilevel inheritance?

14.	What is a binary tree?

15.	How are exceptions handled in C++ language?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the building blocks of a C++ program.									(7)

b.	Write a C++ program to print the Fibonacci series.								(8)

(OR)

17.	Write a C++ program to print the prime numbers ranging from 1 to n. Obtain the value of n as input from the user.

18.	Explain the array pointers and its application in C++ language with a suitable example.

(OR)

19.	Describe the process of overloading operators and functions in C++ language with suitable examples.

20.	Explain the different types of inheritance possible in C++ with suitable examples.

(OR)

21.	Explain the friend and virtual functions with suitable examples in C++ language.

22.	Write a C++ program to illustrate the use of circular linked list in order to solve a real world problem.

(OR)

23.	Explain the operations performed on the queue data structure using a sample C++ program.

24.	Describe the merge sort algorithm with a suitable C++ program.

(OR)

25.	Explain the concept behind the binary search algorithm with a sample C++ program.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	CIRCUIT ANALYSIS AND NETWORKS			      	     Time : 3 hours

Subject Code:	12EI201								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	If a resistor has 5.5 V across it and 3 mA flowing through it, what is the power?

2.	Determine total inductance of a parallel combination of 100 mH, 50 mH, and 10 mH

3.	Superposition theorem is valid only for __________.

4.	Three equal resistance of 3 Ω are connected in star. What is the resistance in one of the arms of the equivalent star circuit?

5.	How many cycles does a sine wave go through in 10 s when it frequency is 50 Hz?

6.	If the peak value of a certain sine voltage is 5 V, what is the rms value?

7.	What is the maximum value of the coefficient of coupling?

8.	How many minimum numbers of coils associated with the property of mutual inductance?

9.	The number of possible combinations generated by four variables taken two at a time in two-port network is __________.

10.	If Z11= 2 Ω; Z12= 1 Ω;Z21= 1 Ω andZ22= 3 Ω, what is determinant of admittance matrix?

PART – B  (5 x 3 = 15 MARKS)

11.	State Kirchhoff’s voltage law and current law.

12.	Write short notes on voltage and current source transformations.

13.  	What is damping factor and write the equation for damping factor.

14. 	Define single tuned and double tuned circuits.

15. 	Differentiate the low pass and high pass filter.

PART – C  (5 x 15 = 75 MARKS)

16. 	For the circuit shown, find (a) i1 (b) V1 and (c) power supplied by the 3V battery.

(OR)

17.	Find the current through 8 Ω branch using mesh analysis.

[P.T.O.]

18. 	With neat sketch derive the equation for delta to star transformation.

(OR)

19.	Find the current through 5 ohm resistance using superposition theorem.

20.	a.	A sinusoidal voltage is applied to the circuit shown in fig. Determine rms current, average

current, peak current and peak to peak current. Derive the expressions used. 			    (8)

b.	Derive an expression for step response of series R-C circuit.						    (7)

(OR)

21. 	An alternating current varying sinusoidally, with a frequency of 50 Hz has an rms value of 20 A. Write down the equation for the instantaneous value and find this value at (a) 0.0025 s, and (b) 0.0125 s after passing through a positive maximum value. At what time, measured from a positive maximum value will the instantaneous current be 14.14A?

22.	a.	Prove that, for a coupled circuitM=kL1L2.									    (8)

b.    A series RLC circuit is connected to a 220V, 50Hz supply, when L is varied, the maximum

current obtained is 0.4 Amps and the voltage across the capacitor then is 330V.  Find the

circuit constants. 																    (7)

(OR)

23.	a.	Derive the inductance value in two coils coupled in parallel both for aiding and opposing  	connections.																	    (8)

b.	When two coils are connected in series their effective inductance is found to be 10 H. 			When the connections of one coil are reversed, the effective inductance is 6 H. If the co-			efficient of coupling is 0.6, calculate the self inductance of each coil.				    (7)

24.	Explain about

a.  	Reciprocal network. 															    (4)

b. 	Symmetrical network.															    (4)

c. 	Obtain h-parameters of the network shown in Figure.								    (7)

(OR)

25.	Calculate the y-parameters for the network shown in Figure.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December -2015

Subject Title :	ELECTRICAL MACHINES							      	     Time : 3 hours

Subject Code:	12EI202							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the Torque equation mentioning all the variables.

2.	Define statically and dynamically induced E.M.F.

3.	What is a step-up transformer?

4.	Give the principle of transformers.

5.	Define Slip of an induction motor.

6.	Give any two applications of synchronous motor.

7.	What is Tacho-generator?

8.	Mention any two applications of Stepper Motor.

9.	Draw the symbol of Electrical Heater.

10.	List the methods of Electrical Heating.

PART – B  (5 x 3 = 15 MARKS)

11.	Describe the torque equation of DC generator.

12.	List out the differences between Shell and Core transformer.

13.	Write and explain the EMF equation of synchronous motor.

14.	Draw and explain the working principle of stepper motor.

15.	List out the recent trends in Electric traction.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss how a D.C generator builds up E.M.F.

(OR)

17.	Draw and explain the characteristics of D.C. shunt and series motor.

18.	Explain how the efficiency of the transformer may be found from the open circuit and short circuit tests.

(OR)

19.	a. 	Describe the constructional features of any one type of single phase transformer. 	    (7)

b. 	A 600 kVA single phase transformer has an efficiency of 94% both at full load and half load at unity power factor. Determine the efficiency at 75% of full load at 0.9 power factor.

(8)

20.	Derive the Torque Equation and explain the Torque Slip characteristics of a single phase induction motor.

(OR)

21.	Describe the constructional features of a Synchronous Motor with Construction and Principle of operation and derive the EMF equation with proper sketch.

22.	Explain the following types of motors.

a.	Repulsion type motor.

b.	Servo motor.

(OR)

23.	What is Double field revolving theory and derive the equations?

24.	Draw and explain the various methods of electric heating in detail.

(OR)

25.	Draw and explain the working principle of Welding Generator.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ELECTRONIC CIRCUITS							      	     Time : 3 hours

Subject Code:	12EI203							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The output frequency of a full-wave rectifier is ________ the input frequency.

2.	What type of diode circuit is used to add or restore a dc level to an electrical signal?

3.	A transistor amplifier has a voltage gain of 100. If the input voltage is 75 mV, the output voltage is________.

4.	The cutoff region is defined by IB ________ 0 A.

5.	A certain amplifier has a bandwidth of 22.5 kHz with a lower cutoff frequency of 600 Hz. What is the value of fcu?

6.	The main feature of a large-signal amplifier is the circuit's ________.

7.	Only the condition βA= ________ must be satisfied for self-sustained oscillations to result.

8.	Which of the following improvement is a result of the negative feedback in a circuit?

9.	To which BJT amplifier, a common-source amplifier is similar in configuration?

10.	For what value of ID is gm equal to 0.5 gm0?

PART – B  (5 x 3 = 15 MARKS)

11.	Define Line regulation and load regulation.

12.	Draw the low frequency and high frequency model of BJT.

13.	Give the effect of negative feedback on amplifier characteristics.

14.	State Barkhausen criterion.

15.	List the different types of biasing in FET amplifiers.

PART – C  (5 x 15 = 75 MARKS)

16.	Sketch and explain the working of full wave rectifier circuit and derive its expression.

(OR)

17.	With a neat sketch, discuss how the Zener diode works as a voltage regulator.

18.	Derive the expression for h parameters of BJT amplifier.

(OR)

19.	Analyze the different types of biasing techniques applied for transistors.

20.	Describe the working of class AB power amplifier with its merits and demerits.

(OR)

21.	Draw the circuit diagram of voltage series feedback amplifier and derive the expression for input and output impedances.

22.	Draw and explain the working principle of RC phase shift oscillator. Also derive the expression for frequency of oscillation and condition for sustained oscillation.

(OR)

23.	Discuss about the one shot multivibrator circuit and derive the expression for time period.

24.	Derive the expression for small signal model of FET circuits.

(OR)

25.	Draw and explain the function of low frequency model of common drain amplifier with its necessary equation.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	THERMODYNAMICS AND FLUID MACHINERY      	     Time : 3 hours

Subject Code:	12EI204								 			      Maximum Marks: 100

#### Answer ALL questions

(Use of Steam tables,Heat and Mass Transfer Data book is permitted)

PART – A  (10 x 1 = 10 MARKS)

1.	What is a thermodynamic system?

2.	What is a steady flow process?

3.	What are three basic modes in which heat is transferred?

4.	What is a Stefan-Boltzmann Law?

5.	Define boiler efficiency.

6.	Mention two types of heat exchanger.

7.	Define specific weight.

8.	Define the term centrifugal pump.

9.	Define the term incompressible flow.

10.	Define impulse turbine.

PART – B  (5 x 3 = 15 MARKS)

11.	Write short note on Clausius’ statement.

12.	How do you understand by natural convection and forced convection?

13.	How boilers are classified?

14.	List the different types of fluids.

15.	Define the term discharge.

PART – C  (5 x 15 = 75 MARKS)

16.	Air flows steadily at a rate of 0.5 kg/sec through an air compressor, entering at 7 m/sec,          100 kPa pressure and 0.95 m3/kg volume and leaving at 5 m/sec, 700 kPa pressure and 0.19 m3/kg. The internal energy of the air leaving is 90 kJ/kg greater   than that of the air entering. Cooling water in the compressor jackets absorbs from the air at the rate of 58 kW. (a) Compute the rate of shaft work input to the air in kW. (b) Find the ratio of the inlet pipe diameter to the outlet pipe diameter.

(OR)

17.	Write short notes on

a.	First law of thermodynamics,

b.	Kelvin-Planck statement of second law and

c.	Distinguish between the term path, process and cycle.

18.	Derive the one dimensional steady state heat conduction equation in plain wall.

(OR)

19.	Water flows through a 22 mm internal diameter tube at a velocity of 2 m/sec. The water is heated from 15˚C to 60˚C by condensing the steam at 150˚C on the outer surface of the tube. Using the equation.

Nu = 0.023 (Re)0.8 (Pr)0.4

Find out the heat transfer coefficient.

[P.T.O.]

20. 	An oil cooler for a lubrication system has to cool 1000 kg/hr of oil (Cp = 2.09 kJ/kg-K) from  80˚C to 40˚C by using a cooling water flow of 1000 kg/hr available at 30˚C. Give your choice for a parallel flow or counter flow heat exchanger, with reason. Estimate the surface area of heat exchanger, if the overall heat transfer coefficient is 24 W/m2 K.

(Cp of water = 4.18 kJ/kg-K)

(OR)

21. 	Explain the procedure to conduct a boiler trial and to draw up the heat balance sheet.

22. 	Explain the working principle of reciprocating pump with a neat sketch.

(OR)

23. 	A U-Tube manometer is used to measure the pressure of water in a pipe line, which is in excess of atmospheric pressure. The right limb of the manometer contains mercury and is open to atmosphere. The contact between the water and mercury is in the left limb. Determine the pressure of the water in the main pipe, if the difference in the level of mercury in the limbs of U-tube is 10 cm and the free surface of mercury is in the level with the centre of the pipe. If the pressure of water in pipe line is reduced to 9810 N/m2, calculate the new difference in the level of mercury. Sketch the arrangements in both cases.

24. 	A Pelton wheel is to be designed for the following specifications:

Shaft power = 11772 kW; Head = 380 m; Speed = 750 r.p.m.; Overall efficiency = 86%; Jet diameter is not to exceed one-sixth of the wheel diameter. Determine;

(i) The wheel diameter, (ii) Diameter of the jet and (iii) The number of jets required.

(OR)

25. 	Describe the principles and working of a Kaplan turbine with a neat sketch.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ELECTRICAL MEASUREMENTS					      	     Time : 3 hours

Subject Code:	12EI208											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.   What are the basic requirements of any measurement?

2.   State the major categories of error.

3.   What is the function of counter weight in an instrument?

4.   Define calibration?

5.   What type of movement is used for Iron vane ammeter?

6.   Mention the types of power factor meters.

7.   Define the term null as it applies to bridge measurement.

8.   What two conditions must be satisfied to make an AC bridge balance?

9.   List typical frequency response of a strip chart recorder.

10. What is galvanometer recorder?

PART – B  (5 x 3 = 15 MARKS)

11.  Compare the terms accuracy and precision of an instrument.

12.  What are the requirements of a shunt?

13.  Why an electrodynamometer is called a square law device?

14.  Compare Wheatstone bridge and Kelvin’s bridge.

15. 	What is the difference between a strip chart recorder and an x-y recorder?

PART – C  (5 x 15 = 75 MARKS)

16.  Explain about the static and dynamic characteristics of an instrument.

(OR)

17.  Explain the various functional elements of an instrument.

18.	a.	Explain the principle of operation of permanent magnet moving coil type Instrument.  (8)

b.	Explain the calibration of series type ohm meter.		                                  			    (7)

(OR)

19.  Explain the operation of a simple multimeter with necessary diagrams.

20.	a.	With a neat sketch explain the working of dynamometer type wattmeter.               (10)

b.	Write short notes on Instrument transformer.                                                       	    (5)

(OR)

21.	a.	Draw the neat sketch of a single phase energy meter and explain its operation.    		  (10)

b.	Explain the operation of dynamometer type power factor meter.                              	    (5)

22.	a.	Explain in detail about Maxwell bridge.                                                                    	    (8)

b.	A balanced AC bridge has the following constants.                                                 	    (7)

Arm AB = R  1 kΩ in parallel with C  0.047 µF,

Arm BC = R  2 kΩ in series with    C  0.047 µF,

Arm DC = Unknown.

Arm DA = C 0.25 µF.

The frequency of oscillation is 1KHz. Determine the constants of the arm CD.

(OR)

[P.T.O.]

23.	a.	Explain with neat diagram Wagner’s ground connection.		         		 		  (10)

b.	What are the precautions to be taken when using a bridge for measurement?  	 	    (5)

24.	a.	Explain the basic elements of a magnetic tape recorder.                                        	 	(8)

b.	Write notes on XY recorder.                                                                                     	(7)

(OR)

25.   Explain about the various types of potentiometric recorders.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	SIGNALS AND SYSTEMS							      	     Time : 3 hours

Subject Code:	12EI209							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Sketch the signal x[n] = { 2,3,4,1,0,3,4}

2.	x(t) = sin(2t) is a __________ signal.(Odd/Even)

3.	y[n] = x[n] * h[n]. If x[n] is a DT sequence with 5 samples, and h[n] has 4 samples, then the length of y[n] is __________.

4.	The order of the DT system represented by the equation y[n] = x[n] + 2 x[n-1] - x[n-3] + y[n-1] is __________.

5.	State Parseval’s theorem for continuous time periodic signal.

6.	Determine the Discrete Time Fourier Series (DTFS) of x[n] = [n].

7.	If the CTFT of x(t) is X(jΩ), then the CTFT of x(-t) is __________.

8.	State Sampling theorem.

9.	The Laplace transform of a unit step function is __________.

10.	Define ROC of Z transform.

PART – B  (5 x 3 = 15 MARKS)

11.	Test the periodicity and find the period of the DT signal x[n] = 10cos(16πn)

12.	Determine the homogeneous solution of the system represented by the differential equation

d2y(t)dt2+3dy(t)dt+2y(t)=x(t)

13.	State Dirichlet’s condition for existence of Fourier series.

14.	Determine the DTFT of the DT signal x[n] = 3n u[n]

15.	Determine the Z transform of x[n] = 0.5n u[n].

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Let x(n)be a signal with x(n)=n2+n0

Find y(n)in graphical representation.

(i) y(n)=x(n)		(ii) y(n)=x(n+1)

(iii) y(n)=x(2n+1)+x(2−2n)    											    (7)

b. 	Determine whether the system  y(t) = 14 x(t) +3 is

i)	Static or Dynamic   	ii)  	Linear or Non-linear 		iii)  Shift invariant or variant

iv) 	Causal or Non- causal  	v) 	Recursive or Non-Recursive     				    (8)

(OR)

17.	a.	Plot x (2t-4) and x (0.5 – (t/3)) if

x(t)	= 1,		          -1 < t < 0

= e-t,	             t   0

= 0, otherwise.												    (8)

b. 	Test the properties of the system  y(n) = x(-n-2)+y[n-1]                                                    (7)

[P.T.O.]

18.	a.	Determine the overall response of the systems described by the differential equation  			 	d2y(t)dt2+6dy(t)dt+8y(t)=2x(t); y(0) = -1; dy(t)dt|t=0= 1, x(t) = e-t u(t).                     (8)

b. 	An LTI system is characterized by its impulse response h[n] = {2,1,3,4}. Determine the response of the system to an input x[n] = {1,2,0,2} graphically.                         	     (7)

(OR)

19.	a.	Perform convolution of the following signals x(t) = e-2t u(t); h(t) = e-3t u(t). 			    (7)

b. 	Determine the overall response of the system represented by the equation

y[n] – ½ y[n-1] = 2x[n], y[-1] =3,x[n] = (-1/2)n u[n]. 								    (8)

20.	Determine the Fourier series representation of periodic square wave defined over one period

and draw the magnitude response.

x(t) 	  = 		1;	 |t| < T1

0; 	T1<|t|< T2

(OR)

21.	a  . State and prove any four properties of CTFS.         								  	  (10)

b.   Find the Discrete Fourier Series of

x[n] = 1+ sin (2π/5)n + 3 cos(	2π/5)n+ 	cos(	4π/5	 n +4π/2)	   						     (5)

22.	a. 	Consider a CTLTI system described by dy(t)dt+2y(t)=x(t).  Using FT find output y(t) to 	the input signal x(t) = e –t  u(t)   	                                                                                  (7)

b. 	Determine the DTFT of x[n] = cosπ2n u[n]		                                                     (8)

(OR)

23.	a. 	State and prove sampling theorem.												  (10)

b. 	Find the CTFT of x(t) = e-at u(t)												    (5)

24.	Determine the inverse Z transform of X(z) = 11+0.8z−1+0.12z−2 if ROC is

a.	z>0.6			b.	z<0.2		c.	0.2< z<0.6

(OR)

25.	Using Z transform, find the impulse response and step response of the DT system

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	CONTROL SYSTEM								      	     Time : 3 hours

Subject Code:	12EI210						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	List the components of the closed loop control system.

2. 	Find the closed loop gain of the system shown in fig.

3. 	Mention the nature of system, when the roots of second order system are real and unequal.

4. 	Find the damping factor for a unity feedback system with function Gs=1ss+1.

5. 	Define rise time.

6. 	The steady state error of the first order system subjected to unit step input is ____________.

7.  	Mention the techniques used to analyze system in frequency domain.

8. 	Define gain cross over frequency.

9.  	Analyze the location of poles in s-plane and State whether the system if Gs=1s+3s+1.

is stable.

10. 	Centroid of the open loop system with unity feedback Gs=1s+1s+7s+13.

PART – B  (5 x 3 = 15 MARKS)

11. 	Find the transfer function of the given feedback control system.

12. 	State the effect of a PI controller on the system response?

13. 	Calculate the peak overshoot of the system with damping factor=0.316, for the step input of 10 units.

14. 	Calculate the acceleration error constant for the open loop unity feedback system

Gs=10s2s+3s+1

15.  Verify the stability of the system with characteristic equation,  S3+3 S2+2 S+3 = 0

[P.T.O.]

PART – C  (5 x 15 = 75 MARKS)

16.  Using Mason’s gain formula obtain the transfer function of the given system.

(OR)

17. 	Obtain the electric equivalent network for the given rotational system using Torque voltage analogy.

18.	Determine the transfer function of the given system:

(OR)

19. 	Derive the expression for response of second order under damped system with unit step input.

20. 	A unity feedback system expressed by its open loop transfer function.

Gs=kss+10.	Determine the gain K, so that the damping factor is 0.5.

Determine the settling time, peak overshoot, and peak time.

(OR)

21. A unity feedback system having open loop transfer function Gs=10s+2s+1,  determine 	steady state error, and static error constants.

22. 	Sketch the bode plot and obtain the frequency domain analysis of

Gs=30s0.25s+10.0125s+1.   Obtain the gain and phase cross over frequency.

(OR)

23. 	Apply Routh criterion and examine the stability, for a system with characteristic equation

F(s) = S6 + 4 S5+4 S4+6 S3+5 S2+3 S+2 = 0.

24.	Sketch the root locus of the system whose open loop transfer function is

G(s) = K / s (s+2) (s+3).

(OR)

25. 	Obtain the state model of system given as Y(s)/U(s)= 10/[ S3+4 S2+2 S+1].

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DIGITAL ELECTRONICS							      	     Time : 3 hours

Subject Code:	12EI211							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a digital-to-analog converter?

2.	Digital systems have _____________ states.

3.	The invalid range for an input to TTL logic is from _____________.

4.	In the decimal numbering system, what is the MSD?

5.	Give the decimal value of binary 10010.

6.	The output of an two input exclusive-NOR gate is 1. What is the correct input combination of the input?

7.	Demultiplexing is a logic function that _____________.

8.	What kind of logic device or circuit is used to store information?

9.	Which of the following is a semiconductor memory?

10.	PALs are relatively simple integrated circuit devices. Write True or False

PART – B  (5 x 3 = 15 MARKS)

11.	How schottky transistors are formed and state its use.

12.	Show that (X + Y' + XY)( X + Y')(X'Y) = 0.

13.	Explain the design procedure for combinational circuits.

14.	Explain the flip-flop excitation tables for JK flip-flop.

15.	Write short note on shared row state assignment.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain with neat diagrams TTL.

(OR)

17.	Explain with necessary diagrams MOS & CMOS.

18.	Design a logic circuit to convert the BCD code to Excess – 3 code.

(OR)

19.	Prove that ABC + ABC' + AB'C + A'BC = AB + AC + BC using Karnaugh map.

20.	Implement F(A,B,C,D)= (1,3,4,11,12,13,14,15) using multiplexer.

(OR)

21.	Design a 4-bit binary Adder/Subtractor circuit.

22.	Explain the working of BCD Ripple Counter with the help of state diagram and logic diagram.

(OR)

23.	Explain in detail about serial in serial out shift register.

24.	Implement W (A,B,C,D) = (2,12,13).

(OR)

25.	Explain in detail about RAM and ROM.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	C++ AND DATA STRUCTURES						      	     Time : 3 hours

Subject Code:	12EI212											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A linear collection of data element given by mean of a pointer is called __________.

2.	A data structure where elements can be added or removed at either end but not in the middle is called __________.

3.	Merge sort uses __________ method.

4.	State True of False. Insertion sort is efficient for small data sets.

5.	State True of False. Friend function is not the member of a class.

6.	__________ are contiguous data elements with a single name.

7.	State True of False. Multiple inheritance is not possible in C++ language.

8.	A pointer variable can store the __________ of another variable.

9.	Error in opening a file returns __________ value to the file pointer.

10.	State True of False. this pointer points to the current object.

PART – B  (5 x 3 = 15 MARKS)

11.	Give any three applications of stack.

12.	When does quick sort take more time to sort the given data?

13.	How many instances of an abstract class can be created?

14.	What are the pitfalls of operator overloading?

15.	List the stream errors in C++ language.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain with an example the creation of a linked list, insertion and deletion of nodes and swapping of any two nodes.

(OR)

17.	Describe circular queue implementation in detail giving all the relevant features.

18.	Explain the heap sort technique with a suitable example.

(OR)

19.	Give an algorithm for Quick sort and explain its time complexity. Trace the algorithm for the following data:

65	75	70	80	85	60	55	50	45

20.	Write a C++ program to reverse a string defined with-in classes.

(OR)

21.	With the help of a simple program explain how constructors are overloaded in C++ language.

22.	Explain the method of overriding member functions using inheritance with a sample C++ program.

(OR)

23.	Explain the need for defining pointers to other pointer variables using a suitable C++ program.

24.	Explain the need for defining friend and virtual functions in C++ programs.

(OR)

25.	Explain the development of function templates and class templates with a suitable C++ program.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	COMMUNICATION ENGINEERING				      	     Time : 3 hours

Subject Code:	12EI215								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The carrier amplitude after AM varies between 4 V and 1 V. Calculate depth of modulation.

2.	Modulation index in FM: m = __________.

3.	What is the bandwidth required for an FM signal in which the modulating frequency is 2 kHz and maximum deviation is 10 kHz.

4.	When the AM equals FM?

5.	In pulse modulation systems of PAM and PPM, which one is, required higher bandwidth.

6.	Write the relationship between the minimum bandwidth required and bit rate of FSK system.

7.	State Snell’s law.

8.	What is multiplexing?

9.	Define aspect ratio.

10.	Geosynchronous satellites have an orbital time of approximately __________ hours.

PART – B  (5 x 3 = 15 MARKS)

11.	A broadcast transmitter radiates 20 kW when the modulation percentage is 75. Calculate carrier power and power of each sideband.

12.	Compare amplitude modulation and frequency modulation.

13.	Give short notes on: Data MODEM.

14.	Give short notes on: Twisted pair cables.

15.	What if frequency reuse in cellular communication, explain in detail.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the generation of FM using Ring modulator. 								    (6)

b.	Derive an expression of AM envelope.											    (9)

(OR)

17.	a.	Explain the generation of SSBSC using Filter method.							    (7)

b.	Explain the phase shift method to generate SSB signal.							    (8)

18.	Explain in detail about super heterodyne receiver.

(OR)

19.	Explain in detail about AM Transmitters.

20.	a.	Explain the principle operation of PCM.											    (9)

b.	Compare FSK, ASK and PSK.													    (6)

(OR)

21.	Explain the principle operation of delta modulation.

22.	Explain in detail about general optical communication system.

(OR)

23.	a.	Explain in detail about TDM system.											    (9)

b.	Explain about the optical sources.												    (6)

24.	Draw and explain the function of colour TV Receiver.

(OR)

25.	Explain in detail about basic Satellite system.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	SIGNAL CONDITIONING CIRCUITS				      	     Time : 3 hours

Subject Code:	12EI216								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.    If an op-amp is specified as being compensated, what does this mean?

2.    What is the output voltage?

3.	Division is __________of multiplication.

4.	What is the difference between clipper and clamper?

5.	List some features of instrumentation amplifier.

6.	What are the two types of filters?

7.	Series regulators use   a __________transistors connected in series between the unregulated dc

input and the load.

8.	Draw the pin diagram of IC723 general purpose regulator.

9.	State any two applications of IC 555.

10.	The voltage to frequency converter (Kv) of VCO is defined as _________.

PART – B  (5 x 3 = 15 MARKS)

11. 	Write the function of active load.

12.	For the non inverting op-amp shown in the figure below, find the output voltage Vo with v1=1V, v2=2V, v3=3V,R=1k, R1=1k and Rf=10k.

13.	What do you mean by universal filter?

14.	What are the advantages of IC voltage regulators?

15.	Give three applications of a comparator.

PART – C  (5 x 15 = 75 MARKS)

16.	List and explain the non-ideal DC characteristics of an operational amplifier.

(OR)

17.	What is a current mirror?  Explain the concept of Wilson current source used in op-amp with a neat diagram and necessary equations.

[P.T.O]

18.	a. 	Derive the output expression of an integrator and discuss the frequency response. 	    (7)

b. 	Explain any two application of comparator with neat sketch.						    (8)

(OR)

19. 	Briefly explain the following:

a. 	Antilog amplifier																    (8)

b. 	Sample and hold circuit														    (7)

20.	a. 	Explain Instrumentation amplifier with a neat circuit diagram.						    (8)

b. 	Write short note on isolation amplifier.											    (7)

(OR)

21.	a. 	Design a second order Butterworth low-pass filter having cut-off frequency 1kHz. 	  (10)

Determine its frequency response.

b. 	With suitable diagrams, explain the working of a switched capacitor filter. Also explain 	how a resistor can be realized using switched capacitor filter.						    (5)

22.	a. 	Explain the working of a 723 general purpose regulator.							    (8)

b. 	Discuss the working of series op-amp voltage regulator with neat diagram.			    (7)

(OR)

23.	Describe in detail how multiplication leads to doubling.

24. 	Explain how IC 555 can be used as a mono-stable multivibrator. Derive an expression for its

pulse width.

(OR)

25. 	For PLL IC NE/SE 565, derive the expression for lock in range and capture range.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MICROPROCESSORS AND MICROCONTROLLERS    	     Time : 3 hours

Subject Code:	12EI217								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	How many flags are there in 8085 architecture?

2.	How many T states are required for executing MOV M, A instruction?

3.	What is the purpose of an ALE signal?

4.	What is an interrupt I/O?

5.	How many ports in 8255?

6.	List the two modes of operation of 8255 Programmable Peripheral Interface

7.	Define OUT 8-bit port address instruction

8.	How many bytes of internal RAM and ROM supported by 8051 micro controller?

9.	The architecture of 8051 consists of
a) 4 latches.
b) 2 timer registers.
c) 4 on-chip I/O ports.
d) all of the mentioned.

10.	Define port.

PART – B  (5 x 3 = 15 MARKS)

11.	What do you mean by 8-bit and 16-bit processor? Mention some systems in each type.

12.	What is DMA and explain the need for DMA data transfer.

13.	Give the differences between Timer and Counter.

14.	What are the types of Addressing modes included in 8051?

15.	Give the applications of Microcontroller in Medical Field.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe about the architecture of 8085 Microprocessor and its functional blocks in detail with diagram.

(OR)

17.	Explain about opcode fetch, memory read and memory write machine cycle with Timing diagram.

18.	Discuss about the Instruction format and Addressing modes of 8085 Microprocessor.

(OR)

19.	Explain about a typical DMA controller with a neat block diagram.

20.	With a neat sketch explain the functions of 8255 PPI.

(OR)

21.	Explain in detail about Stepper Motor control using 8085

22.	Draw and explain the architecture of 8051 Microcontroller.

(OR)

[P.T.O.]

23.	Explain how ADC 0801 is interfaced with 8085 microprocessor.

24.	a. 	Describe in brief the interfacing method for hex keyboard with 8051 Microcontroller.

b. 	Explain how DAC are interfaced with 8051 with relevant interface diagrams.

(OR)

25.	a.	Elaborate on Interrupts. What are the various types of Interrupts in 8051? Discuss with an 	example.

b. 	Write an assembly language program to multiply any two numbers using 8051.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	PROCESS DYNAMICS AND CONTROL			      	     Time : 3 hours

Subject Code:	12EI219								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In the process control system, the controller output is given to __________.

2.	If three storage tanks connected in non-interacting fashion the effective order of the system is __________.

3.	The proportional gain of a PID controller can be expressed in terms of its proportional band (PB) as  __________.

4.	In boiler drum level control the controlling variable is __________.

5.	ISE criterion is used to suppress __________.

6.	The primary variable in cascade controller is __________.

7.	Which control scheme does not require plant model, Model reference adaptive control or Split range control?

8.	Rangeability of control valve is given by __________.

9.	A part of accumulated liquid which is returned to column is __________.

10.	In which point the vapour feed first starts to condense __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the block diagram of typical process control loop and mention the elements and variables associated in it.

12.	What is offset in proportional mode?

13.	State the conditions for selecting IAE, ISE and ITAE criteria.

14.	What is inferential control scheme?

15.	What is the function of Re-boiler in Distillation Equipment?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Derive the transfer functions H1(s)/Q(s) and H2(s)/Q(s) for the liquid level system shown 		in figure.																	(8)

[P.T.O.]

b.	Explain briefly about the processes with inverse response.							    (7)

(OR)

17.	a.	Obtain for the level process shown in figure, the transfer function relating h2 and other 	parameters. The area of cross section of tank I is A1 and that of tank II is A2. 		    (8)

b.	With neat sketch explain the process of self regulating processes.					    (7)

18.	List out the steps for the selection of control modes for processes for Pressure, 	Temperature, Level and Flow.

(OR)

19.	Explain briefly about Electronic PID controller with neat circuit diagram

20.	a.	Explain briefly about Electronic PID controller with neat circuit diagram.			  (10)

b.	Illustrate with neat sketch explain the process of Ziegler-Nichols Method for tuning 	controllers.																	    (5)

(OR)

21.	Describe the process reaction curve method for tuning controller parameters.

22.	a.	Explain with a sketch the working of a pneumatic actuator with Positioner.			    (7)

b.	Discuss about inherent and installed characteristics of control valve.				    (8)

(OR)

23.	a.	Explain briefly about I/P Converter.											    (7)

b.	Briefly explain cavitations and flashing in control valve.							    (8)

24.	Illustrate with an example the process of cascade control and Ratio control.

(OR)

25.	a.	Discuss the control schemes employed in heat exchangers.						    (7)

b.	With a neat sketch explain the case study for Boiler drum level control.				    (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ELECTRONIC INSTRUMENTATION				      	     Time : 3 hours

Subject Code:	12EI223								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List out the different connections in Q-meter.

2.	The __________ power means the energy which flows in an a.c. circuit but is not getting dissipated in the load.

3.	Name any four display devices.

4.	Mention the main parts of CRT.

5.	A Colpitt Oscillator uses __________ capactiors __________ inductors in the feedback path.

6.	A __________ is an instrument designed to measure relative amplitudes of a single frequency components in a complex waveform.

7.	The accuracy of frequency counter depends on __________.

8.	Integrating type digital voltmeters are classified as __________.

9.	A history of virtual instrumentation is characterized by continuous increase of __________ and __________ of measurement equipment.

10.	The sensor interface is classified as __________ and __________ interface.

PART – B  (5 x 3 = 15 MARKS)

11.	The self capacitance of a coil is to be measured by Q meter. The first measurement result is f1=1.5 MHz and C1=550 pF. The second measurement result is f2=3 MHz and new value of tuning capacitor is 110 pF. Find the distributed capacitance and the inductance.

12.	List out the advantages of Sampling Oscilloscope.

13.	What is the dynamic range of a spectrum analyzer with a third-order intercept point of +25dBm and a noise level of -85dBm?

14.	What are the precautions must be taken to obtain maximum frequency?

15.	Define Virtual Instrumentation.

PART – C  (5 x 15 = 75 MARKS)

16.	Summarize   the working principle of a Vector voltmeter with the help of a neat  block   diagram.

(OR)

17.	Define Q-Meter. Explain the working principle of Q meter and discuss the different connections.

18.	Explain with a neat block diagram the function of Cathode ray oscilloscope and also define deflection sensitivity of a CRT.

(OR)

19.	Explain how the waveform is stored in Digital storage oscilloscope.

20.	Design a spectrum analyzer (block diagram only) given the following parameters:

Frequency range: 200 MHz to 500 MHz; I IF : 400 MHz, RBW – 10 KHz;    II IF 25 MHz, RBW – 100 Hz. Identify the type of filters, bandwidth of filters and frequency of local oscillators.

(OR)

[P.T.O.]

21.	With the necessary block diagrams explain the function of Frequency Synthesizer.

22.	Explain in detail about digital frequency counter with neat block diagram.

(OR)

23.	Discuss in detail about

a.	Gating error

b.	Time base error

c.	Trigger level error

and explain how the maximum accuracy achieved for frequency measurement?

24.	Why is LabVIEW Ideal for Creating Virtual Instruments?

(OR)

25.	How virtual instrumentation used in the Engineering process?

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DIGITAL SIGNAL PROCESSING					      	     Time : 3 hours

Subject Code:	12EI224											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The _________ is called periodic convolution.

2.	The process of recovering the input from the response of a system is called _________.

3.	In _________ butterfly diagram the _________ is multiplied after add-subtract operations.

4.	The number of complex additions and multiplications in radix-2 FFT are _________ and _________ respectively.

5.	In impulse invariant or bilinear mapping the poles on the imaginary axis in S –plane are mapped into the _________ in Z-plane.

6.	The two properties which are preserved in analog to digital transformation are _________ and _________.

7.	The oscillation developed due to truncation of impulse response is called _________.

8.	In FIR filters with constant phase delay the impulse response is _________.

9.	The _________ architecture employs separate buses for program and data.

10.	The TMS320C5x processors have a total memory space of ___________.

PART – B  (5 x 3 = 15 MARKS)

11. 	List the basic set of operations on signals.

12. 	What is phase factor or twiddle factor?

13. 	What is frequency warping?

14. 	What is Gibb’s phenomenon?

15. 	Distinguish between Von Neumann and Harvard architecture.

PART – C  (5 x 15 = 75 MARKS)

16. 	Determine the response of the system whose input x (n) = {1, 2, 3, 1} and

h(n)= {1, 2, 1, -1} using Linear convolution by Graphical method only.

(OR)

17. 	Perform the linear convolution of the following sequence by overlap add method for the following sequences x(n) = {1,2, 3, -1, 2, 3, 4, 5, 6 } and h(n) ={2, 1, -1}.

18. 	An 8-point sequence is given by x (n) = {2, 2, 2, 2, 1, 1, 1, 1}.Compute 8-point DFT of x(n) by radix-2 DIT FFT.

(OR)

19. 	Compute IDFT of the sequence X(k)= { 7,-0.707-j0.707, 0.707-j0.707, 1, 0.707 +j0.707, j,

-0.707+j0.707, 7} using DIF algorithm.

20.	 The specification of the desired lowpass filter is

0.8 ≤│H (ω) │≤ 1.0;      0 ≤ ω ≤     0.2π

│H (ω) │≤ 0.2;       	0.32 π ≤ ω ≤π.

Design Butterworth digital filter using impulse invariant transformation.

[P.T.O.]

(OR)

21. 	Design a digital Chebyshev filter to meet the constraints

1/√2 ≤H (ω)≤1             ;        for 0≤ω≤0.2π

0≤│H(ω)│≤0.1   		 ;   for 0.5π≤ω≤π by using bilinear transformation and assume                              sampling period T= 1 Sec.

22. 	Design a filter with	Hdejω=e−j3ω−π4≤ω≤π40π4≤ω≤π

Using a Hamming window with N=7.

(OR)

23. 	Use frequency sampling method to design a FIR low pass filter with ωc= π/4 for N=15. Plot the magnitude response.

24. 	Draw the architecture of TMS320C5x and explain.

(OR)

25. Discuss the various types of addressing modes for TMS320C5X family.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	LOGIC AND DISTRIBUTED CONTROL SYSTEMS	      	     Time : 3 hours

Subject Code:	12EI225								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are Data loggers?

2.	What is the sampling consideration that has to be taken for data acquisition system?

3.	What is the need for input and output module in a PLC?

4.	What is On delay timer?

5.	What type of programming languages are used in PLC?

6.	What are interlocks?

7.	Define redundancy.

8.	What is Local Control Unit?

9.	Give the importance of high level operator interface.

10.	Give some advantages of DCS.

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish between Direct Digital control and Supervisory control system.

12.	Give short notes on registers in PLC.

13.	Write a PLC program to explain comparison functions.

14.	Brief about evolution of DCS.

15.	Write about the use of general purpose computers in DCS.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain about the functional block diagram of computer control system in detail.

(OR)

17.	Explain the working of Supervisory control with an example.

18.	Explain different types of timer with suitable examples

(OR)

19.	Write a PLC Program to obtain the following task

[P.T.O.]

- The conveyer motor is ON when the start push button is pressed

- Motor ON condition is indicated by pilot light Run

- Whenever the package senses the limit switch LS the motor stops.

- Motor OFF condition is indicated by Stop

- When emergency stop push button is pressed the motor stops at wherever the package is.

20.	Draw PLC Ladder program for sequentially energizing solenoids A,B,C &D after every 5 second increment.

(OR)

21.	Write Short notes on the following with suitable example.

a.	Sequencer.

b.	PLC-PID.

c.	Interlocks and Alarms.

22.	Explain the Architecture of Distributes control system in detail.

(OR)

23.	Explain the concept of redundancy implemented in DCS.

24.	Explain the Low level operator interfaces in DCS.

(OR)

25.	Explain High level Engineering Interfaces in DCS.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	SIGNAL CONDITIONING CIRCUITS				      	     Time : 3 hours

Subject Code:	EI258								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The unit of slew rate is _________.

2.	Write the expression for gain of an op-amp connected in non-inverting mode.

3.	The output voltage of a summer with three resistors in inverting terminal when 	R1=R2=R3=Rf is _________.

4.	In a 555 monostable multivibrator, the R=100kΩ, C=0.1µF and the time delay is _________.

5.	“A Capacitively coupled amplifier amplifies only AC signals”. Say True or False.

6.	Which filter is also called as notch filter?

7.	The purpose of input capacitance in fixed voltage regulator is to _________.

8.	An ideal regulated power supply should have _________ percentage of regulation.

9.	The Low Pass Filter controls _________ and _________ of PLL

10.	A digital phase detector uses _________ gate to detect phase difference.

PART – B  (5 x 3 = 15 MARKS)

11. 	List the ideal op-amp characteristics.

12.	Draw the circuit of an integrator using op-amp.

13.  Why are active filters preferred?

14.	Compare switching voltage regulators and linear voltage regulators.

15.	Mention some typical applications of PLL.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the various frequency compensation techniques of op-amp.

(OR)

17.	Explain the DC characteristics of operational amplifier in detail.

18.	With appropriate circuit diagrams and waveforms, explain the following applications of an op-amp.

a.	Precision Rectifier.

b.	Logarithmic Amplifier.

c.	Current to Voltage Converter.

d.	Clamper.

(OR)

19.	Describe the functional block diagram of 555 Timer and explain the operation of 555 timer in monostable mode with neat diagrams.

20. 	Elucidate the need for an instrumentation amplifier and give a detailed analysis for the same.

(OR)

21.	a.	Explain the operation of State Variable Filter with neat diagrams. 					  (10)

b.	Design a wide band reject filter having fh=500 Hz and fl=2 kHz and pass band gain as 2. 																						    (6)

[P.T.O.]

22.	Explain the internal structure of voltage regulator IC 723 and show how this can be used 	as low voltage regulator with neat diagram.

(OR)

23.	Explain the basic structure of a multiplier and any two applications of the same with necessary diagrams.

24.	With a neat block diagram, Discuss the operation of VCO and also derive the expression for f0

(OR)

25.	Draw the block diagram of PLL and derive the expression for Lock Range and Capture Range.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ADVANCED PROCESSORS						      	     Time : 3 hours

Subject Code:	12EI229							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give an example for immediate addressing mode in modern microprocessor.

2.	Let ta1, ta2 are the access time of the cache and main memory respectively and let H is the hit ratios of cache, give an expression to calculate the average access time for the system with a one level cache.

3.	Give an instruction in 8086 to find 2’s complement of a number.

4.	If the data segment register (DS) contains 3000h, what physical addresses will the instruction

MOV   AL, [234B] read?

5.	List any two features rejected by ARM processor but included in RISC processor.

6.	Give an example for multiply and accumulate instruction in ARM.

7.	Name any two operating modes of ARM processor.

8.	Give the difference between 3-stage and 5-stage pipelining.

9.	Name any two power management modes of VLSI Ruby II Advanced Communication Processor.

10.	Draw the architecture of GSM handset.

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish between page and segment in memory hierarchy.

12.	What do you mean by forwarding scheme in pipelining?

13.	Write an assembly language program in ARM to swap two numbers.

14.	What do you mean by memory bottleneck in pipelining?

15.	What is Bluetooth Piconet?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Name the two basic types of pipelining and explain various types of pipeline Hazards with 	examples.																	  (10)

b.	Compare the features of different instruction level parallelism architectures.			    (5)

(OR)

17.	a.	Discuss different levels of memory hierarchy in modern processors	.				    (8)

b.	Discuss various instruction format used in modern processors.						    (7)

18.	a.	Draw the internal block diagram of 8086 and explain its functional parts.			  (10)

b.	Write an assembly language program in 8086 to transfer a block of data from memory location (2100 to 2109) to other memory location (3100 to 3109)	.				    (5)

(OR)

19.	a.	Illustrate with examples, the various addressing modes of 8086.					  (10)

b.	Specify the 8086 addressing modes for the following instruction

(i) MOV AX, [1234]

(ii) MOV BL, [SI]

(iii) MOVAL, [BX+SI+1234]

(iv) STC

(v) ADD AX,BX																    (5)

[P.T.O.]

20.	a.	Explain various levels of abstraction in hardware design.							  (10)

b.	Discuss the advantages and disadvantages of RISC processor.						    (5)

(OR)

21.	a.	Which features of the ARM architecture are not shared by most other RISCs and which 	features does ARM have in common with many other RISC architectures?			  (10)

b.	Write short note on ARM programmer’s model.									    (5)

22.	a.	With neat sketch explain 3 stage ARM organization.								  (10)

b.	Write a short note on cross bar switch barrel shifter.								    (5)

(OR)

23.	a.	With neat sketch explain 5 stage ARM organization.								  (10)

b.	Give an assembly language program to translate the following C code.

for i=0; i<8;i++

{

a	[i] = b[7-i];

}																			    (5)

24.	With neat sketch explain the architecture of Ruby II advanced communication controller.

(OR)

25.	With neat sketch explain the architecture VLSI ISDN Subscriber Processor.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	BIOMEDICAL INSTRUMENTATION				      	     Time : 3 hours

Subject Code:	12EI232								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the frequency range of EEG signal

2.	What is absolute refractory period?

3.	Define oxygen saturation.

4.	What is cardiac output?

5.	What is the need for pacemaker?

6.	Mention the types of Oxygenators.

7.	What is the pH value of arterial blood?

8.	Define packed cell volume.

9.	List the different modulation systems in Biotelemetry

10.	Define angiography.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the block diagram of a medical instrumentation system.

12.	What are the different lung volumes?

13.	Explain the principle of surgical diathermy.

14.	Write short notes on colorimeter.

15.	Discuss the importance of echo cardiograph in biomedical application.

PART – C  (5 x 15 = 75 MARKS)

16.	Illustrate in brief the working of an ECG recording. Discuss the lead configurations used in ECG.

(OR)

17.	a.	What is meant by central nervous system? Explain the different parts of it and their 	activity.																		  (10)

b.	What are resting and action potentials?											    (5)

18.	a.	Draw the block diagram of basic audiometer and discuss the various blocks involved in 	it.																			  (10)

b.	Describe the principle of Fick’s method for measurement of cardiac output.			    (5)

(OR)

19.	a.	What are the performance characteristics of transducers?							    (5)

b.	Draw the measurement setup for electromagnetic flow meter and explain with their 	advantages.																	  (10)

20.	a.	Explain the working principle of demand pacemaker with the help of block diagram. (12)

b.	What is the need of a cardiac defibrillator?										    (3)

(OR)

21.	a.	Write short notes on nerve stimulator.											    (3)

b.	With neat sketch, illustrate the role of Ventilator in biomedical instrumentation.		  (12)

[P.T.O.]

22.	 a.	What are the different types of blood cell?										    (3)

b.	List the different methods for blood cell counting. Explain the working principle of 	automatic optical method.														  (12)

(OR)

23.	a.	Explain the working of a clinical flame photometer with suitable diagram.			    (8)

b.	Write short notes on automated continuous flow type biochemical analysis system.	    (7)

24.	a.	Explain the working of an X ray machine with the help of block diagram.			    (8)

b.	Write short notes on the technique of producing computed tomography images.		    (7)

(OR)

25.	a.	What is biotelemetry? 														    (3)

b.	Explain the working of a single channel ECG telemetry system with block diagram.	  (12)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	NEURAL NETWORKS AND FUZZY LOGIC CONTROL

Time : 3 hours

Subject Code:	12EI234								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Biologically inspired computing techniques are called __________. Give its features.

2.	Write the general form of delta learning rule and its significance.

3.	Neural Nets in which the source acts as the target are called _________ networks. Give example.

4.	Name the type of network in which clustering of inputs occurs.

5.	Bring out the role of the neural network in the given schematic diagram.

6.	What is the goal of the inverted pendulum task?

7.	If A and B are two fuzzy sets defined on two different Universal sets, write the set theoretic operation behind the fuzzy rule “IF A THEN B”.

8.	Give the expression for a trapezoidal membership function.

9.	What are adaptive fuzzy systems?

10.	Bring out the advantages of fuzzy controllers over conventional controllers.

PART – B  (5 x 3 = 15 MARKS)

11.	Illustrate the concept of linear inseparability using the XOR problem.

12.	What is meant by vigilance test in an ART network?

13.	Draw the schematic blocks for indirect learning architecture using neural networks.

14.	If A and B are fuzzy sets given A={(1,0.1),(2,0.8),(3,0.5)} and B={(1,0.5),(2,0.1),(3,0.1)}, then find A U B’.

15.	Specify the significance of fuzzy based articulation control.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Signal transmission in a neuron is an electrochemical activity. Justify this statement with 	necessary explanations and diagrams.											(7)

b. 	Implement the perceptron rule training for a single layer network using a unipolar continuous activation function, =1 and the following data specifying the initial weights, and the two training pairs.																(8)

(OR)

[P.T.O.]

17.	With neat block diagrams, explain the weight adjustment mechanism in a backpropagation network and derive the generalized delta learning rule for it.

18.	Describe in detail the discrete Hopfield NN with its architecture and training algorithm.

(OR)

19.	With relevant diagrams, explain the Kohonen’s self-organizing network.

20.	Show how a neural network learns an approximation of the system characteristics and uses it to generate the appropriate control signal.

(OR)

21.	With a neat block diagram, demonstrate feedforward control of a plant with plant inverse learning and the specialized online learning control architecture for a dynamic plant.

22.	For research on the human visual system, it is sometimes necessary to characterize the strength of response to a visual stimulus based on a magnetic ﬁeld measurement. When working with a new subject, a researcher will make preliminary measurements to determine if the type of stimulus being used evokes a good response in the subject. The magnetic measurements are in units of femtotesla. Inputs are defined on the universe X=[0,50,100,150,200] femtotesla, and outputs on the universe Y=[0,50,100,150,200] femtotesla. Two fuzzy sets are defined for two different stimuli, on universe X as follows

and one fuzzy set on the output universe Y, as follows

Construct the proposition: IF ‘‘weak stimulus’’ THEN not ‘‘severe response,’’ using classical implication. Use approximate reasoning to find the new consequent if the antecedent is for the proposition IF ‘‘medium stimulus’’.

(OR)

23.	a.	Comment on the various defuzzification methods.         							(8)

b. 	Classify the graphical techniques of fuzzy inference with relevant diagrams.   		(7)

24.	With a neat diagram, show how a fuzzy controller can be used to control a temperature process.

(OR)

25.	Design a fuzzy controller for inverted pendulum.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	DIGITAL CONTROL SYSTEMS					      	     Time : 3 hours

Subject Code:	12EI235							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Sampling Theorem.

2.	List out the basic building blocks of State variable model.

3.	Find the transfer function of zero order hold.

4.	For the given routh array, determine the location of roots in the r-plane. i.e., List the number of roots in left half of r-plane and in right half of r-plane.

r4 :  1      5     8

r3 : -2     6      0

r2 :  8     8

r1 :  8     0

r0 :  8

5.	Draw the graph of Quarter decay ratio.

6.	Mention the two steps involved in implementing a control algorithm on a computer.

7.	Draw the basic structure of Digital control system.

8.	Define Rise time.

9.	__________ is the other name of Modal Matrix.

10.	Write the formula for State Transition Matrix.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the advantages offered by digital control?

12.	Describe about the Reconstruction of analog signals.

13.	Distinguish between Recursive and Non-Recursive Realization.

14.	Draw the Typical unit step response of a digital control system and label all the time domain specifications.

15.	Find the Inverse of the given Modal Matrix023821421.

PART – C  (5 x 15 = 75 MARKS)

16.	a.   	Determine the one-sided z-transform of the discrete sequence generated by sampling the 	given Continuous time functions mathematically.			  	            				    (5)

x(t)=e−atCosωt

b.   	State and prove the properties of z-Transform.		  			           			    (6)

c.   	Determine the Initial Value x(0) and Final value x(∞) of the given z-domain signal.    (4)

X(z)=2−4z−11+2z−1−3z−2

(OR)

[P.T.O.]

17.	a. 	Explain the principles of Signal Conversion in detail. And describe the working of R-2R 	ladder type D/A Converter with its equivalent circuits.				        		  (10)

b. 	With a neat sketch, explain the configuration of the basic digital control scheme.   (5)

18.	a. 	Check whether the given sampled data control system which is represented by the given 	Characteristic equation is Stable or Not, using Jury’s Stability Test.   	                	 (7)	  	z4−1.7z3+1.04z2−0.268z+0.024=0

b. 	Describe the working of the Hold Operation in detail with necessary diagrams. Also determine the model of Sample-and-Hold Operation.				        			    (8)

(OR)

19.	a. 	Derive the relation between the spectrums of the continuous-time signal to that of the 	discrete-time sequence and illustrate the process of sampling in detail.		       		  (10)

b. 	Explain about Direct Realization of Recursive method in detail.			        	    (5)

20.	a. 	Realize the given Pulse Transfer function D(z)=5(z2+3z+2)(z2+8z+15)(z+0.5)using Cascade 	realization.      										       						    (5)

b. 	Using Parallel realization, realize the given pulse transfer function

D(z)=z(z+2)(z+1)(z+3)(z+4)				      							    (5)

c. 	Illustrate the procedure for tuning a controller using Ziegler-Nichols tuning method based 	on Process Reaction Curve.								      					    (5)

(OR)

21.	Explain the hardware features, control schemes and the design of control algorithm for a Digital Temperature Control in an Air-flow System.

22.	a. 	Derive the Steady state errors for various inputs and systems. Also determine the 	Positional error constant, Velocity error constant and Acceleration error constant.   	    (7)

b. 	Describe the procedure for constructing Root Locus on z-plane.			    		    (8)

(OR)

23.	a. 	Explain the basic routes to the design of digital controller in detail.		    			    (7)

b. 	Describe the effect of pole and zero cancellation.					    				    (4)

c. 	Write short notes on Realizability of digital controller.				    			    (4)

24.	Transform the given state model into a canonical state model and also compute the state transition matrix,eAt for the same.

x1x2x3=010001−6−11−6x1x2x3+002[u] ;        y=100x1x2x3

(OR)

25.	a. 	A discrete-time system has the transfer function,Y(z)U(z)=4z3−12z2+13z−7(z−1)2(z−2). Determine 	the state model of the system in Phase Variable form and Jordan canonical form.    	  (10)

b. 	Check whether the given system is Controllable and Observable.		     			    (5)

A=001−2−3002−3;B=020;C=100

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	INSTRUMENTATION AND CONTROL IN PETROCHEMICAL INDUSTRIES											     Time : 3 hours

Subject Code:	12EI236								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Abbreviation of TF is __________.

2.	Draw the symbol of connector

3.	__________ is the most common type of Batch Reaction process.

4.	Define Low Water Interlock.

5.	Name any one method of controlling the absolute pressure.

6.	__________  dryer has a fluidized bed of material

7.	State the application of Short Tube Vertical Evaporator.

8.	What is Boiling Point Rise?

9.	Name any two process of waste water treatment.

10.	A factor in the Precipitation of calcium carbonate is a chemical phenomenon known as __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish the manipulated and load variables in Distillation Process.

12.	Differentiate between Batch reactor and Continuous reactor.

13.	Define Fuel Control.

14.	What are the types of Evaporators?

15.	Give a note on Precipitation.

PART – C  (5 x 15 = 75 MARKS)

16.	a.   Write a short note on Reboilers design.											    (8)

b.	Explain the working of Batch Distillation with neat diagram.						    (7)

(OR)

17.	Elucidate on the column pressure control in Vacuum Distillation with a neat diagram.

18.	Explain the principle and working of the advanced Batch Dryer Control system with neat sketch.

(OR)

19.	With neat sketch explain the construction and operation of Fluid Bed Dryer.

20.	Explain with neat diagram the operation of steam heater under feed forward control.

(OR)

21.	Explain the methods by which the condenser control is achieved.

22.	Explain the common methods used to measure the product density in an evaporator control scheme.

(OR)

23.	a. 	Discuss briefly the concept of a multiple effect evaporator.                               		    (8)

b. 	Describe with a neat sketch the operation of a forced circulation Evaporator.			    (7)

24.	a.	Explain the operation of continuous oxidation of cyanide waste with chlorine.		    (8)

b.	Give a note on calcium carbonate Precipitation control system.						    (7)

(OR)

25.	Explain the process of effluent and waste water treatment by Neutralization.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	INSTRUMENTATION AND CONTROL IN PAPER INDUSTRIES													      	     						      Time : 3 hours

Subject Code:	12EI237								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.  	Name the primary raw material used in the manufacturing process of paper.

2.  	What is sulfate pulping?

3. 	List the use of blow tank.

4. 	Mention the different methods used to measure moisture.

5. 	List the different types of flow meters used in paper industry.

6. 	How smelt is formed?

7. 	Name the instrument used to measure the water level.

8. 	Mention the advantage of magnetic flow meters.

9. 	Name the different display devices used in computer control of processes.

10. 	Mention the importance of alarm devices in process industries.

PART – B  (5 x 3 = 15 MARKS)

11. 	Define the term pulping.

12. 	What is the need for oxidation reduction potential measurement?

13. 	What is meant by Feed pump?

14. 	Define the term Conductivity.

15. 	What is the need for computer application in process industries?

PART – C  (5 x 15 = 75 MARKS)

16. 	With a neat block diagram explain the function of each block in detail of a Kraft mill.

(OR)

17. 	Explain the chemical recovery and liquor making process with a neat flow diagram.

18. 	Explain in detail any two methods used to measure density in paper industries.

(OR)

19. 	Discuss in detail the following	(a) Moisture Measurement.							    (7)

(b) Temperature Measurement.  								    (8)

20.  Explain in detail the Digester Liquor feed pump with a neat diagram.

(OR)

21.  Discuss the Yankee dryer temperature control in detail with a neat flow diagram.

22. Discuss in detail about the dissolving tank density Control.

(OR)

23.  Write short notes on

a. 	Condensate Conductivity Control.												    (7)

b. 	White liquor flow control loop.													    (8)

24. 	Discuss in detail how direct digital process control plays a vital role in paper mills.

(OR)

25.  	Discuss about the computer applications in Measurement & Control of pulping process.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	INSTRUMENTATION AND CONTROL IN IRON AND STEEL INDUSTRIES											      Time : 3 hours

Subject Code:	12EI238								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List the raw materials needed for iron making.

2.	Name the various steps in iron making.

3.	Define steel casting.

4.	What is teeming?

5.	Identify type of transducer is used for the measurement of pressure.

6.	Infer the basic elements of a radio frequency level measurement system.

7.	Define control system.

8.	Point out the parameters that are controlled in BOF.

9.	List any two computer applications in industries.

10.	What are the basic equipments used in the initial cod rolling of sheet coils?

PART – B  (5 x 3 = 15 MARKS)

11.	Mention the various processes involved in the making of raw steel.

12.	List the advantages and disadvantages of the casting of steel.

13.	How graphic displays are formed?

14.	Write short note on sand casting old control.

15.	State soak temperature.

PART – C  (5 x 15 = 75 MARKS)

16.	Illustrate the iron and raw steel making process.

(OR)

17.	Outline the working of following furnaces.

a.	Blast furnaces.

b.	Basic oxygen furnace.

18.	Illustrate the primary rolling process of casting of steel.

(OR)

19.	With neat sketch explain the cold rolling process and finishing process of casting of steel.

20.	Write a note on following measurements.

a.	Measurement of level.

b.	Measurement of flow weight.

(OR)

21.	Analyze the following.

a.	Measurement of pressure

b.	Graphic displays and alarms.

22.	Explain the blast furnace stove combustion control system.

(OR)

23.	Elucidate the gas and water controls in BOF furnace.

24.	Summarize the centralized computer approach in the steel plant.

(OR)

25.	Draw and explain the annealing process control.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ULTRASONIC INSTRUMENTATION				      	     Time : 3 hours

Subject Code:	12EI239								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The power transmission coefficient of sound wave is given by________.

2.	The velocity of sound wave is given as ________.

3.	The ultrasonic piezoelectric transducers can be designed precisely for a frequency depending on ________.

4.	Mention one application of ultrasound cross correlation.

5.	What is transit time?

6.	Based on what ultrasonic effects ,physical parameters can be measured?

7.	Mention two applications of ultrasonic waves.

8.	Can ultrasounds be used for flaw detection in isotropic materials?

9.	The diagnostic medical ultrasound is in the range of ________.

10.	What is acoustical holography?

PART – B  (5 x 3 = 15 MARKS)

11.	Why does attenuation of sound wave happen?

12.	Can ultrasonic transducers based on magnetostrictive effects be used at high temperatures? Why?

13.	Write the principle behind the operation of ultrasonic flowmeters.

14.	Define and explain SONAR.

15.	In short explain one application of ultrasound in medical therapy.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail the means of producing and transmitting ultrasonic waves.

(OR)

17.	Explain the concept of sound attenuation.

18.	Discuss about the electrical means of detecting ultrasounds.

(OR)

19.	How are piezoelectric and magnetostrictive type ultrasonic transducers constructed? Explain their characteristics.

20.	Elaborate on the method of detecting flaws in materials using ultrasounds.

(OR)

21.	How are ultrasounds useful in level measurement? Where do they find their application?

22.	In what type of quality inspections, ultrasounds used? Explain any two in detail.

(OR)

23.	How can you measure thickness and depth? Give the construction and working.

24.	Discuss in detail the various imaging techniques using ultrasonic waves.

(OR)

25.	Mention two areas in medical field where you use ultrasounds for therapy.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	OPTO ELECTRONICS AND LASER BASED INSTRUMENTATION													      	     						      Time : 3 hours

Subject Code:	12EI241								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The inner core of an optical fiber is ___________ in composition.

2.	When a beam of light travels through media of two different densities, if the angle of incidence is greater than the critical angle, ___________ occurs.

3.	For long distance communications ___________fiber is preferred.

4.	___________ Fiber optic sensors are used to measure the displacement.

5.	The first step to getting output from a laser is to excite an active medium. What is this process called?

6.	What colour does the ruby laser emit?

7.	What are the types of trimming?

8.	A projection weld is a type of ___________.

9.	List the advantage of Acoustical hologram.

10.	What are the different laser interactions with tissues?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw basic components of fiber optics transmission link.

12.	What are the features of fiber optic sensors?

13.	Define internal quantum efficiency of a laser.

14.	What are the types of laser welding?

15.	What are the advantages of laser surgery?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	A silica optical fiber with a core refractive index of 1.50 and a cladding refractive index of 	1.47. Determine the critical angle at the core cladding interface, numerical aperture.	    (5)

b.	Find out the core radius of a single mode step index fiber whose core and cladding refractive indices are 1.48 and 1.478 at an operating wavelength of 1320nm.Calculate maximum value of the angle of acceptance of the fiber. 							  (10)

(OR)

17.	a.	Describe the principle of propagation of light through an optical fiber and   properties with   	necessary diagrams.                          											   (5)

b.	Explain the requirements for an optical source in fiber optic communication and also explain the construction and working of optical source with relevant diagrams.		 (10)

18.	a.	Explain the different types of fibers with   necessary diagrams.				        	   (5)

b.	With relevant diagrams, explain the principle of fiber optic sensors used for measurement temperature.                                                                      							 (10)

(OR)

19.	a.	Explain with neat block diagrams active and passive sensor.						   (5)

b.	With relevant diagrams, explain the principle of fiber optic sensors used for measurement of   pressure.                                                                      							  (10)

[P.T.O.]

20.	a.	Describe the characteristics of lasers. With help of diagrams explain the principle of three  	level   and four level Lasers.													    (5)

b.	With help of diagrams explain the principle of Q-Switching and Mode locking. 		  (10)

(OR)

21.	a.	Explain the principle of population inversion concept of resonator for the generation of 	laser beam.																	    (5)

b.	What are the types of lasers and explain the working principle of solid Lasers.		  (10)

22.	a.	Explain the principle of laser for the measurement of Measurement of Current.		    (5)

b.	Explain the principle of laser for the measurement of distance and Length with neat diagrams.																	  (10)

(OR)

23.	a.	Explain the two types of laser melting Process?									    (5)

b.	Describe the laser apparatus used for welding and mention the advantage of laser welding 	and laser cutting.																  (10)

24.	a.	Describe the different stages involved in the holography with suitable diagrams.		    (5)

b.	Explain in detail the principles of holography for non-destructive testing. 			  (10)

(OR)

25.	a.	Discuss the different lasers used in the medical field along with their applications.	    (5)

b.	What are endoscopes? Describe any one of the endoscope along with its application.    (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	POWER PLANT INSTRUMENTATION				      	     Time : 3 hours

Subject Code:	12EI242								 		       	      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the major disadvantage of wind power generation method?

2.	Sketch the P&ID for control valve

3.	Very high voltages are measured with ___________.

4.	Mention the range of Temperature transmitter output.

5.	pH value of feed water to boiler is ___________.

6.	Name the two phases of chromatography.

7.	Steam pressure measured in ___________ location of boiler

8.	What is the advantage of feed forward control?

9.	The function of turbine is converting ___________ energy into ___________.

10.	___________ Valve is used as safety mechanism in turbine monitoring system.

PART – B  (5 x 3 = 15 MARKS)

11.	Define the term sustainable energy.

12.	What is the need for current transformer in electrical measurement?

13.	Define conductivity.

14.	What is shrinking & swelling in boiler process?

15.	List the vital parameters measured in steam turbine.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Discuss about the piping and instrumentation diagram of boiler operation.			    (8)

b.	Write short notes on nuclear power plant.                                                  			    (7)

(OR)

17.	a.	With neat sketch explain the working principle of thermal power plant.				  (10)

b.	Write short note on solar power plant with neat sketch.							    (5)

18.	a.	Write short notes on Water level measurement in boiler.							    (7)

b.	Draw the circuit for power measurement in thermal power plant & also explain its 	operations.																	    (8)

(OR)

19.	a.	The steam at a pressure of 10 bar is to be measured. Which device is suitable? Explain.(8)

b.	Explain the function of smoke detector using optical method.						    (7)

20.	a.	With neat sketch explain the working principle of pH analyzer.				    (7)

b.	Discuss in detail about oxygen measurement in flue gas.						    (8)

(OR)

21.	a.	With neat diagram explain about gas chromatography.							    (7)

b.	 Explain Wallace and Tierman dissolved oxygen analyzer.							    (8)

[P.T.O.]

22.	a.	With neat sketch explain the drum level control loops in water circuit.				  (10)

b.	Draw & explain the generalized structure of cascade control scheme for steam temperature control. 														    (5)

(OR)

23.	a.	What are the difference levels of DCS in power plant and explain each of its level.	  (10)

b.	Write short note on combustion control in thermal power plant.					    (5)

24.	a.	Explain in detail about basic safety interlocks used in boiler.						    (7)

b.	With neat sketch explain shell temperature monitoring and control.					    (8)

(OR)

25.	a.	 How are speed and vibration monitored in power plant?							  (10)

b.	Why should be the pressure of steam be raised in the boiler and turbine?			   (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December – 2015

Subject Title :	AUTOMOTIVE CONTROL SYSTEMS				      	     Time : 3 hours

Subject Code:	12EI243								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The term TDC refers to ___________.

2.	What is meant by ignition timing?

3.	The ___________ always applies the same amount of torque to each wheel.

4.	Mention the use of clutch in an automobile.

5.	___________ is the highest data rate of flex ray.

6.	Define bit stuffing and its purpose.

7.	Brake assist systems help to apply the brakes under ___________ conditions

8.	A system that improves the grip of driven wheels when accelerating is known as ___________.

9.	List the sensors used for car navigation system.

10.	Gasoline is a mixture of chemical compounds that are called ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	Define the term ‘Ignition’.

12.	Legislation has a considerable effect on the development of automotive system. Justify.

13.	List the different bus systems used in automobile.

14.	Define the term ‘slip’

15.	What is a three way catalyst?

PART – C  (5 x 15 = 75 MARKS)

16.	List the components and elaborate on the operations of Distributor Less Electronic Ignition System with a neat sketch.

(OR)

17.	Draw the configuration of electronic fuel control system and explain the functions of each.

18.	Draw the block diagram of Collision avoidance warning system and explain.

(OR)

19.	With neat sketch explain the construction, operation and working principle of power train system used in automobile systems.

20.	Explain the concept of CAN network used in automotive electronics.

(OR)

21.	a.	Explain the communication architecture and data format of USB.					    (5)

b.	With relevant diagrams explain the signal transfer using I2C.						  (10)

22.	a.	Explain the need for traction control in automotive systems.						   (5)

b.	Write a brief note on antilock braking system.									  (10)

(OR)

23.	Mention the various parameters to be measured in an automotive system and explain how mass air flow is measured.

24.	Illustrate with neat diagram the function of Navigation Systems in Automotive Systems.

(OR)

25.	Explain the concept of chassis dynamometer for simulating driving conditions.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FIBER OPTICS AND LASER INSTRUMENTATION      Time : 3 hours

Subject Code:	12EI247								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define numerical aperture and its relation to acceptance angle and refractive indices of fiber components.

2.	What is dispersion?

3.	Name any two types of gas lasers.

4.	List out the various materials used for optical fiber manufacture.

5.	Write a note on Q switching.

6.	List a few resonator configurations.

7.	State the principle used in laser current measurement.

8.	Which are the types of laser welding used in industry?

9.	State the significance of acoustic holography.

10.	Mention one diagnostic medical application of laser.

PART – B  (5 x 3 = 15 MARKS)

11.	Calculate the total number of guiding modes propagating in multimode step index fiber having diameter 40μm and numerical aperture of 0.3 and operating at a wavelength of 1μm.

12.	Write a short notes on the properties of laser.

13.	Explain the working of optical domain reflectometer.

14.	Give a short notes on various levels of laser.

15.	What is photodynamic therapy?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Elaborate with illustration the different types of fibers and differentiate their characteristic 	features.																	 	    (8)

b.	Briefly explain the mechanical characteristics of optical fiber.						    (7)

(OR)

17.	a.	With neat diagram explain the construction and working of PIN diode.				  (10)

b.	With a neat block diagram, give a note on the general optical communication system.	    (5)

18.	a.	Explain the various methods of displacement measurement using optical fibers.		  (10)

b.	Give a short note on intensity modulated temperature sensor.						    (5)

(OR)

19.	a.	Explain the method of measurement of liquid level using optical fibers.				    (8)

b.	Illustrate the measurement of Pressure and Temperature using fiber optic sensors.	    (7)

20.	a.	Explain the basic physical processes involved in laser production.					    (7)

b.	Elaborate the construction and working of solid state laser.						    (8)

(OR)

21.	a.	Illustrate the various components of a laser source.								    (7)

b.	Explain the construction and operation of semiconductor laser and list the types.		    (8)

[P.T.O.]

22.	a.	How is Laser is used for the measurement of fluid velocity?						    (9)

b.	Elucidate the application of  Laser in material removal and vaporization.			    (6)

(OR)

23.	a.	Explain in detail the principle of laser welding and melting.						  (10)

b.	Illustrate the working and application of LIDAR.									    (5)

24.	a.	What is holography? Explain its construction and reconstruction with neat illustrations.																						  (10)

b.	How is laser is used in vocal cord surgery?										    (5)

(OR)

25.	a.	Write detailed notes on types of holographic interferometry.                                             (7)

b.	Elaborate on the various tissue interactions during absorption and transmission of laser. (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	AIRCRAFT INSTRUMENTATION					      	     Time : 3 hours

Subject Code:	12EI249											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the alternate system, in case of failure of pitot static system?

2.	The most common forms of data display applied to aircraft instruments are _________ & _________.

3.	List the advantage of heating element in the Pitot Static tube.

4.	The ratio of aircraft speed and speed of sound is known  as  _________.

5.	The methods used  for driving the rotor of gyroscope is _________.

6.	List the limitations of a free gyroscope.

7.	The region in which the force exerted by a magnet can be detected is known as _________.

8.	What is magnetic moment?

9.	The temperature sensing of solid component is measured through _________ type of thermocouple.

10.	List the purposes of pressure switches required in aircraft.

PART – B  (5 x 3 = 15 MARKS)

11.	Define qualitative display.

12.	Draw a line diagram of pitot static system.

13.	Write the principle of gyro horizon.

14.	Define the term magnetic dip.

15.	State the principles of thermocouple.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Describe the four principal elements which make up an instrument?				    (5)

b.	Describe the construction and operation of head-up display.						  (10)

(OR)

17.	a.	Bring out the characteristics of display used in high-range measurements.			    (5)

b.	Briefly describe one form of light emitting display.								  (10)

18.	a.	What are the principle components and instruments which comprise an aircraft pitot-static 	system?																		    (5)

b.	Describe the construction and operation of airspeed indicator.						  (10)

(OR)

19.	a.	Describe the function of Maximum safe airspeed indicator.						    (5)

b.	Explain the construction and operation of a vertical speed indicator.				  (10)

20.	a.	How are the gyroscopic properties utilized in flight instruments?					    (5)

b.	Explain how a rate Gyroscope principle is applied for Turn and Bank indicator.		  (10)

(OR)

21.	a.	What do you understand by the term gimbal lock and gimbal error?					    (5)

b.	With neat diagram, describe the principle of directional gyro.						  (10)

[P.T.O.]

22.	a.	What do you understand by the term hard-iron and soft-iron magnetism?			    (5)

b.	Describe the construction and operation of direct reading magnetic compasses.		  (10)

(OR)

23.	a.	Explain how the effects of temperature change in an aircraft compass and compensated.

(5)

b.	Explain the types of errors present in direct reading magnetic compasses.			  (10)

24.	a.	Explain how the Wheatstone bridge circuit may be utilized for the measurement of 				temperature.																	    (5)

b.	Describe the construction of a thermocouple probe used for turbine engine exhaust gas 	temperature measurement.														  (10)

(OR)

25.	a.	Define the terms absolute pressure and gauge pressure.							    (5)

b.	Explain the fundamental operating principle of the Bourdon tube in aircraft.			  (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	INSTRUMENTATION AND PROCESS CONTROL FOR FOOD ENGINEERS											      Time : 3 hours

Subject Code:	12EI252								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define system.

2.	What is process control?

3.	State the transfer function of control system.

4.	List the uses of control valve.

5.	Define the stability in a linear system.

6.	Write any two applications of stability analysis.

7.	Define sensor.

8.	Define state gauge factor.

9.	List any two capacitive sensing element.

10.	Mention any two applications of mass spectrometer.

PART – B  (5 x 3 = 15 MARKS)

11.	Define controller and control element.

12.	Distinguish between open and closed loop control systems.

13.	State Mason’s Gain formula.

14.	Compare thermistors and thermocouples.

15.	Mention the measurement composition of pH meter.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the block diagram of process control system and explain the same with illustrative example.

(OR)

17.	Summarize the Laplace transforms of step function, exponential function and ramp function.

18.	Outline the concepts of hydraulic and pneumatic control systems.

(OR)

19.	Illustrate the construction and working of oneumatically operated valve and spring.

20.	A closed loop system is represented by the differential equation d3c/dt3 + d2c/dt2+4dc/dt = 20e where e = r–0.8c. Identify the quantities c, r, e and draw a block diagram to represent the system. Find the overall transfer function of the system.

(OR)

[P.T.O.]

21.	Determine the overall transfer function of the system whose signal flow graph shown in figure.

22.	Illustrate the construction and working of capacitive pressure sensor.

(OR)

23.	Explain the construction and working of RTD.

24.	Outline the working of Ultrasonic level sensor and Utype densitometer.

(OR)

25.	Draw and explain the working of electrical conductivity cell and Gas chromatograph.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	INSTRUMENTATION AND CONTROL SYSTEMS		      Time : 3 hours

Subject Code:	12EI254								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give examples for integrating instruments.

2.	The process of comparison of a given instrument with a standard instrument to determine its accuracy is called as __________.

3.	List the devices used for low pressure measurement.

4.	Mention any two types of accelerometer.

5.	Write the application of hot-wire anemometer.

6.	Write examples for Piezo electric materials.

7.	In signal flow graph, a path which originates and terminates at the same node is  called as __________.

8.	Write the transfer function of PI controller.

9.	Define the term “rise time” in a second order system.

10.	If the system having the complex poles on jω-axis, the stability of the system is said to be __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish between precision and accuracy.

12.	Write the principle and applications of thermocouple.

13.	Write the principle and list the major components of mechanical torsion meter.

14.	Differentiate open and closed loop systems with suitable examples.

15.	Draw the time response of a first order system with step input and write the significance of time constant.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the generalized functional block diagram of an instrumentation system. Explain      the significance of each block. Discuss the functional block diagram for pressure   measurement using Bourdon tube.

(OR)

17.	What are the different errors encountered in measurements? Explain with examples and the measures taken to minimize these errors.

18.	a.	Explain the principle of measuring pressure using pressure cell with constructional 	diagram.                                                                                                                 		    (7)

b.	Draw the schematic diagram of a radiation pyrometer and explain its operation in 	measurement of temperature.                                                                                 		    (8)

(OR)

19.	Explain the principle and characteristics of LVDT and discuss how LVDT can be used for Displacement measurements.

20.	Discuss the various types of strain gauges.  Explain how the strain gauges are used to measure the force.

(OR)

[P.T.O.]

21.	Explain the principle of operation, constructional features, advantages and disadvantages of electromagnetic flow meter.

22.	Determine the transfer function of the system represented by the block diagram shown below using block diagram reduction technique.

+

-

+

-

+

-

R(s)

C(s)

(OR)

23.	Explain the principle and working of a pneumatic system.  Differentiate it from hydraulic system.

24.	a.	Draw the plot of time response of second order system with step input under different 	damping conditions.                                                                                				    (3)

b.	The characteristic equation of a feedback control system is found as

s4+9s3+11s2+6s+K=0

Determine the value of K for which the system is absolutely stable and marginally Stable. Also determine the frequency of sustained oscillation.                            				  (12)

(OR)

25.	A unity feedback control system hasG(s)=40s(s+2)(s+5). Draw the Bode plot. Find gain margin and phase margin

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination –  November/December - 2015

Subject Title: 	INDUSTRIAL INSTRUMENTATION				       Time : 3 hours

Subject Code: 	12EI302										       	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

Describe the different types of manometers with appropriate sketch and discuss about source of error in the manometer.

2.	a.	With neat diagram explain about rotameter and  derive its flow equation.		      (10)

b.	With a neat sketch explain the principle and operation of ultrasonic flow meters.	      (10)

(OR)

3.	a.	Explain the theory of fixed restriction variable head type flow meters with necessary 	diagrams.																      (10)

b.	Illustrate measurement of mass flow rate using coriolis mass flow meter.		      (10)

4.    	a.	Discuss the construction and operation of  filled system thermometers with sketch.    (10)

b.	With a sketch explain the construction details and principle of a thermocouple transducer 	used for temperature measurement.											      (10)

(OR)

5.  	a.	Illustrate about IC temperature sensor. 	         								      (10)

b.	With neat sketch explain about non contact type temperature measuring technique.    (10)

6. 	Describe any two direct method of liquid level measurement for measuring the level of corrosive liquid  with necessary diagram.

(OR)

7. 	a.	With neat diagram explain about air purge system of level measurement.        	      (10)

b.	Explain the Non- contact type level sensor.									      (10)

8.	Discuss about smart intelligent transducer with appropriate sketch. Also discuss about smart transducer versus conventional transducer.

(OR)

9. 	a.	Discuss the smart transmitter with HART communicator.                                            (10)

b.	Write notes on reliability engineering.										      (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	CONTROL SYSTEMS								      	     Time : 3 hours

Subject Code:	EI203							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What do you mean by branch?

2.	Can block diagrams be used to model non linear systems?

3.	For a second order undamped system, the poles are _________.

4.	Define delay time

5.	The open loop transfer function of a closed loop system is G(s)= K/S(S+2)(S+4). The range of values of K for stable operation is _________.

6.	The number of sign changes in the elements of the first column of the Routh array denotes _________.

7.	What is a minimum phase system?

8.	Define gain margin

9.	What do you mean by derivative control?

10.	Due to integral control, the order of control system _________.

PART – B  (5 x 3 = 15 MARKS)

11.	Write Mason’s Gain Formula.

12.	Give the relation between generalized and static error coefficients.

13.	Write short notes on BIBO stability.

14.	What is Phase Cross over frequency & gain cross over frequency?

15.	What is the effect of proportional controller on the performance of the system?

PART – C  (5 x 15 = 75 MARKS)

16.	Find the overall gain of the system whose signal flow graph is shown below.

(OR)

17.	Derive the transfer function of armature Controlled DC Servo motor.

[P.T.O.]

18.	For a unity feedback control system the open loop transfer function G(s)=10(S+2)/S2(S+1). Find the

a.	position, velocity and acceleration error constants,

b.	the steady state error when the input is  Rs=3s−2s+13s3.

(OR)

19.	A positional control system with velocity feedback is shown below. What is the response of the system for unit step input.

20.	a.	Use Routh stability criterion to determine the location of roots on the S- plane and hence 		the stability for the system represented by the characteristic equation 	s5+4s4+8s3+8s2+7s+4=0.  														  (10)

b.	What are break away and break in points in root locus.							   (5)

(OR)

21.	A unity feedback control system has an open loop transfer functionGs=Kss2+4s+13. Sketch the root locus.

22.	Plot the bode diagram for the following transfer function and find Gain cross over frequency, Phase cross over frequency, Gain Margin & Phase Margin. G(s)= KS2/(1+0.2S)(1+0.02S).

Determine the value of K for a gain cross over frequency of 5 rad/s

(OR)

23.	The open loop Transfer function of a unity feedback system is given by G(S) = 1/S (1+S) 2 sketch the polar plot and determine its phase margin and gain margin.

24.	a.	Explain PID Controller with block diagram                                                                        (5)

b.	What is Fuzzy Controller? Explain the Construction of Fuzzy Controller with example.(10)

(OR)

25.	Explain Fuzzy logic based temperature control system.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	EMBEDDED INSTRUMENTATION SYSTEM		      	     Time : 3 hours

Subject Code:	EI206								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	68HC11 is a ______ bit microcontroller.

2.	Define an Interrupt.

3.	What is synchronization?

4.	Define semaphore.

5.	What is interrupt latency?

6.	Define response time.

7.	Define reliability.

8.	What is locator?

9.	Which language is used in www pages?

10.	Give an example of multitasking, multiuser operating system.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the instructions of 68HC11, give examples?

12.	Describe about inter task communication.

13.	What is interrupt latency?

14.	What is fault tolerance?

15.	What is real time database?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail the architecture of 68HC11 with neat block diagram.

(OR)

17.	Explain the interrupt handling methods in 68HC11 with necessary diagram.

18.	Explain in detail about real time kernels.

(OR)

19.	Explain in detail about real time memory management.

20.	Discuss about time loading and its measurement in detail.

(OR)

21.	Discuss the ways by which system performance is improved and the response time calculation.

22.	Explain the techniques of fault tolerance in detail.

(OR)

23.	Discuss the goals of real-time system integration in detail.

24.	Explain about real-time databases and real-time image processing.

(OR)

25.	Explain about real-time UNIX in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	BIOMEDICAL INSTRUMENTATION				      	     Time : 3 hours

Subject Code:	EI252								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by cell?

2.	What is synapse?

3.	Give any two electrodes used for ECG?

4.	Define EOG?

5.	What is auscultation?

6.	Give two methods for measuring blood flow.

7.	What are the methods involved in direct blood pressure measurement?

8.	What is a Defibrillator?

9.	What is MRI?

10.	What is NSR?

PART – B  (5 x 3 = 15 MARKS)

11.	What is endoscopy?

12.	What is the cardiac pacemaker and why is it used?

13.	What is cardiac fibrillation?

14.	What is micro shock?

15.	What is Biotelemetry?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain a neuron with neat diagram.

(OR)

17.	a.	What is an Action potential? Draw an Action potential and mark its periods.			  (10)

b.	Explain the methods of measuring EMG.										    (5)

18.	a.	Explain a direct method of measuring blood pressure. 							    (8)

b.	Explain a transducer which measures the electrical signal of heart. 					    (7)

(OR)

19.	Explain the respiratory system.

20.	Explain the working principle of an ECG machine with a neat block diagram.

(OR)

21. 	Explain Heart-Lung machine.

22. 	Explain the spectrophotometer.

(OR)

23. 	Explain the Gas chromatography and PH measurement.

24. 	Explain the MRI Instrumentation with neat diagram.

(OR)

25.	Explain the Biotelemetry system with neat block diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	INDUSTRIAL INSTRUMENTATION				      	     Time : 3 hours

Subject Code:	EI256								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give two pressure units.

2.	Give one transducer for pressure measurement.

3.	What is the flow meter used for solid measurement?

4.	What is coriolis?

5.	Where thermometers are used?

6.	Give two units of temperature.

7.	Define humidity.

8.	Define Newtonian fluids.

9.	What is HART communicator?

10.	What is DAS?

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the principle of strain gauge.

12.	What are the methods of flow meter calibration?

13.	Explain the whetstone bridge with one arm replaced by RTD.

14.	Define Bernoulli equation.

15.	What are smart Transmitters?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain vacuum gauges.

(OR)

17. 	Explain dead weight tester and Ionization gauge.

18.	Explain a Rotameter with neat diagram. Explain the method of flow Measurement using Rotameter.

(OR)

19.	How the flow meters are calibrated? Explain in detail.

20.	What is cold junction compensation? How it is done with RTD? Derive.

(OR)

21.	 Explain any two IC temperature sensors.

22. 	Explain the Air purge system?

(OR)

23. 	What is the working principle of pressure gauge? Explain.

24. 	What are the salient features of Smart transmitters? Discuss.

(OR)

25.	Explain the PC based temperature control system.