Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FOOD ENGINEERING OPERATIONS				      	     Time : 3 hours

Subject Code:	09FP201										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The inclination of cylinder sorter for dry granular material is ___________.

2.	In the velvet roll separator, the grains are separated based on the ___________ and ___________.

3.	The rotating speed of hammer in hammer mill is ___________.

4.	Define Rittenger’s law.

5.	Blanching temperature for fruits and vegetable is ___________.

6.	What is lye peeling?

7.	Define triple point.

8.	Freezing time is estimated using ___________ equation.

9.	___________ blender is used to mix powder particle.

10.	Name three stages of bread moulder.

PART – B  (5 x 3 = 15 MARKS)

11.	Write notes on cyclone separator with neat sketch.

12.	Explain about mixers used in bakery industry.

13.	Explain ball mill with neat sketch.

14.	List the advantages of enrobing.

15.	What is freeze concentration?

PART – C  (5 x 15 = 75 MARKS)

16.	Elucidate types of screens used in food processing industries

(OR)

17.	Write notes on

a.	Indent cylinder separator.

b.	Air screen cleaner.

18.	Give brief account on mixing equipments used in food industries

(OR)

19.	Explain in detail about pie and biscuit former and moulders used in bakery and confectionery industries.

20.	Elucidate the laws involves in size reduction operation.

(OR)

21.	Write notes with neat sketch on

a.	Jaw crusher.

b.	Crushing roll.

22.	Explain in detail about different peeling methods and its applications

(OR)

23.	Give brief account on blanchers and its advantages

24.	Explain about food freezing.

(OR)

25.	Explain in detail about freezing equipments

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	HEAT AND MASS TRANSFER						      	     Time : 3 hours

Subject Code:	12FP209						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the modes of heat transfer?

2.	Define convection.

3.	Define overall heat transfer co efficient.

4.	What is dimensional number used in forced convection?

5.	What are different types of heat exchangers?

6.	What is meant by LMTD?

7.	Define Radiation.

8.	What is molecular diffusion?

9.	What are the modes of mass transfer?

10.	What is free Convective mass transfer?

PART – B  (5 x 3 = 15 MARKS)

11. 	State the Fouriers law of heat conduction.

12. 	State the Stephan Boltzman law.

13. 	What is difference between natural and forced convection?

14. 	Write short notes on film wise condensation.

15. 	What is Ficks law of diffusion?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Derive an expression for the rate of heat transfer through a composite plane wall consisting 	of three heterogeneous layers having thermal conductivity K1, K2 and K3 respectively.

b.	A plane brick wall, 25 cm thick, is faced with 5 cm thick concrete layer. If the temperature of the exposed brick face is 70°C and that of the concrete is 25°C, find out the heat lost per hour through a wall of 15 m x10 m. Also, determine the interface temperature. Thermal conductivity of the brick and concrete are 0.7 W/m.K and 0.95 W/m.K respectively.

(OR)

17.	A cold store has a wall comprising 11 cm of brick on the outside, then 7.5 cm of concrete and then 10 cm of cork. The mean temperature within the store is maintained at -18°C and the mean temperature of the outside surface of the wall is 18°C. 
Calculate the rate of heat transfer through the wall. The appropriate thermal conductivities are for brick, concrete and cork, respectively 0.69, 0.76 and 0.043 J m-1 s-1 °C-1.Determine also the temperature at the interfaces between the concrete and cork layers, and the brick and concrete layers.

18.	Explain natural convection in heat transfer and its correlations over flat plate and cylinder.

(OR)

19.	a.	Explain drop wise condensation.

b.	Water is flowing at 0.3 m s-1 across a 7.5 cm diameter sausage at 74°C. If the bulk water temperature is 24°C, estimate the heat-transfer coefficient. Properties of water at 49°C are: 
cp  = 4.186 kJ kg-1°C-1, k = 0.64 J m-1 s-1°C-1, μ= 5.6 x 10-4 N s m-2, ρ = 1000 kg m-3.and

Nu =0.26(Re)0.6(Pr)0.3

[P.T.O.]

20.	a.	Explain the terms emissivity, absorptivity and reflectivity.

b.	Calculate the radiation heat exchange for unit area between two parallel infinite walls, the temperature of which are 400oC and 100oC if

i.	both are black surfaces

ii.	wall at 400oC is black body and wall of 100oC is grey with emissivity of 0.5

iii.	both walls are grey with emissivities of 0.8 and 0.5 respectively

Assume Stefan-Boltzmann constant as 4.92 x 10-8 kcal/hr.m2.K4

(OR)

21.	a.	State the Planck's distribution law.

b.	A solid cube of side 30 cm at an initial temperature of 1000 K is kept in vacuum at absolute zero temperature. Calculate the time required to cool it to 500 K. The material has the following properties:

Density = 2700 kg/m3

Specific heat = 0.9 kJ/kg.K

Emissivity = 0.1

Stefan-Boltzmann constant, s = 5.67 x 10-8 W/m2.K4.

22.	a.	Classify heat exchangers. With the help of a neat sketch, explain the working and 	construction of a 1-2 exchanger.

b.	Explain how the fouling resistance of heat exchanger can be calculated.

(OR)

23. 	A single effect evaporator is required to concentrate a solution from 10% solids to 30% solids at the rate of 250 kg of feed per hour. If the pressure in the evaporator is 77 kPa absolute, and if steam is available at 200 kPa gauge, calculate the quantity of steam required per hour and the area of heat transfer surface if the overall heat transfer coefficient is 1700 J m-2 s-1 °C-1. Assume that the temperature of the feed is 18°C and that the boiling point of the solution under the pressure of 77 kPa absolute is 91°C. Assume, also, that the specific heat of the solution is the same as for water, that is 4.186 x 103 J kg-1°C-1, and the latent heat of vaporization of the solution is the same as that for water under the same conditions.

24.	a.	Oxygen is diffusing in a mixture of oxygen-nitrogen at 1 std atm, 25C.  Concentration of 	oxygen at planes 2 mm apart are 10 and 20 volume % respectively.  Nitrogen is non-	diffusing.

a.	Derive the appropriate expression to calculate the flux oxygen.  Define units of each 	term clearly.

b.	Calculate the flux of oxygen.  Diffusivity of oxygen in nitrogen = 1.89 * 10 –5 m 2/sec.

(OR)

25.	a.	Derive linear concentration profile for equimolar counter diffusion.

b.	Methane diffuses at steady state through a tube containing helium.  At point 1 the partial pressure of methane is p A1 = 55 kPa and at point 2, 0.03 m apart P A2 = 15 kPa.  The total pressure is 101.32 kPa, and the temperature is 298 K.  At this pressure and temperature, the value of diffusivity is 6.75 * 10 –5 m 2/sec.

i.	Calculate the flux of CH 4 at steady state for equimolar counter diffusion.

ii.	Calculate the partial pressure at a point 0.02 m apart from point 1.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ENGINEERING PROPERTIES OF FOOD MATERIALS

Time : 3 hours

Subject Code:	12FP215								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define the term density.

2.	What is meant by sphericity?

3.	Write the Newton’s law of viscosity.

4.	Define kinematic viscosity.

5.	Define heat capacity.

6.	Write the methods of measuring diffusivity.

7.	Define lift co efficient.

8.	What is meant by water activity?

9.	What meant by term food irradiation?

10.	What is meant by microwave heating?

PART – B  (5 x 3 = 15 MARKS)

11.	Explain porosity.

12.	Classify the fluids and explain.

13.	Name some viscometers.

14.	Explain terminal velocity.

15.	Indicate the direct effect of irradiation.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss the effect of size and shape on quality of food products.

(OR)

17.	Explain the frictional properties and coefficient of friction

18.	Discuss on viscosity and momentum flux and   effect of temperature on viscosity of fluids

(OR)

19.	Explain the texture and texture measuring instruments with diagram.

20.	Explain the surface heat transfer coefficient    and indicate the method of finding the same.

(OR)

21.	Explain enthalpy, conductivity and diffusivity and measurement methods of same.

22.	Discuss the application of terminal velocity in   handling and separation of food materials

(OR)

23.	Explain the sorption isotherms in detail

24.	Write note the following :

a.	Future for food irradiation.								     						    (5)

b.	Dose determining factors.							      	  						  (10)

(OR)

25.	a.	Explain the mechanism of microwave heating.   					     	     			    (5)

b.	How microwave heating differs from conventional heating?                                    	   (5)

c.	Explain the principle of ohmic heating.						     			   		    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title : 	DRYING TECHNOLOGY								      Time : 3 hours

Subject Code:	09FP202										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The number of Kg of moisture carried by one Kg of bone-dry air is ____________.

2. 	Define drying.

3. 	Give any two applications of Double-drum dryers.

4. 	What is meant by Osmosis?

5. 	Mention any one dryer which evaporate solutions entirely to dryness by thermal means.

6. 	Give one advantage of using spray dryer.

7. 	Name any one example for Indirect dryers.

8. 	Freeze drying is recommended for ____________ materials.

9. 	Mention the two types of Continuous Fluidized bed dryers.

10. 	The residence time is almost ____________ for all particles in plug flow dryers.

PART – B  (5 x 3 = 15 MARKS)

11.  	Write a short note on falling-rate period.

12.  	Write a short note on Film Drum dryers.

13. 	Heat sensitive materials can be dried in Spray dryers - Explain.

14.  	Explain the essential components of a vacuum freeze-drying plant.

15.  	Define Incipient fluidization.

PART – C  (5 x 15 = 75 MARKS)

16. Discuss in detail about humidity chart and its application in chemical processes.

(OR)

17. 	Write a brief note on (a) Vacuum drying and (b) Critical moisture content and Falling rate period.

18. 	Describe with a neat sketch the construction and working of a Double-drum dryer.

(OR)

19. 	Explain the working principle of Foam Mat Dryer with a neat diagram.

20. 	Discuss in detail about Spray dryer with a neat sketch and also indicates its advantages.

(OR)

21. 	Write a brief notes on (a) Foam Spray drying and (b) Powder separators.

22.  Explain the working principle of Microwave and Dielectric drying.

(OR)

23. 	Explain with a neat sketch the construction and working of a Freeze dryer.

24.  Discuss in detail the working principle of Flash dryer with a neat sketch.

(OR)

25. 	Explain in detail about the working principle of Fluidized bed dryer with a neat diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FOOD SAFETY REGULATIONS & CONTROL		      	     Time : 3 hours

Subject Code:	09FP205								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	When and where was the WHO established?

2.	What is the controversy about WTO?

3.	Give two responsibilities of food business operators.

4.	Who is the licensing authority of food business?

5.	Name two biotechnological products.

6.	Give two advantages of irradiation.

7.	Who are the members of the HACCP team?

8.	Point one critical point in fruit product manufacture.

9.	Name two diseases caused by drinking contaminated water.

10.	Give two regulations for bottled water in India.

PART – B  (5 x 3 = 15 MARKS)

11.	Give the codex standards of maximum residue limits for any one food product.

12.	What are the duties of the food safety officer?

13.	Give the health claims of any one functional food.

14.	Why is it necessary to state a food products’ intended use?

15.	What is the type of package that can be used for water?

PART – C  (5 x 15 = 75 MARKS)

16. 	Give the details of codex standards for three fruit products.

(OR)

17. 	Give an over-view of the operations of WHO.

18. 	Elaborate on the regulations of food analysis laboratories.

(OR)

19. 	Explain the laws pertaining to two food sectors.

20. 	Discuss the rules and regulations relating to genetically modified foods.

(OR)

21. 	Elaborate on the need for labelling and the standards to be followed.

22.	Explain the principles of HACCP.

(OR)

23. 	Explain the sanitary concerns in a food plant and it is maintained.

24. 	Give an account of diseases caused by drinking contaminated water.

(OR)

25. 	Explain in detail how the microbiological quality of water is tested. Give the guideline values for bottled water.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December -2015

Subject Title :	UNIT OPERATIONS IN FOOD PROCESSING II	      	     Time : 3 hours

Subject Code:	09FP206								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give the relationship to calculate the settling velocity of particles.

2.	What is centrifugal separation?

3.	What are the types of filtration processes?

4.	What is filter cake resistance?

5.	List the membrane separation processes.

6.	Why micro filtration is important in membrane separation process?

7.	What is mixing index?

8.	Name the mixing devices suitable for mixing dough and paste.

9.	Name the size reduction equipments working on the principles of impact forces.

10.	What is critical velocity in ball mill?

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between gravity sedimentation and centrifugal sedimentation.

12.	Differentiate between constant rate filtration and constant pressure filtration

13.	Explain membrane fouling and the methods to overcome.

14.	Explain the characteristics of a mixing device.

15.	Write a note on particle size analysis.

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	Derive an expression to find the radius of the neutral zone in a centrifugal separator for two 	immiscible liquids.															  (10)

b. 	If the cream separator has discharge radii of 5 cm and 7 cm and if the density of milk and cream are 1035 and 870 kg/m3, respectively, calculate the radius of neutral zone so that the feed inlet can be designed. 													    (5)

(OR)

17. 	a. 	Explain with a neat sketch the construction and working of a basket type centrifuge and 			give its suitability of application in food industries.								    (5)

b. 	Explain the process of decanting. 												  (10)

18. 	a.	Derive an expression for the discharge rate  in a centrifugal filtration under constant 	volume rate process.															  (10)

b. 	Explain the working of a vacuum filtration system with a neat sketch. 				    (5)

(OR)

19.	a. 	Explain the working of a plate and frame filter press.								  (10)

b. 	Write a note on rotary filters 													   (5)

[P.T.O.]

20.	a.  	Explain the principles of  membrane separation.									   (5)

b.  	Explain the  types of membranes and the materials of construction, used in membrane 	separation process.															  (10)

(OR)

21. 	a. 	With a neat sketch explain the construction and working of a reverse osmosis membrane 	separation equipment. 														  (10)

b. 	Discuss the cleaning process of the membrane on completion of the process. 		    (5)

22. 	a. 	Derive an expression for the mixing index.										    (5)

b. 	Mention the various types of mixers with their suitability of applications. 			  (10)

(OR)

23. 	a. 	Explain the construction and working of a mixing device suitable for powder and particles.    																				  (10)

b. 	Explain the construction and working of a planetary mixer with a suitable diagram.	    (5)

24.	a. 	With a neat sketch explain the working of a jaw crusher.							  (10)

b.  	With a neat sketch explain the working of a hammer mill. 							    (5)

(OR)

25.	a. 	Derive an expression for the critical speed of a ball mill.							  (10)

b. 	A ball mill 1.5 m in diameter is operated at 50 rpm speed and the size of the balls used are 80 mm in diameter. It is found that the mill is not functioning satisfactorily.  What is your suggestion to improve the working condition. 									    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	INSTRUMENTAL TECHNIQUES OF FOOD ANALYSIS													       							       Time : 3 hours

Subject Code: 	10FP302										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

Explain briefly about basic concepts involved in Chromatography technique and its application in food analysis.

2. 	Give brief explanation about FTIR Spectroscopy and its application in food analysis.

(OR)

3.	Explain briefly about the types of detectors used in Gas Chromatography and its application in food analysis.

4. 	Discuss briefly about the application of NMR in Food analysis

(OR)

5. 	Write and explain the Components of an Atomic Absorption spectrometer and its application.

6.	Explain in detail about Nuclear Magnetic Resonance spectroscopy.

(OR)

7.	Write briefly about Atomic Fluorescence Spectrometry and its application.

8. 	Write and explain about LC/MS and FAB/MS.

(OR)

9.  	Give detail account on:

a.	Capillary Gel Electrophoresis (CGE).

b.	Capillary Isotachophoresis (CITP).

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	PROCESS ENGINEERING CALCULATIONS		      	     Time : 3 hours

Subject Code:	11FP202								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Classify the physical quantities.

2.	Define normality.

3.	Write the ideal gas law.

4.	What is meant by psychrometry?

5.	Write the law of conservation of mass.

6.	What is meant by distillation?

7.	What is meant by drying operation?

8.	Differentiate drying from evaporation.

9.	State the first law of thermodynamics.

10.	Define the term heat capacity.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the pressure and energy with units.

12.	State the Boyle’s law and Dalton’s law.

13.	Write the details in a flow sheet.

14.	What are the major respects to evaporate water in a closed cycle dryers?

15.	Define the latent heat of fusion latent heat of vaporization and latent heat of sublimation.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Steam is flowing at the rate of 2000 kg /h in a 3.068 inch pipe at 440 K pa absolute and 	453 K with specific volume 0.462m3/ kg. Calculate the velocity of steam in the pipe line.						 				                  									    (5)

b.	A saturated solution of salicylic acid in methanol contains 64kg salicylic acid per 100kg methanol at 298.15K. Find (a) mass% and (b) mole % composition of the solution.	  (10)

(OR)

17.	a.	A solution of sodium chloride contains 20% Nacl (by mass) at 333K. The density of 	solution is 1.127kg/L. Find the molarity, normality and molality of the solution.             (6)

b.	 Sodium chloride weighing 600kg is mixed with 200kg potassium chloride. Fine the composition of the mixture in (a) mass % (b) mole%                                       		    (9)

18.	Calculate the specific volume of super heated steam at 100 bar and 6213.15K using (a) ideal gas law (b) the Vander walls equation. If the specific volume of steam at the above condition is 0.02242m3 /kg, find the percentage error in the above cases.

Data:Pc=220.76 bar , Tc=647.11K for water

(OR)

19.	Define the following

Dry bulb temperature,Absolute humidity, Percentage humidity ,Relative humidity , Humid heat, Humid volume and dew point .

[P.T.O.]

20.	It is desired  to make 1000kg mixed acid containing 60% H2So4 , 32%HNO3,and 8% water by blending (a) spent acid containing 11.3% HNo3 ,44.4% H2So4 and 44.3% H2o (b) aqueous 90% HNo3 and (c) aqueous 98% H2So4. Calculate the each of three acids required for blending

(OR)

21.	An absorption tower is used to absorb carbon dioxide in an aqueous mono ethanol amine (MEA) solution. The volumetric flow rate of the incoming dry gas mixture is 1000m3 /h at 318 K and 101.3 KPa. The CO2content of the gas is 10.4mole % while the outgoing gas mixture contains 4.5 mole% CO2.

A3.2M MEA solution is introduced at the top of the tower at the rate 0.625 L/S. Dissolved CO2 concentration of the entering solution of .166 K mol /K mol of MEA. Find the concentration of dissolved CO2 in the solution leaving the tower.

Data: specific volume of gas at 318k and 101.3k Pa is 26 .107 m3/k mol

22.	a.	A double effect evaporator system concentrates weak liquor containing 4% (by mass) 	caustic soda to produce a lye containing 25% solids (by mass) calculate the evaporation of 	water per 100kg feed in the evaporator.                                                           		    (4)

b.	A feed water to the reverse osmosis plant has dissolved solids to the extent of 5000mg/L. 	The feed to product ratio (on mass basis) is 4:3 . The treated water (product) from the 	plant contains 600mg /L of solids. Find dissolved solids in the reject stream.          (5)

c.	A thin liquor from the digester is concentrated in a evaporator .The flow of thin liquor is 50 L/S and a solid content of 15% (by mass). The concentrated solution leaves the system at 55 % solids (by mass).The density of thin liquor is 1.08kg/L. The saturated steam is consumed at the rate of 28500 kg /h. Calculate the (i) Total evaporation in the system

(ii) steam economy.                                                                                				    (6)

(OR)

23.	In a textile mill wet cloth passes through a hot air dryer. The cloth enters with 90% moisture and leaves at 6% moisture at a speed of 1.15m/s. The width of the cloth is 120cm and its specific density on bone dry basis is 0.095 kg /m3. The humidity of ambient air enter is 0.018kg of moisture /kg of dry air. The humidity of outlet air is 0.0832 kg of moisture / kg of dry air, the humid volume of entering air is 0.8837 m3/kg dry air. Calculate (a) The bone dry production of dryer. (b) Rate of evaporation in the dryer (c) Rate of air circulation.

24.	a.	Illustrate the step by step procedure of calculating bubble points and dew point of ideal 	mixture. 		                                                                                       		 	    (7)

b.	Discuss on sensible heat changes in gases at constant pressure.                       		    (8)

(OR)

25.	a.	Tolune is heated from 290 K to 350 K at the rate 0.25kg/s.   Calculate   the heat   	required to  be   added  to  Tolune  using  the  constants

a = 1.8083, b x 103= 812.223,c x 106= -1512.67, d x 109 = 1630.01.					    (7)

b.	For O- xyline  calculate (i) latent heat of vaporization at 417.5 K using Ridel equation

(ii) latent heat of vaporization at 298.15 K using Watson equation Data :Pc=3730 k Pa and Tc = 630.3 K                                                                 		    (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	PRINCIPLES OF FLUID MECHANICS AND HEAT TRANSFER													      	     							      Time : 3 hours

Subject Code:	11FP204								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What do you mean by vapour pressure?

2.	What are the disadvantages of pitot tube?

3.	State zeroth law of thermodynamics.

4.	What is meant by extensive property?

5.	What is sublimation curve?

6.	What is meant by  adiabatic process?

7.	Why does the thermal conductivity of gases increases with temperature?

8.	Give any two examples of one dimensional steady state conduction type heat transfer.

9.	Define Prandtl number.

10.	Define the term reflectivity.

PART – B  (5 x 3 = 15 MARKS)

11.	Briefly explain the working principle of venturi meter.

12.	Explain why you cannot cool your kitchen by leaving your refrigerator door open on a hot day. (Why does turning on a room air conditioner cool down the room, but opening a refrigerator door does not?)

13.	Define wet steam and superheated steam.

14.	State Fourier’s law of heat of conduction.

15.	Define hydrodynamic boundary layer or velocity boundary layer.

PART – C  (5 x 15 = 75 MARKS)

16.	Water with a density of 998 kg/m3 (62.3 lb/ft3) enters a  50 mm (1.969 –in.) pipe horizontally, as shown in Figure, at a steady velocity of 1.0m/s (3.28ft/s) and a guage pressure of 100 kN/m2 ( 2088.5 lb/ft2). It leaves the fitting horizontally, at the same elevation, at an angle of 45° with the entrance direction. The diameter at the outlet is 20 mm (0.787 in.) Assuming the fluid density is constant the kinetic energy and momentum correction factors at both entrance and exit are unity, and the friction loss in the fitting is negligible, calculate (a) the guage pressure at the exit of the fitting and  (b) the forces in the  and Y directions exerted by the fitting on the fluid.

[P.T.O.]

(OR)

17.	a.	A water tank is 30 ft in diameter and the normal depth is 25 ft. The outlet is a 4-in. 	horizontal pipe at the bottom. If this pipe is sheared off close to the tank, what is the initial 	flow rate of water from the tank? (Neglect friction loss in the short stub of pipe.) (Cross-	sectional area = 0.0884 ft2).

b.	How long will it take for the tank to be empty?

c.	Calculate the average flow rate and compare it with the initial flow rate.

18.	a.	A steady-state flow water power plant has its water inlet 15m above its water outlet. The 	water enters the plant with a velocity of 3 m/s and leaves with a velocity of 10 m/s. What is 	the work done by the plant per kilogram of water passing through it? (Assume there is no 	heat transfer)

b.	A Carnot engine works between two heat reservoirs at temperatures Th= 300 K and Tc= 	200 K.

(i) What is its efficiency?

(ii) If it absorbs 100 J of heat from the hot reservoir during each cycle, how much work does it do each cycle?

(iii) How much heat does it release during each cycle?

(iv) What is the COP of this engine when it works as a refrigerator between the same two reservoirs?

(OR)

19.	a.	State first law of thermodynamics and show that perpectual motion mechine of the first 	kind is impossible

b.	A simple steam power cycle receives 100,000 kJ/min as heat transfer to the working fluid 	at 800K and rejects energy as heat transfer from the working fluid at 32K. If the pump 	power required is 1400 kJ/min, determine

(i) The thermal efficiency of an internally reversible cycle and

(ii) The turbine power output in kilowatts

20.	Consider a mole of a gas initially at 1(P1, V1) and finally at 2(P2, V2). Since S2-S1 is path independent, choose the simple path shown in figure by first changing pressure at constant volume and then volume at constant pressure. Let 0 (P0, T0) be the intermediate point you go through. Show that

S2−S1=CplnT2T−RlnP2P1.

0

2

1

V

P

Show that 1 and 2 lie on an adiabatic curve, this difference vanishes. Assume Cp = CV + R, but not a particular value to CV.

(OR)

21.	A balloon filled with 0.505 mol of gas contracts reversibly from 1.0 L to 0.10 L at a constant temperature of 5.0 °C. It simultaneously loses 1270 J of heat. Calculate q, w, ΔU, and ΔH for the process

22.	a.	Derive the equation for steady state heat transfer through a thick walled cylinder.

b.	Discuss in detail the three modes of heat transfer.

(OR)

23.	A flat furnace wall is constructed of a 4.5 in. (114 mm) layer of soil- o-cel brick, with a thermal conductivity of 0.08 Btu/ft-h-°F  (0.138 W/m-°C) backed by a 9 in. (229 mm) layer of common brick, of conductivity 0.8 Btu/ft-h-°F (1.38 W/m-°C). The temperature of the inner face of the wall is 1400°F (760°C), and that of the outer face is 170°F (76.6°C).  (i) What is the heat loss through the wall?  (ii)  What is the temperature of the interface between the refractory brick and the common brick? (iii) Supposing that the contact between the two brick layers is poor and that a “contact resistance” of 0.50°F-h-ft2/Btu (0.088°C-m2/W) is present, what would be the heat loss?

24.	a.	Air at 300°C flows over a flat plate of dimensions 0.50 m by 0.25 m. If the convection heat 	transfer coefficient is 250 W/m2K, determine the heat transfer rate from the air to one side 	of the plate when the plate is maintained at 40°C.

- A thick soup is being boiled in a pan and because of inadequacy of stirring a layer of soup builds up on the bottom of the pan to a thickness of 2 mm. The hot plate is at an average temperature of 500°C, the heat-transfer coefficient from the plate to the pan is                   600 J m-2 s-1 °C-1, and that from the soup layer to the surface of the bulk soup is               1400 J m-2 s-1 °C-1. The pan is of aluminium 2 mm thick. Find the temperature between the layer of soup and the pan surface. Assume the thermal conductivity of the soup layer approximates that of water. (thermal conductivity of the stagnant thick soup layer k ≈ 0.5 Jm-1s-1 oC-1 and for aluminium k = 220 Jm-1s-1 oC-1)

(OR)

25.	Oil at 50°F is heated in a horizontal 2-in Schedule 40 steel pipe 60 ft long having a surface temperature of 120°F. The oil flow rate is  150gal/h at inlet temperature. What will be the oil temperature as it leaves the pipe and after mixing? What is the average heat-transfer coefficient? Properties of the oil are given in the following table: (Flow is laminar)

|  | 60°F | 120°F |
| --- | --- | --- |
| Specific gravity, 60°F/60°F | 0.79 | 0.74 |
| Thermal conductivity, Btu/ft-h-°F | 0.072 | 0.074 |
| Viscosity, cP | 18 | 8 |
| Specific heat, Btu/lb-°F | 0.75 | 0.75 |

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FOOD BIOCHEMISTRY AND NUTRITION			      	     Time : 3 hours

Subject Code:	11FP205								 		      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is ETC?

2. 	Give an example of high energy compound.

3. 	Give any two examples of pyrimidine bases present in DNA.

4. 	What is PUFA? Give example.

5. 	Give the energy value of carbohydrate and protein.

6. 	What are essential aminoacids?

7. 	What are lectins?

8. 	Name any one inborn disease of liver?

9. 	What are functional foods?

10.	List the types of diabetic.

PART – B  (5 x 3 = 15 MARKS)

11. 	Give an account of gluconeogenesis.

12. 	State the role of carnitine in fat metabolism.

13. 	Explain the basic concept of nutrition.

14. 	What are ANF’s? Explain with suitable example.

15. 	What is ageing?

PART – C  (5 x 15 = 75 MARKS)

16. 	Enumerate the urea cycle.

(OR)

17. 	Explain the metabolic interrelationships between the processes.

18. 	Explain the process of fatty acid bio synthesis.

(OR)

19. 	Explain the cholesterol biosynthesis.

20. 	Discuss the various causes of malnutrition and its problems.

(OR)

21. Explain the effects of processing on protein quality.

22. Write an essay on inborn errors of carbohydrate metabolism.

(OR)

23. 	Give an account of naturally occurring anti-nutritional factors.

24. 	Explain the various theories of ageing.

(OR)

25. 	Write an essay on sports nutrition and geriatric nutrition.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	TECHNOLOGY OF MEAT, POULTRY AND FISH PROCESSING													      	     						      Time : 3 hours

Subject Code:	11FP210										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The average water content in meat is __________% of the weight of the muscle.

2.	The muscle in meat is subdivided into bundles of muscle cells also known as __________.

3.	Intensity of electric shock used for stunning of animals must be no less than __________ mA.

4.	The average temperature of a cooler used in storage of carcasses must be between __________ and __________°C.

5.	__________ addition to meat is responsible for the distinct color and flavor of products such as ham and bacon.

6.	Meats frozen in a cold room is typically at temperatures between __________ and __________0C.

7.	Aerobic storage of poultry meat under chill conditions leads to the development of a microflora termed __________.

8.	Lean poultry meat refers to proceed poultry meat that is low in __________.

9.	__________ has been the most common method used to immobilize poultry for slaughter in poultry-processing plants.

10.	Small pelagic fish species are normally stored in a tank system with __________.

PART – B  (5 x 3 = 15 MARKS)

11.	How does raw meat get its color?  What is the coloring pigments of raw meat?

12.	What are the main stunning methods used in the meat industry?

13.	What does the term curing refer with respect to processed meat?

14.	What does the term scalding refer with respect to poultry processing?

15.	What does the term skinning refer with respect to fish processing?

PART – C  (5 x 15 = 75 MARKS)

16.	What are the major postmortem changes to muscle after the slaughter of animals?

(OR)

17.	Describe muscle metabolism with emphasis on ATP production in muscles.

18.	Write a descriptive note on the stunning of animal prior to slaughtering.

(OR)

19	Write a note on carcass processing with flowcharts.

20.	Describe the various methods used for the curing of meat.

(OR)

21.	Write a detailed note on HACCP in meat industries.

22.	Discuss the structure, composition and nutritive value of egg.

(OR)

23.	Write a note on modern poultry processing.

24.	Write a detailed note on the manufacture of fish meal and fish oil.

(OR)

25.	Elucidate the pre-cooking method of canning of fish.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FOOD CHEMISTRY								      	     Time : 3 hours

Subject Code:	11FP212						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define water activity.

2.	Name two structural polysaccharides and two storage polysaccharides.

3.	Give two property of pectins.

4.	Differentiate dietary fibre from crude fibre.

5.	Name two confectionary fats.

6.	Write the chemical structure of a triglyceride.

7.	Define denaturation of protein.

8.	Name two enzymes used in the fruit processing industry.

9.	What are the various ways in which vitamin C is lost in food preparation?

10.	Name two food sources of vitamin A.

PART – B  (5 x 3 = 15 MARKS)

11.	Write the structure of water and ice.

12.	Give two properties of maltodextrins.

13.	How is oil hydrogenated?

14.	Give two physical properties of proteins.

15.	List two functions of vitamin B2.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Explain four chemical properties of glucose. 									    (8)

b. 	Classify the mono saccharides with one example each. 							    (7)

(OR)

17.	Explain three types of sorption isotherm, with one example each. 					      (9+ 6)

18. 	Give an account of three properties of dietary fibre.

(OR)

19. 	Explain five properties of starch.

20. 	With a process flow sheet explain the manufacture of edible oil. 					     (5+10)

(OR)

21. 	Explain five chemical characteristics of fats and oils.

22. 	Elaborate on the isolation and identification of proteins. 								(8+7)

(OR)

23. 	a.	Explain three techniques for the immobilization of enzymes. 						    (9)

b. 	Elaborate on the chemical nature of enzymes. 									    (6)

24. 	Explain the functions and bioavailability of thiamine. 								(9+6)

(OR)

25. 	Elaborate on the chemistry and functions of vitamin D. 								(7+8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FRUIT AND VEGETABLE PROCESSING TECHNOLOGY

Time : 3 hours

Subject Code:	11FP214								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.  	Why spoilage of fruits and vegetables occur?

2.  	What is cut fruits and cut vegetables?

3.  	Define blanching.

4.  	Give the TSS of jam and squash.

5.  	What is clarification of juice?

6.  	What are the advantages of fruit powders?

7.  	What are intermediate moisture foods?

8.  	Why dehydration of fruits and vegetables are followed?

9.  	What is aseptic packaging?

10. 	What is minimal processing?

PART – B  (5 x 3 = 15 MARKS)

11. 	Name the factors causing spoilage in fruits and vegetables.

12. 	Explain the process of production of tomato ketchup.

13. 	Explain the hot and cold break processes.

14. 	Explain the process of freeze drying.

15.  	Explain the importance of aseptic conditions required in fruit processing units and name the methods of achieving.

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	Explain the field level post harvest operations performed for fruits and vegetables.	   (5)

b.  	Write a note on the packaging methods and the packaging materials suitable for fruits and whole fruits and vegetables for domestic and export markets.						  (10)

(OR)

17. 	a. 	Explain the process of cut fruits and vegetables. 									    (5)

b. 	Name the various preservation treatments for the freshly harvested fruits and vegetables to enhance shelf life.															  (10)

18.	a. 	Give the importance of washing of fruits and vegetables prior to use in processing.	   (5)

b.  	Explain the process of canning of fruits and vegetables. 				     			  (10)

(OR)

19.	a. 	Explain the concept of hurdle technology. 										   (5)

b. 	Explain the filling, closing and sterilization operations followed in fruit processing units to increase the production capacity.  												  (10)

20.	a. 	Explain the hydraulic pressing method of pulping of fruits with a neat sketch.		  (10)

b. 	Discuss the process of production of fruit powders.								   (5)

(OR)

[P.T.O.]

21.	a. 	Explain the process of production of mango pulp and concentration at industry level.    (10)

b.  	Explain the concept of fruit juice aroma recovery and its importance. 				    (5)

22.	a. 	What are the advantages of vacuum drying?										    (5)

b. 	Explain the working of a spray dryer with neat sketch and give its suitability.		  (10)

(OR)

23.	a. 	List the merits and demerits of freeze drying.									    (5)

b. 	Explain the working of a drum dryer with neat sketch and give its suitability.		  (10)

24.	a. 	Explain the working of a heat exchanger used for sterilizing and concentrating the fruit 	pulp. 																		  (10)

b. 	Explain the methods of filling of fruit juice products under aseptic conditions. 	     	    (5)

(OR)

25.	a. 	Explain the methods of bulk storage of juice/ pulp in bag and boxes, and bag and drums.

(10)

b. 	What are the storage requirements for the storage of aseptically packaged products? 	   (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FOOD PACKAGING TECHNOLOGY				      	     Time : 3 hours

Subject Code:	11FP218										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is shelf life?

2.	Differentiate packaging and packing.

3.	Why annealing is done in glass manufacturing?

4.	What are the colouring agents used for glass?

5.	Define flexible packaging.

6.	What are the advantages of Corrugated Fibre Board (CFB)?

7.	Expand HDPE and LDPE.

8.	Distinguish lap seal and fin seal.

9.	What do you understand by oxygen scavenging?

10.	Define tensile strength.

PART – B  (5 x 3 = 15 MARKS)

11. 	Write a short note on vacuum packaging.

12. 	Explain Drawn and Iron method of Can manufacturing.

13. 	Explain the importance of retort pouch packaging.

14. 	Enumerate the advantages of impulse sealing.

15. 	Write a note on active packaging.

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain various testing methods used for testing packaging materials.

(OR)

17.  	Explain in detail the functions of packaging material.

18. 	a. 	Explain the various steps involved in three-piece Can manufacture.					  (10)

b. 	Write the advantages and disadvantages of metal cans.				          		    (5)

(OR)

19.  a. 	Explain the various steps involved in three-piece can manufacture. 					  (10)

b. 	Explain the basic operations involved in canning of foods. 		            			    (5)

20. 	Explain in detail the various types of polyester film used in flexible packaging.

(OR)

21. 	Give a detailed note on the manufacturing methods of corrugated and solid paper board for food packaging.

22. 	Explain the construction and working of form, fill and sealing machine with a neat sketch.

(OR)

23. 	Describe different filling methods and equipments for liquid and solid products.

24. 	Describe the principle and applications of modified atmospheric packaging with suitable example.

(OR)

25. 	Write the classification, principle and procedure for aseptic packaging system with suitable diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	MILLING AND BAKERY TECHNOLOGY			      	     Time : 3 hours

Subject Code:	11FP219								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Amylograph is used for _________.

2.	Ferinograph is used for _________.

3.	Atta is the _________.

4.	Wheat has _________ % of endosperm.

5.	Proofing is used for _________.

6.	Baking is carried out at _________oC

7.	_________ shortening is used in baking of foods.

8.	_________ enzyme is used for the preparation of modified dough.

9.	Middlings are obtained from _________.

10.	The cooking temperature of toffee is _________oC

PART – B  (5 x 3 = 15 MARKS)

11.	How to measure the hardness of the wheat grain?

12.	Mention the uses of plan sifter.

13.	Write a short note on pneumatic conveying.

14.	Write about dough retarding.

15.	Mention about layered products.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss the lab testing methods used for wheat grain.

(OR)

17.	Discuss the lab testing methods used for wheat flour.

18.	With a schematic diagram explain the cleaning operations used before wheat milling.

(OR)

19.	With a flow chart, explain the wheat milling operations.

20.	Discuss in detail about the types and methods of preparing dough’s.

(OR)

21.	Discuss the different types of packaging materials used in bakery and confectionery items.

22.	Write the bread manufacturing process in detail.

(OR)

23.	Enumerate the specialty fermented goods in detail.

24.	Enumerate the biscuit manufacturing process in detail.

(OR)

25.	Discuss the process of making different types of cakes.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	PLANTATION PRODUCTS AND SPICES TECHNOLOGY													      	     								      Time : 3 hours

Subject Code:	11FP220										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Enlist the chemical constituents in coffee.

2.	Give the Process flow sheet for the manufacture of Instant coffee powder.

3.	Mention the types of tea.

4.	What are the Grades of tea?

5.	Write the methods of fermenting cocoa.

6.	Define conching.

7.	Give the Process flow sheet for the manufacture of Cardamom.

8.	Differentiate between Oleoresins and essential oils.

9.	Write the process flowchart for Clove processing.

10.	What is meant by flavoridenticals?

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the structure of coffee cherry with neat diagram?

12.	Differentiate between green tea and ctc tea.

13.	Discuss Instant tea manufacturing.

14.	Elucidate on cocoa liquor manufacture?

15.	How is white pepper manufactured?

PART – C  (5 x 15 = 75 MARKS)

16.	 Elaborate on the manufacture of instant coffee powder with flowchart.

(OR)

17.	Discuss the processing of coffee bean from harvesting to drying.

18.	Describe the Chemistry and technology of CTC tea manufacturing process

(OR)

19. 	Elucidate on the manufacturing process for oolong tea with flowchart.

20. 	Explain briefly on the processing of cocoa powder with a flow sheet.

(OR)

21.	Enunciate on the technology of chocolate manufacturing with process line.

22.	Explain in detail about turmeric processing?

(OR)

23.	Explain in detail about extraction of volatiles from chilly.

24. 	Describe the Vanilla processing steps in detail with process sheet.

(OR)

25. 	Elucidate on the Synthesis of volatiles using micro-organisms.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	ENGINEERING PROPERTIES OF FOOD MATERIALS													      	 									      Time : 3 hours

Subject Code:	11FP221								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the formula to calculate porosity from true and bulk density.

2.	How the volume of irregular fruit is measured?

3.	Give two examples of non-newtonian food products.

4.	List any two food textures used to represent snack products.

5.	Write the formula for thermal conductivity.

6.	Expand DSC.

7.	List any two applications of pneumatic handling in food industries.

8.	What is meant by drag?

9.	Mention any two examples of dielectric materials.

10.	Give any two applications of microwaves in food industries.

PART – B  (5 x 3 = 15 MARKS)

11.	What is meant by critical moisture content? When the product attains its equilibrium moisture content?

12.	Distinguish between thixotropic fluids and rheopectic fluids.

13.	List the methods used for the determination of thermal diffusivity.

14.	List the possible chances for the presence of foreign materials in bulk grains and on what basis, the separation of these foreign materials takes place?

15.	Relate the mechanism of heat development and polar molecules inside the food product during microwave heating?

PART – C  (5 x 15 = 75 MARKS)

16.	Elaborate the direct methods of determination of moisture content.

(OR)

17.	a.	How will you determine the angle of repose of paddy grains? Mention with a neat sketch 	and necessary formulae. 														    (8)

b.	Describe the method to measure the volume and density of fruit samples with neat sketch.

(7)

18.	Classify the broad groups of Rheology and describe the terms with suitable examples.

(OR)

19.	Recall the working principles of capillary flow viscometers with neat sketch.

20.	Describe the following methods to determine the thermal conductivity of materials.

a.	Heat of vaporization method and

b.	DSC method with necessary diagram and formulae.

(OR)

21.	Elaborate the method of comparison calorimeter to find the specific heat of the given materials.

[P.T.O.]

22.	Suggest the suitable materials that can be dried using fluidized bed dryer and also analyze the necessary conditions for the velocity of air used in a fluidized bed dryer so that it keeps the product in fluidized state.

(OR)

23.	Explicate the applications of aerodynamic and hydrodynamic properties of foods in food processing industries.

24.	How microwave oven helps in cooking of food product? Elucidate the construction and working of principle a domestic microwave oven.

(OR)

25.	Describe the coaxial probe method and cavity perturbation method to determine the dielectric properties of foods.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FOOD REFRIGERATION & COLD STORAGE CONSTRUCTION													      	     						      Time : 3 hours

Subject Code:	11FP229								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Ton of refrigeration.

2.	What is meant by psychrometry?

3.	Mention any two properties of insulating materials.

4.	Enlist any four examples of refrigerants.

5.	Give the need for defrosting.

6.	Write the principle of working of MAS.

7.	Differentiate between primary and secondary refrigerants.

8.	Define Evaporative cooling.

9.	What is meant by freezing rate?

10.	State any two applications of Individual quick freezing.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the characteristics of different refrigerants.

12.	Describe the properties of insulating materials.

13.	Write the advantages of Controlled atmosphere storage.

14.	Elucidate on the design considerations for chillers and chilled Storages.

15.	Explain the principle and working of blast freezing.

PART – C  (5 x 15 = 75 MARKS)

16.	Elaborate on the Components of a Refrigeration system with a neat diagram.

(OR)

17. 	Discuss the properties of atmospheric air .Give the significance of psychometrics in food processing.

18. 	Describe the construction and working of Refrigerated container trucks and its applications.

(OR)

19.	Elucidate on the construction and working of air diffusion equipment with neat sketch.

20.	Explain briefly on the principle and construction of Modified atmosphere storage.

(OR)

21.	Enunciate on the concept of cold chain supply with applications.

22.	Explain in detail about evaporative cooling and its construction with neat diagram. Discuss the applications.

(OR)

23.	Explain in detail about chilled foods microbiology.

24. 	Describe the construction and working of Cryogenic Freezing with applications.

(OR)

25. Elucidate on the construction and working of conveyorized quick freezers.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	THERMODYNAMICS								      	     Time : 3 hours

Subject Code:	12FP202						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is meant by thermodynamic system?

2. 	Differentiate closed and open system.

3. 	State the First law of thermodynamics

4. 	Define the term enthalpy.

5. 	Define an isentropic process.

6. 	Write down the characteristic gas equation.

7.	State the Kelvin – Plank statement of second law of thermodynamics?

8.	What you mean by air standard diesel cycle?

9.	Define entropy of a pure substance?

10.	Define Intensive and Extensive properties?

PART – B  (5 x 3 = 15 MARKS)

11. 	State Zeroth law of Thermodynamics?

12. 	What is the difference between Otto and diesel cycle?

13. 	Explain Ideal Rankin cycle.

14. 	Write short notes on Joule Thomson effect.

15. 	What is Quasi-Static process?

PART – C  (5 x 15 = 75 MARKS)

16.	An existing process consists of two steps:(a) One mole of air T1 = 900 K and P1 = 3 bar is cooled at constant volume to T2 = 300 K.(b) The air is then heated at constant pressure until its temperature reaches 900 K. It is proposed to replace this two-step process by a single isothermal expansion of the air from 900 K and 3 bar to some final pressure P. What is the value of P that makes the work of the proposed process equal to that of the existing process? Assume mechanical reversibility and treat air as an ideal gas with Cp = 7R/2 and Cv = 5R/2.

(OR)

17.	220 kg of CO2 gas at 27°C and 1 atm is compressed adiabatically to 1/5th of its volume. It is then cooled to its original temperature at constant volume. Find Q, ΔU and W for each step and for the entire process.

18.	a.	Liquid water at 100°C and 1 bar has an internal energy (on an arbitrary scale) of 419 kJ/kg 	and a specific volume of 1.044 cm3/g.(a) What is its enthalpy?(b) The water is brought to 	the vapor state at 200°C and 800 kPa, where its enthalpy is 2838.6 kJ/kg and its specific 	volume is 260.79 cm3/g. Calculate ΔU and ΔH for the process.

b.	A 28 liter rigid enclosure contains air at 140 kPa and 20°C. Heat is added to the container 	until the pressure reaches 345 kPa. Calculate the heat added.

(OR)

[P.T.O.]

19.	a.	Write short notes polytropic process.

b.	A thermodynamic system undergoes a cycle composed of a series of three processes for which Q1 = +10 kJ, Q2 = +30 kJ, Q3 = -5 kJ. For the first process, ΔE = +20 kJ, and for the third process, ΔE = -20 kJ. What is the work in the second process, and the net work output of the cycle?

20. 	a.	State Carnot theorem. Explain Carnot cycle in detail. List out the advantages and 	disadvantages of Carnot cycle.

(OR)

21.	a.	Write down different Maxwell relation.

b.	Using an appropriate cyclic rule and an appropriate Maxwell relation, show that

(∂P / ∂V)S = γ (∂P / ∂V)T where γ = CP / CV

22.	Explain Clapeyron-Clausius Equation and also give its derivation.

(OR)

23.	a. 	Calculate the change in entropy when 10 kg of air is heated at constant volume from a 	pressure of 101325 N/m2 and a temperature of 20°C to a pressure of 405300 N/m2.

CV =20.934 kJ/kmol.° C.

b. 	A heat exchanger uses 5000 kg/hr of water to cool hydrocarbon oil from 140°C to 65°C. The oil, flowing at the rate of 2500 kg/hr has an average specific heat of 0.6 kcal/kg.°C. The water enters at 20°C. Determine

i.	The entropy change of the oil.

ii.	Entropy change of water.

24.	One kilogram of air is heated reversibly at constant pressure from an initial state of 300 K and 1 bar until its volume triples. Calculate W, Q, ΔU, and ΔH for the process. Assume air obeys the relation PV/T = 83.14 bar cm3 mol-1 K-1 and that Cp = 29 J mol-1 K-1.

(OR)

25.	a. 	Explain the concept of Fugacity?

b.	From the following vapor pressure data, construct the temperature - composition diagram 	at 1 atm, for the system benzene-toluene, assuming ideal solution behavior.

| Temperature °C | Vapor pressure, mm Hg |  |
| --- | --- | --- |
|  | Benzene | Toluene |
| 80 | 760 | 300 |
| 92 | 1078 | 432 |
| 100 | 1344 | 559 |
| 110.4 | 1748 | 760 |

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	TECHNOLOGY OF FRESH CUT FRUITS AND VEGETABLES													       							Time : 3 hours

Subject Code: 	15FP3002										  Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

Write notes on Physiological effects of cutting on tissues.

a.	Secondary metabolites accumulation.

b.	Respiration.

c.	Factors Affecting Response to Cutting.

d.	Enzymatic effect on flavor and texture.

2.	Give brief explanation on microbial enzymes associated with fresh cut produce.

(OR)

3.	Give brief explanation on effect of microbial growth on quality of fresh cut salad.

4.  	Explain in brief about sensory quality evaluation of fresh cut produce.

(OR)

5.	Write notes on modified atmospheric packaging of fresh cut produce.

6. 	Write notes on safety and sanitation standards for fresh cut produce.

(OR)

7.	Explain in brief about processing of fresh cut produce.

8. 	Explain in detail about pre and post harvest factors affecting the quality of fresh fruits and vegetables.

(OR)

9.	Give brief explanation on the ways to improve the industry size and market size of freshcut produce.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	PLANTATION PRODUCTS AND SPICES TECHNOLOGY

Time : 3 hours

Subject Code:	12FP217								 	  	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Flavor development in coffee processing is during _________ operations.

2.	Instant coffee is obtained  by drying in __________.

3.	Partially fermented tea is known as _________.

4.	CTC means __________.

5.	Pepper oil is obtained by ___________ process.

6.	Oleoresin is obtained by __________ process.

7.	__________ alkaloid in pepper is responsible for biting state of pepper.

8.	__________ apparatus is used for the moisture determination of spices .

9.	The alkaloid present in cardamom is __________.

10.	The volatile and non-volatile component of spice is ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	Mention various parts of a coffee bean.

12.	Mention the processing effects on tea polyphenol content.

13.	Briefly explain the process of making cocoa powder.

14.	Mention the products of pepper.

15.	Mention the essential oil from garlic.

PART – C  (5 x 15 = 75 MARKS)

16. 	With flow chart, explain the wet and dry method of manufacture of parchment coffee.

(OR)

17. 	Explain the process of roasting, grinding and blending of coffee beans.

18. 	Discuss the method of manufacture of black tea under CTC process.

(OR)

19. 	Explain the types of tea. Also explain the process of making green tea.

20. 	With diagram, explain the dryers used for processing of cocoa.

(OR)

21. 	Write a short note on Processing of cocoa

22. 	Explain the methods of manufacture of essential oil and oleoresin from pepper and ginger.

(OR)

23. 	Explain about the turmeric boiling, drying and polishing operations.

24. 	Explain the complete processing scenario of garlic

(OR)

25. 	Explain the methods of manufacture of volatiles from minor spices

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	PROCESS EQUIPMENT DESIGN  					      	     Time : 3 hours

Subject Code:	12FP224										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Poisson’s ratio.

2.	What are pressure vessels?

3.	Name any two materials commonly used for the construction of storage vessel.

4.	What is mechanical fatigue?

5.	What are the two types of reaction vessels according to heating type?

6.	Give any one method used to control the corrosion of underground steel storage tank?

7.	Define effectiveness of heat exchanger.

8.	Give any two application of evaporation in food processing.

9.	How are dryers classified based on operation types?

10.	How is heat transferred to food in a tray dryer?

PART – B  (5 x 3 = 15 MARKS)

11.	What are the different parts of a pressure vessel?

12.	Why stiffeners are always recommended in storage vessels?

13.	Write short note on half coil jackets.

14.	Find the Overall heat transfer coefficient for a shell and tube counter flow heat exchanger where the heat exchanged is 11.27 GJ/hour with the heat transfer area of 96 m2. Assume the LMTD as 76°C.

15.	What are the steps involved in the algorithm for the selection of industrial dryers?

PART – C  (5 x 15 = 75 MARKS)

16.	A long steel tube, 7.5 cm internal diameter and 0.15 cm thick, has closed ends, and is subjected to an internal fluid pressure of 3 MN/m2. If E = 200 GN/m2, and v = 0.3, estimate the percentage increase in internal volume of the tube.

(OR)

17.	A cylindrical steel pressure vessel 400 mm in diameter with a wall thickness of 20 mm, is subjected to an internal pressure of 4.5 MN/m2. (a) Calculate the tangential and longitudinal stresses in the steel. (b) To what value may the internal pressure be increased if the stress in the steel is limited to 120 MN/m2? (c) If the internal pressure were increased until the vessel burst, sketch the type of fracture that would occur.

18.	Briefly explain different types of storage tanks.

(OR)

19.	There is an oxygen cylinder which has been kept in storage for a long time, with Do=219mm using 40Mn2A and rolled by seamless steel. The actual Sn= 6.5mm and sb = 784.8MPa,            ss = 510.12MPa, d5 = 18%, the design T is normal T. If the working pressure Pw=15MPa, try to find whether the thickness is enough or not. If not, what is the maximum allowable working pressure in this cylinder? (for seamless steel, f = 1.0)

[P.T.O.]

20.	Design the thicknesses of cylinder and heads of a storage tank. Calculating respectively the thickness of each heads if it’s semi-spherical, elliptical, dished and flat head as well as comparing and discussing the results.

Known: Di = 1200 mm     Pc = 1.6Mpa     Material: 20R     [s]t = 133Mpa     C2 = 1mm

The heads can be punch formed by a complete steel plate. (Negative deviation of plate and tube for cylindrical, semi-spherical, elliptical, dished and flat heads are 0.8mm, 0.5mm, 0.8mm and 0.8mm respectively)

(OR)

21.	A solution of NaOH in water is prepared by diluting a concentrated solution in an agitated, jacketed, vessel. The strength of the concentrated solution is 50 percent w/w and 2500 kg of      5 percent w/w solution is required per batch. Calculate the heat removed by the cooling water if the solution is to be discharged at a temperature of 25°C. The temperature of the solutions fed to the vessel can be taken to be 25°C.

22.	a.	Milk is flowing into a pipe cooler and passes through a tube of 2.5 cm internal diameter at 		a rate of 0.4 kg s-1. Its initial temperature is 49°C and it is wished to cool it to 18°C using a 		stirred bath of constant 10°C water round the pipe. What length of pipe would be required? 		Assume an overall coefficient of heat transfer from the bath to the milk of

900 J m-2 s-1 °C-1, and that the specific heat of milk is 3890 J kg-1 °C-1.				    (7)

b.	In a counter flow heat exchanger, water is being chilled by sodium chloride brine. If the rate of flow of the brine is 1.8 kg s-1 and that of the water is 1.05 kg s-1, estimate the temperature to which the water is cooled if the brine enters at -8°C and leaves at 10°C, and if the water enters the exchanger at 32°C. If the area of the heat-transfer surface of this exchanger is 55 m2, what is the overall heat-transfer coefficient? Take the specific heats to be 3.38 and 4.18 kJ kg-1 °C-1 for the brine and the water respectively.				    (8)

(OR)

23.	Air enters a direct expansion type, fin-and-tube evaporator at a temperature of 17°C and leaves the evaporator at 11°C. The evaporator operates at a constant temperature of 7°C and has total refrigerant side area of 12m2, while the bare tube and finned areas on airside are 10m2 and 212m2, respectively. Find the refrigeration capacity of the evaporator assuming only sensible heat transfer on airside and counter flow type arrangement. Neglect fouling and resistance offered by the tube wall. The fin effectiveness for airside is 0.75. The average heat transfer coefficient on refrigerant and airside are 1700 W/m2.K and 34 W/m2.K, respectively.

24.	Briefly explain the design of rotary dryer and derive the expression for material balance and thermal energy requirements.

(OR)

25.	a.	Cabbage containing 89% of moisture is to be dried in air at 65°C down to moisture	content on a dry basis of 5%. Calculate the heat energy required per tonne of raw cabbage 	and per tonne of dried cabbage, for the drying. Ignore the sensible heat.				  (12)

b.	The efficiency of a spray dryer is given by the ratio of the heat energy in the hot air supplied to the dryer and actually used for drying, divided by the heat energy supplied to heat the air from its original ambient temperature. Calculate the efficiency of a spray dryer with an inlet air temperature of 150°C, an outlet temperature of 95°C, operating under an ambient air temperature of 15°C. Suggest how the efficiency of this dryer might be raised.																			    (3)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FOOD ADDITIVES									      	     Time : 3 hours

Subject Code:	12FP226						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define food additive.

2.	Write any four functions of food additives.

3. 	Define anti-oxidant.

4. 	Define emulsifiers.

5. 	What are Humectants?

6. 	Name any two humectants used in food.

7. 	Name any two synthetic colors.

8. 	Define flavor enhancers.

9. 	Name any two nutrient supplements added to food.

10. 	What are Chelating agents?

PART – B  (5 x 3 = 15 MARKS)

11. 	Differentiate intentional and unintentional additives.

12. 	Write a note on food additives classification.

13. 	Explain origin of emulsifiers.

14. 	Write a note on application of humectants.

15. 	Write a note on limitations of natural colour.

PART – C  (5 x 15 = 75 MARKS)

16. 	Write a detailed note on application of food additives.

(OR)

17. 	Explain general principles for use of food additives.

18. 	Discuss acidulants and its functions.

(OR)

19. 	Discuss (a) Emulsifiers (b) Gums.

20. 	Discuss in detail about humectants as food additives.

(OR)

21. 	Write a detailed note on functional effect of humectants on dough.

22. 	Discuss in detail about food colourants and current regulations.

(OR)

23. 	Write a note on fat substitutes and replacers.

24. 	Discuss in detail about natural and synthetic sweetners.

(OR)

25. 	Write a detailed note on nutritional additives.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	TECHNOLOGY OF MEAT, POULTRY AND FISH      	     Time : 3 hours

Subject Code:	12FP230								 		      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	__________ is the primary oxygen carrying pigment of muscle tissues.

2.	__________ is an infection caused by Salmonella species in fresh meat.

3.	__________ intensity is applied during electric shock stunning process of pigs.

4.	PSE stands for __________.

5.	Nitric oxide is derived from nitrite in the presence of ___________.

6.	HACCP stands for ____________.

7.	__________ refers to removal of internal organs.

8.	Amount of live weight of animal retained in the carcass is known as __________.

9.	__________ method is used to preserve perishable fish using liquid nitrogen.

10.	__________ is a nodal agency for promotion of export of marine products from India.

PART – B  (5 x 3 = 15 MARKS)

11.	Briefly summarize on rigor mortis.

12.	Identify the processing effects on meat tenderization and report it.

13.	Describe in brief about smoking of meat and its types.

14.	Briefly discuss the quality characteristics of poultry meat.

15.	Examine the spoilage factor that affects the quality of fish and fishery products.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe in detail the composition of muscle and its modifiers

(OR)

17.	Explain the postmortem changes in muscle in detail.

18.	a.	Discuss the different types of stunning methods used in meat processing operations.	    (8)

b.	Summarize the practices affecting the quality of meat.							    (7)

(OR)

19.	Explain in detail about the unit operations of carcass processing of meat.

20.	What is sausage? Discuss the types and process involved in preparation of sausages.

(OR)

21.	Differentiate GHP and HACCP. Give a detailed account on HACCP for meat processing.

22.	Enumerate the poultry meat operations in detail with highlights on equipments used for processing.

(OR)

23.	a.	Discuss briefly the effects of refrigeration on quality characteristics of poultry meat.	    (7)

b.	State the process procedure for egg powder preparation.							    (8)

24.	a.	Describe the major biochemical components of fish.								    (7)

b.	What is RSW? Discuss the changes observed during storage of fish in RSW and CSW.

(8)

(OR)

25.	Define canning. Describe the steps involved in canning operations of fish in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	BAKERY AND CONFECTIONERY  TECHNOLOGY     	     Time : 3 hours

Subject Code:	12FP233								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The time- temperature combination for moisture estimation in wheat flour is _________ and _________.

2.	Dockage refers to _________.

3.	What are the constituents of a double acting baking powder?

4.	What is the importance of Calcium propionate in bread?

5.	ICUMSA is an acronym for _________.

6.	What is a massecuite?

7.	What is 0proof?

8.	If a beverage contains 10% w/v of sugar, what is the maximum possible alcohol produced in the beverage is fermented?

9.	What is graining?

10.	Conching refers to _________.

PART – B  (5 x 3 = 15 MARKS)

11.	Discuss briefly on mixograph.

12.	What are the characteristics features of short dough for biscuits?

13.	What is Jaggery? How is it prepared?

14.	Write briefly on sparkling wine.

15.	Write briefly on the method of production of Marshmallow by Continuous method.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss in detail on the following:

a. 	Hagberg Falling number.														    (8)

b.	Extensograph curve and its interpretation.  										    (7)

(OR)

17.	With a neat flow diagram, discuss in detail on the steps involved in milling of wheat, highlighting the importance of each step.

18.	With the help of neat diagrams, discuss on the methods of biscuit dough preparation.

(OR)

19.	With the help of a neat flow diagram, discuss in detail on the method of production of bread by the Plastic sponge dough method.

20.	Discuss briefly on the following:

a.	Standards for cane evaluation (any three).										    (7)

b.	Evaporators used for sugar manufacture. 										    (8)

(OR)

[P.T.O.]

21.	With a neat flow diagram, discuss in detail on the steps involved in manufacture of sugar from cane, highlighting the significance of each step.

22.	Discuss in detail on the method of manufacture of lager beer.

(OR)

23.	Discuss in detail on the method of manufacture of a carbonated beverage, highlighting the significance of each operation.

24.	Discuss on the following:

a.	Cookers used for confectionery manufacture	.									  (12)

b.	What is a toffee?																   (3)

(OR)

25.	With the help of a neat flow diagram, discuss in detail on the method of manufacture of chocolates.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	FOOD PACKAGING TECHNOLOGY				      	     Time : 3 hours

Subject Code:	12FP234								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the basic function of packaging.

2.	Mention any two tests carried out for paper & paper boards.

3.	What is the importance of orientation of a packaging material?

4.	Differentiate between a laminate and coextruded films.

5.	Abbreviate: (a) EVA	(b) BOPP

6.	Name any 2 methods for flexible plastic film manufacture.

7.	Mention the importance of nutritional labelling in a package.

8.	Name any 2 methods of plastic laminations.

9.	Abbreviate MAP.

10.	Give an example for carbon-di-oxide emitter used in active package.

PART – B  (5 x 3 = 15 MARKS)

11.	How to determine the migration characteristics of a given packaging material?

12.	Write a note on Double Seaming Operation in metal cans.

13.	Discuss about properties of Polypropylene in food packaging.

14.	What is Flexography?

15.	Mention the role of widgets in active packaging.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain in detail ASLT procedures adopted in designing shelf life test for a food product.

b.	Assume the product has a shelf life of 2 weeks at 60˚C and a Q10 of 2.5. Estimate the shelf life of the product at 30˚C.

c.	Write the formula for determining the frequency of testing product by ASLT procedure.

(10+4+1)

(OR)

17.	Write a brief note on various tests performed in packaging material:

a.	WVTR.																		    (5)

b.	Tearing strength.																    (5)

c.	Tensile strength.																    (5)

18.	Enumerate with neat sketch about the method of manufacture of glass bottles.

(OR)

19.	a.	Discuss in detail about two piece can manufacture with neat sketch.				  (10)

b.	Write a note on importance of exhausting process in canning process.				    (5)

20.	Discuss in detail with neat sketch wherever necessary about paper and paperboard manufacture.

(OR)

21.	a.	Explain about any three types of lamination process performed in plastic packaging.	    (7)

b.	Write a note on retort pouch packaging.						    					    (8)

[P.T.O.]

22.	Explain the working principle of  Form fill Sealing machine with a neat labeled  sketch.

(OR)

23. 	Write a note on:

a.	Barcodes.																	(8)

b.	Printing on packages															(7)

24.	Discuss in detail about the importance of active packaging in foods with necessary examples.

(OR)

25. 	Explain in detail about modified atmosphere packaging and its applications in foods.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	STORAGE ENGINEERING							      	     Time : 3 hours

Subject Code:	12FP235						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The unit of thermal conductivity is ___________.

2.	Enthalpy is the total ___________ content of a material.

3.	The temperature of the air determined by a thermometer bulb covered with a piece of wet cloth is called ___________.

4.	The moisture content at the beginning of falling rate period is called ___________.

5.	Pusa bin is an example for ___________.

6.	The angle between horizontal and inclination of heap is called ___________.

7.	Sitophilusoryzae is called ___________.

8.	The insects which depend on other types of insects are called ___________.

9.	Phosphine is used for ___________ of the stored grains

10.	Increasing the level of ___________ gas extends the shelf life of fruits and vegetables.

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate static and kinetic friction.

12.	Define thermal diffusivity.

13.	What are the direct damages caused to the grains by the improper storage?

14.	What are the common types of insects that affect grain storage?

15.	What is the advantage of air tight storage?

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss about the importance of physical properties of grains in designing storage vessels.

(OR)

17.	Write note on

a.	Drag co efficient.   															    (8)

b.	Frictional Properties.															    (7)

18.	Name four direct methods of determining the moisture contents of grains and explain in detail.

(OR)

19.	Define equilibrium moisture content and discuss the methods of determination of equilibrium moisture content.

20.	Name any four traditional storage structures used grain and discuss about the advantages and disadvantages of each.

(OR)

21.	What are silos and what are the types of silos used for storage of grains?

22.	What are the common insects found in stored grains and write the extent of damage caused.

(OR)

23.	What are rodenticides? How it is used and discuss the other rat control measures.

24.	What is the basic principle behind controlled atmosphere storage? Explain in detail.

(OR)

25.	How effective is modified atmosphere storage for cereal grain storage? Explain with example.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title :	PROCESSING OF FOOD COMMODITIES			      	     Time : 3 hours

Subject Code:	12FP242								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	___________ is the most commonly used species of Yeast in Bread manufacture

2.	___________ processing of fruits and vegetables generally involves washing, peeling, slicing, or shredding before packaging and storage at low temperature.

3.	___________ mill is used for pulse flour production process.

4.	___________ is the raw material for making tortillas.

5.	Name any 2 hurdles that can be used in preserving foods.

6.	The true constituents of milk are ___________, ___________ and ___________.

7.	___________ refers to the skeletal muscle from the carcasses of animals.

8.	What is cryoprotectant?

9.	___________ and ___________ are the commercial species of coffee grown worldwide.

10.	What is wilting process?

PART – B  (5 x 3 = 15 MARKS)

11.	Write down the different types of sausages.

12.	What is explosion puffing?

13.	Write notes on different methods of stunning animals.

14.	What is homogenization? Write the need for milk homogenization.

15.	Write notes on green tea manufacturing.

PART – C  (5 x 15 = 75 MARKS)

16.	Write in detail about  the process of rice milling in detail.

(OR)

17.	Elucidate the mixing methods used for the following types of cakes.

a.	Shortening cakes.

b.	Foam style cakes.

c.	Chiffon cakes.

18.	What is canning? Explain the process of canning of fruit in detail.

(OR)

19.	Explain the dehydration of fruits and vegetables by the following methods.

a.	Microwave drying. 															    (5)

b.	Solar drying.																	    (5)

c.	Osmotic drying.																    (5)

20.	Explain in detail the process of manufacturing cheese with flowcharts.

(OR)

[P.T.O.]

21.	Discuss the working and construction of the following types of heat exchangers used in dairy industries with diagrams.

a.	Plate heat exchangers.  														    (8)

b.	Tubular heat exchangers.														    (7)

22.	Explain the various aspects of poultry processing in detail.

(OR)

23.	Discuss the processing of the following fish products:

a.	Fish meal and oil.  															    (8)	b.	Canned Fish.																	    (7)

24.	a.	How is parchment coffee manufactured?  										    (8)

b.	Explain the steps involved in ginger processing. 									    (7)

(OR)

25.	a.	Describe the cocoa and chocolate processing. 									  (10)

b.	What is the use of vanilla? How is it prepared?									    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December – 2015

Subject Title :	TECHNOLOGY OF PACKAGING					      	     Time : 3 hours

Subject Code:	12FP243										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define biological environment.

2.	What is Nitrogen flushing in food packaging?

3.	What is a lacquer?

4.	Define polyethylene.

5.	Expand DRD cans and DWI process.

6.	Define white flint glass.

7.	Define injection moulding.

8.	What is fin seals?

9.	What is Purge Absorption?

10.	Define CAP.

PART – B  (5 x 3 = 15 MARKS)

11.	Write the advantages of Vacuum packaging.

12.	Draw the design of three piece cans.

13.  Why is retort pouch packaging used in food industry?

14.	Write about various types of closures.

15.	Differentiate Active and Passive modified atmosphere.

PART – C  (5 x 15 = 75 MARKS)

16.	Write in detail about the factors affecting quality of food products.

(OR)

17.	Explain the functions of Packaging with reference food and consumers.

18.	Describe the process of making of Three Piece cans with diagram.

(OR)

19.	Discuss the canning operations with a flow diagram.

20.	a.	Write in detail about polyethylene and its classification.							 (10)

b.	Describe the characteristics of polythene used in food industry.					   (5)

(OR)

21.	a.	Write about Paper board carton packaging with its applications.					    (7)

b.	Write about Corrugated card board packaging with its applications.					    (8)

22.	Explain in detail about various types of vertical form-fill-seal seal equipment with diagrams.

(OR)

23.	a.	Write about the Caps and Lids used in Food Packaging.							    (7)

b.	Explain various Sealing Methods.												    (8)

24.	Write in detail - Modified Atmosphere packaging?

(OR)

25.	Explain the following:

a.	Moisture control in food packaging.											    (5)

b.	Role of O2 in spoilage.														    (5)

c.	Intelligent packaging material.													    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	INSTRUMENTAL TECHNIQUES OF FOOD ANALYSIS													       								      Time : 3 hours

Subject Code: 	12FP316										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

Explain briefly about the high performance liquid chromatography and its application in food analysis.

2. 	Give brief explanation about physical forces and modes of separation used in chromatography technique.

(OR)

3.	Discuss briefly about the application of FTIR Spectroscopy in:

a.	Data handling Techniques

b.	Peak measurements

4.  	Explain the following:

a.	Types of detectors used in Gas Chromatography.

b.	Application of Gas Chromatography in food analysis.

(OR)

5. 	Write and explain the Components of an Atomic Absorption spectrometer and its measurements.

6.	Explain in detail about the application of NMR in Food analysis.

(OR)

7.	Write briefly about Atomic emission spectrometry technique in food analysis.

8. 	Write and explain about GC/MS and FAB/MS.

(OR)

9.  	Give detail account on:

a.	Micellar Electrokinetic chromatography (MECK).

b.	Capillary isoelectric focusing (CIEF).

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December - 2015

Subject Title: 	STABILITY AND SHELF LIFE TESTING OF FOODS

Time : 3 hours

Subject Code: 	15FP3001										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

Write notes on

a.	Major packaging materials in extending shelf life of foods.

b.	Gas and moisture barrier properties.

c.	Moisture sorption isotherm.

d.	Molecular mobility.

2. 	Give brief explanation on predictive model for shelf life of packaged foods and beverages.

(OR)

3.	Give brief explanation on development of mathematical model .

4.  	Explain in brief about on E- nose and E tongue and its application in food industries.

(OR)

5.	Explain in brief about on design and interpretation of sensory shelf life testing of foods.

6. 	Write notes on modelling approach for accelerated shelf life studies.

(OR)

7.	Write notes on smart packaging of foods.

8. 	Explain in detail about the factors influencing the stability of long shelf life milk products.

(OR)

9.	Explain in detail about the factors affecting shelf life and stability in confectionery products.