Reg.No. ____________

End Semester Examination – Nov/Dec – 2017

| Code : | 14AE3003 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | THERMODYNAMICS AND HEAT TRANSFER | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A furnace wall consists of steel plate 20 mm thick with thermal conductivity 16.2 W/mK lined on inside with Silica bricks 150 mm thick with conductivity 2.2 W/mK and on the outside with Magnesia brick 200 mm thick of conductivity 5.1 W/mK. The inside and outside surfaces of the wall are maintained at 650C and 150C respectively. Calculate the heat loss from the wall per unit area. If the heat loss is reduced to 2850 W/m2 by providing an air gap between the Steel and Silica bricks, find the necessary width of air gap if the thermal conductivity of air is assumed as 0.03 W/mK. | CO2 | 15 |
|  | b. | Write the general three-dimensional heat conduction equation in Cartesian, Cylindrical and Spherical coordinates. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | A steel pipeline (k=50 W/mK) of ID 100 mm and OD 110 mm is to be covered with two layers of insulation each having a thickness of 50 mm. The thermal conductivity of the first insulation material is 0.06 W/mK and that of the second material is 0.12 W/mK. Calculate the loss of heat per meter length of pipe and the interface temperatures between the two layers of insulation when the temperature of the inside tube surface is 250C and that of the outside surface of the insulation is 50C. | CO2 | 10 |
|  | b. | The air speed of a turbojet engine in flight is 270 m/s. Ambient air temperature is -15C. Gas temperature of outlet of the nozzle is 600C. Corresponding enthalpy values for air and gas are 260 and 912 kJ/kg respectively. Fuel-air ratio is 0.019. Chemical energy of the fuel is 44.5 MJ/kg. Owing to incomplete combustion 5% of the chemical energy is not released in the reaction. Heat loss from the engine is 21 kJ/kg of air. Calculate the velocity of the exhaust jet. | CO2 | 10 |
| 3. |  | Derive the TdS equation in terms of coefficient of volume expansion () and the isothermal compressibility (KT). | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Explain the ways of entropy generation in closed systems | CO1 | 10 |
|  | b. | Prove that | CO1 | 10 |
| 5. |  | A fluid undergoes a reversible adiabatic compression from 0.5 MPa, 0.2 m3 to 0.05 m3 according to the law pV1.3 = constant. Determine the change in enthalpy, internal energy, entropy, heat transferred and work transfer during the process. | CO1 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | What is critical radius of insulation? Explain. | CO2 | 5 |
|  | b. | A steam pipe 10 cm inner diameter 11 cm outer diameter is covered with an insulating material (K=1W/mK). The steam temperature and the ambient temperatures are 200oC and 20oC respectively. If the convection heat transfer co-efficient between the insulating surface and air is 8W/m2K. Find the critical radius of insulation and the heat lost per meter of pipe for the value of rc. And also find the outer surface temperature. | CO2 | 15 |
| 7. | a. | The velocity distribution in the boundary layer is given by u/U = (3/2)(y/δ) – (1/2)(y2/δ2), δ is being boundary layer thickness. Calculate the ratio of momentum thickness to boundary layer thickness (θ/δ). | CO2 | 10 |
|  | b. | Air at 20o C and at atmospheric pressure flows over a flat plate at a velocity of 1.8 m/s. If the length of the plate is 2.2 m and is maintained at 100oC, calculate the heat transfer rate per unit width using exact and approximate methods. The prperties of air at mean bulk temperature 60oC are ρ = 1.06 kg/m3, CP = 1.005 kJ/kgoC, k = 0.02894 W/moC, Pr = 0.696, υ = 18.97 x 10-6 m2/s. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. |  | Assuming sun to be black body emitting radiation at 6000 K at a mean distance of 12 x 1010 m from the earth. The diamter of the sun is 1.5 x 109 m and that of the earth is 13.2 x 106 m. Calculate the i. total energy emitted by the sun ii. the emission received per m2 just outside the earth’s atmosphere iii. the total energy received by the earth if no radiation is blocked by the earth’s atmosphere iv. the energy received by a 2 x 2 m solar collector whosed normal is inclined at 45o to the sun. The energy loss through the atmosphere is 50% and the diffuse radiation is 20% of direct radiation. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | A horizontal cylinder 6 cm in diameter is exposed to stagnant room air at 1 atm and 300 K. The surface of the cylinder is maintained at a constant temperature of 400 K. Calculate the heat lost by the cylinder per unit length. | CO2 | 20 |

ALL THE BEST