Reg.No. ____________

End Semester Examination – Nov/Dec – 2017

| Code : | 14CE2005 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | APPLIED HYDRAULICS AND HYDRAULIC MACHINERY | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | An open channel of most economical section, having the form of a half hexagon with horizontal bottom is required to give a maximum discharge of 20.2 m3/s of water. The slope of the channel bottom is 1 in 2500. Taking Chezy’s Constant, C = 60 in Chezy’s equation, determine the dimensions of the cross section. | CO1 | 15 |
|  | b. | Distinguish between open channel flow and pipe flow. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Design an earthen trapezoidal channel for water having a velocity of 0.6 m/sec, side slope is 1:1.5 and quantity of water is 3 m3/sec. Assume Chezy’s constant as 65. | CO1 | 15 |
|  | b. | Classify the different types of channel. | CO1 | 5 |
| 3. | a. | Derive an equation for a Gradually varied flow. | CO1 | 15 |
|  | b. | Sketch the specific energy curve. | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. |  | The velocity distribution in the boundary layer over a high spillway face was found to have the following form: u/U = (y/δ)0.22 Find the ratio of the displacement thickness, momentum thickness and the energy thickness. | CO2 | 20 |
| 5. | a. | A jet of water having a velocity of 40m/s strikes a curved vane, which is moving with a velocity of 20m/s. The jet makes an angle of 300 with the direction of motion of vane at inlet and leaves at an angle of 900 to the direction of motion of vane at outlet. Draw the velocity triangles at inlet and outlet so that the water enters and leaves the vane without shock. | CO2 | 12 |
|  | b. | Derive an equation for finding out the length of the back water curve along the bed of the channel. | CO2 | 8 |
| (OR) |  |  |  |  |
| 6. | a. | A jet of water moving at 12m/s impinges on a concave shaped vane to deflect the jet through 120o when stationary. The vane is moving at 5m/s. Find i. The angle of jet that there is no shock at inlet ii. The absolute velocity of the jet at exit both in magnitude and direction and iii. The work done per second per N of water. Assume that vane is smooth. | CO4 | 12 |
|  | b. | A 50 mm diameter jet having a velocity of 25 m/s strikes a flat plate, the normal of which is inclined at 30° to the axis of the jet. Calculate the normal force exerted on the plate. i. When the plate is stationary ii. When the plate is moving with a velocity of 10 m/s in the direction of the jet | CO3 | 8 |
| 7. | a. | A Pelton wheel is to be designed for the following specifications: Shaft Power = 9560kW; Head = 350m; Speed = 750rpm. Overall efficiency = 85%; Jet diameter is not to exceed one sixth of the wheel diameter. Take coefficient of velocity as 0.985; Speed ratio = 0.45. Determine i. The wheel diameter ii. the number of jets required and iii. Diameter of the jet. | CO5 | 10 |
|  | b. | Calculate the diameter and speed of the runner of a Kaplan turbine developing 6000kW under an effective head of 5m. Overall efficiency of the turbine is 90%. The diameter of the boss is 0.4 times the external diameter of the runner. The turbine speed ratio is 2.0 and flow ratio 0.6. Calculate the specific speed of the turbine. | CO5 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | The impeller of a centrifugal pump has an external diameter of 450mm and internal diameter of 200mm and it runs at 1440rpm. Assuming a constant radial flow through the impeller at 2.5 m/s and that the vanes at exit are set back at an angle 250, determine: i. Inlet vane angle, ii. The angle, absolute velocity of water at exit makes with the tangent and iii. The work done per N of water. | CO5 | 12 |
|  | b. | Summarize the various efficiencies of a centrifugal pump | CO5 | 8 |
|  |  | Compulsory: |  |  |
| 9. | a. | The cross sectional area of a plunger of a reciprocating pump equals 1.5 times that of a delivery pipe. The delivery pipe is 60m long and it rises upward at a slope of 1 in 6. If the plunger has an acceleration of 2.4 m/s2 at the end of the stroke and separation pressure is 2.5m of water, find whether separation will take place and, if so at which section of the pipe. Assume simple harmonic motion, and take atmospheric pressure = 10.3 m of water. | CO5 | 15 |
|  | b. | Compare Centrifugal pumps and reciprocating pumps. | CO5 | 5 |

ALL THE BEST