Reg.No. ____________

End Semester Examination – Nov/Dec – 2017

| # Code : | # 14CE3001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # APPLIED ELASTICITY AND PLASTICITY | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | The displacement field is given by U=k(x2 + 2z); v= k(4x + 2y2 + z); w = 4kz2; K is a very small quantity. What are the strains at (2,2,3) in the directions i. nx = 0, ny= 1/√2, nz=1/√2 ii. nx = 1, ny= 0, nz=0 iii. nx = 0.6, ny= 0, nz=0. | CO1 | 12 |
|  | b. | Derive the stress transformation law in 3D rectangular coordinates. | CO1 | 8 |
| (OR) |  |  |  |  |
| 2. | a. | Write down the Navier’s equilibrium equations in three dimensional Cartesian coordinates | CO1 | 5 |
|  | b. | The State of Stress at a point is given by σx = 100, σy = 200, σz = -100, τxy = -200, τyz = 100 and τxz = -300 kpa. Determine i. The stress invariants ii. The principal stresses, and iii. The direction cosines of the principal planes. | CO3 | 15 |
| 3. | a. | Show that the following function is a stress function and investigate what problem it solves,  = (q/8c3)[x2(y3-3c2y-2 c3)-(1/5)( y3(y2-2c2)). | CO2 | 10 |
|  | b. | A beam of narrow rectangular cross section is subjected to uniformly distributed load of q over the entire span. Assuming suitable stress function, derive expressions for stresses and displacements, if the beam is simply supported | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Investigate what problem of plane stress is solved by the stress function: φ=3F4Cxy−xy33C2ρ2y2. | CO2 | 10 |
|  | b. | For a plane stress problem, derive G∇2u+E2(1−γ)∂∂x∂u∂x+∂v∂y+Bx=0 and a companion equation | CO2 | 10 |
| 5. | a. | Obtain an expression for the hoop stress induced in a thick cylinder subjected to internal fluid pressure, deriving the general solution satisfying 4 = 0,  = A log r + Br2 log r + Cr + D | CO3 | 10 |
|  | b. | Derive the constitutive law for an axisymmetric problem. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Derive the strain displacement relations in polar coordinates. | CO3 | 10 |
|  | b. | Determine the stresses in an infinite plate with a hole subjected to tension | CO3 | 10 |
| 7. | a. | A two cell tube shown in Figure.1 is subjected to a torque 10kN.m. Determine the shear stress in each part and angle of twist per metre length. Take G=80 GPa. Figure.1. | CO2 | 10 |
|  | b. | An elliptical bar is subjected to a twisting moment T. Derive the expressions for shear stresses and angle of twist at any point in the bar and hence the maximum shear stress | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | A steel girder has cross section shown below in Figure.2. Wall thickness in uniformly 1.2 cm. The stress due to twisting should not exceed 350 N/mm2. Neglect stress concentration. G=75 GPA. Figure.2. - What is the maximum allowable torque? - What is the twist per unit length under the torque? - What is the shear stress in the middle web? | CO2 | 10 |
|  | b. | Derive the expression for maximum shear stress and unit angle of twist for an equilateral triangular shaft subjected to torque. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Obtain expressions for moment of resistance of a beam of rectangular cross section at i. Incipient yielding ii. elastic plastic stage and iii. fully plastic.stage | CO1 | 10 |
|  | b. | Derive the elastic, plastic and elasto-plastic stress – strain relationship equations for a thick cylinder subjected to internal pressure of p1 . | CO1 | 10 |

ALL THE BEST