Reg.No. ____________

End Semester Examination – Nov/Dec – 2017

| Code : | 14EC2008 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | LINEAR INTEGRATED CIRCUITS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | With necessary circuit diagrams prove that the operational amplifier can be used as an Integrator. State the problems associated with the basic integrator and design a practical integrator to overcome the drawbacks. | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. |  | Show with the help of circuit diagram an op-amp that can be used as, i. Inverting Amplifier ii. Non-Inverting Amplifier iii. Adder Subtractor to perform Vo = V1 + V2 - V3 -V4 | CO1 | 20 |
| 3. | a. | Write short notes on astable multivibrator using op- amp. Derive the expression for frequency of oscillation. | CO1 | 15 |
|  | b. | Design a symmetrical Schmitt trigger using op-amp for a hysteresis width of 4V, Power supply +/- 15V. | CO1 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Design a RC phase shift oscillator for a frequency of 2kHz. | CO1 | 10 |
|  | b. | Write short notes on comparators. How comparator can be used as zero crossing detectors? | CO1 | 10 |
| 5. | a. | Design a Wide-band pass filter having fl=400Hz,fh=2kHz and pass band gain of 4. Find the value of Q of the filter. | CO1 | 10 |
|  | b. | Explain the working of Monostable multivibrator using IC 555 timer and derive the expression for frequency of oscillations. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Produce an expression for the transfer function of a first order low pass filter. Also derive the expression for frequency response. | CO1 | 10 |
|  | b. | Draw the diagram of the functional block diagram of 555 IC timer and explain how it is used for generating accurate time delay or oscillation. | CO2 | 10 |
| 7. | a. | With relevant block diagram, explain the functionality of a PLL circuit and its operation. | CO2 | 12 |
|  | b. | Explain in brief about principle of operation of a Schmitt Trigger using IC555 timer. | CO2 | 8 |
| (OR) |  |  |  |  |
| 8. | a. | Paraphrase on the working of square wave generator using IC 555 timer. Also estimate the frequency calculation for the same. | CO2 | 15 |
|  | b. | What output voltage would be produced by a D/A converter whose output range is 0 to 10V and whose input binary number is 10111100? | CO1 | 5 |
|  |  | Compulsory: |  |  |
| 9. |  | Discuss in detail the steps involved in basic planar process with neat diagrams. | CO3 | 20 |

ALL THE BEST