Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2017

| Code : | 14EC2027 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ADVANCED MICROCONTROLLER | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Illustrate how the 68HC11 uses its data/address buses to handle both data and address lines. | CO1 | 12 |
|  | b. | Write an algorithm and flowchart to interface the blinking of two LEDs using interrupt controller of a microcontroller | CO3 | 8 |
| (OR) |  |  |  |  |
| 2. | a. | Illustrate the Parallel Input/Output (I/O) Port operation in PIC32 microcontroller | CO1 | 7 |
|  | b. | Discuss the memory mapping in 68HC11 microcontroller | CO1 | 7 |
|  | c. | What is the function of condition code register (CCR) in 68HC11 microcontrollers? | CO1 | 6 |
| 3. | a. | Discuss the Expanded Mode of operation in 68HC11 Microcontroller. | CO1 | 10 |
|  | b. | Explain the concept of I2C communication with a suitable application in PIC32 microcontroller. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. |  | Apply the internal timer of MC51 and design a RTC with Hr:Min:Sec. Draw the block diagram, flowchart and algorithm for the designed RTC. | CO2 | 20 |
| 5. |  | Develop an algorithm for generating a square wave with 50% duty cycle using P89V51RD2 microcontroller with crystal frequency of 11.0592 MHz using Timer0 in mode1 of operation. Explain its operation with a neat block diagram. | CO3 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the Timer Clock Divider Chains with its structural diagram | CO1 | 7 |
|  | b. | Write an ISR to read the input from Port C and write it to Port B approximately every 1,000,000 clock cycles. Assume timer Prescalar = 1. | CO2 | 7 |
|  | c. | Mention the importance of Prescaling of timers in PIC32 microcontroller. | CO1 | 6 |
| 7. | a. | Apply the I/O operation of PIC 32 and control the motor used in robotics in forward, reverse and 45 degree step angle. Write the algorithm and explain its interfacing module. | CO2 | 12 |
|  | b. | Discuss ‘Output Compare’ in PIC32 microcontroller. | CO1 | 8 |
| (OR) |  |  |  |  |
| 8. | a. | Illustrate the importance of analog to digital converter in an embedded system. Also explain the 10bit internal ADC of PIC 32 microcontroller with a neat diagram. | CO2 | 12 |
|  | b. | Apply the JTAG concept to debug the system. | CO3 | 8 |
|  |  | Compulsory: |  |  |
| 9. | a. | Design a system to generate a delay of 500ms for an industrial application using the timer module of AVR Atmega microcontroller. Also explain the internal block diagram of timer module. | CO3 | 15 |
|  | b. | List some Characteristics of AVR microcontroller. | CO1 | 5 |

ALL THE BEST