Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2017

| Code : | 14EC2066 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | DIGITAL SYSTEM DESIGN | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Most Calculators and digital watches make use of seven-segment displays for output. Each digit of these displays is comprised of seven bars of light –emitting semiconductors. Create a truth-table that receives a BCD digit as input and provides seven outputs to drive a corresponding display digit and find Boolean expression using K-map for the variables a, b, c and draw the logic diagrams for a, b, c outputs. | CO1 | 5 |
|  | b. | A system of logic is to be designed which has two outputs (F,G) and three inputs (W,X,Y).One output F will be TRUE if an odd number of inputs are TRUE. The other output G will be TRUE if and only input alone is TRUE. Draw the truth table and write the corresponding Boolean equations. | CO1 | 5 |
|  | c. | Simplify the variable using K-map F(W,X,Y,Z)= X Y′Z + X′ Y′ Z′ + XZ′ + W′XZ′ | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Explain in detail about Shannon’s theorem and apply Shannon’s decomposition for realizing any six-variable function Z(a,b,c,d,e,f). Also realize the following six variable function using five-variable and four-variable function generator. Z= abcd'ef ' + a'b'c'def ' + b'cde'f. | CO1 | 7 |
|  | b. | Obtain the AND-OR and two forms of NAND-NAND implementation for the function Z=AB+ CD+ E | CO1 | 5 |
|  | c. | Develop a circuit using combinational logic for 4-bit Binary to Gray code converter by considering D,C,B,A as inputs and W,X,Y,Z as outputs. | CO1 | 8 |
| 3. | a. | Design a synchronous sequential machine for the overlapping input sequence detector “111”sequence for Mealy state machine. | CO1 | 12 |
|  | b. | Simplify the Boolean function: F(w,x,y,z) =∑m (1,3,7,11,15) and the don’t care conditions: d(w,x,y,z)= ∑(0,2,5). | CO1 | 8 |
| (OR) |  |  |  |  |
| 4. | a. | Design a logic diagram for BCD -to-excess -3 code converter by considering A,B,C,D as inputs and W,X,Y,Z as outputs. | CO1 | 8 |
|  | b. | Design a synchronous sequential machine for the overlapping input sequence detector “101”sequence for Moore state machine. | CO1 | 12 |
| 5. | a. | Implement the following output functions using a suitable PLA with PLA table. F1(A,B,C,D)= ∑m ( 3,7,8,9,11,15) F2(A,B,C,D)= ∑m ( 3,4,5,7,10,14,15) F3(A,B,C,D)= ∑m ( 1,5,7,11,15) | CO2 | 10 |
|  | b. | Design an architecture for fundamental mode Asynchronous sequential circuit for the following condition: Consider a sequential circuit with 2 inputs x1 and x2 and one output z. The initial input state is x1=x2=0 and the circuit output is to be 1 if and only if the input state is x1=x2=1 and preceding input state is x1=0 and x2=1. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | A 3-input,4-Output combinational circuit has the following output functions. Implement the circuit using a suitable PAL. A(x,y,z) =∑m (1,2,4,6) B(x,y,z) =∑m (0,1,3,6,7) C(x,y,z) =∑m (1,2,4,6,7) D(x,y,z) =∑m (1,2,3,5,7) | CO2 | 10 |
|  | b. | Design an asynchronous sequential circuit to behave in the following manner. The circuit has two inputs x1 and x2 and two outputs P and Q. Initially both inputs and outputs are equal to 0 (state 0).Also if x1= x2= 1,P=Q=1 (state 4). When x2 changes from 0 to 1,P becomes 1 and Q becomes 0 for state 2.Since the second input x2 is 1 Q becomes 1 and P takes the initial state value i.e 0 for state 3.When x1=1 and x2=0 , P becomes 1 and Q becomes 0 for state 5 and Q becomes 1 and P becomes 0 for state 6. Whenever,P=0 and either of x1 or x2 becomes 1,P becomes 1.When the second input becomes 1, Q also becomes 1. | CO2 | 10 |
| 7. | a. | Define ASM Chart and draw an ASM chart for a 3-bit binary up/down counter. | CO2 | 8 |
|  | b. | Design a BCD to excess-3 code converter using PROM. | CO2 | 12 |
| (OR) |  |  |  |  |
| 8. | a. | With neat block diagram explain in detail about Logic Array block of ALTERA FLEX 10K CPLD. | CO2 | 12 |
|  | b. | Implement a full subtractor using a PAL. | CO2 | 8 |
|  |  | Compulsory: |  |  |
| 9. | a. | With neat block diagram explain in detail about Altera MAX 7000S CPLD. | CO2 | 15 |
|  | b. | Using PROM realize the following function: F1(a,b,c) =∑m (0,1,3,5,7) F2(a,b,c) =∑m (1,2,5,6) | CO2 | 5 |

ALL THE BEST