Reg.No. ____________

End Semester Examination – Nov/Dec - 2017

| Code : | 14EC3027 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | VLSI DIGITAL SIGNAL PROCESSING | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Discuss on the different types of graphical representations used for DSP systems. | CO1 | 10 |
|  | b. | Using Longest path Matrix Algorithm, find the iteration bound of DFG shown. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Explain in detail about pipelining of FIR digital filter. | CO2 | 12 |
|  | b. | In the SFG shown in the figure, the computation time for each is assumed to be 1 u.t. Calculate the critical path computation time. The critical path has been reduced to 2 u.t by inserting 3 extra delay elements as shown in the following figure. Is this a valid pipelining? If not, obtain an appropriate pipelined circuit with critical path of 2 u.t | CO2 | 8 |
| 3. | a. | Describe with an example the method of unfolding a DFG by using an unfolding factor J=3. | CO2 | 6 |
|  | b. | Use Floyd Warshall algorithm to solve the system of inequalities given below r1 – r2 < 0 r3 – r1 < 5 r4 - r1 < 4 r4 – r3 < -1 r3 - r2 < 2 | CO1 | 14 |
| (OR) |  |  |  |  |
| 4. |  | Draw a space-time representation and the systolic architecture of the Design B2 with d = 1−1 pT = 11 and ST=10and design F whose characteristics are fan in results, move inputs and weights stay. | CO2 | 20 |
| 5. |  | Consider a 2 x 3 linear convolution s(p) = h(p) x(p) where, h(p) = h0 + h1p and x(p) = x0 + x1p + x2p2 Use Cook – Toom algorithm to construct an efficient implementation for the given linear convolution. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Discuss in detail about the algorithm architecture transformations used to derive efficient DCT implementations where the number of multiplications is reduced. | CO2 | 20 |
| 7. |  | State and explain the round off noise computation with an example. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Explain how Distributed Arithmetic Architecture is used to design bit level architectures for vector-vector Multiplication | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Explain Additive and Multiplicative number splitting. | CO3 | 20 |

ALL THE BEST