Reg.No. ____________

End Semester Examination – Nov/Dec – 2017

| Code : | 14EE2026 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | HIGH VOLTAGE ENGINEERING | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Enumerate the causes for switching and power frequency overvoltages? How are they controlled in power systems? | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. | a. | With suitable illustrations, explain how insulation level is chosen for various equipment in a 230/132 kV sub-station. | CO1 | 10 |
|  | b. | A long transmission line is energised by a unit step voltage 1.0 V at the sending end and is open circuited at the receiving end. Construct the Bewley lattice diagram and obtain the value of the voltage at the receiving end after a long time. Take the attenuation factor α = 0.8. | CO1 | 10 |
| 3. |  | Discuss in detail the processes that are primarily responsible for the breakdown of a gas are ionization by collision, photo-ionization, and the secondary ionization processes. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Originate the different mechanisms by which breakdown occurs in solid dielectrics in practice. | CO1 | 20 |
| 5. |  | Describe with neat sketch, different types of rectifier circuits for producing high d.c. voltages. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | A Cockcroft-Walton type voltage multiplier has eight stages with capacitances, all equal to 0.05 pF. The supply transformer secondary voltage is 125 kV at a frequency of 150 Hz. If the load current to be supplied is 5 mA, find i) the percentage ripple, ii) the regulation, and iii) the optimum number of stages for minimum regulation or voltage drop. | CO2 | 15 |
|  | b. | Give the Basic and Modified Marx circuit arrangement for multistage impulse generators. | CO2 | 5 |
| 7. | a. | A coaxial shunt is to be designed to measure an impulse current of 50 kA. If the bandwidth of the shunt is to be at least 10 MHz and if the voltage drop across the shunt should not exceed 50 V, find the ohmic value of the shunt and its dimensions. | CO2 | 10 |
|  | b. | Interpret capacitance voltage transformer? Explain with phasor diagram how a tuned capacitance voltage transformer can be used for voltage measurements in power systems. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Discuss the different methods of measuring high d.c. voltages. What are the limitations in each method? | CO2 | 15 |
|  | b. | Design a peak reading voltmeter along with a suitable micro-ammeter such that it will be able to read voltages, up to 100 kV (peak). The capacitance potential divider available is of the ratio 1000:1. | CO2 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Extend the terms i. withstand voltage, ii. flashover voltage, iii. 50% flashover voltage, and iv. wet and dry power frequency tests as referred to high voltage testing. | CO3 | 10 |
|  | b. | Elaborate the partial discharge tests on high voltage cables. How is a fault in the insulation located in this test? | CO3 | 10 |

ALL THE BEST