Reg.No. ____________

End Semester Examination – Nov/Dec - 2017

| Code : | Electrical Measurements | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | 14EI2003 | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Describe the construction and working of PMMC instrument. Derive its torque equation. | CO1 | 15 |
|  | b. | Why is damping torque necessary in indicating instruments? Sketch the curves showing the different damping conditions. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Mention the torque equation for Electrodynamometer type instruments and explain its operation. | CO1 | 10 |
|  | b. | Differentiate between Absolute and secondary instruments with suitable examples. | CO1 | 10 |
| 3. | a. | Discuss the working of Hay’s bridge for measurement of inductance. Derive the equations for balance. | CO3 | 10 |
|  | b. | Explain the operation and working principle of power meters. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Discuss the working of Maxwell’s bridge for measurement of inductance. Derive the equations for balance. | CO3 | 10 |
|  | b. | Explain the operation and working principle of Megger meter. | CO1 | 10 |
| 5. | a. | Figure below show the schematic diagram of a Wheatstone bridge with values of the bridge elements. The battery voltage is 5 V and its internal resistance negligible. The galvanometer has a current sensitivity of 10 mm/μA and an internal resistance of 100 Ω. Calculate the deflection of the galvanometer caused by the 5-Ω unbalance in arm BC. | CO2 | 10 |
|  | b. | Discuss the Loading Effect of the Voltmeter. | CO1 | 10 |
| (OR) |  |  |  |  |
| 6. |  | Describe the working of Wien’s Bridge for measurements of frequency. Derive the equations for balance condition. | CO3 | 20 |
| 7. | a. | Describe the construction, working principle, merits and demerits of Current Transformer. | CO1 | 16 |
|  | b. | Define a sensitivity of voltmeter with example. | CO1 | 4 |
| (OR) |  |  |  |  |
| 8. | a. | With neat sketch explain the construction and working principle of Q meter. | CO3 | 15 |
|  | b. | Briefly explain about units and standards. | CO1 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss the analog type X-Y recorder for plotting instrument characteristics. | CO1 | 10 |
|  | b. | Write short note on Magnetic Tape Recorder. | CO1 | 10 |

ALL THE BEST