Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2017

| Code : | 14EI2023 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | OPTO-ELECTRONICS AND LASER BASED INSTRUMENTATION | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No | Sub Div. | Questions | Course Outcome | Marks |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | a. | Justify the use of fiber optics for communication and elaborate on any two fibers optic sources. | CO1 | 16 |  |  |  |
|  | b. | The speed of light in oil is measured to be 2.1 x 108 m/s. Determine the index of refraction of the medium. | CO1 | 4 |  |  |  |
| (OR) |  |  |  |  |  |  |  |
| 2. | a. | Illustrate the construction and working of Photodetectors and PIN detectors. | CO1 | 12 |  |  |  |
|  | b. | Light travels from air into an optical fiber with an index of refraction of 1.44. i. In which direction does the light bend? ii. If the angle of incidence on the end of the fiber is 22o, what is the angle of refraction inside the fiber? Iii. Sketch the path of light as it changes media. | CO1 | 8 |  |  |  |
| 3. | a. | Identify the use of fiber optic sensors in Industry and also explain how fiber optic cables are used in measuring temperature, pressure and liquid level. | CO1 | 15 |  |  |  |
|  | b. | Outline the concepts of dual nature of light. | CO1 | 5 |  |  |  |
| (OR) |  |  |  |  |  |  |  |
| 4. | a. | Differentiate between intrinsic and extrinsic fiber optic sensors, Intensity modulated and phase modulated fiber optic sensors. Also discuss the measurement of strain and displacement using fiber optic sensor. | CO1 | 15 |  |  |  |
|  | b. | Calculate the energy of a photon of radiation that has a wavelength of 3.3µm. | CO1 | 5 |  |  |  |
| 5. | a. | Explain the construction and working of Ruby LASER in detail. | CO2 | 15 |  |  |  |
|  | b. | Differentiate Pulsed LASERS and Continuous Wave LASERS. | CO2 | 5 |  |  |  |
| (OR) |  |  |  |  |  |  | (OR) |
| 6. | a. | LASER applications for material processing requires high intensity short duration pulses. How are those pulses produced? Elaborate on it. | CO2 | 12 |  |  |  |
|  | b. | Analyze how LASERS can be used in measuring velocity. | CO2 | 8 |  |  |  |
| 7. |  | Explain the different types of LASER interaction with tissues. Also discuss in detail the application of LASERS in medicine. | CO3 | 20 |  |  |  |
| (OR) |  |  |  |  |  |  | (OR) |
| 8. |  | Elaborate on the application of LASER in Material Processing | CO3 | 20 |  |  |  |
|  |  | Compulsory: |  |  |  |  |  |
| 9. |  | Compare Holography and Photography. Explain the construction and reconstruction of hologram. | CO3 | 20 |  |  |  |

ALL THE BEST