Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2017

| Code : | 14EI2024 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | POWER PLANT INSTRUMENTATION | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | With a neat sketch, explain the hydro electric power generation system. | CO1 | 10 |
|  | b. | Discuss the different methods involved in generating solar power. | CO1 | 8 |
|  | c. | Distinguish between Nuclear Fission and Nuclear Fusion. | CO1 | 2 |
| (OR) |  |  |  |  |
| 2. | a. | Examine how heat energy of steam contributes in generation of electrical energy. | CO1 | 14 |
|  | b. | Identify the role of instrumentation in power plants. | CO2 | 4 |
|  | c. | If a boiler consumes 110 T/hr feed water and delivers 85 T/hr steam throughout the day, calculate i. average rate of blow down and ii. total blow down for a day assuming there is no other water loss in the system. | CO2 | 2 |
| 3. | a. | Illustrate the operation of control rooms present in the thermal power plant. | CO1 | 14 |
|  | b. | Generators of 4 poles configuration and 8 poles configuration is available to be used with steam turbine and hydro turbine. Select the suitable generator after finding out their synchronous speed. Calculate the speed of the generator with a frequency of 50Hz. | CO1 | 4 |
|  | c. | Outline the differences between an economizer and air preheater. | CO1 | 2 |
| (OR) |  |  |  |  |
| 4. | a. | Design a V/I converter used in process industries for grounded load and floating load conditions. | CO2 | 12 |
|  | b. | Represent the steam flow control using P&I diagram. | CO2 | 6 |
|  | c. | List any two merits of Nuclear Power Plant. | CO1 | 2 |
| 5. | a. | Summarize the techniques used to analyze the impurities present in the feed water. | CO2 | 10 |
|  | b. | Explain the working of CT and PT with necessary diagrams. | CO2 | 8 |
|  | c. | A 250:5 current transformer is used along with an ammeter. If ammeter reading is 4.6A . Determine the line current. | CO2 | 2 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the methodology used in measuring the steam flow in power plants. | CO2 | 10 |
|  | b. | Give an outline on the internal and external treatment methods adopted in treating the boiler feed water. | CO2 | 10 |
| 7. | a. | Illustrate how water level is measured in the boiler drum and transmitted to a remote location. | CO2 | 12 |
|  | b. | Relate how shrinking and swelling affects the drum level measurement. | CO2 | 8 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the working of Infrared Gas Analyser. | CO3 | 10 |
|  | b. | Examine how smoke and dust are detected from the flue gas using smoke detector and dust monitor. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | Discuss the hardware and software systems of DCS for power plant automation with necessary diagrams. | CO3 | 20 |

ALL THE BEST