Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2017

| Code : | 14EI3016 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | SCADA SYSTEMS AND APPLICATIONS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Develop a ladder logic to detect and reject faulty components. Components are transported on a conveyor past a retro-reflective type photoelectric switch. The photoelectric switch is positioned at a height (H) above the conveyor where (H) represents a tolerance value for component height. Good components pass underneath the photoelectric switch and no signal is generated. Faulty components break the light beam twice as they pass the photoelectric switch. | CO1 | 15 |
|  | b. | List the various functions of SCADA in Process Industries. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. |  | Explain in detail the architecture of SCADA with the necessary diagrams. | CO1 | 20 |
| 3. | a. | Calculate the Scan interval for a SCADA system with the following Specifications: - It has initially 7 RTU’s and has increased to 40. - The total number of bits to be transmitted is 1050 - Data rate used is 1200 bps - Communication efficiency is 60% | CO1 | 10 |
|  | b. | Elaborate on the different communication philosophies of SCADA. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. |  | Explain in detail the Reference Communication OSI model and elaborate on the DNP3 protocol used in SCADA communication. | CO2 | 20 |
| 5. |  | Comprehend the need of RTU in a SCADA and discuss in detail the Signals and protocols involved in RTU communication. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | How are the control signals classified in a RTU and elaborate on the different methods of monitoring the control signals? | CO2 | 20 |
| 7. | a. | Configure a SCADA system to keep track of the fluid flowing in a pipeline and alert the MTU under leakage conditions. Assume a system with Three RTU’s communicating with one MTU. | CO2 | 15 |
|  | b. | With a neat block diagram explain how MTU stores data in Central Data Store. | CO3 | 5 |
| (OR) |  |  |  |  |
| 8. |  | Elaborate on the economic and business factors that are considered in evaluating the benefits of using SCADA for a system. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Present a detailed case study on SCADA monitoring and control of a Gas lift system. | CO3 | 20 |

ALL THE BEST