Reg.No. ____________

End Semester Examination – Nov/Dec - 2017

| Code : | 14FP2007 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | UNIT OPERATIONS IN FOOD PROCESS ENGINEERING-I | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain the principle and working of Fluidized bed dryer with a neat sketch. | CO1 | 10 |
|  | b. | Calculate the amount of moisture evaporated from 500 kg of grain for drying it from an initial moisture content of 28% (wet basis) to a final moisture content of 13% (wet basis). | CO2 | 10 |
| (OR) |  |  |  |  |
| 2. |  | Determine the values of “c” and “n” from Henderson’s equation for the following data. RH (%); t (℃); Me (%). 50; 50; 6.5. 70; 50; 12.4. | CO2 | 20 |
| 3. |  | Derive the formulae for constant rate and constant pressure filtration processes. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Sketch the figure of basket type centrifuge and explain its working. | CO2 | 10 |
|  | b. | A centrifuge is used to separate oil in a dispersion of fluid. The oil particles have a diameter of 5.76 x 10-5 m and its density is 960 kg/m3. The centrifuge rotates at 3000 rpm and the effective radius at which separation occurs is 4 cm. The specific gravity of the fluid is 0.8 and viscosity 6.3 x 10 -4 Pa.s. Calculate the following: i. angular velocity and ii. settling velocity of the particle in the centrifuge. | CO3 | 10 |
| 5. |  | Derive equations for material and energy balances in single effect evaporators. | CO1 | 20 |
| (OR) |  |  |  |  |
| 6. |  | A Single effect evaporator is to be used to concentrate a food solution containing 15% (by mass) dissolved solids to 50% solids. The feed stream enters the evaporator at 291 K with a feed rate of 1 kg/s. Steam is available at a pressure of 2.4 bar and an absolute pressure of 0.07 bar is maintained in the evaporator. Assuming that the properties of the solution are the same as those of water and taking the overall heat transfer coefficient to be 2300 W/m2 K, calculate the rate of steam consumption and the necessary heat transfer surface area. The specific enthalpy data obtained from the steam tables are given below: Enthalpy of steam (hS) = 2715 kJ/kg; Enthalpy of condensate(hC)=530 kJ/kg; Enthalpy of feed (hF) = 75.5 kJ/kg; Enthalpy of vapour (hV) = 2572 kJ/kg; Enthalpy of concentrated liquor (hL) = 163 kJ/kg. The temperature of steam (TS) at 2.4 bar is 126.1℃ and the temperature of the saturated liquid water (TE) at the evaporator at a pressure of 0.07 bar is 39℃. | CO3 | 20 |
| 7. | a. | Describe the working of kneader with a neat sketch and enumerate the applications of mixing in food processing. | CO2 | 10 |
|  | b. | A fortified dough was made by adding 20% soya flour to the maida flour. The two dry flours were mixed in a ribbon mixer to make the dough. After a time of 10 minutes, three samples were collected and analysed for soya flour and the fractional composition obtained were: 0.25, 0.23 and 0.22. Calculate the standard deviation, mixing index and the time needed for proper mixing for getting a standard deviation of 0.01. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Draw a neat diagram of planetary mixer and explain its working. | CO2 | 10 |
|  | b. | During the preparation of a dough, 10 kg of sugar was mixed with 990 kg of flour in a sigma mixer. Five samples were taken after 10 minutes of mixing and analysed for the percentage of sugar in the mixture. The results were: 0.76, 0.72, 0.85, 0.76, 0.82. Calculate the mixing index and the time required for perfect mixing. Consider σα = 0.01 | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss the principles and laws of grinding. | CO1 | 10 |
|  | b. | Explain the working of crushers with suitable diagram. | CO2 | 10 |

ALL THE BEST