Reg.No. ____________

End Semester Examination – Nov/Dec – 2017

| Code : | 14FP2010 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | UNIT OPERATIONS IN FOOD PROCESS ENGINEERING-II | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Show that the binary gas mixture have same diffusivity coefficient using molecular diffusion of gases. | CO1 | 10 |
|  | b. | Draw and explain the flash distillation process. | CO2 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | A steam distillation process is used for separating out an oil from mint leaves by passing steam into the chamber at 60 kPa at the rate of 110 kg/h for 10 minutes. If the molecular weight of the mint oil is 165, find the quantity of mint oil distilled. Assume the total pressure as 101.3 kPa. | CO2 | 8 |
|  | b. | Water alcohol mixture of about 100kg is separated in a batch distillation unit until the left out concentration of alcohol in the bottom liquid is 10%. The feed initially contains 40% alcohol. All the composition are on molar basis. Find out the moles of alcohol produced in the process (Assume the value of YD as 0.46 and XD as 0.3145) and derive the material balance for the above process. | CO2 | 12 |
| 3. | a. | The natural food colour from beet root is to be extracted with water. The saturated concentration of the colour in water is found to be 1.6 kg/m3. In a lab scale extractor containing about one litre volume, it has taken 10 minutes to extract the colour from beet root to an extent of 970 ppm. Under similar condition in a commercial plant of 10 m3 capacity, it is desired to extract 12 kg of colour into the water. How much time does it take for extraction? | CO1 | 10 |
|  | b. | Differentiate extraction and distillation. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. |  | Describe the various types of extraction equipments with a neat sketch. | CO2 | 20 |
| 5. | a. | Point out the applications of adsorption in food processing and enlist the generally used adsorbent. | CO2 |  |
|  | b. | Calculate the nucleation rate of sodium chloride crystals at 20°C which have a surface tension of 6.92ergs/cm2. The solution has a fractional super saturation of (s value) of 0.025. the density of NaCl is 2.163 x 103 kg/m3. and molecular weight of NaCl is 58.45. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | State the crystallization principle. | CO1 | 5 |
|  | b. | Describe the batch crystallizer and draft tube baffle crystallizer (DTB) and with a neat diagram and explain its application in food processing. | CO2 | 15 |
| 7. | a. | Discuss the absorption equipment used in food industries. | CO2 | 10 |
|  | b. | List out the application and limitations of SCF. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | List the various applications of absortion in food processing. | CO1 | 10 |
|  | b. | The concentration of whey is being accomplished by using an ultrafiltration membrane to separate water. The 500 kg/hour of feed stream has 6 % solids and is being increased to 24 % total solids. The membrane tube has a 8 cm inside diameter and the pressure difference is 2000 kPa. Estimate the flux of water through the membrane and length of the membrane tube when the hydraulic permeability is 3.7x10-5 kg of water/m2k Pa. s | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss the various membrane materials and various membrane modules | CO1 | 10 |
|  | b. | An ultrafiltration membrane was examined microscopically and found to have about 1,80,000 pores with an average diameter of 0.6 x 10-6 m per mm2 of membrane surface. The thickness of the membrane is 140x10-6 m. The viscosity of the permeate is 0.0013 Pa.s. Estimate the following : i. Porosity of the membrane (€), ii. Hydraulic Permeability (LP) and c) Permeate Flux(J) for a Trans membrane Pressure Difference(∆PTM) of 1.5 Pa. | CO2 | 10 |

ALL THE BEST