Reg.No. ____________

End Semester Examination – Nov/Dec 2017

| Code : | 14FP2016 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | PHYSICAL PROPERTIES OF FOOD MATERIALS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | How do you measure the volume of vegetables using liquid displacement method? | CO1 | 8 |
|  | b. | Illustrate with a diagram the working principle of lactometer. | CO2 | 8 |
|  | c. | The major diameter (L) and the average of the minor and major diameters (H) of barley are measured as 6 mm and 2 mm, respectively. Calculate the minimum and maximum radii of curvature for the barley. | CO3 | 4 |
| (OR) |  |  |  |  |
| 2. | a. | Cherry has a moisture content of 77.5% (wb). The apparent and bulk densities are 615 kg/m3 and 511 kg/m3 at 25◦C, respectively. Assuming cherry contains only carbohydrate and water, calculate the total porosity of cherries when stacked as bulk. The densities of carbohydrate and water are 1586 kg/ m3 and 997 kg/ m3, respectively | CO1 | 4 |
|  | b. | Explain any two methods of determination of porosity of food materials. | CO2 | 8 |
|  | c. | Write in brief about angle of repose. | CO3 | 8 |
| 3. | a. | Describe the factors affecting the rheological parameters of foods. | CO2 | 10 |
|  | b. | Explain the applications of Compression and snapping bending tests for the measurement of texture of biscuits and Chocolates. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Describe the instruments used for the testing of quality of dough. | CO3 | 10 |
|  | b. | Describe with a neat sketch about the Texture Profile analysis | CO2 | 10 |
| 5. | a. | Derive an expression for thermal conductivity of gases from the principles of Kinetic Theory of gases. | CO1 | 10 |
|  | b. | The composition of date fruit (Phoenix dactylifera) and the densities of food components are given below. Determine the thermal conductivity of the fruit at 25◦C, using parallel, and series models. Component; Weight(%); Density (kg/m3). Water; 22.5; 995.7. Carbohydrate; 72.9; 1592.9. Protein; 2.2; 1319.6. Fat; 0.5; 917.15. Ash; 1.9; 2418.2. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Explain any two methods for determination of thermal conductivity in steady state methods. | CO2 | 10 |
|  | b. | Describe in detail about a. Boiling point elevation b. Freezing point depression. | CO1 | 10 |
| 7. | a. | Explain the following i. Henry’s law. ii. Raoult’s law. iii. Boiling point elevation. iv.Osmotic pressure. | CO2 | 10 |
|  | b. | Describe with a neat sketch Water Activity Determination by Vapor Pressure Measurement method. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Explain in brief about dipolar rotation. | CO2 | 8 |
|  | b. | Estimate the penetration depth of a chicken meat during processing in home type microwave oven. Chicken meat has a dielectric constant of 53.2 and dielectric loss factor of 18.1. Assume that dielectric properties are constant during heating. | CO3 | 12 |
|  |  | Compulsory: |  |  |
| 9. | a. | Describe the influence of moisture over Dielectric constant and Dielectric loss factor. | CO2 | 15 |
|  | b. | Summarize the basic principles of microwave heating | CO3 | 5 |

ALL THE BEST