Reg.No. ____________

End Semester Examination – Nov/Dec - 2017

| Code : | 14ME1003 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | BASIC MECHANICAL ENGINEERING | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | With neat sketch, explain the working of Cochran boiler. | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. | a. | Discuss the working of two stroke cycle petrol engine with the help of neat sketches. | CO1 | 15 |
|  | b. | Differentiate between petrol and diesel engines. | CO1 | 5 |
| 3. |  | Draw the layout of a hydro-electric power plant. State its advantages and disadvantages. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | With neat sketch explain the working principle of geothermal power plant. | CO1 | 20 |
| 5. |  | A tension test was conducted on a steel rod at strength of material laboratory and the following observation were made: Initial Diameter = 14mm Gauge length = 100mm Load at yield point = 41.5KN Maximum load applied = 69.2KN Load at breaking = 61.6KN Diameter at the neck = 11.2mm Elongation at break= 38mm Elongation at load 25.1 KN (P) = .08mm (dl) Determine:i.Young’s modulus or Modulus of elasticity. ii. Yield stress. iii. Ultimate Stress. iv. Nominal Stress at breaking. v. Actual Stress at breaking. vi. Percentage elongation. vii. Percentage reduction in cross - sectional area. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Explain the importance of elastic region, plastic region and endurance limit in a stress- strain curve. | CO2 | 20 |
| 7. | a. | How to classify engineering materials. | CO2 | 2 |
|  | b. | List any five mechanical properties of metals. | CO2 | 3 |
|  | c. | What are the various alloys of copper? List their applications. | CO2 | 5 |
|  | d. | Discuss the applications of mild steel, medium carbon, high carbon and stain less steels. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Differentiate between Green sand molding and dry sand molding. | CO2 | 10 |
|  | b. | Explain five types of pattern allowances with simple sketches. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | i. Explain the principle of gas welding process with a neat sketch. ii. Draw the three types of flames. | CO2 | 20 |

ALL THE BEST