Reg.No. ____________

End Semester Examination – Nov/Dec – 2017

| Code : | 14ME2014 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ENGINEERING THERMODYNAMICS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

#### (Use of Steam Tables, Refrigeration Table and Compressibility chart are permitted)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A piston cylinder machine contains a fluid system which passes through a complete cycle of four processes. During a cycle, the sum of all heat transfer is -170 kJ. The system completes 100 cycles per minute. Complete the following table showing the method for each item, and compute the net rate of work output in kW. Process; Q (kJ/min); W (kJ/min); ΔE (kJ/min). a - b; 0; 2170; ---. b - c; 21000; 0; ---. c – d; -2100; ---; -36600. d - a; ---; ---; ---. | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. | a. | Air at a temperature of 150C passes through a heat exchanger at a velocity of 30 m/s where its temperature is raised to 8000C. It then enters a turbine with the same velocity of 30 m/s and expands until the temperature falls to 6500C. On leaving the turbine, the air is taken at a velocity of 60 m/s to a nozzle where it expands until the temperature has fallen to 5000C. If the air flow rate is 2 kg/s, calculate the rate of heat transfer to the air in the heat exchanger and the power output from the turbine assuming no loss. Also find the velocity at the exit nozzle assuming no heat loss. Take the enthalpy of air as h = cpt where cp is the specific heat equal to 1.005 kJ/kg K and t is temperature. | CO1 | 20 |
| 3. | a. | A reversible heat engine operates between two reservoirs at temperatures of 600˚ C and 40˚C. The engine drives a reversible refrigerator which operates between reservoirs at temperatures of 40˚C and 20˚C. The heat transfer to the heat engine is 2000kJ and the net work output of the combined engine refrigerator plant is 360kJ. Evaluate the heat transfer to the refrigerant and the heat transfer to the reservoir at 40˚ C. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | State and explain the Carnot’s theorem with suitable block diagram. | CO1 | 12 |
|  | b. | A refrigeration plant for a food store maintains a temperature of – 10C. The ambient air temperature is 32C. If heat leaks into the plant at the continuous rate of 2 kJ/s, what is the least power necessary to pump this heat out continuously? | CO1 | 8 |
| 5. | a. | One kg of water at 273 K is brought into contact with a heat reservoir at 373 K. When the water has reached 373 K, find the entropy change of the water, the reservoir and the universe. | CO2 | 10 |
|  | b. | If water is heated from 273 K to 373 K by first bringing it in contact with a reservoir at 323K and then with a reservoir at 373 K , what will be the entropy change of the universe? | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Steam at 0.8Mpa, 2500C and flowing at the rate of 1 kg/s passes into a pipe carrying wet steam at 0.8 Mpa, 0.95 dry. After adiabatic mixing the flow rate is 2.3 kg/s. Determine the condition of steam after mixing. The mixture is then expanded in a frictionless nozzle isentropically to a pressure of 0.4 Mpa. Determine the velocity of the steam leaving the nozzle. Neglect the velocity of steam in the pipeline. | CO3 | 20 |
| 7. | a. | A mixture of ideal gases consists of 3 kg of nitrogen and 5 kg of carbondioxide at a pressure of 300 kPa and a temperature of 200C. find - The mole fraction of each constituent - The equivalent molecular weight of the mixture - The equivalent gas constant of the mixture - The partial pressure and partial volumes - The volume density of the mixture - The cp and cv of the mixture Take γ for CO2 and N2 to be 1.286 and 1.4 respectively. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Two vessels A and B both containing nitrogen are connected by a valve which is opened to allow the contents to mix and achieve an equilibrium temperature of 270C. Before mixing vessel A had pressure, temperature and contents as 1.5 Mpa, 500C and 0.5 kg mol respectively. Vessel B had pressure, temperature and contents as 0.6 Mpa, 200C and 2.5 kg mol respectively. Calculate the final equilibrium pressure, and the amount of heat transferred to the surroundings. If the vessel had been perfectly insulated, calculate the final temperature and pressure which would have been reached. Take γ = 1.4 | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Atmospheric air at 1.0132 bar has a DBT of 320C and WBT of 260C. Compute - The partial pressure of water vapour - The specific humidity - The dew point temperature - The relative humidity - The degree of saturation - The density of the air in the mixture - The density of vapour in the mixture - The enthalpy of the mixture. | CO4 | 20 |

ALL THE BEST