Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2017

| Code : | 14ME2029 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | DESIGN OF MACHINE ELEMENTS | Max. marks : | 100 |

(Use of DESIGN DATA book & Approved data sheets are permitted. )

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Define Engineering Design. List the steps involved in a systematic design process. | CO1 | 4 |
|  | b. | A cylindrical member is loaded as shown in Figure. A load of 5000 N acts in a direction parallel to the area of cross section of the member. Determine the maximum shear stress induced in the member. | CO2 | 16 |
| (OR) |  |  |  |  |
| 2. | a. | Describe the material properties: hardness, stiffness and resilience. | CO1 | 4 |
|  | b. | A steel bracket is loaded as shown in the Fig 1. The section at A-A is rectangular whose width is 40 mm and thickness ‘t’ mm. The permissible tensile stress of the material of the bracket is 60 N/mm2. Determine the thickness of the bracket. Fig 1 | CO2 | 16 |
| 3. | a. | How are springs classified? Mention the application of each type. | CO1 | 6 |
|  | b. | Design a Helical compression spring to carry a load of 1.5kN with a deflection of 40mm, spring index is 5, allowable shear stress is 400 N/mm2 and modulus of rigidity is 8x1010 N/m2 . | CO2 | 14 |
| (OR) |  |  |  |  |
| 4. | a. | State the Guest’s theory and mention its contribution to design | CO1 | 4 |
|  | b. | A line shaft running at 1000 rpm transmits 20 kW. It carries a central load of 1 kN. It is supported in bearings, the distance between their centres being 1 m. Determine the diameter of shaft assuming minor shock load with permissible shear stress of 45 N/mm2 and bending stress of 70 N/mm2. | CO3 | 16 |
| 5. | a. | Distinguish between shaft and an axle. | CO1 | 4 |
|  | b. | Design a rigid flange coupling to transmit 20kW at 1000 rpm, assuming suitable allowable stresses. | CO2 | 16 |
| (OR) |  |  |  |  |
| 6. | a. | How are couplings classified? | CO1 | 4 |
|  | b. | Design a muff coupling to transmit 35kW from a shaft running at 120 rpm. The following permissible stresses may be used: Allowable shear stress for shaft and key material 42 N/mm2 , Allowable crushing stress for key 84 N/mm2 , Allowable shear stress for coupling 14 N/mm2 | CO2 | 16 |
| 7. | a. | Classify welded joints and riveted joints with simple sketches. | CO1 | 4 |
|  | b. | Design a Knuckle Joint for a tie rod of a circular section to sustain a maximum pull of 70 kN. The ultimate strength of the material of the rod against tearing is 420 MPa. The ultimate tensile and shearing strength of the pin material are 510 MPa and 396 MPa respectively. | CO3 | 16 |
| (OR) |  |  |  |  |
| 8. |  | The connecting rod of a petrol engine is to be designed for the following data. Piston diameter 80mm, stroke 120mm, weight of reciprocating parts15N, length of connecting rod 240mm, maximum speed 2800 rpm, explosion pressure corresponding to 10° of crank angle is 3MPa, factor of safety 6. If the connecting rod is to be made of 40Cr1 steel, find the dimensions of the I-section connecting rod. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Design a CI piston for a single acting four stroke I.C.engine for the following specifications. Cylinder bore 100 mm, stroke length120 mm, maximum gas pressure 6MPa,Brake mean effective pressure 0.7Mpa,Fuel consumption 0.24kg /kW/hr,Speed 2200rpm | CO2 | 20 |

ALL THE BEST