Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2017

| Code : | 14ME2030 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | DESIGN OF HEAT EXCHANGERS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | What are the types of heat exchangers used in a conventional plant? Classify them and discuss. | CO1 | 15 |
|  | b. | Write the classification of heat exchangers based on heat transfer mechanism? | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | What are the principle types of shell and tube construction? Write the advantages and disadvantages? | CO1 | 15 |
|  | b. | What are the types of fins that are used in heat exchangers? | CO1 | 5 |
| 3. | a. | Explain basic equations to be considered in heat exchanger design? | CO2 | 15 |
|  | b. | Explain “overall heat transfer coefficient” in heat exchangers? | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. |  | Consider a shell and tube heat exchanger constructed from a 0.0254m OD to cool 6.93 kg/s of a 95% ethyl alcohol solution (Cp=3810 J/kg K) from 66 °C to 42°C using 6.3 kg/s of water available at 10 °C (Cp=4187 J/kg K). In the heat exchanger, 72 tubes will be used. Assume that the overall coefficient of heat transfer based on the outer tube area is 568 /m2K. Calculate the surface area and the length of the heat exchanger for each of the following arrangements. - parallel flow shell and tube heat exchanger - counter flow shell and tube heat exchanger | CO2 | 20 |
| 5. | a. | Explain the equations used to calculate the pressure drop on tube side in heat exchangers? | CO2 | 15 |
|  | b. | Discuss the effects of fouling on pressure drop? | CO2 | 5 |
| (OR) |  |  |  |  |
| 6. |  | A double pipe heat exchanger is used to condense steam at a rate of 120 kg/hr at 45°C. Cooling water (sea water) enters the inner tube at a rate of 1.2 kg/s at 15°C. The tube with 25.4 mm OD, 22.1 mm ID is made of mild steel, k= 45 W/mK. The heat transfer coefficient on the steam side ho is 700 W/m2K. Calculate overall heat transfer coefficient under clean and fouled conditions. | CO2 | 20 |
| 7. | a. | Explain different types of compact heat exchangers? | CO1 | 10 |
|  | b. | Explain the heat transfer enhancement in compact heat exchangers? | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Write the operational characteristics of plate heat exchangers? | CO3 | 15 |
|  | b. | Write the main advantages offered by gasketed plate heat exchangers? | CO3 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Write the design and operational considerations during the selection of condensers? | CO3 | 15 |
|  | b. | What are the main reasons for failure of condenser operation? | CO3 | 5 |

ALL THE BEST