Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2017

| Code : | 14ME2036 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | MECHANICAL VIBRATIONS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Formulate the equation of motion for the figure given using Newton’s law. | CO1 | 10 |
|  | b. | For the above question derive the equation of motion using Energy method and compare the results | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Compile the steady state undamped forced vibration response of the general two degree of freedom as in figure | CO1 | 14 |
|  | b. | Derive the equation of motion for undamped free vibration with a single degree of freedom | CO1 | 6 |
| 3. | a. | Interpret the equation of motion for the single degree of freedom damped free vibration with practical examples for each type | CO1 | 13 |
|  | b. | In a single degree damped vibration system a suspended mass of 5 kg makes 20 oscillations in 10 seconds. When disturbed from its equilibrium position. The amplitude of vibration reduces to 50% of its initial value after 5 oscillations. Determine i. stiffness of the spring ii. logarithmic decrement iii. damping factor iv. damping co-efficient | CO2 | 7 |
| (OR) |  |  |  |  |
| 4. | a. | An under damped shock absorber is to be designed for motor cycle shock absorber mass 100kg. The shock absorber of subjected to initial velocity due to road bump. Find the necessary stiffness and damping constant of shock absorber. If the damped period of vibration is to 5 seconds. If the amplitude reduces to ¼ in ½ cycle. Find also the minimum initial velocity that leads to maximum displacement of 200mm. | CO2 | 14 |
|  | b. | A gun barrel of mass 600 kg has a recoil spring of stiffness of 294000N/m. If the barrel recoil is 1.3m on firing. Determine i. initial velocity of barrel ii. the critical damping coefficient of dashpot which is engaged at the end of the recoil stock iii. the time required for the barrel to return to position from the initial position. | CO2 | 6 |
| 5. | a. | Predict the equation of motion of under damped forced vibrations (single degree of freedom system). | CO1 | 15 |
|  | b. | Enumerate the dynamic magnifier and derive the same. | CO4 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | A mass of 10kg is suspended from one end of helical spring and other end being fixed the stiffness 10N/m. The viscous damping causes amplitude to decrease to 1/10 of initial value in four complete oscillations. If the periodic force of 150 cos 50t is applied in the mass in vertical direction. Find the amplitude of force vibration and the resonance value. | CO2 | 10 |
|  | b. | A machine part of mass 2 kg vibrate in viscous medium. Determine the damping coefficient if harmonic existing force is 25N. Results in a resonance produces an amplitude 25mm in 0.2 seconds. If the system is excited by harmonic force or frequency force, what will be the increase in amplitude of vibration when damper is removed as compared without damper and with damper. | CO2 | 10 |
| 7. | a. | Explain about the various vibration measuring devices. | CO5 | 5 |
|  | b. | Develop the equation of motion for a longitudinal vibration of a bar | CO3 | 15 |
| (OR) |  |  |  |  |
| 8. | a. | Derive the equation of motion of a transverse vibration of beam. | CO5 | 15 |
|  | b. | Describe about vibration exciters. | CO5 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | For a two degree of freedom pendulum, construct the equation of motion. | CO1 | 15 |
|  | b. | Examine the role of masses and the lengths in the frequency of the system. | CO1 | 5 |

ALL THE BEST