Reg.No. ____________

End Semester Examination – Nov/Dec - 2017

| Code : | 14ME2015 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | THERMAL ENGINEERING I | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | The following readings are taken during the test on a boiler for one hour. Steam generated =5400 kg, calorific value of coal = 31500 kJ/kg, coal burnt = 700kg, dryness fraction of steam entering superheater = 0.92, rated pressure of boiler = 11.5 bar, temperature of steam leaving super heater = 250°C, temperature of hot wall = 45°C. Determine : - Equivalent evaporation per kg of fuel with and without super heater - Thermal efficiency of the boiler with and without super heater - Amount of heat supplied by the super heater per hour. | CO1 | 15 |
|  | b. | Write the difference between water and fire tube boiler? | CO1 | 3 |
|  | c. | Define Evaporative capacity. | CO1 | 2 |
| (OR) |  |  |  |  |
| 2. | a. | The following observation were recorded during a boiler trial: Pressure of steam = 10 bar, Steam condenser = 540 kg/hr Fuel used = 65 kg/hr, Moisture in fuel = 2% by mass, Mass of dry flue gases = 9 kg/kg of fuel, Lower calorific value of fuel = 32,000 kJ/kg Temperature of the flue gases = 325° C Temperature of boiler house = 28° C , Feed water temperature = 50° C , Mean specific heat of flue gases = 1 kJ/kg K, Dryness fraction of steam = 0.95, Specific heat of superheated steam = 2.1kJ/kgK. Draw up a heat balance sheet for the boiler. | CO1 | 20 |
| 3. | a. | Estimate the mass flow rate of steam in a nozzle with the following data: Inlet pressure = 10bar, Inlet temperature =200°C, back pressure =0.5bar, throat diameter =12mm: | CO2 | 10 |
|  | b. | Dry saturated steam at 10 bar is expanded isentropically in a nozzle to 0.1bar. Find the dryness fraction of the steam at exit. Also find the velocity of steam leaving the nozzle when i. initial velocity is negligible ii. initial velocity of the steam is 135m/s | CO2 | 7 |
|  | c. | Define nozzle efficiency. | CO2 | 3 |
| (OR) |  |  |  |  |
| 4. | a. | Derive an equation for the mass of steam discharged through nozzle and also the equation for the critical pressure ratio. | CO2 | 20 |
| 5. | a. | Draw the combined velocity triangle for an impulse turbine and derive the power produced by an impulse turbine. | CO3 | 15 |
|  | b. | Write the difference between impulse and reaction turbine? | CO3 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | The velocity of steam, leaving the nozzle of an impulse turbine, is 1200m/s and the nozzle angle is 20°. The blade velocity is 375 m/s and the blade velocity coefficient is 0.75. Assuming no loss due to shock at inlet, calculation for a mass flow of 0.5 kg/s and symmetrical blading: i. blade inlet angle; ii. driving force on the wheel; iii. axial thrust on the wheel; and iv. power developed by the turbine. | CO3 | 15 |
|  | b. | Elaborate the advantages of steam turbines over reciprocating steam engines. | CO3 | 5 |
| 7. | a. | Determine the size of the cylinder for a double acting air compressor of 40kW indicated power, in which air in at 1 bar and 15°C and compressed according to the law pv1.2= constant, to 6 bar. The compressor runs at 100rpm with average piston speed of 152.5m/min. Neglect clearance | CO4 | 10 |
|  | b. | Define i. single acting compressor ii. Single stage compressor iii. Volumetric efficiency iv. Displacement of the compressor v. Free air delivery. | CO4 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | A single stage, single acting reciprocating air compressor has a bore of 200mm and a stoke of 300mm. It receives air at 1 bar and 20°C and delivers it at 5.5 bar. If the compression follows the law pv1.3= constant and clearance volume is 5 percent of the stroke volume, determine: i. The mean effective pressure ii. the power required to drive the compressor, if it runs at 500 rpm. | CO4 | 10 |
|  | b. | List the advantages of multi stage compression. | CO4 | 5 |
|  | c. | Estimate the minimum work required to compress 1 kg of air from 1 bar 27°C to 16 bar in two stages, if the law of compression is pv1.25= constant and the intercooling is perfect. Take R = 287 J/kgK. | CO4 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain briefly the principle of vapour compression refrigeration system. | CO5 | 15 |
|  | b. | Differentiate vapour compression and absorption refrigeration systems | CO5 | 5 |

ALL THE BEST