Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2017

| Code : | 15EI2036 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | RADIATION AND NUCLEAR MEDICINE | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Discuss the working and application of the different types of modern X-ray tubes. | CO1 | 12 |
|  | b. | List the possible outcomes when an X ray impinges on a material. | CO1 | 5 |
|  | c. | Predict the reason behind the evacuation of X-ray tubes and how it is done? | CO1 | 3 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the Characteristic spectrum in the production of X-Rays. | CO1 | 10 |
|  | b. | Compare and contrast the relationship between the filament current and tube current that flows in the X-ray tube. | CO1 | 5 |
|  | c. | Write short notes on the types of X-rays. | CO1 | 5 |
| 3. | a. | Explain the working operation and the system components of a CT scanner. | CO2 | 14 |
|  | b. | Discuss on Hounsfield units. | CO2 | 6 |
| (OR) |  |  |  |  |
| 4. | a. | Explain how images are captured in digital radiography. | CO1 | 12 |
|  | b. | Identify the limitations of conventional radiography. | CO1 | 5 |
|  | c. | Determine brightness gain, if an image intensifier tube has a magnification gain of 16 and a flux gain of 2000? | CO1 | 3 |
| 5. | a. | Analyse the working of semiconductor detector in forward bias mode and reverse bias mode when it is exposed to ionizing radiation. | CO2 | 12 |
|  | b. | Predict the role of Electromagnetic radiation in Diagnostic Imaging. | CO2 | 8 |
| (OR) |  |  |  |  |
| 6. | a. | Analyze the image intensification process in fluoroscopy. | CO1 | 12 |
|  | b. | Suggest an instrument to detect the charges created by direct ionization within the gas through the application of an electric field. Explain its working. | CO2 | 8 |
| 7. | a. | Discuss the principle, system components, detector system and the electronics involved in the working of a Gamma Camera. | CO2 | 14 |
|  | b. | Illustrate the direct and indirect effects of radiation with pictorial representation. | CO3 | 6 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the methodologies involved in treatment planning and dosage in radiotherapy. | CO3 | 10 |
|  | b. | Discuss on Positon Emission Tomography. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain the procedure in Brachy therapy and its advantages. | CO3 | 12 |
|  | b. | Examine the types of personal dosimeter that is used to protect the wearer from harmful effects of radiation. | CO3 | 8 |

ALL THE BEST