Reg.No. ____________

End Semester Examination – Nov/Dec – 2017

| Code : | 16AE2001 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | STRUCTURAL MECHANICS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | An axial pull of 35000N is acting on a bar consisting of three lengths as shown in fig. below. If the Young’s modulus =2.1×105 N/mm2,determine: i.Stresses in each section and ii. Total extension of the bar. | CO1 | 10 |
|  | b. | A member ABCD is subjected to point loads P1,P2,P3 and P4 as shown in fig. below. Take A1=625mm2;A2=2500 mm2 ;A3=1250 mm2 Calculate the force P2 necessary for equilibrium, if P1=45kN, P3=450kN and P1=130kN.Determine the total elongation of the member, assuming the modulus of elasticity to be 2.1×105N/mm2. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. |  | A Steel rod of 20mm diameter passes centrally through a copper tube of 50mm external diameter and 40mm internal diameter.The tube is closed at each end by rigid plates of negligible thickness.The nuts are tightened lightly home on the projecting parts of the rod.If the temperature of the assembly is raised by 50 ͦ C,calculate the stresses developed in copper and steel.Take E for steel and copper as 200GN/m2 and 100 GN/m2 and α for steel and copper as 12×10-6per ͦ C and 18×10-6per ͦ C. | CO1 | 20 |
| 3. | a. | Derive an expression for the stresses on an oblique plane of a rectangular body, when the body is subjected to a simple shear stress. | CO1 | 10 |
|  | b. | A point in a strained material is subjected to following stresses. Major principal stressσ1=65 N/mm2; Minor principal stressσ2=35 N/mm2; shear stress τ=25 N/mm2.Determine the normal and tangential stresses across the oblique plane which is at an angle of 45 ͦ analytically or using Mohr’s circle method. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Draw shear force and bending moment diagrams of a simply supported beam of length 7m carrying uniformly distributed loads as shown in fig. below. | CO2 | 10 |
|  | b. | Draw shear force and bending moment diagrams for a cantilever beam of length 2m carries a uniformly distributed load of 1.5kN/m run over the whole length and a point load of 2kN at a distance of 0.5m from the free end. | CO2 | 10 |
| 5. | a. | What are the assumptions for simple bending? | CO2 | 5 |
|  | b. | Derive an expression for bending stress at a layer in a beam in terms of young’s modulus. | CO2 | 15 |
| (OR) |  |  |  |  |
| 6. | a. | Derive an expression for the slope and deflection of a simply supported beam of length L, carrying a point load W at the centre by double integration method. | CO3 | 10 |
|  | b. | Derive an expression for the slope and deflection of a cantilever beam of length L, carrying a point load at the free end by moment area method. | CO3 | 10 |
| 7. |  | A beam of length 6m is simply supported at its ends and carries two point loads of 48kN and 40kN at a distance of 1m and 3m respectively from the left support. Find: i.deflection under each load ii. maximum deflection iii. the point at which maximum deflection occurs. Given E=2×105N/mm2 and I=85×106 mm4 | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | A leaf spring carries a central load of 3000N.The leaf spring is to be made of 10 steel plates 5cm wide and 6mm thick.If the bending stress is limited to 150N/mm2 determine: i.Length of the spring ii. Deflection at the centre of the spring.Take E=2×105N/mm2 | CO3 | 10 |
|  | b. | A closed coil helical spring is to carry a load of 500N.Its mean coil diameter is to be 10 times that of the wire diameter. If the maximum shear stress in the material of the spring is to be 80 N/mm2, the stiffness of the spring is 20N/mm and modulus of rigidity=8.4×104N/mm2, find the number of coils in the closely coiled helical spring. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | A solid circular shaft and a hollow circular shaft whose inside diameter is (3/4) of the outside diameter, are of the same material,of equal lengths and are required to transmit a given torque.Compare the weights of these two shafts if the maximum shear stress developed in the two shafts are equal. | CO3 | 20 |

ALL THE BEST