Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2017

| Code : | 16EC2004 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ELECTRON DEVICES AND CIRCUITS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain in detail the operation of a PN junction Diode with appropriate circuit diagram, graph and support the same with the shockleys equation. | CO1 | 15 |
|  | b. | Determine the thermal voltage VT at a temperature of 27C given the Boltzmann’s constant (k) = 1.38 x 10-23 J/K and the magnitude of electronic charge (q)=1.6 x 10-19 C. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | With neat circuit diagram and corresponding waveforms explain in detail the operation of a center tapped full wave rectifier. | CO1 | 15 |
|  | b. | Determine VD, VR and ID for the series diode configuration shown below. Also reverse the diode and find VD |  | 5 |
| 3. | a. | Identify the configuration which is used the most and why? i. Common emitter ii.Common collector iii. Common base | CO1 | 2 |
|  | b. | Describe in detail Base Biasing in Bipolar Junction Transistor. | CO1 | 18 |
| (OR) |  |  |  |  |
| 4. | a. | Explain in detail the Voltage divider biasing and comment on its stability. | CO1 | 10 |
|  | b. | In the circuit shown below, assume that the transistor is in active region. It has a large beta and its base emitter voltage is 0.7 V.Find IC. | CO2 | 10 |
| 5. |  | Describe the Depletion Type MOSFET operation with a detailed Transfer curve and equation. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Represent the circuit given below in its AC equivalent model, Using re model for the transistor. | CO2 | 10 |
|  | b. | State the roles of the capacitors shown in the figure below. | CO2 | 10 |
| 7. | a. | Analyse the frequency response curve of a RC coupled Amplifier. | CO2 | 15 |
|  | b. | Determine the Voltage, Current and Power Gain of an amplifier that has an input signal of 1mA at 10mV and a corresponding output signal of 10mA at 1V. Also, express all three gains in decibels, (dB) | CO2 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Give the block diagram representation of Voltage Series feedback amplifier and derive the expression of input impedance, output impedance and gain of the amplifier. | CO3 | 10 |
|  | b. | Derive and Compare the maximum Efficiency of Class A and Class B amplifier and list out the advantages of class B amplifier over Class A amplifier. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | For a transistorized RC oscillator, select the value of capacitor C in order to provide 2 kHz oscillator frequency with resistance Rc=10 kOhms , R=8 kOhms. | CO3 | 5 |
|  | b. | State the Barkhausen Criterion | CO3 | 5 |
|  | c. | Describe the operation of a transistor astable multivibrator using two identical transistors Q1 and Q2 | CO3 | 10 |

ALL THE BEST