Reg.No. ____________

End Semester Examination – Nov/Dec – 2017

| Code : | 17EC3003 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | OPTICAL NETWORKS AND PHOTONIC SWITCHING | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Criticize the research problems of the fiber bragg grating based add-drop multiplexer, Describe in detail about the theoretical concept with a neat sketch. | CO1 | 10 |
|  | b. | Discuss the working of mechanical and thermo-optic switches. | CO2 | 6 |
|  | c. | Defend the concept of polarization-independent acousto-optic tunable filters, with a neat sketch. | CO2 | 4 |
| (OR) |  |  |  |  |
| 2. | a. | Write a short note on the working of optical circulator and multiplexers. | CO2 | 6 |
|  | b. | Enumerate the concept of optical amplifier over regenerators. | CO2 | 4 |
|  | c. | Validate a few research problems of an Erbium doped filter amplifier with its literature; explain in detail with a neat sketch. | CO2 | 10 |
| 3. | a. | How the mapping of lower-speed asynchronous streams into virtual tributaries in SONET? With a neat sketch. | CO1 | 10 |
|  | b. | Explain in detail about the different types of SONET equipment deployed in a network. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Describe in detail about reconfigurable OADM architectures. - A partially tunable OADM using a serial architecture with fixed-wavelength transponders. - A fully tunable OADM using a parallel architecture with tunable transponders. | CO2 | 10 |
|  | b. | Write a short note on i. Broadcast select ROADM, ii. wavelength blocker, and iii. WSS based ROADM. | CO2 | 10 |
| 5. | a. | Which sublayer within the optical layer would be responsible for handling the following functions? - Setting up and taking down lightpaths in the network. - Montoring and changing the digital wrapper overhead in a lightpath. - Detecting a fiber cable cut in a WDM line system. | CO2 | 10 |
|  | b. | Show that in a ring architecture if the protection capacity is less than the working capacity, then service cannot be restored under certain single failure conditions. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Consider a WDM ring built using optical add/drop multiplexers, supporting two interconnected SONET line terminals and two interconnected IP routers using protection provided by the SONET and IP layers respectively. Carefully characterize the set of simultaneous multiple fiber cuts that can be handled by this arrangement. | CO2 | 10 |
|  | b. | Construct a traffic distribution for which the traffic-carrying capacity of a BLSR/4 is maximized. What is this capacity as a multiple of the bit rate on the working fibers? | CO2 | 10 |
| 7. | a. | Explain synchronization in photonic packet switching network, with an example. | CO3 | 5 |
|  | b. | How a wavelength assignment is done? Illustrate the concept with an example. | CO3 | 10 |
|  | c. | Differentiate between photonic switching and electronic switching. | CO3 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Demonstrate the concept of allocating bandwidth in an enhanced HFC network with a neat sketch. | CO3 | 15 |
|  | b. | How to measure the performance and fault in an optical layers? | CO3 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Consider the open fiber control protocol in the fiber channel standard. How would you choose the parameters  and ’ a function of the maximum link propagation delay dprop? | CO1 | 10 |
|  | b. | What are the bottlenecks of the first generation optical networks over second generation? | CO1 | 10 |

ALL THE BEST