Reg.No. ____________

End Semester Examination – Nov/Dec – 2017

| Code : | 17EC3063 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ADVANCED DIGITAL SIGNAL PROCESSING | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Test the periodicity of the signal x[n] = [-1]n u[n] and determine the fundamental period. | CO1 | 5 |
|  | b. | Check whether the signal x[n] is a power or energy signal and find the value of power or energy. | CO1 | 5 |
|  | c. | Check whether the following systems are linear, causal, dynamic and time invariant i. y[n] = x[1/2n] ii. y[n] = cos x[n] | CO4 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Let x2ev[n] & x2od[n] denote, respectively the even and odd parts of a square- summable sequence x[n]. Prove the following result: n=−∞∞x2n=n=−∞∞xev2n+n=−∞∞xod2n. | CO2 | 10 |
|  | b. | If x[n]=n2+n0. Plot the sequence x[n] and hence find i. 2 x[2-2n] ii. x[n+1] +x[n-1] iii. x[3-n/4] | CO2 | 10 |
| 3. | a. | The following signals are defined on the interval n = 0, 1, 2, 3. x1[n] = (1/2)n x2[n] = (-1)n. Compute the 4 point circular convolution using DFT. Use matrix method for solving DFT. | CO3 | 6 |
|  | b. | Convolve the signals x1[n] = [n] - [n-1] + 2[n-3] and x2[n] = 3[n] -[n-2]+ [n-4] using the z transform. | CO3 | 6 |
|  | c. | Determine the Transfer Function and Impulse response for the causal LTI System described by the difference equation Y[n]-(1/4)y[n-1]-(3/8)y[n-2]= -x[n]+2x[n-1]. | CO4 | 8 |
| (OR) |  |  |  |  |
| 4. | a. | Obtain the response of an LTI system whose impulse response h[n] = {2,1} and input x[n] = { 1,2,3} using DIT FFT algorithm. | CO2 | 12 |
|  | b. | Prove that IDFT can be evaluated by using DFT Algorithm. | CO2 | 4 |
|  | c. | Find the DFT of Discrete signal {x[n-1]} in terms of X[k], if X[k]=DFT of the Discrete signal{x[n]}. | CO2 | 4 |
| 5. | a. | Obtain the direct form I, direct form II and Cascade realization of the LTI system H (z) =1+3z−1+2z−21+0.375z−1−0.09z−2−0.015z−3. | CO6 | 10 |
|  | b. | The system function of the analog filter Ha(s) is given below. Determine H[z] using bilinear transformation method. Assume T=0.1sec. Ha(s)=2s2+3s+2. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Derive the expression for 3 phase realization of a length-9 FIR filter. Draw the canonical realization for the same. | CO6 | 10 |
|  | b. | Enumerate the steps to design an FIR filter using Frequency sampling Technique. | CO3 | 5 |
|  | c. | Realize the canonic cascade structure of H(z) = 15+12z−1+15z−217+14z−1+17z−2 | CO6 | 5 |
| 7. | a. | Design a linear phase FIR band pass filter to pass frequencies in the range c1 = 0.35π radians/sample to c2 = 0.48π radians/sample using rectangular window by taking 7 samples of the window sequence. | CO3 | 15 |
|  | b. | Write the algorithm for software implementation of digital Butterworth IIR filter design. | CO3 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Design a linear phase FIR digital filter for the given specifications using Hamming window of length M=7. Hd(ω)=e−j3ω for ω ≤ π6 0 for π6< ω ≤ π | CO3 | 10 |
|  | b. | What are the characteristics of the window that affect the frequency response of an FIR filter? Compare the characteristics of a few windows used in FIR filter design. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain with block diagram the implementation of Audio-Sub band Coding and Decoding system. | CO5 | 10 |
|  | b. | Prove the following Cascade Equivalence. | CO6 | 5 |
|  | c. | Explain how a fraction of sampling time (kTs/L, k and L are integers) phase shift can be effected for the input signal using multi-rate signal processing. | CO5 | 5 |

ALL THE BEST